3GPP TSG-RAN WG1 Meeting #104bis-e	Tdoc R1-2103662

eMeeting, April 12th – 20th, 2021

Agenda Item:	7.2.11

Source:	Ericsson

Title:	Remaining details of Rel-16 NR UE features

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss the remaining details of Rel-16 NR UE features, in particular the FG definition for the interpretation of pdcch-MonitoringAnyOccasionsWithSpanGap for cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell.

The discussion is based on the latest RAN1 agreed feature list in [1].

# 2	Discussion

## 2.1	MR-DC/CA enhancement

Below agreements were made in RAN1#104-e meeting.

Agreement

Regarding the interpretation of UE capabilities in case of cross-carrier operation, for cross-carrier scheduling with the same SCS in the scheduling cell and the scheduled cell, RAN1 clarifies that support of pdcch-MonitoringAnyOccasionsWithSpanGap is based on both the support of this capability for the band of the scheduled/triggered/indicated cell and the support of this capability for the band of the scheduling/triggering/indicating cell.

- Note: For pdcch-MonitoringAnyOccasionsWithSpanGap, the supported set (set1, set2 or set 3) for cross-carrier scheduling with the same SCS in the scheduling cell and the scheduled cell is still based on the indicated value for the band of the scheduling cell.

Agreement

For cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell, RAN1 introduces new Rel-16 UE capability to clarify the interpretation of pdcch-MonitoringAnyOccasionsWithSpanGap in case of cross-carrier operation.

- The detailed design of this new Rel-16 UE capability is to be discussed under Rel-16 UE feature session.

Agreement

LS to RAN2 on interpretation of UE features in case of cross-carrier operation is endorsed in R1-2102085

For the new Rel-16 capability, we propose the following FG definition.

Proposal : Adopt below FG definition for the interpretation of pdcch-MonitoringAnyOccasionsWithSpanGap for cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell.

| 18. MR-DC/CA enhancement | 18-5e | Handling of pdcch-MonitoringAnyOccasionsWithSpanGap) for cross-carrier scheduling and with different numerology between scheduling cell and scheduled cell. | Indicates condition under which the UE supports pdcch-MonitoringAnyOccasionsWithSpanGap for cross-carrier scheduling with different SCS in the scheduling cell and the scheduled cells. Candidate values are {‘both’, ‘scheduling cell’} - When ‘both’ is reported, - Condition is satisfied only when UE indicates support of pdcch-MonitoringAnyOccasionsWithSpanGap for the band of the scheduled/triggered/indicated cell and indicates support of pdcch-MonitoringAnyOccasionsWithSpanGap for the band of the scheduling/triggering/indicating cell. - When ‘scheduling cell only’ is reported, - Condition is satisfied when UE indicates support of pdcch-MonitoringAnyOccasionsWithSpanGap for the band of the scheduling/triggering/indicating cell. | 18-5/5b, 3-5b | Yes | N/A |  | Per UE | N/A | N/A | N/A | Note: For pdcch-MonitoringAnyOccasionsWithSpanGap, the supported set (set1, set2 or set 3) for cross-carrier scheduling with different SCS in the scheduling cell and the scheduled cell is still based on the indicated value for the band of the scheduling cell. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 3	Conclusions

In this contribution we discussed the remaining details of Rel-16 NR UE features and make the following proposals:

- Adopt FG definition for the interpretation of pdcch-MonitoringAnyOccasionsWithSpanGap for cross-carrier scheduling with different SCSs in the scheduling cell and the scheduled cell as described in section 2.1.

# References

- R1-2102006, "Updated RAN1 UE features list for Rel-16 NR after RAN1#104-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#104-e, February 2021.