3GPP TSG-RAN WG1 Meeting #104b-e	R1-2103740

e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.16

Source:	Ericsson, Polaris Wireless, Verizon, Intel, NTT Docomo, China Telecom, FirstNet, Deutsche Telekom

Title:	NR positioning support for TA-based positioning in E-CID (TEI)

Document for:	Discussion, Decision

- Introduction

The E-UTRA-based positioning solutions defined in LPPa and NRPPa included the use of Timing Advance (TA)-based measurement as part of E-CID. As this feature has no equivalent in NR, it was observed during the Rel-17 study item that there was a gap in the NR positioning solution compared to LTE. It was in fact previously proposed by [1] to align NR and LTE by including the TA-based methods in NR E-CID.

In this TEI contribution, we discuss how important it is to address the gap in the NR specifications by aligning the NR positioning functionality with LTE at least on the TA type 2, since it seems to have the minimum impact. We also propose to discuss whether RAN1 can agree to introduce the full TA reporting in NR including type 1 as well:

- Introduce the NR type 2 TA measurement type, as defined in LTE, into the NR specification;

- Discuss whether to introduce NR TA Type 1 as well, in order to have the full reporting of NR TA in NRPPa

- discussion

## Motivation

TA-based methods were already available in LTE for E-CID positioning, which were mainly used for meeting important regulatory requirements (e.g. localization during emergency calls). It is essential that when operators deploy NR Network and migrate from the LTE-based TA type 1/type2 solutions, NR positioning should also be capable of providing this same important functionality, instead of a downgrading, which we have today.

Specifically, not every network element (gNBs, LMFs) or UEs may support Rel-16 UL SRS or DL-PRS based positioning – this is especially true in multi-vendor scenario environments, where network elements are coming from different vendors, and hence the possibility of interoperability issues which may lead to longer deployment period. Therefore, solutions similar to LTE which are based upon simple basic communication procedure such as TA should be made available to address such migration issue in a timely manner, and meet the same requirement for 5GS on providing UE location information during emergency calls.

Measurements and reporting similar to LTE TA Type1, Type2 are also needed in NR to provide positioning solutions to meet regulatory requirements and to ensure seamless positioning solutions when migrating from LTE to NR.

Besides helping to comply with regulatory requirements, TA measurements could also improve latency. Release 17 NR positioning use cases have considerably lower latency requirements compared to Release 16 and in [1] it was indeed observed that the latency could be greatly reduced if the network can report existing measurements based on timing advance, instead of using the PRS-based gNB Rx-Tx time difference measurement.  Moreover, re-using already available measurement such as TA could help complementing the other measurement reports (such as PRS/SRS based reports) without introducing more RS overhead.

The network can report TA-based gNB RxTx time difference measurement without additional RS overhead cost or additional LPP signalling.

## TA-based positioning in the specifications

When looking at the LPPa/NRPPa specifications, in E-UTRA RAT the eNBs/ng-eNBs are able to report E-UTRA Angle of Arrival and Timing Advance Type 1/Type 2 in E-CID to the location server. Timing Advance Type 2, which is eNB Rx-Tx time difference measurement, is based upon PRACH (as seen in TS 36.214 extract below), which is considered a usual and required measurement to be signalled by UE to gNB in order to perform communication. Thus, it should already be supported by all networks.

### 5.2.4 Timing advance (TADV) Definition; Type1: Timing advance (TADV) type 1 is defined as the time difference TADV = (eNB Rx – Tx time difference) + (UE Rx – Tx time difference), where the eNB Rx – Tx time difference corresponds to the same UE that reports the UE Rx – Tx time difference. Type2: Timing advance (TADV) type 2 is defined as the time difference TADV = (eNB Rx – Tx time difference), where the eNB Rx – Tx time difference corresponds to a received uplink radio frame containing PRACH from the respective UE or similarly NPRACH from the respective NB-IoT UE...

In this respect, Type 2 TA is only dependent on the gNB reporting and does not introduce any new measurement or reporting from the UE side, or any big system level impact. Therefore, the specification impact to support TA type 2 would be very limited, and it would not impact the UE.

TA type 2 does not impact the UE

To provide the full solution as in LTE and address this RAT migration downgrade, reporting of TA type 1 is also necessary. However, the inclusion of type 1 TA may have more specification impacts on RRC and on the UE. RAN1 can further discuss whether it is worth introducing both TA types in NR as TEI or introducing Type 2 only and further studying the introduction of type 1 in RAN1 during release 17.

While the addition of TA type 2 can be simple and straightforward, from 3GPP point of view, type1 TA can have more specification impacts (e.g RRC) and on Rel-16 UEs

Considering the important issues of interoperability and RAT migration, as well as the difficult contexts of 2020 and 2021 years - making positioning a crucial technology - we propose to focus this TEI on addressing the gap between LTE and NR by introducing the NR type-2 TA, similar to LTE definition. The specifications changes will be the timing advance definition in TS 38.215, and support of NRPPa reporting of NR TA as part of NR E-CID. For the NRPPa change, the reader can find an example in the Annex section of this paper. For the 38.215 changes, a draft CR can be found in [2].

Define the timing advance measurement for NR as follow:

- Type2:

- Timing advance (TADV) type 2 is defined as the time difference

- TADV = (gNB Rx – Tx time difference),

- where the gNB Rx – Tx time difference corresponds to a received uplink radio frame containing PRACH from the respective UE..

Extend the gnodeB Rx-Tx definition to include the PRACH based measurement:

- TgNB-RX is the Transmission and Reception Point (TRP) [18]  received timing of uplink subframe #i containing SRS or PRACH associated with UE, defined by the first detected path in time.

- TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE.

- Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS.

- PRACH is used to determine the start of one subframe containing PRACH.

RAN1 to discuss whether to include type 1 TA measurement and reporting as well, or leaving  it for further study for RAN1 during rel17 positioning enhancements

Send an LS to RAN2 and RAN3 with the agreement to add Type 2 TA reporting for NR so that their corresponding specification changes can be updated.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Measurements and reporting similar to LTE TA Type1, Type2 are also needed in NR to provide positioning solutions to meet regulatory requirements and to ensure seamless positioning solutions when migrating from LTE to NR.

Observation 2	The network can report TA-based gNB RxTx time difference measurement without additional RS overhead cost or additional LPP signalling.

Observation 3	TA type 2 does not impact the UE

Observation 4	While the addition of TA type 2 can be simple and straightforward, from 3GPP point of view, type1 TA can have more specification impacts (e.g RRC) and on Rel-16 UEs

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define the timing advance measurement for NR as follow:

Type2:

Timing advance (TADV) type 2 is defined as the time difference

TADV = (gNB Rx – Tx time difference),

where the gNB Rx – Tx time difference corresponds to a received uplink radio frame containing PRACH from the respective UE..

Proposal 2	Extend the gnodeB Rx-Tx definition to include the PRACH based measurement:

	TgNB-RX is the Transmission and Reception Point (TRP) [18]  received timing of uplink subframe #i containing SRS or PRACH associated with UE, defined by the first detected path in time.

	TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE.

	Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS.

	PRACH is used to determine the start of one subframe containing PRACH.

Proposal 3	RAN1 to discuss whether to include type 1 TA measurement and reporting as well, or leaving  it for further study for RAN1 during rel17 positioning enhancements

Proposal 4	Send an LS to RAN2 and RAN3 with the agreement to add Type 2 TA reporting for NR so that their corresponding specification changes can be updated.

- References

R1-2006732, Discussion on potential techniques for NR Positioning Enhancements, NTT DOCOMO, inc., RAN1#102e, 2020

R12103741 draft CR for 38.215, Ericsson, RAN1#104b-e

- Annex

The following presents the example of changes that can be introduced to NRPPA TS 38.455 at tabular level in order to add the reporting of NR TA type 2 in NR E-CID.

START OF CHANGES TO TS 38.455

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate E-CID measurements.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15 ,…,256) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min applies only for ng-eNB. | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | EACH | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SS-RSRP, SS-RSRQ, CSI-RSRP, CSI-RSRQ, NR Angle of Arrival, NR Timing Advance Type 2) |  | - | - |
| Other-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >Other-RAT Measurement Quantities Item | M |  | ENUMERATED (GERAN, UTRAN,…, NR, EUTRA) |  |  |  |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

NEXT CHANGE

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. If the Geographical Coordinates IE is used, the NG-RAN Access Point Position IE shall be ignored. | - |  |
| Measured Results |  | 0 .. <maxnoMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>Value Angle of Arrival EUTRA | M |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] | - |  |
| >>Value Timing Advance Type 1 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36. 214 [17] | - |  |
| >>Value Timing Advance Type 2 EUTRA | M |  | INTEGER (0..7690) | According to mapping in TS 36. 214 [17] | - |  |
| >>Result RSRP EUTRA |  | 1 .. <maxCellReport> |  |  | - |  |
| >>> PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>EARFCN | M |  | INTEGER (0.. 262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  | - |  |
| >>Result RSRQ EUTRA |  | 1 . <maxCellReport> |  |  | - |  |
| >>> PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>EARFCN | M |  | INTEGER (0..262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>> CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  | - |  |
| >>Result SS-RSRP |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>Value SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>SS-RSRP per SSB Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>Result SS-RSRQ |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>Value SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>SS-RSRQ per SSB Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>Result CSI-RSRP |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>Value CSI-RSRP Cell | O |  | INTEGER (0..127) | CSI-RSRP measurement aggregated at cell level | - |  |
| >>>CSI-RSRP per CSI-RS Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>Value CSI-RSRP | M |  | INTEGER (0..127) | CSI-RSRP measurement per CSI-RS resource | - |  |
| >>Result CSI-RSRQ |  | 1 .. <maxCellReportNR> |  |  | YES | ignore |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>Value CSI-RSRQ Cell | O |  | INTEGER (0..127) | CSI-RSRQ measurement aggregated at cell level | - |  |
| >>>CSI-RSRQ per CSI-RS Resource |  | 0 .. <maxIndexesReport> |  |  | - |  |
| >>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>Value CSI-RSRQ | M |  | INTEGER (0..127) | CSI-RSRQ measurement per CSI-RS resource | - |  |
| >>Angle of Arrival NR | M |  | UL Angle of Arrival 9.2.38 |  | YES | ignore |
| >> Value Timing Advance Type 2 NR | M |  | gNB Rx-Tx Time Difference 9.2.40 | Only based on PRACH measurement, as defined in TS 38.215 [18] | YES | ignore |
| Geographical Coordinates | O |  | 9.2.46 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |
| maxCellReportNR | Maximum no. of NR cells that can be reported with one message. Value is 9. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |