3GPP TSG RAN WG1 #104bis-e			R1-2103815

e-Meeting, April 12th – 20th, 2021

Agenda item:	7.2.12

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on [104b-e-NR-Rel16-TEI-02]

Document for:	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion.

[104b-e-NR-Rel16-TEI-02] Email discussion/approval on other Rel-16 NR related issues, till 4/16 (Hiroki, DCM)

- Whether/how to adopt CR for TR38.821 with considering R1-2103719 and R1-2103720

- Whether/how to adopt CR for correction on RIM RS resource and set ID mapping with considering R1-2103747

- Discussion on the issues

  - Correction of round trip delay drift rate for NTN scenarios

Following proposals are made in contributions.

| [12] | The propagation delay between the LEO satellite and UE can be calculated using the associated propagation conditions. In TR 38.821, it was assumed that the round-trip delay between the gNB and UE (i.e., Timing Advance) for transparent NTN is four times the delay between the satellite and UE. For a LEO satellite at altitude h, assuming the earth radius is a constant R≈6378 km, the satellite speed can be calculated as Vsat=gR2R+h; (1). where g≈9.81 m/s2 is the earth’s gravitational constant. Assuming the LEO orbit is a circle of radius (R+h) centered around the earth, the timing advance (TA) drift rate can also be calculated by using (1). Figure 1 (a) and (b) shows the TA of a transparent LEO at h=600 km and its drift rate as a function of the UE’s elevation angle, where it is assumed the TA is 4× the LEO-to-UE propagation delay and the UE is stationary. From the figure it can be seen that the TA drift rate is higher at a lower elevation angle, e.g. at 10⁰ elevation angle, the TA drift rate is approximately 90.8 μs/s. When considering the worst case scenario with the UE at 10⁰ elevation angle and VUE=1200 km/h (based on Table 4.2-2 of [1]), 2 VUE/c≈2.22 μs/s (corresponding to the UE moving in the direction towards the satellite) and results in the worst case TA drift rate of ±93.0 μs/s. The positive rate is when the LEO and UE are moving away from each other (when the elevation angle is decreasing) and the negative rate is when the LEO and UE are moving toward each other (when the elevation angle is increasing). Table 1, for both transparent and regenerative LEO at 600 km altitude. (a); (b). Figure 1. Round-trip delay (i.e., TA) and round-trip delay variation (i.e., TA drift rate) for LEO at 600 Km altitude in (a) and (b) respectively, when the UE is stationary Table 1 TA drift values for GEO600 case UE velocity; LEO transparent payload TA drift value; LEO regenerative payload TA drift value. 0 km/h; ±90.8 s/sec; ±45.4 s/sec. 1200 km/h; ±93.0 s/sec; ±47.6 s/sec. The TA drift rate, or “round-trip delay variation,” can also be calculated analytically. One easy approach is suggested in [2] by considering the relative velocity between the satellite and UE as illustrated in Figure 2. Assuming feeder link delay is the same as service link delay, i.e., the gateway is located near the UE. The drift rate of feeder link part of TA can then be calculated as dTAFLdt=2 Vsat'c; (2). and the corresponding drift rate of service link part of TA dTASLdt=2 Vsat'-VUE'c ,; (3). where Vsat' and VUE' are respectively the satellite’s velocity and the UE’s velocity projected onto the direction of UE-to-satellite. The worst case scenario is when the UE’s elevation angle is 10⁰ and the UE is moving in the direction of UE-to-satellite. In that case, dTAdt=±4Vsat'+2VUE'c ,; (4). where Vsat' is evaluated at 10⁰ elevation angle. Using equation (4) results in a TA drift rate of above +/-90μs/sec for the worst case in scenarios C1/C2, as mentioned above. This result is consistent with the mathematical framework in [2] with the note that the calculations in [2] did not consider the necessary additional factor 2 in order to account for the round-trip nature of the TA drift rate from both service and feeder links. This is the reason why values shown in the highlighted area of Table 7.1-1 above differ from the values analytically calculated here. Figure 2. LEO satellite and UE in TA drift rate calculation. Observation 1: The upper bound of TA drift values for the LEO600 case will exceed ±90 s/sec Proposal 1: Update the TA drift values of Table 7.1-1 of 38.821 to reflect its round-trip nature by considering values as presented in Table 1. |
| --- | --- |
| [13] | ## 7.1 Requirements and key issues ### 7.1.1 Delay In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint. Table 7.1-1: NTN scenarios versus delay constraints, Source [2] NTN scenarios; A; B; C1; C2; D1; D2. GEO transparent payload; GEO regenerative payload; LEO transparent payload; LEO regenerative payload. Satellite altitude; 35786 km; 600 km. Relative speed of Satellite with respect to earth; negligible; 7.56 km per second. Min elevation for both feeder and service links; 10° for service link and 10° for feeder link. Typical Min / Max NTN beam foot print diameter (note 1); 100 km / 3500 km; 50 km / 1000 km. Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE; 541.46 ms (Worst case); 270.73 ms; 25.77 ms; 12.89 ms. Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE; 477.48 ms; 238.74 ms; 8 ms; 4 ms. Maximum Round Trip Delay variation as seen by the UE (note 2); Negligible; Up to +/- 93.0 µs/sec (Worst case); Up to +/- 47.6 µs/sec. NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time. The worst case is when the satellite moves towards/away from the UE at 10⁰ elevation angle, assuming the UE speed is 1200 km/h. It is expressed in µs/s and is negligible for GEO scenario. NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s.. |

Based on the above contributions, following proposal can be discussed.

FL proposal #1

- Adopt CR for TR38.821 as in R1-2103720

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| Huawei | Agree |
| ZTE | Agree with the identified issue, and the worst case occurs once the UE and gateways are co-located with minimum elevation angle. But w.r.t this CR, since the previous value is done by assuming different minimal elevation angle, prefer to discuss it under section 8.4.2 to achieve the alignement on the assumption firstly. |
| Nokia, NSB | We obviously can accept the proposal and as such are not asked to comment. Just to note in response to ZTE, that this is a TP related to a REl-16 technical report and as such belongs under Rel-16 maintenance and not under any ongoing Rel-17 item. Normally, one would not consider elevation angles lower than 10 degrees (basically satellite would be more or less under the horizon or obstructed by buildings/vegetation), and UE speed higher than 1200 km/h does not make sense either. So we targeted worst case scenario for the worst case number and now the question is if we can agree to a CR based on these assumptions. The one procedural thing to note is that RAN3 is in charge of the TR, so we as RAN1 have the power to endorse the TP, but in principle need RAN3 to agree to the CR (or have RAN do this directly) |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! Based on the feedbacks, let’s check if the proposed change can be endorsed as TP so that we can send LS to RAN3. Draft TP is available in the draft folder. |

Updated FL proposal #1

- Endorse TP for TR 38.821 in R1-21xxxxx

  - Send LS to RAN3 to inform the endorsed TP

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |

Based on the discussion, RAN1 chair declared following.

- TP for TR 38.821 in R1-2103936 is endorsed.

  - It is to be enclosed as the LS to RAN3

  - Correction on RIM RS resource and set ID mapping

Following proposals are made in contributions.

| [15] | In the RAN1#96, the following definition of the mapping between set ID and RIM-RS resource indies was agreed in R1-1903589. For transmission, a gNB is configured with one or multiple set ID(s) nsetID. Then the RIM RS resource indices it,if,is for transmission are determined as follows: is=Sstart+nsetID mod ns it=Tstart+itR+r, with r=0,1,⋯,R-1, and it=modnsetIDns, NT if=modnsetID NTns, NF where - nsetID is the set ID configured for RS1 or RS-2; - Sstart is start sequence index; - For RS-1 with “Enough”/ “Not enough” indication as OFF, Sstart=0 - For RS-1 with “Enough”/ “Not enough” indication as ON, - Sstart=0, for indicating “Not enough” - Sstart= S1/2, for indicating “Enough” - For RS-2, Sstart=0 - ns is the number of sequence candidates for the current functionality - For RS-1 with “Enough”/ “Not enough” indication as OFF, ns=S1 - For RS-1 with “Enough”/ “Not enough” indication as ON, ns=S1/2 - For RS-2, ns=S2 - Tstart is starting time-domain offset; - For RS-1, Tstart=0 - For RS-2, Tstart is determined as Tstart=NSetIdRS1NF∙S1∙R1，if the indication for “enough” or “not enough” is OFF or Tstart=2∙NSetIdRS1NF∙S1∙R1, if the indication for “enough” or “not enough” is ON - NT is the number of set IDs to be differentiated via time-domain; - For RS-1, NT =NSetIdRS1NF∙S1，if the indication for “enough” or “not enough” is OFF or - For RS-1, NT =2∙NSetIdRS1NF∙S1, if the indication for “enough” or “not enough” is ON - For RS-2, NT=NSetIdRS2NF∙S2 - R is the number of consecutive TDD DL/UL switching periods; - For RS-1, R=R1 - For RS-2, R=R2 Conversely, for reception, gNB obtains the set ID from the RIM RS resource indices it,if,is of detected RIM RS based on the mapping rule, as follows, nsetID=is-Sstart+nsit-TstartR+NTnsif. But in the latest version of specification TS 38.211 clause 7.4.1.6.4.5, the definition of R is missing. The resource indices itRIM, ifRIM, and isRIM are determined from the index r in the set ID nsetID according to itRIM=Tstart+nsetIDNsRIM mod NtRIMR+r ifRIM=nsetIDNtRIMNsRIM mod NfRIM isRIM=Sstart+nsetID mod NsRIM where - NtRIM is given by NtRIM=NsetIDRIM,1NfRIMNsRIM,1for RIM-RS type 1 and if EnoughIndication is disabled2NsetIDRIM,1NfRIMNsRIM,1for RIM-RS type 1 and if EnoughIndication is enabled NsetIDRIM,2NfRIMNsRIM,2for RIM-RS type 2 - NfRIM∈1,2,4 is the number of candidate frequency resources configured in the network; - NsRIM is the number of sequence candidates for the current RIM-RS resource given by NsRIM=NsRIM,1for RIM-RS type 1 and if EnoughIndication is disabledNsRIM,12for RIM-RS type 1 and if EnoughIndication is enabled NsRIM,2for RIM-RS type 2 - Tstart is the starting time offset given by Tstart=0for RIM-RS type 1NsetIDRIM,1NfRIMNsRIM,1R1for RIM-RS type 2 and if EnoughIndication is disabled 2NsetIDRIM,1NfRIMNsRIM,1R1for RIM-RS type 2 and if EnoughIndication is enabled - Sstart is given by Sstart=NsRIM,12if EnoughIndication is enabled and 'enough mitigation' is to be indicated 0otherwise where NsRIM,1 is the number of candidate sequences assigned for RIM-RS type 1 - r∈0,1,…,Ri-1 where Ri is the number of consecutive uplink-downlink periods for RIM-RS type i as given by clause 7.4.1.6.4.2;. - Obviously, the above blue color highlighted part is not the exact definition of R. Thus, it is neccesary to add the definition of R in order to make the specification clear. ###### 7.4.1.6.4.5 Mapping between resource triplet and set ID The resource indices itRIM, ifRIM, and isRIM are determined from the index r in the set ID nsetID according to itRIM=Tstart+nsetIDNsRIM mod NtRIMR+r ifRIM=nsetIDNtRIMNsRIM mod NfRIM isRIM=Sstart+nsetID mod NsRIM where - NtRIM is given by NtRIM=NsetIDRIM,1NfRIMNsRIM,1for RIM-RS type 1 and if EnoughIndication is disabled2NsetIDRIM,1NfRIMNsRIM,1for RIM-RS type 1 and if EnoughIndication is enabled NsetIDRIM,2NfRIMNsRIM,2for RIM-RS type 2 - NfRIM∈1,2,4 is the number of candidate frequency resources configured in the network; - NsRIM is the number of sequence candidates for the current RIM-RS resource given by NsRIM=NsRIM,1for RIM-RS type 1 and if EnoughIndication is disabledNsRIM,12for RIM-RS type 1 and if EnoughIndication is enabled NsRIM,2for RIM-RS type 2 - Tstart is the starting time offset given by Tstart=0for RIM-RS type 1NsetIDRIM,1NfRIMNsRIM,1R1for RIM-RS type 2 and if EnoughIndication is disabled 2NsetIDRIM,1NfRIMNsRIM,1R1for RIM-RS type 2 and if EnoughIndication is enabled - Sstart is given by Sstart=NsRIM,12if EnoughIndication is enabled and 'enough mitigation' is to be indicated 0otherwise where NsRIM,1 is the number of candidate sequences assigned for RIM-RS type 1 - r∈0,1,…,Ri-1 where Ri is the number of consecutive uplink-downlink periods for RIM-RS type i as given by clause 7.4.1.6.4.2; - R is given by R=R1for RIM-RS type 1 R2for RIM-RS type 2 The set ID is determined from the resource triplet according to nsetID=isRIM-Sstart+NsRIMitRIM-TstartRi+NtRIMNsRIMifRIM |
| --- | --- |

Based on the above contributions, following proposal can be discussed.

FL proposal #2

- Adopt CR for correction on RIM RS resource and set ID mapping as in R1-2103747

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |
| Intel | The proposed CR is reasonable as it clarifies the definition of the parameter R in clause 7.4.1.6.4.5 of TS 38.211. Alternatively, the set ID mapping can be corrected as itRIM=Tstart+nsetIDNsRIM mod NtRIMRi+r |
| Qualcomm | The proposal is correct. Intel’s version is a cleaner fix. |
| Huawei | Agree |
| Ericsson | Fine with the proposal and the version suggested by Intel. |
| Huawei, HiSilicon | Thank Intel for the suggested alternative verision. More comments to follow up. We prefer our version. But we are OK with Intel’s revision if the following revision is also accepted. Because Ri becomes the direct parameter in the concerned i^RIM_t, it is supposed to be moved out at the same level of indentation as parameter r. In other words, better to have this companion revision for Intel’s version. |
| Moderator (NTT DOCOMO) | Thanks for the feedbacks! Based on the feedbacks, let’s check if the Intel’s version with Huawei’s additional revision is acceptable to all. Draft CR is available in the draft folder. |

Updated FL proposal #2

- Adopt CR for correction on RIM RS resource and set ID mapping in R1-21xxxxx

Companies are encouraged to check above FL proposal and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposal:

| Company | Comment |
| --- | --- |

Based on the discussion, RAN1 chair declared following.

- CR for TS38.211 for correction on RIM RS resource and set ID mapping in R1-2103937 is approved.

- Conclusion

Based on the discussion, RAN1 chair declared followings.

- TP for TR 38.821 in R1-2103936 is endorsed.

  - It is to be enclosed as the LS to RAN3

- CR for TS38.211 for correction on RIM RS resource and set ID mapping in R1-2103937 is approved.

# Reference

[1]	R1-2102376	Enhancement on uplink power control for M-TRP	OPPO

[2]	R1-2102377	Text Proposals for Tx Switching between Two Uplink Carriers	OPPO

[3]	R1-2102491	Remaining Issues of Rel-16 UL Tx Switching	ZTE

[4]	R1-2102492	Remaining Issues of Half-duplex CA	ZTE

[5]	R1-2102585	Correction on half-duplex operation in CA with unpaired spectrum	CATT

[6]	R1-2102586	Discussion on SSB resources for RLM	CATT

[7]	R1-2102951	Discussion on Configuration for CA Half duplex operation	vivo

[8]	R1-2103149	Remaining issues for 1Tx-2Tx switching	Qualcomm Incorporated

[9]	R1-2103457	TP for 38.214 alignment CR	NEC

[10]	R1-2103502	Summary of Rel-16 uplink Tx switching	Moderator (China Telecom)

[11]	R1-2103647	Maintenance for other Rel16 topics	Ericsson

[12]	R1-2103719	Correction of round trip delay drift rate for NTN scenarios	Nokia, Nokia Shanghai Bell, Thales

[13]	R1-2103720	Correction of round trip delay drift rate for NTN scenarios	Nokia, Nokia Shanghai Bell, Thales

[14]	R1-2103746	Discussion on the remaining problems of supporting Tx switching between two uplink carriers	Huawei, HiSilicon

[15]	R1-2103747	Correction on RIM RS resource and set ID mapping	Huawei, HiSilicon