3GPP TSG RAN WG1 Meeting #104bis-e	R1-2103900

E-meeting, April 12 – 20, 2021

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

This summary includes three high level aspects to address HARQ-ACK feedback, PDSCH repetition, and CSI feedback as in the last meeting. In each of high level issue, a sub-level list of issues are organized.

For each of listed issue, proposal(s) is/are suggested from moderator’s perspective according to the submitted individual company’s proposal(s). Companies are welcome to make comments in the table “collect views”. The proposals may be updated in subsequent rounds according to the comments collected in previous rounds so as to strive to converge to consensus. Note that moderator may only tend to collect concerns when time is right for some specific rounds, for which companies are expected to only provide concerns in the table “collect concerns” if any instead of inputting views again and again to alleviate efforts.

People can use “navigation pane” to quickly overview the organization of the summary and proposal(s) for each issue for discussion and provide views/comments into the table of “collect view”/“collect concerns” under each proposal.

# HARQ-ACK feedback

## HARQ-ACK feedback options

Background

ACK/NACK based HARQ-ACK feedback has been agreed in the last meeting. This meeting will discuss whether to support NACK-only based feedback and how to indicate the HARQ-ACK feedback option if NACK-only based option is supported as well.

Submitted Proposals

(OPPO) Proposal 7:

  - NACK-only based HARQ feedback mechanism should be supported for RRC_CONNECTED UEs receiving MBS.

(Spreadtrum) Proposal 7:

  - For RRC_CONNECTED UEs receiving multicast, support NACK-only based HARQ-ACK feedback.

(ZTE) Proposal 8:

  - If NACK-only feedback is supported, consider the following method to determine the feedback mechanism for the UE between ACK/NACK feedback and NACK-only feedback:

- PUCCH resource sets containing ACK/NACK PUCCH resources and NACK-only PUCCH resources is configured for multicast for the UE.

- PRI in DCI is used to indicate the feedback mechanism and PUCCH resources from the PUCCH resource set.

(vivo) Proposal 3:

  - For RRC_CONNECTED UE,

- -	NACK only feedback is support for multicast (for both PTM scheme 1 & 2).

- -	ACK/NACK feedback is not supported for multicast with PTM transmission scheme 1.

- -	ACK/NACK feedback is supported for multicast with PTM transmission scheme 2.

(CATT) Proposal 6:

  - NACK-only based HARQ-ACK feedback is supported in MBS, and shared PUCCH resource is supported from the perspective of UEs in the same group.

(Nokia) Proposal 4:

  - NACK-only feedback on group-common PUCCH resources is the default HARQ-ACK feedback mechanism for MBS.

(Nokia) Proposal 8:

  - From the UE perspective, the UE should be configured to use a single mechanism per MBS service.

(Nokia) Proposal 9:

  - RAN1 further studies whether all UEs receiving an MBS service use the same HARQ-ACK feedback mechanism or whether different UEs can be configured with different mechanisms.

(Nokia) Proposal 10:

  - RAN1 further studies the means for semi-static / dynamic (re-)configuration of ACK / NACK and NACK-only based HARQ-ACK feedback mechanisms.

(MediaTek) Proposal 1:

  - support NACK-only based HARQ-ACK feedback mode for Rel-17 NR multicast service.

(MediaTek) Proposal 3:

  - Network can flexibly choose the HARQ-ACK mode and the HARQ feedback mode can be indicated dynamically by DCI field, e.g., “HARQ feedback option” field.

(FUTUREWEI) Proposal 5:

  - Support NACK-only feedback a working assumption and confirm later after the design is set

(ETRI) Proposal 1:

  - Support NACK-only based HARQ-ACK feedback scheme for MBS.

(ETRI) Proposal 2:

  - Support at least RRC configuration for UEs to decide which HARQ-ACK feedback scheme to use.

(Potevio) Proposal 1:

  - For RRC_CONNECTED UEs, NACK-only based HARQ-ACK feedback should be supported.

(Potevio) Proposal 2:

  - gNB could configure either one of HARQ-ACK feedback based on different scenarios/conditions/requirements.

(CMCC) Proposal 14:

  - NACK-only based HARQ-ACK feedback can be supported, especially for the case with a large amount of UEs in a multicast group.

(Intel) Proposal 1:

  - For RRC_CONNECTED UEs receiving multicast

- Both ACK/NACK based and NACK-only HARQ feedback is supported.

- ACK/NACK based feedback is used for delivery mode with PTP or PTM Scheme 2

- NACK only feedback is used for delivery modes 1 and 2 with PTM Scheme 1

- UEs within a group receiving multicast transmission can be configured with different HARQ feedback modes.

- (Intel) Proposal 4:

  - For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

    - Option 1: Semi-static RRC configuration of ACK/NACK or NACK only mode

    - Option 2: The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

    - Option 3: If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

- (Qualcomm) Proposal 1:

  - For RRC_CONNECTED UEs, support NACK-only for multicast HARQ feedback.

    - Support selection of UE-specific ACK/NACK and NACK-only for different UEs in the same group

- (Samsung) Proposal 6:

  - NACK-only HARQ-ACK feedback is not supported.

- (LGE) Proposal 1:

  - NACK only based HARQ-ACK is supported at least for PTM scheme 1.

- (Chengdu TD Tech) Proposal 1:

  - Support the following HARQ-ACK feedback methods for each SPS MRB of the PTM bearer for an MBS session:

    - ACK/NACK based HARQ-ACK feedback with the different SPS PUCCH resources for the different RRC_CONNECTE UEs.

      - FFS: details for the ACK/NACK based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

    - NACK-ONLY based HARQ-ACK feedback with the different RRC_CONNECTED UEs sharing the same SPS PUCCH resources

      - FFS: details for the NACK-ONLY based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

      - FFS: whether or not other information can be fed back with the NACK-ONLY information.

- (Convida) Proposal 1:

  - NACK-only based HARQ-ACK feedback should be supported for MBS.

- (Lenovo) Proposal 1:

  - NACK-only based HARQ-ACK feedback is supported for UEs receiving multicast.

- (Lenovo) Proposal 6:

  - Either Option 1 (NACK-only based HARQ-ACK feedback) or Option 2 (ACK/NACK-based feedback) is adopted based on gNB’s scheduling policy and PUCCH resource capacity.

- (NTT DOCOMO) Proposal 1:

  - Proposal 1: Support NACK-only based HARQ-ACK feedback for multicast.

- (Ericsson) Proposal 2:

  - NACK-only based HARQ-ACK feedback for multicast shall be supported.

- (Ericsson) Proposal 3:

  - We propose that the UEs can be dynamically reconfigured between NACK-only mode, ACK/NACK mode, and no feedback mode.

### Round-1

FL’s Comments

Per the submitted proposals, there are substantial support of NACK-only based HARQ-ACK feedback for multicast. Also, there are proposals regarding how to indicate which HARQ-ACK feedback option is to be in use, which could be next step discussion for details.

FL’s Proposal:

#### Proposal 2.1.1 (GTW/email)

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

- FFS details.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Support FL proposal. |
| ZTE | Ok with the proposal. But we do have some concerns on the workload of supporting three different HARQ-ACK mechanisms in Rel-17 (type1, type2 ACK/NACK and NACK-only). If NACK-only based mechanism is supported for MBS, we would prefer to define simplified version of it to reduce the workload. |
| CMCC | Support |
| LG | Support |
| CATT | Agree with FL proposal. |
| Nokia | Support From the analysis and link-level simulations presented in [8], we have the observations below that in our view, counter previously argued “problems” of NACK-only feedback : - With any reasonable system model, the concurrent reception of identical NACK-only signals sent from different UEs on the same physical resource, DOES NOT increase the probabilities of low received signal power or failure to detect a NACK, not even for the example of two UEs transmitting with identical average power. - It is true that the fast-fading state of the channel over which the receiver effectively receives the NACK, converges to having a complex Gaussian distribution with variance proportional to the number of UEs simultaneously transmitting the NACK (though not power squared). However, the previous conclusion drawn from this, i.e. that this leads to a requirement for an arbitrarily low detection threshold, is wrong. - Any energy threshold that can be used for detecting NACK of a single UE can also ( even more reliably) detect a NACK if multiple UEs transmit simultaneously and on the same radio resources. - A conventional PUCCH receiver can be readily applied for NACK-only detection on group-common resources. - There is no necessity to adjust the PUCCH DTX threshold depending on the number of UEs that are transmitting NACK on the common PUCCH resource. - In scenarios with an MBS audience of a meaningful size, PUCCH overhead of NACK-only feedback is—even with additional PTM-specific CSI reporting, cf. Section 2.4 of [8]—considerably lower than what is required in the case of ACK / NACK feedback, cf. Appendix 5.1 of [8]. - In combination with SPS there is no risk of DTX / ACK ambiguity in NACK-only feedback once the UE has received the SPS configuration and activation. |
| OPPO | Support the proposal |
| Samsung | Requires further discussion. For non-Rayleigh channels, there are error floors. Avoiding a “PDCCH DTX to ACK” error is not generally possible. |
| Qualcomm | Agree |
| Convida | Support. |
| FUTUREWEI | Support the proposal |
| ETRI | We support the proposal. |
| MTK | Support the proposal |
| Spreadtrum | We support the proposal |
| vivo | support the proposal. |
| NTT DOCOMO | We support the proposal. |
| Ericsson | We agree. As we have explained in our contribution, we wish to point out that there is a use case for NACK-only where only UEs with good link quality use NACK-only and UEs in more difficult conditions use ACK/NACK. |

## On how to organize the discussion of 2.3/2.4/2.5

### Round-1

Background

Given the situation of Round-1 discussion and before continuing the detailed proposals discussion, it would be good for us to strive for the same understanding on how to organize the discussion of sections 2.3/2.4/2.5

It is mainly regarding the PUCCH resources discussion in section 2.3, codebook generation discussion in section 2.4, and multiplexing/prioritizing discussion in section 2.5 since the issues could be twisted together.

The idea as moderator to organize the proposals is as follows:

- Whether network needs to configure UE to generate separate HARQ-ACK codebooks for unicast and multicast, respectively.

  - Since we have agreed a separate PUCCH-config for multicast can be configured and in URLLC feature two separate PUCCH-config is corresponding two codebooks to be transmitted.

  - Discuss it in section 2.4.1.2, two alternatives for down-selection later if controversial for now.

- Whether network needs to configure UE to generate a joint HARQ-ACK codebooks for unicast and multicast.

  - For Type-2: No way

    - We have agreed to concatenate the two sub-codebooks since DAI cannot be jointly counted between unicast and multicast.

  - For Type-1: No need

    - We have agreed to construct Type-1 codebook to base on the union TDRA set which solves the cases of TDM-ed unicast and multicast. How Type-1 codebook is constructed for the FDM-ed case is to be solved.

      - This is also essentially how the Type-1 codebook is constructed for multiplexing the codebook in the same PUCCH resource.

- Section 2.4 focuses on the Type-1 and Type-2 codebook construction itself.

  - So far the discussion is mainly about the construction for both unicast and multicast

    - The reason is that for multicast itself, Type-1/Type2 codebook construction has no difference for that for unicast. The cases of more than one multicast could be FFS separately.

    - The construction for both unicast and multicast is also essentially how the codebook is generated for multiplexing.

      - So the details for multiplexing the codebook is discussed in Section 2.4, need to look at Type-1 and Type-2 separately.

- Section 2.5 focuses only on the high level rule, i.e., multiplexing for the same priority and prioritizing for different priorities.

  - Sub-slot based PUCCH, two non-overlapping slot based PUCCH, and the PUCCH resources used for actual transmission could be discussed in this section.

  - The priority for multicast is discussed in this section as well, since it originates from the rule discussion of multiplexing/ prioritizing.

Based on the above thinking, FL suggests the following proposal on how to organize the discussion of 2.3/2.4/2.5. Note: This proposal aims to align the understanding on how to proceed the discussion, not intend to have this proposal to be captured in chair notes as agreement.

Any constructive comments/suggestion are welcome to help us move forward!

FL’s Proposal:

#### Proposal 2.2.1(closed)

- Whether network needs to configure UE to generate separate HARQ-ACK codebooks for unicast and multicast, respectively.

  - Discuss it in section 2.4.1.2, two alternatives for down-selection later if controversial for now.

- Whether network needs to configure UE to generate a joint HARQ-ACK codebooks for unicast and multicast.

  - No need.

- Section 2.5 focuses only on the high level rule, i.e., multiplexing for the same priority and prioritizing for different priorities.

  - Sub-slot based PUCCH, two non-overlapping slot based PUCCH, and the PUCCH resources used for actual transmission could be discussed in this section.

  - The priority for multicast is discussed in this section as well, since it originates from the rule discussion of multiplexing/ prioritizing.

- Section 2.4 focuses on the Type-1 and Type-2 codebook construction itself.

  - The details for multiplexing the codebook is to be discussed in Section 2.4, need to look at Type-1 and Type-2 separately.

Collect views:

| Company | Comments |
| --- | --- |
| ZTE | Regarding the first bullet, we would suggest to discuss it directly under Section 2.4.3 and 2.4.4 instead of making a general proposal for both type1 and type2 codebook. Detailed comments please see our response below. |
| Lenovo, Motorola Mobility | Regarding the first bullet and second bullet, we don’t know why UE generates separate HARQ-ACK codebook and joint HARQ-ACK codebook for unicast and multicast is dependent on gNB configuration. Regarding the third bullet, current URLLC is discussing multiplexing low priority HARQ-ACK feedback and high priority HARQ-ACK feedback in same PUCCH. I wonder whether it is considered for MBS HARQ-ACK feedback. |
| CATT | We are not OK with the first bullet. The proposal in section 2.4.2 confuses us, since the prerequisite is not clear. We share the same views with ZTE that the Type-1 and Type-2 codebooks construction can be discussed separately in advance as shown in section 2.4.3 and 2.4.4. After this, we can come back to see whether a general proposal can be achieved for both Type-1 and Type-2 codebooks. |
| Spreadtrum | For the first bullet, we share the similar view as ZTE. |
| MTK | We are generally OK with the FL proposal except for the first bullet. For the first bullet, ZTE’s suggestion is fine for us. |

### Round-2

FL’s Comments

FL suggests focusing on this proposal firstly to get the common understanding of what a specific proposal in the corresponding section is actually talking about, especially for the cases the same term (e.g., separate codebook, multiplexing, etc) is used but may be interpreted differently.

FL’s Proposal:

#### Proposal 2.2.2 (GTW for clarification)

- Whether network needs to configure UE to generate separate HARQ-ACK codebooks for unicast and multicast, respectively.

  - Note: This is similar to URLLC UE is configured with a codebook list corresponding two PUCCH-config for separate HARQ-ACK codebook. In this case, network may guarantee the PUCCH resources for the two codebooks are not overlapping, so that the two codebooks can be transmitted to network.

  - Discuss it in section 2.4.1.2, two alternatives for down-selection later if controversial for now.

- Whether network needs to configure UE to generate a joint HARQ-ACK codebooks for unicast and multicast.

  - No need.

  - Note:

    - For Type-2: No way

      - We have agreed to concatenate the two sub-codebooks since DAI cannot be jointly counted between unicast and multicast.

    - For Type-1: No need

      - We have agreed to construct Type-1 codebook to base on the union TDRA set which solves the cases of TDM-ed unicast and multicast. How Type-1 codebook is constructed for the FDM-ed case is to be solved.

        - This is also essentially how the Type-1 codebook is constructed for multiplexing the codebook in the same PUCCH resource.

- Section 2.5 focuses only on the high level rule, i.e., multiplexing for the same priority and prioritizing for different priorities.

  - Sub-slot based PUCCH, two non-overlapping slot based PUCCH, and the PUCCH resources used for actual transmission could be discussed in this section.

  - The priority for multicast is discussed in this section as well, since it originates from the rule discussion of multiplexing/ prioritizing.

  - The detail how to multiplex the codebook to be transmitted in the same PUCCH resources is essentially how Type-1/-2 codebook is generated, will be discussed in section 2.4.2 and 2.4.3

- Section 2.4 focuses on the Type-1 and Type-2 codebook construction itself.

  - Since no need to have a joint codebook, the details of Type-1/-2 codebook generation in section 2.4.2 and 2.4.3 is essentially how the codebook is constructed for multiplexing all the HARQ-ACK bits to be transmitted in the same PUCCH resources (at least for the same priority case).

Collect views:

| Company | Comments |
| --- | --- |
| vivo | We are not ok with the first bullet and second bullet. We think it has some relation with the discussion on multiplexing/prioritization between unicast and multicast. In UE always multiplexes HARQ-ACK for unicast and multicast in a slot in a PUCCH, UE can jointly construct type 1 codebook and separately construct type 2 codebook. If UE multiplexes HARQ-ACK for unicast and multicast in a slot when their PUCCHs are overlapped (if not overlapped, separately transmits), UE separately constructs type 1 codebook and separately construct type 2 codebook. This is different from URLLC, where URLLC HARQ-ACK and eMBB HARQ-ACK are never multiplexed, but for unicast HARQ-ACK and multicast HARQ-ACK, they can be multiplexed in some cases. |
| Ericsson | We think the questions Proposal 2.2.2 raises are very complex and with companies having different understanding about the terminology and definitions. We therefore think more discussion in the email thread is needed before we can converge enough for real progress – the time in the on-line tomorrow might be too limited and it may be better to use the on-line time to reach agreement on other, more limited, topics, where there is better common understanding. At least, the scope of current Proposal 2.2.2 is too wide for discussion and agreement already tomorrow, except possibly for some limited definition, see below. Regarding terminology and definitions, we think there may be a mismatch regarding the relationship between multiplexing, concatenation, and joint codebook. In our view, multiplexing and concatenation are means to achieve a joint codebook. A joint codebook is simply an organized way of sending multiple HARQ feedback and there are different ways to achieve this. So, although separate codebooks for unicast and multicast can be constructed, when these are combined a joint codebook is created, even if a simple concatenation is used. We think it could help the discussion if companies could have the same understanding on this terminology. Possibly, we could have agreement about the use of the terminology for joint codebooks, e.g. that a joint codebook may e.g. be formed by the concatenation of separately constructed codebooks. This can also be seen as multiplexing. One way is to construct codebooks for unicast and multicast separately and then combine them to form a joint codebook. The way the separate codebooks are combined could be defined by the specification and does not need to be configured. There also needs to be rules for when codebooks for unicast and multicast are separately transmitted and when a joint codebook is to be constructed. One possible implicit way is the following: If a separate PUCCH-ConfigurationList for multicast is configured, then the UE constructs and transmits separate codebooks and otherwise a joint codebook is used, except for one case: if the multicast and unicast PUCCH configuration together with the respective PRI and K1 value in DCI and with multicast and unicast dl-DataToUL-ACK point to the same PUCCH resource. In this case a joint codebook is used, or priorization rules define which codebook to transmit. |

## HARQ-ACK feedback resource

Background

Last meeting has agreed a separate PUCCH-Config for multicast can be optionally configured and otherwise, PUCCH-Config for unicast applies. This meeting, for multicast transmission, the HARQ-ACK feedback resources discussion will focus on how to determine the UE specific PUCCH resource for HARQ-ACK feedback for a given multicast transmission, including resources sets, resources, k1 value, etc.

Submitted Proposals

The following proposals are applied to either HARQ-ACK option per FL’s guess of the intention of the proposal.

(ZTE) Proposal 1:

  - Regarding configuration of PUCCH resource sets for multicast (reuse unicast mechanism):

- A maximum of 4 PUCCH resource sets are configured for multicast for the UE.

- NACK-only and ACK/NACK feedback share the same PUCCH resource set.

- UCI size limit for each PUCCH resource sets is configured, except for the PUCCH resource set 0, which only supports no more than 2 UCI bits.

(CATT) Proposal 17:

  - For HARQ-ACK feedback mechanism in MBS, the following methods can be considered:

- Group-common PDCCH to indicate PUCCH resource for common PDSCH.

- Group-common PDCCH to indicate UE-specific periodic PUCCH resources.

(NTT DOCOMO) Proposal 5:

  - A list of k1 values for DCI format 1_0 for multicast is configurable if DCI format 1_0 is used for scheduling group-common PDSCH of PTM scheme 1.

(Ericsson) Proposal 1:

  - An RRC-configured additional time offset can be individually configured to each UE receiving multicast traffic. The HARQ ACK feedback delay is then the addition of the PDSCH-to-HARQ_feedback timing indicator in DCI plus this extra time offset.

### For ACK/NACK based feedback

Submitted Proposals

(Huawei) Proposal 5:

  - PUCCH-Config for multicast can be configured with one or two sub-configurations.

    - When two sub-configurations are configured, they are associated with PUCCH of priority index 0 and 1, respectively.

(OPPO) Proposal 5:

  - Up to 2 separate PUCCH-config can be configured for MBS to accommodate 2 HARQ-ACK priorities for MBS.

(OPPO) Proposal 12:

  - For ACK/NACK based HARQ-ACK feedback for multicast, both shared and separate PUCCH resources among UEs within the group are supported.

(OPPO) Proposal 13:

  - For ACK/NACK based feedback and NACK only based feedback for multicast, RSRP based PUCCH resource configuration is supported.

(OPPO) Proposal 14:

  - Whether shared or separate PUCCH resources are used can be up to gNB configuration or scheduling.

(CATT) Proposal 3:

  - For ACK/NACK based HARQ-ACK feedback for multicast, PUCCH resources are configured separate among UEs within the same group

(Nokia) Proposal 2:

  - In case UE-specific PUCCH resources are to be used for PTM ACK / NACK feedback, the PUCCH resource scheduling is based on a group-common PDCCH containing a single PRI in case of PTM scheme 1, which based on UE-specific configurations of PUCCH resource sets indicates UE-specific PUCCH resources.

(Nokia) Proposal 3:

  - For ACK / NACK based feedback for RRC_CONNECTED UEs receiving multicast, the UEs can be optionally configured with up to two new PUCCH-configs based on the MBS services received, i.e., one for low priority eMBB and one for high priority URLLC services.

(CMCC) Proposal 3:

  - For separate PUCCH configuration case, two PUCCH-Config can be configured for multicast HARQ-ACK, one for priority index 0, one for priority index 1.

(LGE) Proposal 6:

  - UE specific PUCCH resources are configured on UE’s active BWP.

(LGE) Proposal 8:

  - For PTM scheme 1, group common DCI indicates a single PUCCH resource indicator and a single PDSCH-to-HARQ_feedback timing indicator at least for ACK/NACK based HARQ-ACK.

(LGE) Proposal 9:

  - For UE specific PUCCH resource allocation for ACK/NACK based HARQ-ACK to group common DCI, different UEs in the group can be configured with different values of at least PUCCH-Resource and dl-DataToUL-ACK in UE dedicated PUCCH-config for multicast or for unicast (unless PUCCH-config for multicast is configured). So, different UEs can be allocated with different PUCCH resources by the same PUCCH resource indicator and the same PDSCH-to-HARQ_feedback timing indicator of the group common DCI.

(LGE) Proposal 10:

  - For PTP retransmission, the PUCCH resource indicator and the PDSCH-to-HARQ_feedback timing indicator in UE specific DCI are interpreted based on PUCCH-config for unicast, regardless of whether PUCCH-config for multicast is configured or not.

(Lenovo) Proposal 4:

  - For ACK/NACK based HARQ-ACK feedback, it is up to network to configure a PUCCH resource shared by two or more UEs or a specific PUCCH resource for each UE in the group of UEs receiving multicast.

(Lenovo) Proposal 5:

  - For ACK/NACK based HARQ-ACK feedback, each UE in the group of UEs receiving multicast is configured with a specific PUCCH resource for support of PTP-based retransmission.

(Lenovo) Proposal 8:

  - For PTM transmission scheme 1, from per UE perspective, PUCCH resource configuration for ACK/NACK based feedback can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast.

#### Round-1

FL’s Comments

Last meeting agreed a separate PUCCH-Config for multicast can be optionally configured. For ACK/NACK based HARQ-ACK feedback, it is up to network to configure orthogonal PUCCH resources among UEs within the same group. Based on these agreements, the issues we can proceed to discuss include details for the separate PUCCH-Config for multicast and how to determine the UE specific PUCCH resources based on the group-common DCI for PTM scheme 1.

FL’s Proposal:

#### Proposal 2.3.1.1-1 (closed)

When a separate PUCCH-Config for multicast is optionally configured,

- The separate PUCCH-Config can be a list which includes up to PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are not sure about “up to PUCCH-Config configurations”. Would it be better to elaborate it a bit more? |
| ZTE | We are ok with the proposal with the understanding that there can be up to 4 PUCCH-Config configurations where up to 2 are configurations for unicast and up to 2 are for multicast. BTW, it seems to be missing one number “2” in the 1st sub-bullet. |
| CMCC | We have the same understanding with ZTE, which there can be up to total 4 PUCCH-config. |
| CATT | OK with the proposal. |
| Nokia, NSB | Support the intention of the proposal, but like Lenovo, we support more elaboration regarding the number PUCCH-config configurations (see our response for 2.2.2.1, where we see 5 potential configs in total). |
| OPPO | Support the proposal assuming “up to 2 PUCCH-Config configurations”. |
| Samsung | Simpler to agree on additional PUCCH-Config for multicast only – unicast is also affected by other WIs (and there is PUCCHConfig-Common). |
| Apple | The number of PUCCH-Config configurations need to be clarified in the proposal, it includes the configurations for multicast or configurations for both multicast and unicast. |
| Qualcomm | It would be better to say: “up to 2 PUCCH-Config configurations” |
| Convida | Support the intention of the proposal. Agree with other companies that some rewording may be needed. |
| MTK | Similar view with OPPO/QC. |
| Spreadtrum | We share the similar view with OPPO/QC. |
| vivo | If it tends “up to 2 PUCCH-Config configurations”, fine in principle. |
| NTT DOCOMO | It seems to be a typo as other companies have said. “up to two PUCCH-Config configurations” |

#### Round-2

FL’s Comments

This proposal intends for the PUCCH-Config for multicast at least for ACK/NACK based feedback, the configuration can be a list containing up to two configurations as for the configuration for unicast corresponding URLLC and eMBB.

FL’s Proposal:

#### Proposal 2.3.1.2-1 (closed)

For a separate PUCCH-Config for multicast that is optionally configured,

- The separate PUCCH-Config for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We are OK with this proposal. |
| Samsung | OK with the proposal. |
| Ericsson | The proposal is a bit confusing. It looks like the first “PUCCH-config” in the sentence refers to the pucch configuration list. We propose the following changes: For a separate PUCCH-ConfigurationList for multicast that is optionally configured, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively. - FFS other configurations We also think that the proposal should only be applicable to ACK/NACK and with NACK-only FFS. |
| Convida | Fine with the proposal. One suggestion is to highlight the changes between the updated proposal and the original proposal which can help us to track the changes. |
| Qualcomm | Ok |
| Spreadtrum | We are fine with the proposal. |
| CMCC | OK |
| OPPO | support |
| CATT | OK with this proposal. |
| ZTE | OK with the proposal. |
| NTT DOCOMO | We are fine with the proposal. |
| Nokia, NSB | Similar view to Ericsson – the proposal should only be applicable to ACK/NACK and with NACK-only FFS |
| Apple | OK with updates from Ericsson |
| vivo | Ok with the proposal. |

FL’s Proposal:

#### Proposal 2.3.1.2-2 (closed)

For ACK/NACK based HARQ-ACK feedback for multicast,

- PRI indicated in the DCI for PTM scheme 1 is mapped onto one UE specific PRI value.

- k1 indicated in the DCI for PTM scheme 1 is mapped onto one UE specific k1 value.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Support FL proposal. |
| ZTE | We are supportive of this proposal. In order to avoid the PUCCH congestion for MBS, it is beneficial to map PRI/K1 indicated in the group-common DCI to UE specific PRI/K1 value. We can add an FFS to further study how to achieve this mapping. FFS: How to achieve the above mapping |
| CMCC | Support |
| LG | We are fine with the proposal |
| CATT | Agree with FL proposal. |
| Nokia, NSB | Support the proposal with FFS FFS: If Group-common PDCCH PRI can also be configured to indicate UE-specific periodic PUCCH resources. |
| OPPO | Support the proposal |
| Samsung | No need for the proposal. We understand the proposal’s intention to be that PUCCH resources and slot timing values are configured by UE-specific RRC instead of UE-common RRC (SIB). That is part of PUCCH-Config and does not need a separate agreement. |
| Apple | We are fine with this proposal. |
| Qualcomm | Agree |
| MTK | Support the proposal |
| vivo | Can’t fully understand the proposal, can it be explained more how to map one PRI value in DCI for PTM scheme 1 onto one UE-specific PRI value? Reuse the existing mapping rule or something new. |
| NTT DOCOMO | We are fine with FL’s proposal. We prefer to add an FFS. “FFS: PRI and/or k1 for enabling/disabling HARQ feedback by DCI if supported” |
| Ericsson | We agree |
| Apple | OK with the proposal. |

#### Round-3

FL’s Proposal:

#### Proposal 2.3.1.3-1 (email 1st CP)

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Collect views:

| Company | Comments |
| --- | --- |
| Ericsson | We are ok with the main sentence of the proposal. regarding the first sub-bullet, our comments from the previous round remains. The proposal is a bit confusing. It looks like the first “PUCCH-config” in the sentence refers to the pucch configuration list. We propose the following changes: For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively. - FFS other configurations |
| Samsung | Support the proposal and the modification by Ericsson. |

#### Proposal 2.3.1.3-2 (email 1st CP)

For ACK/NACK based HARQ-ACK feedback for multicast,

- PRI indicated in the DCI for PTM scheme 1 is mapped onto one UE specific PRI value.

- k1 indicated in the DCI for PTM scheme 1 is mapped onto one UE specific k1 value.

- FFS the mapping details.

Collect views:

| Company | Comments |
| --- | --- |
| Ericsson | OK with the proposal |
| Samsung | Support the proposal |

### For NACK-only based feedback

Submitted Proposals

(OPPO) Proposal 8:

  - Both PUCCH format 0 and PUCCH format 1 should be supported for NACK-only based HARQ feedback mechanism.

(OPPO) Proposal 9:

  - In NACK-only based HARQ feedback scheme, only 1 HARQ-ACK bit is transmitted in one PUCCH.

(Spreadtrum) Proposal 8:

  - At least PUCCH format 0 is supported for NACK-only based HARQ-ACK feedback.

(ZTE) Proposal 5:

  - If NACK-only feedback is supported for MBS

- PUCCH resource configuration per MBS service is supported.

- PUCCH format 0 is supported for NACK-only feedback.

- PUCCH format 0 supports PUCCH repetition.

- (CATT) Proposal 7:

  - PUCCH format 0/1 can be configured by gNB to be used for NACK-only feedback for MBS.

- (CATT) Proposal 8:

  - NACK-only based feedback framework is designed based on Rel-15 NR ACK/NACK-based feedback mechanism by considering PRI, DCI CCE index and k1 to indicate PUCCH resources.

- (CATT) Proposal 9:

  - When configuring PUCCH resource set for NACK-only feedback in MBS, 8-32 PUCCH resources can be supported based on configuration.

- (CATT) Proposal 10:

  - The PUCCH resource set can be used by all the MBS services using NACK-only based feedback mechanism.

- (Nokia) Proposal 5:

  - Both PUCCH format 0 and 1 are supported for NACK-only based HARQ-ACK feedback.

- (Nokia) Proposal 6:

  - For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH-config for HARQ-ACK feedback from per UE perspective is separate from PUCCH-config for HARQ-ACK feedback for unicast and from optional PUCCH-config for ACK / NACK feedback for MBS.

- (Nokia) Proposal 7:

  - Proposal 7: For a proper operation of NACK-only HARQ-ACK feedback for MBS, a UE can be optionally configured to support more than one PUCCH carrying HARQ-ACK feedback per slot with a method that is to be down-selected from the list below:

    - Option 1:  Allowing multiple HARQ-ACK feedback carrying PUCCHs per slot.

    - Option 2:  Based on Rel-16 sub-slot PUCCH mechanism.

    - Option 3:  DL-heavy slot configuration.

- (MediaTek) Proposal 2:

  - From UEs within the group perspective, the PUCCH resource configuration is shared for NACK-only based HARQ-ACK feedback.

- (Intel) Proposal 6:

  - For NACK-only feedback, PUCCH formats 0 and 1 are supported.

- (LGE) Proposal 2:

  - NACK only based HARQ-ACK is transmitted on group common PUCCH resource.

- (LGE) Proposal 3:

  - Different group common PUCCH resources can be related to different RS e.g. in terms of PRB and/or sequence for PUCCH.

- (LGE) Proposal 4:

  - Support PUCCH format 0 and 1 for NACK based HARQ feedback.

- (LGE) Proposal 7:

  - Group common PUCCH resources are configured on UL CFR configured within UE’s active UL BWP.

- (Lenovo) Proposal 2:

  - Both PUCCH format 0 and PUCCH format 1 can be configured for UEs receiving multicast to transmit NACK-only based HARQ-ACK feedback.

- (Lenovo) Proposal 3:

  - For NACK-only based HARQ-ACK feedback, a common PUCCH resource for transmitting the NACK is configured to the group of UEs receiving multicast.

- (NTT DOCOMO) Proposal 2:

  - Support PUCCH format 0 and 1 for NACK-only based HARQ-ACK feedback for multicast.

- (Ericsson) Proposal 5:

  - PUCCH formats 0 and 1 can be used for semistatic codebook.as a basis for NACK-only signaling

- (Ericsson) Proposal 6:

  - Denoting the number of bits in the NACK-only codebook by N, downselect from the  following variants to create up to N NACK-only signals in the same uplink slot:

    - a) Use multiple PUCCH resources in the same slot, M=2^N-1 PUCCH resources for N is the total number of PDSCHs for which the UE needs to provide feedback, each UE transmits on one of the resources according to the subset of PDSCHs for which the UE needs to signal NACK.

    - b) Use multiple PUCCH resources in the same slot, where each PUCCH resource represents one bit in the NACK-only codebook and the UE needs to transmit multiple NACK signals, one on each PUCCH resource corresponding to a bit in the codebook for which the UE has to signal a NACK.

      - FFS: Use the PUCCH format 0 or format 1 phase rotations and for format 1 the OCCs as dimension in addition to OFDM-symbol and PRB, i.e associate each rotation with a HARQ process.

      - FFS: Associate each NACK signal with a subset of bits in the NACK-only codebook, where multiple UEs use the same PUCCH resource for the NACK-only signal relating to the same subset and the subset size may reduce to 1. A UE transmits the NACK signal if at least one bit of the associated subset of bits in the NACK-only codebook is cleared, i.e. indicates a PDSCH decoding failure. The gNB accordingly retransmits the transport blocks of all HARQ processes of the subset.

#### Round-1

FL’s Comments

This discussion can be pending up to the decision of support of NACK-only based HARQ-ACK feedback.

FL’s Proposal:

#### Proposal 2.3.2.1 (TBD)

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | Although other issues of NACK-only can be pending, we would like FL to clarify a relevant previous agreement to the RAN group. We believe that the following agreement from RAN103-e can be understood differently by different companies (one of PUCCH-config / PUCCH resource set / PUCCH resource): Agreement: For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast. - FFS PUCCH format Although we raised our concern during e-mail discussions of 103e meeting and the FL had clarified that the agreement reflects a new PUCCH-config for NACK-only, we believe that a new clarifying agreement, such as the one below, is needed for a better understanding of the RAN1 group: For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, the following PUCCH-configs are separated: (a) PUCCH-config for multicast HARQ-ACK feedback based on NACK-only (b) PUCCH-config for multicast HARQ-ACK feedback based on ACK / NACK for eMMB (c) PUCCH-config for multicast HARQ-ACK feedback based on ACK / NACK for URLLC (d) PUCCH-config for unicast HARQ-ACK feedback for eMBB (e) PUCCH-config for unicast HARQ-ACK feedback for URLLC. |

## HARQ-ACK codebook

Submitted Proposals

The proposals maybe applicable to both Type-1 and Type-2 HARQ-ACK codebook are listed here:

(Huawei) Proposal 4:

  - A separate HARQ-ACK codebook may be generated depending on whether PUCCH-Config for multicast is configured:

    - When it is configured, UE generates a separate HARQ-ACK codebook using PUCCH-Config for multicast.

    - When it is NOT configured, multiplexing/prioritizing the HARQ-ACK feedback between unicast and multicast will be executed to determine the PUCCH resources.

(OPPO) Proposal 6:

  - If separate PUCCH-config(s) is configured for MBS, HARQ-ACK codebook for MBS and unicast are constructed separately and transmitted in PUCCH resource dedicated for MBS or unicast.

(vivo) Proposal 4:

  - The HARQ-ACK codebook type for multicast and unicast should be separately configured.

(LGE) Proposal 14:

  - UE may not need to send HARQ-ACK feedback to all multicast transmissions within a CFR because UE is typically interested in only a few services. Further discuss whether/how to consider UE’s interest in multicast services for construction of HARQ-ACK codebook.

### Separate HARQ-ACK codebook

#### Round-1

FL’s Comments

- Last meeting has agreed UE can be optionally configured a separate PUCCH-Config for multicast and otherwise PUCCH-Config for unicast applies. The proposal for this section discusses the issues for separate cases, i.e., separate PUCCH-Config for multicast or not.

FL’s Proposal:

#### Proposal 2.4.1.1 (closed)

When PUCCH-Config for multicast and PUCCH-Config for unicast are separately configured,

- UE is expected to construct separate codebooks for multicast and for unicast corresponding to PUCCH-Config for multicast and PUCCH-Config for unicast, respectively.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Same priority or different priority? When the HARQ-ACK feedback for multicast and HARQ-ACK feedback for unicast is to be transmitted in same slot, will the UE transmit two PUCCHs carrying separate codebooks in the slot? |
| ZTE | More clarification for this proposal is needed. First of all, it seems that separate PUCCH-Config for multicast and unicast doesn’t necessary mean separate codebooks for multicast and unicast. For type1 HARQ-ACK codebook, separate PUCCH-Config for multicast and unicast can also lead to one united codebook. On the other side, the same PUCCH-Config for multicast and unicast doesn’t mean the one united codebook, separate codebook for multicast and unicast can also be generated in this case. Thus, it seems the main bullet is not necessary. Regarding the sub-bullet, it is fine for type2 codebook. However, it seems to conflict with the agreements achieved in last meeting (copied below). In last meeting, we have already agreed that the construction of Type-1 HARQ-ACK codebook is based on the union of the PDSCH TDRA sets of the unicast service and the multicast service, which means we will take all the potential SLIVs from multicast and unicast together to construct codebook, instead of constructing codebook for multicast and unicast separately. Agreement: For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast. - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported. - FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size. |
| CMCC | Similar question as Lenovo, there are serval cases as the following, 1) different priorities for multicast and unicast UE should separately construct codebooks for multicast and unicast respectively, indeed. 2) same priority for multicast and unicast 2-1)The PUCCHs not overlaps in time UE can separately construct codebooks for multicast and unicast respectively 2-2)The PUCCHs overlaps in time UE should construct a single codebook using the method in Proposal 2.3.2.1-1 Therefore, we think this proposal is only supported in case 1) and case 2-1). |
| Nokia, NSB | We do not support the proposal. We see that our proposals regarding both Type-1 and Type-2 HARQ-ACK codebook are not listed herein, but rather in multiplexing/prioritization part and are not treated. Pleas could the FL please add the following proposals to this part? Proposal 18: The UE constructs separate HARQ-ACK sub-codebooks using Rel-15 / 16 mechanisms for each MBS service and one sub-codebook for unicast services. Proposal 19: The UE concatenates the constructed sub-codebooks and sends them in the same PUCCH resource in case their HARQ-ACK feedback is scheduled for the same time instance (slot or sub-slot). Proposal 21: The PHY identification of PDSCH HARQ-ACK to MBS sub-codebook mapping is the group-common RNTI value. Proposal 22: The UE maps the PDSCH HARQ-ACK of unicast services scrambled with a UE-specific RNTI to the unicast sub-codebook. Proposal 23: The order of concatenation of the sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing or decreasing order of the RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook. Regardless of the PUCCH-config configured, we believe that the UE needs to construct separate HARQ-ACK sub-codebooks for each MBS service and one sub-codebook for unicast services. We have listed the reasons why this is needed in our contribution. In case scheduled for the same time instance, the sub-codebooks are to be concatenated. Therefore, the proposal can be changed as: - UE is expected to construct separate codebooks for each multicast service and one codebook for unicast services, respectively. - FFS details. |
| OPPO | We support the proposal. This proposal is only related to HARQ-ACK codebook construction, how to transmit the constructed codebooks(multiplexing or drooping one of them) based on priority or PUCCH resource overlapping is a next step issue. |
| Samsung | Do not support the proposal. Whether the codebooks are joint or separate does not relate to having joint or separate PUCCH-Configs – unicast and multicast can have separate PUCCH-Configs due to different payload requirements (e.g. CA for unicast, single cell for multicast) but a joint codebook can be supported. As for M-TRP, a separate RRC parameter can indicate joint or separate coding. |
| Apple | If separated codebooks are simultaneously constructed, what’s the combination of unicast/multicast and slot-base/sub-slot based codebooks |
| Qualcomm | Not sure we need to this proposal to be agreed for now. Our understanding is that except for the cases that UE can multiplex the ACK/NACK-based feedback for multicast and unicast, the UE should construct separate codebooks. |
| Convida | We are generally fine with the principle proposed by the moderator if the assumption is that how the UE will transmit the constructed codebooks, e.g., concatenate and transmit as one codebook, or drop one of them, etc., will be further discussed. |
| MTK | It needs to consider the priority as Lenovo commented. |
| vivo | Share the view with ZTE that separate PUCCH-Config for multicast and unicast doesn’t necessary mean separate codebooks for multicast and unicast and the same PUCCH-Config for multicast and unicast doesn’t mean the one united codebook. In the last meeting, we have agreed that for the same priority, if both HARQ-ACK for unicast and multicast are available in a slot, for the PUCCH overlapping case, UE will multiplex HARQ-ACK for unicast and multicast into one PUCCH. If we agree that even for the non-overlapping case, or UE can support only one PUCCH with HARQ-ACK in a slot, UE will also multiplex the HARQ-ACK together, when there is no need to separately construct HARQ-ACK codebook for unicast and multicast. UE can directly construct one HARQ-ACK for unicast and multicast. |
| NTT DOCOMO | We are generally fine with FL’s proposal. Our understanding is same as OPPO, so one FFS should be added as FFS: whether/how to multiplex/drop |

#### Round-2

FL’s Comments

The intention for this proposal is about whether UE is indicated to generate separate HARQ-ACK codebook somehow. The precedent to be referred is the feature introduced for URLLC, i.e., UE is configured with separate PUCCH-config for constructing two separate codebooks. Alt 1 is general proposal which includes the cases of with same or different priorities. Whether UE is to transmit two codebooks with the same priority in the same slot will be discussed in section 2.5.2.

Regarding the proposal from Nokia, per my understanding of the proposals, they are more like the guidance for multiplexing the sub-codebook to be transmitted in the same PUCCH resources, and that is why the proposals are summarized in section 2.5.2. When we talk about concatenating sub-codebook, we need to differentiate what we are talking about is Type-1 or Type-2. For Type-2, concatenating has been agreed. For Type-1, based on union of TDRA sets has been agreed and FFS the FDM-ed case.

FL’s Proposal:

#### Proposal 2.4.1.2 (closed)

When PUCCH-Config for multicast and PUCCH-Config for unicast are separately configured, down-select between the following alternatives:

- Alt 1: UE is expected to construct separate codebooks for multicast and for unicast corresponding to PUCCH-Config for multicast and PUCCH-Config for unicast, respectively.

- Alt 2: UE constructs the HARQ-ACK codebook following the rule of multiplexing/prioritizing.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We don’t support this proposal as the two alternatives are not quite clear to us. For Alt 1: Whether the HARQ-ACK feedback for multicast and HARQ-ACK feedback for unicast is to be transmitted in same slot or not is not clear. Will the UE transmit two PUCCHs carrying separate codebooks in the slot? OR, Alt 1 implies separate PUCCH transmissions for the multicast and unicast? For Alt 2: How UE constructs HARQ-ACK codebook via multiplexing or prioritizing is not clear. |
| Samsung | Neither Alt. 1 nor Alt. 2. The UE can be provided a separate indication whether or not to multiplex – that is a separate issue and does not relate to whether one or two PUCCH-Config are configured. |
| Ericsson | We think the proposal needs some further clarification. It is not clear what it means “to construct separate codebooks”. Will the two codebooks be sent via one PUCCH resource or not? Even if multicast is configured with separate PUCCH-config, it does not mean two separate codebooks. It also depends on whether Type1 or Type2 codebook are configured or not. For Type 1, when priority between MBS and unicast is the same, then joint codebook. For Type 2, when priority, first construct codebooks separately, but then concatenate them together. If different priority, then of course, only high priority codebook is sent. |
| Convida | Don’t support the proposal. The meanings of the two alternatives are not clear to us. We also have the similar questions for Alt1 and Alt2 as Lenovo asked. |
| Qualcomm | We think Alt. 1 and Alt. 2 are both possible, dependent on the gNB configuration or defined rules. #### Proposal 2.4.1.2 When PUCCH-Config for multicast and PUCCH-Config for unicast are separately configured, further discussion how/when to use the following alternatives: - Alt 1: UE can construct separate codebooks for multicast and for unicast corresponding to PUCCH-Config for multicast and PUCCH-Config for unicast, respectively. - Alt 2: UE can construct the HARQ-ACK codebook following the rule of multiplexing/prioritizing. |
| Spreadtrum | Don’t support the proposal. Similar questions as Lenovo for both Alt1 and Alt2. |
| CMCC | Need more detailed clarification. If there is prioritizing, UE also needs to construct separate codebooks. |
| OPPO | Our suggestion is to keep Alt 1 only, to address the issue raised by Lenovo, maybe we can rephrase Alt 1 as below: - Alt 1: For a given UL slot or sub-slot (if supported) for multicast and unicast HARQ-ACK feedback, UE is expected to construct separate codebooks for multicast and for unicast corresponding to PUCCH-Config for multicast and PUCCH-Config for unicast, respectively. When PUCCH-Config for multicast and PUCCH-Config for unicast are separately configured, there are 3 possibilities: - HARQ-ACK bits for multicast and unicast are multiplexed; - HARQ-ACK bitts for only one of multicast or unicast are reported due to prioritization; - HARQ-ACK bits for multicast and unicast are transmitted on separate PUCCH; In cast of multiplexing, either joint codebook or concatenated separate codebooks are workable, however, concatenated separate codebooks seems much simpler, especially for type-2 codebook. For the other 2 cases, only separate codebooks are possible. In general, seems Alt 1 is beneficial for a unified and simple HARQ-ACK feedback mechanism. |
| CATT | We share the same questions that Lenovo asked, where Alt1 and Alt2 are not quite clear to us |
| ZTE | We still think the current proposal is not clear. For type2 codebook, it has been agreed that separate codebook will be used. Then what’s the motivation to have this agreements again? For type1 codebook, it has been agreed that at least for TDM-ed MBS and unicast, the codebook will be constructed based on the union of TDRAs for both MBS and unicast. To us, this is to say a joint codebook will be used. Then, the current proposal is against our previous agreements for type1 codebook. Thus, we would propose to discuss the detailed proposal for type1 and type2 codebook separately instead of making a generic proposal here . |
| NTT DOCOMO | Alt 2 is not clear to us. Does Alt2 mean to construct a single codebook instead of two separate codebooks(Alt1)? |
| Nokia, NSB | Do not support the proposal. Regardless of the PUCCH-config configured, we believe that the UE needs to construct separate HARQ-ACK sub-codebooks for each MBS service and one sub-codebook for unicast services, due to at least following reasons: - Assume that the UE receives an MBS service and a unicast service both of priority 0 (or both 1). If we assume that both use Type-2 HARQ-ACK codebook, the DAI fields should be different. Therefore, the UE should construct two separate sub-codebooks in this situation. This does not depend on the configured PUCCH-configs. - Assume that the UE receives an MBS service and a unicast service both of priority 0 (or both 1). If we assume that both uses Type-1 HARQ-ACK codebook, and unicast and multicast PDSCHs are FDM-ed, then the UE cannot use the same HARQ-ACK codebook for both services. In that case, the UE needs to construct 2 different sub-codebooks. - The cases above also apply if multiple MBS services are received, instead of an MBS service and a unicast service. Therefore, the proposal can be changed as: - UE is expected to construct separate codebooks for each multicast service and one codebook for unicast services, respectively. - FFS details. |
| vivo | Don’t support the proposal. Agree with Samsung’s view. |
| MTK | The proposal is not clear for us. Does the wording “construct separate codebooks” in Alt 1 mean that the HARQ codebook is dedicated for Type 2 codebook? If the unicast and multicast with same priority generate the HARQ codebook within same slot, does it need to generate the separate codebook if sub-slot PUCCH is not supported? |

#### Round-3

FL’s Proposal:

#### Proposal 2.4.1.3 (GTW first)

When PUCCH-Config for multicast and PUCCH-Config for unicast are separately configured, consider the following alternatives:

- Alt 1: UE is expected to construct separate codebooks for multicast and for unicast corresponding to PUCCH-Config for multicast and PUCCH-Config for unicast, respectively.

  - Note: Alt 1 means network configures/indicates the PUCCH resources for multicast and for unicast will not overlap.

- Alt 2: UE constructs the HARQ-ACK codebook following the rule of multiplexing/prioritizing.

  - Note: Alt 2 means UE will generate a single codebook regardless separate PUCCH-Config for multicast or shared PUCCH-Config with unicast.

- Alt 3: Alt 1 with another configuration configures UE to generate two separate codebooks.

  - Note: Alt 3 is similar to URLLC with configuration of pdsch-HARQ-ACK-CodebookList together with two separate PUCCH-Config for generating two separate HARQ-ACK codebooks.

- FFS whether/how down select.

Collect views:

| Company | Comments |
| --- | --- |
| Ericsson | Support the proposal for future downselection. For clarification, our understanding of the difference between alt1 and alt3 is that alt3 allows dynamic switching between pucch configs for a given PDSCH (via the priority bit in DCI) while in alt1 the configuration is always the same. |
| Samsung | OK in principle with the proposal but down-selection is not necessary – instead, it can be something like “consider further”. For example, Alt 1 can apply as for M-TRP (or as for eMBB+URLLC) when the codebooks are separate, Alt 2 can apply as for M-TRP when the codebooks are joined (or even as for eMBB+URLLC in Rel-17), Alt.3 can apply as for URLLC in Rel-16. Our understanding of Alt. 3 is as described in the note (but with both codebooks being slot-based for multicast as there is no agreement for sub-slot). |

### Type-1 HARQ codebook

Submitted Proposals

(Huawei) Proposal 6:

  - When RRC configures that PDSCHs of unicast and multicast can be scheduled as FDMed, Type-1 HARQ-ACK codebook is constructed by concatenating the codebook for unicast followed by the codebook for multicast.

(OPPO) Proposal 7:

  - Type-1 HARQ-ACK codebook for MBS and unicast are constructed separately;

  - No optimization on Type-1 HARQ-ACK codebook is needed for payload size reduction;

  - Same HARQ-ACK codebook type is used for MBS and unicast.

(Spreadtrum) Proposal 5:

  - For type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, support that HARQ-ACK bits for multicast are appended after HARQ-ACK bits for unicast.

(ZTE) Proposal 2:

  - Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast or FDM-ed unicast and multicast, for a PDSCH SLIV group, the number of HARQ-ACK bits generated by the UE is min (N, M), where N is the number of FDM-ed PDSCHs in the PDSCH SLIV group, and M is the UE receiving capability for FDM PDSCHs.

(vivo) Proposal 5:

  - For ACK/NACK based feedback for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the K1 sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

    - Candidate of PDSCH reception occasions are determined based on the union of PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured) only in the slots corresponding to the intersection of K1 set of unicast service and multicast service.

(vivo) Proposal 6:

  - For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast, and Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

    - Construction of Type-1 HARQ-ACK codebook based on the concatenation of the HARQ-ACK codebooks of the FDMed unicast and multicast, or FDMed multicast and multicast, is supported.

(CATT) Proposal 11:

  - To reduce the HARQ-ACK feedback payload size, the mechanism for optimizing the Type-1 codebook construction should be studied and can be based on the UE’s capability.

(Nokia) Proposal 24:

  - In a resource limited system, construction of semi-static HARQ-ACK sub-codebooks per PTM service can be avoided. Instead, for the FDM-ed PDSCH occasions, one unified bit can be included in the HARQ-ACK codebook that is to be constructed using the Rel-15 / 16 methods.

(Nokia) Proposal 25:

  - Enabling / disabling of this unification mechanism at the UE can be done via RRC signaling or DCI.

(CMCC) Proposal 9:

  - When constructing Type-1 HARQ-ACK codebook for TDM-ed unicast and multicast PDSCH, the union of the PDSCH TDRA sets is used to determine PDSCH candidate occasions only when a specific slot timing value is in the intersection of different K1 lists.

(CMCC) Proposal 10:

  - When constructing Type-1 HARQ-ACK codebook for FDM-ed unicast and multicast PDSCH, each slot timing value in the set K1 corresponds to two HARQ-ACK bits. The order of HARQ bits can be unicast first, then multicast.Determine which service HARQ corresponds to

(CMCC) Proposal 11:

  - When constructing Type-1 HARQ-ACK codebook for FDM-ed multicast and multicast PDSCH, the number of HARQ bits corresponding to each slot timing value should be the same as the number of multicast services that the UE is receiving.

(CMCC) Proposal 15:

  - For NACK-only based HARQ-ACK feedback, Type-1 HARQ-ACK codebook can be constructed by defining the association between HARQ-ACK codebook value and combinations of cyclic shifts/PUCCH resources index/OCC.

(Intel) Proposal 8:

  - If MBS and unicast PDSCH are FDM, the Type 1 HARQ codebook can be generated by concatenating two sub-codebooks, each generated by considering separately the PDSCH TDRA tables of unicast and multicast.

(Apple) Proposal 2:

  - FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot can be supported according to UE capability.

(Apple) Proposal 3:

  - For Type-1 HARQ-ACK feedback, to support FDM reception of unicast and multicast PDSCH, HARQ-ACK codebook are determined separately for unicast PDSCH and group common PDSCH.

(LGE) Proposal 13:

  - For FDMed multicast and unicast, separate HARQ-ACK sub-codebooks for multicast and unicast are individually constructed and concatenated for Type-1 HARQ-ACK codebook.

(Ericsson) Proposal 10:

  - When MBS traffic is configured with its own dl-DataToUl-Ack in PUCCH configuration, the number of bits in joint HARQ codebook is determined by the union of elements in the sets of K1 of both multicast and unicast where K1 of multicast is provided by dl-DataToUl-Ack in multicast PUCCH configuration and K1 of unicast is provided by dl-DataToUl-Ack in unicast PUCCH configuration or is predefined as {1,2,3,4,5,6,7,8}. The union of TDRA sets is considered at the DL slots given by the intersection of both K1 sets in multicast and unicast.

(Ericsson) Proposal 11:

  - When multicast and unicast or multicast and multicast traffic can be FDMed in a slot, multicast traffic scheduled by one G-RNTI is treated as coming from a virtual carrier, and the HARQ codebook construction rule before R-17 can be reused for this joint type-1 HARQ-ACK codebook.

(Ericsson) Proposal 12:

  - The index of virtual carrier associated with multicast traffic can be either explicitly configured via RRC signaling or implicitly determined by predefined rules. The predefined rule to determine virtual carrier index can be FFS.

(Ericsson) Proposal 4:

  - For NACK-only transmission of HARQ feedback for group scheduling, a semi-static codebook is supported and dynamic codebook is not supported

#### Round-1

FL’s Comments

Last meeting has agreed to construct the Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service for the same priority. There are proposals to further clarify that the union of the PDSCH TDRA sets only applies to the slots corresponding to the intersection of K1 set for payload optimization.

Further, for the FDM-ed unicast and multicast, the proposal of Type-1 codebook construction details is appending the Type-1 codebook for multicast at the end of that for unicast, which also reflects the multiplexing details of the HARQ-ACK codebook with the same priority for Type-1 HARQ-ACK codebook.

FL’s Proposal:

#### Proposal 2.4.2.1-1 (closed)

For Type-1 HARQ-ACK codebook construction, determining PDSCH reception candidate occasions based on the union of PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured) is applied to the slots corresponding to the intersection of K1 set of unicast service and multicast service.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Support. |
| ZTE | More clarification is needed for the above proposal. First of all, it seems this proposal is conflicting with Proposal 2.3.1. Proposal 2.3.1 proposes to construct codebooks for multicast and unicast separately, while this proposal seems to propose to determine the codebook for multicast and unicast together. Even if separate codebooks are multicast and unicast are generated, it is not clear how to generate separate codebook. There are at least the following two potential interpretations. Alt.1 {separate HARQ-ACK bits for all K1 values for unicast} + {separate HARQ-ACK bits for all K1 values for multicast}; Alt.2 { separate HARQ-ACK bits for slot n for unicast + separate HARQ-ACK bits for slot n for multicast } + { separate HARQ-ACK bits for slot n+1 for unicast + separate HARQ-ACK bits for slot n+1 for multicast } + …. |
| CMCC | Echo ZTE’s comment, we think this proposal is used in the case that HARQ-ACK for multicast and unicast are multiplexed in a single PUCCH. For example, the priority of HARQ-ACK for multicast and unicast is the same in shared PUCCH configuration case, or the PUCCHs overlaps in time in separate PUCCH configuration case. |
| CATT | OK with this proposal. |
| Nokia, NSB | Support the proposal. |
| OPPO | Support the proposal |
| Samsung | Do not support the proposal - the intersection of K1 does not work as multicast is UE-group common (even if K1 is UE-specific). It should be the union of K1. |
| Apple | We are fine with this proposal. |
| Qualcomm | Ok in principle, but need to add the condition is that “ACK/NACK-based feedback for multicast and unicast are multiplexed in a single PUCCH”. |
| MTK | Support the proposal |
| Spreadtrum | We share the similar view with CMCC. |
| vivo | Agree. It seems more accurate to say “is only applied to the slots corresponding to the intersection of K1 set of unicast service and multicast service.” Because we want to say that in the other slots, i.e., the slots not corresponding to the intersection of K1 set of unicast service and multicast service, determining PDSCH reception candidate occasions is only based on PDSCH TDRA sets of the unicast service or the multicast service, but the current formulation does not say anything about other slots. |
| NTT DOCOMO | We share the comments with ZTE, more clarifications are needed. |

#### Proposal 2.4.2.1-2 (closed)

For Type-1 HARQ-ACK codebook construction,

- for FDM-ed unicast and multicast, the HARQ-ACK bits for unicast precedes the HARQ-ACK bits for multicast.

- FFS for FDM-ed multicast and multicast.

Collect views:

| Company | Comments |
| --- | --- |
| ZTE | Similar comments as above. More clarification is needed. It is not clear how to generate separate codebook separately. There are at least the following two potential interpretations. Alt.1 {separate HARQ-ACK bits for all K1 values for unicast} + {separate HARQ-ACK bits for all K1 values for multicast}; Alt.2 { separate HARQ-ACK bits for slot n for unicast + separate HARQ-ACK bits for slot n for multicast } + { separate HARQ-ACK bits for slot n+1 for unicast + separate HARQ-ACK bits for slot n+1 for multicast } + …. |
| CMCC | Support |
| LG | WE are fine with the proposal. |
| CATT | We agree with the FL proposal. We support that the HARQ-ACK bits for unicast are followed by the HARQ-ACK bits for multicast when constructing Type-1 HARQ-ACK codebook. We also support concatenating multiple multicast HARQ-ACK bits into a Type-1 HARQ-ACK codebook based on the increasing order of G-RNTI. |
| Nokia, NSB | We support the proposal. We believe that the concatenation order of the HARQ-ACK bits for multicast and multicast should follow increasing or decreasing order of the RNTI values that are used to map PDSCH HARQ-ACK to sub-codebook. This would easily also allow to concatenate more than one sub-codebooks created for MBS, when needed. We propose the following addition to the proposal: For Type-1 HARQ-ACK codebook construction, - for FDM-ed unicast and multicast, the HARQ-ACK bits for unicast precedes the HARQ-ACK bits for multicast. - for FDM-ed multicast and multicast, the order of HARQ-ACK bits follow increasing / decreasing order of the RNTI values used to map PDSCH HARQ-ACK to corresponding sub-codebook. |
| OPPO | Support the proposal |
| Samsung | Fine with the proposal |
| Apple | Share the comments with ZTE, the proposal needs to be clarified. If we are following the CA codebook construction principle, i.e., MBS could be considered as another CC, Alt.1 is the way to go. |
| Qualcomm | We think the first subbellet is not limited to FDM-ed unicast and multicast, but also applicable to other TDMed cases. The order of slot index, serving cell index, TRP index (if configured) and C-RNTI/G-RNTI can be further discussed. |
| MTK | Support the proposal |
| Spreadtrum | Support. |
| vivo | Agree. We support to separately construct HARQ-ACK codebooks for unicast and multicast if they are FDMed. |
| NTT DOCOMO | We are fine with FL’s proposal. |

#### Round-2

FL’s Comments

As commented by other companies, the intention was to discuss the issue how to construct Type-1 codebook to be multiplexed in the same PUCCH resource, for which last meeting has agreed to base on the union of TDRA sets and FFS the case of FDM-ed unicast/multicast and multicast. I’d like to keep FFS on the case of FDM-ed multicast and multicast which has been in discussion in AI 8.12.1.

FL’s Proposal:

#### Proposal 2.4.2.2-1 (closed)

For Type-1 HARQ-ACK codebook construction to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions based on the union of PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured) is applied to the slots corresponding to the intersection of K1 set of unicast service and multicast service.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| Samsung | Do not support the proposal. What happens with slots that are not in the intersection of the K1 sets – no HARQ-ACK reporting for them? Why did the gNB even configure that K1 set for unicast (when it is possible to configure)? Please check the Rel-15/Rel-16 specifications for Type-1 HARQ-ACK codebook construction when the UE monitors multiple DCI formats. It is the union of K1 sets between DCI 1_0 and DCI 1_1 in Rel-15, it is again the union of K1 sets when DCI 1_2 was introduced in Rel-16, it does not make sense and it does not work to change the specifications for multicast. |
| Ericsson | We agree, but suggest a clarification: “For Type-1 HARQ-ACK codebook construction for unicast and multicast to be multiplexed…” |
| Qualcomm | Here assumption is to find the intersection of K1 for unicast service and multicast service with same priority? What about the intersection of K1 set of different multicast services or unicast service+different multicast services? |
| Spreadtrum | OK |
| CMCC | Support. Echo Samsung’s comment, there are two cases: 1) slots that are not in the intersection of the K1 sets: The PDSCH reception candidate occasions are determined based on the PDSCH TDRA set(s) of the unicast service or the multicast service. Which TDRA table set(s) to be used is according to whether the K1 set(s) is associated with multicast or unicast 2) slots that corresponding to the intersection of K1 sets: The PDSCH reception candidate occasions are determined based on the union of PDSCH TDRA sets of the unicast service and the multicast service. |
| OPPO | Share the view as Qualcomm, some refinement on the proposal is needed considering that there may be multiple priorities for unicast and multicast, and there may be multiple multicast services. |
| CATT | OK with this proposal. |
| ZTE | What’s the underlying assumption for this proposal, separate codebook for MBS and unicast or joint codebook for MBS and unicast? |
| Nokia, NSB | OK |
| Apple | OK with this proposal. |
| vivo | Support in principle. Same comment as CMCC. |
| MTK | We are generally fine with the proposal |

#### Proposal 2.4.2.2-2 (closed)

For Type-1 HARQ-ACK codebook construction to be multiplexed in the same PUCCH resource,

- for FDM-ed unicast and multicast, the HARQ-ACK bits for unicast precedes the HARQ-ACK bits for multicast.

- FFS for FDM-ed multicast and multicast.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| Samsung | OK |
| Ericsson | We agree, for the case of one unicast and one multicast PDSCH. |
| Qualcomm | ‘precedes…’ here means - in the order of {unicast, multicast} of the same slot index in the same serving cell and then slot index, serving cell index? - Or {unicast per slot index per serving cell and then slot index, serving cell index}, appended by {multicast per slot index per serving cell and then slot index, serving cell index}? |
| Spreadtrum | OK |
| CMCC | OK As Qualcomm’s question, we prefer in the order of {unicast, multicast} of the same slot index in the same serving cell and then slot index, serving cell index. We can further clarify it. |
| OPPO | Support |
| CATT | Agree. |
| ZTE | Similar question as Qualcomm. Besides, in last meeting, we have agreed that joint codebook will be used for at least the TDMed unicast and multicast case per the following agreements. Does it mean now we need to support separate codebook for the FDMed unicast and multicast case? If that’s the case, we have concerns on supporting two different mechanisms for type1 codebook. We would propose to support joint codebook for type1 codebook for both TDMed and FDMed case. Agreement: For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast. - FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported. - FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size. |
| NTT DOCOMO | We are fine with the proposal. |
| Nokia, NSB | Support |
| Apple | Same comments as last round. |
| vivo | ok |
| MTK | We are generally fine with the proposal |

#### Round-3

FL’s Proposal:

#### Proposal 2.4.2.3-1

For Type-1 HARQ-ACK codebook construction for unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions based on the union of PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured) is based on a set of slot timing values K1, where the set is

- Alt 1: the intersection of K1 set for unicast and K1 set for multicast if separately configured.

- Alt 2: the union of K1 set for unicast and K1 set for multicast if separately configured.

- Down-select to one alternative.

Collect views:

| Company | Comments |
| --- | --- |
| Ericsson | Support. |
| Samsung | Do not support. Rel-16 operation is the union of K1 sets (Alt. 2). Unless a reason/problem is identified for doing something different, so that there can be a corresponding agreement, Rel-16 remains default. We don’t understand how the intersection of K1 sets can work (as it would result to dropped HARQ-ACKs) or how it won’t restrict the network deployment. |

#### Proposal 2.4.2.3-2

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast scheduled in the same slot to be multiplexed in the same PUCCH resource,

- the HARQ-ACK bits for unicast precedes the HARQ-ACK bits for multicast for the same scheduled slot.

- FFS for FDM-ed multicast and multicast.

Collect views:

| Company | Comments |
| --- | --- |
| Ericsson | Support. |
| Samsung | Support |

### Type-2 HARQ codebook

Submitted Proposals

(Huawei) Proposal 7:

  - For UE multiplexing the HARQ-ACK feedback for unicast and for multicast, Type-2 HARQ-ACK codebook is constructed by concatenating the codebook for unicast followed by the codebook for multicast.

(OPPO) Proposal 4:

  - The concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is achieved by appending Type 2 MBS HARQ-ACK codebook to the end of Type 2 unicast HARQ-ACK codebook.

  - At most one Type 2 MBS HARQ-ACK codebook is concatenated with type 2 unicast HARQ-ACK codebook.

(Spreadtrum) Proposal 6:

  - For type-2 HARQ-ACK codebook construction, support that HARQ-ACK bits for multicast are appended after HARQ-ACK bits for unicast.

(ZTE) Proposal 3:

  - Type-2 HARQ-ACK codebook construction for MBS:

    - The DAI count is performed per MBS service.

    - If the UE receives multiple MBS services, multiple sub-codebooks is generated separately for each MBMS service, then UE cconcatenates these sub-codebooks based on the G-RNTI of each MBS service in ascending order to form a type-2 codebook for the multiple MBS services.

    - If the UE receives unicast and multicast, the sub-codebooks for unicast and multicast are generated separately, and then the multicast sub-codebook is concatenated after the unicast sub-codebook.

- (vivo) Proposal 7:

  - For ACK/NACK based HARQ-ACK feedback, for type 2 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

    - If UE is configured with multiple g-RNTIs

      - If PTM transmission scheme 1 is used for group-common PDSCH, separate DAI counting for different g-RNTIs is used.

        - Concatenating more than one Type-2 HARQ-ACK codebook for multicast is supported.

    - If PTM transmission scheme 2 is used for group-common PDSCH, the existing mechanism can be reused to construct a HARQ-ACK codebook for different PDSCHs.

- (CATT) Proposal 12:

  - The concatenation of TB-based and CBG-based HARQ-ACK mechanism can be reused to concatenate the MBS-based and unicast-based HARQ-ACK codebooks.

- (CATT) Proposal 13:

  - Support concatenating more than one Type-2 HARQ-ACK codebooks for multicast scheduling.

- (CATT) Proposal 14:

  - For concatenating more than one Type-2 HARQ-ACK codebooks for multicast, the order between sub-codebooks can be based on the increasing order of G-RNTI.

- (CMCC) Proposal 12:

  - DCI format of multicast does not include total DAI.

- (CMCC) Proposal 13:

  - The concatenation order of Type-2 HARQ-ACK codebook for unicast and multicast can be  unicast first, and then in ascending order of G-RNTI values for different multicast services.

- (Apple) Proposal 1:

  - Whether to support more than one Type-2 codebook for MBS  is waiting for the outcome of the CFR design.

- (Lenovo) Proposal 11:

  - For multicast PDSCHs, don’t support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

- (Ericsson) Proposal 9:

  - For type-2 HARQ codebook, UE first constructs codebook for unicast and multicast traffic separately and then concatenate them together. Within this joint codebook, unicast HARQ bits precede, followed by multicast HARQ bits. If there are more than one type-2 codebook for multicast, the codebook associated with a smaller G-RNTI precedes the one associated with a larger G-RNTI

#### Round-1

FL’s Comments

Last meeting has agreed to concatenate Type-2 HARQ-ACK codebook for unicast and multicast, this meeting focuses the details on how to concatenate them. The proposal itself reflects how to multiplex the Type-2 HARQ-ACK codebook for unicast and for multicast with the same priority.

FL’s Proposal:

#### Proposal 2.4.3.1 (closed)

For Type-2 HARQ-ACK codebook concatenation,

- the first Type-2 HARQ-ACK codebook for unicast precedes the second Type-2 HARQ-ACK codebook for multicast.

- FFS the number of Type-2 HARQ-ACK codebooks for multicast.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Generally OK. Regarding the first bullet, the two Type-2 HARQ-ACK codebook may be confusing with the Type-2 HARQ-ACK codebook in the main bullet. It is better to replace “sub-codebook” with “codebook” in the two bullets. |
| ZTE | We are fine with the above proposal. |
| CMCC | Fine We don’t know the meaning of number of Type-2 HARQ-ACK codebooks for multicast, does it means multiple type-2 HARQ-ACK codebooks corresponds to multiple multicast services? |
| LG | We are fine with the proposal. |
| CATT | Agree with the FL proposal. |
| Nokia, NSB | Support |
| OPPO | Support the proposal |
| Samsung | Fine with the proposal |
| Apple | We are fine with this proposal. |
| Qualcomm | Better to change to sub-codebook in the first subbullet to avoid confusion. |
| Convida | We are fine with the proposal. |
| MTK | Support the proposal |
| Spreadtrum | Support the proposal. |
| vivo | If it is for DG PDSCH case, agree in principle. One question, does it also include the case that unicast HARQ-ACK including HARQ-ACK for DG PDSCH and SPS, and multicast HARQ-ACK including HARQ-ACK for DG PDSCH and SPS. In that case, how to construct codebook for SPS has not been discussed. Different options are observed for SPS HARQ-ACK. E.g., - Option 1: UE constructs HARQ-ACK codebook for all SPS including multicast SPS and unicast SPS. - Then SPS HARQ-ACK codebook is appended to HARQ-ACK for multicast DG PDSCH - Option 2: UE constructs HARQ-ACK codebook for multicast SPS and unicast SPS separately, Then, there will be HARQ-ACK codebook for unicast DG PDSCH, HARQ-ACK codebook for multicast DG PDSCH, HARQ-ACK codebook for unicast SPS and HARQ-ACK codebook for multicast SPS. The order to concatenate these four HARQ-ACK codebooks will be more than one. |

#### Round-2

FL’s Comments

To address the comment of “sub-codebook” in sub-bullets.

FL’s Proposal:

#### Proposal 2.4.3.2 (closed)

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- the first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS the number of Type-2 HARQ-ACK sub-codebooks for multicast.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| Samsung | OK |
| Ericsson | We agree |
| Convida | Fine with the proposal. |
| Qualcomm | Ok |
| Spreadtrum | OK |
| CMCC | OK |
| OPPO | Support |
| CATT | Agree |
| ZTE | OK |
| NTT DOCOMO | We are fine with the proposal. |
| Nokia, NSB | OK |
| Apple | We are fine with this proposal. |
| vivo | We still think the applicable case needs to be clarified. it is for DG PDSCH only case or DG PDSCH and/or SPS PDSCH case? For DG PDSCH only case, we agree, for DG PDSCH and/or SPS PDSCH case, we think it need further consideration. |
| MTK | We are OK with the proposal. |

#### Round-3

FL’s Proposal:

#### Proposal 2.4.3.3 (email 1st CP)

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- the first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS the number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Collect views:

| Company | Comments |
| --- | --- |

### Enh Type-2 / Type 3 HARQ codebook

Submitted Proposals

(vivo) Proposal 8:

  - For ACK/NACK based HARQ-ACK feedback, support enhanced type 2 and type 3 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

(Nokia) Proposal 11:

  - Enhanced Type-2 and Type-3 HARQ-ACK codebooks that target LBT failure problems for Rel-16 Unlicensed Band are not supported for PTM.

(Qualcomm) Proposal 3:

  - For RRC_CONNECTED UEs, also support Type 3 HARQ-ACK codebook for multicast ACK/NACK based HARQ-ACK feedback.

(Ericsson) Proposal 13:

  - Enhanced Type 2 or Type 3 HARQ-ACK codebooks are not supported for PTM traffic feedback

#### Round-1

FL’s Comments

Only four proposals submitted for this discussion. The survey is as follows:

Enh Type-2:

- Support: vivo

- Not support: Nokia, Ericsson

Type-3:

- Support: Qualcomm

- Not support: Ericsson

Given limited proposals and diverse views, FL would not suggest proposals at this stage.

FL’s Proposal:

#### Proposal 2.4.4.1 (TBD)

Collect views:

| Company | Comments |
| --- | --- |
| Nokia, NSB | We support the Ericsson proposal. Note, you can add us to the list of companies who do not support Type-3 codebooks. Enhanced Type 2 or Type 3 HARQ-ACK codebooks are not supported for PTM traffic feedback |
| Samsung | No need for additional codebook types. Neither enhanced Type-2 or Type-3 can possibly outperform Type-2 on non-shared spectrum. Even Type-1 is unnecessary for single-cell operation. |
| NTT DOCOMO | We also prefer to preclude enhanced type-2 and type 3. They are required for unlicensed band, which is not included in this WI scope. |

## UCI multiplexing/prioritizating

### Priority for MBS and unicast

Submitted Proposals

(Huawei) Proposal 1:

  - Low and high priority with index 0 and index 1 respectively for multicast is introduced.

(Huawei) Proposal 2:

  - At least for DCI format 1_1/1_2, priority index is included in the DCI scheduling multicast.

(Huawei) Proposal 3:

  - The priority across unicast and multicast is up to network configuration.

    - The comparison of the priority index of unicast and multicast is configured per UE.

(OPPO) Proposal 1:

  - HARQ-ACK for MBS can be configured to either priority 0 or priority 1 via DCI, the priority of MBS HARQ-ACK feedback and unicast HARQ-ACK feedback are the same if they are configured to the same priority level.

(OPPO) Proposal 2:

  - Same priority rule as that in unicast is used for the priority between HARQ-ACK for unicast and other unicast UCI.

(Spreadtrum) Proposal 2:

  - Two types of priority are introduced for multicast. The low priority multicast is equal to eMBB unicast priority, and high priority multicast is equal to URLLC unicast priority.

(ZTE) Proposal 10:

  - Proposal 10: Regarding priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast:

    - The unicast priority indication field in the DCI is reused for the multicast PDSCH;

    - The high/low priority of multicast is equal to high/low priority of unicast, respectively.

(vivo) Proposal 10:

  - Support 2-level PHY priority for HARQ-ACK feedback for multicast.

    - HARQ-ACK with priority index 0 for multicast has the same priority with HARQ-ACK with priority index 0 for unicast

    - HARQ-ACK with priority index 1 for multicast has the same priority with HARQ-ACK with priority index 1for unicast

- (CATT) Proposal 4:

  - Two priorities across multicast and unicast are supported.

- (CATT) Proposal 5:

  - To reflect the priority for the HARQ-ACK feedback for multicast, the RRC configuration is supported.

- (Nokia) Proposal 12:

  - As in Rel-16 unicast framework, there are two priorities for HARQ-ACK defined also for multicast, i.e., low-priority for eMBB and high-priority for URLLC transmissions, that can be indicated by the DCI, and those priorities are equal to their unicast counterparts, e.g., low-priority unicast eMBB transmission has the same priority with low-priority multicast eMBB transmission.

- (Nokia) Proposal 13:

  - Priority between HARQ-ACK feedback for multicast and other UCI (SR, CSI) / PUSCH for unicast follow the same rules between HARQ-ACK feedback for unicast and those channels.

- (CMCC) Proposal 4:

  - The ‘priority indicator’ field in DCI format can be reused for priority indication of HARQ-ACK for multicast.

- (CMCC) Proposal 7:

  - The priority of multicast HARQ-ACK can be lower, higher than or equal to the priority of other UCIs for unicast.

    - Multicast HARQ-ACK and other UCIs with the same priority index have the same priority.

- (Intel) Proposal 5:

  - When a PUCCH resource carrying HARQ ACK for MBS overlaps in time domain with a PUCCH resource carrying other UCI types, a default priority order for PUCCH dropping can be defined in specification where HARQ-ACK feedback of unicast transmission > HARQ-ACK feedback of MBS transmission > SR > CSI report

- (LGE) Proposal 11:

  - The high or low L1 priority of HARQ-ACK feedback is indicated by group common DCI for PTM scheme 1 or UE specific DCI for PTP retransmission.

- (LGE) Proposal 12:

  - The following rules of prioritization between MBS HARQ-ACK and unicast HARQ-ACK are applied:

    - The high L1 priority of MBS HARQ-ACK feedback has a lower priority than the high L1 priority of unicast HARQ-ACK feedback.

    - Any high L1 priority of HARQ-ACK feedback has a higher priority of any low L1 priority of HARQ-ACK feedback, regardless of multicast and unicast.

    - The low L1 priority of MBS HARQ-ACK feedback and the low L1 priority of unicast HARQ-ACK feedback have an equal priority.

- (NTT DOCOMO) Proposal 3:

  - Reuse existing mechanisms to configure/indicate the priority index of HARQ-ACK feedback.

    - When priority indexes for unicast and multicast are same, the priority of HARQ-ACK feedback for multicast is equal to the priority of HARQ-ACK feedback for unicast.

    - When the priority index for multicast is larger than the priority index for unicast, the priority of HARQ-ACK feedback for multicast is higher than the priority of HARQ-ACK feedback for unicast.

    - When the priority index for multicast is smaller than the priority index for unicast, the priority of HARQ-ACK feedback for multicast is lower than the priority of HARQ-ACK feedback for unicast.

- (NTT DOCOMO) Proposal 4:

  - Proposal 4: For relations between the priority of HARQ-ACK feedback for multicast and other UCI for unicast or PUSCH, reuse existing mechanisms.

- (Ericsson) Proposal 14:

  - There are still two priorities at PHY layer, Multicast traffic can be configured to have either the high priority, or low priority. The indication of priority reuse the rules from R-16.

#### Round-1

FL’s Comments

There is clear support of two priorities for multicast. Regarding how to indicate the priority for multicast, there are proposals that the priorities are configured per RRC. Given the priority index for unicast is indicated in DCI since the scheduling is dynamic for UE supporting both eMBB and URLLC, it is nature to indicate the priority in DCI for multicast as well considering UE supports the case of low/high priority multicast and the case of multicast and unicast with different priorities. For comparing the priority for unicast and multicast, there are also different proposals but it could be FFS.

FL’s Proposal:

#### Proposal 2.5.1.1 (email 1st CP)

Two priority indexes are introduced for multicast, with

- index 0 meaning low priority and index 1 meaning high priority.

- priority index included in the DCI scheduling the group-common PDSCH.

- FFS the priority comparison between multicast and unicast with the same priority index.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| ZTE | We are supportive of the above proposal. |
| CMCC | OK |
| LG | We are fine with the proposal. |
| CATT | We agree with this proposal. |
| Nokia, NSB | Support the proposal. We believe also that two priorities for multicast are equal to their unicast counterparts, e.g., low-priority unicast has the same priority with low-priority multicast eMBB transmission. |
| OPPO | Support the proposal |
| Samsung | Fine with the proposal |
| Apple | We are fine with this proposal. |
| Qualcomm | We prefer the proposal with the following modifications: Two priority indexes are introduced for multicast ACK/NACK-based feedback, with - index 0 meaning low priority and index 1 meaning high priority. - priority index included in the DCI scheduling the group-common PDSCH. - the priority between multicast and unicast with the same priority index is same - FFS the priority between multicast and unicast with the same priority index can be different. FL’s responses: the newly added 3rd sub-bullet, to my understanding, is saying the same thing as the FFS sub-bullet with different views. |
| Convida | We are fine with the proposal |
| MTK | Support the proposal |
| Spreadtrum | Support. |
| vivo | Ok. We prefer that low priority unicast transmission has the same priority with low-priority multicast transmission and high priority unicast transmission has the same priority with high multicast transmission. |
| NTT DOCOMO | We are fine with FL’s proposal. To simplify HARQ-ACK feedback mechanism, multicast and unicast with the same priority index should have the same priority. |
| Ericsson | We agree |

### Multiplexing/prioritizing

Submitted Proposals

(Huawei) Proposal 9:

  - For UE supporting multicast, subject to UE capability, UE supports:

    - Transmitting up to two PUCCHs with HARQ-ACK information in different symbols within a slot.

    - Sub-slot based PUCCH for HARQ-ACK feedback.

(Huawei) Proposal 10:

  - For the cases of HARQ-ACK feedback is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

    - Support multiplexing for the same priority and prioritizing for different priorities when the corresponding PUCCH resources overlap in time in a sub-slot.

(OPPO) Proposal 3:

  - If PUCCH-config for unicast is used for MBS, HARQ-ACK bits of unicast and MBS with same priority are multiplexed and transmitted in the PUCCH of the same priority level.

(Spreadtrum) Proposal 3:

  - Not support multiple non-overlapping PUCCH resources for HARQ-ACK in the same slot.

(Spreadtrum) Proposal 4:

  - Sub-slot based PUCCH transmission for HARQ-ACK is supported.

(ZTE) Proposal 4:

  - Regarding the case of non-overlapping PUCCHs resources for HARQ-ACK in the same UL slot.

- HARQ-ACK PUCCH overriding mechanism should be reused for multicast with the same priority.

- if HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast are determined in the same UL slot, then they should be multiplexed for the same priority (regardless of whether they overlap in the time domain).

  - The multiplexed PUCCH resource is determined based on the PRI in the last DCI corresponding to the unicast HARQ-ACK PUCCH.

- sub-slot based PUCCH transmission for HARQ-ACK is supported.

(ZTE) Proposal 6:

  - If the NACK-only feedback mode is supported, RAN1 further discusses the multiplexing method for scenarios where NACK-only PUCCHs overlap.

(ZTE) Proposal 7:

  - If the NACK-only feedback mode is supported, RAN1 further discusses the multiplexing method for scenarios where NACK-only PUCCH and other PUCCH/PUSCH overlap.

(vivo) Proposal 9:

  - For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast,

- For the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot, separate transmission of HARQ-ACK PUCCH for unicast and HARQ-ACK PUCCH for multicast is supported.

(CATT) Proposal 15:

  - The multiplexing / prioritizing mechanisms shall subject to UE capability. If UE can’t support multiplexing / prioritizing capability, gNB shall void scheduling PUCCHs overlapping in time domain.

(CATT) Proposal 16:

  - When the priority indicator is introduced for multicast HARQ-ACK feedback, the rules in R16 for unicast can be used on prioritizing/multiplexing of the UCI.

(Nokia) Proposal 14:

  - Multiplexing of HARQ-ACK feedback of same priority and prioritizing of HARQ-ACK feedback of different priorities are supported in case PUCCH transmissions are in the same (sub-)slot, not only when the corresponding PUCCH resources physically overlap.

(Nokia) Proposal 15:

  - UE capabilities on multiplexing / prioritization are discussed at the end of the WI by RAN1 group, given that there may be multiple dependencies with as yet undefined features.

(Nokia) Proposal 16:

  - Rel-15 / 16 handling rules are followed for multiplexing / prioritization of HARQ-ACK with other UL transmissions.

(Nokia) Proposal 17:

  - If for a UE a scheduled group-common PUCCH resource for PTM NACK-only feedback overlaps in time with a UE-specific PUCCH resource for other UCIs or a PUSCH transmission with the same priority, this UE should multiplex the PTM HARQ-ACK feedback with the other UCIs on the UE-specific PUCCH resource or with the PUSCH transmission, by treating NACK-only feedback as a 1-bit ACK / NACK feedback.

(Nokia) Proposal 18:

  - The UE constructs separate HARQ-ACK sub-codebooks using Rel-15 / 16 mechanisms for each MBS service and one sub-codebook for unicast services.

(Nokia) Proposal 19:

  - The UE concatenates the constructed sub-codebooks and sends them in the same PUCCH resource in case their HARQ-ACK feedback is scheduled for the same time instance (slot or sub-slot).

(Nokia) Proposal 20:

  - When group-common NACK-only HARQ-ACK feedback is used as the HARQ-ACK scheme, in case the UE has UE-specific HARQ-ACK resource for unicast services along with group-common NACK-only resource for PTM, the UE utilizes the UE-specific PUCCH resource by constructing separate HARQ-ACK sub-codebooks, as if ACK / NACK based approach is being used for PTM.

(Nokia) Proposal 21:

  - The PHY identification of PDSCH HARQ-ACK to MBS sub-codebook mapping is the group-common RNTI value.

(Nokia) Proposal 22:

  - The UE maps the PDSCH HARQ-ACK of unicast services scrambled with a UE-specific RNTI to the unicast sub-codebook.

(Nokia) Proposal 23:

  - The order of concatenation of the sub-codebooks to construct a HARQ-ACK codebook, when the HARQ-ACK feedback of different services are scheduled for the same time instance, follows the increasing or decreasing order of the RNTI values that are used to map PDSCH HARQ-ACK to MBS sub-codebook.

(CMCC) Proposal 1:

  - Support sub-slot based PUCCH transmission for HARQ-ACK for multicast.

(CMCC) Proposal 2:

  - Not support transmitting two slot based PUCCHs for HARQ-ACK for multicast in a slot.

(CMCC) Proposal 5:

  - If separate PUCCH(s) is configured for HARQ-ACK for multicast, the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK with the same priority index should be the same. The PUCCH structure for PUCCH-Config for multicast and unicast HARQ-ACK with different priority index can be different.

(CMCC) Proposal 6:

  - When the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK is the same, the PUCCH resource for HARQ-ACK multiplexing is determined based on the last unicast DCI.

(CMCC) Proposal 8:

  - The multiplexing/prioritization rule between the HARQ-ACK for multicast and other UCIs for unicast can reuse Rel-16 multiplexing/prioritization rule between the HARQ-ACK for unicast and other UCIs for unicast.

(CMCC) Proposal 16:

  - If there is a collision between NACK-only feedback PUCCH for multicast and other UCIs, NACK-only feedback PUCCH can be dropped to solve the multiplexing issue.

(Qualcomm) Proposal 2:

  - For multiplexing of ACK/NACK feedback for unicast + multicast, or multicast + multicast

    - PUCCH resource is determined by the PRI in the latest DCI of unicast or multicast

    - For Type-1 codebook, HARQ-ACK information bits are concatenated by appending multicast after unicast based on X-RNTI (C-RNTI for unicast, G-RNTI1, G-RNTI2… for multicast).

    - For Type-2 codebook, the number of HARQ-ACK bit(s) and the bit order for multicast are according to the DAI of DL DCI per multicast G-RNTI.

(Samsung) Proposal 1:

  - When multiplexing unicast HARQ-ACK and MBS HARQ-ACK is not supported/configured, a UE drops MBS HARQ-ACK.

(Samsung) Proposal 2:

  - When a UE does not support transmission of PUCCH with unicast HARQ-ACK and of PUCCH with MBS HARQ-ACK in a slot, the UE drops the PUCCH with MBS HARQ-ACK.

(Samsung) Proposal 3:

  - Multiplexing MBS HARQ-ACK and CSI report in a PUCCH resource is enabled by RRC configuration; if not provided, the UE drops the CSI report.

(Samsung) Proposal 4:

  - Multiplexing MBS HARQ-ACK and SR in a PUCCH resource is default (Rel-15 UE behavior).

(Lenovo) Proposal 9:

  - HARQ-ACK feedback for PDSCHs scheduled by group-common DCI is multiplexed in one sub-codebook and HARQ-ACK feedback for PDSCHs scheduled by UE-specific DCI is multiplexed in another sub-codebook.

(Lenovo) Proposal 10:

  - In the HARQ-ACK codebook, HARQ-ACK sub-codebook for PDSCHs scheduled by the group-common DCI is placed at the beginning and followed by HARQ-ACK sub-codebook for PDSCHs scheduled by UE-specific DCI.

(NTT DOCOMO) Proposal 7:

  - When HARQ-ACK feedback for unicast and multicast is indicated as transmitted at the same slot/sub-slot, the UE multiplexes HARQ-ACK with same priority regardless whether PUCCH resources are overlapped or not.

(Ericsson) Proposal 15:

  - When multicast and unicast traffic has same priority and their PUCCH resource for HARQ feedback is overlap in time, the HARQ feedback from multicast and unicast traffic can be multiplexed. The PUCCH resource set is determined according to the total number of HARQ feedback bits of both multicast and unicast traffic, and the specific PUCCH resource is determined by the PRI in DCI associated with unicast traffic.

(Ericsson) Proposal 16:

  - When PUCCH resource for multicast and unicast is not overlapped in time, if UE UE has capability to transmit more than one PUCCH with HARQ per slot, then UE transmit HARQ feedback for both multicast and unicast traffic. If UE can only transmit one PUCCH with HARQ, multiplex multicast and unicast HARQ feedback into one PUCCH resource if they are configured with same priority. Otherwise, low priority PUCCH is dropped.

#### Round-1

FL’s Comments

The issues discussed but not concluded during the last meeting are whether support non-overlapping slot-based PUCCH in the same slot, whether support sub-slot based PUCCH, how to hand the case of sub-slot PUCCH HARQ-ACK feedback for multicast overlapping with sub-slot PUCCH HARQ-ACK feedback for unicast, etc. The proposals in this section will address these discussions.

For the discussion of generating the HARQ-ACK codebook for multiplexing cases, the detailed proposals will be reflected in the HARQ-ACK codebook construction as in section 2.4 for Type-1 and Type-2, respectively.

FL’s Proposal:

#### Proposal 2.5.2.1-1 (closed)

For RRC_CONNECTED UEs receiving multicast,

- support sub-slot based PUCCH for multicast subject to UE capability.

- not support up to two non-overlapping slot-based PUCCHs for multicast in the same slot.

- FFS support of up to two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, in the same slot.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | For the first sub-bullet, it is OK. For the second sub-bullet, “not support up to two non-overlapping PUCCHs…”, should “up to” be deleted? Otherwise, it implies a single PUCCH is not supported. For the third sub-bullet, “up to” is not needed. |
| ZTE | Current, UE is not required to support more than one slot-based PUCCHs for multicast in the same slot for the same priority. Thus, it seems the second bullet needs further clarification. - not support up to two non-overlapping slot-based PUCCHs for multicast in the same slot for the same priority. - FFS support of up to two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, in the same slot for the same or different priority. |
| CMCC | As the comment from Lenovo, “up to” is not needed in the second sub-bullet and the FFS. |
| CATT | We are NOT ok with this proposal. The motivation of the last two sub-bullet is not clear to us and should be clarified. For the case of non-overlapping PUCCH resources for HARQ-ACK in a slot, the multicast can reuse the mechanism in R16, where UE will feedback HARQ-ACK codebook on the PUCCH that the last DCI indicates. |
| Nokia, NSB | Do not support the proposal. Suggest the proposal is split into 2 proposals, differentiated by the type of FB being applied for the MCast service (ACK/NACK or NACK-only). |
| OPPO | Share the view as Lenovo, “up to” in the last 2 bullets should be removed. |
| Samsung | Do not agree with the proposal. Introduction of sub-slot based PUCCH is not required and can significantly complicate specifications. |
| Qualcomm | We feel the last two subbullets are contradictory to each other and should merge them into the following: For RRC_CONNECTED UEs receiving multicast, - support sub-slot based PUCCH for multicast subject to UE capability. - FFS support of up to two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, in the same slot, subject to UE capability. |
| Spreadtrum | Share the same view as Lenovo. |
| vivo | For the first sub-bullet, it is OK For the second and third bullet, agree with ZTE’s clarification and assuming it is for the case that sub-slot is not configured. |
| NTT DOCOMO | We are fine with FL’s proposal. Regarding the third sub-bullet, we don’t support two non-overlapping PUCCHs in a slot/sub-slot. To simplify UE processing multicast HARQ-ACK and unicast HARQ-ACK should be multiplexed in one PUCCH regardless whether PUCCH resources are overlapped or not. |

#### Proposal 2.5.2.1-2 (closed)

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- support multiplexing for the same priority and prioritizing for different priorities when the corresponding PUCCH resources overlap in time in a sub-slot.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We think it could be better to discuss the PUCCH multiplexing/prioritizing together when the corresponding PUCCH resources overlap in time in a slot, a sub-slot subject to UE capability and non-overlap in time in a slot. |
| ZTE | We are supportive of the above proposal. |
| CMCC | More clarification on “in a sub-slot” For example, in separate PUCCH -config case, if the priority is the same between multicast and unicast, but the PUCCH configuration structure is different, e.g., slot-based PUCCH config for multicast, and sub-slot based PUCCH config for unicast, how we define the “overlap in time in a sub-slot”, since there is even no sub-slot for slot based PUCCH config. From our perspective, the PUCCH structure for PUCCH-Config of multicast and unicast HARQ-ACK with the same priority index should be the same in separate case, and the multiplexing for the same priority occurs when the corresponding PUCCH resources overlap in time in the same slot/sub-slot. |
| CATT | Fine with the FL proposal. |
| Nokia, NSB | This proposal depends on the outcome of the previous proposal, which is not clear to us. We believe that multiplexing of HARQ-ACK feedback of same priority and prioritizing of HARQ-ACK feedback of different priorities are supported in case PUCCH transmissions are in the same (sub-)slot, not only when the corresponding PUCCH resources physically overlap |
| OPPO | Support the proposal |
| Samsung | As previously mentioned, we do not agree with introducing sub-slots for multicast. |
| Qualcomm | Agree with several companies that “in a sub-slot” needs to be clarified. |
| Spreadtrum | We share the same view with Lenovo. |
| vivo | Agree in principle. We think we can discuss HARQ-ACK in slot level first and if sub-slot is supported, it can reuse what we have supported for slot level HARQ-ACK by replacing slot with sub-slot. |
| NTT DOCOMO | As commented above, to simplify UE processing, a UE should multiplex HARQ-ACK with same priority regardless whether PUCCH resources are overlapped or not. ‘Overlapping’ condition is not needed. |

#### Proposal 2.5.2.1-3 (closed)

For multiplexing the HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on

- the PRI, K1 indicated in the last DCI.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | Not clear to us. When the unicast or the multicast is SPS, there is no PRI. K1 is used to determine whether to multiplex HARQ-ACK feedback for multicast and unicast, so it could be better to remove the K1 in the subbullet as the main bullet is focused on the PUCCH resource determination. |
| ZTE | We propose to determine the PUCCH resources for transmission based on the PRI indicated in the last unicast DCI. Unicast DCI can naturally indicate UE-specific PUCCH resource for each UE within the same group. On the other hand, it seems all the K1 has to indicate to the same slot, otherwise, there is no need to perform multiplexing. In this sense, it seems not accurate to say that the PUCCH resource is determined by the K1 in the last DCI. |
| CMCC | Similar view is as ZTE, we also think the PUCCH resources for transmission is based on the PRI indicated in the last unicast DCI. One reason about that in separate PUCCH configuration case, gNB can configure smaller PUCCH resources for multicast than unicast. If there is no multiplexing, the HARQ-ACK for multicast is transmitted on the PUCCH resources configured for multicast. If multiplexing occurs, the HARQ-ACK for multicast AND unicast is transmitted on the PUCCH resources configured for unicast. In this way, gNB can only configure small separate PUCCH resources for multicast to reduce system PUCCH overhead. |
| LG | We are fine with change to the last unicast DCI. |
| CATT | We agree the FL proposal. Regarding the last unicast DCI indicating that ZTE/CMCC mentioned, until now we did not see the need that the last DCI has to be the last unicast DCI. We think that both indicting UE-specific PYCCH UE-specific PUCCH resource for each UE within the same group and reducing system PUCCH overhead can be based on gNB’ implement. Please correct me if my understanding is not right. |
| Nokia, NSB | We support the proposal. However, NACK-only feedback needs more discussion, since the group-common resource cannot be used to send a multiplexed feedback. Thus, we propose: For multiplexing the HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on - the PRI, K1 indicated in the last DCI. - FFS NACK-only feedback. |
| OPPO | We support the proposal, more specifically, the last DCI does not need to be unicast DCI, as gNB may not know if there will be MBS transmission(s) later when it is transmitting a unicast, the PRI indicated in unicast DCI is set based on the HARQ-ACK codebook size up to now, it may not be usable if HARQ-ACK bits for more MBS are incorporated later. |
| Samsung | Do not support the proposal - agree with previous comments that it should be the last unicast DCI. |
| Qualcomm | Agree with adding ACK/NACK-based feedback in the main bullet. Regarding ‘last unicast DCI’, there will be some ambiguity for codebook generation if the last DCI is not the unicast DCI and the multicast DCI is missing. |
| Convida | We propose to determine the PUCCH resources for transmitting the multiplexed HARQ feedbacks based on the earliest PUCCH resource that meets the minimum processing timeline from latency consideration. |
| vivo | Share the view with ZTE and CMCC. |
| NTT DOCOMO | We are generally fine with FL’s proposal. For clarification, we prefer to change sub-bullet as below. • the PRI, K1 indicated in the last DCI among DCIs scheduling unicast PDSCH and DCIs scheduling multicast |

#### Round-2

FL’s Comments

The updated proposals clarified the cases of two non-overlapping PUCCH. As what has been supported for URLLC, support of two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, with different priorities, in the same slot seems nature and could FFS on the case of with the same priority.

Regarding the multiplexing rule/prioritizing for sub-slot case, it could depend on whether sub-slot based PUCCH is supported or not for multicast. There was one comment regarding the complex case of support slot-based PUCCH for multicast/unicast and sub-slot based for the other. Perhaps, the solution could be simplified in this release to not expect to be configured with such cases. The proposals are updated accordingly.

FL’s Proposal:

#### Proposal 2.5.2.2-1 (closed)

At least for ACK/NACK based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast,

- FFS: support sub-slot based PUCCH for multicast subject to UE capability.

- not support two non-overlapping slot-based PUCCHs for multicast of the same priority in the same slot.

- support two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, with different priorities, in the same slot.

- FFS support of two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, with the same priority, in the same slot.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | OK. |
| Samsung | Further discussion is needed. For a UE capable of M-TRP operation, why should it be precluded to support two non-overlapping slot-based PUCCHs for multicast of the same priority in the same slot as the TRPs for different services can be different? Why should multicast + multicast be precluded when multicast + unicast is allowed? |
| Ericsson | We agree |
| Qualcomm | Similar question as Samsung, why multicast + multicast is treated differently than multicast + unicast. |
| Spreadtrum | OK |
| CMCC | Support Regarding the M-TRP operation, we are not sure whether all UEs in one MBS group supports M-TRP capability. If only partial UEs support, there are two methods for gNB, which 1) only configure single TRP transmission for all UEs ; 2) partial UEs are configured with single TRP and partial UEs are configured with M-TRP. |
| OPPO | ok |
| CATT | OK |
| ZTE | Ok |
| NTT DOCOMO | We are fine with the proposal. |
| Nokia, NSB | Further discussion needed – same query about multicast+multicast as Samsung/Qualcomm. |
| vivo | ok |

#### Proposal 2.5.2.2-2 (closed)

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- for the case of sub-slot based PUCCH is configured for both unicast and multicast (if supported), support multiplexing for the same priority and prioritizing for different priorities when the corresponding PUCCH resources overlap in time in a sub-slot.

- UE is not expected to be configured with the following cases:

  - slot-based PUCCH for unicast and sub-slot based PUCCH for multicast (if supported)

  - slot-based PUCCH for multicast and sub-slot based PUCCH for unicast

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | For the main bullet, minor modifications on the main bullet as below: For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast and unicast, for determining the PUCCH resource, For the second sub-bullet, we are not pretty sure about why slot-based PUCCH for multicast and sub-slot based PUCCH for unicast can’t be configured simultaneously. At least when URLLC is used for unicast, sub-slot based PUCCH can be configured for unicast for latency reduction purpose. However, for MBS, in case of not latency sensitive service, slot-based PUCCH can be configured for the UE. |
| Ericsson | We agree, but would like to suggest the following reformulation in order to clarify the first bullet: For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource, - for the case of sub-slot based PUCCH is configured for both unicast and multicast (if supported) when the corresponding PUCCH resources overlap in time in a sub-slot. - support multiplexing for the same priority - define dropping rules for different priorities - UE is not expected to be configured with the following cases: - slot-based PUCCH for unicast and sub-slot based PUCCH for multicast (if supported) - slot-based PUCCH for multicast and sub-slot based PUCCH for unicast |
| Qualcomm | Ok with the first subbullet. Need further study on second subbullet. |
| Spreadtrum | We are fine with the first sub-bullet in general. We are confused with the motivation for the second sub-bullet. |
| CMCC | We think this proposal is about the multiplexing/prioritizing rule when separate PUCCH are configured for multicast and unicast, there are two aspects should also be considered, one is whether the PUCCH is slot based or sub slot based, and the second is the sub-slot length. These two aspects will influent the definition of “overlap in time” when multiplexing/prioritizing occurs. For example, regarding the first sub-bullet, if the sub slot lengths are different between unicast and multicast, e.g., one is 2*7 and another is 7*2, how we define “in a sub-slot”? We list all the combination of PUCCH configuration cases as this table: 1) if the priority of multicast and unicast is different, we think all the PUCCH configuration combinations can be supported, because the priority index 0 PUCCH will be dropped if it overlaps with at least a symbol with priority index 1 PUCCH, regardless the sub-slot length is the same or not. 2) if the priority of multicast and unicast is the same, we think the case of both slot based PUCCH configuration or the case of same sub-slot length PUCCH configuration can be supported, because it is simple to define the “PUCCHs overlap in time in slot/sub-slot”. Otherwise, PUCCH for multicast may overlap with more than one PUCCHs for unicast, how to define the ‘overlapping’ and “determination of multiplexed PUCCH” is much complicated. Priority; PUCCH-Config (multicast & unicast); Support; Comment. multicast = unicast; slot based PUCCH & slot based PUCCH; Y; Multiplex in the same slot. slot based PUCCH & sub-slot based PUCCH; N; The spec impact of multiplexing is complicated. E.g., how to define “overlap in same sub-slot”. sub-slot based PUCCH & sub-slot based PUCCH (same sub-slot length); Y; Multiplex in the same slot. sub-slot based PUCCH & sub-slot based PUCCH (different sub-slot length); N; The spec impact of multiplexing is complicated. E.g., how to define “overlap in same sub-slot”. multicast ≠ unicast; slot based PUCCH & slot based PUCCH; Y; Drop priority index 0, reuse Rel-16 rule.. slot based PUCCH & sub-slot based PUCCH. sub-slot based PUCCH & sub-slot based PUCCH (same sub-slot length). sub-slot based PUCCH & sub-slot based PUCCH (different sub-slot length). |
| OPPO | Prefer to further study the proposal. |
| CATT | Since this proposal also is related with the slot-based PUCCH which is still under discussion, we agree with OPPO to postpone it. |
| ZTE | Also prefer some further discussion on the second bullet. |
| NTT DOCOMO | We also prefer to postpone this proposal. The reason is as CATT says. |
| Nokia, NSB | Similar view to Lenovo. Support the main bullet, but disagree with the restrictions imposed by the second bullet, because we expect some concurrent configurations of slot-based and sub-slot based feedback to overcome the limitations of pure sub-slot based configuration, e.g. k1 is also sub-slot. |
| vivo | Same view with Qualcomm |

#### Proposal 2.5.2.2-3 (closed)

For multiplexing the HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on

- the PRI indicated in the last DCI.

  - Details of the last DCI

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | “last DCI” is better to be “last received DCI”. It is better to clarify whether the HARQ-ACK feedback in the main bullet includes HARQ-ACK feedback for SPS. |
| Ericsson | We agree |
| Convida | Fine with the proposal. |
| Qualcomm | Agree |
| Spreadtrum | OK |
| CMCC | We can support to further discuss the details of the last DCI |
| OPPO | Agree. |
| CATT | Agree |
| ZTE | We still think it should be “the last unicast DCI” instead of the “the last DCI”. The case here is “multiplexing the HARQ-ACK feedback for multicast and unicast”, there of course is unicast DCI. It can provide UE specific PUCCH resource for each UE naturally. |
| NTT DOCOMO | We are fine with the proposal. |
| Nokia, NSB | OK |
| vivo | Agree. We also support to use the last unicast DCI. |

#### Round-3

FL’s Proposal:

#### Proposal 2.5.2.3-1

At least for ACK/NACK based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast,

- FFS: support sub-slot based PUCCH for multicast subject to UE capability.

- support two non-overlapping slot-based PUCCHs for multicast of the same priority in the same slot.

- support two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, with different priorities, in the same slot.

- support of two non-overlapping slot-based PUCCHs for multicast and unicast, respectively, with the same priority, in the same slot.

Collect views:

| Company | Comments |
| --- | --- |
| Ericsson | #### We think this proposal could be redundant with Proposal 2.4.1.3. |
| Samsung | Support – it does relate to 2.4.1.3 but there is no conflict. |

#### Proposal 2.5.2.3-2

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE, for determining the PUCCH resource,

- for the case of sub-slot based PUCCH is configured for both unicast and multicast (if supported) when the corresponding PUCCH resources overlap in time in a sub-slot,

  - support multiplexing for the same priority

  - define dropping rules for different priorities.

- FFS: UE is not expected to be configured with the following cases:

  - slot-based PUCCH for unicast and sub-slot based PUCCH for multicast (if supported)

  - slot-based PUCCH for multicast and sub-slot based PUCCH for unicast

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | No need for any agreement now. First, a need to support sub-slot based HARQ-ACK reporting need to be identified |

#### Proposal 2.5.2.3-3

For multiplexing the HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on

- the PRI indicated in the last received DCI.

  - Details of the last received DCI

Collect views:

| Company | Comments |
| --- | --- |
| Samsung | Do not support. If the last DCI is for multicast, the proposal does not work. The last DCI has to be the unicast one. |

## Enable/disable HARQ-ACK feedback

Submitted Proposals

(Huawei) Proposal 8:

  - Support group-common DCI indicating enabling/disabling HARQ-ACK feedback.

(OPPO) Proposal 10:

  - For HARQ feedback enabling/disabling, only Option 3 is supported.

    - if RRC signalling does not configure the function, HARQ feedback is disabled in default;

    - if RRC signalling configures the function, 2 bits in DCI are used to indicate whether/which HARQ feedback is enabled;

- (Spreadtrum) Proposal 9:

  - At least RRC signaling is used to indicate enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast.

- (Spreadtrum) Proposal 10:

  - Not support enabling/disabling HARQ-ACK feedback by MAC-CE.

(ZTE) Proposal 9:

  - Regarding enabling/disabling HARQ-ACK feedback for MBS, option 2, 3 and 4 are supported together

    - Option 2: RRC indicates enabling/disabling

    - Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

    - Option 4: MAC-CE indicates enabling/disabling

- (vivo) Proposal 2:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, RRC indicates enabling/disabling (i.e. option 2) is supported.

- (CATT) Proposal 1:

  - When RRC signaling does not configure the function, enabling the HARQ-ACK feedback is the default mode.

- (CATT) Proposal 2:

  - Supporting Option 3 to indicate enabling/disabling,  1 extra bit indication in DCI or a special state index of k1/PRI can be the candidate schemes.

- (Nokia) Proposal 30:

  - RRC-based enabling / disabling (Option 2) of HARQ-ACK feedback is used for MBS and Option 3 is not supported.

- (Nokia) Proposal 31:

  - If use cases that require dynamic enabling / disabling are found, MAC-CE is preferred over the methods proposed in Option 3.

- (MediaTek) Proposal 4:

  - RRC signalling configures the enabling/disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback is supported for multicast services.

- (MediaTek) Proposal 5:

  - Disabling the feedback is as the default mode if the RRC signalling doesn’t configure the HARQ feedback function.

- (FUTUREWEI) Proposal 4:

  - Support option 3 for enabling/disabling HARQ-ACK feedback.

- (ETRI) Proposal 3:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, RRC signalling configuration for the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback is supported.

- (CMCC) Proposal 17:

  - Option 2 and Option 3 can be supported for enabling/disabling HARQ-ACK feedback for multicast.

- (CMCC) Proposal 18:

  - For Option 2, RRC can indicate enabling/disabling per UE. For Option 3, RRC can configure enabling/disabling HARQ-ACK feedback function per service, DCI can indicate enabling/disabling of HARQ-ACK feedback per group.

- (Intel) Proposal 3:

  - For NR MBS, Option 3 can be supported where if RRC configures enabling and disabling of HARQ, the default mode is HARQ ON if no DCI indication is provided.

- (Apple) Proposal 4:

  - HARQ-ACK feedback is only enabled or disabled by RRC signaling for PTM scheme 1.

- (Qualcomm) Proposal 4:

  - For RRC_CONNECTED UEs, HARQ-ACK feedback can be enabled/disabled by RRC signaling.

    - The configuration of HARQ-ACK feedback can be configured for a given G-RNTI (corresponding to a service) or for a UE receiving a service.

- (Samsung) Proposal 5:

  - Support enabling/disabling of HARQ-ACK reports for MBS PDSCH by UE-specific RRC. For Type-2 HARQ-ACK codebook, consider to also enable/disable HARQ-ACK reports by DCI scheduling MBS PDSCH.

- (LGE) Proposal 5:

  - Support both Option 2 and/or 3 for enabling/disabling HARQ-ACK feedback. Which option is used is up to gNB configuration.

- (Convida) Proposal 2:

  - For MBS HARQ-ACK feedback enabling/disabling, option 3 (i.e., RRC configures the enabling/ disabling function and DCI indicates enabling /disabling) is supported.

- (Lenovo) Proposal 7:

  - For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast, RRC signaling configures a non-numerical value in the K1 set and PDSCH-to-HARQ_timing indicator in the DCI indicates a numerical value or the non-numerical value for enabling or disabling the HARQ-ACK feedback.

(NTT DOCOMO) Proposal 6:

  - Support both Option 2 and Option 3 for enabling/disabling HARQ-ACK feedback for multicast.

    - Introduce an RRC parameter with 3 states {enabled, disabled, dynamic}.

      - When ‘enabled’, HARQ-ACK is enabled.

      - When ‘disabled’, HARQ-ACK is disabled.

      - When ‘dynamic’, if PRI is zero and PDSCH-to-HARQ feedback timing indicator is zero, HARQ-ACK feedback is disabled. Otherwise, HARQ-ACK feedback is enabled.

- (Ericsson) Proposal 7:

  - Enabling/disabling HARQ-ACK feedback for MBS is supported by both RRC configuration and MAC CE.

- (Ericsson) Proposal 8:

  - If enabling/disabling HARQ-ACK feedback for MBS is supported by MAC CE, the MAC CE message shall include a PUCCH resource indicator so that the UE knows which PUCCH resource to use. The mapping between PUCCH resource indicator and PUCCH resource is RRC configured.

### Round-1

FL’s Comments

The situation does not change significantly since the last meeting, i.e., substantial support of option 3 for dynamic indicating. For moving forward to find the common ground the different camps can live with, we can try the following proposal.

FL’s Proposal:

#### Proposal 2.6.1 (closed)

For supporting enabling/disabling HARQ-ACK feedback for multicast,

- UE-specific RRC signalling configures the function.

  - If the function is NOT configured, HARQ-ACK feedback is ON

  - If the function is configured,

    - Alt1: HARQ-ACK feedback is OFF

    - Alt2: UE checks the indication in the DCI/MAC-CE.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | This proposal is not clear to us. What does it mean “If the function is NOT configured, HARQ-ACK feedback is ON”? What is the “function”? |
| ZTE | We are fine with the above proposal. |
| CMCC | Fine |
| LG | We are not sure about this proposal. We wonder how no use of the function is configured for broadcast, if UE specific RRC signalling is used for broadcast. It seems natural that no configuration of enabling/disabling HARQ-ACK feedback means HARQ-ACK feedback is OFF. |
| CATT | Generally OK with this proposal. Regarding the Alt1 and Alt2, we think whether to down-select or support both of them should be clarified. |
| Nokia, NSB | Support the intention of the proposal, however, we propose the following wording … For supporting enabling/disabling HARQ-ACK feedback for multicast, - UE-specific RRC signalling configures the function. - If the function is NOT configured, - HARQ-ACK feedback is ON - If the function is configured, further configuration is provided indicating either of the following alternatives: - Alt1: HARQ-ACK feedback is OFF Alt2: UE checks the indication in the DCI/MAC-CE to decide dynamically on whether to provide HARQ-ACK feedback. |
| OPPO | We do not understand the meaning of “function” either. Or the proposal is trying to say the following: - HARQ-ACK feedback enabling is indicated by UE-specific RRC signalling; - HARQ-ACK feedback disabling is: - Alt1: indicated by UE-specific RRC signalling; - Alt2: indicated by UE-specific RRC signalling and DCI/MAC-CE; |
| Samsung | OK with the intention of the proposal but agree with the comment from LGE - no configuration means no HARQ-ACK report. Configuration can be implicit based on whether PUCCH-Config is provided – if nothing is provided, there is no HARQ-ACK and there is no need for a configuration to tell the UE there is no HARQ-ACK. |
| Apple | From our side, Alt.1 or Alt.2 needs to down select, not support both. |
| Qualcomm | Need to clarify whether to down-select Alt1 and Alt2, and whether to down-select DCI and MAC-CE. |
| Convida | We are not clear what are the two alternatives for? Are they two options to be down selected? Or they are two possible UE behaviours that both need to be supported? More clarifications are needed. |
| FUTUREWEI | We generally agree to have a default operation if configuring is not present, and can look at different alternatives when the signaling is present, one of which is related to DCI. |
| ETRI | Principle of the proposal is fine, but further discussion is required for the default HARQ-ACK feedback operation when the function is not configured. |
| MTK | The proposal is not clear for us. Whether the down select is needed? |
| Spreadtrum | Support the proposal with Alt.1. |
| vivo | The motivation to support dynamic enabling/disabling HARQ-ACK for a group of UE is not clear to us. For us, the enabling/disabling should be per UE, but for PTM scheme 1, DCI indicating method can only be per MBS group. It does not make sense to use a group-common DCI to enable/disable. |
| NTT DOCOMO | We share the view with Qualcomm, more clarifications are needed. |

### Round-2

FL’s Comments

The updated proposal aims to clarify the purpose of down-selection later and address how the UE-specific signalling is going to be like which could be FFS.

FL’s Proposal:

#### Proposal 2.6.2

For supporting enabling/disabling HARQ-ACK feedback for multicast,

- UE-specific RRC signalling configures the function.

  - If the function is NOT configured, HARQ-ACK feedback is ON/OFF (FSS)

  - If the function is configured, down-select between

    - Alt1: HARQ-ACK feedback is OFF

    - Alt2: UE checks the indication in (for down-selection).

      - Alt2-1: DCI

      - Alt2-2: MAC-CE

  - FFS: UE-specific RRC signalling details.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | This proposal is not clear to us. What does it mean “If the function is NOT configured, HARQ-ACK feedback is ON/OFF”? What does it mean “if the function is configured, HARQ-ACK feedback is OFF in Alt 1? |
| Samsung | How is the proposal different from/advancing the current state? |
| Ericsson | We also think the proposal is unclear. As a basis, we propose that enable/disable of HARQ-ACK is RRC configured with one of the following: - HARQ ACK/NACK or no HARQ feedback (if NACK-only is not supported) - HARQ ACK/NACK, NACK-only, or no HARQ feedback (if NACK-only is supported). This is sufficient for “infrequent” changes of HARQ-ACK state. For more frequent changes we propose that UEs may be RRC configured to support enable/disable and PUCCH resource reconfiguration via (UE-specific) MAC-CE. We do not see any use case for DCI-based enable/disable. We would like to ask proponents of DCI-based enable/disable to describe the intended use case for this. |
| Convida | This proposal is still not clear to us and we have the following questions. - What does ‘FSS’ mean in the first sub-bullet? Do you mean ‘FFS’? - What is the intention for Alt1? Why is HARQ feedback disabled (OFF) when the enabling/disabling function is configured? What is the use case for it? - If Alt1 is the HARQ feedback is always OFF, then in the first sub-bullet ‘If the function is NOT configured, HARQ-ACK feedback is ON/OFF (FSS)’, the HARQ feedback can be only always On, right? Otherwise, how is the HARQ feedback enabled and disabled? What is the intention to have ON/OFF here? Some further clarification would be appreciated. |
| Qualcomm | The enabling/disabling HARQ-ACK feedback is only applied to multicast RRC_CONN UEs, not applicable to broadcast for IDLE/INACTIVE. So no need FFS in the first subbullet and should be - If the function is NOT configured, HARQ-ACK feedback is ON |
| Spreadtrum | We are not clear with the first bullet. For the second bullet we prefer Alt.1 for simplicity. |
| OPPO | Seems the proposal does preclude any possibility. |
| CATT | The updated proposal is not clear for us. We understand the intention of this proposal is try to combine Option 2 and Option 3 which discussed in last meeting. But the meaning of ‘the function’ in this proposal should be clarified. Dose ‘the function’ means the enabling/disabling function which same as the last related agreement mentioned? Also, we do not understand ‘ON/OFF (FSS)’ in the first sub-bullet. In our understanding, if the function is not configured, the default mode is enabling (i.e. ON), as shown in the last proposal. |
| ZTE | The current proposal is not clear to us as commented by many other companies. We would prefer to resolve these FFSs before we jump into any agreements. |
| NTT DOCOMO | We interpreted ‘ON/OFF(FFS)’ in first sub-bullet as follows. - If Alt1 is selected, HARQ-ACK feedback should be ON when the function is not configured. Otherwise, HARQ-ACK feedback is always OFF. - If Alt2 is selected, the default behaviour is something to consider. That’s why the default behavior is ‘ON/OFF(FFS)’ We think that it will be beneficial to dynamically indicate HARQ-ACK feedback ON/OFF to minimize PUCCH overhead when providing multiple MBS services with different QoS levels. |
| Nokia, NSB | Share the same views as Ericsson and also want proponents of the DCI-based configuration method to clarify the intended use cases. |
| Apple | The issue was discussed in several meetings. Including all alternatives doesn’t help, down-selection can be performed. We prefer Alt.1. |
| vivo | The updated proposal is not clear for us. The meaning of ‘the function’ needs to be clarified. |
| MTK | The wording of “function” is needed to further clarify. HARQ ACK enabling/disabling dynamically indication by DCI is benefit, especially when repetition is configured, it doesn’t need to feedback each slot’s ACK/NACK. |

## HARQ-ACK feedback for multicast SPS

Submitted Proposals

(Spreadtrum) Proposal 1:

  - For PUCCH carrying HARQ-ACK information corresponding to SPS group-common PDSCH reception and SPS release, the priority is configured by RRC signalling.

(ZTE) Proposal 11:

  - Regarding HARQ-ACK feedback for SPS-based MBS transmission, if NACK-only feedback is configured

    - The NACK-only feedback mode applies to ‘PDSCH without scheduling PDCCH’ only

    - The feedback mode for ‘PDSCH with scheduling PDCCH’ can be,

    - Option 1: fixed to ACK/NACK feedback

    - Option 2: follow the configuration of feedback mode for DG-PDSCH

    - FFS: the feedback mode for deactivation PDCCH

- (Google) Proposal 1:

  - If NACK-only based HARQ-ACK feedback is supported, consider following methods to secure the reception of a SPS activation command

    - Option 1: It is based on base station implementation. (e.g. sending multiple activation commands)

    - Option 2: In response to the reception of a SPS activation command in a group-common PUCCH, the UE reports an ACK on a UE-specific PUCCH resource.

      - NACK-only based HARQ-ACK feedback is applied to SPS PDSCH.

    - Option 3: Utilize unicast uplink resources (e.g. MAC CE or RRC) to report ACK of the reception of the SPS activation command.

      - Detailed signalling design is up to RAN2.

- (Google) Proposal 2:

  - If NACK-only based HARQ-ACK feedback is supported, consider following methods to secure the reception of a SPS deactivation command

    - Option 1: It is based on base station implementation. (e.g. sending multiple deactivation commands)

    - Option 2: In response to the reception of a SPS deactivation command in a group-common PUCCH, the UE reports an ACK on a UE-specific PUCCH resource.

      - NACK-only based HARQ-ACK feedback is applied to SPS PDSCH.

    - Option 3: Utilize unicast uplink resources (e.g. MAC CE or RRC) to report ACK of the reception of a SPS deactivation command.

      - Detailed signalling design is up to RAN2.

    - Option 4: Introducing SPS configuration inactive timer/counter.

      - Detailed signalling design is up to RAN2.

(Nokia) Proposal 32:

  - Both ACK / NACK and NACK-only based HARQ-ACK feedback schemes are supported also in case of SPS.

(Nokia) Proposal 33:

  - Enhancements to SPS activation / deactivation mechanisms are needed to have reliable SPS grant in case of NACK-only feedback is used, in particular a mechanism for the gNB to be certain that all UEs have received the SPS grant would be desirable.

(Nokia) Proposal 34:

  - For NACK-only HARQ operation a mechanism should be used, in which UEs are made aware via RRC signalling that SPS might be used for an MBS and request retransmission of an SPS activation PDCCH only if they have not received it in a certain amount of time.

(Nokia) Proposal 35:

  - While gNBs can send SPS deactivation commands, that are in NACK-only mode not acknowledged by UEs, UEs can assume that SPS has been deactivated if they have not been able to decode a PDSCH for a certain period of time.

(Nokia) Proposal 36:

  - In NACK-only HARQ operation, a method is supported for UEs to check with the gNB whether an SPS (re-)activation has been sent by the gNB but missed by the UE. Options include:

    - (a) Option 1: Using a group-common uplink resource

    - (b) Option 2: Using UE-specific signalling (MAC-CE or RRC message)

    - (c) Other methods are not precluded.

(FUTUREWEI) Proposal 3:

  - Developing the SPS configuration for MBS should use the current specified SPS configuration as a baseline.

(Qualcomm) Proposal 6:

  - For ACK/NACK-based feedback of RRC_CONNECTED UEs receiving SPS GC-PDSCH,

    - UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS GC-PDSCH configurations.

    - Otherwise, a common SPS-PUCCH-AN-List applies to all SPS PDSCH and SPS GC-PDSCH configurations.

(LGE) Proposal 17:

  - For group common SPS, both group common NACK only based HARQ-ACK and UE specific ACK/NACK based HARQ-ACK are supported.

(LGE) Proposal 18:

  - For group common SPS, at least UE specific confirmation to group common SPS release can be supported by PUCCH A/N.

(LGE) Proposal 19:

  - For group common SPS activation/deactivation to multiple UEs in a group, (de)activation DCI can be repeated on multiple CORESETs with same TCI state or different TCI states.

(Convida) Proposal 5:

  - Both ACK/NACK based HARQ-ACK feedback and NACK-only based HARQ-ACK feedback should be considered for MBS SPS PDSCH.

(Convida) Proposal 6:

  - Consider to support HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI.

(Lenovo) Proposal 12:

  - It is up to gNB to allocate the total SPS configurations between multicast and unicast.

(Lenovo) Proposal 13:

  - For group-common SPS configuration, a UE-specific PUCCH resource is configured for each UE to transmit ACK upon reception of activation/deactivation DCI.

(Lenovo) Proposal 14:

  - For group-common SPS configuration, the UE-specific PUCCH resource for confirming reception of activation/deactivation DCI is also used for the UE to transmit ACK for the SPS PDSCH.

### Round-1

FL’s Comments

There are quite a lot of proposals for HARQ-ACK feedback for SPS multicast. However, most proposals talk about the HARQ-ACK feedback option as well as the detailed issues for supporting NACK-only based option for SPS multicast. FL assesses it would be more efficient to focus on the HARQ-ACK feedback options for dynamic scheduling first and see how to proceed the discussion for SPS multicast later. Regarding the SPS configuration, it might be discussed in AI 8.12.1.

For AI 8.12.2, the first-step issue we can discuss the priority and PUCCH resources indication for SPS multicast, for which SPS unicast precedent can be referred to.

FL’s Proposal:

#### Proposal 2.7.1 (closed)

For HARQ-ACK feedback for SPS multicast, UE-specific RRC signaling configures

- the HARQ-ACK codebook index.

- PUCCH resources ID.

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | - We don’t understand why HARQ-ACK codebook index is configured. - How many PUCCH resources are configured? |
| ZTE | Regarding the last bullet, we believe some more clarification is needed. Currently, 4 PUCCH resources can be configured for SPS for unicast, where each PUCCH resource corresponds to a particular bits range of HARQ-ACK bits. In the above proposal, do we plan to reuse the same mechanism? If yes, then maybe it is better to further clarify this a little bit. |
| CMCC | More clarification on HARQ-ACK codebook index, does it mean the priority index? |
| CATT | The intention of this proposal is not clear to us. We share the same concerns with ZTE/CMCC, more clarification is needed. |
| Nokia, NSB | Support the proposal |
| OPPO | For the first sub-bullet, we do not understand what HARQ-ACK codebook index is, maybe more clarification is needed. For the second sub-bullet, seems it is for the indication of PUCCH resource for HARQ-feedback of SPS multicast, basically we think mechanism in unicast SPS can be reused. |
| Samsung | The proposal is unclear. |
| Apple | The proposal needs more clarifications. |
| Qualcomm | The current proposal is not clear. We think the ACK/NACK-based for SPS GC-PDSCH could be similar as legacy unicast SPS PDSCH, but using a separate SPS-PUCCH-AN-List. |
| FUTUREWEI | The proposal appears to use the existing SPS configuration as a baseline for MBS. But it may need clarification which PUCCH-AN list is being indexed. |
| Spreadtrum | The current proposal needs some clarifications. |
| vivo | Similar view with ZTE. Multiple multicast SPS is supported. and a PUCCH resource list for multicast SPS similar as that for unicast SPS is needed. |
| NTT DOCOMO | We are fine with FL’s proposal. |

### Round-2

FL’s Comments

The intention was to follow the precedent of SPS-config for SPS unicast, where the HARQ-ACK codebook index indicates the HARQ-ACK codebook index for the corresponding HARQ-ACK codebook for SPS PDSCH and ACK for SPS PDSCH release. Also, the PUCCH resources ID is indicated in a specific SPS-config configuration with actual PUCCH resource configured in PUCCH-config.

FL’s Proposal:

#### Proposal 2.7.2

For support of HARQ-ACK feedback for SPS multicast, SPS-config configuration for SPS multicast at least includes

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH and ACK for SPS PDSCH release.

- PUCCH resources ID with actual PUCCH-Resource configured in PUCCH-Config.

  - FFS on SPS-PUCCH-AN-List

Collect views:

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | This proposal is not clear to us. We don’t know why HARQ-ACK codebook index is needed. |
| Samsung | OK in principle but why can’t the proposal be what was mentioned in the FL comments (i.e. SPS-config for multicast is as SPS-config for unicast for the HARQ-ACK codebook index and the PUCCH resources). |
| Ericsson | We agree with the other comments that the proposal could be more detailed. It seems the intention is to use unicast sps-config as a baseline. If so, we propose to either include the other fields from SPS-config in the proposal, or update the proposal to say that the discussion on the UE-specific RRC signaling for multicast SPS configuration uses the RRC configuration SPS-config as a baseline. |
| Qualcomm | If similar as unicast configuration, SPS-PUCCH-AN-List can be configured jointly for all the multicast SPS-Configs in a CFR, separate from that for all the SPS-Configs for unicast. |
| FUTUREWEI2 | OK in principle |
| CMCC | Ok in principle |
| OPPO | ok |
| CATT | Fine with this proposal. |
| ZTE | We are not sure about the intention about the FFS here. Do we want to introduce some new mechanisms for PUCCH resources ID here? It is preferred to reuse the current unicast SPS mechanism. |
| NTT DOCOMO | We are fine with the proposal. |
| Nokia, NSB | OK |
| Apple | Ok in principle |
| Spreadtrum | We are not clear about the FFS in the second bullet. If multiple MBS SPS PDSCHs are supported, we prefer to use SPS-PUCCH-AN-List for multicast SPS PDSCHs instead of PUCCH resources ID mentioned in the second bullet. |
| vivo | The second sub-bullet is not clear to us. It seems the intention of the proposal is to reuse the configuration way of HARQ-ACK codebook index and PUCCH resource for unicast SPS to multicast SPS. For unicast SPS, for each SPS-config, it includes n1PUCCH-AN, harq-CodebookID-r16. If multiple unicast SPS is configured, gNB will configure SPS-PUCCH-AN-List-r16 in PUCCH-config rather than SPS-config. |
| MTK | We are fine with the proposal. |

## Retransmission

Submitted Proposals

(CATT) Proposal 20:

  - When PTM scheme 1 is used as initial transmission, retransmission scheme supports PTM scheme 1 and PTP.

(CATT) Proposal 21:

  - CBG based PTP retransmission can be supported in multicast retransmission when a UE is configured with CBG transmission for unicast.

(Nokia) Proposal 11:

  - CBG-based (re-)transmissions are not supported for PTM (re-)transmissions.

(MediaTek) Proposal 6:

  - PTM scheme 1 or PTP retransmission is supported for ACK/NACK based HARQ feedback mechanism.

(MediaTek) Proposal 7:

  - PTM scheme 1 retransmission is supported for NACK-only based HARQ feedback mechanism.

(Intel) Proposal 9:

  - For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

- (Convida) Proposal 3:

  - For retransmission, option 3 (i.e., both option 1 and option 2) is supported.

- (Convida) Proposal 4:

  - PTM transmission scheme 2 should be supported for MBS retransmission.

(Lenovo) Proposal 15:

  - For SPS PDSCH transmission in PTM manner in initial transmission, PTP based retransmission is supported.

### Round-1

FL’s Comments

At this moment, we only have PTM scheme 1 and ACK/NACK based HARQ-ACK feedback being agreed. Also, PTM scheme1 and PTP used for retransmission for initial transmission of PTM scheme 1 has also been agreed in AI 8.12.1.

FL does not see necessary proposal to proceed at this moment for AI 8.12.2.

FL’s Proposal:

#### Proposal 2.8.1 (TBD)

Collect views:

| Company | Comments |
| --- | --- |

# PDSCH repetition

Submitted Proposals

(vivo) Proposal 1:

  - For PDSCH repetition of group-common PDSCH, if a UE can be configured with multiple g-RNTIs,

- The PDSCH aggregation factors for group-common PDSCHs with different g-RNITs should be separately configured.

(Nokia) Proposal 26:

  - For Config A, pdsch-AggregationFactor is per MBS service.

(Qualcomm) Proposal 5:

  - Support semi-static and dynamic slot-level repetition for SPS GC-PDSCH for RRC_CONN UEs receiving multicast.

    - Repetition configuration for SPS GC-PDSCH can be different than that of dynamic GC-PDSCH.

(LGE) Proposal 15:

  - For a group common SPS configuration, UE can be optionally configured with either pdsch-AggregationFactor or TDRA table with repetitionNumber as part of the TDRA table.

(LGE) Proposal 16:

  - Discuss whether different TCI states can be configured for group common SPS received by different UE, e.g. different slots of group common SPS PDSCH repetitions or different SPS configurations can be associated to different TCI states for the same group of UEs.

## Round-1

FL’s Comments

Per FL’s assessment, the proposals submitted for this issues either depend on the number of multicast service UE supports which would be better to be discusses in AI 8.12.1, or are interested by only a couple of companies.

FL would suggest it could be discussed later at an appropriate time.

FL’s Proposal:

#### Proposal 3.1 (TBD)

Collect views:

| Company | Comments |
| --- | --- |

# CSI feedback

Submitted Proposals

(ZTE) Proposal 12:

  - UE supports reporting multiple candidate {CQI, PMI, RI} sets in one CSI report for MBS

(ZTE) Proposal 13:

  - RAN1 further discusses the issues on CSI subband determination for MBS transmission

    - If common frequency resource is defined as an MBS frequency region within the unicast BWP, mechanism to align different UE’s CSI subband size is needed.

    - If common frequency resource is defined as an MBS BWP, no additional mechanism is needed (i.e., just reusing the previous MBS mechanism).

- (CATT) Proposal 22:

  - CSI feedback enhancement for MBS can be further studied and discussed but with low priority.

(Nokia) Proposal 27:

  - When using NACK-only based HARQ-ACK feedback along with CSI reporting, CQI measurements are done based on actual (time-averaged) BLER measurements at the UEs, rather than (instantaneous) CSI-RS based measurements.

(Nokia) Proposal 28:

  - New compact CSI report formats are defined for multicast transmission, where only a CQI or CQI along with an RI can be reported, and these formats are used in CSI reporting when NACK-only based HARQ-ACK feedback on group-common PUCCH resources is used.

(Nokia) Proposal 29:

  - The configuration for CQI reporting for PTM is extended to include not only the reliability target but also the number of HARQ transmissions per transport block after which the reliability target should be met.

(Qualcomm) Proposal 7:

  - For RRC_CONNNECTED UES, configure the CSI-RS resource per MBS CFR.

    - CSI-RS bandwidth is limited within the MBS CFR.

    - CSI-RS power is associated with GC-PDSCH power.

(Qualcomm) Proposal 8:

  - Support GC-PDCCH to trigger A-CSI-RS transmission in MBS CFR.

(Qualcomm) Proposal 9:

  - For RRC_CONNECTED UEs, support beam management for multicast assisted by unicast connection.

(Qualcomm) Proposal 10:

  - Consider SRS configuration for CSI measurement of multicast transmission in MBS CFR.

(Samsung) Proposal 7:

  - Support triggering of CSI reports by DCI scheduling MBS PDSCH or by MAC CE in MBS PDSCH.

(Ericsson) Proposal 17:

  - Group-common PDCCH can be used to trigger aperiodic CSI-RS transmission across multiple UEs of the same MBS group

## Round-1

FL’s Comments

Before diving into diverse different detailed proposals, one common high-level issues brought up by a few companies can be a starting point for the discussion, i.e., whether the group-common DCI for PTM scheme 1 scheduling can be used for A-CSI report triggering.

FL’s Proposal:

#### Proposal 4.1

For RRC_CONNECTED UEs receiving multicast, support group-common DCI for PTM scheme 1 scheduling

- indicating A-CSI report triggering

  - FFS: details.

Collect views:

| ZTE | This proposal seems to suggest to use DL DCI to trigger A-CSI report. It seems that triggering A-CSI via DL DCI is being discussing under URLLC session now. We would suggest to wait for URLLC outcome before we decide whether to support this in MBS session. |
| --- | --- |
| CMCC | Not support. From our perspective, gNB can re-use Rel-15/16 A-CSI triggering framework, this topic is not very critical to be discussed. |
| LG | We do not support this proposal. |
| CATT | The use cases of A-CSI are not clear to us. So we are open for this proposal. |
| Nokia, NSB | We support the proposal, but with the added FFS: FFS Schemes to trigger CSI using UE specific DCI |
| OPPO | It is not clear for us why a new A-CSI triggering scheme is needed for MBS. |
| Samsung | Support the proposal – a same functionality exists since Rel-15 for SRS triggering. If the proposal is not supported, how a UE without unicast traffic can provide CSI reports should be described (assuming there is no ‘always-on’ CSI-RS for each multicast service that may have different transmission points). |
| Qualcomm | We support this proposal. The GC-PDCCH to trigger the reporting based on a common A-CSI-RS is more important for multicast case. No need to wait for URLLC outcome. |
| Spreadtrum | The motivation of introducing a new A-CSI triggering scheme is not clear for us. |
| Convida | The motivation and the urgency of this proposal is not clear to us. We suggest to treat CSI enhancement for MBS as low priority. |
| vivo | Same view with ZTE. |
| MTK | We have the similar view with Convida. |

# Other miscellaneous proposals

Submitted Proposals

(OPPO) Proposal 11:

  - For a UE receiving group-common PDSCH transmitted with PTM scheme 1 a TPC-PUCCH-RNTI different from that for unicast should be configured.

(CATT) Proposal 18:

  - To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

(CATT) Proposal 19:

  - UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

(MediaTek) Proposal 8:

  - The total HARQ process number (e.g., 16) is unchanged for UE receiving unicast and multicast service.

(MediaTek) Proposal 9:

  - Independent HARQ process is allocated at gNB to PTM and PTP for downlink multicast transmission.

(MediaTek) Proposal 10:

  - A combined HARQ process is allocated at UE to receive the data from both PTM and PTP HARQ process.

(MediaTek) Proposal 11:

  - The total HARQ process number defined in Rel-15/16 (e.g., 16) is unchanged for UE receiving unicast and multicast service.

(MediaTek) Proposal 12:

  - The maximum HARQ process used for MBS is up to network implementation.

(MediaTek) Proposal 13

  - The HARQ process ID indicated in DCI for PTP retransmission is kept the same value with corresponding PTM scheme 1 initial transmission.

(FUTUREWEI) Proposal 1:

  - For the same service, all RRC_CONNECTED UEs receiving multicast are configured with the same MCS table.

(FUTUREWEI) Proposal 2:

  - For MBS, FBRM should be used to improve performance for small sized TBs.

(Intel) Proposal 2:

  - For the case when unicast and MBS PDSCH partially overlap in time on different PRBs, out-of-order HARQ feedback may be supported wherein the HARQ feedback corresponding to the PDSCH ending earlier in time may be transmitted after the HARQ feedback for the PDSCH ending later in time

(Intel) Proposal 7:

  - For NR MBS, no additional HARQ processes are defined and MBS shares HARQ process ID with unicast i.e., the total of 16 HARQ processes is unchanged.

FL’s Comments

Other miscellaneous proposals listed in this section might be discussed in AI 8.12.1, e.g., HARQ process management and related issues.

Collect views:

| Company | Comments |
| --- | --- |

# References

R1-2102319	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2102415	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2102470	Mechanisms to improve reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2102502	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2102543	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2102552	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

R1-2102610	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2102656	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2102703	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	MediaTek Inc.

R1-2102783	Discussion on improving reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2102845	Discussion on HARQ-ACK feedback method for MBS	ETRI

R1-2102875	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio Company Limited

R1-2102901	Discussion on reliability improvement	CMCC

R1-2103051	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2103125	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2103187	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2103261	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2103358	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2103363	Further discussion on reliability for RRC_CONNECTED UEs	Chengdu TD TECH, TD Tech

R1-2103420	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2103547	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2103595	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED Ues	NTT DOCOMO, INC.

R1-2103739	Discussion on reliability mechanisms for NR MBS	Ericsson

# Appendix Agreements summary for AI 8.12.2

## 102e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

## 103e

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS details of HARQ-ACK codebook design.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

## 104e

Agreement:

For ACK/NACK based feedback if supported for RRC_CONNECTED UEs receiving multicast, UE can be optionally configured a separate PUCCH-Config for multicast. Otherwise, PUCCH-Config for unicast applies.

Agreement:

The priority for HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast can be,

- Lower, higher than or equal to the HARQ-ACK feedback for unicast

  - FFS: How to reflect the priority in specification, e.g., whether it is configured or indicated to the UE

  - FFS: The total number of priorities across multicast and unicast

- FFS the priority between HARQ-ACK feedback for multicast and other UCI for unicast (SR, CSI) or PUSCH for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1,

- DAI for unicast and DAI for multicast are separately counted.

- Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported.

  - FFS details on   the codebooks.

- FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, support the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - It is up to network to configure orthogonal PUCCH resources among UEs within the same group.

- FFS: NACK-only based HARQ-ACK feedback for multicast,

  - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group.

- FFS details.

Agreement:

For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource,

- Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot.

  - FFS whether it is subject to UE capability.

- FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot.

- FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported.

- FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast.

Agreement:

For ACK/NACK based feedback if supported for multicast, construction of Type-1 HARQ-ACK codebook based on the union of the PDSCH TDRA sets of the unicast service and the multicast service (if they are separately configured), at least of the same priority, is supported

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast.

- FFS details of Type-1 HARQ-ACK codebook construction for FDM-ed multicast and multicast if supported.

- FFS: whether/how to optimize the Type-1 codebook construction to reduce the HARQ-ACK feedback payload size.

Agreement:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

For slot-level repetition for group-common PDSCH for RRC_CONNECTED UEs receiving multicast,

- (Config A) UE can be optionally configured with pdsch-AggregationFactor.

- (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table.

- If UE is configured with Config B, UE does not expect to be configured with Config A for the same group-common PDSCH.