3GPP TSG-RAN WG1 Meeting #104-bis-e	R1-2103990

e-Meeting, April 12th – April 20th, 2021

Agenda Item:	8.4.2

Source:	Moderator (Thales)

Title:	FL Summary on enhancements on UL time and frequency synchronization for NR NTN

Document for:	Discussion

# Introduction

This feature lead summary document captures the issues related to UL time and frequency synchronization in NR NTN. It contains a summary of the contributions under 8.4.2 at TSG-RAN WG1 #104-bis-e. together with identified key open issues and recommends topics/questions to be handled via email discussions. The goal of this document is also to provide recommendation on prioritization of discussion and whether any issues should be postponed.

Please note the following checkpoints for agreements: Checkpoints for agreements on Apr-15, Apr-20

# Content

Introduction	1

Content	1

1	Issue#1: NTN UE Timing Advance formula	3

1.1	Company views	5

1.2	Updated proposal based on company views (First round of email discussions)	9

1.3	Updated proposal based on company views (2nd round of email discussions)	11

2	Issue#2: Indication of Common TA drift rate	11

2.1	Company views	14

2.2	Updated proposal based on company views (First round of email discussions)	18

2.3	Updated proposal based on company views (2nd round of email discussions)	20

3	Issue#3: Granularity and signalling of Common TA	20

3.1	Company views	22

3.2	Updated proposal based on company views (First round of email discussions)	25

3.3	Updated proposal based on company views (2nd round of email discussions)	26

4	Issue#4: The need and the indication of TA margin	26

4.1	Company views	27

4.2	Updated proposal based on company views (First round of email discussions)	28

4.3	Updated proposal based on company views (2nd round of email discussions)	29

5	Issue#5: TA command in RAR	29

5.1	Company views	29

5.2	Updated proposal based on company views (First round of email discussions)	31

6	Issue#6 : TA update in RRC_CONNECTED state	31

6.1	Company views	34

6.2	Updated proposal based on company views (First round of email discussions)	38

6.3	Updated proposal based on company views (2nd round of email discussions)	38

7	Issue#7 : NTN UE Time Alignment Timers	39

7.1	Company views	39

7.2	Updated proposal based on company views (First round of email discussions)	42

7.3	Updated proposal based on company views (2nd round of email discussions)	42

8	Issue#8: TA reporting	42

8.1	Company views	43

8.2	Updated proposal based on company views (First round of email discussions)	45

8.3	Updated proposal based on company views (2nd round of email discussions)	45

9	Issue#9: Broadcasting the position of a reference point	45

9.1	Company views	45

9.2	Updated proposal based on company views (First round of email discussions)	47

9.3	Updated proposal based on company views (2nd round of email discussions)	48

10	Issue#10 : Common Doppler shift pre/post compensation on the feeder link	48

10.1	Company views	49

10.2	Updated proposal based on company views (First round of email discussions)	51

10.3	Updated proposal based on company views (2nd round of email discussions)	52

11	Issue#11: Indication of common frequency pre-compensation offset on DL service link	52

[TR38.821] :	52

6.3.2	DL synchronization	52

11.1	Companies views	54

11.2	Updated proposal based on company views (First round of email discussions)	56

11.3	Updated proposal based on company views (2nd round of email discussions)	57

12	Issue#12: Indication of common compensation frequency offset for Uplink	57

12.1	Companies views	58

12.2	Updated proposal based on company views (First round of email discussions)	59

12.3	Updated proposal based on company views (2nd round of email discussions)	59

13	Issue#13: Close control loop for UL frequency alignment	59

[TR38.821] :	59

13.1	Companies views	60

13.2	Updated proposal based on company views (First round of email discussions)	62

13.3	Updated proposal based on company views (2nd round of email discussions)	62

14	Issue#14: Serving satellite ephemeris format	62

14.1	Company views	64

14.2	Updated proposal based on company views (First round of email discussions)	68

14.3	Updated proposal based on company views (2nd round of email discussions)	71

15	Issue#15 Orbit propagator model in UE	72

15.1	Company views	72

15.2	Updated proposal based on company views (First round of email discussions)	74

15.3	Updated proposal based on company views (2nd round of email discussions)	74

16	Issue#16 SRS enhancements	75

16.1	Updated proposal based on company views (First round of email discussions)	76

17	References	77

18	Appendix I: RAN1 agreements on UL time and frequency synchronization for NR NTN	77

19	Appendix II: Summary of proposals	79

# Issue#1: NTN UE Timing Advance formula

The Timing Advance to be applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED was discussed in RAN1 Meeting #104-e, the following formula was proposed but not yet agreed:

Updated proposal 1-1:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

When common TA is indicated by the Network:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

   FFS:  Signaling and the granularity of NTA,common

   FFS: When common TA is not indicated

-        Note-1: Definition of NTA is different from that in RAN1 #103-e  agreement.

-        Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

-        Note-3: This agreement does not preclude aligned DL & UL timing at gNB or unaligned DL & UL timing at gNB with decision up to future discussion if any.

-        Note-4: NTA,common is the common timing offset as agreed in RAN1 #103-e

In RAN1 Meeting #104-e, the group was converging on this issue, but for some companies the proposal needs to clarify  the behaviour of the UE when the Common TA is not indicated.

In the contributions submitted to RAN1#104-bis-e, companies provided inputs to clarify the behaviour of the UE when the Common TA is not indicated and some inputs on the signalling and the granularity Common TA.

Proposals related to this issue are summarized in the following table:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 1: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: When common TA is indicated by the Network: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc When common TA is not indicated by the Network: TTA=NTA+NTA,UE-specific+NTA,offset×Tc Where: - NTA and NTA,offset are defined as in Release-16. - NTA,UE-specific is UE self-estimated TA - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. |
| OPPO | Proposal 1: apply TA as TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc, where - NTA is adjusted by RAR or TAC from MAC-CE - NTA,UE-specific is a UE-specific TA estimated based on NTN satellite ephemeris - NTA,common is a common TA indicated by the network - NTA,offset is a cell-specific offset Proposal 2: When the common TA is not provided by the network, UE assumes NTA,common=0. |
| Spreadtrum Communications | Proposal 2: The value of common TA defaults to zero when not indicated by the network. |
| PANASONIC R&D Center Germany | Proposal 1: We propose the Common Timing Offset value formulation expressed by multiples of Tc (TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc) with NTA,common having a granularity in the order of slot or half slot duration. |
| CATT | Proposal 3: When the common TA is not indicated by the gNB, common TA should be set to zero. |
| MediaTek Inc. | Proposal 1: Configuration of NTA,common is up to gNB depending on assumption for reference point for DL subframe and UL subframe alignment - NTA,common = RTD of feeder link if reference point is gNB - NTA,common =0 if reference point is satellite |
| ZTE | Proposal 1: The indication of common TA should always be assumed. |
| Xiaomi | Proposal 2: The UE should assume that NTA,common=0, when common TA is not broadcasted by network. Proposal 3: NTA is controlled by RAR and TAC in MAC-CE as in Release-16. Proposal 4: Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode assume NTA =0. |
| Mitsubishi Electric RCE | Proposal 2: In NTN, the network broadcasts a beam-specific default value NTA_default of the user-specific NTA value for initial access purposes. |
| Ericsson | Proposal 2: The TA for NTN should use the legacy granularity of Tc units or a multiple of Tc units, i.e., the common TA component X should be placed within the brackets as follows: NTA+NTA,offset +X×Tc Proposal 3: The TA to be used by NTN UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states should be as follows: TTA=NTA+NTA,offset+NTA,UE-specific+NTA,common×Tc where: NTA and NTA,offset are defined as in Rel-16. NTA,UE-specific is UE-autonomous TA calculated based on the GNSS-acquired UE position and the serving satellite ephemeris to pre-compensate for the service link RTT. NTA,common is network-controlled common TA to compensate (e.g.) for feeder link RTT. Proposal 4 The UE calculates the UE-specific TA (in Tc units) as follows: NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the signal the UE is about to transmit Proposal 6 The UE determines the common TA (in Tc units) as follows: NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to be transmitted by the UE will be relayed by the satellite. |
| Apple | Proposal 1: The TA used for Msg1/MsgA transmission is given by TA=NTA+NTA, UE-specific+NTA, common+NTA,offset×Tc, where - NTA, NTA,offset and Tc are defined as in Release-16 - NTA, UE-specific is UE self-estimated TA - NTA,common is network-controlled common TA if indicated by network, or is 0 if not indicated by network. |
| Qualcomm Incorporated | Proposal 1: - In NTN, the network may broadcast a common timing offset value with granularity of one slot assuming SSB subcarrier spacing. - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: TA= NTA+NTA, offset×Tc+X where X is derived from the common timing offset. |
| Samsung | Proposal 3: The TA update with TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc is used. |
| InterDigital, Inc. | Proposal 1: X is expressed as the legacy granularity of Tc unit. |
| LG Electronics | Proposal 3. Support the latest updated proposal 1-1 in R1-2102215 [3] |
| CEWiT, IITM, IITH | Proposal 1: The final equation for the full TA at UE should be, TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA and NTA,offset are defined as in Release-16. - NTA,UE-specific is UE self-estimated TA - NTA,common is network-controlled common TA and may include any timing offset considered necessary by the network. Proposal 9: In UE specific TA estimation, TA value estimated by UE is quantized using predefined step size to get the final TA. The step size can be the TA margin configured to the UE. |
| CAICT | Proposal1: In the scenario of transparent satellites, setting whether the satellite or the gNB as the reference point, both options have advantages and disadvantages and shall be supported. |
| Intelsat | Proposal 1: The TA used for Msg1/MsgA transmission is given by TA=NTA+NTA, UE-specific+NTA, common+NTA,offset×Tc, where - NTA, NTA,offset and Tc are defined as in Release-16 - NTA, UE-specific is UE self-estimated TA NTA,common is network-controlled common TA if indicated by network, or is 0 if not indicated by network. |

## Company views

Based on the submitted contributions  to RAN1#104-bis-e the majority is supportive of the formula of TA  discussed in  RAN1 #104-e [Oppo, Spreadtrum Communications, CATT, Xiaomi, Apple] proposed to clearly state in the proposal that UE assumes NTA,common=0 when the common TA is not provided by the network. According to [MediaTek ] configuration of NTA,common is up to gNB depending on assumption for reference point for DL subframe and UL subframe alignment: NTA,common = RTD of feeder link if reference point is gNB and 	NTA,common =0 if reference point is satellite.

For [ZTE] The indication of common TA should always be assumed.

On the other hand, [Qualcomm] observed that compensation of feeder link timing drift by UE can impose significant implementation complexity and may not be feasible. It is more efficient to have network compensate feeder link timing drift in a way transparent to UE. And proposed that a part of the feeder link delay can be compensated by UE for ease of network implementation. the network may broadcast a common timing offset value with granularity of one slot assuming the subcarrier spacing of the SSB of the cell.

[Moderator’s view]: When the reference point for time synchronization is the satellite, the gNB may not need to indicate the Common TA which is de facto equal to 0 (if it does not absorb TA margin as proposed by some companies). Thereby, when Common TA is not present in the SIB the UE shall assume NTA,common= 0.

Given the views expressed on this issue, the Initial Proposal 1 is made as follows:

Initial Proposal 1:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

When common TA is indicated by the Network.

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

When common TA is not indicated by the Network.

TTA=NTA+NTA,UE-specific+NTA,offset×Tc

Where:

- NTA and NTA,offset are defined as in Release-16.

- NTA,UE-specific  is UE self-estimated TA calculated by the UE based on the GNSS-acquired UE position and satellite ephemeris.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

Note-1: Definition of NTA is different from that in RAN1 #103-e  agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset as agreed in RAN1 #103-e.

- Companies are encouraged to provide their comments and views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree. This can be the baseline for further discussion. |
| Thales | We support the proposal |
| OPPO | OK with the main formula BUT we think that NTA and NTA,offset may NOT be exactly same as R16. This should be discussed later depending on how the NTA is adjusted and whether a TA margin is needed or not and if needed, whether the TA margin can be included in NTA,offset |
| Nokia, Nokia Shanghai Bell | The description of N_TA, UE-specific should simply refer to”… based on GNSS-acquired information” to ensure that the referenceTimeInfor-r16 is not precluded. |
| MediaTek | Support moderator proposal |
| Intel | Support the proposal |
| Apple | We support the proposal. The proposal covers both cases of timing reference point is at satellite and gNB, and hence provides configurability and flexibility. |
| Intelsat | We support the proposal. |
| Ericsson | We support the proposal with the following proposed clarification: - NTA,UE-specific is UE self-estimated TA calculated by the UE based on the GNSS-acquired UE position and satellite ephemeris to pre-compensate for the service link delay. Besides this (and not contradicting Initial Proposal 1), we want to highlight an aspect (further discussed in R1-2103059) that should be considered when calculating the service and feeder link delays: to accurately calculate the delay that a given signal has experienced (or will experience), it is the distance between the position of the transmitting node at the time of transmission to the position of the receiving node at the time of reception that determines the delay. Since the time of transmission is different from the time of reception, this does not correspond to the distance between the nodes at any given point in time. In other words, accurately determining the link delays requires a dynamic view of the signal propagation, as opposed to a static snapshot. This is illustrated in the figure below. |
| Sony | We agree with the proposal. |
| Xiaomi | Support the proposal |
| CMCC | Not agree. The indication of common TA should always be assumed to absorb TA margin and/or to compensate for the RTT of the feeder link and possibly other latencies in the satellite-gNB path. Furthermore, for an NR NTN UE in RRC_CONNECTED, there is no need to update NTA,common regularly, i.e., NTA,common=NTA,common0 and TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc where NTA,common0 is the common TA obtained in initial access procedure. |
| QC | Support if we clarify that signaling detail and granularity of NTA_common is FFS. |
| LG | Agree in principle, but we prefer the unified formula regardless of common TA indication. Therefore, we propose the following modified proposal: Modified Initial Proposal 1: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: When common TA is indicated by the Network. TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc When common TA is not indicated by the Network. TTA=NTA+NTA,UE-specific+NTA,offset×Tc Where: - NTA,common is 0 if common TA is not indicated by network. *** Unchanged text omitted *** |
| ZTE | We still prefer the unified framework to support both aligned and unaligned case. And the common TA should be always here and we can further discussion the value of this indication. More specifically, if the TA margin is not needed with tight requirement defined in RAN4, the value of common TA can be 0. Otherwise, the minimal value will be determined by the error. It’s not clear why we need to define the dedicated separate branch. |
| APT | Support Initial Proposal 1. In 3GPP (TS 38.331), we may use the term “if absent”, e.g., if the field of common TA is absent, the UE applies the value of 0 for common TA. However, to be honest, whether common TA is optional or mandatory shall not be discussed now. UE behaviour is the same whether UE knows the RP is at a gNB or at a satellite. |
| NTT DOCOMO, INC. | We think this proposal can be discussed after the issues on common TA drift rate (Issue#2) and common TA margin (Issue#4) are resolved. When the agreements on those two issues are achieved, the TA formula would be clear. |
| CATT | Support the proposal. The proposal reflects current discussion’s outcome, and covers both cases of timing reference point is at satellite and gNB. This is the baseline for further discussion, otherwise, not sure how to move forward. |
| Spreadtrum | Support the proposal. |
| Huawei, HiSilicon | We would like to point that for the case when common TA is not signalled, there will be a lot of issues we need to resolve including the ones that are being discussed in 8.4.1, e.g. MAC-CE action timing, start time for RAR window, etc. Our understanding is that for these cases, some parameter reflecting the feeder link delay is still needed so that the difference between the UL and DL frame timing at the gNB can be know at the UE. It is not entirely clear why we have the parameter anyway but not used for UL TA and complicate a lot of other UE procedures. |
| Panasonic | In our view, NTA,common should be always signaled to absorb any margin deemed necessary by the network. Modified Initial Proposal 1: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA and NTA,offset are defined as in Release-16. - NTA,UE-specific is UE self-estimated TA calculated by the UE based on the GNSS-acquired UE position and satellite ephemeris. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. Note-1: Definition of NTA is different from that in RAN1 #103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset as agreed in RAN1 #103-e. |
| Lenovo/MM | Agree with moderator’s proposal. |
| Fraunhofer IIS, Fraunhofer HHI | We support the proposal. |
| Sequans | Support the proposal. Details on common TA can be decided next. |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | We support this proposal |

## Updated proposal based on company views (First round of email discussions)

25 companies provided views on this proposal. 23 are supportive of the proposal 1

[Oppo] highlights that  NTA and  NTA,offsetmay NOT be exactly same as R16.

In Moderator’s view NTA,offsetshould remain the same: It was introduced for specific purposes: a TDD base station requires this margin to allow for the delay associated with activating its transmitter. It was also used to avoid gNB to gNB interference. It applies also to FDD to ensure that UL transmissions are time synchronized when using FDD/TDD Carrier Aggregation. It depends also on band and LTE/NR coexistence.

Further, RAN1 might discuss if NTA,offsetis really needed for NTN.

W.r.t to NTA  its definition is modified in the updated proposal.

[Nokia]: to ensure that the referenceTimeInfor-r16 is not precluded and [Ericsson]: add clarification on  NTA,UE-specific. The proposal is updated following Nokia’s comment.

[CMCC]: not agree because the indication of common TA should always be assumed to absorb TA margin and/or to compensate for the RTT of the feeder link

In Moderator’s view if the reference point is satellite, it could be possible that the Network does not need to indicate it. The updated proposal is a compromise to be more generic and support the different RP positions.

[QC]: Support if we  clarify that signalling detail and granularity of  NTA_common is FFS.:

[Moderator]: Granularity and  signalling of Common TA are discussed in Issue2 and Issue3 respectively. But I think it is fine to add this FFS  to this proposal. Let’s see other feedbacks from the group.

[LG, ZTE, Panasonic, Huawei ] agree but prefer the unified formula regardless of common TA indication.

[Moderator]: The updated proposal is a compromise to be more generic and support the different RP positions

[NTT DOCOMO]: To be discussed after the issues on common TA drift rate (Issue#2) and common TA margin (Issue#4) are resolved[Moderator]: In Moderator’s view: We need this fundamental proposal as starting point then Issue#2 and 3 can be discussed next. I added an FFS as proposed by QC.

Based on the first round of email discussions and the discussions via the reflector Initial Proposal 1 is updated as follows:

Updated Proposal 1:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and should include any timing offset considered necessary by the network.

- NTA,common is 0 when common TA is not indicated by network.

  - FFS:  the signaling and the granularity of NTA,common

- NTA,offset is as defined in Release-16.

Note-1: Definition of NTA is different from that in RAN1 #103-e  agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset as agreed in RAN1 #103-e.

Note-4: This does not preclude explicit  indication of TA margin if needed.

- Companies are encouraged to provide their comments and views on the updated proposal 1:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree |
| Apple | We support this proposal. |
| CATT | Agree |
| LG | Agree |
| CMCC | We support this proposal. |
| Sony | We support this proposal. |
| Spreadtrum | We support this proposal. |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | This proposal has been discussed intensively on the reflector so we will not relate to this proposal as such. |
| Ericsson | We support the proposal with the modifications proposed on the reflector. |
| Fraunhofer IIS, Fraunhofer HHI | We support the updated proposal over the reflector. |
| ZTE | This proposal has been discussed intensively on the reflector and attest version is up to the discussion |
| DOCOMO | We support the main bullet (i.e., the unified TA formular) in this proposal with details of the sub-bullets discussed on the reflector. |

## Updated proposal based on company views (2nd round of email discussions)

Updated Proposal 1 has been discussed intensively on the reflector. Based on the discussions via the reflector the proposal is modified as follows:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA  is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA,UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

Note-1: Definition of NTA is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

# Issue#2: Indication of Common TA drift rate

The discussions on the necessity of indicating the Common TA drift rate were started since RAN1#103e, but such indication is still FFS.

When the Common TA needs to be considered in the TA calculation, the network shall provide/indicate assistance information to be used by the UE for the estimation of Common TA when needed (i.e. before each uplink transmission).

To assist the UE to self-estimate the NTA,common  at time t, the gNB shall broadcast the Common TA (NTA,common  ) regularly (e.g. every 160ms). But as already discussed in RAN1#104-e, indicating only the Common TA is not enough.

Many contributions provided inputs to RAN1#104-bis-e on the need and benefit of indicating the Common TA drift rate.

Furthermore, some contributions discussed the benefits of indicating the Common TA drift variation rate as well [Thales, MediaTek]:

Indeed, using indicated Common TA drift rate, the UE can self-estimate the Common TA based on a first-order approximation as follows :

Solution 1:

NTA,Common t= NTA,common t0+   TACommondrift, t0×(t-t0+δt0)

Moreover, for better accuracy of self-estimated Common TA a second-order approximation may be needed, thereby, both Common TA drift rate and Common TA drift variation rate may need to be indicated.

Solution 2:

NTA,Common t= NTA,common t0+   TACommondrift, t0×t-t0+δt0+12 TACommondrift_variation_rate, t0×t-t0+δt02

NTA,common t0, TACommondrift, t0 and TACommondrift_variation_rate, t0 denote respectively the Common TA, Common TA drift rate and Common TA drift variation rate acquired by the UE at SIB period t0.

δt0 denotes one-way propagation delay from gNB to the UE at time t0 (UE’s timing). This needs to be considered  in both equations to take into account the ageing of NTA,common t0 values (the NTA,common t0 at time t0 is outdated by δt0 at the reception by the UE).

The following table recaps the proposals submitted to RAN1#104-b-e regarding Issue#2:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 6: Indicate a common TA drift rate as part of the common TA applied by UE. Proposal 7: For Msg1/MsgA transmission and TA maintenance, use a common TA drift rate to compensate the TA drift between SIB decoding and UL transmission. |
| OPPO | Proposal 5: the network indicated common TA can be updated by a drift rate, e.g. NTA,common_new= NTA,common_old + drift rate * t |
| PANASONIC R&D Center Germany | Proposal 4: We propose to at least allow for a non-zero DL-UL timing difference manged by the gNB FFS: The need and feasibility of a common timing drift rate to accurately compensate the feeder link delay including feeder link switch operation. |
| CATT | Proposal 5: Whether to broadcast the common timing drift rate needs further investigation. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 1 Support the common TA drift rate to improve the required periodicity of the SI-message to signal the common TA from every 80ms to every 1.6 seconds. |
| MediaTek | Proposal 2: In case reference point for UL-DL subframe timing alignment is gNB, broadcast on NTN SIB - Common delay NTA,common in a 18 bit field - Common delay drift rate NTA,common,drift, rate in a 8 bit field - Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field |
| CMCC | Proposal 1: The common TA is determined as NTA,commonn=NTA,common, base+(n-nTA,common,base)∙NTA,common,drift where: n is the slot number of the targeted UL slot. nTA,common,base is a “time stamp” slot number. NTA,common, base is the common TA (in Tc units) at slot number nTA,common,base. NTA,common,drift is the common TA drift rate (in Tc units per slot). and, NTA,common, base=A+S∙X NTA,common,drift=S∙Y where: A represents the synthetical impact of time-invariant parameters. S is a scaling factor. X, Y are time-variant parameters, and they need to be updated with time Proposal 2: Indication of parameter A (a time-invariant offset of NTA,common, base) should be supported. - For DL and UL aligned at satellite, A can be used to absorb TA margin. - For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link. Proposal 3: Indication of parameter X and Y at least should be supported. - For DL and UL not aligned at satellite (including aligned at gNB), X and Y should be indicated and updated to capture the rapidly changed RTT on the feeder link. |
| ZTE | Proposal 3: Indication of common TA drift rate should be supported to assist TA adjustment. |
| Xiaomi | Proposal 5: The common timing drift rate indicated by network should be supported. |
| Intel Corporation | Proposal 1: • Support common TA drift rate indication - Alternatively, broadcast of reference point for pre-compensation of feeder link delay can be considered to avoid frequent reconfiguration |
| Ericsson | Proposal 5 The network broadcasts parameters describing the common delay by a linear function as follows: Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift where: t is the time the signal passes the satellite Tcommon,base is a reference time of the broadcast common delay Dcommon, base is the common one-way delay at time Tcommon,base Dcommon, drift is the common one-way delay drift rate Proposal 7 The characterization of the common delay should include drift rate information. |
| Apple | Proposal 3: Consider common TA drift rate is broadcasted together with common TA. If common TA is not indicated, then the common TA drift rate is also not indicated. Proposal 4: The network-controlled common TA NTA, common is obtained by NTA, common=NTA,common, RX +DTA, common,RX ⋅Δt, where NTA,common, RX is the latest received common TA, DTA,common, RX is the latest received common TA drift rate and Δt is the time gap between the latest common TA reception and the corresponding uplink transmission, in the unit of Tc. |
| Samsung | Proposal 7: The gNB signals common TA drift rate to enable autonomous TA update at UE. Proposal 8: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa. |
| Sony | Proposal 3: RAN1 should support the signalling of timing drift rate information to the UEs in a beam specific manner. |
| Lenovo, Motorola Mobility | Proposal 3: Support indication of common timing offset drift rate. |
| NTT DOCOMO, INC. | Proposal 1: Common timing drift rate can be indicated to users to cover all the foreseen implementations and scenarios. Proposal 5: One or multiple common TAs can be broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy. |
| LG Electronics | Proposal 4. Support additional signaling by the network in order to apply proper common timing offset according to time changes at UE side. Potential solutions can include - Alt 1) providing the reference time corresponding to common timing offset - Alt 2) providing series of common timing offset. Proposal 8. It can be considered that the common TA is calculated based on K_offset in case when the UE cannot acquire the accurate TA. |
| Fraunhofer IIS, Fraunhofer HHI | Proposal 2: The characteristic of the common RTT/delay (or feeder link RTT/delay) shall be taken into account for the design of its corresponding signaling. Proposal 4: RAN1 to consider the signaling of the parameter set NTA,scale, drift scale rate, NTA,cons, a constant, and NTA,power, an exponent (can take integer values), for signaling of the common RTT/delay in NTN. Proposal 5: RAN1 to consider UE autonomous drift rate calculation for common RTT/delay signaling. |
| Nokia, Nokia Shanghai Bell | Proposal 8: It should be left up to the gNB to decide whether to broadcast the TA drift value or not. |
| Sequans Communications | Proposal 1: For common TA broadcast design, RAN1 shall consider drift rate accuracy and signalling overhead aspects. |
| CEWiT, IITM, IITH | Proposal 3: In NTN, the network may broadcast a common timing drift rate and update duration to update the common TA by UE in periodic interval. It can be broadcasted in the NTN specific SIB. |

## Company views

According to the contributions submitted to RAN1#104-bis-e the benefit of providing a drift rate is clear to the majority of companies.

The indication of common TA drift rate should be supported as part of the characterization of the common TA according to [ZTE, CMCC, Xiaomi, Intel Corporation, Ericsson, Apple, Samsung, Sony, Lenovo, Motorola Mobility, NTT DOCOMO, INC. Fraunhofer IIS, Fraunhofer HHI, Sequans Communications, CEWiT, Huawei, Thales].

[Huawei] observed that applying a common TA drift rate at the UE can reduce the closed loop signaling overhead and avoid frequent SIB1 decoding and proposed to indicate a common TA drift rate as part of the common TA applied by UE.

[Thales, MediaTek] proposed to use for self-estimated Common TA a second-order approximation for better accuracy.

[Thales] observed that by considering the most restrictive case (i.e. UE in Connected state and 120kHz SCS) the UE can self-estimate the Common TA with sufficient accuracy provided that an updated related assistance information is available at the UE during the last 0.7 s when Common TA drift rate is indicated or during the last 5s when both Common TA drift rate  and Common drift variation rate are indicated. Alternatively, [Thales] proposed to indicate averaged Common TA Drift rate instead of Instantaneous Common TA Drift rate if Common TA drift variation rate is not broadcast.

[Asia Pacific Telecom] Support the common TA drift rate to improve the required periodicity of the SI-message to signal the common TA from every 80ms to every 1.6 seconds

According to [Nokia] It should be left up to the gNB to decide whether to broadcast the TA drift value or not.

For [PANASONIC, CATT] Whether to broadcast the common timing drift rate needs further investigation.

Based on the inputs from the contributions submitted to RAN1#104-bis-e, the initial proposal 2 is made as follows:

Initial proposal 2:

When Common TA shall be considered in UE autonomous TA calculation, the Network shall periodically broadcast:

- NTA,common

- And other assistance information to be used by the UE for the self-estimation of Common-TA:

  - Option 1: Broadcast of Instantaneous Common TA Drift rate, without Common TA drift variation rate indication

  - Option 2: Broadcast of averaged Common TA Drift rate, without Common TA drift variation rate indication

  - Option 3: Broadcast of Instantaneous Common TA Drift rate, With Common TA drift variation rate

| Companies |  |  | First preference | Second preference |  |  | Unacceptable option(s) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung |  |  | Option 1 | Option 2 |  |  | We are not sure “instantaneous” and “average” common TA drift rate is important. We can just discuss how a UE applies the indicated TA drift and then, how to define the drift rate is gNB implementation. |  |
| Thales |  |  | Option 3: Based on simulation it is clear that it would be beneficial to indicate as well the Common TA drift variation rate. This will avoid to some degree frequent decoding of system information: e.g. in case of 120kHz SCS, UE will need to read the SIB almost every half-second, and in case of 15 kHz SCS, every 2 seconds. if the Common TA drift variation rate is not indicated. But a field of 6 bits will be need to indicate Common TA drift variation rat. | Option 2 is our second preference |  |  |  |  |
| OPPO |  |  | It should be clarified what the different UE behaviour is between option 1 and option 2? |  |  |  |  |  |
| Nokia, Nokia Shanghai Bell |  |  | General comment: The formulation prior to broadcast options is way too strict. In our preference is should focus on the information that the gNB is providing for the UE to apply. That is: When broadcasting information related to common TA, the information may contain: N_TA,common, TA drift rate, etc. And then extend the explanations afterwards. Our first preference would be to have the gNB broadcasting a common TA drift rate without TA drift variation rate. The UE should be indifferent to which metric is used, as it would simply extrapolate this value from a given reference time point. If a “bias” is added from gNB side, this might reduce the average error (so a mixture of option 1&2) |  |  |  | Option 3: this would simply introduce an additional overhead. |  |
| MediaTek |  |  | Option 3 | Option 2 (less accurate than Option 3) |  |  | Option 1 is not sufficiently accurate and would require UE to read SIB carrying common TA every second |  |
| Intel |  |  | Option 1 or Option 2 depends on gNB implementation | Option 3 |  |  |  |  |
| Apple |  |  | Option 1 or Option 2 Comments: - From specification point of view, we do not see the difference on whether the broadcast common TA is instantaneous or averaged. - If common TA is not broadcasted as in Issue #1, we do not need to broadcast common TA drift rate. | Option 3 |  |  |  |  |
| Intelsat |  |  | Option 1 or Option 2 It is dependent somewhat on the implementation details. |  |  |  |  |  |
| Ericsson |  |  | We support broadcasting a drift rate but cannot select between the three options for the following reasons: - How the drift rate is calculated (e.g., averaged or instantaneous) should be a network implementation choice (transparent to UE). - Further study needed for drift variation rate indication. Further, we propose to broadcast a first-order approximation of the one-way common delay instead of the (two-way) TA. The reason is that it is beneficial to consider DL and UL delays separately since the DL signal (to which the UE has synchronized) and the UL signal (for which the TA is calculated) are offset in time and therefore subject to different delays on the feeder link. The common TA is the sum of the common DL delay and common UL delay. More information can be foind in R1-2103059. |  |  |  |  |  |
| Sony |  |  | Option 1 We support the timing drift rate to improve the throughput as our tdoc. On the other hand, we should agree to support the timing drift rate at first, and then we should consider the detail of timing drift rate. | Option 2 |  |  | Option 3 |  |
| Xiaomi |  |  | Option 1 or Option 2 depends on gNB implementation |  |  |  | We can just discuss how a UE applies the indicated TA drift and then, how to define the drift rate is gNB implementation. |  |
| CMCC |  |  | Option 1 or Option 2 From specification point of view, we do not see the difference on whether the broadcast common TA is instantaneous or averaged. |  |  |  |  |  |
| QC |  |  | There has been no agreement that common TA drift rate will be signalled. No TA drift signalling should also be one of the option. Many questions need to be answered before we even consider the siganling of a drift rate: - Benefits and necessity from system point of view - Impacts on UE implementation and requirements: UE TA adjustment rate is at least doubled - Impacts on timing accuracy requirements: additional source of timing error and its impact on UE GNSS error and ephemeris update rate - Impacts on signaling overhead: SIB signaling overhead and impacts on UE power consumption in SIB reading We don’t see the need of signalling of common TA drift rate. |  |  |  |  |  |
| LG |  |  | Option 1 | It is necessary to discuss that the network can provide the reference time corresponding to common timing offset. Moreover, it can be considered that the network can provide series of common TA instead of common TA drift rate. |  |  |  |  |
| ZTE |  |  | We are supportive for the indication of common TA drift and only the 1st order is enough. Moreover, there is no need to specify the how to calculate the drift, e.g., Instantaneous or averaged since from gNB’s implementation, the indicated value is just a factor to approximate the potential changes of common TA. Other factors may also be considered instead of simple action. |  |  |  |  |  |
| APT |  |  | Option 3 | Option 1 |  |  | Option 2. The average common TA drift rate will be zero if the observation window is long enough. It is unclear how to define the window size (introducing a new timer?). |  |
| NTT DOCOMO, INC. |  |  | Option 1 or Option 2 Whether to broadcast instantaneous or averaged common TA drift rate should be network implementation. |  |  |  | Option 3: Additional overhead and complexity |  |
| CATT |  |  | Current initial proposal 1 has not been agreed. It is too early to down-select which option, need more time to discuss its benefit and its usage. |  |  |  |  |  |
| Huawei, HiSilicon |  |  | There seems no difference from the UE perspective between option 1 and option 2, whether the indicated common TA drift rate is instantaneous or averaged is transparent to UE. And the gNB will update the indicated value by selecting a proper periodicity. |  |  |  |  |  |
| Panasonic |  |  | In our view, it is premature to decide for adjusting the timing advance in any way at this meeting. As described in our contribution R1-2102555 the presence of a timing drift rate complicates the feeder link switch operation. As an alternative we proposed that the feeder link drift is managed by the gNB. This implies a misaligned DL/UL frame timing common among all UEs at the gNB. |  |  |  |  |  |
| Lenovo/MM |  |  | Our first preference is Option 2 and second preference is Option 3. |  |  |  |  |  |
| Fraunhofer IIS, Fraunhofer HHI |  |  | We support the signaling of the assistance information to UE for UE autonomous N_TA,common calculation at this stage. Details of the signaling can be FFS, as several aspects mentioned by QC should be investigated first. Also, it seems that this proposal is closely related to initial proposal 3-2. |  |  |  |  |  |
| Sequans |  |  | Agree with first part, i.e. N_(TA,common) broadcasting. Drift rate and its variation rate need to be examined more to understand feasibility. |  |  |  |  |  |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | Option 1or option 2 It does not really matter whether the provided parameter is instantaneous or average from UE perspective. It is up to network to decide how it want to signal the parameter |  |  | Option 3. Need to further study the overhead and expected gain due to TA drift variation rate |  |  |

## Updated proposal based on company views (First round of email discussions)

In first round of email discussions, 24 companies provided views on initial proposal 2: the large majority selected Option 1 as first or second preference: [OPPO, Samsung, Nokia (a mixture of option 1&2), Intel, Apple, Intelsat, Sony, Xiaomi, CMCC, LG, ZTE, APT, NTT DOCOMO, INC., Huawei, HiSilicon, Sequans, CEWiT, IITM, IITH, Reliance Jio, Tejas Networks].

Option 3 was selected as first or second preference by : [Thales, MediaTek, APT, Intel, Apple, Lenovo/MM, CEWiT, IITM, IITH, Reliance Jio, Tejas Networks]

[Ericsson] support broadcasting a drift rate, further study needed for drift variation rate indication.

[QC] don’t see the need of signalling of common TA drift rate. No TA drift signalling should also be one of the option

For [Panasonic] it is premature to decide for adjusting the timing advance in any way at this meeting.

[CATT] need more time to discuss its benefit and its usage.

[Fraunhofer] support the signaling of the assistance information but details of the signaling can be FFS.

Unacceptable options are: Option 3 for [Nokia, Sony, NTT DOCOMO, INC.] because of additional overhead [Nokia, NTT DOCOMO, INC.]. Option 1: [MediaTek] because it is not sufficiently accurate and would require UE to read SIB carrying common TA every second. Option 2 [APT] because it is unclear how to define the window size.

In Moderator’s view: RAN1 needs to specify which assistance information is needed/to be indicated: Common TA drift rate and/or Common TA drift  variation rate. At least Common TA drift rate is needed. How the gNB will calculate the Common TA drift rate can be left to the implementation. But, the accuracy of Common TA calculated by the UE and required timer period for new SIB acquisition will depend  on the provided assistance information.

Based on expressed views, it is clear that the large majority is supportive of Common TA drift broadcast. The indication of Common TA drift variation rate can be left FFS. Thereby, the updated proposal is as follows:

Updated proposal 2:

If Common TA is to be considered in UE autonomous TA calculation, the Network may periodically broadcast:

- Common delay NTA,common

- Common delay drift rate

- FFS: Common delay drift rate variation

- Companies are encouraged to provide their comments and views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | We can remove “If Common TA is to be considered in UE autonomous TA calculation” |
| Apple | Agree with the proposal. |
| CATT | Agree |
| LG | We are generally fine with this proposal, but the UE decoding complexity could be increased when the indication period of broadcast channel is too short. Therefore, instead of indication of common TA drift rate, it may be desirable that the network may provide series of common TAs using appropriate indication period. So, we suggest following modified proposal. Modified proposal 2: If Common TA is to be considered in UE autonomous TA calculation, t The Network may periodically broadcast: - Common TA, delay NTA,common - FFS: Whether to support Common TA delay drift rate or series of common TAs - FFS: Common TA delay drift rate variation - FFS: Implicit/Explicit indication of reference time corresponding to common TA |
| CMCC | We support this proposal. |
| Sony | We share the Samsung’s view. |
| Spreadtrum | We support this proposal. |
| Panasonic | We think it is premature to adopt a timing drift rate. It requires more study. Therefore, we propose to remove common delay drift rate from the list as follows: Updated proposal 2: If Common TA is to be considered in UE autonomous TA calculation, the Network may periodically broadcast: - Common delay NTA,common - FFS: Common delay drift rate and common delay drift rate variation |
| Nokia, Nokia Shanghai Bell | We agree with LG and Panasonic and prefer that the delay drift rate is kept as FFS. |
| Huawei, HiSilicon | We support the proposal. |
| Ericsson | - We agree with Samsung that “If Common TA is to be considered in UE autonomous TA calculation” can be removed. If the network broadcasts the parameter(s), the Common TA shall be considered. This has already been agreed at RAN1#103-e (although it was called “X” then). - If Updated proposal 1 is agreed, NTA,common is the common TA to apply by the UE. This should be derived from the broadcast parameters but is not necessarily the broadcast parameter. In Updated proposal2, the parameter name should be removed to avoid confusion. |
| Fraunhofer IIS, Fraunhofer HHI | We agree with LG and Panasonic. In our perspective, at this stage, it is important to keep the investigation regarding the signaling of the common delay open to, generally, assistance information not limited to drift rate, and drift rate variation only. We propose the following: Updated proposal 2: If Common TA is to be considered in UE autonomous TA calculation, the Network may periodically broadcast: - Common delay NTA,common FFS: Signaling of assistance information for UE autonomous NTA,common calculation, e.g. Common delay drift rate, common delay drift rate variation. |
| ZTE | We support this proposal in general with following updates： - Remove: If Common TA is to be considered in UE autonomous TA calculation, And we also prefer to keep to parameters name is the 1st sub-bullet |
| DOCOMO | We support this proposal in general. However, before further discussion on the details of the proposal, we think that the difference between the wording of “delay” and “TA” as well as “timing” should be firstly clarified. Besides, we agree with LG’s view on broadcasting multiple common TAs. As analyzed in our tdoc (R1-2103579), UEs may have different capabilities on TA self-estimation/pre-compensation. For example, some UEs can calculate the TA with considering common TA drift rate, while some UEs cannot. For UEs with different capabilities, multiple common TAs can be broadcasted. |

## Updated proposal based on company views (2nd round of email discussions)

Based on the views expressed during the second round of emails discussions and through the reflector, the Proposal 2 is updated as follows:

Updated proposal 2:

The Network may periodically broadcast:

- Common TA

- Common TA drift rate

  - Common TA draft rate with value of 0 is supported

- FFS: Common TA drift variation rate

# Issue#3: Granularity and signalling of Common TA

The discussion about the granularity and signalling of Common TA was already started in RAN1#104-e. This issue is also discussed in many contributions submitted to RAN1#104-bis-e.

Granularity of Common TA needs to be clarified because it is related to the signalling overhead introduced by Common TA indication but more importantly to the accuracy of Common TA. Discussion on the accuracy of Common TA should consider the quantization error which depends on the granularity of common TA.

Let’s first discuss the granularity of Common TA:

- Granularity of Common TA

These are the proposals on Common TA granularity submitted to RAN1#104-bis-e:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: The granularity of common timing TA is set to be the same as the granularity of NTA, i. e. 16∙64/2μ*Tc |
| ZTE | Proposal 2: The ignalling granularity of common TA can be chosen as. |
| Lenovo, Motorola Mobility | Proposal 1: Support the granularity of NTA,common same as granularity of TA command. |
| Fraunhofer IIS, Fraunhofer HHI | Proposal 2: The characteristic of the common RTT/delay (or feeder link RTT/delay) shall be taken into account for the design of its corresponding ignalling. |
| Spreadtrum Communications | Proposal 1: N_(TA,common) should be expressed in the legacy granularity of Tc units. |
| CATT | Proposal 4: The granularity of common TA can apply 1024 Tc or 2048Tc of step size. |
| Xiaomi | Proposal 1: With consideration on the signalling overhead, the large granularity of NTA,common i.e. 2048 Tc is preferred. |
| Apple | Proposal 2: The broadcast common timing offset is in the unit of Tc⋅(16⋅64/2μ). |
| Qualcomm Incorporated | Proposal 1: - In NTN, the network may broadcast a common timing offset value with granularity of one slot assuming SSB subcarrier spacing. - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: TA= NTA+NTA, offset×Tc+X where X is derived from the common timing offset. |

- Signalling of Common TA:

In current meeting we can have some preliminary discussions on  the signalling design of Common TA. But a final agreement on this topic will depend on the outcome of the discussions  on Issue#2.

These are the proposals related to Common TA signalling submitted to RAN1#104-bis-e:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 3: If Common TA shall be considered in UE autonomous TA calculation, the Network shall periodically broadcast: - in case of GEO based non-terrestrial access network: - Common TA (NTA,common ) in a field of 23 bits - in case of LEO/MEO based non-terrestrial access network: - Common TA (NTA,common ) in a field of 19 bits - Common TA drift rate( NCommondrift) in a field of 13 bits - FFS: Common TA drift rate variation (NCommondrift_variation_rate ) in a field of 6 bits |
| PANASONIC R&D Center Germany | Proposal 2: The common TA is always indicated in SIB. Value 0 for NTA,common should be supported. |
| MediaTek Inc. | Proposal 2: In case reference point for UL-DL subframe timing alignment is gNB, broadcast on NTN SIB - Common delay NTA,common in a 18 bit field - Common delay drift rate NTA,common,drift, rate in a 8 bit field - Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field |
| ZTE | Proposal 1: The indication of common TA should always be assumed. |
| Samsung | Proposal 5: A gNB signals residual common TA value to Ues such that Ues can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information. |
| Sony | Proposal 3: RAN1 should support the signalling of timing drift rate information to the Ues in a beam specific manner. |
| LG Electronics | Proposal 5. Within pre-defined set of TA offsets, the TA offset can be provided by gNB via higher layer signing (e.g., SIB or dedicated RRC signaling). - The TA offset can be independently corresponding to different Ros (or RO groups) Proposal 10. - At least for the case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the additional information via semi-static signaling. - In case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling. |
| Fraunhofer IIS, Fraunhofer HHI | Proposal 1: RAN1 to strive for a unified design of common RTT/delay signaling in order to avoid duplicate signaling of common RTT/delay. Proposal 2: The characteristic of the common RTT/delay (or feeder link RTT/delay) shall be taken into account for the design of its corresponding ignalling. Proposal 3: RAN 1 should agree on studying common delay ignalling design with reduced ignalling overhead. |
| CEWiT, IITM, IITH | Proposal 2: gNB broadcast the common TA value in the NTN specific SIB message along with other NTN specific broadcast messages. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 2 Support the GW location pre-stored in u-sim to prevent the common TA signalling, if there is no security concern. |

## Company views

Regarding the granularity:

[Spreadtrum Communications] proposed  the unit is Tc.

[Lenovo, Motorola Mobility]: same granularity as of TA command.

[Xiaomi] proposed the large granularity of N_(TA,common) i.e. 2048 T_c is preferred (With consideration on the signalling overhead)

[CATT] proposed 1024 Tc or 2048Tc of step size.

[Huawei ] observed, if the granularity of common TA is larger than the granularity of TAC, a larger quantization error will be introduced. Therefore, it seems reasonable to select the same granularity of N_TA for common TA. The granularity of common timing TA is set to be the same as the granularity of N_TA,

[Huawei, Apple]	proposed same as the granularity of N_TA, i. e. 16∙64/2^μ*Tc

[Qualcomm] proposed for a common timing offset value a granularity of one slot assuming SSB subcarrier spacing.

[Moderator’s view]: the granularity (time resolution) should be proportional to the subcarrier spacing as for N_TA:  .Tc. Thus, the finer time resolution is given by SCS = 120kHz subcarrier spacing; that is Tc.

Based on the views expressed regarding the granularity, the following Initial Proposal is made:

Initial Proposal 3-1:

The granularity of Common TA is set to be the same as the granularity of NTA, i. e. 16∙64/2μ*Tc

- Companies are encouraged to provide their comments and views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree |
| Thales | Agree |
| Nokia, Nokia Shanghai Bell | Ok with FL proposal to have similar granularity for indicating the N_TA. |
| Intel | OK |
| Apple | We support the proposal. The granularity of common TA is the same as TA command saves the ignalling overhead, comparing with the granularity of Tc. |
| Intelsat | Agree |
| Ericsson | We propose a somewhat finer granularity, e.g., 4∙64/2μ∙Tc. A granularity of 16∙64/2μ∙Tc corresponds to 11% of the CP for PUSCH/PUCCH. The total UL timing error is the sum of many components, including DL sync inaccuracy, UE-specific TA inaccuracy and granularity, closed-loop TA granularity, common TA linear (or 2nd order) approximation and granularity. Finer granularity of common TA allows more inaccuracy to be “spent” on the more challenging UE-specific TA calculation. |
| Sony | We agree with this proposal. |
| QC | We don’t see the need of such a fine granularity. The question should be answered after decision of ignalling of common TA drift rate. |
| LG | Agree |
| APT | Support Initial Proposal 3-1. |
| Huawei, HiSilicon | Agree |
| Fraunhofer IIS, Fraunhofer HHI | We support the proposal. |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | Support the proposal. |

Regarding the signalling of Common TA:

Based on proposals from different contributions:

Initial Proposal 3-2:

If Common TA shall be considered in UE autonomous TA calculation, the Network shall periodically broadcast:

- Common delay NTA,common in a 18 bit field

- Common delay drift rate NTA,common,drift, rate in a 8 bit field

- FFS: Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field

- Companies are encouraged to provide their comments and views:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree. We can delete FFS, since drift rate is enough to indicate. |
| Thales | We think that in case of LEO/MEO based non-terrestrial access network: - Common TA (NTA,common ) in a field of 19 bits - Common TA drift rate( NCommondrift) in a field of 13 bits - FFS: Common TA drift rate variation (NCommondrift_variation_rate ) in a field of 6 bits To reduce the overhead, some optimization maybe needed. E.g. indicate only relative values. |
| OPPO | RAN1 only needs to discuss the common TA granularity and RP position, the number of bits may be decided later. |
| Nokia, Nokia Shanghai Bell | The starting point here should be focused on the needed span of the different parameters rather than the bit field size. According to our understanding the needed time span for these parameters would be depending on the physical propagation time, and hence independent on the subcarrier spacing. Therefore we would need to define the bit fields according to highest subcarrier spacing. This discussion applies to both N_TA, common and N_TA, common, drift_rate. The FFS should by default be excluded as there has not been any agreement on drift rate variation under initial proposal 2. |
| MediaTek | Support moderator proposal |
| Intel | We are fine with the proposal |
| Apple | Before we agree on the bit field sizes, it is better to understand how these detailed numbers are derived. For example, what is the granularity of common delay drift rate, what is the value range of common delay and common delay drift rate, etc. |
| Intelsat | Agree. Further agreement on the details may be needed. |
| Ericsson | We propose to remove the field lengths. |
| Sony | We support to indicate the common TA drift rate with common TA. On the other hand, this proposal is related to Proposal 2. So, we should agree the proposal 2 at first. |
| Xiaomi | We propose to remove the field lengths. |
| CMCC | Same view with OPPO, Apple and Ericsson. The number of bits may be decided later. |
| LG | It is desirable to discuss the initial proposal 2 first and then discuss this issue later. Moreover, the bit field size of each parameter should be further discussed. |
| ZTE | The exact value can be decided later. |
| APT | Support Initial Proposal 3-2. However, MTK’s proposal, i.e., 18, 8, and 6 bits, that is not aligned with Thales’ proposals. It may be too early to the bit field sizes. |
| CATT | This proposal is not urgent. We need to firstly reach the consensus on the need of common TA and its granularity. |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Agree with other companies that the value range for common TA should be decided first and the bit field size can be determined later when the granularity is determined. Our view on the value of common TA is that the common delay NTA,common should under the gNB control which means that the RP for NTA,common can be located at service link or feeder link. The field of NTA,common should take the extra 1 bite for negative values of NTA,common. |
| Panasonic | If the network shall periodically broadcast the value, then before taking the decision, RAN1 needs to conclude how often a UE shall attempt to read the SIB. In our understanding, the UE is not required to read the SIB frequently. Based on TS 38.214 Section 5.1, for simultaneous operation with SIB and user traffic, it needs to be concluded whether it’s a P-RNTI triggered SI acquisition or an autonomous SI acquisition. |
| Lenovo/MM | Suggest to further discuss this proposal once the granularity is decided. |
| Fraunhofer IIS, Fraunhofer HHI | Agree with other companies. The value range for common TA should be determined first before discussing the bit field size. |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | Would like echo the other companies’ view that first range of value should be decided then bit field size can be derived. |

## Updated proposal based on company views (First round of email discussions)

[Samsung, Thales, Nokia, Nokia Shanghai Bell, Intel, Apple, Intelsat, Sony, LG, APT, Huawei, HiSilicon, Fraunhofer IIS, Fraunhofer HHI, CEWiT, IITM, IITH, Reliance Jio, Tejas Networks] agree with proposal to use similar granularity for Common TA as the one N_TA. (i. e. 16∙64/2μ*Tc)

[QC] don’t see the need of such a fine granularity and prefer to discuss the granularity of common TA  after decision of signaling of common TA drift rate.

[Ericsson] propose a  finer granularity, e.g., 4∙64/2^μ∙T_c.

w.r.t to Initial Proposal 3-2, several companies prefer to discuss issue#2 and Common TA granularity first.

Based on the above, this FL recommendation is made:

FL recommendation 3:

On the signaling and the granularity of Common TA ( NTA,common ) companies are encouraged to conduct more investigations and provide inputs to RAN1#105-e.

- Companies are encouraged to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Ok |
| Apple | Fine with the recommendation. |
| CATT | OK |
| LG | OK |
| CMCC | Fine with the recommendation. |
| Sony | Support FL recommendation. |
| Spreadtrum | We support this proposal. |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | OK |
| Huawei. HiSilicon | Fine |
| Ericsson | It could be clarified that input is needed both for the signaling and granularity of the broadcast parameters used to derive the Common TA, and the granularity of the derived TA value (NTA,common). |
| Fraunhofer IIS, Fraunhofer HHI | Agree |
| ZTE | Support, We should focus on the granularity and value range to address potential all cases. |

## Updated proposal based on company views (2nd round of email discussions)

# Issue#4: The need and the indication of TA margin

The need of TA-margin was discussed in RAN1#103-e and RAN1#104-e meetings and it seems needed depending on requirements for UE-autonomous TA error, PRACH preamble format, common TA error, and common timing drift rate error.

In Moderator ‘s view:

- TA-margin is needed to account for TA estimation uncertainty when applying the TA pre-compensation in initial access: During the first acquisition of its UE-specific TA, the UE can overestimate the TA. In case of overestimation, the PRACH preamble will be received at gNB side in advance w.r.t. the PRACH occasion leading to unwanted interference with previous slot or PRACH occasion.

- If initial TA does not include a margin for maximum TA estimation error. A bipolar TA command is needed in msg2. i.e support negative TA adjusting in RAR. It is preferable to avoid such specification impact

W.r.t TA margin indication, there mainly two different views:

- including TA-margin within the Common TA. i.e.; Common TA configuration absorbs the maximum TA uncertainty.

- TA-margin indication in the SIB; for at least two main reasons:  In case when the RP is located at satellite, the Common TA will be zero and it may not be necessary to provide the Common TA by gNB. Further, the need of TA_margin is only relevant in case of TA acquisition for PRACH msg1/mgsA transmission.  New value of Common TA acquired by the UE in connected state should not include TA_margin.

The companies proposals on the need and indication of TA margin are collected in the following table:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: The TA margin for the maximum estimation error is included in common TA offset. |
| THALES | Proposal 4: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements |
| OPPO | Proposal 3: NTA,offset should factor in TA margin. |
| Spreadtrum Communications | Proposal 3: TA margin is indicated in SIB should be supported. |
| PANASONIC R&D Center Germany | Proposal 3: We don’t see a need for the UE to handle a TA margin. As a value transparent to the UE, the TA margin can be included in the Common TA by the network. Setting of the TA margin (and any other margin) is up to the network implementation. |
| CATT | Proposal 6: TA margin can be one fixed value or not needed at all if UL timing requirement of RRC-IDLE mode and RRC-connected mode for UE is same. |
| MediaTek Inc. | Proposal 3: TA margin can be included in scope of discussions for timing synchronization requirements in RAN4. |
| China Telecom | Proposal 1: The TA margin shall be included in the common TA. |
| CMCC | Proposal 2: Indication of parameter A (a time-invariant offset of NTA,common, base) should be supported. - For DL and UL aligned at satellite, A can be used to absorb TA margin. - For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link. Proposal 4: TA margin should be absorbed in common TA configuration, and it should be transparent to the UE. |
| ZTE | Proposal 4: Postpone the discussion on TA margin until RAN4 determines time synchronization requirements. Proposal 5: If TA margin is needed, it should be absorbed in common TA to save signaling. |
| Apple | Proposal 5: RAN1 to determine the necessity of specifying TA margin, depending on RAN4’s conclusion. |
| Lenovo, Motorola Mobility | Proposal 2: TA margin indication is not supported. |
| NTT DOCOMO, INC. | Proposal 2: Common TA margin and user-specific TA margin should be considered separately. Proposal 3: Common TA margin can be included in the common TA. The indication of common TA margin is not necessary. |
| Nokia, Nokia Shanghai Bell | Proposal 4: The cyclic prefix of the random access preamble must be able to cover the aggregate contribution of all sources of time inaccuracy and multipath propagation delays. Proposal 7: There is no need to indicate a TA margin. Any uncertainty related to TA should be covered by the common TA value and CP of random access preamble. |
| Sequans Communications | Proposal 2: If TA margin is introduced in NR NTN, its indication should be absorbed within common TA signalling. |
| CEWiT, IITM, IITH | Proposal 4: TA margin should be configured to the UE directly or indirectly to control the uncertainty in the full TA estimation at the UE. Full TA equation including TA margin will be TA=NTA+NTA,UE-specific+NTA,common+NTA,offset-NTA, margin ×Tc. Proposal 5: TA margin can be configured indirectly as faction or multiple of the CP of the configured PRACH. NTA, margin=Y* CPPRACH. FFS Y value. |

## Company views

Based on the view expressed in RAN1#104-bis-e contributions, it seems reasonable to postpone the discussion on TA margin until RAN4 determines time synchronization requirements:

- Initial proposal 4:

RAN1 to discuss the need and the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements

- Companies are encouraged to provide their comments and views:

| Companies | Comments and Views |
| --- | --- |
| Samsung | TA Margin can be considered as one of the requirement. So, RAN4 discuss and conclude that topic. |
| Thales | Ok |
| OPPO | RAN1 could discuss if the TA margin is needed, where to indicate this margin. This is related to the FL proposal 1. |
| Nokia, Nokia Shanghai Bell | As a starting point we are ok to wait for the RAN4 response to the LS on timing synchronization requirements. However, we would still like to emphasize that we do not see any need for providing the UE with any TA margin. It is the UE’s responsibility to comply with the timing requirements such that the random access preambles are received with proper time alignment at the gNB. |
| MediaTek | Support Moderator proposal |
| Intel | OK |
| Apple | We support the proposal. We could wait for RAN4’s conclusion before further discussions in RAN1. In our view, RAN1 may not be necessary to specify TA margin if it is handled in RAN4. |
| Intelsat | Agree |
| Ericsson | We are fine with the proposal. |
| Sony | Support. |
| Xiaomi | Ok |
| CMCC | We are fine with the proposal. |
| QC | Agree |
| LG | Agree |
| ZTE | Support with this proposal. But the if needed, this value should be considered within common TA indication. |
| APT | Support Initial proposal 4. TA margin can be handled by NW via 1) providing common TA; 2) shifting preamble reception window, or by UE via 1) shifting UL timing. The intention is to reuse TAC in RAR. |
| NTT DOCOMO, INC. | We support this proposal. However, we would like to clarify that even if timing synchronization requirements can be roughly satisfied without considering common TA margin, better timing synchronization and performance can be achieved if considered. Therefore, common TA margin can be included in the common TA for better performance. |
| CATT | Agree. But in any case, we don’t think this margin should be indicated by the gNB. |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Ok this the proposal. When TA margin is needed, we propose to contain it in the common TA as it can save some signalling overhead. In addition, TA margin is also a kind of common TA to all the UE, when RP related common TA is zero, the indicated common TA is equal to TA margin, there is no for UE to differentiate whether RP related common TA is zero or not. |
| Panasonic | Agreed. |
| Lenovo/MM | Agree with moderator’s proposal. |
| Fraunhofer IIS, Fraunhofer HHI | Agree with the proposal. We think TA margin can be absorbed in N_TA,common. |
| Sequans | Agree |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | Ok |

## Updated proposal based on company views (First round of email discussions)

25 companies provided views during  the first round of email discussions. All are supportive of the proposal.

Based on the expressed views, the following conclusion is made:

FL Recommendation 4:

RAN1 to discuss the need and the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#5:	TA command in RAR

In RAN1#103-e meeting, the following working assumption was made on TA command in RAR

Working assumption:

It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

The intention of the working assumption on TA command in RAR made in RAN1#103 is to have as design target  not extending  existing TAC 12-bit field in msg2 (or msgB). Basically TA command in RAR will be used by gNB to indicate the residual error made on UE-specific TA, if the accuracy requirements of time pre-compensation are appropriately set there will be no need to extend TAC field in Msg2. As consequence, with this working assumption the discussion about the bit size of the TAC field in msg2 (or msgB) is postponed.

There some proposals in the contributions submitted to RAN1#104-bis-e to confirm this working assumption:

| Companies | Proposals |
| --- | --- |
| Spreadtrum Communications | Proposal 4: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. |
| CATT | Proposal 7: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. |
| CMCC | Proposal 5: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. |
| Apple | Proposal 6: The requirement that the existing TAC 12-bit field in Msg2/MsgB is reused is that a UE pre-compensates an accurate UE specific TA and TA margin in its Msg1/MsgA transmission. |
| LG Electronics | Proposal 9. Regarding TA command in RAR, support enhancement approaches to cover large cell coverage. - Increase the step size of TA command field in RAR. - Support multiple reference points. |
| CEWiT, IITM, IITH | Proposal 6: Agree the working assumption on TAC 12-bit field reuse. |

## Company views

Based on the view expressed in RAN1#104-e and in the contributions to RAN1#104-bis-e, the reasonable way forward  is to postpone the confirmation of this working assumption. We will come back on this topic when the requirement on UL time pre-compensation for Msg1/MsgA transmission are defined.

- FL recommendation 5:

Postpone the confirmation of the working assumption on non-extension of  TAC 12-bit field in msg2 (or msgB) until the requirement on UL time pre-compensation for Msg1/MsgA transmission is defined

- Companies are encouraged to provide the comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Ok. RAN1 can discuss based on WA. |
| Thales | Agree |
| Nokia, Nokia Shanghai Bell | Ok to wait for the RAN4 response to the LS on timing synchronization requirements before confirming the Working Assumption. |
| MediaTek | Support FL recommendation |
| Intel | OK |
| Apple | Fine to confirm the working assumption after the requirement on UL time pre-compensation for Msg1/MsgA transmission is defined. |
| Intelsat | Agree |
| Ericsson | We are fine with the FL recommendation. |
| Sony | Support. |
| Xiaomi | Agree |
| CMCC | We are fine with the FL recommendation. |
| QC | Agree |
| LG | Agree |
| ZTE | Agree |
| ATP | Support FL recommendation 5. Prefer to reuse TAC in RAR to minimize spec impact. |
| CATT | Agree |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Agree. |
| Panasonic | Agreed. |
| Lenovo/MM | Agree with moderator’s proposal. |
| Fraunhofer IIS, Fraunhofer HHI | Agree |
| Sequans | Agree |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | Agree |

## Updated proposal based on company views (First round of email discussions)

After the first round of email discussion, the following FL recommendation is made:

- FL recommendation 5:

Postpone the confirmation of the working assumption on non-extension of  TAC 12-bit field in msg2 (or msgB) until the requirement on UL time pre-compensation for Msg1/MsgA transmission is defined

# Issue#6 : TA update in RRC_CONNECTED state

In RAN1#104-e, the following agreement was made:

Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control Agreement: For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. FFS: Details of the combination of open and closed loop TA control

In current meeting we need to start the discussions on the operation of closed loop and open loop TA control.

If we consider Timing formula discussed in Issue#1:
TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

For TA maintenance, the UE needs to update NTA based on closed loop and (NTA,UE-specific+NTA,common) based on open loop mechanism.

w.r.t TA update based on closed loop: some preliminary discussions were started in RAN1#104-e, It seemed that  several companies are with reusing existing  NTA update in RRC_CONNECTED based on TA Command  field in msg2/msgB and MAC CE TA command without specification change.

However, the support of negative TA Command in RAR should be further investigated if TA-margin is not supported.

As in current specification, closed loop can dynamically controls the component  NTA as follows:

- When TAC (TA) in msg2/msgB is received,  UE receives the first adjustment and NTA is updated as follows:

NTA= TA× 16×64

where, TA is the TAC field in msg2/msgB

- When TACs (TA) provided within the MAC CE is received, NTA is updated as follows:

NTA_new=NTA_old+TA-31.16.642μ

where, TA is the TAC field receivd in MAC CE  command

w.r.t TA update based on open loop: The necessity of  open loop TA update was already discussed in previous RAN1 meetings and it is linked to high RTT drift on service link and feeder link especially in case of NGEO scenario. Using closed loop only is not practicable. With TA update based on open loop, the UE can autonomously track the RTD variation on both service link and feeder link to keep the residual timing error within a maximum tolerable range that could be absorbed by the cyclic prefix.

The main issue to be solved now is the combination of open and closed loop TA control

The following proposals on TA update in RRC-Connected state were provided by the different companies:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 5: Existing NTA update in RRC_CONNECTED based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change - FFS: Whether to support negative TA Command in RAR Proposal 6: NTN UE in RRC_CONNECTED shall have an active BWP with common search space configured to monitor system information. |
| OPPO | Proposal 4: TA updating by MAC-CE or RAR can have the following relationship NTA_new = NTA_old + adjustment, where for TA updated by MAC-CE, the adjustment is and TA is the TAC indication in the range of [0, 63]; for TA updated by RAR, the adjustment is and the TA is the RAR TAC indication in the range of [0, 3846]. |
| CAICT | Proposal3: FFS the condition when to apply the TA through combined closed loop and open loop. |
| CATT | Proposal 8: UE can stop autonomous TA compensation or subtract the accumulated TA compensated by autonomous TA compensation when combining the close-loop TA compensation. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 3 For RP at gNB, disabling the open-loop TA control shall be supported, and as a result, the UL timing can be maintained by the closed-loop TA control with 6-bit TA commands and TA drift rates. Proposal 8 For RP at a satellite, disabling the open-loop TA control shall be supported, and as a result, the UL timing can be maintained by the closed-loop TA control with 6-bit TA commands and TA drift rates. |
| China Telecom | Proposal 2: NTA and NTA,common are updated together by a combined TA command based on closed loop updating. Proposal 3: NTA,UE-specific is updated by adding the newly estimated offset directly: NTA,UE-specificnew= NTA,UE-specificold+ΔNTA, UE specific |
| CMCC | Proposal 6: Enhanced TA command based TA determination mechanism for UE in RRC_CONNECTED should be supported, where, the enhanced TA command consists of (TA,δ,TA,drift, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. Proposal 7: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as NTAti=NTAaccumm+TA,driftm∙ti-tm' where, - m is the last received enhanced TA command before ti. - NTAaccumm is the m-th accumulated TA command. - Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0 |
| ZTE | Proposal 6: In RRC_CONNECTED state, if the expression of applied TA determined as: , where - , is estimated by UE based on GNSS and indicated information. - , is the common TA drift rate indicated by BS, and is the interval between the time when is valid and current time. - , is indicated in MAC CE TA command. |
| Xiaomi | Proposal 3: NTA is controlled by RAR and TAC in MAC-CE as in Release-16. |
| Mitsubishi Electric RCE | Proposal 3: Support network assistance indicating to the UE whether to skip timing advance acquisition during handover. |
| Ericsson | Proposal 4 The UE calculates the UE-specific TA (in Tc units) as follows: NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the signal the UE is about to transmit Proposal 6 The UE determines the common TA (in Tc units) as follows: NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to be transmitted by the UE will be relayed by the satellite. |
| Apple | Proposal 7: In RRC connected mode, UE updates its TA value using TA=NTA+N'TA, UE-specific+N'TA, common+NTA,offset×Tc, where - NTA=NTA,old+TA-31⋅16⋅64⋅2-μ is based on the updated TA command MAC CE - N'TA, UE-specific is based on the updated GNSS positioning signal or satellite ephemeris - N'TA, common=NTA,common, RX +DTA, common,RX ⋅Δt is based on the updated common TA (NTA,common, RX ) and common TA drift rate (DTA, common,RX ). |
| Qualcomm Incorporated | Proposal 2 : Send an LS to RAN4 to inform RAN1’s decision on timing control and to enquire details of combination of open and closed-loop TA |
| Sony | Proposal 1: In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs. Proposal 4: UE should update the common TA with common timing drift rate when UE transmits uplink data. |
| LG Electronics | Proposal 11. RAN1 should discuss how to update and/or report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states. |
| Nokia, Nokia Shanghai Bell | Proposal 5: The GNSS-assisted pre-compensation solution used by the UE shall meet the demands of the preamble format chosen by the operator. The UE shall ensure that requirements in TA adjustment and frequency pre-compensation for all preamble formats are met at any time. Proposal 9: Network must be in control of the timing advance updates applied at the UE in RRC connected mode. Proposal 10: Closed-loop TA control in RRC connected mode is preferable to ensure stability of the TA control loop. Proposal 11: Open loop TA control in RRC connected mode should be applied only in a way that does not impact the closed loop TA control messages. Proposal 12: For RRC connected mode, self-estimated UE-specific TA estimation based on GNSS-provided time reference and the time provided by referenceTimeInfo-R16 is a beneficial solution and should be standardized as well |
| Sequans Communications | Proposal 3: GNSS-specific measurement gaps should be defined to accommodate various UE design options for NR NTN. |
| CEWiT, IITM, IITH | Proposal 7: gNB should provide the set of instructions to refine the TA estimated by the UE for better control of the gNB over UE specific TA estimation. Proposal 11: gNB can provide the TAmin value to the UE per beam/cell where TAmin is the minimum timing advance UE can experience in the beam/cell. Proposal 12: UE will correct the TA in connected mode using velocity information of satellite apart from the MAC-CE TA based update. Proposal 13: In connected mode, combination of open and close loop TA update should be adopted. New TA value update equation will be, TA=NTA,old ± ∆NTA+Ta-31.16.642μ )×Tc ×Tc . Proposal 14: The ∆NTA will be determined by UE using estimated drift value and additional drift provided by gNB. |

## Company views

Based on the contributions submitted to RAN1#104-b-e, some companies proposed to reuse existing Release-16 closed loop to update NTA: [THALES, Oppo, ZTE, Xiaomi, Apple]. Further [OPPO] proposed to support negative  TA Command in RAR.

The most complex topic is the combination of both closed loop and open loop. Some companies provided proposals, see proposals from [China Telecom, CMCC, ZTE, Ericsson, Apple, CEWiT, IITM, IITH].

[Asia Pacific Telecom] proposed to support disabling the open-loop TA control depending on the RP position.

[Sony] In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs.

For [Nokia] Network must be in control of the timing advance updates applied at the UE in RRC connected mode. And proposed that Open loop TA control in RRC connected mode should be applied only in a way that does not impact the closed loop TA control messages.

[MediaTek] observed if the common delay drift rate NTA,common,drift, rate and NTA,common,drift,rate, variation are broadcast the UE could autonomously predict and correct to common delay drift rate with an accuracy within 0.01 μs without need to receive a MAC CE TAC.

According to [Qualcomm] details of combination of open and closed-loop TA should be defined by RAN4.

[Moderator’s view] : In RRC_Connected state the NTN UE shall update ( NTA,UE-specific+NTA,common ) so that the cyclic prefix could absorb the propagation delay variation due to satellite movement . With Open loop TA update, the UE compensates the delay so that the residual delay can be absorbed by the CP in use. It looks like, as depicted in Figure below, the UE puts and maintains itself within 700m from reference point when using 15 kHz SCS or within 88m from reference point when using 120 kHz SCS.

Furthermore, it was observed via simulations, presented in  [2] that the UE could autonomously predict and correct the common delay within 0,15 µs provided that an updated Common TA related assistance information is available at the UE during the last 0.7 s when Common TA drift rate is indicated or during the last 5s when both Common TA drift rate  and Common drift variation rate are indicated. Moreover, the UE could autonomously predict and correct the delay on service link within a timing error range of 0.15 µs provided that an updated ephemeris data is available at the UE during the last 45 s. As a result, by updating NTA,UE-specific+NTA,common the UE could autonomously track the RTD variation on both service link and feeder link to keep the residual timing error within a maximum tolerable range that could be absorbed by the cyclic prefix when using 120kHz SCS.

Finally, before solving the  FFS on the details of the combination of open and closed loop TA control and possibly send an LS to RAN4 to inform RAN1’s decision on timing control and to enquire details of combination of open and closed-loop TA as proposed by [Qualcomm] . The Moderator’s view is to first discuss the operation of closed loop and open loop TA control separately.

The initial Proposal 6 is made as follows:

Initial Proposal 6:

In RRC_CONNECTED state:

- Existing NTA update based on TA Command  field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change.

  - FFS: Whether to support negative TA Command in RAR

- NTA,UE-specific is updated autonomously by the UE based on its GNSS acquired position and satellite ephemeris

- FFS: Update of NTA,common .

  - This FFS will be solved when the issue on Common drift rate indication is solved

Companies are encouraged to provide their comments on Initial Proposal 6:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree |
| Thales | We support the proposal |
| OPPO | Not agree. It is too early to conclude that the specification change is not needed. It may depend on how the UL TA correction is performed. To us, the TA used for PRACH transmission may need a negative or positive correction depending on whether and how a TA margin is used. Thus, at this stage, it is too early to conclude the specification change is none. |
| Nokia, Nokia Shanghai Bell | We have several issues related to this initial proposal 6: First, the FFS on supporting negative TA command in RA response should be taken out. In our opinion the UE should ensure that it transmits its random access preambles with such a time offset that it will guarantee to the gNB that there is no random access preambles arriving before “time=0”. Hence there will be no need for such support for negative TA command in RA response. Second, the second bullet should be rephrased such that it also accounts for the situation where the approach using ReferenceTimeInfo-r16 is taken into accountThird, this proposal does not take into account the aspect of having two adjustment loops, i.e. open and closed loop, potentially acting on top of each other. It is crucial that the UE is blocked from overcompensating any TA adjustments which would cause the UE UL transmissions to happen in such a way that UE time requirements are not fulfilled. |
| MediaTek | Support proposal with revision. We think this sub-bullets with FFS can be removed |
| Intel | Support the proposal |
| Apple | We support the proposal. |
| Intelsat | Support |
| Ericsson | Proposal 1 already seems to cover this. |
| Sony | We think update of NTA,common is needed in RRC_CONNECTED state. This can be resolved by common TA drift rate. If common TA drift rate is supported, the FFS should be removed. |
| Xiaomi | Support |
| CMCC | Partial agree. Regarding with 1st main bullet, legacy TA Command field in msg2/msgB and Enhanced MAC CE TA command which comprises of a delta TA TA,δ and a TA drift rate TA,drift is preferred to update NTA. Proposal 6: Enhanced TA command based TA determination mechanism for UE in RRC_CONNECTED should be supported, where, the enhanced TA command consists of (TA,δ,TA,drift, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. Proposal 7: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as NTAti=NTAaccumm+TA,driftm∙ti-tm' where, - m is the last received enhanced TA command before ti. - NTAaccumm is the m-th accumulated TA command. - Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0. Regarding with 1st sub bullet, negative TA Command in RAR is not preferred. TA margin can be absorbed in NTA,common. Regarding with 2nd main bullet, agree. Regarding with 3th main bullet, for an NR NTN UE in RRC_CONNECTED, there is no need to update NTA,common regularly, i.e., NTA,common=NTA,common0 and TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc where NTA,common0 is the common TA obtained in initial access procedure. Note that the system information NTA,common can either be periodically broadcasted on DL-SCL, or send in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. If NTA,common is only periodically broadcasted on DL-SCL, the UE in RRC_CONNECTED may miss-detect some of them, which resulting in requirement of very frequent signalling of TAC (TA) to track the rapidly changed RTT in the feeder link. Otherwise. to avoid the miss-detection issues, the gNB may always send NTA,common related system information in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. In this case, there are redundant closed loop TA control mechanisms, i.e., two UE dedicated signalling for TA adjusting, one for NTA,common determination, and one for NTA determination. Therefore, an enhanced MAC CE TA command within the MAC CE is suggested for a lean closed loop TA control mechanism. Option 1: Legacy MAC CE TA command; Option 2: Enhanced MAC CE TA command. TTA determination; TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc; TTA=NTA+NTA,UE-specific+NTA,common0+NTA,offset×Tc. Signalling overhead; TA in legacy MAC CE TA command, Common TA, Common TA drift rate; TA,δ,TA,drift in enhanced MAC CE TA command. |
| QC | OK |
| LG | Agree in principle, but we think it is reasonable to remove the first both sub-bullet and last main bullet for FFS points. |
| ZTE | Fine with this proposal in principle |
| APT | Support Initial Proposal 6. Remove FFS: Whether to support negative TA Command in RAR. |
| CATT | Agree |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Fine with the proposal. We do not see the need to support negative TA Command in RAR when TA margin is considered to ensure that the preamble will not arrived in advance. |
| Panasonic | Agreed. |
| Lenovo/MM | From our perspective, we think we should study the combination of close-loop and open-loop TA control before making decisions whether enhancement on close-loop TA is necessary or not, and on whether autonomously open-loop TA update can be enabled or not. Without combination of open-loop and close-loop TA, proposal 6 seems fine. However, when we consider combination of open-loop and close-loop TA, we think the contradiction between close-loop and open-loop TA should be solved, either by indication the time instance to determine TAC in MAC CE or by gNB control the open-loop TA control. So we suggest we discuss the issue related to combination of close-loop and open-loop TA firstly. |
| Fraunhofer IIS, Fraunhofer HHI | Agree |
| Sequans | Fine with the proposal |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | Agree on condition that FFS is removed. We believe negative TA in RAR is over specifying TA range whereas this can be managed by other parameters like NTA,UE-specific and NTA,common |

## Updated proposal based on company views (First round of email discussions)

Based on the expressed views during the first round of email discussions. It seems that companies need more time to investigate the combination of open and closed loop TA control.

The FL recommendation is made as follows:

FL recommendation 6:

Companies are encouraged to:

- Provide inputs to RAN1#105-e about the combination of open and closed loop TA control

- Companies are encouraged to provide the comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | We are discussing Updated Proposal 1. Isn’t it already cover this topic? |
| Apple | Fine with the proposal. |
| CATT | Agree it |
| LG | OK. We believe this issue could be discussed separately from the Updated proposal 1. |
| CMCC | Fine with the proposal. |
| Sony | Support. |
| Spreadtrum | We support this proposal. |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | Agree with LG that this is something that should be treated separately from updated proposal 1. |
| Huawei, HiSilicon | Fine |
| Ericsson | We are fine with the FL recommendation. |
| Fraunhofer IIS, Fraunhofer HHI | Agree |
| ZTE | Agree |

## Updated proposal based on company views (2nd round of email discussions)

# Issue#7 : NTN UE Time Alignment Timers

The legacy timeAlignmentTimer is used for closed loop TA update. Similarly specific timers may be needed for open loop TA update. The reasons for introducing such timers are the following:

For NTA,UE-specific update the UE needs to acquire satellite ephemeris and use its own propagator to predict its specific TA during a certain time period without the need of acquiring new ephemeris data. The newly acquired ephemeris data will be valid only  during a period depending on propagator model used by the UE and the maximum tolerable error on the estimation of NTA,UE-specific.

Similarly, for NTA,common update, the assistance information (Common TA, Common TA drift rate) acquired in the SIB will be valid only during a validity time period which depends on the maximum tolerable error on common TA estimation and the order of the Common TA approximation to be carried out by the UE. At the reception of new assistance information, the UE shall start/restart related timer. At expiry of this timer a new acquisition of Common TA estimation assistance information is required.

Regarding the Time Alignment Timers for NTN UE these proposals were submitted to RAN1#104-bis-e:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 7: A validity timer configured for UE specific TA defines the maximum time during which the UE can track the RTD on service link without having acquired new ephemeris data to be used for UE specific TA estimation. - This timer is restarted each time the UE receives new ephemeris data - The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris data is not available. Proposal 8: A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA estimation. - This timer is restarted each time the UE receives new assistance information. - The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available. |
| PANASONIC R&D Center Germany | Proposal 5: In RRC_CONNCTED mode and after expiration of the TA timer, a UE triggers the random-access procedure based on GNSS-acquired TA similar to RRC_IDLE with the same timing advance equation. |
| Sony | Proposal 2: RAN1 should consider indicating the time at which a closed loop TA was calculated to the UE. |
| LG Electronics | Proposal 13. Support validity timing window for satellite ephemeris information in Rel-17 NTN. |
| CEWiT, IITM, IITH | Proposal 3: In NTN, the network may broadcast a common timing drift rate and update duration to update the common TA by UE in periodic interval. It can be broadcasted in the NTN specific SIB. |

## Company views

Based on the above. RAN1 to discuss the necessity of the following proposal:

Initial proposal 7:

A validity timer configured for UE specific TA defines the maximum time during which the UE can track the RTD on service link without having acquired new ephemeris data to be used for UE specific TA estimation.

- This timer is restarted each time the UE receives new ephemeris data

- The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris data is not available.

A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA estimation.

- This timer is restarted each time the UE receives new assistance information.

- The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available.

Companies are encouraged to provide their comments  and views:

| Companies | Comments and Views |
| --- | --- |
| Samsung | We think this can be discussed in RAN2. |
| Thales | We think this should be discussed in RAN1. Indeed, the ranges to be defined for such timers depend on maximum tolerable error on UE specific RTD estimation, on maximum tolerable error on UE Common RTD estimation by the UE, on assistance information indicated for Common TA self-estimation, on propagator model used at the UE, on ephemeris accuracy, on the maximum error on the Common RTD estimated by the gNB (before being broadcast) and the error of quantization of assistance information. Some typical max ranges for both two timers are given in the flowing table: (to be revisited to take into account quantization errors) |
| OPPO | support |
| Nokia, Nokia Shanghai Bell | As both ephemeris data and Common TA is expected to be carried in SIB, we believe it would be enough to have “UE UL time synchronization” covered in a single validity timer. In our opinion it should be studied a bit more whether we need a new validity timer for this purpose, or if it is sufficient to simply reuse the TA timer (having RAN2 adding text on including ephemeris and/or reference time information from SIBs). |
| MediaTek | We are generally supportive of the introduction of a validity timer. We think discussion in RAN1 on validity timer is too early. In current understanding it is up to UE when to read ephemeris on the SIB. UE prediction of delay and Doppler shift is up to UE implementation. |
| Intel | We are open to consider the validity timer, however it may be early to make agreement to support it at this meeting. |
| Apple | Overall, we are fine with the principle of the proposal. The validity timer for UE specific TA may be discussed together with the ephemeris data format (i.e., issue #14). The validity timer for common TA may be discussed together with the common TA and common TA drift rate (i.e., issues #1 and #2). |
| Intelsat | Should be discussed in RAN 1. |
| Ericsson | We are fine with discussing this issue further. |
| Sony | Support |
| Xiaomi | We think this should be discussed in RAN2. |
| QC | Prefer to discuss the issue after the signaling of ephemeris, common TA, and requirements are clearer. |
| LG | Agree. We think it is beneficial to introduce separate validity timers for UE specific TA and common TA. |
| ZTE | This is fine to define a valid timer for common TA part since the should be take the responsibility to indicate the valid duration for the indicated value. For the UE specific TA value, no support to define the valid timer. The reason is that this value is calculated based on the GNSS information and satellite ephemeris, it’s preferred to directly define the valid time for these two components instead of final results. |
| APT | Open to discussion. We share Nokia’s view. The legacy TA timer might be enough. If there is no timer for UE-specific TA and Common TA, NW can still use TA commands to control UL timing. The existing TA timer, TimeAlignmentTimer, provides the sufficient parameters: ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, and infinity. If NW has a concern about UL interference, a smaller value, e.g., ms500 or ms750 can be used. UE shall stop UL transmission if no TAC is received within 500ms. Also, NW can trigger a PDCCH ordered RACH to ask a UE to rebuild UL timing if NW cannot receive any UL signal from the UE. |
| CATT | We are open to discuss it, but the details would be too early to make the decision. Actually we don’t know how to use this timer, or if it is really needed, or just up to implementation. |
| Huawei, HiSilicon | We are not convinced about the necessity of the validity timer. The periodicity of the common TA and satellite ephemeris are configurable by the gNB, which can satisfied the time/frequency synchronization accuracy requirement for the UE. |
| Panasonic | This can be decided in RAN2. |
| Lenovo/MM | We are fine to introduce these two timers for the UE-specific TA and common TA indication. In addition we want to mention that a timer should also be associated with the TAC in MAC CE. With these three timers, the UE can determine whether it is out of sync or in sync based on the state of these three timers. And we prefer that the timer associated with MAC CE is configured for each MAC CE command rather than common for all TACs in MAC CE to differentiate different satellite positions, beams, etc. |
| Fraunhofer IIS, Fraunhofer HHI | We believe this discussion can be postponed until further progress achieved regarding Issue#2 and Issue#3. |
| Sequans | Open to discuss the introduction of such validity timers |

## Updated proposal based on company views (First round of email discussions)

Based on the views expressed on this issue, companies are open to discuss the need of validly timer after the signalling of ephemeris, common TA, and requirements are clearer.

FL recommendation 7:

Companies are encouraged to:

- Provide inputs to RAN1#105-e about the need of the validity timers for Common TA and UE specific TA

- Companies are encouraged to provide the comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | This topic can be discussed in RAN2. |
| Apple | Fine with the FL recommendation. |
| CATT | Agree |
| LG | OK. We believe this issue could be discussed separately from the Updated proposal 1. |
| CMCC | Fine with the FL recommendation. |
| Sony | Support. |
| Spreadtrum | We support this proposal. |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | This is a topic that is better suited for RAN2, and could potentially be covered already under the existing TA timer. |
| Huawei, HiSilicon | Fine |
| Ericsson | We are fine with the FL recommendation. |
| Fraunhofer IIS, Fraunhofer HHI | Support |
| ZTE | Support and it should be noticed that the valid timer should be only applied for the configured value from gNB directly instead of UE-specific TA. |

## Updated proposal based on company views (2nd round of email discussions)

# Issue#8: TA reporting

TA reporting was already discussed in RAN1#104-e. The following FL recommendation was made:

Handle TA Reporting proposals under A.I. 8.4.1 or under A.I. 8.4.2 once sufficient progress has been made in A.I. 8.4.1.

Companies [MediaTek, Huawei, Xiaomi, ZTE, CATT, APT, Samsung, vivo, CMCC, LG, Panasonic, OPPO, Apple] provided views in RAN1#104-e. They were all supportive of FL recommendation.

Within the contributions to RAN1#104-b-e, proponents companies provided the following proposals:

| Companies | Proposals |
| --- | --- |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 4 Support UE autonomous TA adjustment if and only if NW can still know the absolute TA value. Proposal 5 Support TA report in RRC_CONNECTED to provide UE-gNB RTT to NW. Proposal 6 Support UE location report in RRC_CONNECTED to prevent frequent TA reporting, if there is no privacy concern. Proposal 7 If TA reporting in RRC_CONNECTED is supported, the following values shall be considered between consecutive TA reports: 1) the maximum amount of the magnitude of the timing change in one adjustment and 2) the minimum/maximum aggregate adjustment rate per second. |
| Mitsubishi Electric RCE | Proposal 1: UEs using self-acquired timing advance for initial access signal the pre-compensation value NTA to the network during the initial access process. |
| Samsung | Proposal 4: UE’s estimated TA value is reported to gNB. |
| LG Electronics | Proposal 6. Support implicit reporting of TA estimated by the UE. - The different TA (or the range of TA) can be mapped to different ROs (or RO groups). Proposal 11. RAN1 should discuss how to update and/or report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states. |
| CEWiT, IITM, IITH | Proposal 8: UE should report the applied TA to the gNB for better control over UE’s behaviour. Proposal 10: UE will report the applied TA to the gNB in terms of number of steps used in the quantization of TA. |
| InterDigital, Inc. | Proposal 2: a PRACH resource set is configured per distance group from the UE to satellite and a UE determine a PRACH resource set for Msg1 transmission based on the distance group the UE belongs to. |

## Company views

[Asia Pacific Telecom, Mitsubishi Electric RCE, Samsung, LG Electronics] proposed to support TA reporting to gNB.

The reasons provided are the following: [CEWiT] UE should report the applied TA to the gNB for better control over UE’s behaviour. [Asia Pacific Telecom] support UE autonomous TA adjustment if and only if NW can still know the absolute TA value.

Some methods for TA reporting were proposed:  [LG] proposed to support implicit reporting of TA estimated by the UE. and according to [CEWiT] UE will report the applied TA to the gNB in terms of number of steps used in the quantization of TA. [Asia Pacific Telecom] Support UE location report in RRC_CONNECTED to prevent frequent TA reporting, if there is no privacy concern. On UE-specific TA value indication, [InterDigital] proposed a PRACH resource set is configured per distance group from the UE to satellite and a UE determine a PRACH resource set for Msg1 transmission based on the distance group the UE belongs to.

[Moderator’s view]: TA reporting would be beneficial only for timing relationships, e.g if K_offset is updated UE-specifically. And therefore, such discussion should be handled under AI 8.4.1 on timing relationships. Once sufficient progress has been made on the update of  K_offset after initial access.

FL recommendation 8:

Handle TA Reporting proposals under A.I. 8.4.1 or under A.I. 8.4.2 once sufficient progress has been made in A.I. 8.4.1.

- Companies are encouraged to provide their comments on Moderator’s recommendation 8:

| Companies | Comments and Views |
| --- | --- |
| Samsung | In order for gNB to schedule with proper processing timeline, the TA information is needed. So, TA reporting is necessary with having TA_{UE,specific} value that the UE autonomously calculate. |
| Nokia, Nokia Shanghai Bell | OK to postpone discussions on this topic. |
| MediaTek | We are supportive of UE TA reporting. We can agree with FL recommendation. |
| Apple | We agree with the FL recommendation. The TA reporting is useful for gNB to update Koffset. We need to consider the signaling overhead of reporting TA. |
| Ericsson | We are fine with the FL recommendation. |
| Sony | Support. |
| Xiaomi | Ok |
| CMCC | We are supportive of UE TA reporting. We can agree with FL recommendation. In our view, UE may report its location or initial UE specific TA (NTA,UE-specific) via Msg A/Msg 3 in initial access procedure and report the delta value of changed TA between two reports via MAC CE after initial access procedure. |
| QC | We support TA reporting. But it is better to be handled in AI 8.4.1 |
| LG | Agree |
| ZTE | Supportive on TA reporting and fine to discuss it later once progress is made. |
| APT | Support FL recommendation 8. TA report is expected to provide more benefit: 1) share a common understanding about UE-gNB RTT between UE and gNB; 2) share rough UE location; 3) enhancement on DRX and RRC_CONNECTED mobility. |
| CATT | Agree |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Fine with the recommendation. |
| Panasonic | Agreed. |
| Lenovo/MM | Agree with moderator’s proposal. |
| Fraunhofer IIS, Fraunhofer HHI | Support FL recommendation. |
| Sequans | Agree |
| CEWiT, IITM, IITH, Reliance Jio, Tejas Networks | We support the reporting TA value to gNB which will provide gNB better control over UE behaviour. Agree with FL recommendation. |

## Updated proposal based on company views (First round of email discussions)

Based on first round of email discussions, the FL recommendation is as follows:

FL recommendation 8:

Handle TA Reporting proposals under A.I. 8.4.1 or under A.I. 8.4.2 once sufficient progress has been made in A.I. 8.4.1.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#9: Broadcasting the position of a reference point

Broadcasting the position of a reference point is an alternative solution that would simplify the time and frequency compensation mechanisms was proposed by [Ericsson, CMCC]. Such potential solution was already discussed in RAN1#104-e.

The principles of this UE centric pre-compensation solution is captured in the following observation made by [Ericsson]: If the position of a reference point of the feeder link and the UL and DL carrier frequencies of the feeder link are signalled to the UE, the UE can autonomously determine the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link, which would simplify the time and frequency compensation procedures.

[CMCC] observed that broadcasting the position of a reference point of the feeder link is beneficial to simplify the time compensation procedures and reduce signalling overhead for frequent update of N_(TA,common). And Security issue can be fixed by broadcasting the position of a reference point of the feeder link with certain artificial bias.

[CMCC] and [Ericsson] proposed to support broadcasting a reference point of the feeder link:

| Companies | Proposals |
| --- | --- |
| CMCC | Proposal 12: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. |
| Ericsson | Proposal 11 Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link. |

## Company views

According the views expressed in RAN1#104-e on this issue it seemed there is no need to broadcast the position of a reference point.

[Moderator’s view]: the main concern is how a reference point located on the feeder link can be characterized without indicating the position of the GW? It follows which orbit?  That is, any reference point cannot be indicated without indicating the GW position as well. But, the broadcast of the GW position was discussed since RAN1#102-e.  The NTN GW position may or may not be a critical information depending on the deployment scenario. Then, it is essential to support the cases where it is not acceptable to share this information with the users.

FL recommendation 9:

On support of broadcasting a reference point, proponents are encouraged to provide more details on the feasibility of such solution by taking into account companies’ concerns on this issue.

| Companies | Comments and Views |
| --- | --- |
| Samsung | Ok. Does this solution support flexible placement of the reference point (e.g. at the satellite, or even on the service link)? Also, what would be the update frequency of this reference point location (e.g. once per second)? |
| OPPO | If broadcasting of RP position is feasible, it is definitely beneficial for UE to handle the time and frequency synchronization compensation on the feeder link. |
| Nokia, Nokia Shanghai Bell | We can see the problem from both sides. Providing the reference point would allow the UE to perform calculations that could pre-compensate any Doppler and time variations as a function of time, as UE would have knowledge of all nodes positions in the system (with the accuracy of the ephemeris information being the main source of error). For such situations any support information for compensating for time and frequency variability in the feeder link would not anymore be needed. On the other hand, there is the concern related to disclosing the network architecture, which would be at the discretion of the network operator. Given that Rel-16 is already providing means for informing the UE of the TRP location, this could be something to consider for NTN. The relevant IE would be nr-TRP-LocationInfo in NR-PositionCalculationAssistance-r16. |
| MediaTek | Broadcast of reference point will allow UE to calculate TA and Doppler shift on feeder link in similar way as in the service link. He issue with this solution is whether satellite GateWay can be shared, and whether the delay over the GateWay – gNB interface can be known accurately in case these are not co-located. |
| Intel | In our view it the broadcasting of reference point is a good solution which we should consider. If where are concerns on disclosing gateway position common TA and common TA drift rate indication can be used (it is up to the gNB). |
| Apple | The reference point for time synchronization can be different from the reference point for frequency synchronization. Also, we do not see the necessity of broadcasting the reference point. For uplink time synchronization, we prefer to broadcast common TA and common TA drift rate. For uplink frequency synchronization, we prefer to set reference point at satellite directly to avoid any additional signaling. |
| Ericsson | Broadcast of a reference point would be a good solution for some deployment scenarios and should be further considered (in addition to the solution with broadcast of common TA, common TA drift, etc. To address the question raised by the Moderator above, “It follows which orbit?”, our view is that the reference point would be a fixed point in ECEF coordinates that could be the GW location of different from the GW location but close enough to make the residual time/frequency offsets manageable by the gNB. |
| Sony | At first, we should consider the reference point position. In our view, the reference point should be located in the access link which is shown as figure 6.3.4-1 in the TR 38.821. And, common TA compensate for time synchronization from gNB to reference point, then UE-specific TA compensates the time synchronization from reference point to UE. Figure 6.3.4-1 in TR38.821 |
| Xiaomi | Introducing common TA has been agreed for uplink time synchronization. We do not see the necessity of broadcasting the reference point. |
| CMCC | Broadcast of a reference point would be definitely beneficial for UE to handle the time and frequency synchronization compensation on the feeder link. Regarding to the concern related to disclosing the network architecture, it would be at the discretion of the network operator. Furthermore, the position of a reference point of the feeder link may be broadcasted with certain artificial bias to address the security issue. Regarding to the concern related to gNB location away from GW location, GW location (maybe with certain artificial bias) may be broadcasted in addition to the broadcast of fixed common TA, where the fixed common TA is used to compensate for the RTT of GW-to-gNB and possibly other latencies. |
| ZTE | There is no need to broadcast the location for the reference point, all needs can be addressed by singling of parameters. |
| APT | If GW location can be shared or pre-stored in u-sim, then no common TA, no common TA drift, no common TA timers, no more association between new TA timers and the existing TAG is needed. This could be feasible for some operators since we all know that STARLINK shares worldwide GW locations online. If GW location cannot be shared, we have a concern about whether UE can obtain UE-gNB RTT. If UE can have UE-gNB RTT, then UE may estimate GW locations via the multiple-RTT scheme. An agreement may be needed, otherwise, companies will keep sending the same proposals on this topic (at least from APT). Proposal 9 (APT): If gNB has no permission/authority to share GW locations, the GW locations cannot be known by UE. FFS: whether UE-gNB RTT cannot be known by UE if the GW locations cannot be known. |
| CATT | We think it is overlapped with other proposals, like as common TA indication. |
| Huawei, HiSilicon | To broadcast a reference point is an alternative to broadcast the common TA directly. When the UL and DL timing difference at the gNB is a constant value, we see some benefit to broadcast the RP as it will not change as quickly as common TA. |

## Updated proposal based on company views (First round of email discussions)

Based on the views expressed during the first round of email discussions, broadcasting a reference point might be beneficial  however the main concern with this solution is related to disclosing the network architecture, which would be at the discretion of the network operator.

The FL recommendation is the following:

FL recommendation 9:

On support of broadcasting a reference point, proponents are encouraged to provide more details on the feasibility of such solution by taking into account the concern related to disclosing the network architecture.

| Companies | Comments and Views |
| --- | --- |
| CMCC | Regarding to the concern related to disclosing the network architecture, it would be at the discretion of the network operator. Furthermore, the position of a reference point of the feeder link may be broadcasted with certain artificial bias to address the security issue. Regarding to the concern related to gNB location away from GW location, GW location (maybe with certain artificial bias) may be broadcasted in addition to the broadcast of fixed common TA, where the fixed common TA is used to compensate for the RTT of GW-to-gNB and possibly other latencies. |

## Updated proposal based on company views (2nd round of email discussions)

# Issue#10 : Common Doppler shift pre/post compensation on the feeder link

In RAN1#103e, it was agreed that an NR NTN UE shall be capable to perform frequency pre-compensation to counter shift the Doppler experienced on the service link. But there is not agreement yet on Doppler shift pre/post compensation on the feeder link. It is not yet clear if it has to be performed by the UE or the Network (gNB and/or GW/Satellite).

This issue is linked to the discussion on reference point for UL timing and frequency synchronization. Such discussion has been started during RAN1#103e.

The following options have been proposed:

Option 1: The reference point for frequency in an NTN is located at the satellite. 

The UE shall be responsible for determining the frequency offset required for frequency alignment at the satellite. The gNB shall manage the other sources of frequency error (e.g. satellite transponder, feeder links).

Option 2: The reference point for frequency in an NTN is located at the gNB: 

The UE shall be responsible for determining the frequency offset required for frequency alignment at the satellite. However, the network shall configure additional frequency offsets that are signalled to the UE. The signalled offsets are determined by the gNB. These offsets are applied on top of the one determined by the UE.

Option 3 : The reference point for frequency in an NTN is under control of the network:

The UE shall be responsible for determining the frequency offset required for frequency alignment at the satellite. However, the network may configure additional frequency offsets that are signalled to the UE. The signaled offsets are determined by the gNB and may be used to move the reference point. This is the most flexible option

The following is proposed by the companies w.r.t to Common Doppler shift pre/post compensation on the feeder link:

| Companies | Proposals |
| --- | --- |
| THALES | Proposal 9: RAN1 working assumption is that GW compensates common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE and gNB. |
| MediaTek Inc. | Proposal 4: RAN1 working assumptions for UL synchronization: - GW pre/post compensates common Doppler shift / Doppler shift variation over the feeder link - GW pre/post compensates any transponder frequency error at the satellite |
| Intel Corporation | Proposal 3: • It is assumed by RAN1 that compensation of feeder link Doppler shift is done at the gNB side for UL/DL |
| Ericsson | Proposal 9 The gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. The value of this parameter should be configurable. It may be used for compensating for the Doppler shift observed on the feeder link. Proposal 10 RAN1 should investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE. |
| Nokia, Nokia Shanghai Bell | Proposal 13: The location and determination of the frequency reference point must be agreed before developing further solutions for open technical aspects in NTN standardization. Proposal 14: If the frequency reference point is at the satellite, the satellite is responsible for pre-compensating the feeder link Doppler shifts applicable for both uplink and downlink. Proposal 15: The satellite has the responsibility of compensating the feeder link Doppler shift in DL and if needed in UL. The feeder link should be seen as an ideal link without any Doppler shift by the gNB. Proposal 16: The reference point for frequency should be under control of the network. In case the frequency reference point is not at the gNB, the satellite is responsible for compensating the feeder link Doppler shift in the uplink and downlink. |
| Qualcomm Incorporated | Observation 1: Compensation of feeder link timing drift and Doppler by UE can impose significant implementation complexity and may not be feasible. It is more efficient to have network compensate feeder link timing drift and Doppler in a way transparent to UE. |

## Company views

For [THALES, MediaTek, Intel] it is assumed that the network does pre/ post-compensation of Doppler shift over the feeder link. [Ericsson] proposed to use similar design as for Common TA; that is, the gNB to indicate necessary assistance information to be used by the UE to compensate the Doppler shift observed on the feeder link. Further, [Ericsson] observed  that having the feeder link Doppler shift performed by the GW is a convenient solution from the gNB and UE perspective but since it is currently unclear how to view the gateway from a 3GPP architecture point of view, the possibility to put requirements on the gateway to perform Doppler shift compensation needs further study. [Ericsson] then proposed for RAN1 to investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE.

In [Nokia]’s proposal the satellite has the responsibility of compensating the feeder link Doppler shift in DL and if needed in UL. [Qualcomm] observed that it is more efficient to have network compensate feeder link timing drift and Doppler in a way transparent to UE.

[Moderator’s view] It is not necessary that reference point of UL time synchronization and reference point for frequency synchronization are the same. At least for frequency synchronization it is viable to consider the reference point is on-board the satellite and the GW/Satellite compensate common Doppler shift / Doppler shift variation over the feeder link  and any transponder frequency error  in a transparent way to the UE and gNB.

Based on the view expressed in the companies’ contributions the following working assumption:

Working assumption:

The gateway and satellite-payload compensate common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error  in a transparent way to the UE and gNB.

- Companies are encouraged to provide their comments on this working assumption in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree. |
| Nokia, Nokia Shanghai Bell | In principle we could be Ok with this, but this discussion is somewhat coupled to the issue #9 where the information on a reference point could potentially allow for full UE autonomous compensation of any impairments from both the feeder link and the service link. |
| MediaTek | Agree working assumption. |
| Intel | Agree |
| Apple | We support the working assumption. Since the feeder link Doppler is common to all UEs, it is better for gateway to pre/post compensate it. This saves the signaling overhead to notify UE of feeder link Doppler so that UE to pre/post compensate it. |
| Intelsat | Agree |
| Ericsson | Fine as a working assumption but doubtful if 3GPP has any possibility to put requirements on gateway and satellite-payload. |
| Sony | Support |
| Xiaomi | Agree |
| CMCC | Regarding Ericsson’s concern, we support proposal with revision, to include the option that gNB compensates common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE. Updated Working assumption The gateway and satellite-payload or gNB compensate common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE and gNB. |
| QC | Agree |
| LG | Agree |
| ZTE | Agree |
| APT | Agree. It seems a working assumption since SI. |
| CATT | Agree |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Agree |
| Panasonic | It seems to be a reasonable working assumption. In our understanding, gNB and UE then only have to cope with the service link Doppler shift. It is not clear if a functional split between gNB and gateway has any specification impact. |
| Lenovo/MM | Agree with moderator’s view. |
| Sequans | Agree |

## Updated proposal based on company views (First round of email discussions)

20 companies provided views on this working assumption : All support or are fine with it.

[CMCC] support the proposal with revision: to include the option that gNB may also compensate common Doppler shift.

This working assumption was also discussed via the reflector. And the following proposal is made as follows:

Modified Proposal 10:

RAN1 should investigate whether a solution is possible where the gateway and/or satellite-payload compensate the common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error for both Downlink and Uplink in a way transparent to the UE and gNB.

- Companies are encouraged to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | The updated WA suggested by CMCC as below is fine. Updated Working assumption The gateway and satellite-payload or gNB compensate common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE and gNB. |
| Apple | It seems the main argument is which of gNB or gateway should compensate the feeder link Doppler shift, and there is no objection that the feeder link Doppler shift compensation is transparent to UE. Also, we think the “common” before Doppler shift is not needed, since it already refers to Doppler shift on feeder link. Hence, could we think about the following modification? Compensation of Doppler shift/Doppler shift variation over the feeder link and any transponder frequency error for both Downlink and Uplink is transparent to UE. FFS gateway/satellite compensation or gNB compensation |
| CATT | The common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error should be compensated by the network. For gNB or gateway implementation, it is transparent to UE, so we think detailed solution is out of RAN1 scope. Hence, we don’t think there is a need to recommend RAN1 to do further study. Instead, the following proposal is suggested: Compensation of Doppler shift/Doppler shift variation over the feeder link and any transponder frequency error for both Downlink and Uplink is transparent to UE. |
| LG | We are OK with modified proposal from Apple and CATT. |
| CMCC | The updated proposal suggested by Apple as below seems better. Compensation of Doppler shift/Doppler shift variation over the feeder link and any transponder frequency error for both Downlink and Uplink is transparent to UE. FFS gateway/satellite compensation or gNB compensation |
| Sony | We share Samsung’s comment. |
| Panasonic | We think that Doppler compensation for the feeder link part should be transparent to the UE. We don't see the need to specify a boundary between gateway and gNB. From a RAN1 perspective this will also simplify the discussion. We support the modifications proposed by Samsung and Apple. |
| Nokia, Nokia Shanghai Bell | There seems to be a lively debate on this topic directly on the reflector. Nokia’s view is that this topic should be investigated further (perhaps during RAN1#105-e). |
| Ericsson | We are fine with the proposal with the modifications proposed on the reflector. |
| ZTE | Views are shared over reflector. Prefer to have a clear statement/conclusion on the required RAN1’s behavior. |

## Updated proposal based on company views (2nd round of email discussions)

Based on the discussion via the reflector, the WF is to make the proposal as a working assumption, which RAN1 can revisit later if it requires update:

Updated Proposal 10:

Working assumption:

The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated in a way transparent to UE.

# Issue#11: Indication of common frequency pre-compensation offset on DL service link

In RAN1#104-e, the following conclusion was made:

Conclusion: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary. • FFS: support of DL frequency compensation for the service link Doppler.

Regarding the FFS in the conclusion above, as already discussed in [TR38.821] It was observed via simulations that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of GEO and LEO with beam specific pre-compensation of common frequency shift :

### [TR38.821] :

### 6.3.2	DL synchronization

According to the simulation assumptions in Table 6.1.2-1, the performance evaluation on the DL synchronization performance is conducted. The corresponding results from [43], [44], [45], [46], [47] are summarized in [48]. It is observed that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of GEO and LEO with beam specific pre-compensation of common frequency shift, e.g., conducted with respect to the spot beam center at network side, respectively.

However, for the LEO without pre-compensation of the frequency offset, additional complexity is needed at UE receiver to achieve robust DL initial synchronization performance based on Rel-15 SSB. No further enhancement on the SSB is needed.

Additionally, w.r.t the performance on the DL timing/frequency tracking, no issues have been identified based on Rel-15/16 NR design. Potential optimization can be further considered in potential normative phase if necessary.

Companies proposals regarding Issue#11 are collected in the following table:

| Companies | Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 8: The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing. |
| THALES | Proposal 10: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary: - In case of earth-fixed cell, the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB. - In case of earth-moving beam, the beam-specific common Doppler shift value. |
| PANASONIC R&D Center Germany | Proposal 6: Support of a common frequency offset relative to the UE frequency source and indicated via SIB. |
| CATT | Proposal 9: Common Doppler shift compensation value of DL and UL for service link can be indicated to UE, which can be defined with Khz granularity. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 9 Confirm the support of DL frequency compensation for the service link Doppler. |
| China Telecom | Proposal 4: gNB applies a common frequency pre-compensation on the DL link and a common post-compensation for the same amount on the UL link. Proposal 5: As the amount of the pre-compensation for the DL link is indicated to the UE, the post-compensation for the UL does not need another indication. Proposal 6: UE only take charge of the frequency shift cause by UE’s own behaviour, i.e. UE’s location or speed. |
| CMCC | Proposal 8: Support indication of DL frequency compensation for the service link Doppler, i.e., - If NR NTN gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of frequency pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. |
| ZTE | Proposal 7: The indication of frequency offset is sufficient to address issues for both DL and UL. |
| Xiaomi | Proposal 6: Pre-compensation value for DL frequency should be indicated by network. |
| Intel Corporation | Proposal 2: • Support DL frequency pre-compensation for the service link Doppler o Indication of the applied frequency pre-compensation value is required |
| Ericsson | Proposal 8 If gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. |
| Apple | Proposal 9: In downlink transmissions, support gNB pre-compensates and indicates a frequency offset for the service link Doppler shift with respect to a reference point. |
| BUPT | Proposal 1: It is necessary to compensate for doppler code deviation in low orbit satellite system. Proposal 2: Configuration of small bandwidth and large subcarrier spacing should be considered to weaken the effect of doppler code deviation. |
| Nokia, Nokia Shanghai Bell | Proposal 17: In the downlink a common frequency offset on service link is pre-compensated to limit the UE search space for the synchronization signals. Proposal 18: The amount of common frequency pre-compensation in DL in a cell may be indicated to the UE and thereby be used for determining the amount of UL frequency pre-compensation. |
| Sequans Communications | Proposal 4: RAN1 to consider indication to UE regarding frequency pre-compensation performed at neighbouring cells. |

## Companies views

For [Huawei] the amount of frequency compensation is normalized to a predefined subcarrier spacing. It can be defined with Khz granularity according to [CATT].

For [THALES]: how to indicate the amount of frequency compensation , will depend on the deployment scenario i.e. GEO, NGEO, Earth moving beam and Earth fixed beam.

[PANASONIC, Asia Pacific Telecom, China Telecom, CMCC, ZTE, Xiaomi, Intel, BUPT, Nokia] confirm the support the indication of DL frequency compensation for the service link Doppler.

For [Ericsson]  gNB may optionally apply frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link.

[Apple] In downlink transmissions, support gNB pre-compensates and indicates a frequency offset for the service link Doppler shift with respect to a reference point.

Furthermore, for [Sequans ] RAN1 to consider indication to UE regarding frequency pre-compensation performed at neighbouring cells.

[Moderator ‘s view]: The following step w.r.t this issue is to further investigate how to indicate the amount of frequency compensation on the service link and provide detailed signalling design.

We may discuss later on if it would be needed/beneficial to indicate the pre-compensation performed at neighbouring cells/beams as proposed by  [Sequans ].

Based on the views expressed within the different contributions, the Initial Proposal is made as follows:

Initial Proposal 11:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary:

- In case of Earth-fixed cell (GEO) or Earth-Quasi-fixed cell (NGEO), the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB.

- In case of Earth-moving beam, the beam-specific common Doppler shift value corresponding to the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link is indicated.

  - FFS: the granularity and how to indicate the amount of frequency compensation

Companies are encouraged to provide their comments and views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | It seems unclear why DL frequency compensation is needed to a UE. The main bullet was already made as Conclusion in RAN1#104-e. We think, regardless of Earth-fixed beam or Earth-moving beam, this indication of DL frequency compensation can be used for a UE to determine the nominal UL TX frequency. |
| Thales | Regarding the question from Samsung on the need of DL service link common frequency compensation: As captured in [TR38.821] It was observed via simulations that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of GEO and LEO with beam specific pre-compensation of common frequency shift. In case of Earth-Quasi-fixed cell (LEO/MEO): It is enough to indicate the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB. Then, the UE can derive at any given time : - Its own estimation of the residual Doppler experienced on the DL : fResidual Doppler,DL=VSat,USat_UEcUSat_UE -VSat,USat_RPcUSat_RP [ppm] - Its own estimation of the Doppler experienced on the UL : f Doppler,UL=VSat,USat_UEcUSat_UE [ppm] The UE shall take into account these two elements when generating its UL frequency carrier based on proprietary implementations. For instance, one can apply the following formula: fTX,UL,service= Cfactor×fRX LOCKED, DL, service Where fTX,UL,service refers to the UL frequency carrier generated by the UE, fRX LOCKED, DL, service refers to DL frequency carrier w.r.t which the UE is synchronized, and the Cfactor is computed as follows: Cfactor=fref, UL, servicefref,DL,service×1-f Doppler,UL1+fResidual Doppler,DL Where fref, UL, service and fref,DL,service are the reference UL and DL frequency carriers. |
| OPPO | support |
| Nokia, Nokia Shanghai Bell | We prefer a common signalling for both earth-fixed and earth-moving cells. The amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE. |
| MediaTek | Support proposal |
| Intel | OK |
| Apple | We are fine with the proposal. |
| Intelsat | Agree |
| Ericsson | For the first bullet, pre-compensation w.r.t. a fixed reference point means that the UE assumes that the pre-compensation changes continuously with time. The gNB DL frequency needs to be continuously updated to match this assumed offset by the UE. Deviation between the assumed and actual pre-compensation will result in a residual frequency error on the received UL signal. This adds unnecessary complexity to the gNB. From the UE perspective, the purpose of DL pre-compensation (i.e., to reduce synchronization complexity) is fulfilled with a much coarser pre-compensation that is updated less frequently (e.g., once every several seconds) and with a coarser granularity (e.g., in the order of kHz). Therefore, we prefer to have explicit signaling of the DL frequency pre-compensation value also for Earth-fixed cells. |
| Xiaomi | Ok |
| CMCC | We have the same view with Nokia to prefer a common signalling for both earth-fixed and earth-moving cells. The amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE. |
| QC | Don’t see the need of the agreement. The precompensated value is up to gNB. RAN1 can discuss the signaling part. |
| ZTE | For the DL part, we only need to define the possible parameters for indication to align the transmission among beams and following impacts on UL. How to compensate is only up to gNB. |
| APT | Support Initial Proposal 11 with the following text change. In case of Earth-fixed cell (GEO) or Earth-Quasi-fixed cell (NGEO), the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB is indicated. We also share the view from Nokia and Ericsson. It shall be enough by explicit signalling of the DL frequency pre-compensation value for Earth-moving/fixed/quasi-fixed cells. We may go back if companies report issues on signalling overhead in the future. |
| CATT | From UE prospective, the pre-compensated value should be indicated to UE. The proposal can be changed as: Indicating the amount of frequency compensation for DL is supported. |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Our preference is also have the same solution for different deployments. In addition, the tolerance of frequency error relates to SCS not Tx frequency, which means that requirement to the quantization error should be different under different SCS. Then, it is more reasonable to normalize the indication to SCS. |
| Panasonic | Agreed. |
| Lenovo/MM | Agree with initial proposal 11. |
| Sequans | Fine with the proposal |

## Updated proposal based on company views (First round of email discussions)

[MediaTek, Thales, Oppo, Intel, Apple, Intelsat, Xiaomi, APT, Spreadtrum, Panasonic, Lenovo/MM, Sequans] are supportive of the proposal.

[QC]:  Don’t see the need of the agreement , RAN1 can discuss the signaling part.

For [CATT] the pre-compensated value should be indicated to UE.

[Nokia , Ericsson, CMCC, Huawei] prefer to have explicit signaling of the DL frequency pre-compensation value for both  Earth-fixed cells and Earth-moving cells.

Based on the views expressed in first round of email discussions, the proposal 11 is updated as follows:

Updated  Proposal 11:

RAN1 to discuss the granularity and how to indicate the amount of frequency compensation when DL frequency compensation for the Doppler on service link is applied.

Companies are encouraged to provide their comments and views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Ok. |
| Apple | Fine with the proposal. |
| CATT | Agree |
| CMCC | We support this proposal. |
| Sony | Support. |
| Spreadtrum | We support this proposal. |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | OK to discuss this further |
| Huawei, HiSilicon | Fine |
| Ericsson | We are fine with the proposal. |
| ZTE | Fine |

## Updated proposal based on company views (2nd round of email discussions)

Based on the views expressed during the second round of mail discussions the proposal 11 is made as follows:

FL Recommendation 11:

RAN1 to discuss the granularity and how to indicate the amount of frequency compensation when DL frequency compensation for the Doppler on service link is applied.

# Issue#12: Indication of common compensation frequency offset for Uplink

This issue was already discussed in RAN1#104-e. Based on these discussions, the necessity of network indication of common frequency offset on UL is unclear to many companies. After the 2nd round of email discussions the FL recommendation was made as follows:

FL recommendation 3-3: On the indication of compensation frequency offset on UL, proponents are encouraged to have offline discussions with other companies.

The necessity of network indication of common frequency offset on UL was not clear to many companies because: It should be further discussed whether the post compensation is for the access link or feeder link:

-If the post compensation is for the feeder link. It is better to wait for the progress on issue#10 on pre/post common compensation of Doppler shift on the feeder link.

-If the compensation is for the service link: It was recommended to use UL frequency compensation indication during release-16 NR NTN study Item under the assumption that a UE without GNSS capability cannot estimate residual Doppler. With GNSS capability, there is no need for UL frequency compensation indication if the UE pre-compensation of Doppler is done with sufficient accuracy.

The companies proposals on this issue are collected in the following table:

| Companies | proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 9: Adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment. Proposal 10: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment. |
| CMCC | Proposal 9: If NR NTN gNB applies frequency post-compensation in UL, the gNB should broadcast a parameter giving the amount of frequency post-compensation, to achieve a common understanding between UE and gNB. This parameter should indicate the RX frequency offset at the satellite receiver relative to the nominal UE RX frequency of the service link. |
| Xiaomi | Proposal 7: The residual offset value of UL frequency at the reference point should be indicated by network. |
| Apple | Proposal 10: UE only pre-compensates its estimated Doppler shift on the service link in its uplink transmissions, and additional network indication of frequency offset to be pre-compensated by UE is not supported. |
| Samsung | Proposal 9: The gNB indicates the additional UL frequency offset value. |

## Companies views

[Huawei] proposed to adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment. For [CMCC] if NR NTN gNB applies frequency post-compensation in UL, the gNB should broadcast a parameter. According to [Xiaomi] the residual offset value of UL frequency at the reference point should be indicated by network. [Samsung] proposed that gNB indicates the additional UL frequency offset value. [Apple] additional network indication of frequency offset to be pre-compensated by UE is not supported. UE only pre-compensates its estimated Doppler shift on the service link in its uplink transmissions.

[Moderator’s view] wait for the progress on Issue#10. If the working assumption on common Doppler shift compensation on the feeder link is agreed. There is no need for indicating a common compensation frequency offset on Uplink.

Initial proposal 12:

Wait until progress is made on common Doppler shift pre/post compensation on the feeder link before to discuss the necessity of indicating a common compensation frequency offset to be applied by the UEs on Uplink.

Companies are invited to provide their comments/views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Thales | Agree |
| Nokia, Nokia Shanghai Bell | OK to wait. |
| MediaTek | Agree |
| Apple | Agree |
| Intelsat | Agree |
| Ericsson | Ok |
| Xiaomi | Ok |
| CMCC | OK |
| LG | OK |
| ZTE | Fine to discuss it later. It should be also coupled with DL frequency offset indication since the determination of nominal frequency is mainly for UL transmission. |
| APT | Support Initial proposal 12 |
| CATT | OK |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Agree. However, we don’t see any need to indicate a common compensation frequency offset to be applied by the UEs on Uplink |

| Panasonic | Agreed. |
| --- | --- |
| Sequans | Agree |

## Updated proposal based on company views (First round of email discussions)

17 companies provided views. All support or are fine with this proposal.

This proposal was further discussed via the reflector and the following FL recommendation is made:

FL Recommendation 12:

Wait until progress is made on common Doppler shift pre/post compensation on the feeder link before to discuss the necessity of indicating a common compensation frequency offset to be applied by the UEs on Uplink.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#13: Close control loop for UL frequency alignment

It was recommended to use UL frequency compensation indication during release-16 NR NTN study Item under the assumption that a UE without GNSS capability cannot estimate residual Doppler. With GNSS capability, there is no need for UL frequency compensation indication if the UE pre-compensation of Doppler is done with sufficient accuracy.

### [TR38.821] :

For the UL frequency compensation, at least for LEO system, the following solutions are identified with consideration on the beam specific post-compensation of common frequency offset at the network side:

●	Option-1: Both the estimation and pre-compensation of UE-specific frequency offset are conducted at the UE side. The acquisition of this value can be done by utilizing DL reference signals, UE location and satellite ephemeris.

●	Option-2: The required frequency offset for UL frequency compensation at least in LEO systems is indicated by the network to UE. The acquisition on this value can be done at the network side with detection of UL signals, e.g., preamble

In RAN1#103e, it was agreed to adopt Option-1 discussed in [TR38.821]:

Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

In RAN1#104-e, This issue was discussed and some companies want to consider option-2 in [TR38.821] based on a closed loop uplink frequency control.

However, several companies do not see the need for UL frequency control loop. The Moderator recommended for companies willing to support UL frequency control loop should provide more details on the benefits of such mechanism. And the following recommendation was made after the first round of email discussions:

FL recommendation : RAN1 to further investigate the needs and benefits to support closed-loop UL frequency compensation for GNNS equipped NR NTN UE

Closed-loop frequency control is discussed within 3 contributions submitted to RAN1#104-bis-e:

| Companies | proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 11: For GNSS UE, closed-loop UL frequency compensation is not needed. |
| Qualcomm Incorporated | Proposal 3 : Support closed-loop frequency control commands by MAC-CE. Proposal 4 : Consider group-common DCI for UL time and frequency control. |
| Nokia, Nokia Shanghai Bell | Proposal 20: Closed-loop control in RRC connected mode can be applied for UE frequency alignment in UL. The UE must follow commands provided by the gNB and ensure stability with any open-loop frequency alignment. |
| Spreadtrum Communications | Proposal 5: Autonomous frequency adjustment based on UE GNSS implementation is enough for UL frequency synchronization. |
| OPPO | Proposal 6: for uplink frequency synchronization, a UE shall pre-compensate the UE-specific Doppler shift on service link w.r.t a gNB’s uplink nominal frequency. |

## Companies views

[Qualcomm] observed that in case UE synchronizes to DL signal and estimates DL Doppler and UL Doppler based on the geolocations of the UE and of the satellite, a frequency error occurred in the feeder link from gNB to satellite will lead to a UE transmit frequency error twice of the feeder link error. Further according to [Qualcomm]  that there will be scenarios where CL UL frequency control would be needed due to UE movement and infrequent GNSS reading. Therefore, [Qualcomm] prosed to support closed-loop frequency control commands by MAC-CE and to consider group-common DCI for UL time and frequency control.

[Nokia] observed that closed-loop control in RRC connected mode can ensure UE frequency alignment for UL transmission also in case of GNSS loss. And proposed that closed-loop control in RRC connected mode can be applied for UE frequency alignment in UL. The UE must follow commands provided by the gNB and ensure stability with any open-loop frequency alignment.

According to [Huawei ] for GNSS UE, closed-loop UL frequency compensation is not needed. And for [Spreadtrum] Autonomous frequency adjustment based on UE GNSS implementation is enough for UL frequency synchronization. [Oppo] proposed for uplink frequency synchronization, a UE shall pre-compensate the UE-specific Doppler shift on service link w.r.t a gNB’s uplink nominal frequency.

[Moderator’s view]: With GNSS capability, there is no need for UL frequency compensation indication as long as the UE pre-compensation of Doppler shift is done with sufficient accuracy.

Based on the above, the following conclusion is made as follows:

Conclusion

On potential support of closed-loop frequency control, wait until more progress is made on the issues related to the accuracy of UE pre-compensation.

Companies are invited to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Ok. |
| Thales | Agree |
| Nokia, Nokia Shanghai Bell | OK to wait until further progress on accuracy of UE pre-compensation. |
| MediaTek | Agree |
| Apple | We are fine with the conclusion. |
| Intelsat | Agree |
| Ericsson | Ok |
| Xiaomi | Ok |
| CMCC | OK |
| QC | OK |
| LG | OK |
| ZTE | Ok |
| APT | Agree |
| CATT | OK |
| Spreadtrum | Agree |
| Huawei, HiSilicon | Agree |
| Panasonic | Agreed. |
| Lenovo/MM | Agree with moderator’s proposal. |
| Sequans | Agree |
| ZTE | Fine |

## Updated proposal based on company views (First round of email discussions)

19 companies provided views. All support or are fine with this conclusion. This issue was further discussed via the reflector and the following FL recommendation is made:

FL Recommendation 13

On potential support of closed-loop frequency control, wait until more progress is made on the issues related to the accuracy of UE pre-compensation.

## Updated proposal based on company views (2nd round of email discussions)

# Issue#14: Serving satellite ephemeris format

In RAN1#104-e, the compromise taken as way forward is to support both satellite ephemeris broadcast based on Ephemeris format based on satellite position and velocity state vectors and Ephemeris format based on orbital elements.

The following agreement was achieved:

Agreement: - RAN1 to support satellite ephemeris broadcast based at least on one of the following format options: - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats - FFS: Details on time reference provisioning/format - FFS: Whether down-selection is needed or both options are supported

Therefore, RAN1 need to continue the discussions on the design of both satellite ephemeris format: the content of ephemeris data, the signalling details and the Epoch time associated to ephemeris data. Whether down-selection is needed or both options are supported can be discussed at a later stage.

The following proposals were submitted to RAN1#104-bis-e:

| Companies | proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 12: At least Satellite ephemeris format based on the following orbital elements is supported - Semi-major axis a - Eccentricity e - Argument of periapsis ω - Longitude of ascending node Ω - Inclination i - Mean anomaly M0 |
| THALES | Proposal 11: RAN1 to discuss whether: - Option 1: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast - Option 2: Epoch time is explicitly indicated within the ephemeris data Proposal 12: RAN1 to support satellite ephemeris format: based on satellite position and velocity state vectors: - position X,Y,Z absolute in ECEF (m) in a field of 75 bits - velocity VX,VY,VZ absolute in ECEF (m/s) in a field of 75 bits based on orbital elements: - Da relative in J2000 (m) in a field of 19 bits - ex (absolute in J2000) in a field of 19 bits - ey (absolute in J2000) in a field of 19 bits - Di (relative in J2000) (rad) in a field of 19 bits - RAAN (absolute in J2000) (rad) in a field of 27 bits - AOL (absolute in J2000) (rad) in a field of 27 bits |
| PANASONIC R&D Center Germany | Proposal 7: We support to adopt both ephemeris format (orbital elements, and PVT format). |
| CATT | Proposal 1: Ephemeris format based on satellite position and velocity state vectors and Ephemeris format based on orbital elements can be supported both for different scenarios. Proposal 2: For ephemeris format based on orbital elements, existing KeplerianSet defined in TR 37.355 can be one starting point, while for the ephemeris format based on satellite position and velocity state vectors, satellite position and velocity indication could be the baseline. |
| Asia Pacific Telecom, FGI, ITRI, III | Proposal 10 Support the down selection of satellite ephemeris between 1) format based on satellite position and velocity state vectors and 2) format based on orbital elements. Proposal 11 For the ephemeris format, if the epoch time is not included in the ephemeris, then NW shall ensure UE knows the UE-gNB RTT to estimate the ephemeris at the current time. Proposal 12 If the epoch time is linked to the DL subframe, then it is corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which NTN SIB is transmitted. |
| MediaTek Inc. | Proposal 5: Epoch time is implicitly known as a reference time linked to DL subframe where NTN SIB is broadcast. Proposal 6: Support satellite ephemeris format bit allocations - Position and velocity state vector ephemeris format (16 bytes payload). - The field size for position [m] is 84 bits - The field size for velocity [m/s] is 60 bits - Orbital parameter ephemeris format (18 byte payload) - Semi-major axis α [m] is 33 bits - Eccentricity e is 19 bits - Argument of periapsis ω [rad] is 24 bits - Longitude of ascending node Ω [rad] is 21 bits - Inclination i [rad] is 20 bits - Mean anomaly M [rad] at epoch time to is 24 bits |
| CMCC | Proposal 10: Support both ephemeris formats based on satellite position and velocity state vectors (Option 1) and based on orbital elements (Option 2), if possible. Proposal 11: If down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Option 1) should be supported for implicit compatibility to support HAPS and ATG scenarios. |
| ZTE | Proposal 8: Ephemeris format based on instant state vectors with implicit time should be at least supported. |
| Ericsson | Proposal 13 RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation. |
| Apple | Proposal 11: Support both ephemeris formats, i.e., state vectors and orbital elements. |
| Samsung | Proposal 1: A gNB signals the serving satellite ephemeris to UEs in system information, including the followings: - index to a pre-defined table of satellite altitude levels and altitude offset scaling factors, i.e., NTN type - satellite altitude offset - satellite position - satellite velocity - reference time for satellite position and velocity. Proposal 2: Support Ephemeris format based on satellite position and velocity state vectors. |
| InterDigital, Inc. | Proposal 3: For ephemeris format, both formats are supported. Proposal 4: Option 1 is used for GEO deployment and Option 2 is used for LEO deployment. |
| NTT DOCOMO, INC. | Proposal 6: Two options can be both supported with different indication accuracy and periodicity. - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats FFS: Details on time reference provisioning/format |
| LG Electronics | Proposal 12. Regarding satellite ephemeris format, - Support one option to reduce both specification work and UE implementation complexity. - If two options need to be supported, only one option should be mandatory feature. Proposal 13. Support validity timing window for satellite ephemeris information in Rel-17 NTN. |
| Sequans Communications | Proposal 5: RAN1 to discuss signaling overhead and frequency update of satellite ephemeris broadcast design, considering altogether the serving satellite, constellation, and neigbouring cells’ ephemeris data. |

## Company views

Based on the views expressed in the Tdocs submitted to RAN1#104-bis-e, the companies have still different preference w.r.t the appropriate satellite ephemeris format to be supported. But the large majority is supportive of the compromise taken as way forward to support both satellite ephemeris broadcast. Only [LG, Asia Pacific Telecom] proposed to support one mandatory option to reduce both specification work and UE implementation complexity.

From Moderator’s perspective, it is beneficial to continue designing both formats. whether down-selection is needed or both options are supported can be discussed at a later stage.

w.r.t accuracy of serving-satellite ephemeris: [Ericsson] observed  that satellite ephemeris with sufficient accuracy to support timing and frequency offset pre-compensation shall be made available to the NR NTN UE. This can come with low frequency updates. And proposed for RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation. [Thales] observed that even for a satellite system with “low quality” orbit determination algorithm, challenging operations relying on accurate prediction of satellite trajectories such as Doppler compensation can be performed reliably.

[Moderator’s view] : The required accuracy for the satellite ephemeris format shall be driven based on the prediction budget error on satellite position and velocity vectors and the foreseen  prediction time horizon. It can be further discussed once the timing and frequency synchronization requirements are defined (waiting for RAN4 response to RAN1 LS on Time/frequency synchronization requirement).

It is necessary that the ephemeris is broadcast with low latency and high accuracy . And the signalling overhead introduced by satellite ephemeris broadcast shall be carefully quantified: [MediaTek] observed that a UE first coming into coverage of a satellite needs to immediately access if it is paged or if it needs to transmit data. The UE must be able to receive the satellite ephemeris on NTN SIB broadcast with periodicity 0.5s or 1 s. Further [MediaTek] prosed to indicate  PV state vector ephemeris format  in16 bytes payload (with a resolution of 1.3m and 0.06 m/s ) and orbital parameter ephemeris format in18 byte payload.

According to [Thales] to reduce the signalling overhead in case of ephemeris format based on satellite position and velocity state vectors, separate formats for GEO orbits, LEO orbits and HAPS/ATG may be defined. To reduce the signalling overhead in case of ephemeris format based on orbital elements, pre-provisioned ephemeris based on orbital elements can be used as reference. Only delta corrections can be broadcast in order to reduce the overhead. Further [Thales] proposed to indicate PV in Cartesian format in a payload of 19 bytes (with a resolution of 0.5 m and 0.0005 m/s ) and orbital parameters in adapted format in 17 bytes.

w.r.t time reference or Epoch time of the satellite ephemeris. The first approach is to transmit this time explicitly, as proposed by [Samsung] . It is the simplest approach but it increases signalling overhead. It can also be done in an implicit way to limit the signalling overhead as proposed by companies [Asia Pacific Telecom, MediaTek, ZTE].

To continue designing both satellite ephemeris formats, the following 3 Initial Proposals (15-1, 15-2 and 15-3) are made:

Initial Proposal 14-1:

Support serving-satellite ephemeris broadcast based:

- on satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

    - FFS: Whether separate format for GEO orbits, LEO/MEO orbits and HAPS/ATG shall be defined

- on orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both options are supported

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Okay. |
| Nokia, Nokia Shanghai Bell | Slight preference for Orbital parameters for better tracking of satellite. |
| Apple | We support this proposal. |
| Ericsson | We are fine with the proposal. Are ECEF or ECI coordinates preferable? |
| Sony | We think with satellite position and velocity state, we can get a unified solution that is also applicable to HAPS. |
| CMCC | We are fine with the proposal. |
| LG | Support. And, we prefer to support one option to reduce both specification work and UE implementation complexity. If two options need to be supported, only one option should be mandatory feature. |
| ZTE | Support for this proposal, but only one (PV) is preferred with unified format. No need to support the orbit and PV value in parallel |
| APT | Support Initial Proposal 14-1. Prefer PV. |
| Huawei, HiSilicon | We are fine with proposal. |
| Sequans | Support |
| CATT | Regarding the details of orbital parameter ephemeris format, existing format can be reused as defined in TR 37.355, which is used for GNSS based positioning. Parameter set ::= SEQUENCE { keplerToe INTEGER (0 .. 16383), keplerW INTEGER (-2147483648..2147483647), keplerDeltaN INTEGER (-32768..32767), keplerM0 INTEGER (-2147483648..2147483647), keplerOmegaDot INTEGER (-8388608.. 8388607), keplerE INTEGER (0..4294967295), keplerIDot INTEGER (-8192..8191), keplerAPowerHalf INTEGER (0.. 4294967295), keplerI0 INTEGER (-2147483648..2147483647), keplerOmega0 INTEGER (-2147483648..2147483647), keplerCrs INTEGER (-32768..32767), keplerCis INTEGER (-32768..32767), keplerCus INTEGER (-32768..32767), keplerCrc INTEGER (-32768..32767), keplerCic INTEGER (-32768..32767), keplerCuc INTEGER (-32768..32767), ... } |

Initial proposal 14-2:

RAN1 to discuss the required accuracy of serving-satellite ephemeris after RAN4 response to the LS on timing synchronization requirements

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | If RAN4 defines the requirement, RAN1/2 can discuss the information format size to meet the requirements. |
| Nokia, Nokia Shanghai Bell | Ok to wait with discussing needed accuracy when we have more information from RAN4 response. |
| Apple | We are fine with the proposal |
| Ericsson | Ok |
| CMCC | We are fine with the proposal. |
| ZTE | Fine |
| APT | Support Initial proposal 14-2 |
| Huawei, HiSilicon | Agree |
| CATT | OK |

Initial proposal 14-3:

RAN1 to discuss whether:

- Option 1: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast

- Option 2: Epoch time is explicitly indicated within the ephemeris data

- Companies are encouraged to indicate their preference in the following table (please elaborate):

| Companies | First preference | Second preference | Unacceptable option(s) |
| --- | --- | --- | --- |
| Samsung | Option 1 | Option 2 |  |
| OPPO | Option 1 (if workable) |  |  |
| Nokia, Nokia Shanghai Bell | Option 1. Normally, information that is broadcast by gNB which is associated to “time” is having this information coupled to the transmission timing. | Option 2 |  |
| MediaTek | Option 1 |  | Option 2. concern for payload on NTN SIB, and added complexity for UE which needs to have an internal clock synchronized to epoch time (e.g. internal clock based on GNSS time) |
| Intel | Option 1 | Option 2 |  |
| Apple | Option 1 | Option 2 |  |
| Ericsson | Fine to discuss. No preference on the options as of now. |  |  |
| Xiaomi | Option 1 |  |  |
| CMCC | Option 1 | Option 2 |  |
| ZTE | Option 1 | Option 2 |  |
| APT | Option 1. It is simple but this may not convert to the latest ephemeris when only one sample is received. | Option 2 It is a timestamp if NW can ensure good quality of granularity. The downside is the same as having a timestamp pointed out by MTK. |  |
| CATT | Option 1 | Option 2 |  |
| Huawei | Option 1 | Option 2 |  |
| Panasonic | Option 1 | Option 2 |  |
| Lenovo/MM | Option 1 |  |  |

## Updated proposal based on company views (First round of email discussions)

w.r.t Initial Proposal 14-1: 11 companies provided views. All support or are fine with this proposal

[Nokia] has slight preference for Orbital parameters for better tracking of satellite.

[Sony] has preference for satellite position and velocity state, because we can get a unified solution that is also applicable to HAPS.

[LG] prefer to support one option to reduce both specification work and UE implementation complexity. If two options need to be supported, only one option should be mandatory feature.

In Moderator’s view  the intention is to continue the discussions on the design of both satellite ephemeris format: the content of ephemeris data, the signalling details and the Epoch time associated to ephemeris data. Whether down-selection is needed or both options are supported can be discussed at a later stage.

Initial proposal 14-1 was further discussed via the reflector and it is updated as follows:

Updated  Proposal 14-1:

Support serving-satellite ephemeris broadcast based:

- on satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- on orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both options are supported

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | For the last FFS, will RAN1 discuss to down-selection or supporting both? This may contradict to the main bullet saying “Support …” Then we can change the main bullet as below. Support at least one format between the following formats for serving-satellite ephemeris broadcast: - Format-1: satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Format-2: orbital parameter ephemeris format: … |
| Apple | Fine with the proposal. |
| CATT | Regarding the details of orbital parameter ephemeris format, existing format can be reused as defined in TR 37.355, which is used for GNSS based positioning. Parameter set ::= SEQUENCE { keplerToe INTEGER (0 .. 16383), keplerW INTEGER (-2147483648..2147483647), keplerDeltaN INTEGER (-32768..32767), keplerM0 INTEGER (-2147483648..2147483647), keplerOmegaDot INTEGER (-8388608.. 8388607), keplerE INTEGER (0..4294967295), keplerIDot INTEGER (-8192..8191), keplerAPowerHalf INTEGER (0.. 4294967295), keplerI0 INTEGER (-2147483648..2147483647), keplerOmega0 INTEGER (-2147483648..2147483647), keplerCrs INTEGER (-32768..32767), keplerCis INTEGER (-32768..32767), keplerCus INTEGER (-32768..32767), keplerCrc INTEGER (-32768..32767), keplerCic INTEGER (-32768..32767), keplerCuc INTEGER (-32768..32767), ... } |
| LG | We agree with Samsung. In order to make clearer, it could be better to replace the words “format(s)” to “option(s)” in modified proposal from Samsung as below. Support at least one option between the following options for serving-satellite ephemeris broadcast: - Option-1: satellite position and velocity state vectors: - position X,Y,Z in ECEF (m) - velocity VX,VY,VZ in ECEF (m/s) - Option-2: orbital parameter ephemeris format: … |
| Sony | Support |
| Panasonic | Agree with the proposal |
| Nokia, Nokia Shanghai Bell | As different formats may be used under different conditions (LEO/GEO), it should be considered to outline this in the potential agreement – that is, listing the two sets of information (do we need separation into the sets?), which may be used and potentially combined. |
| Huawei, HiSilicon | We support the proposal. Regarding the comment from CATT, our understanding is that the NavModelKeplerianSet defined in 37.355 is used for navigation. Some of the parameter are not needed in the context of NTN such as the correction terms. |

w.r.t initial proposal 14-2: 9 companies provided views. All support this proposal

Initial proposal 14-2: was further discussed via the reflector and the following FL recommendation is made:

FL Recommendation 14-2:

RAN1 to discuss the required accuracy of serving-satellite ephemeris after RAN4 response to the LS on timing synchronization requirements

- Companies are encouraged to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Ok |
| Apple | Fine with the FL recommendation. |
| CATT | OK |
| LG | OK |
| Sony | Support |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | Ok to discuss. |
| Huawei, HiSilicon | Okay |

w.r.t initial proposal 14-3: 15 companies provided views:

[Samsung, OPPO, Nokia, Nokia Shanghai Bell, MediaTek, Intel, Apple,  Xiaomi, CMCC, ZTE, APT, CATT, Huawei, Panasonic, Lenovo/MM] prefer option 1. [Ericsson]: no preference.

[Option 2] is unacceptable option for MediaTek because  there is a concern for payload on NTN SIB, and add complexity for UE which needs to have an internal clock synchronized to epoch time (e.g. internal clock based on GNSS time).

Updated proposal 14-3:

Epoch time is implicitly known as a reference time linked to DL subframe where NTN SIB is broadcast.

- Companies are encouraged to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree |
| Apple | Agree |
| CATT | Agree |
| LG | Agree |
| CMCC | We support this proposal. |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | Agree |
| Huawei, HiSilicon | Agree |
| Ericsson | Implicit epoch time based on DL subframe number requires that the gNB TX timing is closely coordinated with an absolute time reference. The feasibility of this should be investigated. It should also be clarified what the reference point of time is – the gNB transmitter or the satellite transmitter. |
| ZTE | Support. Moreover, further consideration on the indication of “error range” /degree of confidence of indicated satellite ephemeris should be considered. |

## Updated proposal based on company views (2nd round of email discussions)

Based on the views expressed during the second round of mail discussions the proposal 14-1 is as follows:

Agreement:

Support serving-satellite ephemeris broadcast based on one or more of the following:

- Set 1: Satellite position and velocity state vectors:

  - position X,Y,Z in ECEF (m)

  - velocity VX,VY,VZ in ECEF (m/s)

- Set 2: At least the following parameters in orbital parameter ephemeris format:

  - Semi-major axis α [m]

  - Eccentricity e

  - Argument of periapsis ω [rad]

  - Longitude of ascending node Ω [rad]

  - Inclination i [rad]

  - Mean anomaly M [rad] at epoch time to

    - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead

- FFS: The field size for each parameter

- FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris

- FFS: Whether down-selection is needed or both sets are supported

Based on the views expressed during the second round of mail discussions the proposal 14-3 is made as follows:

FL recommendation:

RAN1 to study whether epoch time for ephemeris information is implicitly (derived from DL subframe) or explicitly signaled.

# Issue#15 Orbit propagator model in UE

Depending on adopted ephemeris format; Option 1 or Option 2, different propagator models can be considered for implementation at UE side. Some have been developed for orbital elements and others for PV state vectors.

To enhance the propagator model additional elements may be included in the ephemeris data.

Further adequate fitting can be performed based on the reference orbit and the prediction time horizon considered to further enhance UE pre-compensation accuracy. Of course, this is possible only if the propagator model to be used at UE side is known by the network. Thus, an agreement on a specified model to be used can be beneficial.

The following is proposed by [Thales]:

| Companies | proposals |
| --- | --- |
| THALES | Proposal 13: RAN1 to support one of the following options regarding the NR NTN UE propagator model: - Option 1: a specified propagation model is to be implemented at UE side - Option 2: the propagator model to be used at UE side can be left to implementation Observation 13. Propagator model at UE side can be left to implementation. But, the adopted ephemeris format can strongly orientate the choice of model. Further, an agreement on an specified model can be beneficial. This is useful to perform suitable fitting at network side to enhance orbit prediction by the NR NTN UEs. |

## Company views

Based on the above observation,  RAN1 to discuss the necessity of the following proposal:

- Initial Proposal 15:

RAN1 to support one of the following options regarding the NR NTN UE propagator model:

- Option 1: a specified propagation model is to be implemented at UE side

- Option 2: the propagator model to be used at UE side can be left to implementation

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| THALES | We prefer option 1: a specified propagation model is to be implemented at UE side It was observed based on simulation that the prediction time period before overshooting the delay and Doppler pre-compensation target is linked to the broadcast satellite format and to the propagation model used in the UE. As shown in the table below prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for RTD error and within 45.93 Hz for Doppler error (in FR2) when Kepler model is used. This is sufficient to accommodate UE pre-compensation requirement when using 120 kHz SCS. Prediction 180 s ahead for UE pre-compensation is accurate within 1.74 µs for RTD error and within 17.60 Hz (in S band) for Doppler error when Kepler model is in use. This is sufficient to accommodate UE pre-compensation requirement when using 15 kHz SCS. Using other propagation models in the UE will significantly enhance UE pre-compensation accuracy during a larger prediction time period. For example, by using analytical Eckstein-Hechler model, prediction 5 minutes ahead for UE pre-compensation is accurate within 0.29 µs for RTD error and within 6.20 Hz (in FR2) for Doppler error. Orbit propagation model comparison: |
| Samsung | Support Option 2. |
| OPPO | Option 2 |
| Nokia, Nokia Shanghai Bell | Option 2: No need to define specific model for the UE to use for compensation. As stated by Thales the model to use can vary by the ephemeris format and should be left to UE implementation. That would also follow the 3GPP concept of not defining actual algorithms to use at the UE side. |
| MediaTek | Option 2 |
| Intel | Option 2. It can be validated by tests, if needed |
| Apple | Option 2 |
| Ericsson | We prefer Option 2. |
| Xiaomi | Option 2 is preferred |
| LG | Support Option 2 |
| ZTE | Option 2, no need and it’s also impractical to specify the mandated solution. |
| APT | Option 2. Not sure if this enhancement is needed in Rel-17. We tend to agree that RAN1 shall discuss the required accuracy of serving-satellite ephemeris after RAN4 responds to the LS on timing synchronization requirements. |
| CATT | Support option 2, but the propagation model can be discussed from technical clarification point of view. |
| Huawei, HiSilicon | Support Option 2, and the propagation model can be adapted according to different scenarios. |
| Panasonic | Option 2: the propagator model to be used at UE side can be left to implementation |

## Updated proposal based on company views (First round of email discussions)

15 companies provided views. 15 prefer Option 2.

Based on the inputs provided during the first round of email discussions, the  following conclusion is made :

Conclusion

The propagator model to be used at UE side can be left to implementation.

- Companies are encouraged to provide their comments in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | Agree |
| Apple | To clarify the conclusion, we could modify as The orbital propagator model to be used at UE side is left to implementation. |
| CATT | OK |
| LG | Agree |
| CMCC | Agree |
| Panasonic | Agree |
| Nokia, Nokia Shanghai Bell | Agree with Apple that orbital may be included in the proposed conclusion. |
| Huawei, HiSilicon | Fine and agree with change proposed by Apple. |
| Ericsson | OK |
| ZTE | Support |

## Updated proposal based on company views (2nd round of email discussions)

Based on the views expressed during the second round of mail discussions and via the reflector, the conclusion is updated as follows:

Conclusion

The orbital propagator model to be used at UE side can be left to implementation.

# Issue#16 SRS enhancements

According to [Qualcomm ] for UL, existing Sounding Reference Signal (SRS),  is inefficient for frequency estimation. Thereby, there is a need to enhance SRS for time and frequency estimation.

The justification can be found in [R1-2103169]. It is recopied hereafter:

Time and frequency synchronization in NTN are much more challenging than in terrestrial networks. Hence, there is a need to enhance reference signals for time and frequency estimation in both DL and UL. For DL, multiple-symbol TRS is supported that should be sufficient for frequency estimation in the DL. For UL, existing SRS, however, is inefficient for frequency estimation.

To support efficient frequency error estimation, SRS with multiple coherent symbols should be supported as illustrated in Figure 2. To support good trade-off between frequency pulling range and accuracy, configurable symbol gaps between the SRS symbols should be considered and supported. In addition, the comb position can be alternated among the SRS symbols to remove the ambiguity of half-symbol time uncertainty in comb 2.

[Qualcomm] proposed:

| R1-2103169 | Qualcomm Incorporated | Proposal 5 : Support SRS with multiple coherent symbols with configurable gaps. |
| --- | --- | --- |

Initial Proposal 16[Qualcomm]:

RAN1 to discuss the necessity of the following proposal:

Support SRS with multiple coherent symbols with configurable gaps.

- Companies are encouraged to provide their views in the following table:

| Companies | Comments and Views |
| --- | --- |
| Samsung | RAN1 can discuss after concluding the discussion on whether to support frequency offset indication and the requirements that will be defined in RAN4. |
| Nokia, Nokia Shanghai Bell | In our view, such optimization is not needed. |
| MediaTek | Wait for RAN4 discussion on frequency error requirements. We see no necessity for SRS-based transmission assuming accurate UE pre-compensation of Doppler shift on service link, and assuming accurate common Doppler shift pre/post compensation on the feeder link by the GateWay/gNB. |
| Intelsat | Discuss further in RAN 1 |
| Ericsson | We don’t see a need for this. |
| Xiaomi | We think the necessary to support this is not clear. |
| CMCC | This discussion is related to issue #13 (Close control loop for UL frequency alignment). If issue 13 is not supported, enhancement on SRS seems meaningless, since gNB cannot adjust the UL frequency error of UE. |
| ZTE | No need |
| APT | We may need an agreement, e.g., SRS with multiple coherent symbols with configurable gaps for NTN is not supported in this release (or Rel-17). This proposal has been here since RAN1#97 in R1-1907278. |
| CATT | Configuring gap based SRS would be beneficial to NTN scenarios. So we support this proposal. |
| Huawei, HiSilicon | We don't see the need. |
| Panasonic | We think SRS across multiple coherent symbols is a possible optimization connected to closed loop frequency control which can be considered by RAN1 after the details of closed loop frequency control (Proposal#13) are clearer. |
| Thales | No need |

## Updated proposal based on company views (First round of email discussions)

Such optimization is not needed according to the majority of companies who provided view [Nokia, Nokia Shanghai Bell, MediaTek, Ericsson, Xiaomi, CMCC, ZTE, Huawei, HiSilicon, Thales]

Based the views expressed during the first round of email discussions, the following FL recommendation is made:

FL Recommendation 16:

On SRS enhancements  in NTN, proponents are encouraged to have offline discussions with other companies.

# References

- R1-2102342	Discussion on UL time and frequency synchronization enhancement for NTN	Huawei, HiSilicon

- R1-2102360	Considerations on UL timing and frequency synchronization in NTN	THALES

- R1-2102398	Discussion on UL time and frequency synchronization	OPPO

- R1-2102459	Consideration on enhancements on UL time and frequency synchronization	Spreadtrum Communications

- R1-2102555	Discussion on UL time and frequency synchronization for NTN	PANASONIC R&D Center Germany

- R1-2102573	Considerations on Enhancements on UL Time Synchronization in NTN	CAICT

- R1-2102634	UL time and frequency compensation for NTN	CATT

- R1-2102733	UL time and frequency synchronization in NTN	Asia Pacific Telecom, FGI, ITRI, III

- R1-2102752	Enhancements on UL Time and Frequency Synchronisation for NR-NTN	MediaTek Inc.

- R1-2102865	Discussion on UL time and frequency synchronization for NR NTN	China Telecom

- R1-2102885	Enhancements on UL time and frequency synchronization for NTN	CMCC

- R1-2102915	Discussion on UL synchronization for NR-NTN	ZTE

- R1-2102986	Discussion on UL time and frequency synchronization for NTN	Xiaomi

- R1-2103011	UL time synchronization acquisition for NTN	Mitsubishi Electric RCE

- R1-2103033	On UL synchronization for NTN	Intel Corporation

- R1-2103059	On UL time and frequency synchronization enhancements for NTN	Ericsson

- R1-2103108	On Uplink Time and Frequency Synchronization for NTN	Apple

- R1-2103169	UL time and frequency synchronization for NTN	Qualcomm Incorporated

- R1-2103242	Enhancements on UL time and frequency synchronization for NTN	Samsung

- R1-2103277	UL time/frequency synchronization for NTN	InterDigital, Inc.

- R1-2103305	Enhancement for UL time synchronization	Sony

- R1-2103533	Discussion on NTN uplink time synchronization	Lenovo, Motorola Mobility

- R1-2103544	Discussions on doppler effect in  time synchronization for NTN	BUPT

- R1-2103579	Discussion on UL time synchronization enhancements for NTN	NTT DOCOMO, INC.

- R1-2103620	Discussions on UL time and frequency synchronization enhancements in NTN	LG Electronics

- R1-2103655	Discussion on UL Time Synchronization for NTN	Fraunhofer IIS, Fraunhofer HHI

- R1-2103670	Discussion on time and frequency synchronization for NTN operation	Nokia, Nokia Shanghai Bell

- R1-2103687	On UL time and frequency synchronization enhancements for NR NTN	Sequans Communications

- R1-2103731	Further discussion on UL time synchronization methods for NTN systems	CEWiT, IITM, IITH

# Appendix I: RAN1 agreements on UL time and frequency synchronization for NR NTN

RAN1 Meeting #104-e (e-Meeting, January 25th – February 5th, 2021): Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control Agreement: For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. FFS: Details of the combination of open and closed loop TA control Conclusion: It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation Agreement: RAN1 should send an LS to RAN4 with the following questions: Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Conclusion: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary. - FFS: support of DL frequency compensation for the service link Doppler. Agreement: - RAN1 to support satellite ephemeris broadcast based at least on one of the following format options: - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats - FFS: Details on time reference provisioning/format - FFS: Whether down-selection is needed or both options are supported RAN1 Meeting #103-e (e-Meeting, October 26th – November 13th, 2020): Agreement: An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. Agreement: An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. Agreement: - In NTN, the network may broadcast - A common timing offset value - FFS details of the common timing offset - FFS: A common timing drift rate - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: TA= NTA+NTA, offset[+X]×Tc[+X] Where: NTA is derived from the User specific TA self-estimation X is derived at least from the common timing offset value if broadcasted by the network. The granularity of X and whether X is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed. - NTA, offset depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2. - Tc is specified in TS 38.211 section 4.1. - Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Working assumption: It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link. RAN1 Meeting #102-e (e-Meeting, August 17th – 28th, 2020): Agreement: • In Rel-17 NR NTN, at least support UE which can derive based on its GNSS implementation one or more of: o its position o a reference time and frequency • And, based on one or more of these elements together with additional information (e.g., serving satellite ephemeris or timestamp) signalled by the network, can compute timing and frequency, and apply timing advance and frequency adjustment at least for UE in RRC idle/inactive mode. • FFS: Details on additional information signalled from network Agreement: In case of GNSS-assisted TA acquisition in RRC idle/inactive mode, the UE calculates its TA based on the following potential contributions: • The User specific TA which is estimated by the UE: o Option 1: The User specific TA is estimated by the UE based on its GNSS acquired position together with the serving satellite ephemeris indicated by the network:  FFS: Details on serving satellite ephemeris indication o Option 2: The User specific TA is estimated by the UE based on the GNSS acquired reference time at UE together with reference time as indicated by the network • The Common TA if indicated by the network: o FFS: The need and details of Common TA indication • FFS: The TA margin, if needed and indicated by the network (in order to account for the TA estimation uncertainty)

# Appendix II: Summary of proposals

| TDoc | Source |  |
| --- | --- | --- |
| R1-2102342 | Huawei, HiSilicon | Proposal 1: The delay compensated by the gNB should be a constant value considering the gNB implementation complexity. Proposal 2: The RP for common timing offset is left to gNB implementation to support all the foreseen deployment scenarios. Proposal 3: The common timing offset is determined as the RTD from the reference point to the satellite, i.e. by subtracting the delay compensated at the gNB from the feeder link RTD. Proposal 4: The granularity of common timing TA is set to be the same as the granularity of NTA, i. e. 16∙64/2μ*Tc. Proposal 5: The TA margin for the maximum estimation error is included in common TA offset. Proposal 6: Indicate a common TA drift rate as part of the common TA applied by UE. Proposal 7: For Msg1/MsgA transmission and TA maintenance, use a common TA drift rate to compensate the TA drift between SIB decoding and UL transmission. Proposal 8: The indicate DL frequency pre-compensation is normalized to a predefined subcarrier spacing. Proposal 9: Adopt frequency post/pre-compensation with minimum indication overhead for UL frequency alignment. Proposal 10: To reduce the signaling overhead, only DL pre-compensation indication is needed and sufficient for UL frequency alignment. Proposal 11: For GNSS UE, closed-loop UL frequency compensation is not needed. Proposal 12: At least Satellite ephemeris format based on the following orbital elements is supported - Semi-major axis a - Eccentricity e - Argument of periapsis ω - Longitude of ascending node Ω - Inclination i - Mean anomaly M0 |
| R1-2102360 | THALES | Proposal 1: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: When common TA is indicated by the Network: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc When common TA is not indicated by the Network: TTA=NTA+NTA,UE-specific+NTA,offset×Tc Where: - NTA and NTA,offset are defined as in Release-16. - NTA,UE-specific is UE self-estimated TA - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. Proposal 2: Introduce a flag in the SIB to indicate whether the Common TA is present and need to be applied for TTA calculation Proposal 3: If Common TA shall be considered in UE autonomous TA calculation, the Network shall periodically broadcast: - in case of GEO based non-terrestrial access network: - Common TA (NTA,common ) in a field of 23 bits - in case of LEO/MEO based non-terrestrial access network: - Common TA (NTA,common ) in a field of 19 bits - Common TA drift rate( NCommondrift) in a field of 13 bits - FFS: Common TA drift rate variation (NCommondrift_variation_rate ) in a field of 6 bits Proposal 4: RAN1 to discuss the indication of TA-margin after RAN4 response to the LS on timing synchronization requirements Proposal 5: Existing NTA update in RRC_CONNECTED based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change - FFS: Whether to support negative TA Command in RAR Proposal 6: NTN UE in RRC_CONNECTED shall have an active BWP with common search space configured to monitor system information. Proposal 7: A validity timer configured for UE specific TA defines the maximum time during which the UE can track the RTD on service link without having acquired new ephemeris data to be used for UE specific TA estimation. - This timer is restarted each time the UE receives new ephemeris data - The UE assumes that it has lost uplink synchronization if this timer expires and new ephemeris data is not available. Proposal 8: A validity timer configured for Common TA defines the maximum time during which the UE can track the Common RTD without having acquired new assistance information to be used for Common TA estimation. - This timer is restarted each time the UE receives new assistance information. - The UE assumes that it has lost uplink synchronization if this timer expires and assistance information is not available. Proposal 9: RAN1 working assumption is that GW compensates common Doppler shift / Doppler shift variation over the feeder link and any transponder frequency error in a transparent way to the UE and gNB. Proposal 10: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary: - In case of earth-fixed cell, the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB. - In case of earth-moving beam, the beam-specific common Doppler shift value. Proposal 11: RAN1 to discuss whether: - Option 1: Epoch time is implicitly known as a reference time linked to DL slot where satellite ephemeris is broadcast - Option 2: Epoch time is explicitly indicated within the ephemeris data Proposal 12: RAN1 to support satellite ephemeris format: based on satellite position and velocity state vectors: - position X,Y,Z absolute in ECEF (m) in a field of 75 bits - velocity VX,VY,VZ absolute in ECEF (m/s) in a field of 75 bits based on orbital elements: - Da relative in J2000 (m) in a field of 19 bits - ex (absolute in J2000) in a field of 19 bits - ey (absolute in J2000) in a field of 19 bits - Di (relative in J2000) (rad) in a field of 19 bits - RAAN (absolute in J2000) (rad) in a field of 27 bits - AOL (absolute in J2000) (rad) in a field of 27 bits Proposal 13: RAN1 to support one of the following options regarding the NR NTN UE propagator model: - Option 1: a specified propagation model is to be implemented at UE side - Option 2: the propagator model to be used at UE side can be left to implementation |
| R1-2102398 | OPPO | Proposal 1: apply TA as TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc, where - NTA is adjusted by RAR or TAC from MAC-CE - NTA,UE-specific is a UE-specific TA estimated based on NTN satellite ephemeris - NTA,common is a common TA indicated by the network - NTA,offset is a cell-specific offset Proposal 2: When the common TA is not provided by the network, UE assumes NTA,common=0. Proposal 3: NTA,offset should factor in TA margin. Proposal 4: TA updating by MAC-CE or RAR can have the following relationship NTA_new = NTA_old + adjustment, where for TA updated by MAC-CE, the adjustment is and TA is the TAC indication in the range of [0, 63]; for TA updated by RAR, the adjustment is and the TA is the RAR TAC indication in the range of [0, 3846]. Proposal 5: the network indicated common TA can be updated by a drift rate, e.g. NTA,common_new= NTA,common_old + drift rate * t Proposal 6: for uplink frequency synchronization, a UE shall pre-compensate the UE-specific Doppler shift on service link w.r.t a gNB’s uplink nominal frequency. |
| R1-2102459 | Spreadtrum Communications | Proposal 1: N_(TA,common) should be expressed in the legacy granularity of Tc units. Proposal 2: The value of common TA defaults to zero when not indicated by the network. Proposal 3: TA margin is indicated in SIB should be supported. Proposal 4: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Proposal 5: Autonomous frequency adjustment based on UE GNSS implementation is enough for UL frequency synchronization. |
| R1-2102555 | PANASONIC R&D Center Germany | Proposal 1: We propose the Common Timing Offset value formulation expressed by multiples of Tc (TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc) with NTA,common having a granularity in the order of slot or half slot duration. Proposal 2: The common TA is always indicated in SIB. Value 0 for NTA,common should be supported. Proposal 3: We don’t see a need for the UE to handle a TA margin. As a value transparent to the UE, the TA margin can be included in the Common TA by the network. Setting of the TA margin (and any other margin) is up to the network implementation. Proposal 4: We propose to at least allow for a non-zero DL-UL timing difference manged by the gNB - FFS: The need and feasibility of a common timing drift rate to accurately compensate the feeder link delay including feeder link switch operation. Proposal 5: In RRC_CONNCTED mode and after expiration of the TA timer, a UE triggers the random-access procedure based on GNSS-acquired TA similar to RRC_IDLE with the same timing advance equation. Proposal 6: Support of a common frequency offset relative to the UE frequency source and indicated via SIB. Proposal 7: We support to adopt both ephemeris format (orbital elements, and PVT format). |
| R1-2102573 | CAICT | Proposal1: In the scenario of transparent satellites, setting whether the satellite or the gNB as the reference point, both options have advantages and disadvantages and shall be supported. Proposal2: Support to adopt cell-specific or beam-specific K_offset for TA pre-compensation by the UEs without capability of location acquisition. Proposal3: FFS the condition when to apply the TA through combined closed loop and open loop. |
| R1-2102634 | CATT | Proposal 1: Ephemeris format based on satellite position and velocity state vectors and Ephemeris format based on orbital elements can be supported both for different scenarios. Proposal 2: For ephemeris format based on orbital elements, existing KeplerianSet defined in TR 37.355 can be one starting point, while for the ephemeris format based on satellite position and velocity state vectors, satellite position and velocity indication could be the baseline. Proposal 3: When the common TA is not indicated by the gNB, common TA should be set to zero. Proposal 4: The granularity of common TA can apply 1024 Tc or 2048Tc of step size. Proposal 5: Whether to broadcast the common timing drift rate needs further investigation. Proposal 6: TA margin can be one fixed value or not needed at all if UL timing requirement of RRC-IDLE mode and RRC-connected mode for UE is same. Proposal 7: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Proposal 8: UE can stop autonomous TA compensation or subtract the accumulated TA compensated by autonomous TA compensation when combining the close-loop TA compensation. Proposal 9: Common Doppler shift compensation value of DL and UL for service link can be indicated to UE, which can be defined with Khz granularity. |
| R1-2102733 | Asia Pacific Telecom, FGI, ITRI, III | Proposal 1 Support the common TA drift rate to improve the required periodicity of the SI-message to signal the common TA from every 80ms to every 1.6 seconds. Proposal 2 Support the GW location pre-stored in u-sim to prevent the common TA signalling, if there is no security concern. Proposal 3 For RP at gNB, disabling the open-loop TA control shall be supported, and as a result, the UL timing can be maintained by the closed-loop TA control with 6-bit TA commands and TA drift rates. Proposal 4 Support UE autonomous TA adjustment if and only if NW can still know the absolute TA value. Proposal 5 Support TA report in RRC_CONNECTED to provide UE-gNB RTT to NW. Proposal 6 Support UE location report in RRC_CONNECTED to prevent frequent TA reporting, if there is no privacy concern. Proposal 7 If TA reporting in RRC_CONNECTED is supported, the following values shall be considered between consecutive TA reports: 1) the maximum amount of the magnitude of the timing change in one adjustment and 2) the minimum/maximum aggregate adjustment rate per second. Proposal 8 For RP at a satellite, disabling the open-loop TA control shall be supported, and as a result, the UL timing can be maintained by the closed-loop TA control with 6-bit TA commands and TA drift rates. Proposal 9 Confirm the support of DL frequency compensation for the service link Doppler. Proposal 10 Support the down selection of satellite ephemeris between 1) format based on satellite position and velocity state vectors and 2) format based on orbital elements. Proposal 11 For the ephemeris format, if the epoch time is not included in the ephemeris, then NW shall ensure UE knows the UE-gNB RTT to estimate the ephemeris at the current time. Proposal 12 If the epoch time is linked to the DL subframe, then it is corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which NTN SIB is transmitted. |
| R1-2102752 | MediaTek Inc. | Proposal 1: Configuration of NTA,common is up to gNB depending on assumption for reference point for DL subframe and UL subframe alignment - NTA,common = RTD of feeder link if reference point is gNB - NTA,common =0 if reference point is satellite Proposal 2: In case reference point for UL-DL subframe timing alignment is gNB, broadcast on NTN SIB - Common delay NTA,common in a 18 bit field - Common delay drift rate NTA,common,drift, rate in a 8 bit field - Common delay drift rate variation NTA,common,drift,rate,variation in a 6 bit field Proposal 3: TA margin can be included in scope of discussions for timing synchronization requirements in RAN4. Proposal 4: RAN1 working assumptions for UL synchronization: - GW pre/post compensates common Doppler shift / Doppler shift variation over the feeder link - GW pre/post compensates any transponder frequency error at the satellite Proposal 5: Epoch time is implicitly known as a reference time linked to DL subframe where NTN SIB is broadcast. Proposal 6: Support satellite ephemeris format bit allocations - Position and velocity state vector ephemeris format (16 bytes payload). - The field size for position [m] is 84 bits - The field size for velocity [m/s] is 60 bits - Orbital parameter ephemeris format (18 byte payload) - Semi-major axis α [m] is 33 bits - Eccentricity e is 19 bits - Argument of periapsis ω [rad] is 24 bits - Longitude of ascending node Ω [rad] is 21 bits - Inclination i [rad] is 20 bits - Mean anomaly M [rad] at epoch time to is 24 bits |
| R1-2102865 | China Telecom | Proposal 1: The TA margin shall be included in the common TA. Proposal 2: NTA and NTA,common are updated together by a combined TA command based on closed loop updating. Proposal 3: NTA,UE-specific is updated by adding the newly estimated offset directly: NTA,UE-specificnew= NTA,UE-specificold+ΔNTA, UE specific Proposal 4: gNB applies a common frequency pre-compensation on the DL link and a common post-compensation for the same amount on the UL link. Proposal 5: As the amount of the pre-compensation for the DL link is indicated to the UE, the post-compensation for the UL does not need another indication. Proposal 6: UE only take charge of the frequency shift cause by UE’s own behaviour, i.e. UE’s location or speed. |
| R1-2102885 | CMCC | Proposal 1: The common TA is determined as NTA,commonn=NTA,common, base+(n-nTA,common,base)∙NTA,common,drift where: n is the slot number of the targeted UL slot. nTA,common,base is a “time stamp” slot number. NTA,common, base is the common TA (in Tc units) at slot number nTA,common,base. NTA,common,drift is the common TA drift rate (in Tc units per slot). and, NTA,common, base=A+S∙X NTA,common,drift=S∙Y where: A represents the synthetical impact of time-invariant parameters. S is a scaling factor. X, Y are time-variant parameters, and they need to be updated with time. Proposal 2: Indication of parameter A (a time-invariant offset of NTA,common, base) should be supported. - For DL and UL aligned at satellite, A can be used to absorb TA margin. - For DL and UL not aligned at satellite (including aligned at gNB), A can be used to absorb TA margin and the minimum RTT on the feeder link. Proposal 3: Indication of parameter X and Y at least should be supported. - For DL and UL not aligned at satellite (including aligned at gNB), X and Y should be indicated and updated to capture the rapidly changed RTT on the feeder link. Proposal 4: TA margin should be absorbed in common TA configuration, and it should be transparent to the UE. Proposal 5: Confirm the working assumption that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension. Proposal 6: Enhanced TA command based TA determination mechanism for UE in RRC_CONNECTED should be supported, where, the enhanced TA command consists of (TA,δ,TA,drift, where, TA,δ indicates a delta TA, and TA,drift indicates a TA drift rate. Proposal 7: For i-th uplink transmission occasion at ti (ti≥tm'), NTAti can be determined as NTAti=NTAaccumm+TA,driftm∙ti-tm' where, - m is the last received enhanced TA command before ti. - NTAaccumm is the m-th accumulated TA command. - Note: When UE received the m-th enhanced TA command TA,δm,TA,driftm at tm', then it updates the m-th accumulated TA command NTAaccumm as NTAaccumm=TA,δ0m=0NTAaccumm-1+TA,driftm-1∙tm'-tm-1'+TA,δmm>0 Proposal 8: Support indication of DL frequency compensation for the service link Doppler, i.e., - If NR NTN gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of frequency pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. Proposal 9: If NR NTN gNB applies frequency post-compensation in UL, the gNB should broadcast a parameter giving the amount of frequency post-compensation, to achieve a common understanding between UE and gNB. This parameter should indicate the RX frequency offset at the satellite receiver relative to the nominal UE RX frequency of the service link. Proposal 10: Support both ephemeris formats based on satellite position and velocity state vectors (Option 1) and based on orbital elements (Option 2), if possible. Proposal 11: If down-selection is needed, ephemeris format based on satellite position and velocity state vectors (Option 1) should be supported for implicit compatibility to support HAPS and ATG scenarios. Proposal 12: Support broadcasting the position of a reference point of the feeder link with certain artificial bias. |
| R1-2102915 | ZTE | Proposal 1: The indication of common TA should always be assumed. Proposal 2: The signaling granularity of common TA can be chosen as. Proposal 3: Indication of common TA drift rate should be supported to assist TA adjustment. Proposal 4: Postpone the discussion on TA margin until RAN4 determines time synchronization requirements. Proposal 5: If TA margin is needed, it should be absorbed in common TA to save signaling. Proposal 6: In RRC_CONNECTED state, if the expression of applied TA determined as: , where - , is estimated by UE based on GNSS and indicated information. - , is the common TA drift rate indicated by BS, and is the interval between the time when is valid and current time. - , is indicated in MAC CE TA command. Proposal 7: The indication of frequency offset is sufficient to address issues for both DL and UL. Proposal 8: Ephemeris format based on instant state vectors with implicit time should be at least supported. |
| R1-2102986 | Xiaomi | Proposal 1: With consideration on the signalling overhead, the large granularity of NTA,common i.e. 2048 Tc is preferred. Proposal 2: The UE should assume that NTA,common=0, when common TA is not broadcasted by network. Proposal 3: NTA is controlled by RAR and TAC in MAC-CE as in Release-16. Proposal 4: Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode assume NTA =0. Proposal 5: The common timing drift rate indicated by network should be supported. Proposal 6: Pre-compensation value for DL frequency should be indicated by network. Proposal 7: The residual offset value of UL frequency at the reference point should be indicated by network. |
| R1-2103011 | Mitsubishi Electric RCE | Proposal 1: UEs using self-acquired timing advance for initial access signal the pre-compensation value NTA to the network during the initial access process. Proposal 2: In NTN, the network broadcasts a beam-specific default value NTA_default of the user-specific NTA value for initial access purposes. Proposal 3: Support network assistance indicating to the UE whether to skip timing advance acquisition during handover. |
| R1-2103033 | Intel Corporation | Proposal 1: • Support common TA drift rate indication - Alternatively, broadcast of reference point for pre-compensation of feeder link delay can be considered to avoid frequent reconfiguration Proposal 2: • Support DL frequency pre-compensation for the service link Doppler o Indication of the applied frequency pre-compensation value is required Proposal 3: • It is assumed by RAN1 that compensation of feeder link Doppler shift is done at the gNB side for UL/DL |
| R1-2103059 | Ericsson | Proposal 1 The reference point for time and frequency in an NTN should be under control of the network and should at least support the option of having gNB as the reference point. Proposal 2 The TA for NTN should use the legacy granularity of Tc units or a multiple of Tc units, i.e., the common TA component X should be placed within the brackets as follows: NTA+NTA,offset +X×Tc Proposal 3 The TA to be used by NTN UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states should be as follows: TTA=NTA+NTA,offset+NTA,UE-specific+NTA,common×Tc where: NTA and NTA,offset are defined as in Rel-16. NTA,UE-specific is UE-autonomous TA calculated based on the GNSS-acquired UE position and the serving satellite ephemeris to pre-compensate for the service link RTT. NTA,common is network-controlled common TA to compensate (e.g.) for feeder link RTT. Proposal 4 The UE calculates the UE-specific TA (in Tc units) as follows: NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the signal the UE is about to transmit Proposal 5 The network broadcasts parameters describing the common delay by a linear function as follows: Dcommont=Dcommon,base+(t-Tcommon,base)Dcommon,drift where: t is the time the signal passes the satellite Tcommon,base is a reference time of the broadcast common delay Dcommon, base is the common one-way delay at time Tcommon,base Dcommon, drift is the common one-way delay drift rate Proposal 6 The UE determines the common TA (in Tc units) as follows: NTA, common=Dcommontsat,DL+Dcommontsat,UL/Tc where tsat,DL is the time the DL signal to which the UE local time reference is synchronized was relayed by the satellite, and tsat,UL is the time the UL signal to be transmitted by the UE will be relayed by the satellite. Proposal 7 The characterization of the common delay should include drift rate information. Proposal 8 If gNB applies frequency pre-compensation in DL, the gNB should broadcast a parameter giving the amount of pre-compensation. This parameter should indicate the TX frequency offset at the satellite transmitter relative to the nominal DL TX frequency of the service link. Proposal 9 The gNB may broadcast a parameter giving an additional frequency shift that the UE should apply at transmission. The value of this parameter should be configurable. It may be used for compensating for the Doppler shift observed on the feeder link. Proposal 10 RAN1 should investigate the possibility of the solution where the gateway pre/post-compensates the feeder link Doppler shift in a way transparent to the gNB and UE. Proposal 11 Support broadcasting a reference point of the feeder link and UE autonomous determination of the time and frequency offset of both the service link and the link between the satellite and the reference point of the feeder link. Proposal 12 RAN1 to determine the relevance of the case of NTN coverage but no GNSS coverage. Proposal 13 RAN1 to study the required accuracy of satellite ephemeris to support timing and frequency offset pre-compensation. |
| R1-2103108 | Apple | Proposal 1: The TA used for Msg1/MsgA transmission is given by TA=NTA+NTA, UE-specific+NTA, common+NTA,offset×Tc, where - NTA, NTA,offset and Tc are defined as in Release-16 - NTA, UE-specific is UE self-estimated TA - NTA,common is network-controlled common TA if indicated by network, or is 0 if not indicated by network. Proposal 2: The broadcast common timing offset is in the unit of Tc⋅(16⋅64/2μ). Proposal 3: Consider common TA drift rate is broadcasted together with common TA. If common TA is not indicated, then the common TA drift rate is also not indicated. Proposal 4: The network-controlled common TA NTA, common is obtained by NTA, common=NTA,common, RX +DTA, common,RX ⋅Δt, where NTA,common, RX is the latest received common TA, DTA,common, RX is the latest received common TA drift rate and Δt is the time gap between the latest common TA reception and the corresponding uplink transmission, in the unit of Tc. Proposal 5: RAN1 to determine the necessity of specifying TA margin, depending on RAN4’s conclusion. Proposal 6: The requirement that the existing TAC 12-bit field in Msg2/MsgB is reused is that a UE pre-compensates an accurate UE specific TA and TA margin in its Msg1/MsgA transmission. Proposal 7: In RRC connected mode, UE updates its TA value using TA=NTA+N'TA, UE-specific+N'TA, common+NTA,offset×Tc, where - NTA=NTA,old+TA-31⋅16⋅64⋅2-μ is based on the updated TA command MAC CE - N'TA, UE-specific is based on the updated GNSS positioning signal or satellite ephemeris - N'TA, common=NTA,common, RX +DTA, common,RX ⋅Δt is based on the updated common TA (NTA,common, RX ) and common TA drift rate (DTA, common,RX ). Proposal 9: In downlink transmissions, support gNB pre-compensates and indicates a frequency offset for the service link Doppler shift with respect to a reference point. Proposal 10: UE only pre-compensates its estimated Doppler shift on the service link in its uplink transmissions, and additional network indication of frequency offset to be pre-compensated by UE is not supported. Proposal 11: Support both ephemeris formats, i.e., state vectors and orbital elements. |
| R1-2103169 | Qualcomm Incorporated | Proposal 1: - In NTN, the network may broadcast a common timing offset value with granularity of one slot assuming SSB subcarrier spacing. - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: TA= NTA+NTA, offset×Tc+X where X is derived from the common timing offset. Proposal 2 : Send an LS to RAN4 to inform RAN1’s decision on timing control and to enquire details of combination of open and closed-loop TA. Proposal 3 : Support closed-loop frequency control commands by MAC-CE. Proposal 4 : Consider group-common DCI for UL time and frequency control. Proposal 5 : Support SRS with multiple coherent symbols with configurable gaps. |
| R1-2103242 | Samsung | Proposal 1: A gNB signals the serving satellite ephemeris to UEs in system information, including the followings: - index to a pre-defined table of satellite altitude levels and altitude offset scaling factors, i.e., NTN type - satellite altitude offset - satellite position - satellite velocity - reference time for satellite position and velocity. Proposal 2: Support Ephemeris format based on satellite position and velocity state vectors. Proposal 3: The TA update with TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc is used. Proposal 4: UE’s estimated TA value is reported to gNB. Proposal 5: A gNB signals residual common TA value to UEs such that UEs can derive common TA by adding to minimum common TA value, which can be obtained by UE from the satellite ephemeris (or altitude) information. Proposal 6: Multiple reference points and common TA values should be considered for extremely large cells. Proposal 7: The gNB signals common TA drift rate to enable autonomous TA update at UE. Proposal 8: The gNB can jointly signal common TA drift rate and Doppler shift such as the UE derives Doppler shift from common TA drift rate signaled by gNB or vice versa. Proposal 9: The gNB indicates the additional UL frequency offset value. |
| R1-2103277 | InterDigital, Inc. | Proposal 1: X is expressed as the legacy granularity of Tc unit. Proposal 2: a PRACH resource set is configured per distance group from the UE to satellite and a UE determine a PRACH resource set for Msg1 transmission based on the distance group the UE belongs to. Proposal 3: For ephemeris format, both formats are supported. Proposal 4: Option 1 is used for GEO deployment and Option 2 is used for LEO deployment. |
| R1-2103305 | Sony | Proposal 1: In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs. Proposal 2: RAN1 should consider indicating the time at which a closed loop TA was calculated to the UE. Proposal 3: RAN1 should support the signalling of timing drift rate information to the UEs in a beam specific manner. Proposal 4: UE should update the common TA with common timing drift rate when UE transmits uplink data. |
| R1-2103533 | Lenovo, Motorola Mobility | Proposal 1: Support the granularity of NTA,common same as granularity of TA command. Proposal 2: TA margin indication is not supported. Proposal 3: Support indication of common timing offset drift rate. |
| R1-2103544 | BUPT | Proposal 1: It is necessary to compensate for doppler code deviation in low orbit satellite system. Proposal 2: Configuration of small bandwidth and large subcarrier spacing should be considered to weaken the effect of doppler code deviation. Proposal 3: Make further study on whether timing and frequency synchronization schemes in terrestrial network can eliminate doppler code deviation. |
| R1-2103579 | NTT DOCOMO, INC. | Proposal 1: Common timing drift rate can be indicated to users to cover all the foreseen implementations and scenarios. Proposal 2: Common TA margin and user-specific TA margin should be considered separately. Proposal 3: Common TA margin can be included in the common TA. The indication of common TA margin is not necessary. Proposal 4: UE capability on user-specific TA self-estimation/pre-compensation can be defined to indicate the self-estimation accuracy. Proposal 5: One or multiple common TAs can be broadcasted for UEs with different UE capabilities on TA self-estimation/pre-compensation accuracy. Proposal 6: Two options can be both supported with different indication accuracy and periodicity. - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats - FFS: Details on time reference provisioning/format |
| R1-2103620 | LG Electronics | Proposal 1. The option of UE-specific TA calculation based on the time difference between the reference time provided by network and the reference time acquired by the GNSS is not supported in Rel-17 NTN. Proposal 2. Prioritize NTN designs that the reference point is located at the gNB. (i.e., DL timing and UL timing are aligned at the gNB) Proposal 3. Support the latest updated proposal 1-1 in R1-2102215 [3] Proposal 4. Support additional signaling by the network in order to apply proper common timing offset according to time changes at UE side. Potential solutions can include - Alt 1) providing the reference time corresponding to common timing offset - Alt 2) providing series of common timing offset. Proposal 5. Within pre-defined set of TA offsets, the TA offset can be provided by gNB via higher layer signing (e.g., SIB or dedicated RRC signaling). - The TA offset can be independently corresponding to different ROs (or RO groups) Proposal 6. Support implicit reporting of TA estimated by the UE. - The different TA (or the range of TA) can be mapped to different ROs (or RO groups). Proposal 7. It is desirable to support the TA indication from gNB, because the accuracy of the GNSS can be degraded in specific scenario such as indoor, underground, etc. Proposal 8. It can be considered that the common TA is calculated based on K_offset in case when the UE cannot acquire the accurate TA. Proposal 9. Regarding TA command in RAR, support enhancement approaches to cover large cell coverage. - Increase the step size of TA command field in RAR. - Support multiple reference points. Proposal 10. - At least for the case when the UE is in RRC_IDLE and/or RRC_INACTIVE states, it is reasonable to provide the additional information via semi-static signaling. - In case when the UE is in RRC_CONNECTED states, it can be considered that the information is provided by dynamic signaling. Proposal 11. RAN1 should discuss how to update and/or report the UE specific TA in case when the NTN UE is in RRC_CONNECTED states. Proposal 12. Regarding satellite ephemeris format, - Support one option to reduce both specification work and UE implementation complexity. - If two options need to be supported, only one option should be mandatory feature. Proposal 13. Support validity timing window for satellite ephemeris information in Rel-17 NTN. |
| R1-2103655 | Fraunhofer IIS, Fraunhofer HHI | Proposal 1: RAN1 to strive for a unified design of common RTT/delay signaling in order to avoid duplicate signaling of common RTT/delay. Proposal 2: The characteristic of the common RTT/delay (or feeder link RTT/delay) shall be taken into account for the design of its corresponding signaling. Proposal 3: RAN 1 should agree on studying common delay signaling design with reduced signaling overhead. Proposal 4: RAN1 to consider the signaling of the parameter set NTA,scale, drift scale rate, NTA,cons, a constant, and NTA,power, an exponent (can take integer values), for signaling of the common RTT/delay in NTN. Proposal 5: RAN1 to consider UE autonomous drift rate calculation for common RTT/delay signaling. |
| R1-2103670 | Nokia, Nokia Shanghai Bell | Proposal 1: Any UE should only attempt to access the 5G system over NTN for situations where it is absolutely sure that proper time and frequency compensation is applied. Proposal 2: Having the time reference point at the satellite should be de-prioritized as it adds complexity and overhead to the gBN and creates unnecessary specification effort. Proposal 3: The gNB is the default time reference point of the system. Proposal 4: The cyclic prefix of the random access preamble must be able to cover the aggregate contribution of all sources of time inaccuracy and multipath propagation delays. Proposal 5: The GNSS-assisted pre-compensation solution used by the UE shall meet the demands of the preamble format chosen by the operator. The UE shall ensure that requirements in TA adjustment and frequency pre-compensation for all preamble formats are met at any time. Proposal 6: Self-estimated UE-specific TA in RRC idle or inactive mode based on GNSS-provided time reference in conjunction with the time provided by referenceTimeInfo-R16 is independent on satellite ephemeris and should be standardized as well. Proposal 7: There is no need to indicate a TA margin. Any uncertainty related to TA should be covered by the common TA value and CP of random access preamble. Proposal 8: It should be left up to the gNB to decide whether to broadcast the TA drift value or not. Proposal 9: Network must be in control of the timing advance updates applied at the UE in RRC connected mode. Proposal 10: Closed-loop TA control in RRC connected mode is preferable to ensure stability of the TA control loop. Proposal 11: Open loop TA control in RRC connected mode should be applied only in a way that does not impact the closed loop TA control messages. Proposal 12: For RRC connected mode, self-estimated UE-specific TA estimation based on GNSS-provided time reference and the time provided by referenceTimeInfo-R16 is a beneficial solution and should be standardized as well. Proposal 13: The location and determination of the frequency reference point must be agreed before developing further solutions for open technical aspects in NTN standardization. Proposal 14: If the frequency reference point is at the satellite, the satellite is responsible for pre-compensating the feeder link Doppler shifts applicable for both uplink and downlink. Proposal 15: The satellite has the responsibility of compensating the feeder link Doppler shift in DL and if needed in UL. The feeder link should be seen as an ideal link without any Doppler shift by the gNB. Proposal 16: The reference point for frequency should be under control of the network. In case the frequency reference point is not at the gNB, the satellite is responsible for compensating the feeder link Doppler shift in the uplink and downlink. Proposal 17: In the downlink a common frequency offset on service link is pre-compensated to limit the UE search space for the synchronization signals. Proposal 18: The amount of common frequency pre-compensation in DL in a cell may be indicated to the UE and thereby be used for determining the amount of UL frequency pre-compensation. Proposal 19: UE frequency adjustment based on GNSS-acquired frequency reference, DL signals and the time provided by referenceTimeInfo-R16 has benefits and should be standardized as well. Proposal 20: Closed-loop control in RRC connected mode can be applied for UE frequency alignment in UL. The UE must follow commands provided by the gNB and ensure stability with any open-loop frequency alignment. |
| R1-2103687 | Sequans Communications | Proposal 1: For common TA broadcast design, RAN1 shall consider drift rate accuracy and signalling overhead aspects. Proposal 2: If TA margin is introduced in NR NTN, its indication should be absorbed within common TA signalling. Proposal 3: GNSS-specific measurement gaps should be defined to accommodate various UE design options for NR NTN. Proposal 4: RAN1 to consider indication to UE regarding frequency pre-compensation performed at neighbouring cells. Proposal 5: RAN1 to discuss signaling overhead and frequency update of satellite ephemeris broadcast design, considering altogether the serving satellite, constellation, and neigbouring cells’ ephemeris data. |
| R1-2103731 | CEWiT, IITM, IITH | Proposal 1: The final equation for the full TA at UE should be, TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where:  NTA and NTA,offset are defined as in Release-16.  NTA,UE-specific is UE self-estimated TA  NTA,common is network-controlled common TA and may include any timing offset considered necessary by the network. Proposal 2: gNB broadcast the common TA value in the NTN specific SIB message along with other NTN specific broadcast messages. Proposal 3: In NTN, the network may broadcast a common timing drift rate and update duration to update the common TA by UE in periodic interval. It can be broadcasted in the NTN specific SIB. Proposal 4: TA margin should be configured to the UE directly or indirectly to control the uncertainty in the full TA estimation at the UE. Full TA equation including TA margin will be TA=NTA+NTA,UE-specific+NTA,common+NTA,offset-NTA, margin ×Tc. Proposal 5: TA margin can be configured indirectly as faction or multiple of the CP of the configured PRACH. NTA, margin=Y* CPPRACH. FFS Y value. Proposal 6: Agree the working assumption on TAC 12-bit field reuse. Proposal 7: gNB should provide the set of instructions to refine the TA estimated by the UE for better control of the gNB over UE specific TA estimation. Proposal 8: UE should report the applied TA to the gNB for better control over UE’s behaviour. Proposal 9: In UE specific TA estimation, TA value estimated by UE is quantized using predefined step size to get the final TA. The step size can be the TA margin configured to the UE. Proposal 10: UE will report the applied TA to the gNB in terms of number of steps used in the quantization of TA. Proposal 11: gNB can provide the TAmin value to the UE per beam/cell where TAmin is the minimum timing advance UE can experience in the beam/cell. Proposal 12: UE will correct the TA in connected mode using velocity information of satellite apart from the MAC-CE TA based update. Proposal 13: In connected mode, combination of open and close loop TA update should be adopted. New TA value update equation will be, TA=NTA,old ± ∆NTA+Ta-31.16.642μ )×Tc ×Tc . Proposal 14: The ∆NTA will be determined by UE using estimated drift value and additional drift provided by gNB. |