3GPP TSG- RAN WG1 Meeting #104b-e                                                 R1-2104145

# e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.6

Source:	Moderator (Apple Inc.)

Title:	Summary on 104bis-e-NR-eMIMO-02

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide a summary on 104bis-e-NR-eMIMO-02. During the discussion, the following conclusion and TP for editorial change is agreed.

Conclusion For a CSI report with CSI -ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI -RS-ResourceSet with higher layer parameter trs-Info not configured, it is up to UE implementation how to use the configured measurement resources for its Rx beam refinement. For the editors: The editorial spec changes captured in R1-2103991 are provided to improve clarity of RAN1 specifications. Please consider them in the next specification revision.

# MB.12

In R1-2103402, Huawei/HiSilicon proposes one TP to define the CPU occupation for L1-SINR computation when reportQuantity is set to “none”. Based on the discussion in preparation phase, there could be two alternatives. It looks Alt2 does not require a TP.

Proposal for discussion: Down-select one of the following alterantives for UE behavior when reportQuantity is set to “none” and higher layer parameter trs-Info is not configured:

- Alt1: UE can measure L1-SINR based on the CSI-RS/CSI-IM/SSB for channel or interference measurement

- Alt2: UE only measures L1-RSRP based on CSI-RS/SSB for channel measurement

TP for Atl-1 (38.214)

#### 5.2.1.6 CSI processing criteria #### < Unchanged parts are omitted > For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, or periodic or semi-persistent CSI-RS/CSI-IM/SSB resource for channel or interference measurement for L1-SINR computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation or periodic or semi-persistent CSI-RS/CSI-IM/SSB resource for channel or interference measurement for L1-SINR computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation or each CSI-RS/CSI-IM/SSB resource for channel or interference measurement for L1-SINR computation. where (Z3,Z3') are defined in the table 5.4-2. #### < Unchanged parts are omitted >

Companies’ views and comments

| Company | View |
| --- | --- |
| Apple | It seems Alt1 is unclear whether UE should measure the RS based on L1-RSRP or L1-SINR, especially for CMR only case. Slightly prefer Alt2. |
| vivo | Prefer Option1, at least for the case when more than one resource settings are configured. UE would assume L1-RSRP based measurement if only one resource setting configured. |
| Ericsson | Prefer Alt2 without spec change. The selection of measurement is completely up to the UE. |
| Intel | Slightly prefer Alt 1, but also Ok with Alt 2 |
| Samsung | Support Alt2. |
| ZTE | Support Alt2 as our first preference. If going with Alt-1, we need to study how/why to support the case that reportQuantity is set to “none” and higher layer parameter trs-Info is not configured |
| OPPO | Support Alt2. |
| LG | Alt2 seems better. After checking the spec, BM IMR can be configured for L1-SINR report only, meaning that IMR cannot be configured when reportQuantity set to 'none' to our understanding. The text “for L1-RSRP computation” in current spec might create some confusion and could be deleted. |
| Huawei, HiSilicon | In our understanding, the current specs allow the gNB to configure both CMR and IMR, with reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured. In this particular case, our 1st preference is go with Alt-1 above, i.e., UE performs beam measurement/refinement, taken interference into account. After reading the comments, we agree with LG that “for L1-RSRP” can be removed from current specs, which leaves more implementation flexibility to UE. Still, when the IMRs are configured, we suggest including them in terms of CPU occupation. It is still up to UE to perform L1-RSRP or L1-SINR measurement, but the gNB would know how to assume UE CPU occupation timeline. To sum up, we suggest considering the following revised TP: #### < Unchanged parts are omitted > For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel or interference measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel or interference measurement for L1-RSRP computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel or interference measurement for L1-RSRP computation. where (Z3,Z3') are defined in the table 5.4-2. #### < Unchanged parts are omitted > |
| Qualcomm | Support Alt.2, i.e. same behavior as R15 when report Quantity is none. |

# Editorial CR

The editorial CRs for 38.211 and 38.213 are provided at:

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_104b-e/Inbox/drafts/7.2.6/email#2

Companies’ views and comments

| Company | View |
| --- | --- |
| vivo | Supportive |
| Ericsson | Support |
| Intel | Agree with editorial CRs |
| Samsung | Support |
| ZTE | Support. BTW, if our understanding is correct, as per Mr. Chairman guidance, we do NOT need any individual CR for these editorial issues, right? Just like TPs for information to editors. Couple of comments: - Although I understand that everyone is fine with the conclusion, RAN1#104bis-e has not yet started. In order to take something as a RAN1 conclusion, we can only do it next week. - For the editorial corrections, if I don’t see any concerns on the proposed changes next week, I will just capture the proposed correction in my notes for editors. I merged all the editorial corrections into Yushu’s email thread. |
| CATT | Support |
| LG | Support |
| Huawei, HiSilicon | Support the CRs. |
| Qualcomm | Support |