3GPP TSG RAN WG1 Meeting #104bis-e	R1-21xxxxx

E-Meeting, April 12th – April 20th, 2021

Agenda Item:	6.1

Source:	Moderator (ZTE)

Title:	Summary of email discussion on [104b-e-LTE-6.1CRs-01]

Document for:	Discussion and Decision

Introduction

This contribution provides discussion on clarification related to Type1A-MPDCCH common search space and Type2A-MPDCCH common search space for LTE-M:

[104b-e-LTE-6.1CRs-01] Email discussion/approval on R1-2102846 and R1-2102847 by April-15 - Youjun (ZTE)

Discussion

Issue 1: Correction on MPDCCH assignment procedure [1]

| Reason for change: | The value for determining the CCEs for any PDCCH candidate of Type1A-MPDCCH common search space and Type2A-MPDCCH common search space is missing. |
| --- | --- |
| Summary of change: | Add the definition of for Type1A-MPDCCH common search space and Type2A-MPDCCH common search space in the spec. |
| Consequences if not approved: | The CCEs for Type1A-MPDCCH common search space and Type2A-MPDCCH common search space can not be obtained. |

<Changes to TS 36.213>

<Unchanged parts are omitted>

    - 9.1.5	MPDCCH assignment procedure

<Unchanged parts are omitted>

For aggregation level  or  ECCEs, the number of ECCEs refers to the MPDCCH mapping to the REs of the 2+4 PRB set as defined in [3]. An MPDCCH search space at aggregation level  and repetition level is defined by a set of MPDCCH candidates where each candidate is repeated in a set of  consecutive BL/CE downlink subframes starting with subframe . For an MPDCCH-PRB-set , the ECCEs corresponding to MPDCCH candidate m of the search space  are given by

where

,

is the number of MPDCCH candidates to monitor at aggregation level in MPDCCH-PRB-set  in each subframe in the set of  consecutive subframes.

for MPDCCH UE-specific search space is determined as described in Clause 9.1.4, and for Type0-MPDCCH common search space, Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space.

For , if subframe is a special subframe that does not support MPDCCH according to table 6.8B.1-1 in [3], the UE shall calculate  by assuming for normal cyclic prefix and for extended cyclic prefix.

<Unchanged parts are omitted>

Please provide your comments for the proposed CR:

| Companies | Comments |
| --- | --- |

Issue 2: Correction on PDSCH dropping for multicast in special subframe [2]

| Reason for change: | Multicast is supported for TDD and FDD in Release 14. For frame structure type 2, MPDCCH and PDSCH transmission in the special subframe would be dropped. The PDSCH transmission by a BL/CE UE can also be scheduled from Type1A-MPDCCH common search space and Type2A-MPDCCH common search space. However, if the PDSCH is scheduled from Type1A-MPDCCH common search space or Type2A-MPDCCH common search, corresponding PDSCH dropping in the special subframe is missed in the spec. |
| --- | --- |
| Summary of change: | Add the description that a BL/CE UE shall assume PDSCH is dropped in a special subframe when the PDSCH is scheduled from Type1A-MPDCCH common search space or Type2A-MPDCCH common search space. |
| Consequences if not approved: | The PDSCH transmission behavior, when PDSCH is scheduled from Type1A-MPDCCH common search space and Type2A-MPDCCH common search space in special subframe, is not consistent with that scheduled from other search spaces. |

<Changes to TS 36.213>

    - 7.1.7	Modulation order and transport block size determination

<Unchanged parts are omitted>

For frame structure type 2, a BL/CE UE shall assume PDSCH is dropped in a special subframe considered as BL/CE DL subframe according to Clause 6.8B.1 of [3] in the following cases

-	for PDSCH scheduled from UE-specific search space, Type0-MPDCCH common search space, Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space or Type2A-MPDCCH common search space, if an MPDCCH belonging to the corresponding search space is dropped in the special subframe according to clause 9.1.5.

-	if PDSCH carries SI messages.

<Unchanged parts are omitted>

Please provide your comments for the proposed CR:

| Companies | Comments |
| --- | --- |

Summary

- References

- R1-2102846	Correction on MPDCCH assignment procedure	 ZTE,Sanechips

- R1-2102847	Correction on PDSCH dropping for multicast in special subframe	ZTE,Sanechips