3GPP TSG-RAN WG1 Meeting #104b-e											R1-210xxxx

e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			Summary of email discussion for [104-e-NR-2Step-RACH-01]

Document for:	Discussion

# Introduction

This document is intended to address the following corrections for 2-step RACH by email discussion.

[104b-e-NR-2Step-RACH-01] Email discussion/approval of the corrections of 2-step RACH related issues till 4/15 (Li, ZTE)

- CR in R1-2103403, CR in R1-2103495 and two TPs in R1-2103680

# Correction on the configuration of RACH-related power control parameters

In R1-2103403, it is mentioned that when both 2-step RACH and 4-step RACH are configured, and ΔMsgA_PUSCH is equal to ΔPREAMBLE_Msg3, there could be possible misunderstanding that msgA-Alpha will be used for both 2-step RACH and 4-step RACH, which is inconsistent with the agreement.

Proposal 1:

- Adopt the following TP#1 in 38.213, to avoid the ambiguity on the conditions for the configuration of power control parameters.

Reasons for change The msgA-Alpha and msg3-Alpha should be used for the PUSCH transmission of 2-step RACH and 4-step RACH, respectively. However, according to current TS 38.213, when both 2-step RACH and 4-step RACH are configured, and ΔMsgA_PUSCH is equal to ΔPREAMBLE_Msg3, msgA-Alpha will be used for both 2-step RACH and 4-step RACH. Summary of changes Change the conditions for the configuration of power control parameters msgA-Alpha and msg3-Alpha. Consequences if not approved: Use the wrong power control parameter for PUSCH transmission during RACH procedure Specs/Sections impacted TS 38.213, Section 7.1.1 ------------------------- Start of Text Proposal #1 for TS 38.213 ---------------------------- # 7 Uplink Power control ## 7.1 Physical uplink shared channel ### 7.1.1 UE behaviour *** Unchanged text is omitted *** - For - For , - if PO_NOMINAL_PUSCH,f,c0 is determined by =PO_PRE +ΔMsgA_PUSCHand ΔMsgA_PUSCH, and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif - PO_NOMINAL_PUSCH,f,c0 is determined by =PO_PRE+ΔPREAMBLE_Msg3 and ΔPREAMBLE_Msg3, or msgA-Alpha is not provided, and - msg3-Alpha is provided, - is the value of msg3-Alpha - else, *** Unchanged text is omitted *** ------------------------- End of Text Proposal #1-------------------------------

## Comments to proposal 1

| Company | Comments |
| --- | --- |
| Samsung (prep phase) | In the early part of section 7.1.1, there is already conditions for setting PO_NOMINAL_PUSCH,f,c0, and the change of order of msg3-alpha did not change the behavior comparing with old version. The “elseif” part was saying when msgA-alpha is not configured, and msg3-alpha, we will use msg3-alpha. It seems no difference comparing what we have now. |
| Huawei (prep phase) | A quick response to Samsung: the interpretation of “=” in the equation of concerned texts may be misleading, in the sense that “=” only cares about the final results so that when the two delta values are the same then there could be the case that the “if” and “elseif” condition does not differentiate. So the intention is just to make it clear that the behavior is to check how PONOMINALPUSCH,f,c0 is determined by, rather than what value PONOMINALPUSCH,f,c0 is. |
| Ericsson | Agree with the intention of the CR. However, to make it clear, it seems we do not even need to mention PONOMINALPUSCH,f,c0 to determine which alpha is used. Thus, we propose to use following TP, TP#1-a instead: ------------------------- Start of Text Proposal #1-a ------------------------------- *** Unchanged text is omitted *** - For - For , - if a Type-2 random access procedure is used, as described in Clause 8, if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif a Type-1 random access procedure is used, as described in Clause 8,PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, is the value of msg3-Alpha - else, *** Unchanged text is omitted *** ----------------------------- End of Text Proposal #1-a ------------------------------- |
| ZTE | We understand the intention of the TP, but it seems hard to have misunderstanding for UE implementation. If the majority think it is a critical issue, we slightly prefer Ericsson’s version than the original TP because it may cause further ambiguity to UE implementation on how to understand the wording of “determined by”. |
| Qualcomm | We prefer the following TP for clarity: ------------------------- Start of Text Proposal #1-b ------------------------------- *** Unchanged text is omitted *** - For - For , - if a Type-2 random access procedure is used, if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided: αb,f,c(0) is the value of msgA-Alpha, and PO_NOMINAL_PUSCH,f,c0 is determined by =PO_PRE +ΔMsgA_PUSCHand ΔMsgA_PUSCH; - elseif a Type-1 random access procedure is used (0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided: is the value of msg3-Alpha, and PO_NOMINAL_PUSCH,f,c0 is determined by =PO_PRE+ΔPREAMBLE_Msg3 and ΔPREAMBLE_Msg3. - else, *** Unchanged text is omitted *** ----------------------------- End of Text Proposal #1-b------------------------------- We are also ok with TP #1a proposed by Ericsson |
| CATT | In our understanding, the description in current TS 38.213 (g50) is clear and proposed TP is unnecessary. |
| Apple | According to our understanding, the current spec is clear. It seems the CR is not essential correction and not needed. |
| Samsung | We understand the intention of the CR, however, it’s really not practical that such ambiguity could happen based on the mathematical value being “accidently” same. So we prefer current version without change. Besides, the suggested change by CR is not proper as explained by ZTE; the suggested change from E/// or QC are also not proper because the conditions on using Type-1 or Type-2 RA is not exactly same as the original equation, i.e., UE uses Type 2-RA but goes to fallbackRAR, original it should use msg3-Alpha, then by the suggested change, UE should use msgA-Alpha. |
| Intel | We understand the intention of the TP in principle. We slightly prefer the proposal from QC to make it clear. |
| Nokia, Nokia Shanghai Bell | We prefer current version of specifications without changes. |
| Huawei | Ok with the TP or those from E// or QC |
| Spreadtrum | In our understanding, no change is OK. But clear spec description is slightly preferred. Thus, we are fine with the revision from Ericsson or Qualcomm. |
| vivo | We understand the CR is to clarify the conditions for msgA-Alpha and msg3-Alpha to be used. To our understanding, there is no ambiguity in current spec so we prfer no change to the spec. Having said that, we are also fine if majority view is to introduce the CR for better clarification. |
| Huawei2 | The issue Samsung mentioned about E// or QC TP exists. So we prefer our original TP. |

# Editorial corrections on the DMRS description for MsgA

R1-2103495 identified a few editorial issues on the DMRS description for MsgA, copied as follows.

Proposal 2:

- Adopt the following TP#2 in 38.211, to correct the editorial issues of the DMRS description of MsgA.

Reasons for change Some typos and copy-paste errors were found in the latest specification. Summary of changes Editorial corrections for the description of DMRS configurations for MsgA. Consequences if not approved: Cause ambiguity in understanding. Specs/Sections impacted TS 38.211, Section 6.4.1.1.3 -----------------------Start of Text Proposal #2 for TS 38.211 ---------------------------- # 6.4.1.1.3 Precoding and mapping to physical resources < Unchanged parts are omitted > For PUSCH mapping type A, - the case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'; - ld=4 symbols in Table 6.4.1.1.3-4 is only applicable when dmrs-TypeA-Position is equal to 'pos2'. For msgA transmitted using PUSCH mapping type A, - the case msgA-DMRS-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'; - 'dmrs-AdditionalPosition' in Tables Tables 6.4.1.1.3-3 to 6.4.1.1.3-6 shall be replaced by msgA-DMRS-AdditionalPosition; - only PUSCH DM-RS configuration type 1 is supported. For msgA transmitted using PUSCH mapping type B, - 'dmrs-AdditionalPosition' in Tables 6.4.1.1.3-3 to 6.4.1.1.3-6 shall be replaced by msgA-DMRS-AdditionalPosition; - only PUSCH DM-RS configuration type 1 is supported. < Unchanged parts are omitted > -------------------------End of Text proposal #2 ----------------------------

## Comments to proposal 2

| Company | Comment |
| --- | --- |
| Ericsson | Agree with the 3rd update to remove the 2nd “tables”. However, the first and 2nd updates are not necessary since “equal” can be an adjective here. |
| ZTE | Respond to Ericsson: The first and 2nd updates are consistent with the description of DL in the spec, e.g. In section 7.4.1.1.2 For PDSCH mapping type A - the case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'; |
| Qualcomm | OK with TP#2 |
| CATT | We are fine with FL proposal. |
| Apple | We are ok with FL proposal. |
| Samsung | Seems fine. |
| Intel | We are fine with the TP. |
| Nokia, Nokia Shanghai Bell | We are OK with FL proposal. |
| Ericsson2 | Thanks for the clarification from Li which look fine then. |
| Huawei | Agree |
| Spreadtrum | Fine with FL’s proposal |
| vivo | Agree with the proposal |

# Determination of power control parameter in case of 2-step RACH only operation

In R1-2103680, it was mentioned that in NR release 15 and 16, one or multiple sets of p0 and alpha values may or may not be configured for PUSCH power control. When it is not configured, the alpha and P0-nomilal for msg3 are supposed to be used for the power calculation of a nomal PUSCH according to the definition of p0-AlphaSets in RRC specification below.

p0-AlphaSets configuration {p0-pusch, alpha} sets for PUSCH (except msg3), i.e., {{p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3 PUSCH, P0-UE is set to 0 and alpha is set according to msg3-Alpha configured for msg3 PUSCH.

However, msg3 will be not supported when only 2-step RACH is configured, i.e. when 4-step RACH is not configured. In this case, it is not clear which alpha or nominal p0 configuration should be used for a normal PUSCH transmission.

To solve this issue, when p0-AlphaSets is not provided in case of 2-step RACH only operation, for power control of a normal PUSCH other than MsgA PUSCH or Msg3 PUSCH, P0-nominal and alpha for msgA PUSCH are used.

Proposal 3:

- In case of 2-step RACH only operation, when p0-AlphaSets is not provided, for power control of normal PUSCH, P0-nominal and alpha for msgA PUSCH are used, according to TP#3.

Reason for change: Msg3 will be not supported when only 2-step RACH is configured, i.e. when 4-step RACH is not configured. In this case, it is not clear which alpha or nominal p0 configuration should be used for a normal PUSCH transmission when p0-AlphaSets is not provided. Summary of change: In case of 2-step RACH only operation, when p0-AlphaSets is not provided, for power control of normal PUSCH, P0-nominal and alpha for msgA PUSCH are used. Consequences if not approved: It is not clear which alpha or nominal p0 configuration should be used for a normal PUSCH transmission Clauses affected: TS38.213, Section 7.1.1 -----------------------Start of Text Proposal #3 for TS 38.213 ---------------------------- 7.1.1 UE behaviour *** unchanged text omitted*** - is a parameter composed of the sum of a component and a component where . - If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in Clause 8.3, , , and , where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, , is provided by p0-NominalWithoutGrant, or if p0-NominalWithoutGrant is not provided, and is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a value, applicable for all , is provided by p0-NominalWithGrant, or if p0-NominalWithGrant is not provided, for each carrier of serving cell and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the DCI format also includes an open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1', the UE determines a value of from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value. - If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of from - a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00' - a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '1' or '01' - a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '10' - else, the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For - For , - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, is the value of msg3-Alpha - else, - For , is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a set of values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For j=1 or j∈SJ, if P0-PUSCH-AlphaSet is not configured, the UE uses the P0-nominal and msg3-Alpha configured for msg3 PUSCH if a Type-1 random access is configured for the BWP or uses the P0-nominal and msgA-Alpha for msgA PUSCH if a Type-1 random access procedure is not configured for the BWP. *** unchanged text omitted*** -----------------------End of Text Proposal #3 ----------------------------

## Comments to proposal 3

| Company | Comment |
| --- | --- |
| Ericsson | Agree and one minor comment on the text for the cover page is provided above. |
| ZTE | The TP introduce a new behavior which is not supported by the previous agreement. To our understanding, the current specification can work without the TP or without considering the 2-step RACH configuration, i.e. if 4-step RACH is not configured and 2-step RACH is configured for a BWP, the behavior should be the same as that both 4-step RACH and 2-step RACH are not configured for a BWP. |
| Qualcomm | Agree with the TP in principle. Prefer a better wording of the TP for clarity. The conditions that UE uses P0-nominal and msg3-Alpha configured for msg3 PUSCH should include: - Type-1 random access is configured for the BWP - Type-1 random access procedure is selected by UE Similarly, the conditions that UE uses the P0-nominal and msgA-Alpha for msgA PUSCH should be: - Type-2 random access is configured for the BWP Type-2 random access procedure is selected by UE |
| CATT | We have the same view with ZTE. If 2-step RACH is only configured for a BWP, UE behavior should be the same as that both 4-step RACH and 2-step RACH are not configured for a BWP. |
| Apple | In general, we are ok with this proposal. For updates on 4-step RACH part, it maybe not needed which is covered by 38.331. Otherwise, a Rel.15 CR maybe needed as well. For the 2-step RACH only case, according to our understanding, this case was already supported by RAN2 spec, 2-step RACH can work standalone. |
| Samsung | As similar view as ZTE and our comments in proposal 4, In conventional operation without 2step RACH, there could be the case that a BWP has no 4step RACH, so this issue is regardless of 2step RACH, so an UE behavior has already been implemented, UE just follows that same behavior is fine. We did not need to additionally introduce new function/behavior for power control parameter determination due to 2step RACH. |
| Intel | We share similar view as other companies that this TP is not necessary as this is a new behavior which was not agreed before. |
| Nokia, Nokia Shanghai Bell | Agree with ZTE, CATT, Samsung and Intel. We should not add new functionality for 2-step RACH procedure. |
| Ericsson2 | Try to clarify that our intention is not to introduce new functionality, the issue is that in case of 2-step RACH only operation and when p0-AlphaSets is not provided, current specification assumes 4-step RACH only in 38.331, which doesn’t cover 2-step RACH only case. All MsgA PUSCH related and 2-step RACH related text are missed in the specification when describing p0-AlphaSets. p0-AlphaSets configuration {p0-pusch, alpha} sets for PUSCH (except msg3), i.e., {{p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3 PUSCH, P0-UE is set to 0 and alpha is set according to msg3-Alpha configured for msg3 PUSCH.. Although the preferred way is to allow normal PUSCH to use the default configuration that is used by MsgA PUSCH in 2-step RACH only operation which was reflected in our original TP. We’re also fine to simply say the default alpha for a normal PUSCH should be always 1 and default P0-nominal is always PO_NOMINAL_PUSCH,f,c(0) in case of 2-step RACH only operation if this is pursued by all companies. I.e. following TP #3a is (and probably CR in RAN2 is needed as well and an LS can be sent to RAN2 on this) -----------------------Start of Text Proposal #3a ---------------------------- - If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For j=1 or j∈SJ, if P0-PUSCH-AlphaSet is not configured, the UE uses the P0-nominal and msg3-Alpha configured for msg3 PUSCH if a Type-1 random access is configured for the BWP or uses P0-nominal equal to PO_NOMINAL_PUSCH,f,c(0) and Alpha equal to 1 if a Type-1 random access procedure is not configured for the BWP. *** unchanged text omitted*** -----------------------End of Text Proposal #3a ---------------------------- |
| vivo | We share the same view as ZTE. The TP is not needed. |
| Huawei | To understand, according to the part as below whether the case has been specified somewhere or? - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgADeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgADeltaPreamble is not provided, for carrier f of serving cell c |
| Ericsson3 | @yi, the text you copied is for the MsgA PUSCH (j=0) power control, this TP here is to determine the alpha and p0-nominal for a normal PUSCH when dedicated P0-PUSCH-AlphaSet is not provided and when only 2-step RACH is configured. According to comments from companies, it seems we prefer that “alpha should be 1 and p0-nominal should be the PO_NOMINAL_PUSCH,f,c0 in this case, but this is not reflected in the current specification since only RRC tells how the alpha/p0-nominal are determined when P0-PUSCH-AlphaSet are not configured for j=1and j>1 case but RAN2 didn’t update the text below when introducing 2-step RACH feature (as msg3 PUSCH does not exist in 2-step RACH only case): p0-AlphaSets configuration {p0-pusch, alpha} sets for PUSCH (except msg3), i.e., {{p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3 PUSCH, P0-UE is set to 0 and alpha is set according to msg3-Alpha configured for msg3 PUSCH.. |

# Determination of PUSCH waveform in case of 2-step RACH only operation

In R1-2103680, it was also mentioned that the transform precoder is enabled or disabled based on the transformPrecoder configured in ConfiguredGrantConfig for CG based PUSCH transmission or based on the transformPrecoder configured in pusch-Config for DG based PUSCH transmission. When the parameter transformPrecoder is not configured, the configuration for msg3 is used.

transformPrecoder The UE specific selection of transformer precoder for PUSCH (see TS 38.214 [19], clause 6.1.3). When the field is absent the UE applies the value of the field msg3-transformPrecoder.

However in 2-step RACH only operation, there will be no msg3 configuration, and if the transformPrecoder field is absent in the CG PUSCH configuration, the waveform of the CG based PUSCH is not known. This will also make it not clear on how to determine the DRMS sequence since different types of DMRS sequences are supported for different waveforms in NR. Furthermore, the MCS table to be used will be not clear either since for different waveforms different MCS tables are defined for PUSCH transmission in NR.

To make it clear on which waveform to use when the Msg3 is not supported, one simply way is that transform precoding is always enabled or disabled, another way is that msgA-TransformPrecoder is to be used when only 2-step RACH is configured. In our view, the latter is preferred instead of forcing UE to always use CP-OFDM or DFT-s-OFDM in such case. A corresponding text proposal TP2 to 38.214 can be used to correct this.

Proposal 4:

- In case of 2-step RACH only operation, when transformPrecoder is not provided, waveform of normal PUSCH is determined based on msgA-transformPrecoder according to TP#4.

Reasons for change Msg3 will be not supported when only 2-step RACH is configured, i.e. when 4-step RACH is not configured. In this case, it is not clear which waveform should be used for a normal PUSCH transmission when dedicated signaling is not available. Summary of changes In case of 2-step RACH only operation, when transformPrecoder is not provided, waveform of normal PUSCH is determined based on the waveform of MsgA PUSCH.. Consequences if not approved: Waveform of normal PUSCH is not clear in the case of 2-step RACH only operation and when transformPrecoder is not provided.. Specs/Sections impacted TS 38.214, Section 6.1.3 -----------------------Start of Text Proposal #4 for TS 38.214 ---------------------------- 6.1.3 UE procedure for applying transform precoding on PUSCH *** unchanged text omitted*** For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if a Type-1 random access is not configured for the BWP. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if a Type-1 random access is not configured for the BWP. For PUSCH transmission with a configured grant - If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or msgA-transformPrecoder if a Type-1 random access is not configured for the BWP. *** unchanged text omitted*** -------------------------End of Text proposal #4 ----------------------------

## Comments to proposal 4

| Company | Comment |
| --- | --- |
| Ericsson | Agree and some text for cover page are proposed to be updated according to our comments above as well. |
| ZTE | Same comment as proposal 3. The TP introduce a new behavior which is not supported by the previous agreement. To our understanding, the current specification can work without the TP or without considering the 2-step RACH configuration, i.e. if 4-step RACH is not configured and 2-step RACH is configured for a BWP, the behavior should be the same as that both 4-step RACH and 2-step RACH are not configured for a BWP. |
| Qualcomm | Not sure “a Type-1 random access is not configured” is equivalent to “only a Type-2 random access is configured.” Prefer a clarification of the conditions as we commented in proposal 3. |
| CATT | We have the same view with ZTE. |
| Apple | The msgA-transformPrecoder is not mandatory configured. If it’s not configured, the UE disables the transformer precoder. |
| Samsung | As we commented in preparation phase and similar view as ZTE, In conventional operation without 2step RACH, there could be the case that a BWP has no 4step RACH, so this issue is regardless of 2step RACH, so an UE behavior has already been implemented, UE just follows that same behavior is fine. We did not need to additionally introduce new function/behavior for CG PUSCH waveform determination due to 2step RACH. |
| Intel | This TP is not needed as commented before and by other companies. |
| Nokia, Nokia Shanghai Bell | Once more we share the same view of ZTE, CATT, Samsung and Intel. No need to add new functionality for 2-step RACH. |
| Ericsson | Similar to previous proposal 3, we try to clarify that our intention is not to introduce any new functionality. The issue is that in case of 2-step RACH only operation and when transformPrecoder is not provided, current specification assumes 4-step RACH only, which is not correct in 2-step RACH only case. We’re also fine to simply say that transform precoder is always disabled by default for normal PUSCH in case of 2-step RACH only operation, as long as all companies want to do it this way. I.e. with following TP #4a -----------------------Start of Text Proposal #4a for TS 38.214 ---------------------------- 6.1.3 UE procedure for applying transform precoding on PUSCH *** unchanged text omitted*** For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI: - If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or consider transform precoding is disabled if a Type-1 random access is not configured for the BWP. - If the DCI with the scheduling grant was not received with DCI format 0_0 - If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or consider transform precoding is disabled if a Type-1 random access is not configured for the BWP. For PUSCH transmission with a configured grant - If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter. - If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder if a Type-1 random access is configured for the BWP or consider transform precoding is disabled if a Type-1 random access is not configured for the BWP. *** unchanged text omitted*** -------------------------End of Text proposal #4a ---------------------------- |
| vivo | We share the same view as ZTE. The TP is not needed. |

# Summary

The final proposals and the potential CRs are to be updated…

# References

- R1-2103403	Correction on the configuration of RACH-related power control parameters	Huawei, HiSilicon

- R1-2103495	Editorial corrections on the DMRS description for MsgA		ZTE, Sanechips

- R1-2103680	Discussion on corrections for 2-step RACH	Ericsson