3GPP TSG RAN WG1 #104bis-e	R1-21xxxxx

e-Meeting, April 12th – 20th, 2021

Agenda item:		7.2.12

Source:	Moderator (China Telecom)

Title:	[104b-e-NR-Rel16-TxSwitching-01] Summary of email discussion on maintenance of Rel-16 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In [1], maintenance issues are summarized for Rel-16 uplink Tx switching. As per the guidance of Chairman, following issues are identified for email discussion/approval during RAN1 #104b e-meeting:

[104b-e-NR-Rel16-TxSwitching-01] Email discussion/approval regarding potential CRs for the following issues

- Issue#1: Correction on RRC parameter “uplinkTxSwitchRequest” in TS 38.214

- Issue#2: Clarification on SRS carrier switching

- Issue#3: Clarification on SRS antenna switching

  - Whether it is a valid case should be clarified first.

- Issue#4: Clarification on UCI mapping

till 4/16 – Jianchi (China Telecom)

This contribution is the summary of email discussion/approval on maintenance of Rel-16 uplink Tx switching.

# Email discussion (1st round)

## Issue #1: Correction on RRC parameter “uplinkTxSwitchRequest” in TS 38.214

R1-2102377 mentioned that the RRC parameter “uplinkTxSwitchRequest” is misused in section 6.1.6 in TS38.214.

### 6.1.6 Uplink switching < unchanged text omitted> The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod: - If a UE indicated a capability for uplink switching with uplinkTxSwitchRequest BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination - Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or - Configured with uplink carrier aggregation, or - Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink. the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively. < unchanged text omitted>

Companies are encouraged to provide views on the above TP.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | OK. |

## Issue #2: Clarification on SRS carrier switching

SRS carrier switching was intensively discussed in RAN1 #104e. Companies acknowledged that some clarification is needed, but no consensus has been achieved. R1-2102491, R1-2103149, R1-2103746 proposed to further discuss this issue in this meeting.

There are two UE behaviours for SRS carrier switching in the spec, i.e., dropping rule and suspension. R1-2102491 stated it is clear that the suspension is only applicable to the source carrier and target carrier. However, it is not clear whether the current dropping rule is applicable to the carrier other than source carrier and target carrier. R1-2102491 proposed the TP to clarify the UE behaviour.

#### 6.2.1.3 UE sounding procedure between component carriers < -------------------- Other parts are omitted -------------------- > A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2. If the UE is configured with uplink switching with parameter uplinkTxSwitching for carrier c2 and carrier c3, during 2-port SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c3. < -------------------- Other parts are omitted -------------------- >

R1-2103746 proposed to clarify the UE behaviours for both dropping rule and suspension and proposed the TPs.

<Unchanged parts are omitted – 38.214> #### 6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2, and also the uplink transmission on carrier c3 if the UE is configured with uplinkTxSwitching-r16 for uplink switching between uplink carrier c2 and c3. <Unchanged parts are omitted – 38.214>

<Unchanged parts are omitted – 38.214> #### 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell d with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sd={s0d, s1(d)} as the set of carriers of serving cells that meet all the following conditions: - {s0d,s1(d)} are in the same TAG as s0(d). - {s0d,s1(d)} are in the different band as s0(d), s0(d) and s1(d) are both configured with uplinkTxSwitching-r16. The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier and transmission of a physical signal/channel over a carrier of a serving cell in set Sd: - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sd happens to overlap in the same symbol. - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sd happens to overlap in the same symbol. - the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sd configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol. - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in set Sd whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol. <Unchanged parts are omitted>

R1-2103149 proposed the following proposals and provided the corresponding TPs.

Proposal 1: In the prioritization for SRS switching considers the state of carriers configured with UL Tx switching jointly.  As an example, if SRS switching is configured between CC2 and CC3 then in the prioritization the state of CC1 also needs to be considered if CC1 and CC2 are configured with UL Tx switching.

Proposal 2: Define requirements allowing dropping transmissions on a CC due to SRS transmission on another CC, even if this CC is not configured with SRS switching, as long as the CC is configured with UL Tx switching.

Proposal 3: Choose one of the following options:

- During the SRS transmission on CC3 and the interruption time caused by RF tuning, UE is not expected to be scheduled or configured with other transmission requiring UL Tx switching

- Define rules on the order in which the UE state vs. dropping decisions are being made

Proposal 4: During SRS transmission, CC3 is always treated as two ports, even if one-port SRS resource is configured.

#### 6.1.6.2 Uplink switching for carrier aggregation <Unchanged parts are omitted> - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier. - If the UE is configured with SRS-CarrierSwitching, the UE is not expected to be scheduled or configured with any uplink transmission (including a preceding or succeeding duration NTx1-Tx2) overlapping with an SRS transmission on a serving cell not configured for PUSCH/PUCCH transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR), whenever the uplink transmission would be preceded or succeeded by a duration NTx1-Tx2. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. <Unchanged parts are omitted>

<Unchanged parts are omitted> #### 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. In evaluating whether the transmission can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability, the UE may assume uplink transmission occurring in all carriers configured for PUSCH/PUCCH transmission, except for the carrier indicated by srs-SwitchFromCarrier, and if that carrier is configured with parameter uplinkTxSwitching, also the other carrier configured with uplinkTxSwitching, for which the UE assumes the actual configured transmissions. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. In evaluating whether the transmission can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability, the UE may assume uplink transmission occurring in all carriers configured for PUSCH/PUCCH transmission, except for the carrier indicated by srs-SwitchFromCarrier, and if that carrier is configured with parameter uplinkTxSwitching, also the other carrier configured with uplinkTxSwitching, for which the UE assumes the actual configured transmissions. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. In evaluating whether the transmission can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability, the UE may assume uplink transmission occurring in all carriers configured for PUSCH/PUCCH transmission, except for the carrier indicated by srs-SwitchFromCarrier, and if that carrier is configured with parameter uplinkTxSwitching, also the other carrier configured with uplinkTxSwitching, for which the UE assumes the actual configured transmissions. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. In evaluating whether the transmission can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability, the UE may assume uplink transmission occurring in all carriers configured for PUSCH/PUCCH transmission, except for the carrier indicated by srs-SwitchFromCarrier, and if that carrier is configured with parameter uplinkTxSwitching, also the other carrier configured with uplinkTxSwitching, for which the UE assumes the actual configured transmissions. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'. A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2. If the UE is configured with uplinkTxSwitching-r16 for uplink switching between c2 and a third carrier c3, the UE may temporarily suspend the uplink transmission on carrier c3 during the SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR). If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2. <Unchanged parts are omitted>

Companies are encouraged to provide views on the proposals in R1-2103149 and proposed TPs in R1-2102491, R1-2103149 and R1-2103746.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Regarding the first change about suspended uplink carrier, we prefer a concise TP like our TP proposed in R1-2103746. Regarding the change about prioritization rules, the “switch-from” carrier is surely involved in prioritization rules together with the “switch-to” SRS carrier, as shown in the latest LTE specification TS 36.213 for SRS carrier switching and shown in CR R1-1721095. Therefore, we prefer to keep the same UE behaviour instead of excluding the “switch-from” carrier as the TP proposed in R1-2103149. Additionally, for the intra-band case where the same UE hardware PA shared between the “switch-from” carrier and the intra-band carrier, the prioritization rules in LTE SRS carrier switching has been extended to the intra-band carrier as in R1-1721095, and the following agreement, RAN1#90: Agreement in Principle: Capture the following in 36.213: - The “same PA” is implicitly identified by the following. Two “CC with the same PA” are those that: - Are in the same band - Are in the same TAG - Have the same CP - Adopt the following solution for the cases of collision between SRS switching and victim CC: - Extend collision rules to victim CC (i.e., the transmission of SRS depends on the information transmitted in the source CC and the victim CC). CR to be prepared for RAN1#90bis. Similar to the intra-band case, UE hardware are shared between the uplink carrier configured with ULTxswitching, the situation is the same regarding to the prioritization rules, i.e. the uplink carrier sharing the same UE hardware with “switch-from” carrier is also a victim carrier. Therefore, we propose our TP in R1-2103746 with the same text structure as the latest LTE spec for SRS carrier switching. |

## Issue #3: Clarification on SRS antenna switching

SRS antenna switching was discussed in RAN1 #104e. Companies have different understandings whether the proposed issue is a valid case.

UL

SRS

SRS and UL

DL

Y symbol

CC1 UL

CC2

DL

Potential conflict w/o clarification

R1-2103149 raised the issue that there is potential conflict due to SRS antenna switching illustrated in the above figure and have following proposal:

- In the Y-symbol gap between SRS transmissions defined by Table 6.2.1.2-1 in 38.214, the UE is assumed to operate with the same number of ports as before and after the gap.

Companies are encouraged to provide views on whether the above issue is a valid case.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | As commented before, we hope the proponent could take our previous comments into consideration, copied below as well. “Regarding issue#3, we don’t agree the case shown in Figure 2 in R1-2103149 is a valid case simply because it has been precluded by current specification either the text about “no more than 1 UL Tx switching per slot” or the text about switching gap much larger than Y=1 symbol. Additionally, the TP proposed in R1-2103149 seems not relevant to the figure and not necessary simply because the SRS resource set of SRS antenna switching has been already restricted with the same number of SRS ports. Without this TP, it has been the same number of ports anyway. Therefore, we don’t feel issue#3 should be discussed again in this meeting. We are open for it if a reasonable TP is provided in future meeting.” |

## Issue #4: Clarification on UCI mapping

R1-2103149 raised the issue that UE behaviour is not clear for UCI mapping in the case illustrated in the following figure:

R1-2103149 proposed two options to clarify the UE behaviour:

- Option 1: Error case

- Option 2: Do multiplexing first: multiplex the UCI in CC2, then drop CC2.

Companies are encouraged to provide views on the above issue and proposed options.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Option 2. Because in current spec, as long as the timeline conditions for UCI multiplexing are met, UCI multiplexing is performed. The impact from UL Tx switching has been reflected by increased timeline conditions. If any conclusion would be made, the wording of Option 2 would be better to be rephrased a bit to describe the concerned case more precisely. |

# References

- R1-2103502, Summary of Rel-16 uplink Tx switching, Moderator (China Telecom), RAN1#104bis-e, April 12th – 20th, 2021.

- R1-2102377, Text Proposals for Tx Switching between Two Uplink Carriers, OPPO, RAN1#104bis-e, April 12th – 20th, 2021.

- R1-2102491, Remaining Issues of Rel-16 UL Tx Switching, ZTE, RAN1#104bis-e, April 12th – 20th, 2021.

- R1-2103149, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1#104bis-e, April 12th – 20th, 2021.

- R1-2103746, Discussion on the remaining problems of supporting Tx switching between two uplink carriers, Huawei, HiSilicon, RAN1#104bis-e, April 12th – 20th, 2021.