3GPP TSG-RAN WG1 #104b-e	R1-210xxxx

e-Meeting, April 12th – 20th, 2021

Agenda Item:	7.2.3

Source:	Moderator (AT&T)

Title:	Summary of [Prep-104bis-e-NR_IAB] Maintenance of Integrated Access and Backhaul for NR

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of the contributions to AI 7.2.3 of RAN1#104bis-e and proposals for email discussions.

Note that the email budget for Rel-16 IAB in RAN1#104bis-e is up to 2 email threads.

# Summary of Contributions

## Correction on search space definition for Type-3 CSS and USS

Source: R1-2102588

Background: In current specification, DCI format 2_5 with CRC scrambled by AI-RNTI can be transmitted in Type-3 PDCCH CSS and USS, which has been captured in clause 14 in TS38.213 and section 6.3.2 in TS38.331.  However, AI-RNTI is missing in the description related to search space definition for Type-3 PDCCH CSS and USS in clause 10.1. Besides, DCI format 2_5 is also missing for the configuration of Type-3 PDCCH CSS and USS in TS38.213 clause 10.1.

Considering the minimum periodicity for monitoring DCI format 2_4 and DCI format 2_5 is 1 slot, UE should not expect to process information from more than one DCI format with CRC scrambled with the CI-RNTI or AI-RNTI per slot.

Proposed Change:

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, or AI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), AI-RNTI, SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI.

< Unchanged part is omitted >

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a CI-RNTI, a AI-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

< Unchanged part is omitted >

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10  search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace:

-	a search space set index s, 0<s<40 , by searchSpaceId

-	an association between the search space set s  and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration

-	a number of PDCCH candidates Ms(L) per CCE aggregation level L by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set s is either a CSS set or a USS set by searchSpaceType

-	if search space set s is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates, or to monitor one PDCCH candidate per RB set if the UE is provided freqMonitorLocations for the search space set, for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	an indication by dci-Format2-4 to monitor PDCCH candidates for DCI format 2_4

-	an indication by dci-Format2-5 to monitor PDCCH candidates for DCI format 2_5

-	an indication by dci-Format2-6 to monitor PDCCH candidates for DCI format 2_6

-	if search space set s is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 2_5, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

< Unchanged part is omitted >

FL Observation: Parts of R1-2102588 have been discussed in previous meetings with the conclusion that IAB-specific specification should be kept within Section 14 of 38.213 as much as possible.

FL Proposal: Discuss until 4/9 whether R1-2002588 is an essential correction and whether it should be handled in RAN1#104bis-e.

Discussion:

| Company | Is R1-210258 is an essential correction and should be handled in this meeting? | Comments |
| --- | --- | --- |

## Clarification on availability indication for IAB-DU cell

Source: R1-2103498

Background: In TS38.331, two parameters availableCombToAddModList-r16 and availableCombToReleaseList-r16 are used to add and release a list of availabilityCombinations for the IAB-DU’s cells respectively. I.e. two parameters define which set of cells of the IAB-DU is applied for available indicator of soft symbols. However, “a set of cells of the IAB-DU” in TS38.213 is not associated with IAB-DU's cells configured by parameters availableCombToAddModList-r16 and availableCombToReleaseList-r16 in TS38.331. Therefore, it is not clear which set of cells of the IAB-DU is applied for availability indicator of soft symbols. Thus, TS38.213 needs to be modified to be consistent with TS38.331.

Proposed Change:

# 14	Integrated access-backhaul operation

<Unchanged parts are omitted>

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH.

For each cell of an IAB-DU that is included in a set of cells of the IAB-DU configured to the IAB-node by availableCombToAddModList-r16 and availableCombToReleaseList-r16, the IAB-DU can be provided:

-	an identity of the IAB-DU cell by iab-DU-CellIdentity

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI

-	a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, and

-	a mapping for the soft symbol availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

The IAB-DU can assume a same SCS configuration for availabilityCombinations for a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell.

<Unchanged parts are omitted>

FL Observation:  Potentially only an editorial correction given the context of 38.331 makes it clear where/how the IAB-DU cell identity is configured.

FL Proposal: Discuss until 4/9 whether R1-2103498 is an essential correction and whether it should be handled in RAN1#104bis-e.

Discussion:

| Company | Is R1-2103498 is an essential correction and should be handled in this meeting? | Comments |
| --- | --- | --- |

## H/S/NA configuration per pair of IAB-DU cell and IAB-MT’s serving cell

Source: R1-2103713

Background: This contribution addresses the issue of a missing H/S/NA configuration option per IAB-MT carrier. The current specification in TS 38.213 [1] regarding the H/S/NA configuration of a DU cell as well as the reception of DCI 2_5 do not prevent to have multiple configurations of either. However, no prioritisation or conflict resolution is provided for how an IAB-DU should interpret conflicting configurations or availability information.

Furthermore, presently TS 38.213 is purely assuming operation in a single carrier on parent and child links. RAN1 did not intend this in Rel-16 IAB. Not only is an important Rel-16 agreement left unimplemented; but, since IAB in Rel-17 will also enable multi-parent scenarios, an IAB-node operating on uncoordinated though potentially interacting carriers, there may be backwards compatibility issues with the signalling and IAB node behaviour if not addressed already in Rel-16.

Proposed Change:

- - - - - - - - - - - - - - - - Begin extract from TS 38.213 - - - - - - - - - - - - - - - - - -

With reference to slots of an IAB-DU cell, for each pair of an IAB-DU cell and an IAB-MT serving cell, a symbol in a slot of an IAB-DU cell can be configured to be of hard, soft, or unavailable type.

In a downlink, uplink, or flexible symbol, the IAB-DU cell cannot respectively transmit, receive, or either transmit or receive in the symbol if

-	the DU symbol is configured as unavailable (with respect to any MT serving cell), or

-	for any MT serving cell for which the DU symbol is configured as soft,

-	the IAB-MT does transmit or receive in the symbol, and

-	any IAB-MT transmission or reception in the symbol is changed due to a use of the symbol by the IAB-DU, and

-	the IAB-MT does not detect a DCI format 2_5 with an AI index field value indicating the soft symbol as available

A symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot.

If an IAB-node is provided an AvailabilityIndicator, the IAB-node is provided an AI-RNTI by ai-RNTI and a payload size of a DCI format 2_5 by dci-PayloadSizeAI. The IAB-node is also provided a search space set configuration, by SearchSpace, for monitoring PDCCH.

For each pair of an IAB-DU cell and an IAB-MT serving cell, in a set of pairs of an IAB-DU cell and an IAB-MT serving cell, the IAB-DU can be provided:

-	an identity of the IAB-DU cell by iab-DU-CellIdentity

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI

-	a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-DU cell, and

-	a mapping for the soft symbol availability combinations provided by resourceAvailability to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

- - - - - - - - - - - - - - - - End extract from TS 38.213 - - - - - - - - - - - - - - - -

FL Observation:  This issue is related to an incoming RAN3 LS:

R1-2102294	LS on Granularity of the H/S/NA Slot Configurations for the IAB-DU	RAN3, Ericsson

FL Proposal: Handle this contribution and the related discussion including the draft reply tdocs in AI 5 under a single email thread:

Granularity of the H/S/NA Slot Configurations for the IAB-DU (including RAN3 LS response)

Discussion:

| Company | Do you agree with the FL Proposal to consolidate discussion with the related RAN3 LS? | Comments |
| --- | --- | --- |

# Summary

TBD