Proposal 1-1 (round V):

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, traffic type, etc.)

    - Alt.3: A set of Preserve values is (pre-)configured and includes common multiple(s) of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics (e.g. priority, CBR, HARQ error rate, traffic type, etc.)

    - Alt.4: Multiple sets of Preserve values based on transmission priority

- For the k value, down-select to one of the following in RAN1#105-e

    - Alt.1: Only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction

    - Alt.2: k is (pre-)configured, including multiple values with at least k=1. If the corresponding the most recent sensing occasion for a given reservation periodicity and a k value is located after before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, the most recent sensing occasion before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction shall be monitored by the UE.

      - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value from the (pre-)configuration

    - Alt.3: k is from a (pre-)configured bitmap, as in LTE-V

  - Note: companies are encouraged to provide more evaluations

  - FFS: whether sensing should be performed for periodic sensing occasions located between the resource (re)selection triggering and the first slot of the set of Y candidate slots subject to processing time restriction are monitored by the UE for identification of candidate resources within the set of Y candidate slots.

    - Note: this does not cover resource (re)selection trigger by re-evaluation and pre-emption checking

Proposal 1-2 (round V):

- In periodic-based partial sensing,

  - At least for periodic transmissions, the set SA is initialized to the set of all the candidate single-slot resources within the selected set of Y candidate resource slots.

  - FFS the definition of set SA in the case of re-evaluation and pre-emption checking.

- FFS if the set of SA should be restricted by the a set of Y candidate slots within the resource selection window for aperiodic transmissions in a resource pool with reservation for another TB enabled.

  - Note: companies are encouraged to provide evaluation results

- FFS how periodic-based partial sensing process can be applied when resource (re)selection is triggered for PSCCH/PSSCH transmission without enabled reservation for another TB

Proposal 2-1 (round IV):

- Periodic-based partial sensing is performed by UE in a mode 2 Tx resource pool provided by higher layer when at least the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - UE is configured to perform partial sensing by higher layer

  - UE is triggered to performs resource (re)selection procedure at least for periodic transmission (i.e., resource reservation interval Prsvp_TX≠0)

    - FFS the case(s) when UE is triggered to perform resource (re)selection procedure for aperiodic transmission

  - FFS whether and how to deal with the initial period(s) in periodic/aperiodic PSCCH/PSSCH transmission with and without enabled reservation for another TB where the UE have insufficient sensing results, e.g.,

    - whether a TX UE is able to know in advance / expect a resource (re)selection will be triggered in slot n for early sensing,

    - performing random resource selection or contiguous partial sensing,

    - dropping transmission(s),

    - up to UE implementation to handle this case, or

    - etc

Proposal 2-2 (round IV):

- In a resource pool (pre-)configured with at least partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool, (and if pre-emption checking is (pre-)configured in the resource pool), resource exclusion based on at least periodic-based partial sensing is also performed by UE for the purpose of re-evaluation (and pre-emption checking) (if pre-emption is enabled for the resource pool).

  - This is subject to other conditions, if agreed, in which the UE performs periodic-based partial sensing

  - FFS details

Proposal 2-3 (round IV):

- FFS whether UE performing periodic-based partial sensing and/or contiguous partial sensing is further conditioned by one or more of the followings

  - When the priority value of a packet is above a threshold

  - When the congestion/interference level in a resource pool is above a threshold and/or below another threshold

  - When the required reliability level of a packet transmission is above a threshold

  - When the number of received HARQ NACKs or HARQ error rate of a TB is above a threshold

  - When the number of retransmissions of a packet is below a threshold

  - When the PDB/remaining PDB/latency requirement is above a threshold

  - When the available resource ratio in a resource selection window from a sensing process is above a threshold and/or below another threshold

  - When the traffic is periodic and/or aperiodic

  - When the UE has packet to transmit

  - Other conditions are not precluded

Proposed conclusion (round III):

- Further study the issue case of random resource selection (e.g., random selection for low priority transmission) by UE without PSSCH reception does not perform sensing in a resource pool (pre-)configured with mixed RA schemes.

Proposed conclusion (round II):

- Further study the followings for power saving [with low priority]:

  - Support or further study mixed blind and HARQ-feedback based retransmissions

  - Restriction/rule for the minimum distance between consecutive blind (re-)transmission for UEs performing random resource selection

  - [Restriction/rule for the maximum distance between any two resources indicated by a single SCI for UE performing partial sensing]

  - [Dependency of minimum number of candidate slots Y for partial sensing on priority level]

  - [Adaptation b/w partial sensing/ random resource selection and full sensing]