3GPP TSG-RAN WG1 Meeting #105-e	R1-2104179

e-Meeting, 19th – 27th May 2021

Agenda Item:	8.6.1.1

Source:	Ericsson

Title:	Reduced maximum UE bandwidth for RedCap

Document for:	Discussion, Decision

# 1	Introduction

According to the Rel-17 work item description on support of reduced capability NR devices ‎[1], the objective on reduced maximum UE bandwidths is as follows:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

In RAN1#104e, the following agreements on reduced maximum UE bandwidths were reached ‎[2]:

Agreements: - Sharing of the same SSB and CORESET#0 between RedCap and non-RedCap UEs is supported when the bandwidth is no wider than the RedCap UE bandwidth - The initial DL BWP (derived based on MIB/SIB) for RedCap UEs can be the same as the initial DL BWP for non-RedCap UEs at least when the initial DL BWP is no wider than the RedCap UE bandwidth. - FFS: after initial access, whether a RedCap UE is allowed to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth - Discuss further whether or not it is also applicable during initial access - The initial UL BWP (derived based on SIB) for RedCap UEs can be the same as the initial UL BWP for non-RedCap UEs at least when the initial UL BWP is no wider than the RedCap UE bandwidth. - FFS: during and after initial access, whether a RedCap UE is allowed to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth - FFS whether or not to further introduce the following (e.g., for offloading purpose, for differentiation of RedCap vs. non RedCap UEs, for different BWP#0 configuration options, etc.) - Whether an additional CORESET can be configured for scheduling of RACH (msg2 & msg4)/Paging/SI messages for RedCap UEs - Whether the SIB-configured initial DL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial DL BWP for non-RedCap UEs. - Whether the SIB-configured initial UL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial UL BWP for non-RedCap UEs. Conclusion: RAN1 does not consider acquisition time improvements for FR2 RedCap UEs with SSB and CORESET#0 multiplexing patterns 2 and 3 as part of this WI. Agreements: - Study further how to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, with the following options: - Option 1: Proper RF-retuning for RedCap - Option 2: Separate initial UL BWP(s) for RedCap UEs - Option 3: gNB configuration (e.g., restrictions on existing PRACH configurations, or FDM-ed ROs, or always restricting the initial UL BWP to within RedCap UE bandwidth) - Option 4: Dedicated PRACH configurations (e.g., ROs) for RedCap UEs - Other options are not precluded Conclusion: Discuss further in RAN1#104b-e whether or not to send LS to RAN4 regarding RF retuning time, and if so, the RAN1 details associated with question. Agreements: - Study further whether and how to enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, with the following options: - Option 1: Proper RF-retuning for RedCap (if feasible) - Option 2: Separate initial UL BWP(s) for RedCap - FFS more than one starting PRB position - Option 3: Separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation for the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping) - Option 4: gNB configuration (e.g., always restricting the initial UL BWP to within RedCap UE bandwidth, or restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH) - As an example, with restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH, when the initial UL BWP is the same for RedCap and non-RedCap UEs, the PUCCH (for Msg4/[MsgB] HARQ feedback) and PUSCH (for Msg3/[MsgA]) are within the RedCap UE bandwidth - Other options are not precluded

Moreover, the agreements and working assumptions based on RAN1#104bis-e are as follows ‎[2]:

Working assumption: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs. - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS). Working assumption: - After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2) Agreements: - During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Agreements: - After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e: - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Working assumption: - A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

In this section, we present our analysis on the bandwidth (BW) reduction aspects based on the above agreements and working assumptions.

# 2 		Initial DL BWP

For initial DL BWP, according to RAN1#104bis-e working assumptions,

Working assumption: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs. - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS). Working assumption: - After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2)

## 2.1 	During initial access

During the initial access, the DL transmissions (e.g., Msg2 and Msg4) are confined within the CORESET #0 bandwidth, which is within the RedCap UE bandwidth. Therefore, regardless of the size of initial DL BWP, these DL transmissions are within the CORESET #0 bandwidth, which can also be received by the RedCap UEs.

It is FFS whether a separate or additional bandwidth and location for initial DL BWP can be configured for RedCap UEs during initial access. In general, from scheduling complexity, signaling overhead, and resource utilization perspective, it is beneficial to have the same initial DL BWP for RedCap and non-RedCap UEs. In this case, only one set of initial DL BWP configuration is needed to support both RedCap and non-RedCap UEs which is desirable.  Meanwhile, using separate or additional bandwidth and location for initial DL BWP for RedCap can be beneficial by offering more flexibility in placing initial BWPs. Such flexibility can be particularly useful in TDD scenarios where, based on the current NR specifications, the initial DL and UL BWPs need to be co-centered. For example, if the initial UL BWP for RedCap is different than non-RedCap, the location of RedCap initial DL BWP may need to be adjusted based on the location of the RedCap initial UL BWP. Alternatively, the condition for co-centering initial DL and UL BWPs in TDD scenarios could be relaxed to allow placing UL and DL BWPs in different locations thus having more flexibility. In this case, frequency retuning between DL and UL center frequencies is not expected to be an issue as far as the UE implementation is concerned, considering the relaxed required switching time between DL and UL during initial access. Also, such frequency retuning between DL and UL center frequencies is anyway needed for half-duplex FDD UEs which is acceptable from UE complexity perspective.

Moreover, the possibility of configuring a separate initial DL BWP can be beneficial for offloading purposes. For example, transmissions for Msg2, Msg4, paging, and system information other than SIB1 (i.e., SIBx, with x>1) for RedCap can be offloaded to a common CORESET which is defined in a separate initial DL BWP to mitigate the CORESET #0 congestion. It should be noted that after decoding MIB, the UE has the required information (e.g., CORESET #0 configuration) for acquiring SIB1. Therefore, SIB1 must be scheduled using CORESET #0 as it is not possible to configure a separate CORESET for SIB1 in the MIB which only has only 1 spare bit left. However, a separate initial DL BWP in conjunction with an additional CORESET can be used for offloading Msg2, Msg4, paging, and SIBx (with x>1, e.g., SIB2, SIB3, etc.) transmissions.

In Rel-15 NR specifications, the DL transmissions during the initial/random access (e.g., Msg2 and Msg4) are confined within the CORESET #0 bandwidth which does not exceed the RedCap UE bandwidth.

From scheduling complexity, resource utilization, and signaling overhead point of view, it can be beneficial to have a shared initial DL BWPs for RedCap and legacy UEs.

Using separate initial DL BWP for RedCap can be beneficial for flexibility and offloading purposes.

It would be beneficial to have the possibility of configuring a separate or additional bandwidth and location for initial DL BWP for RedCap UEs.

Confirm the working assumption that “During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth” and its sub-bullets.

The network can optionally configure a separate or additional bandwidth and location for initial DL BWP for RedCap.

## 2.2 	After initial access

First, we note that after the initial access the UE capability is known, and the network can schedule transmissions to be within the RedCap UE bandwidth even if the UE is configured with a BWP wider than its maximum bandwidth. For BWP#0 configuration option 2 (as in TS 38.331, Appendix B.2), it is FFS whether a RedCap UE is expected to operate with an initial DL BWP that is restricted to never be wider than the maximum RedCap UE bandwidth or not.

The key advantage of BWP#0 configuration option 2 is that it allows the network to establish a fully operational connection with UEs by only configuring one BWP per cell which potentially spans over the entire carrier bandwidth. If the network needs to configure multiple BWPs from the cell perspective to support RedCap, it loses a major incentive to use BWP#0 configuration option 2. Therefore, supporting BWP#0 configuration option 2 requires a RedCap UE to operate on a BWP wider than its maximum bandwidth.

Nevertheless, it is foreseen that BWP#0 configuration option 1 (as in TS 38.331, Appendix B.2) will be widely adopted by the time that RedCap is commercialized. Also, it is expected that most of the networks that support configuration option 2 today will migrate to BWP#0 configuration option 1 in the next few years. In this regard, there is no need to prioritize solutions for supporting RedCap for BWP#0 configuration option 2. Therefore, it may not be necessary for a RedCap UE to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

Supporting BWP#0 configuration option 2 requires a RedCap UE to operate on a BWP wider than its maximum UE bandwidth. If the network needs to configure multiple BWPs, it loses a major incentive to use BWP#0 configuration option 2 that allows operation with a single large BWP.

Since it is expected that most of the networks that support BWP#0 configuration option 2 today will migrate to BWP#0 configuration option 1 in the next few years, there is no need to prioritize solutions for supporting RedCap for BWP#0 configuration option 2.

Confirm the working assumption that after initial access a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth for both BWP#0 configurations option 1 and option 2.

## 2.3 	Congestion and offloading aspects

Based on RAN1#104e, it is FFS whether an additional CORESET can be configured for scheduling of Msg2/Msg4/Paging/SI messages for RedCap UEs. In case of shared initial BWPs (and CORESETs) between RedCap and non-RedCap UEs, the potential congestion issue depends on the total number of UEs and the number of available time-frequency resources (i.e., size of BWP and CORESETs). In particular, it is not unlikely that CORESET #0 may become congested in case there is a large number of UEs generating a considerable amount of traffic during the initial/random access.

Based on the current NR specifications, UEs can monitor PDCCH candidates intended for RAR/paging/system information messages in CORESET #0 or another CORESET. If the UE is not provided with a CORESET for any of the Type0A-PDCCH/Type1-PDCCH/Type2-PDCCH common search spaces, the corresponding CORESET would be CORESET #0. An additional CORESET (other than CORESET #0) may also be configured and used for RAR/paging/system information. In this case, the network configures the commonControlResourceSet in SIB1 such that it is contained in the bandwidth of CORESET #0. Therefore, according to the Rel-15 specifications, it is already possible to configure additional CORESETs for initial access, but with the limitation that the additional CORESET needs to be within the bandwidth of CORESET #0 and this does not considerably help in increasing the capacity. In fact, given the limited size of CORESET #0 (i.e., at most 48 CCEs), the PDCCH capacity can be limited when there is a need for scheduling many UEs.

To mitigate congestion on CORESET #0, it can be useful to introduce a separate CORESET for RedCap UEs. To achieve offloading purposes, such additional CORESET must be fully or partially non-overlapping with CORESET #0 in the frequency domain. Moreover, this additional CORESET could be defined within a separate initial DL BWP for RedCap UEs (i.e., the configuration of the additional CORESET can be included in the configuration of a separate initial DL BWP). As previously discussed, SIB1 must be scheduled using CORESET #0 as it is not possible to configure a separate CORESET for SIB1 in the MIB (which only has a single spare bit). Therefore, an additional CORESET can be used for offloading Msg2, Msg4, paging, and SIBx (with x>1, e.g., SIB2, SIB3, etc.) transmissions, while relying on CORESET #0 for receiving SIB1, as illustrated in Figure 1.

However, there are some considerations for introducing the new CORESET due to the RedCap UE bandwidth limitations. Moreover, it needs to be ensured that the RedCap UEs can properly receive SSB and SI given their bandwidth limitations. For example, in FR1 with a 20-MHz RedCap UE bandwidth the CORESET #0 bandwidth can be 17.28 MHz. In this case, if both CORESET #0 and the additional CORESET need to be within the RedCap UE bandwidth, there will be a large overlap between these two CORESETs which does not provide considerable offloading benefits. Alternatively, to achieve a significant offloading gain, the total bandwidth of CORESET #0 and the new CORESET can exceed the RedCap UE bandwidth (i.e., to minimize overlap between two CORESETs). Consequently, the RedCap UE can monitor CORESET #0 (for SIB1) and the new CORESET (for paging, random access, SIBx) in different time slots while relying on RF retuning to switch between these two CORESETs. We note that the UE may not be expected to monitor CORESET #0 for SIB1 frequently, and thus the potential RF retuning attempts are not expected to be frequent. Further, since latency is not typically critical for RedCap UEs during initial access, RF retuning delay will not be an issue and, hence, the RedCap UEs can have sufficient retuning time to perform the required RF retuning.

The additional CORESET can be defined within a separate SIB-configured initial DL BWP for RedCap UEs. Moreover, the RedCap initial DL BWP does not necessarily need to contain CORESET #0 as the limited bandwidth of RedCap UEs may not cover both CORESET #0 and the additional CORESET. This is beneficial as it provides flexibility for properly placing CORESET #0 and the initial DL BWP within a given carrier. Also, with such flexibility, in TDD operation, it is feasible to place both UL and DL BWPs (with the same center frequency) at one edge of a given carrier to minimize the PUSCH resource fragmentation in coexistence with non-RedCap UEs.

Figure 1: Additional CORESET for RedCap for offloading purposes.

In scenarios where CORESET #0 might be congested due to a high number of UEs, having an additional CORESET can be beneficial for traffic offloading.

An additional CORESET which is partially or fully non-overlapping (in the frequency domain) with CORESET #0 can be configured for offloading purposes during the initial/random access. However, in introducing an additional CORESET, the RedCap UE bandwidth limitation as well as proper reception of SSB and SIB1 need to be considered.

The additional CORESET can be defined within the RedCap initial DL BWP and used for offloading Msg2, Msg4, paging and SI (other than SIB1) message transmissions, while CORESET #0 is used for scheduling SIB1.

An additional CORESET can be configured for scheduling of Msg2, Msg4, paging and SI (other than SIB1) messages for RedCap UEs.

# 3 	Initial UL BWP

For initial UL BWP, according to RAN1#104bis-e agreements:

Agreements: - During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Agreements: - After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e: - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

In this section, we discuss various aspects of the three options listed in the above RAN1#104bis-e agreement for configuring the initial UL BWPs for RedCap and non-RedCap UEs. Note that, according to the WID ‎[1], coexistence with non-RedCap UEs needs to be ensured. Therefore, while supporting RedCap UEs, it is important to minimize the impact on non-RedCap UEs. In particular, it is essential to minimize or avoid PUSCH resource fragmentation to maintain the performance of non-RedCap UEs, as analyzed in our companion contribution ‎[3].

It is important to note that the PUSCH resource fragmentation results in a significant UL peak data rate reduction for non-RedCap UEs that require contiguous frequency allocation in the uplink. Specifically, for non-RedCap UEs not supporting almost contiguous UL CP-OFDM (Feature 2-7 defined in TR 38.822) or resource allocation Type 0 for PUSCH (Feature 5-2 defined in TR 38.822) features, which are optional with capability signaling, the UL peak data rate significantly decreases due to the PUSCH resource fragmentation. Therefore, it is essential to have proper solutions that prevent such PUSCH resource fragmentation from arising in coexistence scenarios where both non-RedCap and RedCap UEs are present.

A Rel-15/16 UE may not support non-contiguous PUSCH frequency resource allocation as the features of supporting ‘almost contiguous UL CP-OFDM’ (Feature 2-7 defined in TR 38.822) and ‘resource allocation Type 0 for PUSCH’ (Feature 5-2 defined in TR 38.822) are optional with capability signaling.

Any PUSCH resource fragmentation reduces the UL peak data rate for non-RedCap UEs which do not support non-contiguous PUSCH frequency resource allocation.

## 3.1	During initial access

Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP:

During initial access, Option 1 allows sharing the initial UL BWP between RedCap and non-RedCap UEs even when the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth. One potential issue of separate initial UL BWP for RedCap and non-RedCap UEs is resource fragmentation for PUSCH due to PUCCH configurations (as illustrated in Figure 2). With such PUSCH resource fragmentation when there is a RedCap UE sending PUCCH during initial access, the peak UL data rate for non-RedCap UEs significantly decreases, as also discussed in ‎[3]. Therefore, using the same initial UL BWP for RedCap and non-RedCap UEs avoids PUSCH resources fragmentation, as illustrated in Figure 3. Moreover, from scheduling complexity and resource utilization perspective, it may be beneficial to have the same initial UL BWPs for RedCap and non-RedCap UEs.

Meanwhile, as will be discussed in the Section 3.3, enabling a RedCap UE to operate with an initial UL BWP wider than its maximum UE bandwidth requires resolving issues related to PUCCH/PUSCH frequency hopping as well as frequency division multiplexed (FDM-ed) RACH occasions that may fall outside the UE bandwidth.

Figure 2: Example of UL (PUSCH) resource fragmentation due to separate initial BWP and different PUCCH configurations.

Figure 3: Avoiding PUSCH resource fragmentation by allowing RedCap to operate on an initial UL BWP wider than its BW.

During the initial access, a RedCap UE is allowed to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth. This requires resolving issues related to PUCCH/PUSCH frequency hopping as well as FDM-ed RACH occasions that may fall outside the UE bandwidth.

Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs:

The advantage of configuring a separate initial UL BWP for RedCap UEs is that the issues related to PUCCH/PUSCH transmissions and FDM-ed RACH occasions are resolved as the separate initial UL BWP can be defined to be within the RedCap UE bandwidth. That is, the RACH occasion associated with the best/preferred SSB falls within the RedCap UE bandwidth. Moreover, PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access.

However, in the UL, introducing a separate initial UL BWP for RedCap UEs can cause PUSCH resource fragmentation due to the PUCCH transmissions, as also discussed in ‎[3]. The issue of PUSCH resource fragmentation is more pronounced during initial access in which PUCCH frequency hopping is always enabled based on the current NR specifications. In our view, a separate initial UL BWP can be configured only if this PUSCH resource fragmentation is avoided or minimized. For example, to avoid resource fragmentation, the initial UL BWP for RedCap UEs can be placed at one edge of non-RedCap initial UL BWP with disabled PUCCH frequency hopping during initial/random access, as illustrated in Figure 4. Since PUCCH frequency hopping is always used by non-RedCap UEs for sending Msg4/[MsgB] HARQ feedback, an early RedCap UE identification before Msg4/[MsgB] is needed if PUCCH frequency hopping is disabled for RedCap UEs. Note that disabling PUCCH frequency hopping during initial access will not make it coverage limited based on the coverage evaluations in TR 38.875 (as this PUCCH will contain at most a few ACK/NACK bits).

Figure 4: RedCap initial BWP at the edge of non-RedCap BWP with disabled RedCap PUCCH frequency hopping.

If the PUSCH resource fragmentation is avoided, a SIB-configured initial UL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial UL BWP for non-RedCap UEs. This can be achieved by properly adjusting the position of RedCap initial UL BWP and disabling the PUCCH frequency hopping.

The network should be allowed to disable the PUCCH frequency hopping (for Msg4/[MsgB] HARQ feedback) for RedCap UEs during initial access.

In TDD scenarios, if a separate initial UL BWP for RedCap is defined, it may be beneficial to also have a separate initial DL BWP for maintaining the same center frequency for initial UL and DL BWPs. In this case, to minimize the PUSCH resource fragmentation, both initial UL and DL BWPs for RedCap UEs can be placed at one edge of the non-RedCap initial UL BWP. In Figure 5, we illustrate an example of initial DL/UL BWPs configuration for RedCap and non-RedCap in TDD. As we can see, the RedCap initial UL BWP with disabled PUCCH frequency hopping is located at the edge of the non-RedCap initial UL BWP. Also, the RedCap initial DL BWP has the same center frequency as the UL BWP. This configuration has two main benefits: 1) the PUSCH resource fragmentation is avoided thus it does not negatively impact non-RedCap UEs, and 2) RedCap UEs do not need to perform RF retuning between the UL and DL BWPs. However, as discussed in Section 2.3, depending on the location of CORESET #0, RedCap UEs may require RF retuning to monitor CORESET #0 to acquire system information (at least SIB1), but note that such potential RF retuning is not expected to be frequent.

Figure 5: Example of DL/UL initial BWPs configuration for RedCap and non-RedCap in TDD.

In TDD scenarios, if a separate initial UL BWP for RedCap is defined, it may be beneficial to also have a separate initial DL BWP for maintaining the same center frequency for the UL and DL BWPs.

Option 3: The scenario is not allowed:

Option 3 imposes restrictions on the non-RedCap initial UL BWP configuration. Always limiting the bandwidth of the non-RedCap initial UL BWP to be within the RedCap UE bandwidth can have a negative impact on non-RedCap UEs. Specifically, such constraint on the size of non-RedCap initial UL BWP can reduce the frequency diversity and RACH capacity for non-RedCap UEs. Another issue is the potential PUSCH resource fragmentation at the carrier level which can adversely affect resource utilization and scheduling efficiency, as also discussed in ‎[3]. For example, as illustrated in Figure 6, the entire available carrier bandwidth might fragmentated into three parts and cannot be properly utilized if the initial UL BWP is always limited to the bandwidth of RedCap UEs. Since the objective of the WID is to ensure an efficient coexistence with non-RedCap UEs, Option 3 is not generally preferred due to its potential negative impacts on non-RedCap UEs.

Figure 6: PUSCH resource fragmentation at the carrier level.

Nevertheless, when from network perspective it is feasible to limit the size of non-RedCap initial UL BWP to be within the RedCap UE bandwidth, Option 3 is suitable for RedCap operations as the potential issues related to RACH occasion and PUCCH/PUSCH transmissions during initial access are avoided. That is, the RACH occasion associated with the best/preferred SSB falls within the RedCap UE bandwidth. Moreover, PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access.

Always limiting the bandwidth of the non-RedCap initial UL BWP to be within the RedCap UE bandwidth can have a negative impact on non-RedCap UEs.

The initial UL BWP for non-RedCap UEs can be wider than the RedCap UE bandwidth.

For initial UL BWP, Option 3 (which does not allow configuring non-RedCap initial UL BWP wider than the RedCap UE bandwidth) is not further considered.

## 3.2	After initial access

Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP:

After initial access, the use of initial BWP is not very likely for BWP#0 configuration option 1, as the UE typically operates on a non-initial BWP (e.g., BWP#1). However, after initial access, given the BWP switching mechanisms in NR, it is still possible that a UE in connected mode switches back to the initial BWP for random access purposes. Note that in connected mode, random access procedure can be initiated by a PDCCH order from the gNB, for example for uplink re-synchronization (i.e., when the status of UL synchronization status is non-synchronized). In this case, if RACH occasions are not configured for the active UL BWP, it is switched to the initial UL BWP. Hence, if during initial access the UE is allowed to operate on BWP#0 wider than its RF bandwidth, it should also be allowed to operate on BWP#0 after initial access (i.e., in connected mode). Alternatively, the need for switching to the initial UL BWP can be avoided by ensuring that the active UL BWP includes RACH occasions. Note that after initial access the UE capability is known, and thus the network can schedule uplink transmissions to be within the RedCap UE bandwidth although the UE is configured with an UL BWP wider than its maximum UE bandwidth.

For BWP#0 configuration option 2, typically BWP#0 is also used after the initial access to operate with only a single large BWP. For BWP#0 configuration option 2, if used, a RedCap UE should be able to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth during and after the initial access. However, as previously discussed, it may not be necessary to prioritize solutions for supporting RedCap for BWP#0 configuration option 2.

After initial access, the UE capability is known, and thus the network can schedule uplink transmissions to be within the RedCap UE bandwidth although the UE is configured with an UL BWP wider than its maximum UE bandwidth.

After initial access, it is possible that a UE switches back to the initial UL BWP for random access purposes when RACH occasions are not configured for the active UL BWP of the serving cell.

The need for using initial UL BWP after initial access can be avoided by ensuring that the active UL BWP includes RACH occasions.

If during initial access the UE is allowed to operate on UL BWP#0 wider than its maximum bandwidth, it should also be allowed to operate on UL BWP#0 after initial access.

Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs:

After initial access, the UE capability is known, and thus the network can schedule uplink transmissions to be within the RedCap UE bandwidth. Moreover, in the current NR specifications, the PUCCH frequency hopping can be disabled after the initial access. Therefore, compared to the initial access case, the risk of PUSCH resource fragmentation is less after the initial access. In this respect, the RedCap UE can also operate on a separate initial UL BWP after the initial access.

After the initial access, it is already possible to disable PUCCH frequency hopping.

If during initial access a separate initial UL BWP is configured for RedCap, the RedCap UE can also operate on this separate initial UL BWP after the initial access.

Option 3: The scenario is not allowed:

As we previously discussed, Option 3 imposes restrictions on the non-RedCap initial UL BWP configuration. Always limiting the bandwidth of the non-RedCap initial UL BWP to be within the RedCap bandwidth can have a negative impact on non-RedCap UEs. Therefore, Option 3 is not generally desired.

During and after initial access, Option 3 (which does not allow configuring non-RedCap initial UL BWP wider than the RedCap UE bandwidth) is not further considered.

## 3.3 	PRACH, PUCCH, and PUSCH transmissions during initial access

Here, we discuss the potential issues and solutions related to PRACH, PUCCH, and PUSCH transmissions during initial access that may fall outside the UE bandwidth depending on the choice of initial UL BWP.

Option 1 for initial UL BWP: The scenario is allowed, and a RedCap UE can use the same UL BWP

  - With this option, PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) and RACH occasion associated with the best SSB may fall outside the RedCap UE bandwidth. Therefore, using this option requires addressing this issue which will be discussed later.

Option 2 for initial UL BWP: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs

  - With this option, PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) and RACH configurations are determined based on the size of initial UL BWP for RedCap UEs. Hence, PUCCH (for Msg4/[MsgB] HARQ feedback), PUSCH (for Msg3/[MsgA]), and RACH occasion associated with the best/preferred SSB can fall within the RedCap UE bandwidth.

  - However, introducing a separate initial UL BWP for RedCap UEs can cause significant PUSCH resource fragmentation due to the PUCCH transmissions, as also discussed in ‎[3]. In this case, it is essential to avoid or minimize the PUSCH resource fragmentation by disabling PUCCH frequency hopping and proper placement of the RedCap initial UL BWP (e.g., at the edges of carrier or non-RedCap initial UL BWP).

Option 3 for initial UL BWP: The scenario is not allowed

  - With this option, PUCCH (for Msg4/[MsgB] HARQ feedback), PUSCH (for Msg3/[MsgA]), and RACH occasion associated with the best/preferred SSB can fall within the RedCap UE bandwidth.

  - However, restricting the size of initial UL BWP can have negative impacts on non-RedCap UEs. It can reduce the frequency diversity (e.g., for PUCCH and PUSCH transmissions) and RACH capacity for non-RedCap UEs. Another issue potential issue is PUSCH resource fragmentation at the carrier level which can degrade resource utilization and scheduling efficiency, as also discussed in ‎[3].

### 3.3.1	Enabling RACH occasions within the UE BW

Bandwidth reduction has a potential impact on the random access procedure. Specifically, when RACH occasions are frequency-division multiplexed (FDM-ed), the total frequency span of 8 FDM-ed RACH occasions is greater than 20 MHz in FR1 and 100 MHz in FR2 in the following configurations:

- Preamble Format 3 in FR1, L=839 (long preamble), 5 kHz SCS: total BW of 8 FDM-ed RACH occasions = 34.56 MHz

- FR1, L=139 (short preamble), 30 kHz SCS: total BW of 8 FDM-ed RACH occasions = 34.56 MHz

- FR2, L=139 (short preamble), 120 kHz SCS: total BW of 8 FDM-ed RACH occasions = 138.24 MHz

Therefore, for the RedCap UE, the RACH occasion associated with the best/preferred SSB beam may fall outside of its bandwidth, depending on how the UE sets its center frequency. To enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, the following options were considered in RAN1#104e:

Agreements: - Study further how to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, with the following options: - Option 1: Proper RF-retuning for RedCap - Option 2: Separate initial UL BWP(s) for RedCap UEs - Option 3: gNB configuration (e.g., restrictions on existing PRACH configurations, or FDM-ed ROs, or always restricting the initial UL BWP to within RedCap UE bandwidth) - Option 4: Dedicated PRACH configurations (e.g., ROs) for RedCap UEs - Other options are not precluded

In Table 1, we list the key advantages and disadvantages/impacts of each option. Moreover, for some of these impacts we present our view and potential remedies.

Table 1: Key pros and cons of each option (for PRACH transmissions).

| Solution | Pros | Concerns/comments/remedies |
| --- | --- | --- |
| Option 1: Proper RF retuning | - Allows the RedCap devices and the normal UEs to share the same initial UL BWP and PRACH resource. - Uplink resource fragmentation can be avoided. - No/small impact on non-RedCap UEs. | - A new timing relationship between PRACH and RAR (msg2) which shall take the retuning time into account. This may result in a few symbols additional delay during initial access which can be acceptable. - Requires RF retuning. However, RF retuning does not need be done frequently in this case and may not be even needed in most cases. RF retuning is a common behavior for LTE-M UEs and it does not prevent having UEs with low complexity/energy consumption. |
| Option 2: Separate initial UL BWP | - Mostly compatible with existing BWP framework - Also enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access | - Risk of PUSCH resource fragmentation and significant impact on non-RedCap UEs. This can be minimized by allowing network to disable PUCCH frequency hopping during initial access and placing the initial UL BWP at the edges of carrier. - May require different center frequencies for initial UL BWP and DL BWP in TDD. To address this, frequency retuning between DL and UL BWP during initial access can be supported for RedCap (similar to HD-FDD operation). - Increased gNB processing for PRACH. To mitigate this, ROs should be shared between non-RedCap and RedCap UEs as much as possible. - gNB would always configure more than one initial UL BWP even for a very small number of RedCap UEs. - Increased signaling overhead. |
| Option 3: gNB configuration | - Simple and feasible solution - Minimum specification impact. - RedCap UEs can share RACH occasions with legacy UEs. | - The flexibility of the network configuration for legacy UE can be impacted. However, such impact is not significant and sufficient capacity can still be achieved with less than 8 FDM-ed RACH occasions (e.g., 4 FDM-ed RACH occasions). Also, multiplexing in the time domain can be used to increase PRACH capacity. |
| Option 4: Dedicated PRACH configurations | - Early identification support by dedicated resource configuration. - Flexible configuration. | - May complicate the gNB’s resource allocation and the resource utilization efficiency - Increase the overhead and gNB PRACH processing load - gNB would always configure dedicated ROs even for a very small number of RedCap UEs. |

In our view, Option 1 (RF retuning) is a feasible solution with no/small impact on non-RedCap UEs. With proper RF retuning, a UE can retune to an appropriately chosen center frequency for PRACH preamble transmission, so that its preferred RACH occasion is within its transmission bandwidth. In this case, RF retuning delay may need to be accounted for the time gap between the last symbol of the SSB and the start of the PRACH occasion. After transmission of PRACH by RF retuning, the UE may have to retune to another center frequency to monitor for RAR. Since the RAR window may start one symbol after the last symbol of the RACH occasion, the RF retuning time needs to be considered when the network transmits RAR to the RedCap UE. This may require that an early RedCap indication in Msg1 is needed in this scenario. Alternatively, the gNB can apply appropriate RAR transmission time in the cell so that all the UEs, including the RedCap UEs, to have sufficient time to switch between PRACH transmission and RAR reception. This solution may slightly increase the latency, but such a potential increase of latency by only few symbols ought to be acceptable during the initial access.

Option 2 (separate initial UL BWP) can be considered only if the issue of PUSCH resource fragmentation is avoided or minimized. This can be achieved by allowing network to disable PUCCH frequency hopping during initial access and placing the initial UL BWP at the edges of carrier. Moreover, in this case PRACH time-frequency resources for non-RedCap UEs need to be reused for RedCap UEs as much as possible (i.e., having a maximum overlap between PRACH resources for RedCap and non-RedCap), which can prevent degradation of resource utilization since dedicated PRACH resources may not always be utilized.

Option 3 (proper gNB configuration) is a simple solution with minimum specification impacts that can ensure the RACH occasions fall within the RedCap UE bandwidth but with a potential impact on the RACH capacity. Restricting the number FDM-ed RACH occasions can be considered such that the total bandwidth of multiple FDM-ed RACH occasions does not exceed the UE BW. Currently, the number of FDM-ed RACH occasions can be 1, 2, 4, and 8. By using 8 FDM-ed RACH occasions a maximum capacity is achieved. However, a smaller number of RACH occasions can also provide sufficient capacity. Furthermore, in FR2 with the analog beamforming there is no need for having FDM-ed ROs for different SSB beams. Therefore, if needed, the maximum number of FDM-ed RACH occasion can be limited. For example, gNB can configure up to 4 FDM-ed RACH occasions to ensure that all FDM-ed RACH occasions fall within the RedCap UE bandwidth. With proper gNB configuration in cells supporting RedCap UEs, when bandwidth of 8 FDM-ed RACH occasions is greater the RedCap UE bandwidth, the maximum number of FDM-ed RACH occasions (for RedCap and non-RedCap UEs) can be limited to 4.

Option 4 (dedicated PRACH configurations) is a flexible solution and can be used for early indication. However, it increases signaling overhead and gNB processing and can degrade resource utilization efficiency. Also, if not done properly, it can cause PUSCH resource fragmentation due to a separate PRACH configuration.

Overall, our preferred solutions are with the following order:

- Option 1 (RF retuning).

- Option 3 (proper gNB configuration).

- Option 2 (separate initial UL BWP) with no/minimum PUSCH resource fragmentation.

- Option 4 (dedicated PRACH configurations).

For enabling RACH occasion associated with the best/preferred SSB to fall within the RedCap UE bandwidth, Option 4 (dedicated PRACH configurations) is not considered further. Down-select between the following options depending on the choice of initial UL BWP:

    - If a separate initial UL BWP is not introduced for RedCap: Option 1 (RF retuning) or Option 3 (proper gNB configuration).

    - If a separate initial UL BWP is introduced for RedCap: Option 2 (separate initial UL BWP) with no/minimum PUSCH resource fragmentation.

### 3.3.2	Enabling PUCCH and PUSCH within the UE BW

Another potential issue with the bandwidth reduction is related to the frequency hopping for PUCCH and PUSCH in the initial UL BWP during the initial access procedure. PUCCH is used for carrying the ACK/NACK for Msg4. In this case, frequency hopping is configured and the PRBs used for PUCCH are determined based in the initial UL BWP configuration, which may have a bandwidth larger than the maximum RedCap UE bandwidth. A similar problem exists for Msg3 PUSCH if frequency hopping is configured for PUSCH.

To enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth, the following options were considered in RAN1#104e:

Agreements: - Study further whether and how to enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, with the following options: - Option 1: Proper RF-retuning for RedCap (if feasible) - Option 2: Separate initial UL BWP(s) for RedCap - FFS more than one starting PRB position - Option 3: Separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation for the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping) - Option 4: gNB configuration (e.g., always restricting the initial UL BWP to within RedCap UE bandwidth, or restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH) - As an example, with restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH, when the initial UL BWP is the same for RedCap and non-RedCap UEs, the PUCCH (for Msg4/[MsgB] HARQ feedback) and PUSCH (for Msg3/[MsgA]) are within the RedCap UE bandwidth - Other options are not precluded

In Table 2, we list the key advantages and disadvantages/impacts of each option. Moreover, for some of these impacts we present our view and potential remedies.

Table 2: Key pros and cons of each option (for PUCCH/PUSCH transmissions).

| Solution | Pros | Concerns/comments/remedies |
| --- | --- | --- |
| Option 1: Proper RF retuning | - Allows the RedCap devices and the normal UEs to share the same initial UL BWP and PUCCH/PUSCH resource. - Uplink resource fragmentation can be avoided. - No/small impact on non-RedCap UEs. | - Requires RF retuning. However, RF retuning does not need be done frequently in this case and may not be even needed in most cases. RF retuning is a common behavior for LTE-M UEs, and it does not prevent having UEs with low complexity/energy consumption. - Performance loss due to loss of few symbols and loss of orthogonality between time-domain orthogonal cover codes (OCC) used for PUCCH. To resolve this, PUCCH enhancements need to be introduced for RedCap UEs |
| Option 2: Separate initial UL BWP | - Mostly compatible with existing BWP framework - Also address the out-of-range issue of the best RO. | - Risk of PUSCH resource fragmentation and significant impact on non-RedCap UEs. This can be minimized by allowing network to disable PUCCH frequency hopping during initial access and placing the initial UL BWP at the edges of carrier. - May require different center frequencies for initial UL BWP and DL BWP in TDD. To address this, frequency retuning between DL and UL BWP during initial access can be supported for RedCap (similar to HD-FDD operation). - gNB would always configure more than one initial UL BWP even for a very small number of RedCap UEs. - Increased signaling overhead. |
| Option 3: Separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication | - Simple and feasible solution - Small specification impact. | - Has some constraints: e.g., frequency hopping should be disabled. |
| Option 4: gNB configuration | - Minimum specification impact. - RedCap and non-RedCap UEs can share the same initial UL BWP and initial access configuration for non-RedCap can be reused, | - The flexibility of the network configuration for legacy UE is impacted. - Although no PUSCH resource fragmentation within the BWP, there might be fragmentation over the entire carrier bandwidth. |

PUCCH for Msg4/[MsgB] HARQ feedback:

Frequency hopping is supported for long and short PUCCH formats. Before a dedicated RRC connection, the PUCCH configuration in PUCCH-ConfigCommon from SIB1 only supports short Format 0 with 2 symbols and long Format 1 with 4, 10, and 14 symbols. Also, in this configuration frequency hopping is always applied.

PUSCH for Msg3/[MsgA]:

For the Msg3 PUSCH transmissions, the frequency hopping can be enabled or disabled. Moreover, in case of enabled frequency hopping, three different frequency offset values between two hops can be used ‎[7]. Specifically, for BWP with size Bw, the offset values between hops can be Bw2, Bw4, and -Bw4.

Option 1 (RF retuning) can be potentially used to address this issue with no/small impact on non-RedCap UEs. When feasible, the RedCap UE can properly retune its center frequency between the hops to ensure PUCCH/PUSCH is transmitted as in legacy without falling outside the RedCap device BW.

In NR, the RF retuning time for intra-band operation is around 50-200 µs (for sub 6 GHz) ‎[6]. For RedCap UEs, depending on the RF retuning delay in FR1 and FR2, the RF retuning solution can be used for PUCCH and PUSCH transmissions. There will be some performance degradation as some of the symbols will be lost due to the RF retuning delay. Table 3 shows the RF retuning time in terms of number of symbols.

Table 3: RF retuning time in terms of number of symbols.

| SCS | Symbol duration | RF retuning time |  |  |
| --- | --- | --- | --- | --- |
|  |  | 50 µs | 100 µs | 200 µs |
| 15 kHz | 66.67 µs | 0.75 symbols | 1.5 symbols | 3 symbols |
| 30 kHz | 33.33 µs | 1.5 symbols | 3 symbols | 6 symbols |
| 60 kHz | 16.67 µs | 3 symbols | 6 symbols | 12 symbols |
| 120 kHz | 8.33 µs | 6 symbols | 12 symbols | 24 symbols |

As we can see, the performance loss depends on the RF retuning time, SCS, and the number of PUCCH symbols. The RF retuning solution seems to be more feasible in FR1 and long PUCCH formats. Losing a few symbols due to RF retuning delay can result in the loss of orthogonality between time-domain orthogonal cover codes (OCC) used for UE multiplexing in PUCCH long Format 1. Consequently, it can impact the multiplexing of RedCap and non-RedCap UEs in shared PUCCH resources.

Regarding Option 2, a separate initial UL BWP can also be configured to ensure that PUCCH/PUSCH transmissions fall within the UE bandwidth. However, it is essential to prevent PUSCH resource fragmentation by disabling the PUCCH frequency hopping and proper placement of the RedCap initial UL BWP with respect to the location of the non-RedCap initial UL BWP. It is possible to have more than one starting PRB position for the BWP if this does not cause PUSCH resource fragmentation. For example, there can be two starting PRB positions for placing the RedCap initial UL BWP at the high-edge or low-edge of non-RedCap initial UL BWP (or a given carrier).

Option 3 (separate PUCCH/PUSCH configuration/indication) can also be considered as a solution, if done properly. It is important that a new configuration avoids resource fragmentation in the initial UL BWP. One way to achieve this is to introduce a new PUCCH configuration without frequency hopping and PUCCH resources located at a BWP edge. Note that for PUSCH it is already possible to disable frequency hopping during initial access.

Option 4 (gNB configuration) can be considered especially when it is feasible to restrict the initial UL BWP to be within RedCap UE bandwidth and the potential impact on the non-RedCap UEs is acceptable. Restricting the bandwidth of initial UL BWP may not be desired for non-RedCap UEs due to a potential impact on the frequency diversity as well as PUSCH resource fragmentation at the carrier level which degrades the UL peak data rate. In this case, the entire carrier bandwidth may not be utilized, and, with the PUCCH frequency hopping, PUSCH resource fragmentation is unavoidable. The advantage of this option is its minimum specification impact and the possibility of using a single BWP for both RedCap and non-RedCap UEs. Hence, this solution can be considered depending on the scenario (e.g., channel condition) and only if the potential impact on the non-RedCap UEs is acceptable. Alternatively, restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH might be considered to ensure that these transmissions fall within the RedCap UE BW. However, there is only a limited flexibility in terms of location of Msg4 PUCCH and currently the PUCCH frequency hopping cannot be disabled during initial access. For Msg3/[MsgA] PUSCH, gNB configurations can be a more effective solution as it can disable PUSCH frequency hopping and/or choose a suitable frequency offset between two hops (among existing values Bw2, Bw4, and -Bw4) for RedCap and non-RedCap UEs to ensure that PUSCH is within the RedCap UE BW.

Overall, our preferred solutions are with the following order:

- Option 3 (separate PUCCH/PUSCH configuration/indication) with the possibility of disabling PUCCH frequency hopping and avoiding/minimizing the PUSCH resource fragmentation.

- Option 2 (separate initial UL BWP) with no/minimum PUSCH resource fragmentation.

- Option 1 (RF retuning) when feasible.

- Option 4 (proper gNB configuration) only when it is feasible to restrict the initial UL BWP to be within RedCap UE bandwidth and the impact on the non-RedCap UEs is acceptable in terms of frequency diversity and UL peak data rate.

To handle the issue of PUCCH/PUSCH transmissions that may fall outside the RedCap UE bandwidth, the RF retuning solution (Option 1) is only feasible for FR1 and long PUCCH formats.

To ensure that PUSCH transmissions during initial access fall within the RedCap UE BW, it is already possible with gNB configuration to disable PUSCH frequency hopping or adopt a suitable frequency offset value between hops.

In Rel-15 NR specifications, the PUCCH frequency hopping is always enabled during the initial access.

Always restricting the size of a shared initial UL BWP to be within the RedCap UE bandwidth can have negative impacts on non-RedCap UEs.

For enabling PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) to fall within the RedCap UE bandwidth, Option 4 (proper gNB configuration) is not considered further. Down-select between the following options depending on the choice of initial UL BWP:

    - If a separate initial UL BWP is not introduced for RedCap: Option 3 (separate PUCCH/PUSCH configuration/indication) with the possibility of disabling PUCCH frequency hopping and avoiding/minimizing the PUSCH resource fragmentation, or Option 1 (RF retuning, when feasible).

    - If a separate initial UL BWP is introduced for RedCap: Option 2 (separate initial UL BWP) with no/minimum PUSCH resource fragmentation.

# 4	Operation on a non-initial BWP wider than the UE BW

For non-initial BWP operation, we have the following working assumption based on RAN1#104bis-e:

Working assumption: - A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

According to the main bullet in the working assumption, a RedCap UE cannot be configured with a non-initial (DL or UL) BWP wider than the maximum bandwidth of the RedCap UE. We think this makes sense and that the main bullet in the working assumption can be confirmed. Our analysis of the sub-bullet in the working assumption will be discussed further in the remainder of this section.

Confirm the main bullet in the working assumption that “A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE”.

Regarding UE features in TR 38.822 ‎[8], under feature “6. CA/DC, BWP, SUL”, there are a few feature groups (FGs) related to the BWP operation which require consideration for RedCap. Specifically, FGs 6-1 and 6-1a as listed in Table 4.

Table 4: UE feature list for BWP operation ‎[8].

| Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET #0 (if CORESET #0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | n/a | n/a | This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signaling |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET #0 (if CORESET #0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | bwp-WithoutRestriction | BandNR | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signaling |

Note that as listed in the above table, FG 6-1a is applicable to the following features:

- FG 6-1: Basic BWP operation with restriction

- FG 6-2: Type A BWP adaptation with same numerology

- FG 6-3: Type B BWP adaptation with same numerology

- FG 6-4: BWP adaptation with different numerologies

Below, we will discuss two issues: SSB/CORESET #0 multiplexing, and non-initial UL/DL BWPs in TDD.

Issue #1: SSB/CORESET #0 multiplexing:

As we can see from FG “6-1: Basic BWP operation with restriction”, it is mandatory for the non-RedCap UEs to support BWP with bandwidth restriction, i.e., an RRC configured DL BWP includes CORESET #0 and SSB (this helps to avoid RF retuning). However, RedCap UEs are not able to simultaneously receive SSB and CORESET #0 for one special CORESET #0/SSB multiplexing pattern in FR2, namely pattern 2 for 240 kHz SSB and 120 kHz PDCCH SCS.

Based on ‎[7] (TS 38. 213, Table 13-10), only the cases listed in Table 5 result in a total bandwidth larger than 100 MHz (around 126-128 MHz) which exceed the RedCap UE bandwidth in FR2. In the table, kssb is the number of subcarriers indicating SSB offset from the PRB grid.

Table 5: Cases that exceed RedCap UE bandwidth in FR2, {SS/PBCH block, PDCCH} SCS is {240, 120} kHz, multiplexing pattern 2.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 6 | 2 | 48 | 1 | -41 if kssb=0 -42 if kssb>0 |
| 7 | 2 | 48 | 1 | 49 |

These special configurations with 240 kHz SSB SCS and 120 kHz PDCCH SCS are not expected to be commonly used in a network. However, if such a configuration is needed in some special case, BWP operation “without restriction” must be mandatory for RedCap (i.e., BWP does not need to include both SSB and CORESET #0) in FR2. Therefore, one of the following two options can be considered for RedCap:

- Option 1: Like non-RedCap UEs, RedCap UEs are required to support FG 6-1 (mandatory without capability signaling) while FG 6-1a and other applicable FGs are optional. However, a cell supporting RedCap cannot use the specific SSB/CORESET #0 configurations in FR2 that require a bandwidth larger than the RedCap UE bandwidth.

- Option 2: A cell supporting RedCap can use all SSB/CORESET #0 configurations. However, in addition to FG 6-1, RedCap UEs are also required to support FG 6-1a (and its applicable FGs) implying that an RRC configured DL BWP does not need to contain both SSB and CORESET #0.

It can be noted that Option 2 provides somewhat more flexibility than Option 1.

Issue #2: Non-initial UL/DL BWPs in TDD:

Generally, it is desired to place UL BWPs close to the edges of a given carrier to minimize the PUSCH resource fragmentation due to PUCCH transmissions. In TDD scenarios, if the UL BWP and DL BWP should have the same center frequency, the DL BWP also needs to be placed close to the edge of the carrier. For RedCap UEs with reduced bandwidth, the DL BWP may not cover CORESET #0. This case is more likely in FR1 with only 20 MHz RedCap UE bandwidth than in FR2 with a 100 MHz bandwidth. Therefore, the DL BWP may not contain CORESET #0, and thus BWP operation with restriction may not be always feasible for RedCap UEs.

In TDD scenarios, a non-initial DL BWP for RedCap UEs may or may not contain CORESET #0, and thus BWP operation with restriction may not always be feasible for RedCap UEs due to the UE bandwidth limitation.

Considering both Issue #1 and Issue #2 discussed above, we have the following proposal:

RedCap UEs support FG “6-1a-BWP operation without restriction on BW of BWP(s)”, implying that an RRC-configured DL BWP does not need to contain both SSB and CORESET #0.

# 5	BWP switching and RF retuning

## 5.1 	BWP switching options

During RAN1#104bis-e, different aspects of BWP switching, BWP retuning, or BWP hopping were discussed in several contributions. One motivation for such features could potentially be to alleviate any reduced frequency diversity due to the reduced bandwidth of a RedCap device, and they may further provide a tool for managing UL resource fragmentation as discussed e.g. in ‎[3]. Based on the RAN1#104bis-e contributions, the feature lead summary and subsequent e-mail discussion as summarized in ‎[12] included a question presenting three options, not precluding others:

What mechanism(s), if any, should be supported in terms of fast BWP switching, BWP retuning, or BWP hopping? - Option 1: No faster BWP switching or hopping mechanisms are introduced for RedCap UEs. - Option 2: Fast switching between BWPs is introduced, e.g. with only changed centre frequency and/or other constraints. - Option 3: BWP retuning or hopping (for a single BWP) is introduced.

Option 2 would enable faster BWP switching than the Rel-15 behavior by introducing constraints that the properties of the BWPs would be identical, or at least similar, apart from the center frequency. In that aspect, it would be like Option 3 proposed by some companies, in that a BWP retuning or “virtual” hopping would just adjust the starting common PRB of the BWP. One company suggested that this type of “virtual” hopping would be beneficial for FR2 but not for FR1.

Based on the summary in ‎[12], there was no consensus on this topic. Most companies expressed some concern that any deviation from the Rel-15 BWP framework may not be worth the specification effort if no convincing arguments backed up with simulation results are presented. Furthermore, it was commonly argued that RAN1 cannot agree on introducing any behavior different from Rel-15 without having RAN4 confirming the feasibility. We share these basic concerns, but we don’t want to exclude the possibility at this point.

Since both Option 2 and 3 rely on the same constraints in the RF retuning, we see no fundamental difference between them in terms of feasibility. Option 2 would have the advantage that the Rel-15 BWP concept can be retained, at the cost of requiring reservation of one BWP for each configured center frequency. Option 3 would not have this disadvantage but may instead require more specification effort. Furthermore, other impacts on legacy NR devices and networks must be considered. This includes preventing that even higher degree of UL spectrum fragmentation is introduced due to potential PUCCH transmissions occurring at the interior of the UL carrier in case of UL BWP hopping.

Fast BWP switching, retuning, or hopping may be considered, pending RAN4 confirming the feasibility, and provided sufficient gains can be shown. Otherwise, BWP switching behavior for RedCap UEs follows the Rel-15 behavior.

If RAN1 concludes that the introduction of any modified BWP behavior is considered worth the specification effort, we see no reason to restrict such behavior to apply only for some frequency bands or frequency regions, but not for others. Even if some performance gains may be more pronounced in some scenarios related to, for example, operation in FR2, some aspects of modified BWP behavior may prove advantageous irrespective of the frequency region, or even play a bigger role in deployment scenarios typically occurring in FR1. We therefore propose the following.

If any new or modified BWP switching, retuning, or hopping behavior is introduced for RedCap UEs, such behavior shall be supported both in FR1 and FR2.

## 5.2 	RF switching and BWP switching delay

As discussed in the previous sections, there are several open issues awaiting agreements between solutions that require RF switching, BWP switching or other alternatives such as limiting DL and/or UL transmissions within RedCap UE bandwidth. Several techniques mentioned that require RF switching or BWP switching may or may not have adverse impacts on the network and/or RedCap UEs performance. To provide proper analysis and compare different solutions, it would be beneficial to have appropriate understanding of what assumptions for RF switching time and/or BWP switching time are feasible for RedCap UEs.

In general, RF switching is needed in scenarios when there are changes of center frequencies, SCS, and/or bandwidths. Different scenarios may result in different transient delays requirements in different RF (e.g. filtering transients, sampling rate change delays, local oscillator retuning, AGC settling time etc.) and/or baseband (e.g. DCI decoding, configuration reconfigurations, etc.) components. By introducing certain constraints (e.g. keeping the BW, SCS the same but only change in the center frequency), as discussed in Section 5.1, delays in certain components may be eliminated or reduced.

In ‎[6], it is estimated that RF retuning time for intra-band operation for non-RedCap UEs is around 50-200 µs (for sub 6 GHz). And in TS 38.133 ‎[13], it is specified that the DCI-based BWP switching delays are up to 1 ms for FR1 and 0.75 ms for FR2 for non-RedCap UEs with BWP switch delay Type 1 capability (note that Type 2 capability requires longer switching delay). The figures provided in the LS and in the current specification, consists of delay components such as AGC settling time, BWP adaptation delay, PDCCH decoding time, etc. With the proposed BWP switching mechanisms (such as BWP switching with preconfigured pattern or new timer based BWP switching) captured in ‎[12], It may not be suitable to adapt the same numbers in the analysis for RedCap use cases / scenarios and/or devices with reduced complexity. Clarification from RAN4 could be beneficial (at least for comparison purpose).

Table 6: BWP switch delay (Table 8.6.2-1 in 3GPP TS 38.133)

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Nevertheless, in the previous RAN1#104bis-e meeting, consensus for sending a LS to RAN4 could not be reached as some companies have concerns that sending the LS may have implications in favoring specific solutions. In our view, the draft LS ‎[14] has been sufficiently generalized to avoid specific use cases and the intention of sending the LS is purely to seek a basic component to facilitate proper analysis and progress with adequate solutions for RedCap.

Send the draft LS ‎[14] to RAN4 to ask for the RF retuning delays suitable for RedCap UEs.

# 6	Summary

Here, we summarize our views on the open issues concerning the RedCap UE BW reduction aspects.

Terminology:

First, it is important to establish a common understanding of certain important terminology. Specifically,

- “BWP#0” and “initial BWP” are often used interchangeably in the NR specifications.

- An initial BWP (BWP#0) can also be used after the initial access regardless whether it is configured using option 1 (without UE-specific configuration) or option 2 (with UE-specific configuration).

- An initial BWP has an index 0 (i.e., BWP#0) while a non-initial BWP has a non-zero index.

Initial DL BWP:

For DL, we think it would be beneficial to have the possibility of configuring a separate or additional bandwidth and location for initial DL BWP for RedCap UEs.

| Option/solution | Pros | Cons/impacts/remedies |
| --- | --- | --- |
| Separate initial DL BWP during initial access | - Initial BWP is within the RedCap BW. - More flexibility (e.g., in TDD scenarios). - Helps for offloading purposes. | - Higher signaling overhead, impact on scheduling efficiency. - Need to introduce new IE (RedCap-specific initial DL BWP) in SIB1). - Remedy: minimize/avoid duplicating information. |
| Operation on an initial DL BWP wider than the UE BW after initial access for BWP#0 configuration option 2 | Possibility of operating with a single BWP using BWP. No need to prioritize solutions for BWP#0 configuration option 2. | Specification changes needed to allow configuring a BWP wider than the UE BW. |

Initial UL BWP:

For UL, our most important consideration is to avoid PUSCH resource fragmentation, which may result in a significant UL peak data rate reduction for non-RedCap UEs not supporting almost contiguous UL CP-OFDM (Feature 2-7 defined in TR 38.822) or resource allocation Type 0 for PUSCH (Feature 5-2 defined in TR 38.822) in the uplink. Considering these two features are optional with capability signaling, PUSCH resource fragmentation needs to be avoided to achieve good coexistence performance between RedCap and non-RedCap UEs, which is an important objective according to the WID ‎[1] and also discussed in ‎[3].

Sharing an initial UL BWP configured up to the entire carrier bandwidth is one way for avoiding PUSCH resource fragmentation, especially during initial access when PUCCH FH is always enabled. Alternatively, a separate initial UL BWP can be considered for RedCap, but only if the PUSCH resource fragmentation is avoided or minimized somehow.

| Option/solution | Pros | Cons/impacts/important consideration |
| --- | --- | --- |
| Option 1: operation on an initial UL BWP wider than the UE BW | - Avoids PUSCH resource fragmentation - Better resource utilization | - Specification changes needed to allow configuring a BWP wider than the UE BW (e.g., to support PUCCH/PUSCH transmissions and RACH occasions to fall within UE BW). - Important considerations: - Need to prioritize solutions with small specification impacts. - Need to send an LS to RAN4 to ask for the RF retuning delays suitable for RedCap UEs. |
| Option 2: separate initial UL BWP for RedCap | - Initial BWP is within the RedCap BW - PUCCH/PUSCH and FDM-ed RACH occasions can be guaranteed to fall inside RedCap UE bandwidth. | - Risk of PUSCH resource fragmentation. - Important consideration: enhancements needed for avoiding or minimizing PUSCH resource fragmentation - Allow PUCCH FH to be disabled during initial access - Allow flexible DL/UL center frequencies during initial access |
| Option 3: non-RedCap initial UL BWP is always limited within RedCap BW |  | - PUSCH resource fragmentation - Important consideration: least favorable solution among the 3 options as this option cannot avoid PUSCH resource fragmentation in certain scenarios |

Additional CORESET for the scheduling of Msg2/Msg4/Paging/SI messages for RedCap UEs:

| Option/solution | Pros | Cons/impacts/remedies |
| --- | --- | --- |
| Additional CORESET | Can help offloading and congestion mitigation if the additional CORESET is not fully overlapping with CORESET #0. | - The UE cannot receive SSB and SI, while monitoring the additional CORESET if the total frequency span of CORESET #0 and the new CORESET exceeds the UE bandwidth. - Remedy: UE can rely on RF retuning. |

Non-initial BWP:

- Operation on a non-initial BWP (BWP with a non-zero index) wider than the UE BW is not supported.

- Due to potential issues related to SSB/CORESET#0 multiplexing and UL/DL BWPs in TDD, RedCap UEs may need to support feature “6-1a - BWP operation without restriction on BW of BWP(s)” (and its applicable features) implying that an RRC configured DL BWP does not need to contain both SSB and CORESET #0.

Enabling RACH occasions within the UE BW:

As explained in section 3.3.1, we prefer Option 1 (RF retuning) and Option 3 (proper gNB configuration) if a sperate initial UL BWP is not introduced for RedCap. Also, Option 2 (separate initial UL BWP) is acceptable with no/minimum PUSCH resource fragmentation if a separate initial UL BWP is introduced for RedCap.

| Option/solution | Pros | Cons/impacts/remedies |
| --- | --- | --- |
| Option 1: RF re-tuning | - Allows the RedCap devices and the normal UEs to share the same initial UL BWP and PRACH resource. - Uplink resource fragmentation can be avoided. - No/small impact on non-RedCap UEs. | - May need to consider additional delay for starting RAR window: a few symbols additional delay during initial access which can be acceptable. - Requires RF retuning: RF retuning does not need be done frequently in this case and may not be even needed in most cases. RF retuning is a common behavior for LTE-M UEs with low complexity/energy consumption. |
| Option 2: Separate initial UL BWP | - Mostly compatible with existing BWP framework - Also enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access | - Risk of PUSCH/PRACH resource fragmentation: can be minimized by allowing network to disable PUCCH frequency hopping during initial access and placing the initial UL BWP at the edges of carrier. - May require different center frequencies for initial UL BWP and DL BWP in TDD: frequency retuning between DL and UL BWP during initial access can be supported for RedCap. - Increased gNB processing for PRACH: ROs should be shared between non-RedCap and RedCap UEs as much as possible. - Increased signaling overhead. |
| Option 3: gNB configuration | Feasible and flexible solutions, minimum specification impact. | Potential impact on PRACH capacity if restrictions are applied: sufficient capacity can still be achieved with less than 8 FDM-ed RACH occasions (e.g., 4 FDM-ed RACH occasions). Also, multiplexing in the time domain can be used to increase PRACH capacity. |
| Option 4: Dedicated PRACH configurations | FDM-ed RACH occasions can be guaranteed to fall inside RedCap UE bandwidth. | - May complicate the gNB’s resource allocation and the resource utilization efficiency - Increase the overhead and gNB PRACH processing load - gNB would always configure dedicated ROs even for a very small number of RedCap UEs. |

Enabling PUCCH and PUSCH within the UE BW:

As explained in section 3.3.2, we prefer Option 3, e.g. disabling frequency hopping or using a different frequency hopping pattern. Also, Option 2 (separate initial UL BWP) is acceptable with no/minimum PUSCH resource fragmentation if a separate initial UL BWP is introduced for RedCap.

| Option/solution | Pros | Cons/impacts/remedies |
| --- | --- | --- |
| Option 1: RF re-tuning | - Allows the RedCap devices and the normal UEs to share the same initial UL BWP and PRACH resource. - Uplink resource fragmentation can be avoided. - No/small impact on non-RedCap UEs. | - Requires RF retuning: RF retuning does not need be done frequently in this case and may not be even needed in most cases. RF retuning is a common behavior for LTE-M UEs with low complexity/energy consumption. - Performance loss due to loss of few symbols and loss of orthogonality between time-domain orthogonal cover codes (OCC) used for PUCCH: PUCCH enhancements need to be introduced for RedCap UEs. |
| Option 2: Separate initial UL BWP | - Mostly compatible with existing BWP framework. - Also address the out-of-range issue of the best RO. | - Risk of PUSCH resource fragmentation: can be minimized by allowing network to disable PUCCH frequency hopping during initial access and placing the initial UL BWP at the edges of carrier. - May require different center frequencies for initial UL BWP and DL BWP in TDD: frequency retuning between DL and UL BWP during initial access can be supported for RedCap. - Increased signaling overhead. |
| Option 3: Separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication | - Simple and feasible solution - Minimum specification impact. - RedCap UEs can share RACH occasions with legacy UEs. | Has some constraints: e.g., frequency hopping should be disabled. |
| Option 4: gNB configuration | - Minimum specification impact. - RedCap and non-RedCap UEs can share the same initial UL BWP and initial access configuration for non-RedCap can be reused. | Potential impact on non-RedCap UEs if restrictions are applied to support RedCap. Although no PUSCH resource fragmentation within the BWP, there might be fragmentation over the entire carrier bandwidth. |

BWP switching and RF retuning:

- Fast BWP switching, retuning, or hopping may be considered, pending RAN4 confirming the feasibility, and showing sufficient gains. Otherwise, Rel-15 BWP switching mechanisms are used for RedCap UEs.

- Send the draft LS to RAN4 to ask for the RF retuning delays suitable for RedCap UEs.

# 7	Conclusion

In the previous sections we made the following observations:

Observation 1	In Rel-15 NR specifications, the DL transmissions during the initial/random access (e.g., Msg2 and Msg4) are confined within the CORESET #0 bandwidth which does not exceed the RedCap UE bandwidth.

Observation 2	From scheduling complexity, resource utilization, and signaling overhead point of view, it can be beneficial to have a shared initial DL BWPs for RedCap and legacy UEs.

Observation 3	Using separate initial DL BWP for RedCap can be beneficial for flexibility and offloading purposes.

Observation 4	It would be beneficial to have the possibility of configuring a separate or additional bandwidth and location for initial DL BWP for RedCap UEs.

Observation 5	Supporting BWP#0 configuration option 2 requires a RedCap UE to operate on a BWP wider than its maximum UE bandwidth. If the network needs to configure multiple BWPs, it loses a major incentive to use BWP#0 configuration option 2 that allows operation with a single large BWP.

Observation 6	Since it is expected that most of the networks that support BWP#0 configuration option 2 today will migrate to BWP#0 configuration option 1 in the next few years, there is no need to prioritize solutions for supporting RedCap for BWP#0 configuration option 2.

Observation 7	In scenarios where CORESET #0 might be congested due to a high number of UEs, having an additional CORESET can be beneficial for traffic offloading.

Observation 8	An additional CORESET which is partially or fully non-overlapping (in the frequency domain) with CORESET #0 can be configured for offloading purposes during the initial/random access. However, in introducing an additional CORESET, the RedCap UE bandwidth limitation as well as proper reception of SSB and SIB1 need to be considered.

Observation 9	The additional CORESET can be defined within the RedCap initial DL BWP and used for offloading Msg2, Msg4, paging and SI (other than SIB1) message transmissions, while CORESET #0 is used for scheduling SIB1.

Observation 10	A Rel-15/16 UE may not support non-contiguous PUSCH frequency resource allocation as the features of supporting ‘almost contiguous UL CP-OFDM’ (Feature 2-7 defined in TR 38.822) and ‘resource allocation Type 0 for PUSCH’ (Feature 5-2 defined in TR 38.822) are optional with capability signaling.

Observation 11	Any PUSCH resource fragmentation reduces the UL peak data rate for non-RedCap UEs which do not support non-contiguous PUSCH frequency resource allocation.

Observation 12	If the PUSCH resource fragmentation is avoided, a SIB-configured initial UL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial UL BWP for non-RedCap UEs. This can be achieved by properly adjusting the position of RedCap initial UL BWP and disabling the PUCCH frequency hopping.

Observation 13	In TDD scenarios, if a separate initial UL BWP for RedCap is defined, it may be beneficial to also have a separate initial DL BWP for maintaining the same center frequency for the UL and DL BWPs.

Observation 14	Always limiting the bandwidth of the non-RedCap initial UL BWP to be within the RedCap UE bandwidth can have a negative impact on non-RedCap UEs.

Observation 15	The initial UL BWP for non-RedCap UEs can be wider than the RedCap UE bandwidth.

Observation 16	After initial access, the UE capability is known, and thus the network can schedule uplink transmissions to be within the RedCap UE bandwidth although the UE is configured with an UL BWP wider than its maximum UE bandwidth.

Observation 17	After initial access, it is possible that a UE switches back to the initial UL BWP for random access purposes when RACH occasions are not configured for the active UL BWP of the serving cell.

Observation 18	The need for using initial UL BWP after initial access can be avoided by ensuring that the active UL BWP includes RACH occasions.

Observation 19	After the initial access, it is already possible to disable PUCCH frequency hopping.

Observation 20	To handle the issue of PUCCH/PUSCH transmissions that may fall outside the RedCap UE bandwidth, the RF retuning solution (Option 1) is only feasible for FR1 and long PUCCH formats.

Observation 21	To ensure that PUSCH transmissions during initial access fall within the RedCap UE BW, it is already possible with gNB configuration to disable PUSCH frequency hopping or adopt a suitable frequency offset value between hops.

Observation 22	In Rel-15 NR specifications, the PUCCH frequency hopping is always enabled during the initial access.

Observation 23	Always restricting the size of a shared initial UL BWP to be within the RedCap UE bandwidth can have negative impacts on non-RedCap UEs.

Observation 24	In TDD scenarios, a non-initial DL BWP for RedCap UEs may or may not contain CORESET #0, and thus BWP operation with restriction may not always be feasible for RedCap UEs due to the UE bandwidth limitation.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm the working assumption that “During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth” and its sub-bullets.

Proposal 2	The network can optionally configure a separate or additional bandwidth and location for initial DL BWP for RedCap.

Proposal 3	Confirm the working assumption that after initial access a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth for both BWP#0 configurations option 1 and option 2.

Proposal 4	An additional CORESET can be configured for scheduling of Msg2, Msg4, paging and SI (other than SIB1) messages for RedCap UEs.

Proposal 5	During the initial access, a RedCap UE is allowed to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth. This requires resolving issues related to PUCCH/PUSCH frequency hopping as well as FDM-ed RACH occasions that may fall outside the UE bandwidth.

Proposal 6	The network should be allowed to disable the PUCCH frequency hopping (for Msg4/[MsgB] HARQ feedback) for RedCap UEs during initial access.

Proposal 7	For initial UL BWP, Option 3 (which does not allow configuring non-RedCap initial UL BWP wider than the RedCap UE bandwidth) is not further considered.

Proposal 8	If during initial access the UE is allowed to operate on UL BWP#0 wider than its maximum bandwidth, it should also be allowed to operate on UL BWP#0 after initial access.

Proposal 9	If during initial access a separate initial UL BWP is configured for RedCap, the RedCap UE can also operate on this separate initial UL BWP after the initial access.

Proposal 10	During and after initial access, Option 3 (which does not allow configuring non-RedCap initial UL BWP wider than the RedCap UE bandwidth) is not further considered.

Proposal 11	For enabling RACH occasion associated with the best/preferred SSB to fall within the RedCap UE bandwidth, Option 4 (dedicated PRACH configurations) is not considered further. Down-select between the following options depending on the choice of initial UL BWP:

i.	If a separate initial UL BWP is not introduced for RedCap: Option 1 (RF retuning) or Option 3 (proper gNB configuration).

ii.	If a separate initial UL BWP is introduced for RedCap: Option 2 (separate initial UL BWP) with no/minimum PUSCH resource fragmentation.

Proposal 12	For enabling PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) to fall within the RedCap UE bandwidth, Option 4 (proper gNB configuration) is not considered further. Down-select between the following options depending on the choice of initial UL BWP:

i.	If a separate initial UL BWP is not introduced for RedCap: Option 3 (separate PUCCH/PUSCH configuration/indication) with the possibility of disabling PUCCH frequency hopping and avoiding/minimizing the PUSCH resource fragmentation, or Option 1 (RF retuning, when feasible).

ii.	If a separate initial UL BWP is introduced for RedCap: Option 2 (separate initial UL BWP) with no/minimum PUSCH resource fragmentation.

Proposal 13	Confirm the main bullet in the working assumption that “A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE”.

Proposal 14	RedCap UEs support FG “6-1a-BWP operation without restriction on BW of BWP(s)”, implying that an RRC-configured DL BWP does not need to contain both SSB and CORESET #0.

Proposal 15	Fast BWP switching, retuning, or hopping may be considered, pending RAN4 confirming the feasibility, and provided sufficient gains can be shown. Otherwise, BWP switching behavior for RedCap UEs follows the Rel-15 behavior.

Proposal 16	If any new or modified BWP switching, retuning, or hopping behavior is introduced for RedCap UEs, such behavior shall be supported both in FR1 and FR2.

Proposal 17	Send the draft LS ‎[14] to RAN4 to ask for the RF retuning delays suitable for RedCap UEs.

# References

RP-210918, “Revised WID on support of reduced capability NR devices”, Nokia and Ericsson, 3GPP TSG RAN #91e, March 2021.

R1-2104027, “RAN1 agreements for Rel-17 NR RedCap”, Rapporteur (Ericsson), 3GPP TSG RAN1 #104bis-e, April 2021.

R1-2104184, “Ensuring coexistence between RedCap and non-RedCap UEs”, Ericsson, Deutsche Telekom, NTT DOCOMO, Softbank, Telecom Italia, Telstra, Verizon Wireless, Vodafone, 3GPP TSG RAN1 #105e, May 2021.

TS 38.331 V16.4.1, “Radio Resource Control (RRC) protocol specification (Release 16)”, March 2021.

TR 38.875 V17.0.0, “Study on support of reduced capability NR devices (Release 17)”, March 2021.

R1-1704179, “Reply LS on UE RF Bandwidth Adaptation in NR”, 3GPP TSG RAN1 #88bis, April 2017.

TS 38.213 V16.5.0, “Physical layer procedures for control (Release 16)”, March 2021.

TR 38.822 V15.0.1, “User Equipment (UE) feature list”, July 2019.

TS 38.214 V16.5.0, “Physical layer procedures for data (Release 16)”, March 2021.

TS 38.306 V16.4.0, “User Equipment (UE) radio access capabilities (Release 16)”, March 2021.

TS 38.211 V16.5.0, “Physical channels and modulation (Release 16)”, March 2021.

R1-2103944, “FL summary #4 on reduced maximum UE bandwidth for RedCap”, Moderator (Ericsson), 3GPP TSG RAN1 #104bis-e, April 2021.

TS 38.133 V17.1.0, “Requirements for support of radio resource management (Release 17)”, March 2021.

R1-2104046, “[Draft] LS on RF switching time for RedCap UE”, Ericsson, 3GPP TSG RAN1 #104bis-e, April 2021.