3GPP TSG RAN WG1 Meeting #105-e	R1-2104202

e-Meeting, May 19th – May 27th, 2021

Agenda Item:	8.1.2.3

Source:	Futurewei

Title:	Beam management for simultaneous multi-TRP transmission with multi-panel reception

Document for:	Discussion and decision

# Introduction

In RAN1#104b-e meeting, the Rel. 17 NR FeMIMO WID [1] was discussed. The following was agreed [2] on the topic of enhancement on multi-beam operation:

Agreement For beam reporting option 2 - On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e: - Alt1: Support maximum value N = {1, 2} - Alt2: Support maximum value N = {1, 2, 3, 4} - FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17 - On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e - Alt1: The value of N is fixed by RRC configuration - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE Agreement On CMR resource configuration for beam reporting option 2, adopt the following alternative: - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS: extension to aperiodic CMR resource setting - Each reported beam pair in a single CSI-report consists of M = 2 SSB-RI / CRI values, where each SSB-RI / CRI points to a CMR resource in a different CMR resource set or subset. - Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above. Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC. Agreement - Support S-DCI and M-DCI in TRP-specific BFR in Rel.17 - S-DCI is low priority, M-DCI is high priority - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization R1-2103906 Moderator summary #2 on M-TRP simultaneous transmission with multiple Rx panels Moderator (CATT) Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement Adopt the following beam failure detection criteria for each BFD-RS set - The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold - X is max{minimal periodicity of BFD RS in the set, 2ms} Agreement A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group - NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group R1-2103996 Moderator summary #3 on M-TRP simultaneous transmission with multiple Rx panels Moderator (CATT) Agreement For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR - FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1) Agreement On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”: - NOTE: the following has been agreed - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset. - FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

In this contribution, we present our views on beam measurement/reporting enhancement and beam failure recovery for multi-TRP, and proposals for moving forward.

# Beam Measurement/Reporting Enhancement for Inter-TRP Beam Pairing

In RAN1#104-e meeting, the following was agreed regarding beam measurement/reporting enhancement to facilitate inter-TRP beam pairing.

Agreement For beam measurement in support of M-TRP simultaneous transmission - Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously - Support M = 2 - Support extending the maximum value of N > 1, exact value FFS - N=1 and N=2 - FFS: Other values larger than 2 - FFS: Whether the UE could report beams are received with different RX beams - Further study the support of option 1 and option 3 - The above applies at least for L1-RSRP - FFS: L1-SINR

With the above agreement, another FFS item is whether different beams reported by the UE that can be received simultaneously should be associated with different TRPs or different UE panels.  Since the goal of the beam measurement/reporting enhancement is to facilitate inter-TRP beam pairing, the different beams reported by the UE that can be received simultaneously should be from different TRP.  Otherwise, if the different beams reported are from the same TRP, those beams cannot be transmitted from the TRP simultaneously since it is not feasible for a single TRP to transmit multiple beams simultaneously in FR2, defeating the purpose of facilitating inter-TRP beam pairing.  Therefore, the different beams should be associated with different TRPs.  On the contrast, if the different beams are associated with different UE panels, although the UE can receive these beams simultaneously by using different UE panel for different beam, it is possible that those different beams are from the same TRP on different time.  And as mentioned earlier, it is not feasible for the TRP to transmit those beams simultaneously.  Therefore, the CSI report cannot facilitate inter-TRP beam pairing.

Based on the above analysis, we have the following proposal:

Proposal 1: Different beams reported by the UE that can be received simultaneously should be associated with different TRPs.

Regarding the number of beam pairs/groups N, we prefer that N can be {1, 2, 3, 4} to provide more flexibility to the gNB for scheduling.  The value of N can be signaled by RRC configuration.

Proposal 2: For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, in a single CSI-report, UE can report N={1, 2, 3, 4) pairs/groups and M (M=2) beams per pair/group,  and different beams within a pair/group can be received simultaneously and are associated with different TRPs.  The value of N is fixed by RRC configuration.

In RAN1#104b-e meeting, the following was agreed regarding CMR resource configuration for beam reporting option 2.

Agreement On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”: - NOTE: the following has been agreed - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset. - FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

As indicated by the above agreement, it needs to be decided whether two CMR resource “sets” or two CMR resource “subsets” should be configured for beam reporting.  Our preference is “set” with the following reasons.  Based on [3], the maximum number of NZP CSI-RS resources and/or SS/PBCH block resources in a CMR resource set is 64.  If “subset” is used, then two subsets, one for each TRP, have to share the same CMR resource set, and the number of RS resources that can be configured for each of the TRP will be much smaller than 64.  For example, assuming the number of RS resources configured for each of the two TRPs are equal, a maximum of only 32 RS resources can be configured for each of the TRP, which is just half of the 64 RS resources allowed if “set” is used instead.  In addition, with the use of “set”, many of the existing specification related to “set” can be reused with minimal specification impact.  Therefore, we support “set”.

Based on the above analysis, we have the following proposal:

Proposal 3: On CMR resource configuration for beam reporting option 2, adopt “set”.  Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set, and where each CMR resource set is associated with a different TRP.

In Rel. 15/16, group-based reporting is supported when the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled' [3].  In this case, the UE reports in a single reporting instance two different CRI or SSBRI for each report setting, where the CSI-RS and/or SSB resources can be received simultaneously by the UE.  The supported report quantities for group-based reporting are 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', and 'ssb-Index-SINR'.

However, in Rel. 15/16, there is no TRP identification associated with different CSI-RS and/or SSB resources.   In Rel. 17, to facilitate inter-TRP beam pairing and association of different beams to different TRPs, a groupID field can be added in the TCI-state information element, which is also discussed in our companion contribution [4].  The groupID field serves as a TRP identification.  Table 1 shows an example of how to add the groupID field to the TCI-state information element.  In Table 1, the red parts are the proposed additions/modifications to the TCI-state information element.  Below we focus on the introduction of groupID and physCellId which are more relevant to the topic.  Details on other parts can be found in [4].

- physCellId: this is the PCI of the serving cell or non-serving cell.  With the PCI info added to the TCI state, the TCI states of non-serving cell can also be configured for the UE, which could enable the UE with faster candidate beam identification and BFR utilizing the non-serving cell’s beams.

- groupID: in mTRP case, this ID could serve as the TRP identification.  With this ID, the UE can, for example, identify the TRP source of the CSI-RS.  The UE can then avoid reporting multiple beams from the same TRP to facilitate inter-TRP beam pairing, considering the fact that it is not feasible for one TRP to transmit multiple beams simultaneously in FR2.

Table 1: Example of enhancements on content of TCI state

TCI-State information element -- ASN1START -- TAG-TCI-STATE-START TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ... } QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R physCellId PhysCellId OPTIONAL, groupId GroupId OPTIONAL, bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index, srs SRS-ResourceId }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } -- TAG-TCI-STATE-STOP -- ASN1STOP

Based on the above analysis, we have the following proposal:

Proposal 4: FeMIMO supports adding TRP identification (e.g., groupID) to TCI state for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing.

# Beam Failure Recovery

In RAN1#103-e, #104-e, and #104b-e meetings, the following was agreed regarding beam failure recovery.

Agreement - For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set. - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP - Support at least one of explicit and implicit BFD-RS configuration - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured - FFS: Further study QCL relationship between BFD-RS and CORESET - FFS: How to determine implicit BFD-RS configuration, if supported - For M-TRP new beam identification - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured - FFS: detail on association of BFD-RS and NBI-RS - Support the same new beam identification and configuration criteria as Rel.16, including L1-RSRP, threshold Agreement For M-TRP BFR - Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set - FFS: value of N (e.g. fixed in specification, or UE capability) - FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability) Agreement For M-TRP BFR Support 1-to-1 association between each BFD-RS set and an NBI-RS set - FFS: Association details Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1

Regarding explicit and implicit BFD-RS configuration, it is noted that this topic is related to item on enhancements on multi-beam operation that is under discussion in Agenda Item (AI) 8.1.1.  In  AI 8.1.1, it was agreed that on Rel. 17 unified TCI framework, for the separate DL TCI, the source reference signal(s) in M TCIs provide QCL information at least for UE-dedicated reception on PDSCH and for UE-dedicated reception on all or subset of CORESETs in a CC.  However, the value of M (e.g., whether M > 1 is supported or only M = 1 is supported) is not decided.  If the Rel-17 unified TCI framework support M>1 TCIs such that each of the TCI provide QCL information for UE-dedicated reception on a subset of CORESETs in a CC, then for the explicit BFD-RS configuration, the QCL relationship between BFD-RS and CORESET can be defined such that the BFD-RS are QCLed with PDCCH DMRS with respect to 'QCL-TypeD', e.g., similar to those defined in Rel. 15/16.  However, if the Rel-17 unified TCI framework support only M=1 TCI and the TCI provide QCL information for UE-dedicated reception on all of CORESETs in a CC, then the BFD-RS should not be QCLed with PDCCH DMRS with respect to ‘QCL-TypeD’ since there is only one beam available for all the CORESETs.  If the implicit BFD-RS configuration is defined similar to those defined in Rel. 15/16, e.g., the BFD-RS is determined based on the QCL-TypeD RS of the activated TCI states for PDCCH reception, the above consideration also applies.

Based on the above analysis, we have the following proposal:

Proposal 5: FeMIMO supports BFD-RS are QCLed with PDCCH DMRS with respect to 'QCL-TypeD' assuming that the Rel-17 unified TCI framework supports M>1 DL TCIs such that each of the TCI provides QCL information for UE-dedicated reception on a subset of CORESETs in a CC.

# Conclusions

In this contribution, we present our views on enhancement on beam management for multi-TRP.  Based on the discussions in the previous sections we propose the following:

Proposal 1: Different beams reported by the UE that can be received simultaneously should be associated with different TRPs.

Proposal 2: For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, in a single CSI-report, UE can report N={1, 2, 3, 4) pairs/groups and M (M=2) beams per pair/group,  and different beams within a pair/group can be received simultaneously and are associated with different TRPs.  The value of N is fixed by RRC configuration.

Proposal 3: On CMR resource configuration for beam reporting option 2, adopt “set”.  Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set, and where each CMR resource set is associated with a different TRP.

Proposal 4: FeMIMO supports adding TRP identification (e.g., groupID) to TCI state for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing.

Proposal 5: FeMIMO supports BFD-RS are QCLed with PDCCH DMRS with respect to 'QCL-TypeD' assuming that the Rel-17 unified TCI framework supports M>1 DL TCIs such that each of the TCI provides QCL information for UE-dedicated reception on a subset of CORESETs in a CC.

# References

RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung, RAN#86.

3GPP RAN1, RAN1#104b-e, Chairman Notes

TS 38.214, “NR: Physical layer procedures for data (Release 16)”.

R1-2104205, “Enhancement on multi-beam operation”, Futurewei, RAN1#105-e.