3GPP TSG RAN WG1 Meeting #105-e	R1-2104213

e-Meeting, May 19th – 27th, 2021

Agenda Item:	8.2.6

Source:	Futurewei

Title:	Channel access for shared spectrum Beyond 52.6 GHz

Document for:	Discussion/Decision

# Introduction

In RAN # 90 the extensions to WI [1] for NR operation up to 71GHz were approved. According to [1] RAN1 should define:

“Physical layer procedure(s) including [RAN1]:

Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access.

Study, and if needed specify, energy detection threshold enhancement”

Multiple Agreements we reached in RAN1#104-e and RAN1#104b-e based on the above principles. In this document we provide our views on the open issues after recalling the relevant agreements.

# EDT Computation

In RAN1 #104-e the following agreements on energy detection threshold were made:

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

Subsequently in RAN1 #104b-e the following working assumption was reached.

Working assumption:

- For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

We first discuss the merits of the working assumption considering the scenario in which a single LBT process (where each such process has its own sensing/CCA-check) seeks to acquire the channel occupancy time (COT). Here, while we recognize that maximum EIRP can result in a more conservative EDT (as opposed to say a mean over all per-burst EIRP computed over the COT), it allows for easier computation. Indeed, we can also use any upper-bound on the maximum EIRP (where this upper bound need not be achieved by any burst) which would still respect the ETSI BRAN co-existence specification. On the other hand, utilizing the mean EIRP implies much more scheduling restrictions in that all transmissions during the COT must already be known to a degree that allows precise computation of the mean. The latter restrictions can potentially offset any improved spectrum utilization benefits. Moreover, as illustrated in Fig. 1 we can have skewed combinations of peak burst EIRPs and burst durations. Using the mean over per-burst peak EIRP for such a combination can violate the principle behind the ETSI BRAN specification. The latter statement is in the sense that for a substantial duration of the acquired COT the EIRP of a burst can be above Pout, contrary to ETSI BRAN, which we note considers a single transmission burst during the COT and mandates that its EIRP must not exceed Pout.

Fig 1. Skewed bursts within COT

On the other hand, the working assumption does not seem to be quite well matched towards a scenario in which multiple LBT processes (considering different sets of transmit beams) are implemented by an initiator node. The latter scenarios have not been precluded and are still being considered for further study. The principle behind the working assumption should then allow for computing EDT for each such LBT process as the maximum EIRP considering its set of intended transmit beams, i.e., considering each LBT process separately and the transmission bursts it alone enables. We build upon this in a following section where we discuss the relation between sensing and transmit beams.

Observation 1. The working assumption is only well justified when a single LBT sensing is carried out by a node using one sensing beam for all transmit beams intended to be used in the COT.

# LBT Bandwidth

With regards to the operating bandwidth to be used in LBT, the following alternatives were discussed for the single carrier case:

For LBT for single carrier transmission:

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.2. gNB/UE performs LBT over the transmission bandwidth (from the lowest RB to the highest RB used for the transmission)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth.

Our preference is for alternative 1. We realize that alternative 3 allows partial usage in the case that a subset of the checked LBT units is found to be free. However, there is more specification effort involved in terms of signaling to the COT sharing device(s) about the subset LBTs to be used in the COT. Moreover, implementing multiple LBT processes requires new procedures for handling unequal completion times and partial CCA check failures across LBT processes.

Proposal 1: In LBT for single carrier transmission gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

Regarding LBT for multi-carrier transmission in intra-band CA, the following alternatives were discussed in RAN1 #104b-e.

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.3. gNB/UE performs multiple LBT, one for each CC over the transmission bandwidth (from the lowest RB in to the highest RB used for the transmission in the CC)

- Alt CA.4. gNB/UE performs LBT over the transmission bandwidth over all CCs (from the lowest RB in the lowest CC to the highest RB in the highest CC used for the transmission)

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

The down selection among the alternatives for multi-carrier LBT should happen once the same for single-carrier LBT has been accomplished. Our preference is for Alt CA.1 which we view as an intuitive extension of Alt SC.1. We also recognize the benefit of Alt CA.2 which can reduce overall LBT overhead at-least in the low load case wherein the probability of finding all channel bandwidths to be idle is relatively higher. The other alternatives can result in increased complexity and signaling at-least in terms of exploiting subsets of bandwidth units found to be idle across carriers as well as in handling different completion times.

Proposal 2: In LBT for multi carrier transmission gNB/UE support:

- gNB/UE performs multiple LBT, one for each channel bandwidth separately,

- gNB/UE performs single LBT over all CCs.

# LBT Sensing

In the previous two meetings, for a COT with MU-MIMO (SDM) transmission as well as COT with TDM transmission of beams with beam switching, the following two alternatives were discussed:

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover"

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Our position has been to support both these alternatives. In our previous contribution, we proposed to use the spatial domain filters to define the “beam cover” relationship between sensing and transmit as follows:

Proposal 3: For the CCA check procedure, the COT initiating device may use one or multiple spatial domains receive filters. For each transmission during the COT, there should be associated one or multiple spatial domains receive filters used in the CCA check procedure.

The spatial filter domain relation between transmit and receive beams may be used to define an extended beam correspondence type relationship.

Indeed, this relationship is an extension of beam correspondence because it  has to now be defined even for the gNB as well as for beam groups. Hitherto, beam correspondence has been defined as an optional capability at the UE and furthermore on a single beam basis.

An alternate way is to allow for flexible sensing implementation at the LBT initiating node using multiple sensing beams, in which each intended transmit beam is associated with one sensing beam and where multiple intended transmit beams can be associated with this sensing beam. A schematic highlighting this is shown in Fig. 2. We remark here that this set of intended transmit beams is the set that the LBT initiator node anticipates using for transmission during the COT. There is no presumption about knowing either the order in which these beams will be used or the SDM groupings they will be part of, and indeed some beams need not be employed during the COT. Thereby, enough scheduling freedom is still retained which is important given the relatively long duration of a COT (5ms).

The task that remains is to provide a more rigorous definition of “cover”. Towards this end, consider any given subset (or cluster) of transmit beams Cj that the initiating node intends to use. We can define a composite transmit angular power profile (APP) for this subset of beams as follows:

T(j)Θ,ф=maxi∈Cj{TiΘ,ф}                       1,

where we use  TiΘ,ф to denote the APP of the ith beam, with Θ,ф denoting the angular (elevation and azimuth) coordinates.  Here by APP of a beam used for transmission, we mean its far field EIRP profile or pattern. In particular, EIRP profile of a beam at any angular coordinate is defined here as the transmit power required to be given as input to an ideal isotropic radiator so that the power measured from that isotropic radiator at any reference point in the far-field along that angular coordinate,  matches the power measured at the same reference point arising from the transmission of the initiating node that is beamformed using the beam of interest.  EIRP profile (in dBm) is hence equal to the gain profile (in dBi) plus the power supplied (in dBm) to the radiating elements. As a result, the composite transmit APP defined in (1) captures the envelope of the EIRP that can arise from transmissions (either SDM or TDM) using this beam set.

Notice that each composite transmit APP defined in (1) is the “max-hold” pattern over the constituent beam APPs (constituent beam EIRP patterns).

We now proceed to define an intuitive requirement that any sensing beam defined for the set Cj  should satisfy. This requirement is designed to build upon the principle used in the ETSI BRAN document. While the latter document does not explicitly consider beamformed transmissions let alone acquisition of COT in order to transmit along multiple different beams, there is mention of beamformed transmission with regards to a test procedure. Specifically, in Section 5.3.8.2 the test procedure requires placing a continuous-wave jammer (as a proxy for an interferer/ongoing transmission) to test CCA failure at an initiator node. The jammer is aligned to the direction of the intended transmission. While this test only requires the initiator to have sufficient sensing gain at-least in the direction of intended transmission (direction with maximal injected interference to any other nodes), it clearly needs to be expanded to address the scenario of interest to us.

Towards that end, let S(j)Θ,ф denote the angular gain profile (AGP) of the jth sensing beam (this AGP can for instance include sensing beamforming gain along with gain/loss of the sensing assembly). Note here that unlike EIRP of any beam there is no dependence on any transmit power in S(j)Θ,ф. An ideal omni beam sensing pattern will have an AGP S(j)Θ,ф  with identical gain for all angular coordinates (or directions). . Directional sensing patterns on the other hand will favor some directions over the remaining ones. We also let  Ω(j) be a set of angular coordinates representing (elevation and azimuth) angle pairs of interest. One possible choice is to include all angular coordinates {Θ,ф} such that the composite transmit APP  T(j)Θ,ф evaluated at that Θ,ф  is at-least a fraction of the peak, i.e.,

T(j)Θ,ф≥σmaxΘ',ф'TjΘ',ф', where σ is some configurable factor.

Then, the requirement on the sensing beam is given by:

S(j)Θ,ф ≥αj(TjΘ,фTj),  ∀Θ,фϵΩ(j)………….. (2a),

where  α(j)>0 is a scale-factor and Tj=maxΘ',ф'T(j)Θ',ф' is the peak composite transmit APP. In words, (2a) requires that the AGP of the sensing beam dominate the normalized composite transmit APP over the set  Ω(j) of angular coordinates and up-to a factor α(j). One observation on (2a) is that sensing patten should have enough sensing gain at-least directions in which at-least one among the intended TX beams will radiate a larger fraction of input energy, i.e., in directions where T(j)Θ,ф  is relatively larger. Conversely, directions along which all intended TX beams have small gains can be sensed with a relatively smaller gain.  This is a departure from conventional (quasi-)omni sensing generally assumed in 802.11ad and 802.11ay wherein all directions are sensed with (near-)identical gains.

We remark that the scale-factor α(j) (also referred to here as an adjustment factor) can be absorbed in the EDT computation using the following formula that is a simple modification of the one from EN 302 567 v2.1.20. Indeed, we propose to compute EDT for this cluster of beams as

EDT=-80 dBm+10*log10αjPmaxPout+10*log10(Operating Channel BW in MHz) ….(2b)

where Pout is the maximal EIRP over this cluster of beams during the CoT, i.e.,  Pout=maxΘ',ф'T(j)Θ',ф'.  Design principles behind (2b) are the following. Firstly, along each angular direction Θ,ф the LBT initiating node can be considered to be a virtual isotropically radiating node which will transmit with power T(j)Θ,ф upon acquiring the channel. Then, further supposing this virtual node to use omni sensing, appropriate EDT to use would be (2b) but where αj=1 and Pout=T(j)Θ,ф. Given instead a sensing pattern that satisfies (2a) we need to use the EDT in (2b). The latter will ensure that all ongoing transmissions flagged (i.e., detected with enough signal strength to result in the channel being labeled busy) by the virtual node when using omni sensing will also be flagged when using the available sensing satisfying (2a).  Notice here that 1/min⁡(αj,1) can be viewed as a shortfall in the sensing gain over the set of directions Ω(j). This implies that if a given sensing beam is narrower in that it does not have sufficient gain over at-least some prominent directions then it will have a larger shortfall and consequently (conforming to intuition) this will result in smaller EDT as per (2b). Compensating the EDT using this shortfall ensures an appropriate EDT so that lower sensed energy (due to the shortfall in sensing gain) is not compared to the original higher EDT, and thereby respecting LBT requirements.

An issue that can arise with directional LBT is that of blind spots. We illustrate on such situation in Fig.3. Here, there is an ongoing transmission between (TX-A and RX-A) and an LBT process is initiated by TX-B in order to setup a new transmission between (TX-B and RX-B). However, LBT performed at TX-B using a sensing beam-B aligned with the intended transmit beam as depicted Fig.3A cannot sense the ongoing transmission. On the other hand, an augmented sensing beam, wherein a back lobe is added to the original sensing beam, as shown in Fig.3B, can sense the ongoing transmission. Using the augmented sensing beam TX-B will correctly declare channel busy and refrain from transmitting. Note that using the back-lobe is not reverting to an omni sensing pattern and does not lead to excessive exposed nodes. Moreover, the set of directions opposite to the intended transmission can also be determined using the composite transmit APP. The extent or width of the back-lobe and the corresponding sensing gain can be further optimized.  To summarize, our proposal is the following.

Proposal 4: Consider the use of composite transmit angular power profile (APP) of an intended set of transmit beams to design sensing beam that “covers” that intended set:

- Prominent directions of intended transmission, i.e., those for which composite transmit APP is with a fraction of the peak APP, should have relatively large sensing gain.

- For EDT determination, define Pout as the maximum EIRP over that intended set of transmit beams.

- Appropriate EDT incorporates shortfall (if any) in the sensing gain over prominent directions.

- Enable augmented sensing to avoid blind spots without excessive exposed nodes.

Fig: 2. Set of three intended transmit beams are divided into two sets and a sensing beam is defined for each of the two beam sets.

Fig.3A: Sensing beam-B cannot sense existing link TX-A--RX-A. New link TX-B--RX-B creates interference to existing link but is incorrectly allowed. The red tick represents this incorrect pass of CCA check (false positive).

Fig.3B: Sensing beam-B with mirror-lobe (or back-lobe) can sense existing link TX-A--RX-A. TX-B will then correctly refrain from transmitting. The green cross represents the correct fail of CCA check (true negative).

# Cat 2 LBT and Maximum gap

In our opinion Cat 2 LBT, i.e., a short deterministic LBT is useful for a quick evaluation of the channel availability. Such short LBT may be as short as a symbol duration, that is one-shot LBT can be a Cat 2 LBT.

Proposal 5: Introduce Cat 2 LBT for 60GHz unlicensed band operation.

With a Cat 2 LBT supported one can define the device behavior during the COT that facilitates fair coexistence in unlicensed band.

In ETSI EN 302 567 [3] the total time that the equipment initiating transmission makes use of an operating channel is defined as the Channel Occupancy Time. This Channel Occupancy Time (COT) shall be less than 5 ms, after which the initiating device shall perform a new CCA Check.  The document does not specify if the device can have gaps of the channel usage (transmissions) during COT and how long such gaps may be.

For the CCA check procedure a device observes the channel for at least 8us and if the channel is found not occupied defers for a random number (0 to Max number) of empty (not occupied) slots (5us each), where the Max number is 3 or larger.  Thus, the CCA check operation can take up to 23us for the Max number equal to 3. Consequently, if there is a significant gap (i.e., no transmissions from the original node that acquired the channel) during the COT, it might result in another LBT initiator to detect the channel as idle and begin its transmissions. The transmissions from the second initiator node can interfere with and get interfered by the transmissions from the original one when the latter resumes transmitting. A one-shot LBT by the first node will protect the transmissions of the second node. This arrangement also dis-incentivizes adoption of a proactive excessive channel acquisition policy by a node, i.e., a policy under which a node (gNB) acquires a COT (with a relatively long duration of 5ms) even though it anticipates sparse usage. Without a Cat 2 LBT such a node may resume transmissions after a long gap and disrupt transmissions of other co-channel nodes that have initiated transmissions in the interim. However, if a Cat 2 LBT is required before resuming transmissions after a long gap, then that node may be forced to refrain from transmitting once it detects the channel as busy.

Proposal 6: Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, a one-shot LBT is needed to share the COT:

- Where Y (for all SCS) may be the time duration of 3 symbols (@120 kHz SCS

- Where One-shot LBT duration (for all SCS) is the time duration of 1 symbol @ 120kHz SCS

Another use case when CAT 2 LBT can be supported is when per-beam LBT before acquiring a channel is allowed. The justification here is similar to that given for the previous proposal. In particular, a significant gap between the time LBT is carried out for a beam and the time that beam is eventually applied can result in other nodes in the interference footprint of that beam initiating transmission.

Proposal 7:   When independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, an additional requirement on Cat 2 LBT before beam switch during the COT should be specified.

# RX Assistance

It was agreed in the previous RAN1 meetings [2] that the receiver assisted LBT may be beneficial. Further, for the receiver to help, channel sensing and reporting need to be performed.

The goal of the receiver (a.k.a. responding device) assistance is to assess the local resource availability prior to the transmission and inform the transmitter if that resource is available.  A key advantage of receiver assistance is that it allows for mitigation of the hidden node problem. It is recognized that while directional sensing based LBT at the transmitter side reduces number of exposed nodes, it can still be sensitive to the hidden node problem.  Indeed, given the highly directional nature of transmission and reception, interference reciprocity does not hold in many instances, i.e., the transmitter via sensing may be unable to sense a source of interference to the receiver.  In such instances assistance from the receiver is beneficial.   NR already offer multiple tools for interference assessment such as CSI-IM, and NZP-CSI-RS with flexible resource and time configurations (aperiodic, periodic, or semi-persistent). Therefore, a straightforward way to implement a receiver assisted LBT is by extending the existing mechanisms (such as CSI-IM) to the specifics of shared spectrum operation.

Proposal 8: For receiver assisted LBT, support NR CSI-IM based reporting for the clear channel assessment at the receiver.

To provide a report on the resource availability, the receiver needs to measure the received RF power (for instance RSSI) over the provisioned resources (frequency, spatial filter) for a specific duration based on the LBT type and to compare the received energy with the energy detection threshold, and to decide if the resource is clear (available) or not. Then the receiver must report to the transmitter the resource availability for a transmission. The report from the receiver should be timely and as short as possible (to avoid errors and overhead), therefore a simple map of the resource availability is a better tradeoff. Notice here that the transmitter side LBT when done on a per-LBT bandwidth basis can also yield availability information on a per-LBT bandwidth basis, but it incurs the penalty of multiple LBT processes with unequal completion times and furthermore does not fully mitigate the problem of hidden nodes.  Lastly, receiver-side only LBT can address the hidden node problem (by accurately sensing the interference level at that receiver) but it need not be sufficient to protect ongoing transmissions.

Proposal 9: For receiver assisted LBT, the receiver shall report the resource availability prior to the transmission. The RSSI measurement definition may be extended to assess the resource availability, where the resources, type of measurement (for instance Cat2 LBT) shall be provided by the transmitter.

# LBT and No-LBT Signaling and Related Procedures

In some regions LBT is not mandatory for operating in the unlicensed bands in NR_ext_to_71GHz. Over such scenarios, allowing cell specific LBT enable/disable can allow for improved co-existence with a simpler implementation. However, we are open to study use-cases where UE-specific LBT enable/disable signaling can allow further finer optimization.

Proposal 10: For regions where LBT is not mandatory, a no-LBT mode can be defined and switching between LBT mode and no-LBT mode can be supported.

Proposal 11: For indication of LBT mode and no-LBT mode, cell specific (common for all UEs in a cell) indication as part of system information and dedicated RRC signaling should be supported.

• FFS: Supporting UE specific (i.e., different for different UEs in a cell) indication via dedicated RRC signaling.

We do not see enough justification for allowing a per-beam LBT enable/disable signaling. This provides additional option (compared to per-UE LBT enable/disable) mainly in the scenario when a single user communicates using more than one beams, but then it opens up multiple issues in terms different completion times for multiple LBT processes at the same user.

In our opinion operating without LBT and without any additional policy to address coexistence (or managing interference) can result in degraded performance in several cases. In this context, we note that while it is tempting to assume that there is an absence of significant interference in the beyond 52.6 GHz (NR_ext_to_71GHz) frequency range due to narrow beams and high propagation loss, it so happens that such optimistic assumptions need not always hold. Indeed, as demonstrated in [6] for legacy (802.16ad) WiFi devices operating in 60GHz, beams realized via practical devices have considerable sidelobes which, along with out-of-band emissions, can be a source of significant interference. Such interference is particularly detrimental since it is a spurious transmission that does not benefit any receiver.  Moreover, the commonly used default mode in several legacy Wifi devices, at-least in the indoor settings, is to use omni RX beams. This is because while it is recognized that finer RX beamforming can alleviate the interference problem, the caveat is that there can be increased training overhead and higher susceptibility to blocking upon using such beamforming. Thus, there can be vast disparities in the capabilities (in terms of out-of-band emission suppression, TX and RX beamforming capability etc.) and operational settings among co-existing devices. In addition, there is an asymmetry inherent in interference among co-existing links, i.e., one TX-RX link can suffer much more interference from a coexisting active link while imposing substantially less interference on that link. Together, these factors imply that operating without LBT and without any form of interference mitigation policies can lead to reduced system throughput and/or degraded fairness. One approach that seems feasible is for each device to adopt a policy of a certain channel vacation after each acquired CoT, and such a policy will be more effective if it accounts for the disparity prevalent among co-existing devices.

Proposal 12:  In deployments without LBT consider specification of channel vacation policies accounting for disparity among co-existing devices.

- Conclusions

Observation 1. The working assumption is only well justified when a single LBT sensing is carried out by a node using one sensing beam for all transmit beams intended to be used in the COT.

Proposal 1: In LBT for single carrier transmission gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

Proposal 2: In LBT for multi carrier transmission gNB/UE support:

- gNB/UE performs multiple LBT, one for each channel bandwidth separately,

- gNB/UE performs single LBT over all CCs.

Proposal 3: For the CCA check procedure, the COT initiating device may use one or multiple spatial domains receive filters. For each transmission during the COT, there should be associated one or multiple spatial domains receive filters used in the CCA check procedure.

Proposal 4: Consider the use of composite transmit angular power profile (APP) of an intended set of transmit beams to design sensing beam that “covers” that intended set.

- Prominent directions of intended transmission, i.e., those for which composite transmit APP is with a fraction of the peak APP, should have relatively large sensing gain.

- For EDT determination, define Pout as the maximum EIRP over that intended set of transmit beams.

- Appropriate EDT incorporates shortfall (if any) in the sensing gain over prominent directions.

- Enable augmented sensing to avoid blind spots without excessive exposed nodes.

Proposal 5: Introduce Cat 2 LBT for 60GHz unlicensed band operation.

Proposal 6: Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, a one-shot LBT is needed to share the COT:

- Where Y (for all SCS) may be the time duration of 3 symbols (@120 kHz SCS

- Where One-shot LBT duration (for all SCS): the time duration of 1 symbol @ 120kHz SCS

Proposal 7:   When independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, an additional requirement on Cat 2 LBT before beam switch during the COT should be specified.

Proposal 8: For receiver assisted LBT, support NR CSI-IM based reporting for the clear channel assessment at the receiver.

Proposal 9: For receiver assisted LBT, the receiver shall report the resource availability prior to the transmission. The RSSI measurement definition may be extended to assess the resource availability, where the resources, type of measurement (for instance Cat2 LBT) shall be provided by the transmitter.

Proposal 10: For regions where LBT is not mandatory, a no-LBT mode can be defined and switching between LBT mode and no-LBT mode can be supported.

Proposal 11: For indication of LBT mode and no-LBT mode, cell specific (common for all UEs in a cell) indication as part of system information and dedicated RRC signaling should be supported.

• FFS: Supporting UE specific (i.e., different for different UEs in a cell) indication via dedicated RRC signaling.

Proposal 12:  In deployments without LBT consider specification of channel vacation policies accounting for disparity among co-existing devices.

- References

- RP-202925, Revised WID: Extending current NR operation to 71 GHz, CMCC, RAN#90e, 2020.

- Chairman's Notes RAN1#104-e v017

- ETSI EN 302 567 Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum

- TS 38.106 User Equipment (UE) radio access capabilities (Release 16)

- TS 38.101-2 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 17)

- D. Zhang et.al., “Characterizing Interference Mitigation Techniques in Dense 60 GHz mmWave WLANs”, ICCCN 2018.