3GPP TSG RAN WG1 #105-e	Tdoc R1-2104215

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.5

Source:	Ericsson

Title:	Maintenance of PDCCH and SPS for Rel-16 NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

For the Rel-16 maintenance, there are several issues left over from RAN1#104bis. In this contribution, we propose text proposals to close these issues for the PDCCH topic and SPS topic.

# 2	PDCCH

## 2.1	Correction of UE PDSCH processing time for DCI format 1_2

It has been pointed out that the PDSCH processing time should account for the newly introduced DCI format 1_2 as well, see TS 38.214 Section 5.3. The purpose of providing conditions for dmrs-AdditionalPosition = 'pos0' (i.e., left column of  Table 5.3-1 and Table 5.3-2) is, shorter PDSCH processing time is applied when there is no additional DMRS configured. The shorter processing time should be applicable regardless of PDSCH mapping type or DCI formats to ensure in-order processing of PDSCHs in the UE processing pipeline. That is, the PDSCH processing time is determined semi-statically according to RRC configuration only, and has no dependency on the DCI scheduling.

When considering DCI format 1_2 as well, this goal can be achieved by simply adding that dmrs-AdditionalPosition = 'pos0' in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured.

When the strict condition is not satisfied, the slower PDSCH processing time is then applied to all PDSCH scheduled by DCI format 1_1 and 1_2.

There has also been discussion about the PDSCH processing time associated with the fall-back DCI since Rel-15. When receiving DCI format 1_0, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS. This leads to slower PDSCH processing time for DCI format 1_0 always. Since this issue belongs to Rel-15 maintenance, we suggest it to be handled as a separate issue under Rel-15 maintenance agenda item.

In summary, we propose the following TP to be adopted in TS38.214 v16.5.0.

Adopt the text proposal to correct the UE PDSCH processing time for DCI format 1_2.

---------------------------------Start of Text Proposal to TS 38.214 v16.5.0----------------------- Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured. 0; 3. 1; 4.5. 2; 9 for frequency range 1. < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.214 v16.5.0-----------------------

## 2.2	Correction of using the relative SLIV for Type 1 HARQ codebook

The topic of constructing Type-1 HARQ-ACK codebook together with relative SLIV was discussed in RAN1#104bis without conclusion.

For proper Type-1 HARQ-ACK codebook construction, the relative SLIV is applied in every slot, and the same set of row indices of TDRA table should be used in every slot for Type-1 HARQ-ACK codebook.

However, the existing spec needs to be modified to achieve this effect. In current specification, S0 refers to the starting symbols of the PDCCH monitoring occasions in the slot. Since the monitoring occasions can vary from slot to slot, the set of values for S0 can vary from slot to slot as well. See example below in Figure 1. Varying sets of S0 leads to varying sets of (S+S0), which then causes different sets of row indices of TDRA table used for Type-1 HARQ-ACK codebook.

Figure 1. Example of starting symbols of the monitoring occasions vary from slot to slot

Thus, restrictions are needed to ensure that each slot has the same set of S0. As an example, Figure 2 illustrate the PDCCH configuration where the starting symbols of the monitoring occasions are the same in each slot. This restriction would allow each slot to use the same sets of row indices of TDRA table for Type-1 HARQ-ACK codebook construction.

Figure 2. Example of starting symbols of the monitoring stay the same from slot to slot

Thus we propose to adopt the TP below to clarify the condition for constructing Type-1 HARQ-ACK codebook together with relative SLIV.

Adopt the text proposal to clarify the condition for constructing Type-1 HARQ-ACK codebook together with relative SLIV.

---------------------------------Start of Text Proposal to TS 38.213 v16.5.0----------------------- #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel a) if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0. If the UE is provided with referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all slots. < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.213 v16.5.0-----------------------

# 3	DL SPS

In TS 38.213 V16.5.0, the specification has captured the agreement on how to support the case where the SPS release PDCCH is received before the end of the SPS PDSCH reception for the same SPS configuration in a slot.

On the other hand, there are DL SPS releases that are not supported in Rel-16. According to the agreement made in RAN1#101e, the described SPS release in the agreement is not supported.

Agreement (RAN1#101e) It is not supported that a SPS release PDCCH in a slot is received after the end of the SPS PDSCH reception in the slot for the same SPS configuration corresponding to the SPS release PDCCH if HARQ-ACKs for the SPS release and the SPS reception would map to the same PUCCH. - FFS: if HARQ-ACKs for the SPS release and the SPS reception mapping to different PUCCHs

In RAN1#104bis, the following conclusion was drawn.

Conclusion (RAN1#104bis) The following is not supported: - The case that SPS release is received in a slot where SPS PDSCH is configured to be received for the SPS configuration corresponding to the SPS release if the HARQ-ACK for the SPS release and the SPS reception mapping to different PUCCHs.

The agreement and the conclusion above have not yet captured in the specification. We propose that they are captured using the text proposal shown below.

Adopt the text proposal to capture the agreement on SPS release that is not supported.

---------------------------------Start of Text Proposal to TS 38.213 v16.5.0----------------------- ## 9.1 HARQ-ACK codebook determination ... If a UE is configured to receive SPS PDSCHs in a slot for SPS configurations that are indicated to be released by a DCI format, and if the UE receives the PDCCH providing the DCI format in the slot where the end of a last symbol of the PDCCH reception is not after the end of a last symbol of any of the SPS PDSCH receptions, and if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions would be multiplexed in a same PUCCH, the UE does not expect to receive the SPS PDSCHs, does not generate HARQ-ACK information for the SPS PDSCH receptions, and generates a HARQ-ACK information bit for the SPS PDSCH release. < Unchanged parts are omitted > If a UE is configured to receive SPS PDSCH(s) in a slot for SPS configuration(s), the UE does not expect to receive a PDCCH providing a DCI format in the slot to indicate SPS PDSCH release of the these SPS configuration(s), where the end of a last symbol of the PDCCH reception is after the end of a last symbol of any of the SPS PDSCH reception(s), if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH reception(s) would be multiplexed in a same PUCCH. If a UE is configured to receive SPS PDSCH(s) in a slot for SPS configuration(s), the UE does not expect to receive a PDCCH providing a DCI format in the slot to indicate SPS PDSCH release of the these SPS configuration(s), if HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH reception(s) would map to different PUCCHs. < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.213 v16.5.0-----------------------

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the text proposal to correct the UE PDSCH processing time for DCI format 1_2.

Proposal 2	Adopt the text proposal to clarify the condition for constructing Type-1 HARQ-ACK codebook together with relative SLIV.

Proposal 3	Adopt the text proposal to capture the agreement on SPS release that is not supported.

# References

3GPP TS 38.213 V16.5.0 (2021-03), “Physical layer procedures for control (Release 16)”.

3GPP TS 38.214 V16.5.0 (2021-03), “Physical layer procedures for data (Release 16)”.

R1-2104132, “Summary of email discussion [104b-e-NR-L1enh-URLLC-01] on remaining issues on PDCCH enhancements,” RAN1#104b-e.

R1-2104126, “Final Summary of [104b-e-NR-L1enh-URLLC-05] on other aspects for URLLC and IIoT,”  RAN1#104b-e.