3GPP TSG RAN WG1 Meeting #105-e  										R1-2104307

e-Meeting, May 10th – 27th, 2021

Title:	Beam management and BWP operation in NTN

Source: 	Thales

Type:	Discussion

Document for:	Decision

Agenda Item:	8.4.4

Work Item: 	NR_NTN_Solutions: Solutions for NR to support non-terrestrial networks (NTN)

# Introduction

On the Beam management enhancement for NTN, the following agreement achieved during RAN1 Meeting #102-e is recalled hereafter:

Agreement: One-beam per cell and multiple-beam per cell are supported in existing NR specifications and are baseline for NR NTN. - FFS: The need for potential enhancement for beam management - FFS: The need for potential enhancement on association of SSBs, beams and BWPs

On the polarization indication in NTN, the following agreement achieved during RAN1 Meeting #103-e is recalled hereafter:

Agreement: Indication of polarization information for DL and UL by the network is supported. - FFS: Signaling details

In RAN1#104-e, the following was proposed by the moderator [10]:

Proposal:

Discuss whether or not at least following issues are valid and decide whether or not enhancements are needed in addition to current NR specification for supporting NTN beam management:

- Issue 1: NR BWP is not directly associated with a beam. Thus, when using TCI to change beam from beam 1 to beam 2, it does not trigger NR BWP switching. However, in NTN FRF>1 case, beam switching may result in a BWP switching.

- Issue 2: NR BWP switching in UL and DL are not jointly triggered for FDD. However, in NTN FRF>1 FDD scenario, beam switching may result in a BWP switching in both DL and UL.

- Issue 3: NR dynamic BWP switching requires data scheduling. While in NTN FRF>1 scenario, we may need a fast BWP switching triggering without data scheduling.

- Issue 4: NR BWP switching does not require re-synchronization. However, in NTN FRF>1 scenario, when a satellite beam switching is triggered, UE may need to perform re-synchronization in the switched BWP.

- Issue 5: Since satellite beam switching can be frequent and often highly predictable, mechanisms of configured BWP switching (can be a sequence of BWPs) may be  preferred but current NR does not allow it.

- Issue 6: How to deal with BWP switching triggered by bwpInactivityTimer, RA procedure, or simply a need to increase throughput instead of for beam-level mobility.

- Issue 7: NR BWP switching/beam switching is done with UE specific signalling due to UE movement’s. However, in NTN scenario, a satellite BWP/beam switching is common for set of UEs, we may need to a common BWP/beam switching mechanism to save the signalling overhead.

This contribution discusses potential enhancement on Beam management and Bandwidth Parts (BWP) operation for NTN.

All observations and proposals are summarized in Section 4.

# Beam management and BWP operation for NTN with frequency reuse

## Overview

In this section, we discuss the frequency reuse schemes that can be used in NTN to increase the SINR and enhance the spectral efficiency. These include; the traditional frequency reuse, polarization re-use scheme and a Flexible Frequency Reuse based on Bandwidth Part. We discuss also in this section the cell and NR beams to satellite beams mapping as well as the potential usage of the beam management in NTN context.

## Spatial Frequency Reuse

In NR NTN, to mitigate inter-cell/beam co-channel interference, which limits the UE’s throughput, Frequency reuse schemes (Frequency reuse factor, FRF > 1) have been proposed.

Spatial Frequency reuse techniques improve the SINR but inherently limit the per-beam bandwidth and the system capacity. The traditional Frequency Reuse-3 (FRF-3) scheme, for example, offers a protection against inter-cell interference. However, only a third of the spectral resources are used within each cell.

In S-band for example, in case of full frequency reuse (FRF=1), each cell is configured with a bandwidth of 30MHz, while in case of FRF-3, each cell is configured with a bandwidth of 10MHz as the system bandwidth is divided into the three neighboring cells to decrease interference.

Figure 1 Frequency Reuse (FRF-3) and Full Frequency reuse (FRF-1)

Conducted NTN System level simulations have shown potential gains of FRF-3 scheme. Considering a LEO-1200 S-Band scenario, the SLS results provided in [1] and displayed in the Figure 14 , show that for 100% RU (Resource Usage) the majority (more than 80%) of the UEs experience a higher throughput for the case with FRF-3 than for the case with full frequency reuse.

Figure 2 FRF 1 and FRF 3 comparison

The FRF-3 improves the overall SINR but the available cell channel bandwidth is to be divided by a factor of 3; thus, reducing the cell capacity.

- For loaded cells, Frequency Reuse schemes are needed to mitigate the inter-cell/inter-beam interference and improve the overall SINR

- Spatial Frequency reuse schemes reduce significantly the inter-beam Co-channel interference but inherently limiting the per-beam bandwidth and the system capacity

In NTN, one component carrier (CC) supports a scalable bandwidth, 5, 10, 15, 20, 25, and 30 MHz for frequency range FR1, and a scalable bandwidth, 50, 100, 200, 400MHz for frequency range FR2, as shown in Table 8 .

Furthermore, a CC is associated with various RF Requirements (out-of-band emission..) which may reduce the system frequency bandwidth. with guard band ratio from 20% to 2% in FR1 and with guard band ratio from 8% to 5% in FR2.

Table 1 Supported NR NTN carrier bandwidths and transmission bandwidth configuration

| FR1 | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 | 25 | 52 | 79 | 106 | 133 | 160 |
|  |  | 4.50 MHz | 9.36 MHz | 14.22 MHz | 19.08 MHz | 23.94 MHz | 28.8 |
|  | 30 | 11 | 24 | 38 | 51 | 65 | 78 |
|  |  | 3.96 MHz | 8.64 MHz | 13.68 MHz | 18.36 MHz | 23.40 MHz | 28.08 |
|  | 60 | N/A | 11 | 18 | 24 | 31 | 38 |
|  |  | N.A | 7.92 MHz | 12.96 MHz | 17.28 MHz | 22.32 MHz | 27.36 |

| FR2 | SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- | --- |
|  | 60 | 66 | 132 | 264 | N.A |
|  |  | 47.52 MHz | 95.04 MHz | 190.08 MHz | N/A |
|  | 120 | 32 | 66 | 132 | 264 |
|  |  | 46.08 MHz | 95.04 MHz | 190.08 MHz | 380.16 MHz |

The bandwidth per cell must be adapted based on the frequency factor and the polarization re-use option considered. Considering the supported NR NTN carrier bandwidths, given in Table 8 , and depending on the available system frequency bandwidth, not all CC bandwidth could be used with a given FRF, specifically if the CCs allocated for different neighbor cells should have the same bandwidth.

Furthermore, for more flexibility, the BWP can be used to replace component carrier (CC) as will be discussed in an upcoming clause.

- To increase the per-beam bandwidth while ensuring excellent interference isolation between beams, other frequency separation techniques such as polarization re-use scheme should be considered

## Polarization re-use support

Traditional frequency reuse though simple in implementation is suffering from quite reduced spectral efficiency. To increase the per-beam bandwidth while ensuring excellent interference isolation between beams, polarization re-use scheme can be used together with frequency reuse as shown on the figure 15: Neighboring cells use different polarization modes to mitigate inter-cell interference: e.g. Right-Hand-Circular (RHC) which follows a clockwise pattern, and Left-Hand-Circular (LHC) which follows a counterclockwise pattern.

NTN should support UEs with circularly polarized antennas and polarization re-use schemes.To prevent depolarization loss in case of UE with only linear polarization capability, a combination of the two Rx branches in the UE may be used in the downlink, and the satellite reception may implement dual polarization per beam in the uplink.

- NTN should support co-existence of UEs with circularly polarized antennas and linearly polarized antennas on both S-band and Ka-band

Figure 2 Polarization re-use scheme used together with frequency reuse

Additionally, there are other advantages of circular polarization such as its robustness against the Faraday effect which affects linear polarization especially at lower frequencies and it is more resistant to signal degradation due to atmospheric conditions.

More importantly, circular polarization can be used to increase the cell capacity: Both directions (RHC and LHC) can be used simultaneously on the same PRBs.

- Circular polarization can be used to double the cell capacity

# Beam Management and BWP support

## Definitions

- Beam: A beam is the main lobe of the radiation pattern of an antenna array

- Antenna port:

  - As per [7] an antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed.

  - Simply put, it is a logical entity distinguished by a separate Reference Signal sequence

- Satellite beam: a beam created by Satellite Active Antenna System. A satellite beam may not be identified by the UE

- NR beam: a beam which is associated with an antenna port/CSI Reference signal. It can be identified by the UE

## NR Beams and NR Cell to Satellite beams mapping

NR Beamforming needs channel knowledge at the transmitter. Regular beam management operations are based on the control messages which are periodically exchanged between the gNB and the UE.

In NTN, it will be challenging to implement an optimal and dynamic/fast beamforming towards the users. There are different reasons for that: FDD is assumed for core specification work for NR-NTN. As in FDD there is no channel reciprocity, channel information cannot be obtained from UL Sounding Reference Signals (SRS). Also, the propagation delay in NTN may impact the validity of L1 beam measurement. Furthermore, the generated satellite beams are with a large size (tens or maybe hundreds of Km).

- In NTN it might be challenging to implement an optimal and dynamic/fast beamforming towards the users

Therefore, a different approach should be taken in NTN: the gNB should operates on a set of predefined satellite beams in UL and DL. For Earth fixed beam scenario: Beams are static and do not move (from UE perspective) but beams should be steered to cover Earth fixed cell area. In case of LEO based Earth moving cells, the satellite beams are moving on the ground and the Users are jumping from beam to beam.

From our perspective, there are mainly 3 options which can be considered for the mapping of NR beams and NR cells to the satellite beams:

- Option 1; Single NR cell per satellite beam and single NR beam cell:

Option (1) which might be considered as a baseline consist in one-to-one mapping of NR cells to Satellite beams. In option 1, the NR beamforming is not used; the cells are single NR beam cells. NR Beam management operation is not needed.

Figure 3 Single NR cell per satellite beam and single NR beam cell

- Option (1) Single NR cell per satellite beam and single NR beam cell can be used as a baseline. With this option NR Beam management operation is not needed

- Option 2; Multi-Satellite beam NR cell and single NR beam cell: Cell splitting

In Option (2), two or more satellite beams are used for the same PCI. But, NR beamforming is not used. This option can be useful to split the NR cell across several satellite beams (to extend cell coverage). Regular beam management operation (beam Indication, beam measurements and reporting, beam recovery, tracking and refinement)  is not used. The  beam level mobility is not needed for the UE mobility within the cell.

Figure 4 Multi-Satellite beam NR cell and single NR beam cell: Cell splitting

- Option 3; Multi-Satellite beam cell and multi NR beam cell:

Option (3) corresponds to a multi-NR beam cell: multiple NR beams per NR cell (typically 4 NR beams/cell in Frequency Range 1). With this option, the same cell (PCI) is mapped to two or more satellite beams. Each NR beam is transmitted using a satellite beam or multiple satellite beams. Option (3) is to be used with beam management operation.

Figure 5 Multi-Satellite beam cell and multi NR beam cell

- The minimum size of NR beam is the satellite beam size

## NR Beam Management in NTN

The Release-15/16 beam management is to be considered as baseline for 5G NTN.

The beam management will be used with the option (3) described above. Beam level mobility is to be used to control the UE connected mode mobility instead of Handover procedure. This is mainly beneficial in case of Earth moving cell as shown in Figure 19 . As beam level mobility does not require explicit RRC signalling to be triggered - it is dealt with at lower layers and RRC is not required to know  which beam is being used at a given point in time. Thus, the UE moves inside the cell, handled by beam management without extra RRC signalling overhead.

- Beam management can be beneficial in case of multi-beam moving cell

Figure 6 Multi-beam cell: 4 Beams per cell

However, in NTN, when we use several beams per cell (option 3 described above), we increase ipso facto the cell size. Therefore, cell air-interface capacity limit should be taken into account. We believe that deploying multi-beam cell and using beam management will not be applicable to all NTN deployment scenarios.

For example, let’s consider a LEO scenario at 600Km, for which a typical beam is circular at nadir with a beam size of 60Km and becomes an ellipse with a size of 60Km by 175km at the edge of the coverage; The beam size depends on the satellite antenna size, the satellite altitude and it stretch as a function of elevation angle.

The traffic model given in Table B.2-1 [6] has been considered assuming only pedestrian usage scenario: Overall UE density: is fixed at 100 UE per km2 and the percentage value of the amount of simultaneous active users at 1.5 %.

When we compute the number of connected users per cell, we can see that a high number of users need to be served simultaneously in case of a multi-beam cell. Which might not be possible because of limited cell capacity, particularly in FR1.

Table 2 Multi satellite beam cell : 4 beams per cell

| Beam range (km) | beam area (Km^2) | #Connected UE/beam | #Connected UE/cell (4 beams per cell) |
| --- | --- | --- | --- |
| 60Km at nadir | 2827 | 4241 | 16964 |
| At EoC Ellipse 60Km by 175km | 8246 | 12370 | 49480 |

- Deploying multi-beam cell and using beam management will not be applicable to all NTN deployment scenarios

## Flexible Frequency Reuse based on Bandwidth Part

For a Flexible Frequency Reuse, a beam-specific BWP (BBWP) can be configured. The objective is to replace the component carrier (CC) which is not as flexible as a BWP is. The same component carrier  is used on all cells but each beam will be assigned a beam-specific BWP. The inter-beam interference mitigation will be based on  BBWP following a frequency reuse scheme. The Figure 20 shows a flexible frequency reuse based on BBWP reuse. In the proposed solution, the multi-beam cell (option 3) and beam management are assumed. But, as discussed in 4.4.3, deploying multi-beam cell and using beam management will not be applicable to all NTN deployment scenarios.

Basically, Bandwidth parts (BWP) are used to optimize UE operations in frequency domain, whereas the BBWP will be used to support flexible frequency reuse. The beam-specific BWP (BBWP) similar to UE’s dedicated BWPs should be configured independently (from UE’s dedicated BWPs) by network via dedicated RRC signalling. For the configuration of BBWP, the same configuration parameters will be used: starting position, size and the subcarrier spacing. But we need to add an indication of the associated beam: a beam-index (CSI-RS associated with the beam).

When a flexible frequency reuse based on BBWP is to be deployed, an a-priori BBWP planning can be used to allocate the BWP to each beam. Or a dynamic allocation can be performed by the gNB to configure beam-specific BWP based on the traffic distribution between the beams within the cell. Interference coordination may be required between the gNodeBs

- In the proposed solution, an a-priori BBWP planning can be used to allocate the BWP to each beam. Or a dynamic allocation can be performed by the gNB to configure beam-specific BWP based on the traffic distribution between the beams within the cell

Figure 7 Flexible Frequency Reuse using BWP

To support beam-specific BWP, the Release-15/16 BWP operation is to be considered as baseline for 5G NTN. Nevertheless, some enhancements are needed. The required enhancements and their justifications are as follows:

- The new BBWP should be introduced on top of existing UE specific BWP. Indeed, the main usage of the BWP as per Release 15, is to restrict the bandwidth according to UE capabilities, optimize UE operations in frequency domain and dynamically reduce or increase bandwidth according to traffic amount so as to optimize UE power consumption. All these usages of Rel.15 BWP are still valid and we should be able to have the same usage of BWP as before.

- The UE should be configured with as many BBWP as the number of beams within the cell. But, in the current specification, a UE can be configured with up to four bandwidth parts in the downlink and up to four bandwidth parts in the uplink. Therefore, the maximum number of BBWP an NTN UE can be configured with should be only limited by the number of the beams within the cell.

- The UE is not expected to receive CSI-RS outside an active bandwidth part. If BBWP operation is introduced, the UE should be able to perform inter-beam measurements outside its active BWP

- CSI-RS based inter-beam measurement is to be supported in the same manner as the existing CSI-RS based inter-frequency measurement, provided the bandwidth of the CSI-RS resource on the neighbour beam configured for measurement is not within the bandwidth of the CSI-RS resource on the serving beam configured for measurement, or the subcarrier spacing of the two CSI-RS resources is different. The UE cannot be assumed to be able to carry out such measurements without measurement gaps. a measurement gap-assisted configuration is to be provided

- For BBWP, Beam Level Mobility can only be performed based on CSI-RS.

- The new beam-specific BWP (BBWP) concept should be introduced on top of existing UE specific BWP

- The new beam-specific BWP (BBWP) should reuse Release-15/16 BWP operation procedures with the enhancements provided in this TDOC

- Regarding BBWP Operation: In the current specifications, initial-active DL BWP #0, is defined by span of CORESET#0 configured by MIB (or dedicated RRC) for scheduling of SIBs. Supported sizes of initial-active DL BWP are only 24,48,96 RB. Initial-active UL BWP #0: is configured by SIB1 for random access procedure. Initial BWP is compatible with all UE categories and can be used by any UE within the cell before it is reconfigured with BBWP associated to the beam in which the UE is located. Therefore, the size of the common Initial-active BWP should be designed carefully to avoid any congestion and blocking within the cell.

- The size of the common Initial-active BWP should be defined carefully to avoid any congestion and blocking within the cell

- In Connected state, BWP adaptation/switching is based on the Bandwidth part indicator in the DCI.  BBWP switching should be handled by beam management indication which relies on gNB controlled means to determine and select used beams. BBWP switching is illustrated on the protocol flow below. MAC CE transmission configuration indication can be used to indicate and update serving beam and implicitly the BBWP. We do not need to use UE bandwidth part adaptation which use bandwidth part indicator on the DCI, because this should be used only for UE’s dedicated BWPs adaptation as per Release 15/16. We do not need to increase the maximum  number of BWP the UE can be configured with That is the Bandwidth part indicator field on DCI should be unchanged.

- Bandwidth part indicator field on DCI should be unchanged

- MAC CE transmission configuration indication (MAC CE TCI) can be used to indicate and update serving beam and implicitly the BBWP

Figure 8 BBWP change indication, based on TCI

# Conclusion

In this contribution, the following proposals and observations have been made:

Observation 1	For loaded cells, Frequency Reuse schemes are needed to mitigate the inter-cell/inter-beam interference and improve the overall SINR

Observation 2	Spatial Frequency reuse schemes reduce significantly the inter-beam Co-channel interference but inherently limiting the per-beam bandwidth and the system capacity

Proposal 1	To increase the per-beam bandwidth while ensuring excellent interference isolation between beams, other frequency separation techniques such as polarization re-use scheme should be considered

Observation 3	Circular polarization can be used to double the cell capacity

Observation 4	In NTN it might be challenging to implement an optimal and dynamic/fast beamforming towards the users

Observation 5	Option (1) Single NR cell per satellite beam and single NR beam cell can be used as a baseline. With this option NR Beam management operation is not needed

Observation 6	The minimum size of NR beam is the satellite beam’s size

Observation 7	Beam management can be beneficial in case of multi-beam moving cell

Observation 8	Deploying multi-beam cell and using beam management will not be applicable to all NTN deployment scenarios

Observation 9	In the proposed solution, an a-priori BBWP planning can be used to allocate the BWP to each beam. Or a dynamic allocation can be performed by the gNB to configure beam-specific BWP based on the traffic distribution between the beams within the cell

Proposal 3	The new beam-specific BWP (BBWP) concept should be introduced on top of existing UE specific BWP

Proposal 4	The new beam-specific BWP (BBWP) should reuse Release-15/16 BWP operation procedures with the enhancements provided in this TDOC

Observation 10	The size of the common Initial-active BWP should be defined carefully to avoid any congestion and blocking within the cell

Proposal 5	Bandwidth part indicator field on DCI should be unchanged

Proposal 6	MAC CE transmission configuration indication (MAC CE TCI) can be used to indicate and update serving beam and implicitly the BBWP

# Reference

| [1] | N. R. GmbH, R1-20xxxxx Study Cases for System-Level Simulations in NTN WI. |
| --- | --- |
| [2] | "TR38.821 Solutions for NR to support non-terrestrial networks". |
| [3] | "TR38.213 Physical layer procedures for control". |
| [4] | Thales, "R1-2005306 NR_NTN_solutions work plan," RAN1#102-e. |
| [5] | 3GPP, TR 38.821 Solutions for NR to support non-terrestrial networks (NTN), Release 16. |
| [6] | 3GPP, TS 38.211 Physical channels and modulation. |
| [7] | 3GPP, TS 38.331, Radio Resource Control (RRC); Protocol specification. |
| [8] | 3GPP, TS 38.214 Physical layer procedures for data. |
| [9] | Thales, R1-2006676 - NR NTN Reference scenarios definition for Rel-17 normative phase. |
| [10] | OPPO, AI, R1-2102141 Summary_3 of AI 8.4.4. |