3GPP TSG RAN WG1 #105-e	R1-2104323

e-Meeting, May 10th – 27th, 2021

Source: 	ZTE

Title: 	Discussion on PDSCH processing timeline for DCI format 1_2

Agenda item:	7.2.5

Document for:	Discussion/Decision

Introduction

In Rel-16 URLLC, separate DMRS configuration for PDSCH mapping type A and type B is introduced for DCI format 1_2 as follows [1].

dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } OPTIONAL, -- Need M dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2-r16 SetupRelease { DMRS-DownlinkConfig } OPTIONAL, -- Need M

In this contribution, the UE PDSCH processing time related to the new introduced DMRS configuration is discussed.

Discussion

In Rel-15, PDSCH processing time for PDSCH processing capability 1 and capability 2 depends on whether additional DMRS is configured. However, new DMRS parameters dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 are introduced for DCI format 1_2 in Rel-16, and it hasn’t been reflected in current Rel-16 specification.

Note that, the legacy RRC parameter dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB only apply to DCI format 1_1. For DCI format 1_0, our understanding is that UE will always use PDSCH processing capability 1 with a longer processing time (right column in Table 5.3-1) since dmrs-AdditionalPosition = 'pos2' is assumed for PDSCH scheduled by DCI format 1_0.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

There could be two options to include the new introduced RRC parameters for DCI format 1_2.

- Option 1: The PDSCH processing time is independent from DCI formats. That is, only when dmrs-AdditionalPosition = 'pos0' is configured in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, the PDSCH decoding time N1 follows the shorter processing time for PDSCH processing capability 1 (left column in Table 5.3-1) or the processing time for PDSCH processing capability 2. Otherwise it follows the longer processing time for PDSCH processing capability 1 (right column in Table 5.3-1). This would cause a larger processing time even when a PDSCH is scheduled without additional DMRS.

- Option 2: The PDSCH processing time is defined per DCI format. In other words, the PDSCH processing time can be different for PDSCH scheduled by DCI format DCI 1_1 and DCI format 1_2. This is more flexible while it requires UE can dynamically change the UE PDSCH processing time for different PDSCHs scheduled by different DCI formats. It seems not a big issue since the processing time for PDSCH scheduled by DCI format 1_0 may also different from the PDSCH scheduled by DCI format 1_1 in legacy Rel-15.

- Option 2 seems more feasible than option 1 and also follows the same principle as Rel-15. Option 1 would make the processing time dependent on all DMRS configurations from both two non-fallback DCI formats. Option 2 is our first preferred option, while we are also open to discuss solutions based on Option 1.

- Based on above analysis, the following Text proposal #1 or Text proposal #2 is proposed.

Proposal 1: Adopt Text Proposal #1 or Text Proposal #2 below for UE PDSCH processing procedure time.

----------------------------------------Text Proposal #1 for Section 5.3 in TS 38.214 g50------------------------------------

- 5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, or in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

----------------------------------------Text Proposal #2 for Section 5.3 in TS 38.214 g50------------------------------------

- 5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if only DCI format 1_1 is configured, or dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if only DCI format 1_2 is configured, or dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if both DCI format 1_1 and DCI format 1_2 are configured; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured if only DCI format 1_1 is configured, or dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 or if the higher layer parameter is not configured if only DCI format 1_2 is configured, or dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 or dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 or if the higher layer parameter is not configured if both DCI format 1_1 and DCI format 1_2 are configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if only DCI format 1_1 is configured, or dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if only DCI format 1_2 is configured, or dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if both DCI format 1_1 and DCI format 1_2 are configured. 0; 3. 1; 4.5. 2; 9 for frequency range 1.

Conclusion

In this contribution, the UE PDSCH processing time related to the new introduced DMRS configuration is discussed, and we have the following proposal, where the corresponding text proposals can be found in Section 2.

Proposal 1: Adopt Text Proposal #1 or Text Proposal #2 below for UE PDSCH processing procedure time.

Reference

TS 38.213-g50, Physical layer procedures for data, Release 16.

TS 38.331-g41, Radio Resource Control (RRC) protocol specification, Release 16.