3GPP TSG RAN WG1 #105-e		 R1-2104404

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.1.1

Source:	Lenovo, Motorola Mobility

Title:	Enhancements on Multi-beam Operation

Document for:	Discussion

# Introduction

Enhancement of multi-beam operation is an important part of R17 feMIMO WI [1]. Discussion on multi-beam operation has been ongoing since RAN1#102e [2][3][4]. In this contribution, we present our opinion on unified TCI framework, dynamic TCI update, L1/L2-centric inter-cell mobility, UE panel indication, MPE, and latency and overhead reduction.

# Discussion

## Unified TCI framework

The following agreements are reached in RAN1#104e regarding unified TCI framework and dynamic TCI update:

Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, for each of PUSCH, PUCCH, and SRS, in RAN1#105-e, further discuss to down-select or combine from the following alternatives: - AltA. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state - AltB. The setting of (P0, alpha, closed loop index) is also included with UL or (if applicable) joint TCI state - AltC. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state Note: It has been agreed that the setting of (P0, alpha, closed loop index) is associated with UL channel or UL RS (therefore the setting is channel- and signal-specific). Agreement On Rel.17 unified TCI framework, in RAN1#105-e, further discuss to down select or combine from the following three alternatives for PL-RS (note: the text below is based on the agreed description in RAN1#104-e): - AltA. PL-RS can be included in UL TCI state (or, if applicable, joint TCI state). - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state. - AltB. PL-RS can be associated with (but not included in) UL TCI state (or, if applicable, joint TCI state) - FFS: Exact association mechanism - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state - AltC. UE calculates path-loss based on periodic DL RS configured as the source RS for determining spatial TX filter in UL or (if applicable) joint TCI state - FFS: If a PL RS is not included in or associated with the UL TCI state (or, if applicable, joint TCI state), whether the UE can estimate path-loss based on the PL-RS of an UL RS provided in an UL TCI state (or, if applicable, joint TCI state) as a source RS for determining the spatial TX filter. In addition: - FFS (to be decided in RAN1#105-e) whether a fallback scheme is needed and, if so, the details - FFS: Support additional UE capability to report whether above PLRS determination mechanism is supported - Note: As agreed in RAN1#104-e, the total number of maintained PL-RSs per CC is no more than 4 - FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates based on UE capability - FFS: UE capability for maximum number of active PL-RS across CCs per band

- Type of RS DL and UL TCI apply to

We see some benefit for applying the DL TCI (or common DL/UL TCI) to DL RS in addition to PDSCH/PDCCH in terms of quickly updating the DL spatial information without additional signaling. The CSI-RS supported shall include CSI-RS resource for CSI acquisition and TRS. However, if it applies to all the DL RS (CSI-RS for CSI, CSI-RS for BM, TRS) simultaneously, the ability to fine tune the DL beam, for example in the P2 process of beam management, will be lost. It shall be selectively applied to a subset of CSI-RS resources to reduce the signaling overhead without losing the flexibility for fine-tuning. For UL TCI, the TCI state can be applied to SRS for beam management. For the same reason as DL TCI, it is necessary to signal the SRS resources the UL TCI state applies to. The situation for multi-TRP makes it even more necessary to identify which subset of CSI-RS a DL TCI applies to, since not all CSI-RS for CSI measurement are sent from a same TRP. From signaling point of view, it is convenient to divide the CSI-RS or SRS resources into several different subsets, where each subset shares the same DL or UL TCI states. The set of CSI-RS or SRS that a new TCI state applies to can be dynamically signaled.

Proposal 1: DL or joint DL/UL TCI can be applied to subset of CSI-RS for CSI and TRS.

Proposal 2: UL TCI can be applied to subset of SRS resources.

Proposal 3: The subset of CSI-RS and SRS resources a new TCI applies is dynamically signaled.

- Determination of joint DL/UL TCI and separate DL/UL TCI

If there is only a single pool of TCI states covering DL, joint DL/UL and UL TCI, there is no ambiguity in determining what TCI state is signaled by a DCI. The need to distinguish joint DL/UL TCI or separate DL and UL TCI arises when multiple TCI state pools are defined by RRC, and which pool to use depends on the context. If from the TCI signaling DCI, the UE can determine which channels and signals the TCI state applies to, it is necessary for the UE to tell which TCI state pool to use. Therefore we think Alt1 (A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI) shall be supported.

Proposal 4: Whether the TCI field points to joint DL/UL TCI or separate DL/UL TCI depends on the context of the DCI (Alt1).

- Power control parameters

RAN1 has agreed on using SRS for UL TCI state. When a SRS is used for UL TCI state for PUSCH, the same RS cannot be reused as the path loss reference RS. In this case the PL-RS of the SRS used as UL TCI state, such as a TRS or CSI-RS for BM, can be used as PL-RS. Therefore the PL-RS for a TCI state is associated with the TCI state.

In R16, the UL power control parameters for PUSCH, including P0, alpha, and closed loop index, can be dynamically changed with SRI. Different channels are also configured with different UL power parameters. Following the same principle, a TCI state shall not include a set of PC parameters for all the channels. Because the power control parameters of different channels are configured separately in R16, for R17 when different UL channels share the same UL TCI state, the network shall still be able to configure separate power control parameters for them. This makes the channel swith the same UL TCI shall be allowed to have different PC parameters to provide flexibility. Consequently the PC parameters, (P0, alpha, and closed loop index) shall not be part of the TCI state, but multiple sets of PC parameters can be associated with a TCI state. The assignment of PC parameters associated with a TCI state to different UL channels and SRS resources needs further study.

Proposal 5: PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state.

Proposal 6: The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state.

## L1/L2-centric inter-cell mobility

The following agreement is achieved in RAN1#104e on L1/L2-centric inter-cell mobility:

Agreement On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - On the value of K (defined in RAN1#104-e as the number of beam qualities associated at least with non-serving cell(s) can be reported in a single CSI reporting instance), - For the supported maximum value(s) of K, down-select at least one from the following candidates {4, 8, 16} - FFS: whether the maximum value of K is a UE capability - Periodic, semi-persistent, and aperiodic reporting (and the respective measurements) are supported. - Note: Semi-persistent and aperiodic reporting (and their respective measurements) are NW-initiated Agreement On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - In one reporting instance, depending on NW configuration, beam(s) associated with a non-serving cell can be mixed with that associated with serving-cell - FFS: whether this applies to periodic, semi-persistent, and/or aperiodic - FFS: How to report the K beams and corresponding qualities if the Tx power among the non-serving cell and with serving-cell is not the same - Note: The supported numbers of non-serving cells (in terms of measurement/reporting) have not yet been decided. The above description doesn’t imply only one non-serving cell is allowed to be configured for measurement. Nor does this imply that only one non-serving cell is allowed in one reporting instance. Agreement On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, for L1-RSRP measurement and at least aperiodic reporting, investigate and, if needed, specify MAC CE based dynamic activation/deactivation of a subset of higher-layer-configured measurement for non-serving cell SSBs

It has been agreed that SSB from non-serving cell can be used for L1-RSRP measurement during L1/L2 inter-cell mobility. We think CSI-RS for mobility transmitted from non-serving cell shall be used as well. CSI-RS resource for mobility is already used for L3 handover since R15, and since they are already transmitted by the neighboring cells, there is no reason not to use them for L1/L2 mobility measurement. They also have wider bandwidth and can be measured with higher accuracy than SSB. The definition of SS-RSRP in R16 can be reused.

Proposal 7: Use CSI-RS for mobility for L1-RSRP measurement for L1/L2-centric inter-cell mobility.

When a UE is configured with both SSB and CSI-RS for mobility for inter-cell measurement, it may cause complication if both SSB and CSI-RS are included in a same reporting setting and UE may report a mixture of SSB and CSI-RS. The fact that CSI-RS is transmitted with different TX power than SSB (with offset PowerControlOffsetSS) further makes joint reporting complicated. In R15/16 intra-cell beam management, a UE is required to provide L1-RSRP measurement of either SSB or CSI-RS in a single report. To make it simple, and to reduce the specification impact, such limit shall be extended to the L1/L2 inter-cell mobility case. That is, a CSI report can be configured with L1-RSRP measurement of either SSB or CSI-RS, but not both.

Proposal 8: Does not allow mixed SSB and CSI-RS L1-RSRP report in a same CSI report.

## Dynamic beam indication

The following agreements are reached in RAN1#104e regarding dynamic beam indication:

Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - Use ACK/NACK mechanism analogous to that for SPS PDSCH release with both type-1 and type-2 HARQ-ACK codebook: - Upon a successful reception of the beam indication DCI, the UE reports an ACK - Note that upon a failed reception of the beam indication DCI, a NACK can be reported. - For type-1 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined based on a virtual PDSCH indicated by the TDRA field in the beam indication DCI, based on the time domain allocation list configured for PDSCH - For type-2 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined according to the same rule for SPS release - The ACK is reported in a PUCCH k slots after the end of the PDCCH reception where k is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, or provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCI-Format1-2-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI - When used for beam indication: - CS-RNTI is used to scramble the CRC for the DCI - The values of the following DCI fields are set as follows: - RV = all ‘1’s - MCS = all ‘1’s - NDI = 0 - Set to all ‘0’s for FDRA Type 0, or all ‘1’s for FDRA Type 1, or all ‘0’s for dynamicSwitch (same as in Table 10.2-4 of TS38.213) - FFS: Whether HPN is also used - Use the existing TCI field (always present) to signal the following: 1) Joint DL/UL TCI state, 2) DL-only TCI state (for separate DL/UL TCI), 3) UL-only TCI state (for separate DL/UL TCI) - FFS: Whether both DL TCI and UL TCI states can be signaled in one instance of beam indication DCI - FFS: Relation with joint vs separate TCI (DL and/or UL) switching, including M/N>1 if supported - In addition, use the following DCI fields as the fields are being used in Rel-16: - Identifier for DCI formats - Carrier indicator - Bandwidth part indicator - TDRA - Downlink assignment index (if configured) - TPC command for scheduled PUCCH - PUCCH resource indicator - PDSCH-to-HARQ_feedback timing indicator (if present) - The remaining unused DCI fields and codepoints are reserved in R17 - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional - FFS: How to handle the case when there is only UL data - FFS: The case for UE being indicated with separate UL TCI in DCI format 1_1/1_2 with DL assignment - FFS: When more than one TCI codepoints are activated by MAC CE, the activated TCI state(s) for the lowest codepoint is/are applied - Support of this feature is UE optional - The “lowest codepoint” function can be configured on or off. - FFS: Interaction with the DCI based beam update if needed, whether/how to support the case with M or N > 1 if supported - Note: This agreement on DCI beam indication design is not to be used to be against the support of the cases of M/N>1. The support of M/N>1 will be separately discussed and not dependent on the decision here.

- TCI applicability in DCI format 1_1/1_2

During RAN1#104e meeting, RAN1 has agreed to use DCI format 1_1/1_2 with DL assignment. RAN1#104bis-e meeting further saw the use DCI format 1_1/1_2 without DL assignment agreed for TCI indication. When the R16 DCI format 1_1/1_2 with DL assignment is used, the TCI indicated by the TCI field applies to the PDSCH scheduled. Additionally, the TCI can be applied to future PDSCH, PDCCH, PUSCH and PUCCH transmission. A question is how to tell the UE which channels or resources the TCI applies to. This issue is the same for DCI format 1_1/1_2 with and without DL assignment. To apply the signaled TCI to some, not all of the channels requires additional field in the DCI. A bitmap can be added to the DCI format 1_1 and 1_2 for this purpose. Groups of channels and resources always sharing the same (DL and/or UL) TCI may be defined by RRC or MAC-CE, and each group is represented by a single bit in the bit map to indicate whether the new TCI state applies to the group or not. When the DL TCI applies to user specific PDSCH and PDCCH, and the UL TCI applies to PUSCH and PUCCH, two bits are required to signal whether the TCI state applies to the DL channels, to the UL channels, or both DL and UL channels.

Proposal 9: Add a bitmap field to DCI format 1_1/1_2 with DL assignment to signal which channels and resources the TCI applies to.

- Beam application time

When gNB sends a DCI 1_1/1_2 to a UE to update its beams, it can only start applying the new beams after receiving a confirmation ACK from the UE. It is necessary for the gNB to wait for the corresponding ACK before knowing whether it can apply the new beams or not. Therefore the first slot that it can apply the new beam shall be after the last symbol of the ACK of the joint or separate DL/UL beam indication. This is different from the PDSCH scheduled by the same DCI format 1_1/1_2. For the PDSCH scheduled by the same DCI, the R15/16 spec already defines what RX beam the UE shall use to receive it, and there is no need to receive a confirmation of the DCI. Among the alternatives agreed in RAN1#104e meeting, we think Alt 2A best reflects these requirements and shall be adopted.

Proposal 10: Adopt Alt2A for the application time of the beam indication: the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication.

When the new beam indicated by the DCI is different from the previous beam, the UE shall be allowed enough time to switch to the new beam to receive or transmit with it. The R15/16 spec already defines a RRC parameter (timeDurationForQCL) for the DL. When the new beam is indicated by a DCI, the UE is ready to receive with the new DL TCI after timeDurationForQCL from the last symbol of DCI. The UE capability parameter timeDurationForQCL can be reused here. Because the TX and RX circuits are different, the time UE requires to switch to a new TX beam is different than the time required to switch to a new RX beam. This makes it necessary to consider the time for DL beam switching and for UL beam switching time separately. According to RAN4, the switching time within the same panel and with the same UL timing is 5us. RAN4 needs more discussion to conclude the transient period for cases with cross panel beam switch and/or if the spatial filter to transmit the beam is unknown and/or UL timing is different between different UL beams. Different TA values may be needed for different UL TCI states. RAN1 shall make the decision regarding the application time of UL TCI states following RAN4’s decision on this issue.

Proposal 11: Reuse the UE capability timeDurationForQCL for minimal DL beam switching time.

Proposal 12: Introduce a new UE capability parameter for minimal UL beam switching time.

- ACK for DCI format 1_1/1_2 with DL assignment

When DCI format 1_1/1_2 with DL assignment is used to update the TCI, gNB needs a positive acknowledgement from the UE before the new TCI state becomes effective. When a PDSCH is scheduled by the same DCI, while an ACK for the PDSCH is sent by the UE, this ACK is not the proper vehicle for the UE to inform the gNB it has received the DCI. First, the reliability of the PDCCH is much higher than the reliability of the PDSCH. If the ACK/NACK of the PDSCH is reused as the ACK/NACK of the PDCCH carrying the DCI, the will be many instances that the UE receives the DCI correctly but the PDSCH incorrectly, and the TCI state cannot be updated because of the NACK sent to the gNB. This leads to unnecessary retransmission of the TCI updating DCI. The extra delay caused by retransmission of DCI may lead to beam failure unnecessarily because the new TCI cannot be installed in time. This negates the key benefit of TCI updating mechanism with DCI. To improve the latency and reliability of TCI update using DCI, a dedicated bit in the HARQ-ACK codebook corresponding to the PDSCH shall be used as an explicit ACK/NACK for the DCI. Depending on the configuration of the HARQ-ACK codebook of the PDSCH, a bit can be made available for this by compressing the HARQ information bits of the PDSCH, or added to the existing HARQ information bit(s).

Proposal 13: Use a dedicated ACK/NACK bit in the HARQ-ACK codebook of the scheduled PDSCH as acknowledgement of the DCI.

- Update TCI for multiple CCs and BWPs

In R15/16 TCI framework, activation of TCI state or spatial relation information for PDCCH and PUCCH applies the TCI state/spatial relation in the signaled CC and BWP. When simultaneous TCI update lists or simultaneous spatial relation update lists are configured, the TCI state or spatial relation applies to the corresponding channels in other CCs too. We think such mechanism for updating the TCI for multiple CCs should be supported with both L1-based and MAC-CE based TCI updates, and should be applied to PDSCH and PUSCH as well as PDCCH and PUCCH. Similarly, simultaneous TCI updates for multiple BWPs should also be supported.

Proposal 14: Support simultaneous TCI update for multiple CCs and for multiple BWPs.

- Update TCI with MAC-CE

Per previous agreements, MAC-CE can be used to update the TCI for DL and UL TCIs. For UEs with beam correspondence, a single TCI may suffice for both DL and UL. When such a TCI is signaled in a MAC-CE, it may automatically apply to both DL and UL channels. For UE without beam correspondence, SRS used for beam management can be set as the spatialRelationInfo for SRS and PUCCH. In addition to TCI-state, UL-TCI-state, can be introduced for DCI based dynamic TX beam indication for PUSCH. MAC CE based UL-TCI-state activation for UE-specific PUSCH transmission can also be introduced similar with MAC CE based TCI state activation for UE-specific PDSCH transmission. However, both UL TCI state and DL TCI states are usually required to be updated at the same time. Simultaneous UL TCI state and DL TCI states updating by a same MAC CE can be introduced for overhead and latency reduction. For example, the TCI activation MAC-CE for PDCCH indicates the common DL TCI for both PDCCH and PDSCH, and the spatial relation activation MAC-CE for PUCCH indicates the common UL beam for both PUCCH and PUSCH. In the case of TCI update using L1-signals, MAC-CE can also be used to define groups of channel sharing the same TCI state to reduce the latency and overhead for L1 TCI updates.

Proposal 15: Support simultaneous TCI state updating for UL and DL in a same MAC-CE.

Proposal 16: Consider combined MAC CE-DCI approach to reduce the latency and overhead for TCI updates.

- Support of legacy DCI

A DL common beam can be used for all PDSCHs and all or subset of CORESETs and a UL common beam can be used for all PUSCHs and all or subset of PUCCHs. In general, the beams for CORESETs and PUCCHs are wide for better coverage and the beams for PDSCH and PUSCH is narrow to obtain higher throughput. Therefore, a common beam is highly possibly a wide beam to ensure the coverage of CORESETs or PUCCHs. And the beam of a PDSCH or a PUSCH scheduled by a DCI is indicated by the scheduling DCI in R15/16 which can be a narrow beam. However, it’s not clear whether a PDSCH or a PUSCH can use another beam indicated by a legacy DCI which is not used for common beam indication when a common beam is applicable for DL or UL in R17. Considering the efficiency of data transmission, coexistence of common beam indication and beam indication only for one PDSCH/PUSCH is valuable to be supported in our opinion. Therefore, whether a PDSCH or a PUSCH can be use another beam indicated by a legacy DCI should be determined first. And we propose that:

Proposal 17: Determine whether a PDSCH or a PUSCH can use another beam indicated by a legacy DCI which is not used for common beam indication when a common beam is applicable for DL or UL.

If a legacy DCI format 1_1 or format 1_2 can have a TCI filed to indicate a beam for a PDSCH while a common beam can be indicated by a DCI, then how to distinguish DCIs for indicating common beams from legacy DCIs should be studied. For example, a dedicated search space set can be configured for DCIs’ transmission where all these DCIs are for common beam indication. Therefore, we propose that:

Proposal 18: Further study how to distinguish DCIs for indicating common beams from legacy DCIs.

Since DCI format 1_1 and DCI format 1_2 can be used to indicate a common beam, there should always be a TCI filed present in the DCI for beam indication. Therefore, how to ensure there is a TCI field in the DCI for common beam indication should be studied. As we know that, whether there is a TCI field in DCI format 1_1 and DCI format 1_2 is determined by the RRC parameter tci-PresentInDCI for DCI format 1_1 and RRC parameter tci-PresentForDCI-Format1-2-r16 for DCI format 1_2. And the two RRC parameters are configured per CORESET. If a dedicated search space set is configured for DCIs’ transmission which are used to indicate common beams, in order to make sure a DCI for a common beam indication has a TCI field, one way is that the CORESET which the dedicated search space set is associated with is always configured with tci-PresentInDCI as ‘enable’ and tci-PresentForDCI-Format1-2-r16 as ‘enable’; and the other way is that all the DCIs in the dedicated search space set for common beam indication always have a TCI filed and ignore the two RRC parameters tci-PresentInDCI and tci-PresentForDCI-Format1-2-r16 configured for the CORESET where the dedicated search space set is associated. Based on the analysis above, we propose that:

Proposal 19: Determination of a TCI field in the DCI for a common beam indication is FFS.

## UE panel information indication and updating

The following agreements are reached in RAN1#104e regarding multi-panel UE:

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, for CSI/beam measurement/reporting, down select and/or modify from the following candidates: - Opt1-1: A panel entity corresponds to a reported CSI-RS and/or SSB resource index in a beam reporting instance - The correspondence between a panel entity and a reported CSI-RS and/or SSB resource index is informed to NW - FFS: How to inform through CSI/beam reporting framework - FFS: Detailed design of the correspondence including the conveyed information - Note: the correspondence between a CSI-RS and/or SSB resource index and a panel entity is determined by the UE (analogous to Rel-15/16) - Opt1-2: A panel entity is referring to a new panel ID within CSI/beam reports - FFS: Detailed design of the new panel ID including the information conveyed by the new panel ID - Note: The association between the new panel ID and the panel entity is determined by the UE - Opt1-3: No additional specification support - The duration in which the above panel entity reference is valid and the respective setting are FFS Note: “panel entity” is only used for discussion purpose Agreement On Rel.17 enhancements for MPUE, for codebook based UL transmission, decide by August RAN1 meeting whether to support CB-based SRS resources with different numbers of ports - FFS details (e.g. per resource or per resource set) - Note: the above is not for Rel-16 full power transmission but for Rel-17 panel-specific UL transmission - FFS: non-codebook based UL transmission for MPUE - FFS whether existing BWP switch based mechanism (discussed previously in Rel-16 power saving WI) can serve such purpose

UE panel assumption has been agreed to make the design for multiple UE panels easier. And UE can be equipped with multiple panels where one or more panels can be activated in a time. Multiple activated panels on UE side can bring spatial diversity, and UE needs to inform gNB for better scheduling. Explicit definition (panel definition) or implicit definition (antenna or resource groups) can be considered, giving the gNB different degrees of insight to the UE hardware implementation. The original UE panel configuration can be reported to gNB by UE’s capability report. Which panel the UE uses to receive in the DL is as important as which beam to use for reception. This adds another dimension to the signaling design and needs to be incorporated into CSI reporting. Similarly, the gNB also needs to signal to the UE which panel to use for PUSCH or SRS transmission. The status of UE panel also needs to be reported to the gNB. Depending on the UE battery/power saving status, UE may deactivate (power off) some panels to save power.

If gNB does not know the current panel status in UE side, it may schedule the UE with a beam corresponding to the deactivated panel, causing network resource waste and throughput reduction. The updating of UE’s activated panel information should be informed to gNB timely, and how to report the updating information timely need be determined. An explicit panel ID can be defined and is associated with a set of beams. The panel ID can be included in the UE report to facilitate panel selection. Based on the analysis above, we propose that:

Proposal 20: Explicit panel ID should be defined for panel related state information reporting.

After gNB knows the activated panel information of a UE, it should schedule beams corresponding to the activated panels for the UE. As we know, a SSB, a CSI-RS resource or a SRS resource is used to indicate a beam of DL and UL transmission in NR, therefore, gNB should know which activated panel a SSB/CSI-RS/SRS is associated with in advance to schedule a beam for transmitting a UL or DL. The association relation between a beam and an activated panel should be maintained in gNB for scheduling. This association relation can only be originally obtained by the beam management in UE side, therefore, UE should report it to gNB. Explicit way and implicit way of reporting the association relation between beam and activated panel can be designed. For example, an explicit or implicit panel ID can be reported with L1-RSRP/L1-SINR report to indicate the association between a DL beam and an activated panel by an explicit way. On the other hand, if UE reports multiple groups of RSs with enhanced group-based beam reporting, where all the RSs of a group are associated with an activated panel, this can be considered an implicit form of RS-panel association.  Both of explicit way and implicit way have specification impact and which one should be supported should be determined after the further studying the pros and cons of the two ways carefully. Based on the analysis before, we propose that:

Proposal 21: Report the association relation between a beam and an activated panel to gNB as part of beam reporting.

Proposal 22: Study both explicit and implicit method of reporting of the association between a beam and an activated panel.

## MPE mitigation

The following agreements are reached in RAN1#104e regarding MPE:

Agreement On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options: - Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported. - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - FFS: how to determine the virtual PHR or the modified version. - Opt 1D. {Rel.16 P-MPR based (beam/panel-level)} - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting FFS: If differential report is supported when multiple UL beams are reported in the same report Agreement On Rel.17 enhancements for MPUE, investigate and, if needed, specify the following: - UE reporting of panel-specific information as a UE capability, for example: - Information related to the total number of DL/UL panel entities - Information related to the number of (max) antenna ports/layers per panel entity - Information related to the maximum number of resources per panel entity for SRS BM - Information related to panel selection delay - Information related to panel activation delay - UE reporting information related to minimal activation/selection delay for a panel based on L1 or L2 signaling - UE reporting of panel activation/selection status of a panel entity, e.g. active state for both DL and UL, or active state for DL only - FFS: details of this information (e.g. minimal activation/selection delay for a panel) and signaling (e.g. L1 or L2 signaling) - UE-reported information in MPE report (if supported) is used to indicate the minimal activation/selection delay and panel activation/selection status - Note: above ‘panel entity’ is a logical entity and how to map physical panels to the logical entities is up to UE implementation - Note: This will depend on the final outcome of whether specification support for UE-initiated panel activation/selection is agreed

Because only one panel can transmit at a time, power exposure is on a panel basis. When MPE event occur, it is necessary to switch to another panel for transmission. It is more efficient to report PHR on per-panel basis than per-beam basis.

Proposal 23: Virtual PHR shall be reported on per panel basis for panel selection.

## Latency and overhead reduction

Agreement On Rel.17 enhancements to facilitate advanced beam refinement/tracking, perform study (for the purpose of down-selection and/or combining) and, if needed, specify the following candidate schemes from Group 1: - Opt 1-1A: Beam measurement/reporting/refinement/selection triggered by beam indication (without CSI request) - Opt 1-1B: UE-initiated beam selection/activation based on beam measurement (without beam indication or activation from NW) - Opt 1-2: Semi-static NW-configured beam selection (without beam indication and measurement/reporting) - Opt 1-3: SSB grouping to reduce beam training - Opt 1-4: Aperiodic beam measurement/reporting based on multiple resource sets for reducing beam measurement latency - Note: Aim for at most one solution for Group 1 in Rel-17 to address issue 6 Agreement On Rel.17 enhancements to facilitate advanced beam refinement/tracking, perform study (for the purpose of down-selection and/or combining) and, if needed, specify the following candidate schemes from Group 2: - Opt 2-1A: Latency reduction for MAC CE based TCI state activation, or frequency/time/beam tracking - Opt 2-1B: Latency reduction for MAC CE based PL-RS activation - Opt 2-1C: Latency reduction for MAC CE based PUCCH resource/resource group activation - Opt 2-2: Direct SCell TCI state activation - Opt 2-3: Replacing RRC-based with MAC CE (or DCI) based for DL QCL or UL information update - Opt 2-4: One-shot timing update for TCI state update - Note: Aim for at most one solution for Group 2 in Rel-17 to address issue 6 - Note: At least for Opt 2-1A/B, 2-2, and 2-4, RAN2 and RAN4 will at least have to be involved (some may be exclusively RAN2 and/or RAN4 work)

Overhead and latency reduction is a very important topic for the on multi-beam operation, and a lot of enhancements are specified in Rel-16. For example, a lot of new MAC CEs based updating/reconfiguration are designed to reduce the configuration overhead and latency such as MAC CE based spatial relation updating for aperiodic SRS and MAC CE based pathloss reference RS updating for PUSCH and SRS in Rel-16. Besides, simultaneously updating of TCI states and spatial relation for PDSCH/PDCCH and PUSCH/PUCCH/SRS for multiple cells by a MAC CE is also specified in Rel-16. For multi-beam operation, enhancement on overhead and latency reduction is still an important topic and should be discussed and studied in Rel-17. This is especially important for high mobility UEs such as UEs on high speed train. Therefore, those channels whose RRC configurations may be updated frequently should be discussed to see whether it can be updated by dynamic control signaling such as MAC CE or DCI. For example, for SRS resource set configured with the usage as ‘non-codebook’, an associated NZP CSI-RS can be configured for it. Besides, as we know that the spatial relation information of SRS resources in the SRS resource set configured with ‘non-codebook’ and the associated NZP CSI-RS for the SRS resource set should not be configured simultaneously. Consider the overhead of configuring associated NZP CSI-RS for the SRS resource set is smaller than the overhead of configuring the spatial relation information for each SRS resource in the SRS resource set, it’s better to configure an associated NZP CSI-RS for the SRS resource set while not configure spatial relation information for SRS resources in the set. However, the associated NZP CSI-RS can only be configured by RRC for the SRS resource set configured with ‘non-codebook’ whose updating has a large overhead consumption and a large latency especially for aperiodic SRS resource set. In order to reduce the overhead and latency of updating of associated NZP CSI-RS for SRS resource set configured with ‘non-codebook’, a MAC CE can be designed to indicate the associated NZP CSI-RS for the set. Therefore, we propose that:

Proposal 24: Introduce dynamic control signaling such as MAC CE or DCI to further reduce overhead and latency for RRC configuration.

Proposal 25: Design a MAC CE to indicate the associated NZP CSI-RS for SRS resource set configured with ‘non-codebook’ in order to reduce the overhead and latency.

# Conclusion

We have discussed unified TCI framework, dynamic TCI update using DCI and MAC-CE, L1/L2-centric inter-cell mobility, multi-panel UE, MPE, and further optimization of multi-beam operation for R17 FeMIMO. Our proposals are summarized as below:

Proposal 1: DL or joint DL/UL TCI can be applied to subset of CSI-RS for CSI and TRS.

Proposal 2: UL TCI can be applied to subset of SRS resources.

Proposal 3: The subset of CSI-RS and SRS resources a new TCI applies is dynamically signaled.

Proposal 4: Whether the TCI field points to joint DL/UL TCI or separate DL/UL TCI depends on the context of the DCI (Alt1).

Proposal 5: PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state.

Proposal 6: The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state.

Proposal 7: Use CSI-RS for mobility for L1-RSRP measurement for L1/L2-centric inter-cell mobility.

Proposal 8: Does not allow mixed SSB and CSI-RS L1-RSRP report in a same CSI report.

Proposal 9: Add a bitmap field to DCI format 1_1/1_2 with DL assignment to signal which channels and resources the TCI applies to.

Proposal 10: Adopt Alt2A for the application time of the beam indication: the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication.

Proposal 11: Reuse the UE capability timeDurationForQCL for minimal DL beam switching time.

Proposal 12: Introduce a new UE capability parameter for minimal UL beam switching time.

Proposal 13: Use a dedicated ACK/NACK bit in the HARQ-ACK codebook of the scheduled PDSCH as acknowledgement of the DCI.

Proposal 14: Support simultaneous TCI update for multiple CCs and for multiple BWPs.

Proposal 15: Support simultaneous TCI state updating for UL and DL in a same MAC-CE.

Proposal 16: Consider combined MAC CE-DCI approach to reduce the latency and overhead for TCI updates.

Proposal 17: Determine whether a PDSCH or a PUSCH can use another beam indicated by a legacy DCI which is not used for common beam indication when a common beam is applicable for DL or UL.

Proposal 18: Further study how to distinguish DCIs for indicating common beams from legacy DCIs.

Proposal 19: Determination of a TCI field in the DCI for a common beam indication is FFS.

Proposal 20: Explicit panel ID should be defined for panel related state information reporting.

Proposal 21: Report the association relation between a beam and an activated panel to gNB as part of beam reporting.

Proposal 22: Study both explicit and implicit method of reporting of the association between a beam and an activated panel.

Proposal 23: Virtual PHR shall be reported on per panel basis for panel selection.

Proposal 24: Introduce dynamic control signaling such as MAC CE or DCI to further reduce overhead and latency for RRC configuration.

Proposal 25: Design a MAC CE to indicate the associated NZP CSI-RS for SRS resource set configured with ‘non-codebook’ in order to reduce the overhead and latency.

# References

- RP-193133 WID proposal FeMIMO.

- Chainman’s notes of RAN1#102e.

- Chainman’s notes of RAN1#104e.

- Chainman’s notes of RAN1#104e.

- Chainman’s notes of RAN1#104bis-e.