3GPP TSG RAN WG1 Meeting #105-e                            	      R1-2104448

E-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.15.2

Source:	Spreadtrum Communications

Title:	Consideration on enhancements to time and frequency synchronization for IOT NTN

Document for:	Discussion and decision

Introduction

The study on NB-IoT and eMTC over Non-Terrestrial Networks (NTNs) is approved in [1]

This Study will evaluate and confirm solutions to address the minimum necessary specifications for NB-IoT and eMTC according to the following objectives. The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including: - Bands of interest in sub 6 GHz - Device type with PC3 or PC5 (LEO and GEO) - Satellite constellation orbit LEO and GEO - Transparent payload. - Link budget NOTE 1: This first objective will be based on the scenarios documented in TR 38.821. NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions. The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items: - Aspects related to random access procedure/signals [RAN1, RAN2] - Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2] - Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2] - Aspects related to HARQ operation [RAN2, RAN1] - General aspects related to timers (e.g. SR, DRX, etc.) [RAN2] - RAN2 aspects related to idle mode and connected mode mobility [RAN2] - RLF-based for NB-IoT - Handover-based for eMTC - System information enhancements [RAN2] - Tracking area enhancements [RAN2] NOTE 3: GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices. With this assumption, UE can estimate and pre-compensate timing and frequency offset with sufficient accuracy for UL transmission. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. Recommendations for NB-IoT and recommendations for eMTC will be documented in the conclusions.

In this contribution, we share our initial views on time and frequency synchronization enhancements to NB-IoT/eMTC over NTN.

GNSS measurements

In RAN1#104b_e, In RAN1#104e, the following FL recommendation had been made.

FL recommendation – Section 2.1.1: - Companies are encouraged to further discuss scenarios, motivation and solution to ensure there is a sufficient gap for GNSS measurements in idle UE or connected UE and to discuss offline to align on understanding of legacy procedures (i.e. configuration of paging, DRX timers). Further consider whether issue should first be discussed in RAN2.

How frequent the UE need to perform GNSS measurement depends on packet arrival rate, UE speed, etc. For example, a stationary UE may need to do GNSS measurement only once while a moving UE may need to perform GNSS measurement when a significant position change occurs. The duration of GNSS measurement depends on scenarios, e.g. cold/warm/hot start, and also relate to hardware implementation.

Since Rel 17 is focused on the use case of intermittent delay-tolerant small packet transmission and other use cases not requiring significant enhancements. In this case, there is no need to do GNSS measurements in connected mode in Rel 17. Therefore, R17 does not need to introduce measurement gaps for CNSS measurements. Furthermore, regarding of triggering mechanisms for GNSS measurements, our view is that UE should perform GNSS measurements before moving to connected mode.

Proposal 1: R17 does not need to introduce measurement gaps for CNSS measurements.

Proposal 2: UE should perform GNSS measurements before moving to connected mode.

UL timing synchronization for IOT NTN

UL timing synchronization in RRC_IDLE and RRC_INACTIVE states

In RAN 103e meeting, the following agreement on time synchronization have been achieved in NR NTN [2].

Agreement: An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

In IoT-NTN, UEs are assumed with GNSS capability in IOT NTN. Therefore, pre-compensation methods proposed for UL synchronization for RRC_IDLE and RRC_INACTIVE states UE in NR-NTN, e.g., geometric based solution, can also be reused in IoT-NTN.

Proposal 3: UL timing compensation mechansim in RRC_IDLE and RRC_INACTIVE states of NTN WI can be reused in IoT NTN.

UL timing synchronization in RRC_CONNECED states

In RAN 104e meeting, the following agreement on time synchronization have been achieved in NR NTN [3].

Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. - FFS: Operation of closed loop and open loop TA control

In IoT-NTN, UEs are assumed with GNSS capability in IOT NTN. Therefore, pre-compensation methods proposed for UL synchronization for RRC_ CONNECED states UE in NR-NTN, e.g., geometric based solution, can also be reused in IoT-NTN.

Proposal 4: UL timing compensation mechansim for RRC_ CONNECED states UEs of NTN WI can be reused in IoT NTN.

Reference point for autonomous acquisition of the TA at UE

In RAN1#102e, it have identified the following 3 options regarding the RP (Reference point) definition in NR NTN.

- Option1: The RP is located at the gNB.

- Option2: The RP is located at the satellite.

- Option3: The RP localization is not specified and left to the implementation.

For option1, the UE specific TA which is autonomously acquired by the UE based on its GNSS capabilities and additional network indications (e.g. satellite ephemeris or time stamp). In this case, the Common TA corresponds to the RTD experienced between the RP and the satellite need to be indicated by the network.

For option2, the UE specific TA which is autonomously acquired by the UE based on its GNSS capabilities and additional network indications (e.g. satellite ephemeris or time stamp). In this case, the Common TA which corresponds to the RTD experienced between the RP and the satellite does not need to be indicated to the UE.

For option3, the RP localization is not specified and left to the implementation. The initial TA acquisition is computed as the sum of two distinct contributions:

- The UE specific TA which is autonomously acquired by the UE based on its GNSS capabilities and additional network indications.

- The common TA which is indicated by the network. It corresponds to the RTD experienced between the RP and the satellite.

In our view, the TA change caused by the feeder link can be compensated by the network, and the network does not need to broadcast the common TA corresponds to the RTD experienced between the RP and the satellite. Otherwise it will cause a serious signaling overhead problem.

Furthermore, since the duration for PRACH or NPUSCH transmission is very long, especially with large repetition number. Considering the rapid changes of the common TA corresponds to the RTD experienced between the RP, UE needs to perform multiple TA adjustments of feeder link in once transmission time interval if the RP is not located at the gNB. In this case, the Common TA corresponds to the RTD experienced between the RP and the satellite need to be indicated by the network frequently during the repetitions, which needs to be avoided.

Therefore, the RP is located at the satellite in IOT NTN should be supported.

Proposal 5: Reference point for autonomous acquisition of the TA at UE is located at the satellite in IOT NTN.

TA Maintenance procedure

In NTN, the radial velocity between the satellite and UE location can reach significant values especially in NGSO scenarios. As a consequence, the propagation delay changes quickly. In case of LEO transparent payload at 600km, the maximum NTN GW-UE delay variation as seen by the UE can be up to +/- 40 µs/sec. In these conditions, the maintenance procedure based on MAC-CE only may become rather challenging because the UE-specific timing adjustment MAC-CE commands must be sent very often leading to critical DL signaling overhead. Autonomous adjustment of the TA before UL transmission by the UE can significantly reduce signalling overhead in connected mode.

In RAN1#104e, the following agreement on TA update in RRC_CONNECTED state has been achieved in NR NTN [3].

Agreement: For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. - FFS: Details of the combination of open and closed loop TA control

For open loop method, UE can use an acquired GNSS position and satellite ephemeris to calculate the TA for service link and apply the calculated values accordingly in RRC_CONNECTED states. The TA change caused by the feeder link can be compensated by the network. If UE can obtain accurate TA based on GNSS capability and satellite ephemeris automatically, TA Maintenance can be relying on open-loop method only. Otherwise, closed loop method can be considered to help UE to obtain accurate TA, i.e., received TA commands.

Proposal 6: Both open and closed control loops are supported in connected mode for IOT NTN.

UL frequency synchronization for IOT NTN

UL frequency pre-compensation

In RAN 103e meeting, the following agreement on frequency synchronization have been achieved in NR NTN [2].

Agreement: An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

In IoT-NTN, UEs are assumed with GNSS capability. Therefore, pre-compensation methods proposed for UL synchronization in NR-NTN, e.g., geometric based solution, can also be reused in IoT-NTN.

Proposal 7: Frequency compensation mechanism of NTN WI can be reused in IoT NTN.

Reference point for UL frequency synchronization

In RAN1#103e, the following FL recommendations are made in NR NTN.

FL recommendation-3-3: Companies are invited to provide feedback about the three listed options for regarding the reference point definition: - Option 1: The reference point for frequency in an NTN is located at the satellite - Option 2: The reference point for frequency in an NTN is located at the gNB - Option 3 : The reference point for frequency in an NTN is under control of the network

For Option 1, the feeder link Doppler shift and transponder frequency error can be compensated by gNB and UE does not need to handle the frequency offset between satellite and gNB. In this case, there is no need to signal the common Doppler shift and Doppler drift rate due to feeder link and also the transponder frequency error.

For Option 2, as UE cannot estimate the Doppler shift over the feeder link and the transponder frequency error, the common Doppler shift and Doppler drift rate due to feeder link and also the transponder frequency error need to be indicated by the network.

For Option 3, the reference point for frequency in an NTN is under control of the network. In this case, the UE shall be responsible for determining the frequency offset required for frequency alignment at the satellite. However, the network may configure additional frequency offsets that are signalled to the UE. The signalled offsets are determined by the gNB and may be used to move the reference point.

In our view, in order to avoid increasing the complexity of the NB-IOT terminal and signaling overhead, Option 1 should be prioritized. IOT NTN UE does not need to handle the frequency offset between satellite and gNB.

Proposal 8: In IOT NTN, the reference point for frequency synchronization is located at the satellite.

Time and frequency synchronization enhancement specifically for IOT NTN

In NB-IOT, the duration for PRACH or PUSCH transmission is very long, especially with large repetition number. Considering the rapid time-frequency changes in the satellite scenario, UE needs to perform multiple time-frequency offset adjustments in once transmission time interval. In this case, updates on the pre-compensation value of time delay and frequency offset during a transmission time interval is expected. In RAN 104e meeting, the following agreement have been achieved in IOT NTN [3].

Agreement: Study the UE pre-compensation of satellite delay during long UL transmission on (N)PUSCH in NB-IoT and eMTC. Agreement: Study the UE pre-compensation of satellite delay and Doppler during long UL transmission on PRACH in NB-IoT and eMTC. Agreement: Study the UE pre-compensation of satellite Doppler shift during long UL transmission on (N)PUSCH in NB-IoT and eMTC.

In order to solve the problem of frequency drift in UL transmission caused by the low cost crystal oscillations, UL Gap is introduced in Rel-13 NB-IoT. During the UL Gap the UE can correct the frequency drift by its implementation. In the existing specifications, the transmission gap of 40ms is inserted after a period of continuous PRACH and/or PUSCH transmission.

For NPUSCH transmission [36.211]:

After transmissions and/or postponements due to NPRACH of time units, for frame structure type 1, a gap of time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap.

For PRACH transmission [36.211]:

NPRACH transmission can start only time units after the start of a radio frame fulfilling . For frame structure type 1, after transmissions of time units for preamble formats 0 and 1, or 16∙6(TCP+TSEQ) time units for preamble format 2, a gap of time units shall be inserted.

Long UL transmission on PUSCH

In RAN1#104b_e, the following agreement on UE pre-compensation during long UL transmission on PUSH had been achieved [4].

Agreement: UE pre-compensation done per N time units for long PUSCH is the baseline solution. - The pre-compensation does not vary within a block of N time units - FFS: the definition and value of N

Furthermore, in RAN1#104b_e, the following FL recommendation had been made.

FL recommendation – Section 2.4.1.1: Companies are encouraged to further discuss aspects of long PUSCH transmission w.r.t. (segmented) UE pre-compensation (slot-by-slot or over N slots) without UL gap, segmented UE pre-compensation (slot-by-slot or over N slots) with UL gap. Companies are also encouraged to provide more analysis and discussions on the scenarios and the expected behaviour of the UE during the new gaps if configured - i.e. scenario / use case targeted (sporadic short transmission, long connection, ..), whether UE is supposed to read NTN SIB carrying the ephemeris.

UE pre-compensation done per N time units for long PUSCH is the baseline solution and the definition and value of N is FFS. Regarding the definition of N, our view is that PUSCH repetition unit can be used as the granularity of N. The value of N depends on tolerable timing and frequency shift and it can be discussed in normative phase.

Proposal 9: PUSCH repetition unit is used as the granularity of N for long PUSCH should be supported.

Due to the movement of the satellite, the TA value that the UE needs to pre-compensate will also change accordingly. Therefore different segments of long PUSCH (i.e. N time units) generally apply different TA pre-compensation values. When the UE's TA pre-compensation value changes from large to small, two adjacent segments will not overlap, as shown in Figure 1 (TA1>TA2). When the UE's TA pre-compensation value changes from small to large, two adjacent segments will overlap, as shown in Figure 2 (TA1<TA2).

Figure 1: the UE's TA pre-compensation value changes from large to small

Figure 2: the UE's TA pre-compensation value changes from small to large

In our view, inserting a gap between adjacent segments (N time units) to avoid the overlap of segments when the UE's TA pre-compensation value changes from small to large should be supported.

Proposal 10: Inserting a gap between adjacent segments (N time units) to avoid the overlap of segments for long PUSCH should be supported.

Long transmission on PRACH

In IOT NTN, the delay drift can be up to 20 us/s on service link or feeder link. NB-IoT UE supports three CP lengths, 66.7us, 266.7us, and 800us. The maximum transmit error for NB-IoT is 80*Ts=2.6 us (for eMTC, it is 24*Ts=0.78 us). The maximum number of NPRACH transmission is 1024 in NB-IoT, which gives a total RACH transmission of about 20 s. With the maximum time duration of PRACH preamble and without UE pre-compensation of satellite delay, the transmit timing error requirement cannot be met and the cyclic prefix is not large enough for any of the RACH preamble formats. In RAN1#104b_e, the following agreement on UE pre-compensation during long UL transmission on PUSH had been achieved [4].

Agreement: UE pre-compensation done per N time units for long PRACH is the baseline solution. - The pre-compensation does not vary within a block of N time units - FFS: the definition and value of N

Furthermore, in RAN1#104b_e, the following FL recommendation had been made.

FL recommendation – Section 2.5.1: Proponents of new gaps are encouraged to provide more analysis and discussions on the scenarios and the expected behaviour of the UE during the new gaps - i.e. scenario / use case targeted (sporadic short transmission, long connection, ..), whether UE is supposed to read NTN SIB carrying the ephemeris, whether they see an issue with the “baseline” solution requiring new gaps, periodicity and duration of the gaps, and so on) for the long PRACH.

UE pre-compensation done per N time units for long PRACH is the baseline solution and the definition and value of N is FFS. Regarding the definition of N, our view is that preamble repetition unit can be used as the granularity of N. The value of N depends on tolerable timing and frequency shift and it can be discussed in normative phase. Furthermore, in NB-IOT, the total number of symbol groups in a preamble repetition unit is denoted by P and the preamble consisting of P symbol groups shall be transmitted   times. Therefore, P symbol groups can be used as the granularity of N for long PRACH.

TS 36.211

#### 10.1.6.1 Time and frequency structure The physical layer random access preamble is based on single-subcarrier frequency-hopping symbol groups. A symbol group is illustrated in Figure 10.1.6.1-1, consisting of a cyclic prefix of length and a sequence of identical symbols with total length. The total number of symbol groups in a preamble repetition unit is denoted by P. The number of time-contiguous symbol groups is given by G. The parameter values for frame structures 1 and 2 are listed in Tables 10.1.6.1-1 and 10.1.6.1-2, respectively. Figure 10.1.6.1-1: Random access symbol group Table 10.1.6.1-1: Random access preamble parameters for frame structure type 1 Preamble format. 0; 4; 4; 5. 1; 4; 4; 5. 2; 6; 6; 3; 3∙24576Ts.

Proposal 11: Preamble repetition unit (i.e. P symbol groups) is used as the granularity of N for long PRACH is should be supported.

According to the analysis in section 5.1, inserting a gap between adjacent segments (N time units) to avoid the overlap of segments for long PRACH when the UE's TA pre-compensation value changes from small to large should be supported.

Proposal 12: Inserting a gap between adjacent segments (N time units) to avoid the overlap of segments for long PRACH should be supported.

Conclusion

As summary, we have the following proposals.

Proposal 1: R17 does not need to introduce measurement gaps for CNSS measurements.

Proposal 2: UE should perform GNSS measurements before moving to connected mode.

Proposal 3: UL timing compensation mechansim in RRC_IDLE and RRC_INACTIVE states of NTN WI can be reused in IoT NTN.

Proposal 4: UL timing compensation mechansim for RRC_ CONNECED states UEs of NTN WI can be reused in IoT NTN.

Proposal 5: Reference point for autonomous acquisition of the TA at UE is located at the satellite in IOT NTN.

Proposal 6: Both open and closed control loops are supported in connected mode for IOT NTN.

Proposal 7: Frequency compensation mechanism of NTN WI can be reused in IoT NTN.

Proposal 8: In IOT NTN, the reference point for frequency synchronization is located at the satellite.

Proposal 9: PUSCH repetition unit is used as the granularity of N for long PUSCH should be supported.

Proposal 10: Inserting a gap between adjacent segments (N time units) to avoid the overlap of segments for long PUSCH should be supported.

Proposal 11: Preamble repetition unit (i.e. P symbol groups) is used as the granularity of N for long PRACH is should be supported.

Proposal 12: Inserting a gap between adjacent segments (N time units) to avoid the overlap of segments for long PRACH should be supported.

Reference

RP-210868, “Study on NB-IoT/eMTC support for Non-Terrestrial Networks”, MediaTek Inc.

3GPP RAN1#103e Chairman’s Notes, October 26 - November 13, 2020.

3GPP RAN1#104e Chairman’s Notes, January 25th – February 5th, 2021.

3GPP RAN1#104b_e Chairman’s Notes, April 12 - April 20, 2021.