3GPP TSG RAN WG1 #105-e                                             R1-2104506

e-Meeting, May 19th –May 27th, 2021

Agenda Item:	8.15.4

Source:	CATT

Title:	         HARQ operation enhancement for NB-IoT/eMTC

Document for:	Discussion and Decision

Introduction

For the enhancements on NB-IoT/eMTC HARQ, there were some discussions on enhancement on HARQ process number, Disabling/enabling HARQ feedback and other transmission enhancements etc. Moreover, a few agreements on HARQ operation were concluded as follows [1] in last RAN1 meeting:

Agreement:

Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

In this contribution, we further analyze remaining technical issues of NB-IoT/eMTC HARQ operation and provide our views on these issues.

Discussion

  - Disabling HARQ feedback

To avoid reduced throughput caused by HARQ stalling, disabling of HARQ feedback has been agreed in NR NTN. Although the peak data rate requirements of IoT NTN can be met without disabling HARQ feedback, the reduction in peak data rate due to HARQ stalling is considerable, especially for GEO. Since it had been agreed that increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17, disabling HARQ feedback can be reused in some case to provide wide coverage, such as the coverage enhancement mode CEmodeB.

Proposal 1: Reuse disabling HARQ feedback mechansim of NR NTN for CEmodeB in eMTC NTN.

In other cases, considering limited data packet in NB-IoT NTN and CEmodeA eMTC over satellite traffic, disabling HARQ can’t bring benefits to improve data transmission capability. Consequently, it is not recommended to disable HARQ feedback.

Observation 1: Disabling HARQ feedback doesn’t show clear benefit to NB-IoT NTN and CEmodeA eMTC use case.

Proposal 2: No enhancement in disabling HARQ feedback is needed for HARQ in NB-IoT NTN and CEmodeA eMTC over satellite.

Moreover, to ensure reliability, totally disabling the HARQ feedback should be avoided, and at least one HARQ process with feedback needs to be configured. At the same time, it is not allowed to disable HARQ process in case of single HARQ process.

Proposal 3: Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling.

  - Reduced PDCCH monitoring

Since low power consumption is the main characteristic of IoT devices, reducing PDCCH monitoring is an LTE IoT feature, which is related to whether there is an available HARQ process. Since it is not allowed to disable HARQ process in case of single HARQ process, the PDCCH would not be received before RTT after the end of the transmission of the corresponding PUSCH. Therefore for an NTN UE that is configured with one HARQ process, the UE does not monitor PDCCH until the RTT time has elapsed from the end of the PUSCH.

Proposal 4: For an NTN UE configured with one HARQ process and for which HARQ feedback is enabled, the UE will not monitor PDCCH until the RTT time has elapsed from the end of the PUSCH.

When an NTN NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config, and two processes are scheduled together, the interval between two PDCCHs scheduled by the same HARQ ID is RTT + processing time, in which PDCCH monitor is meaningless.

Proposal 5: For an NTN UE configured with two HARQ processes, and two processes are scheduled together, and for which HARQ feedback is enabled, the UE can skip PDCCH monitoring until RTT after the end of the reception of the last PDCCH.

When an NTN NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config, and two processes are scheduled one by one, the UE should monitor PDCCH for new HARQ process after the last PUSCH transmission.

Proposal 6: For an NTN UE configured with two HARQ processes, there is no need for the enhancement on PDCCH monitoring.

Conclusion

In this contribution, we analzyed potential issues of HARQ impact due to longer propogation delay in NB-IoT and eMTC over satellite. In sum, the observations and proposals are listed as follows:

Observation 1: Disabling HARQ feedback doesn’t show clear benefit to NB-IoT NTN and CEmodeA eMTC use case.

Proposal 1: Reuse disabling HARQ feedback mechansim of NR NTN for CEmodeB in eMTC NTN.

Proposal 2: No enhancement in disabling HARQ feedback is needed for HARQ in NB-IoT NTN and CEmodeA eMTC over satellite.

Proposal 3: Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signalling.

Proposal 4: For an NTN UE configured with one HARQ process and for which HARQ feedback is enabled, the UE will not monitor PDCCH until the RTT time has elapsed from the end of the PUSCH.

Proposal 5: For an NTN UE configured with two HARQ processes, and two processes are scheduled together, and for which HARQ feedback is enabled, the UE can skip PDCCH monitoring until RTT after the end of the reception of the last PDCCH.

Proposal 6: For an NTN UE configured with two HARQ processes, there is no need for the enhancement on PDCCH monitoring.

References

- RAN1 #104b-e Chairman notes