3GPP TSG RAN WG1 #105-e                                                                            R1-2104533

e-Meeting, May 19th – 27th, 2021

# Source:             CATT

Title:                  Configuration of TRS/CSI-RS for paging enhancement

Agenda Item:    8.7.1.2

Document for:  Discussion and Decision

Introduction

In RAN1#104b-e meeting, the following was agreed for UE power saving on configuration of TRS/CSI-RS occasions for idle/inactive UEs[1].

Agreement:

SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0.

Agreement:

Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs.

- FFS details of the QCL information, e.g. associated SSB index

Agreement:

IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP.

- Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP.

Working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signaling

Agreement:

Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations

- Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs

- If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured.

  - The parameter trs-info does not need to be provided in the configuration

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported:

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

- FFS whether and how to indicate the ‘availability’ in beam selective manner.

- Other alternatives are not precluded

In this contribution, TRS/CRS configuration details for paging enhancement for UE in RRC-Idle/Inactive mode are discussed.

Discussion

Potential TRS/CSI-RS occasion(s) for paging enhancement

As agreed in RAN1#103-e meeting [2], SIB signaling is utilized to provide the configuration of TRS/CSI-RS occasion(s) for Idle/Inactive-mode UE(s). The following aspects need to be considered for RRC Idle/Inactive mode UEs.

- How to configure TRS/CSI-RS occasion to assist UE achieving significant power saving gain?

- Considering that gNB does not know which cell RRC_IDLE/Inactive mode UE is camped, how the resource configuration and the resource availability indication of TRS/CSI-RS are incorporated in the paging strategy?

- When TRS/CSI-RS is configured on-demand, what is the trigger event?

TRS/CSI-RS configuration

- Higher layer signaling

The legacy TRS/CRS-RS resource/resource set configuration based on RRC signaling for RRC_CONNECTED mode UE can be considered as the framework of signaling in the SIB design  for the configuration of TRS/CSI-RS occasion(s) for RRC IDLE/Inactive mode UE(s). Legacy TRS/CRS-RS resource/resource set configuration procedure/parameters details are shown in Figure 1. We can see from Figure 1 that roughly ten bytes are required for each NZP-CSI-RS Resource configuration. Considering that one TRS Resource set contains 4 NZP-CSI-RS Resources per beam for FR1, the required signaling overhead for maximum 64 beams is significantly higher than the maximum size of SIB1 or SI message of 2976 bits as specified in TS38.331 [3]. Therefore, TRS/CSI-RS configuration with potential large size of signaling may need to be configured at another standalone SIB-X with the present of SIB X indicated by SIB1.

Observation 1: TRS/CSI-RS configuration with potential large size of signaling may need to be configured at another standalone SIB-X with the present of SIB-X indicated by SIB1.

Proposal 1: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit.

Figure 1:  Example of TRS/CRS-RS resources/resource sets configuration

- TRS/CSI-RS resource configuration in Time domain

Two candidates can be utilized to configure TRS/CRS-RS occasions for paging reception using SIB.

- Option1: TRS/CSI-RS resources are configured with SI without association relation with paging occasion(s)

- Option 2:TRS/CSI-RS resources  are associated with the paging occasion(s)

For option1, if  one or multiple TRS/CSI-RS resources/ resource sets can be configured with SIB signalling which TRS/CSI-RS resource configuration usually at least contains parameters of TRS/CSI-RS pattern /resource mapping/gold sequence scrambling ID/ multi-beam QCL information, etc., which will cause huge SIB overhead as shown in Figure 1. gNB can share the TRS/CSI-RS occasions intended for CONNECTED mode UE to IDLE/inactive UEs.  Once such TRS/CSI-RS resource is no longer used by CONNECTED mode UE, gNB needs reconfigure TRS/CSI-RS resource using update of the system information by paging all IDLE/Inactive mode UEs. UE will read system information block update to acquire new TRS/CSI-RS resource configuration information which will penalize power saving gain of TRS/CSI-RS occasion(s) obviously.  Furthermore, when TRS occasion(s) are configured without association relation with PO(s), e.g., TRS/CSI-RS can be configured far behind PO boundary, UE could not use such TRS/CSI-RS configuration for channel tracking which will degrade UE power saving gain significantly.

Observation 2: With CSI-RS resources configured with SI without association with paging occasion(s), UE might not use such TRS configuration for channel tracking which will degrade UE power saving gain significantly.

In general, the TRS/CSI-RS would be configured for all UEs to minimize the overhead in a cell even they are UE-specifically configured. As TRS/CSI-RS resource configuration usually changes infrequently for CONNECTED mode UEs since it is shared by multiple UEs in the cells, the same TRS/CSI-RS resource could be shared to IDLE mode UEs.   From the other angle, the TRS/CSI-RS resource configured for IDLE mode UE for power saving purpose should be prioritized as the TRS/CSI-RS resource to be configured for CONNECTED mode UEs to minimize the system overhead.

Observation 3:  The TRS/CSI-RS resources configured for CONNECTED mode UEs can be shared to IDLE mode UE.

For option2, the potential TRS/CSI-RS resources are configured and associated with the SSB/paging occasion. As shown in Figure 2, the TRS/CSI-RS configuration could be configured similar to PS_offset configured for UE to decode DCI format 2_6 with a window. TRS/CSI-RS occasion could be configured within the dedicated gap between TRS/CSI-RS occasion and paging occasion. The configuration of TRS/CSI-RS resource grid for IDLE/Inactive UEs in SIB-X could be simplified with reduced number of parameters to reduce, signaling overhead.

For TRS/CSI-RS resource used by CONNECTED mode UE, it is the choice of the network implementation to configure such TRS/CSI-RS resource used for both IDLE and CONNECTED mode UE. Since UE starts with IDLE mode first, a specific configuration of TRS/CSI-RS could help IDLE mode UE to reduce the power consumption. Once UE enters CONNECTED mode, gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE. It implies that a set of TRS/CSI-RS resource are always transmitted for IDLE mode UEs and could be used by CONNECTED mode UEs.  Furthermore, considering that the paging indication (sequence or DCI based) would be transmitted on every paging cycle and without transmission of PEI, TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted which is gNB’s implementation, TRS/CSI-RS bundled with SSB/paging occasion should not be considered as always on signal. Considering that option 2 has lower specification efforts and higher UE power saving gain that option 1, TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s).

Figure 2: TRS/CSI-RS occasion is associated with PO/SSB

Observation 4: With TRS/CSI-RS occasion associated with SSB/paging occasion, it will provide significant power saving gain at cost of low configuration signaling overhead.

Observation 5: gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE.

Observation 6: Considering that the paging indication (sequence or DCI based) would be transmitted on every paging cycle, TRS/CSI-RS bundled with SSB/paging occasion should not be considered as always on signal.

Proposal 2: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) to achieve good power saving gain with low SIB signaling overhead.

- TCI state configuration

It was agreed in RAN1#104-e meeting that multiple RS resources can be configured for TRS/CSI-RS occasion(s) for IDLE/Inactive UEs.  While paging information is transmitted in multiple beams, multiple TRS resources configuration with one TRS/CSI-RS resource associated with one beam is needed to achieve the UE power saving. In TRS/CSI-RS configuration structure, TCI state is configured per CSI-RS resource set to support the CONNECTED mode UE with dynamic beamforming operation. Once UE is configured with the TRS resource set for channel tracking, one TRS resource set includes two CSI-RS resources in a slot or four CSI-RS resources in two consecutive slots will be configured with same TCI state. Furthermore, multiple TRS resources with same TCI state configured for different connected UEs could be provided to IDLE/Inactive mode UE for AGC/channel tracking performance improvement of IDLE/Inactive UEs. Up to 64 CSI-RS resources can be configured for one TRS/CSI-RS resources set as in TS38.331 [3]. Compared with configuration of TCI state per CSI-RS resource, one CSI-RS/TRS resource set can be configured with one TCI state which can reduce unnecessary signaling overhead significantly.

Observation 7: Compared with configuration of TCI state per CSI-RS resource, one TRS resource set can be configured with one TCI state which can reduce unnecessary signaling overhead.

Proposal 3: QCL information configuration of TRS for idle/inactive UE should be configured per CSI-RS resource set.

In RAN1#104b-e, the QCL assumption of TRS/CSI-RS occasion(s) for IDLE/Inactive UEs was agreed to be configured by higher layer as follows,

Agreement:

Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs.

	FFS details of the QCL information, e.g. associated SSB index

The configuration of TRS/CSI-RS resource set and the QCL assumption associated with each TRS/CSI-RS resource set are included in SIB-X for IDLE/Inactive UEs.  The TCI state of the TRS/CSI-RS resource would be set different for each beam in a given cell.  In order to inform UE the  TCI state of TRS/CSI-RS resources after detecting a SSB and acquiring coarse timing and channel information, the CSI-RS resource set needs to be linked to the SSB ID. UE can know the TRS/CSI-RS resource to use after detecting SSB and the associated SSB ID at a given cell. QCL information of TRS/CSI-RS occasion(s) should be explicitly indicated to be associated with SSB ID by SIB-X for IDLE/Inactive UEs, which can provide the most direct and flexible QCL assumption of TRS occasion(s) per CSI-RS resource set.

Proposal 4: QCL information of TRS/CSI-RS occasion(s) should be explicitly indicated to be associated with SSB ID by SIB-X for IDLE/Inactive UEs.

- TRS/CSI-RS occasion(s) configuration procedure

It was agreed in RAN1#102-e [4] that TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported. If TRS/CSI-RS resources associated with the paging occasion(s), an example of periodic TRS/CSI-RS configuration procedure is shown as follows,

Periodic TRS/CSI-RS occasion(s) configuration procedure:

Step1) predefined parameters of TRS/CSI-RS resource grid

Step 2) SIB indicates parameters details, including

- QCL assumption of the configured TRS/CSI-RS resources associated with a SSB;

- Code points/bitmap mapping of availability/not availability for a given TRS resources (see section 2.1.2).

Step 3）To derive TRS occasion(s) according to predefined rule and parameters provided by step1 and step 2

The following two alternatives can be considered to support periodic TRS/CSI-RS configuration.

- Alt. 1: TRS/CSI-RS occasion(s) corresponding to single periodic TRS CSI-RS resource/ resource set

- According to TS 38.304 [5], SFN for the Paging Frame (PF) is determined by paging cycle T and total paging frames number N={T, T/2, T/4, T/8, T/16} in T configured by NAS control signaling [6], where N is the function of T. PF frames corresponding to UEs configured with different paging cycles can be aligned in time domain as shown in Figure 3. Similar to derivation of SFN for the PF, gNB can configure TRS_offset including in the SIB X configured for TRS/CSI-RS resources UE can derive the radio frame for TRS/CSI-RS occasion using the following formula

(SFN + TRS_offset) mod T = (T / N)*(UE_ID mod N)

- Although the paging cycle is UE specific, TRS/CSI-RS occasion corresponds to one cell specific periodic TRS/CSI-RS resource/ resource set from gNB perspective.

Figure 3: TRS/CSI-RS occasion(s) corresponding to single periodic TRS/CSI-RS resource/ resource set

- Alt. 2: TRS/CSI-RS occasion(s) corresponding to multiple periodic TRS CSI-RS resources/ resource sets

gNB also can configure multiple periodic TRS/CSI-RS resources/ resource sets to associate with the paging occasion(s). As shown in Figure 4, gNB can configure Ns TRS_offset for each TRS/CSI-RS resource corresponding to one PO of target paging frame, where Ns is the number of paging occasions in a  PF. From gNB perspective, TRS/CSI-RS occasion(s) correspond to multiple periodic TRS/CSI-RS resources/ resource sets, which provides more flexibility in configuration than that of Alt. 1.

Figure 4: TRS/CSI-RS occasion(s) corresponding to multiple periodic TRS CSI-RS resources/ resource sets

Proposal 5: The following procedure can be used for TRS/CSI-RS occasion(s) configuration:

Step1) predefined parameters of TRS/CSI-RS resource grid;

Step 2) SIB indicate parameters details; including

- QCL assumption of the configured TRS/CSI-RS resources associated with a SSB;

- Code points mapping of availability/not availability for a given TRS resources.

Step 3）To derive TRS occasion(s) according to predefined rule and parameters provided by step1 and step 2.

Availability indication

Signaling

It was agreed in RAN1 #104b-e meeting that the assumption on the availability of TRS/CSI-RS at the configured occasion(s) is explicitly informed to the UE and no blind detection of the presence/absence of TRS/CSI-RS at the UE side. In RAN1#104b meeting, two signaling schemes were discussed for availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- Option 1: L1 signaling, e.g. paging DCI or PEI

- Option 2: SIB, e.g. SIB1 or other SIBs

When TRS/CSI-RS at configured occasion(s) available in connected mode is variable frequently for IDLE/Inactive mode UE, L1 signaling based availability indication of TRS/CSI-RS can be beneficial for system overhead reduction. However, there are at least following issues for indication of availability of TRS/CSI-RS at the configured occasion(s) using L1 based signaling.

- When IDLE/Inactive UE moving to a new cell,   L1 signaling of TRS/CSI-RS availability indication received from original cell is invalid.  If TRS/CSI-RS availability from original cell is not valid after moving to a new cell, UE can’t assume the availability of TRS/CSI-RS at all since UE does not know if UE will be in its original cell or next cell after wakeup from deep sleep.

  - If UE assumes TRS/CSI-RS present at next wakeup, UE would wake up later to use SSB(s) and TRS/CSI-RS in combination for the front-end processing before PO.  When UE detects new cell ID and perform cell reselection, UE might not be able to decode the paging DCI and paging message correctly because the TRS/CSI-RS at the new cell is not available indicated by L1 signaling from previous cell.  This will increase the UE mis-detection rate of paging reception.

  - If UE does not assume TRS/CSI-RS present at next wakeup with the consideration of potential moving to a new cell when it wakes up, UE could not use TRS/CSI-RS at any time indicated by L1 signaling.  Thus, the L1 based availability indication is not useful.

- L1 signaling has backward compatibility issues when DCI is used as the L1 signaling. When the reserved bits or states in the paging DCI were used for availability indication of temporary TRS. The additional bit in DCI would have backward compatibility issues since legacy DCI format does not have this field. The DCI used for paging indication would be common for UEs in all Releases.  If some Rel-17 UEs and legacy UEs are in one paging group, the DCI format would only be compatible for one of them but not both.  Thus, additional bit in DCI for the indication of TRS/CSI-RS availability would have backward compatibility issue.  For instance, the reserved bits in the paging DCI could wake up legacy UE without the need of receiving paging/SI update, which will cause additional power consumption of legacy UE.

- It will be concatenated effects in limiting UE power saving gain and missing the paging message when L1 signaling is missed detected.  Since the miss-detection occurs for IDLE/Inactive UE with L1 signaling, it will create misalignment   between gNB and UE’s understandings on the availability of TRS/CSI-RS.  If UE assumes TRS/CSI-RS being available but gNB does not transmit the TRS/CSI-RS, UE will continue to wake up later and use  SSB and TRS/CSI_RS to  perform the front end processing with fine T/F/sampling synchronization and corresponding compensation operations using false signal. This will cause failure in paging reception and UE power saving gain degradation with a high probability.

Proposal 6: Working assumption that support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs should not be confirmed.

Compared with L1 signaling mechanism, SIB based availability indication of TRS/CSI-RS at the configured occasion(s) for IDLE/Inactive mode UE can provide affirmative resource availability.   If IDLE/Inactive UE can decode the SIB-X and get the configuration and TRS/CSI-RS resource information, UE could use the TRS/CSI-RS resource before PO without any additional overhead. If UE moves to a new cell, UE would read SIB-X to retrieve the information of TRS/CSI-RS resource of new cell. If SIB-X present at the new cell, UE could assume TRS/CSI-RS is available all the time. If gNB decides not to transmit the TRS/CSI-RS for IDLE/Inactive UEs, gNB can send the paging indication to the specific cell only for system information change on the TRS/CSI-RS availability.  There is no additional overhead.

Observation 8: Compared with L1 signaling, SIB based availability indication of TRS/CSI-RS at a given cell for IDLE/Inactive mode UE can provide affirmative TRS/CSI-RS resource availability to achieve UE power saving gain and no additional signaling overhead.

The configuration of TRS/CSI-RS resource is included in SIB-X.  If SIB-X of TRS/CSI-RS for IDLE/Inactive UEs is present, UE should assume TRS/CSI-RS is available for IDLE/Inactive mode UE in the serving cell.  Procedure of SIB based availability indication of TRS/CSI-RS is as follows.

- TRS/CSI-RS resource is configured by SIB-X

  - If SIB-X is present,

    - IDLE/Inactive UE would assume the present of TRS/CSI-RS resources for the subsequent paging occasions

      - UE would assume the TRS/CSI-RS based on the location of TRS/CSI-RS resource for AGC tuning, channel tracking and RSRP measurements

        - For explicitly indication of radio frame and the associated offset, TRS/CSI-RS location is included in the SIB-X for all UEs.

        - For implicitly indication for each UE, TRS/CSI-RS location is derived based on UE ID, SFN, and PF_offset in a formula similar to the PO formula

  - Otherwise

    - IDLE/Inactive UE would assume TRS/CSI-RS resources are not available.

If gNB would like to remove the TRS/CSI-RS resources for IDLE/Inactive UE, gNB can depopulate the SIB-X and send the paging information to all UEs for the system information update. IDLE/Inactive UE could also assume the availability of TRS/CSI-RS resource if the SIB-X for TRS/CSI-RS configuration populates in the system information without any blind decoding.   Upon receiving a SI change notification during a modification period for DRX/eDRX, the UE acquires the new SI immediately from the start of the next modification period. If SIB1 does not schedule SIB-X, UE will assume that there is no TRS/CSI-RS transmission at the configured occasion(s). Whether or not idle/inactive UE needs to read SIB other than SIB-X depends on valueTag contained in the si-SchedulingInfo in SIB1. If valueTag associated with the SIB is not changed compared with previous version, UE can use a valid stored version of the SIB without the need of reading unnecessary SIB information as specified in TS38.331 [3].

Proposal 7: The availability of TRS/CSI-RS at a given cell should be informed to the UE by the present/not present of SIB-X TRS/CSI-RS configuration.

Indication content

Two alternatives were agreed for the TRS/CSI-RS resource information provided by a physical layer availability indication to the IDLE/Inactive UEs in RAN1#104b-e.

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

One paging occasion is configured after each actual transmitted SSB in in a multi-beam cell as in TS38.304 [5]. In order to achieve UE power saving in multi-beam cell, one TRS/CSI-RS resource needs to be configured at each beam.   Since TRS/CSI-RS resources are FDM/TDM’d between different beams within a cell, TRS/CSI-RS resources for all beams need to be configured together in the SIB-X with each beam having QCL assumption associated with the SSB ID in a cell.    If one beam is NOT and the other beams are configured with TRS/CSI-RS resource, UE would not be able to assume the present of TRS/CSI-RS since UE does not know which beam UE is under coverage (a given SSI ID) after long deep sleep due to UE mobility. All TRS/CSI-RS resources within a cell should be configured and available/ unavailable for all beams together. The subset of TRS/CSI-RS resources for the selected beams cannot help UE achieving power saving. Thus, the availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell.

TRS/SSS based PEI can be used for the PEI detection as well as the RS for front-end processing, such as AGC, T/F channel tracking operation. Sequence based PEI can also be used for availability indication of TRS/CSI-RS based on the code point from the TRS-based PEI. The mapping of CSI-RS resources and code point values for the configured CSI-RS resources can be included in SIB-X. Code points mapping of availability/not availability for the set of TRS resources can be provided by orthogonal cover code or different sequences with details in [7]. E.g., orthogonal cover code can be used for indication of availability of full set of TRS resources. Different sequences corresponding to different code points can be utilized for indication of availability of configured CSI-RS resource.

Observation 9: Sequence based PEI can also be used for indication of availability for a given set of TRS resources.

Proposal 8: The availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell.

L1 based signaling configuration

If L1 signaling is  supported for availability indication of TRS/CSI-RS resources, the extra overhead of L1 signaling would be present all the time even network would have the TRS/CSI-RS resources available all the time for the UE.  This would increase the resource overhead and UE complexity in decoding the L1 signaling all the time. Thus, it would be more efficient to have mechanism controlled by the network to enable/disable the L1 signaling of available indication of the configured TRS/CSI-RS resources for IDLE/Inactive UEs. The most straight forward way of enable/disable the L1 signaling of TRS/CSI-RS availability indication is to include 1 bit enable/disable parameter together with the TRS/CSI-RS resource configuration in SIB-X. If the parameters enable/disable L1 signaling for TRS/CSI-RS resource availability indication is shown “disable” in SIB-X, UE would assume the TRS/CSI-RS resource is available all the time. gNB does not need to include the L1 signaling for the control TRS/CSI-RS resource availability.  UE does not to decode the L1 signaling for the TRS/CSI-RS resource availability indication when it is available all the time.

Proposal 9: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X.

Conclusion

In this contribution, configuration of TRS/CSI-RS for paging enhancement is discussed.  We have the following observations and proposals,

Observation 1: TRS/CSI-RS configuration with potential large size of signaling may need to be configured at another standalone SIB-X with the present of SIB-X indicated by SIB1.

Proposal 1: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit.

Observation 2: With CSI-RS resources configured with SI without association with paging occasion(s), UE might not use such TRS configuration for channel tracking which will degrade UE power saving gain significantly.

Observation 3:  The TRS/CSI-RS resources configured for CONNECTED mode UEs can be shared to IDLE mode UE.

Observation 4: With TRS/CSI-RS occasion associated with SSB/paging occasion, it will provide significant power saving gain at cost of low configuration signaling overhead.

Observation 5: gNB could configure the CONNECTED mode UE with the TRS/CSI-RS resource bundled with SSB/paging occasion which is configured for IDLE mode UE.

Observation 6: Considering that the paging indication (sequence or DCI based) would be transmitted on every paging cycle, TRS/CSI-RS bundled with SSB/paging occasion should not be considered as always on signal.

Proposal 2: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) to achieve good power saving gain with low SIB signaling overhead.

Observation 7: Compared with configuration of TCI state per CSI-RS resource, one TRS resource set can be configured with one TCI state which can reduce unnecessary signaling overhead.

Proposal 3: QCL information configuration of TRS for idle/inactive UE should be configured per CSI-RS resource set.

Proposal 4: QCL information of TRS/CSI-RS occasion(s) should be explicitly indicated to be associated with SSB ID by SIB-X for IDLE/Inactive UEs.

Proposal 5: The following procedure can be used for TRS/CSI-RS occasion(s) configuration:

Step1) predefined parameters of TRS/CSI-RS resource grid;

Step 2) SIB indicate parameters details; including

- QCL assumption of the configured TRS/CSI-RS resources associated with a SSB;

- Code points mapping of availability/not availability for a given TRS resources.

Step 3）To derive TRS occasion(s) according to predefined rule and parameters provided by step1 and step 2.

Proposal 6: Working assumption that support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs should not be confirmed.

Observation 8: Compared with L1 signaling, SIB based availability indication of TRS/CSI-RS at a given cell for IDLE/Inactive mode UE can provide affirmative TRS/CSI-RS resource availability to achieve UE power saving gain and no additional signaling overhead.

Proposal 7: The availability of TRS/CSI-RS at a given cell should be informed to the UE by the present/not present of SIB-X TRS/CSI-RS configuration.

Observation 9: Sequence based PEI can also be used for indication of availability for a given set of TRS resources.

Proposal 8: The availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell.

Proposal 9: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X.

References

- RAN1 chairman notes, 3GPP TSG RAN WG1 #104b-e.

- RAN1 chairman notes, 3GPP TSG RAN WG1 #103-e.

- TS38.331v16.4.0, Radio Resource Control (RRC) protocol specification.

- RAN1 chairman notes, 3GPP TSG RAN WG1 #102-e.

- TS38.304v16.4.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- TS24.501v16.8.0, Non-Access-Stratum (NAS) protocol for 5G System (5GS).

- R1-2104532, Paging enhancement for UE power saving, CATT.