3GPP TSG RAN WG1 Meeting #105-e  	R1-2104570

e-Meeting, May 10th – 27th, 2021

Agenda Item: 8.15.4

Source: MediaTek Inc.

Title: HARQ in IoT-NTN

Document for: Discussion and Decision

# Introduction

At the RAN#86 meeting, a new Study Item was approved for IoT Non Terrestrial Network (NTN) and revised in RAN#91 [1]. There was an email discussion on [91E][42][NTN_IoT_Roadmap] In RAN#91 with moderator summary and final proposal for GTW input in [2].

In RAN#91-e GTW session, the Chairman endorsed a Way Forward Proposal in [3] on email discussion on [50][New_proposals_approval]. This included guidance from RAN Chairman for NTN NR and NTN IoT as follows

- RAN#92E (June) to finalize the scope and project plan to deliver the essential minimum functionality of both NTN NR and NTN IoT (both NB-IoT and eMTC) within the existing TU allocations

- Detailed scoping exercise (NTN NR WID revision, NTN IoT WID approval) to be undertaken at RAN#92E (June)

This contribution addressed HARQ aspects of NB-IoT NTN.

# Disabling of HARQ Feedback

This section addresses Issue#2 Disabling HARQ feedback in FL summary in [4].  RAN1#104bis-e made the following agreements:

- Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

RAN1#104-e made the following agreements for NB-IoT / eMTC:

- For NTN, further study potential benefits and/or drawbacks of disabling HARQ feedback for NB-IoT.

- Further discuss the potential benefits and/or drawbacks of disabling HARQ feedback for NB-IoT and eMTC, and consider at least the following number of HARQ processes for the analysis

  - NB-IoT:

    - Total: 2, disabled: {1,2}

  - eMTC:

    - Total: 2, disabled: {1,2}

    - Total: 8, disabled: {1,2,7,8}

- Other values for number of HARQ processes below the maximum value can be discussed

- FFS: whether to consider separately LEO and GEO scenarios

- FFS: whether to allow disabling of HARQ feedback in case of single HARQ process

- FFS: whether to allow disabling of all HARQ feedback

- FFS: other details for the evaluation/analysis

It can be discussed whether it is necessary to disable HARQ feedback. The main motivations for disabling HARQ process are:

- HARQ stalling impact on peak data rates.

- HARQ re-transmissions impact on latency

HARQ RTT is in the order of several ms, which will be increased to up to several hundreds of ms if satellite RTD is included. For intermitted delay-tolerant small packet transmissions, this is acceptable considering the following:

- IoT applications are not delay-sensitive: M2M devices may in general support relaxed delay characteristics.  M2M applications (e.g. alarms) may require a delay profile with a delay requirement of 10 seconds for the uplink when measured from the application ‘trigger event’ to the packet being ready for transmission from the base station towards the core network (TR 45.820).

- The data rates for IoT applications are typically low: Packet size, reporting interval combination for battery life analysis - 50 bytes, 2 hours; 200bytes, 2 hours;  50 bytes, 24 hours 200 bytes, 24 hours

Consider required data rate with 10s latency requirements for typical IoT packet sizes can be

- Min: 50 bytes / 10s = 40 bps

- Max: 200 bytes / 10s = 160 bps

## 3.1 HARQ Stalling impact on peak data rates

In Rel-14 NB-IoT, the max TBS=2536 bits is greater than 200 bytes. Table 1 shows the maximum data rates of NB-IoT for Cellular, LEO, and GEO. Though NTN RTT reduces NB-IoT data rates, but these data rates are still sufficient to meet maximum requirement of 160 bps. The lowest data rates with Rel-13 UE with 1 HARQ process, TBS=680 bits (DL) and TBS=1000 bis (UL) can still meet maximum requirement of 160 bps by at least an order of magnitude. The impact of the satellite RTD on the NB-IoT data rates in GEO due to HARQ stalling is in the order of 95% (we took the average in Table 1). Though it seems very large, the data rates after HARQ stalling in GEO are still sufficiently large to support to maximum required data rates of 160 bps for intermitted delay-tolerant small packet transmission.

|  | Rel-13 1 HARQ Process, TBS=680 bits (DL) TBS 1000 bits (UL) |  | Rel-14 1 HARQ Process TBS=2536 bits |  | Rel-14 2-HARQ Processes TBS=2536 bits |  | Rel-17 2-HARQ Processes TBS=2536 bits 16 QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL | DL | UL | DL | UL |
| Cellular | 26.1 kbps | 58.8 kbps | 79.2 kbps | 115.2 kbps | 126.8 kbps | 158.5 kbps | 253.6 kbps | 317.0 kbps |
| LEO | 13.3 kbps (49.0%) | 22.2 kbps (62.2%) | 43.7 kbps (44.8%) | 50.7 kbps (55.9%) | 74.6 kbps (41.2%) | 84.5 kbps (46.7%) | 149.2 kbps (41.2%) | 169.0 kbps (46.7%) |
| GEO | 1.2 kbps (95.4%) | 1.8 kbps (96.9%) | 4.4 kbps (94.4%) | 4.5 kbps (96.0%) | 8.7 kbps (93.1%) | 8.8 kbps (94.4%) | 17.4 kbps (93.1%) | 17.6 kbps (94.4%) |

Table 1: Data rates and reduction in data rates due to HARQ stalling

The impact of the satellite RTD on the NB-IoT data rates in LEO due to HARQ stalling is about a 49% reduction (we took the average in Table 1), which is much smaller than that in GEO. This is because the internal scheduling delay are quite high. On DL, a 4 ms for PDCCH processing of DL assignment and 12 ms for processing delay of UL HARQ feedback are specified. On UL, PDCCH processing of UL grant and UL packet preparations is 8 ms. Further, to schedule maximum TBS of 2536 bits on DL or UL it takes 10 TUs (i.e. 10 ms).

Observation 1: for NB-IoT, HARQ stalling reduces data rates by approximately 95% and 49% for GEO and LEO respectively.

Observation 2: for NB-IoT, HARQ can be used without disabling HARQ feedback with data rates consistent with sporadic short transmissions.

## 3.2 HARQ re-transmissions impact on latency

The latency with 1, 2, and 4 HARQ transmissions are shown in Table 2. The worst case for latency is GEO with 2 HRQ process and 4 HARQ re-transmissions is 2264 ms. This is well within the delay requirement of 10 seconds for the uplink when measured from the application ‘trigger event’ to the packet being ready for transmission from the base station towards the core network (TR 45.820).

Observation 3: for NB-IoT, the maximum latency with 2 HARQ processes with up to 4 HARQ transmissions is 2264 ms.

Observation 4: for NB-IoT, HARQ can be used without disabling HARQ feedback with latency consistent with sporadic short transmissions.

|  | Num HARQ Tx | Rel-13 1 HARQ Process, TBS=680 bits (DL) TBS 1000 bits (UL) |  | Rel-14 1 HARQ Process TBS=2536 bits |  | Rel-14 2-HARQ Processes TBS=2536 bits |  | Rel-17 2-HARQ Processes TBS=2536 bits 16 QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | DL | UL | DL | UL | DL | UL |
| Cellular | 1 | 26 ms | 17 ms | 32 ms | 22 ms | 40 ms | 32 ms | 40 ms | 32 ms |
|  | 2 | 52 ms | 34 ms | 64 ms | 44 ms | 80 ms | 64 ms | 80 ms | 64 ms |
|  | 4 | 104 ms | 208 ms | 108 ms | 88 ms | 160 ms | 128 ms | 160 ms | 128 ms |
| LEO | 1 | 51 ms | 43 ms | 58 ms | 50 ms | 68 ms | 60 ms | 68 ms | 60 ms |
|  | 2 | 102 ms | 86 ms | 116 ms | 100 ms | 136 ms | 120 ms | 136 ms | 120 ms |
|  | 4 | 204 ms | 172 ms | 232 ms | 200 ms | 272 ms | 240 ms | 272 ms | 320 ms |
| GEO | 1 | 567 ms | 549 ms | 574 ms | 556 ms | 584 ms | 566 ms | 584 ms | 566 ms |
|  | 2 | 1134 ms | 1098 ms | 1148 ms | 1112 ms | 1168 ms | 1132 ms | 1168 ms | 1132 ms |
|  | 4 | 2268 ms | 2196 ms | 2296 ms | 2224 ms | 2336 ms | 2264 ms | 2336 ms | 2264 ms |

Table 2: Latency due to HARQ stalling and HARQ re-transmissions

Hence based on observation in the above, we make the following proposals:

Proposal 1: HARQ feedback is not disabled in connected.

## 3.3. Reliability of Message 3 in RACH procedure

Reliability of Message 3 in RACH procedure cannot be based on RLC ARQ as RLC AM is not possible before contention resolution has completed. It is not necessary to disable UL HARQ retransmissions before contention resolution in random access procedure has completed. The message 3 transmission uses a HARQ message 3 buffer during random access procedure, which is separate from HARQ buffer used for UL retransmissions of data in connected mode. UL HARQ retransmissions for message 3 on any HARQ process ID can be used without any HARQ buffer issues. The RRC configuration for disabling of UL HARQ retransmissions per UE per process for data transfer in connected mode can be ignored.

Proposal 2: UL HARQ feedback is not disabled for Message 3 during initial access.

# Conclusion

In this contribution, we discussed scope of HARQ enhancements. We made the following observations and proposals:

Observation 1: for NB-IoT, HARQ stalling reduces data rates by approximately 95% and 49% for GEO and LEO respectively.

Observation 2: for NB-IoT, HARQ can be used without disabling HARQ feedback with data rates consistent with sporadic short transmissions.

Observation 3: for NB-IoT, the maximum latency with 2 HARQ processes with up to 4 HARQ transmissions is 2264 ms.

Observation 4: for NB-IoT, HARQ can be used without disabling HARQ feedback with latency consistent with sporadic short transmissions.

Proposal 1: HARQ feedback is not disabled in connected.

Proposal 2: UL HARQ feedback is not disabled for Message 3 during initial access.

# References

- RP-210868, “New Study WID on NB-IoT/eTMC support for NTN”, MediaTek, RAN#91-e, March 2021

- RP-210915, “Moderator's summary for email discussion [91E][42][NTN_IoT_roadmap]”, Ericsson (RAN1 Vice-Chair)RAN#91-e, March 2021

- RP-210906, Way forward on new proposals, Nokia (RAN Chair), RAN#91-e, March 2021

- R1-2103805, Moderator (Samsung), Summary#3 for enhancements on HARQ, RAN1#104-e, April 2021