3GPP TSG RAN WG1 #105-e		                                                  R1-2104696

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.12.2

Source:	Qualcomm Incorporated

Title:	Views on UE feedback for Multicast RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

In this document, we will present our views on the details of UE feedback and retransmission to improve reliability for Multicast RRC_CONNECTED UEs.

# Discussion

## Multicast HARQ-ACK feedback

We have agreed the following two modes for multicast HARQ-ACK feedback:

- Mode 1: ACK/NACK based HARQ-ACK feedback for multicast

- Mode 2: NACK-only based HARQ-ACK feedback for multicast

The two feedback modes can be complementary to each other in different scenarios. The configuration of feedback mode can be per service or per UE. For example, a UE close to cell-edge can be configured to use Mode 1, based on which gNB can schedule PTP with unicast beam forming for retransmission; otherwise, Mode 2 can be used to save the power by skipping ACK feedback.

Proposal 1: For RRC_CONNECTED UEs, support selection of UE-specific ACK/NACK and NACK-only for different UEs in the same group.

For the PUCCH resource of Mode 2, RAN1 has agreed it is separate from that of unicast.

Agreement:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

However, it is not clear whether PUCCH-config for multicast, if separate from that of unicast, is jointly configured for the two modes or separately configured for each mode. For the PUCCH of Mode 1 and Mode 2, different PC chains should be applied, e.g., separate PUCCH-PowerControl. If PUCCH-config is configured separate for each feedback mode, the PUCCH-PowerControl in PUCCH-Config is applied to corresponding feedback mode. If PUCCH-Config is jointly configured for the two modes, RAN1 needs to discuss how to configure the parameters of PUCCH-PowerControl, which is currently shared by the PUCCH resources in a PUCCH-Config. For PUCCH-power control, at least OL-PC configurations of P0 is supported. RAN1 may need further study on whether/how to support CL-PC for Mode 1 and Mode 2.

Proposal 2: For RRC_CONNECTED UEs, support separate PUCCH power control configuration for ACK/NACK-based and NACK-based multicast HARQ feedback.

- FFS power control parameters

- FFS signaling details

## HARQ-ACK Codebook

In this section, we discuss the details of the HARQ-ACK codebook design in case of the multiplexing of ACK/NACK-based feedback between unicast PDSCH and multicast GC-PDSCH or multiple GC-PDSCHs. The legacy mechanism could be reused as much as possible.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

Here, no matter whether the last DCI is unicast or multicast, the PUCCH resource is a UE-specific field for ACK/NACK-based feedback. gNB has the full control and can schedule sufficient PUCCH resources for multiplexing unicast and multicast. Comparing Alt1 and Alt2, we think Alt2 should be used to avoid codebook ambiguity, which may happen if the UE missed the last DCI for multicast, coming after the last DCI for unicast.

Proposal 3: For multiplexing of ACK/NACK feedback for unicast and multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

For Type-1 HARQ-ACK codebook construction of ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, RAN1 has discussed the following two alternatives to determine PDSCH reception candidate occasions.

- Alt 1 assuming K1 set for unicast termed as set A and K1 set for multicast termed as set B

  - for slot timing values K1 in the intersection of A and B, based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of A and B, based on the union of PDSCH TDRA sets.

We think Alt1 is straightforward without adding redundant bits for multicast or unicast if not overlapping in the same slot.

Proposal 4: For HARQ-ACK codebook construction of ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, prefer Alt1 to determine PDSCH reception candidate occasions.

- Alt 1 assuming K1 set for unicast termed as set A and K1 set for multicast termed as set B

  - for slot timing values K1 in the intersection of A and B, based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

For the order HARQ information bits, we need to consider the legacy Type-1 codebook for unicast (C-RNTI) for backward compatibility, which is in the order of

- PDSCH occasions first,

- and then serving cell index,

- and then CORESETPoolIndex (if configured for mTRP).

Now the question is how to arrange the bits for different RNTIs, where the bit for C-RNTI is for unicast and the bit for G-RNTI(s) is for one or multiple multicast services. For simplicity, we can predefine the bit for C-RNTI before that of G-RNTI(s) and the bit for G-RNTI1 before G-RNTI2. For the general order, a unified design is preferred for multiplexing the HARQ information bits of X-RNTI, i.e., C-RNTI (for unicast) and G-RNTIs (for multicast) multiplexed by FDM, intra-TDM, inter-TDM. There could be several alternatives as illustrated in Figure 1, where the unicast (U) and multicast (M) if in the same PDSCH occasion can be FDMed or intra-slot TDMed. Following the mTRP design principle, we prefer to multiplex the unicast and multicast HARQ bits on each TRP first, i.e., Alt1 or Alt3 rather than Alt2. Considering the FDMed unicast and multicast can be treated in a similar way as CA concept, we slightly prefer Alt3 than Alt1.

For Type-2 HARQ-ACK codebook, the number of HARQ-ACK bit(s) and the bit order for multicast are based on the DAI(s) of the DL DCI(s) for multicast. Each multicast GC-PDSCH with the associated G-RNTI has a separate DAI.

Figure 1 Type-1 codebook generation

Proposal 5: For multiplexing of ACK/NACK feedback for unicast + multicast, or multicast + multicast

- For Type-1 codebook, the HARQ-ACK information bits are concatenated in increasing order of

  - PDSCH reception occasion index at first

  - and then serving cell index

  - and X-RNTI (C-RNTI before G-RNTIs, and G-RNTI1, G-RNTI2… for different multicast)

  - and CORESETPoolIndex (if configured for mTRP)

- For Type-2 codebook, the number of HARQ-ACK bit(s) and the bit order for multicast are according to the DAI of DL DCI per multicast G-RNTI.

Other aspects on the HARQ-ACK feedback codebook need to be discussed include whether to apply the codebook other existing HARQ-ACK codebooks for multicast transmission in a similar way as that of unicast transmission. For example, the HARQ-ACK codebook Type 3 could be considered for aperiodic multicast feedback in at least (not limited to) unlicensed band. Note that the MBS transmission in unlicensed band is within the scope of the WI.

Proposal 6: For RRC_CONNECTED UEs, also support Type 3 HARQ-ACK codebook for multicast ACK/NACK based HARQ-ACK feedback.

## Enabling/disabling of HARQ feedback

Among the options for enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

We think Option 2 is sufficient for the multicast use case. The selection of the HARQ-ACK feedback mode could be dependent on how to enable/disable HARQ-ACK for different users/services:

- Alt1: configure HARQ-ACK for a given a service;

- Alt2: configure HARQ-ACK for a given UE receiving a service.

The motivation is unclear to support dynamic signaling in DCI to enable/disable HARQ-ACK feedback. In addition, the dynamic way would result in ambiguous UE behavior when having PDCCH false detection or PDCCH feedback on/off, which should be carefully considered.

Proposal 7: For RRC_CONNECTED UEs, HARQ-ACK feedback can be enabled/disabled by RRC signaling.

- The configuration of HARQ-ACK feedback can be configured for a given G-RNTI (corresponding to a service) or for a UE receiving a service.

## SPS GC-PDSCH feedback

For SPS GC-PDSCH feedback, we have the following agreements. The other details for SPS GC-PDSCH are discussed in [1].

Agreement:

For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS

- FFS: The retransmission scheme(s)

- FFS: The HARQ-ACK details for SPS PDSCH and activation/deactivation, which can be discussed in AI 8.12.2

For Rel-16 unicast PDSCH, multiple PUCCH resources in SPS-PUCCH-AN-List are commonly configured for all SPS PDSCH configurations per HARQ-ACK codebook. The actual PUCCH resource to be used among PUCCH resources is determined based on HARQ-ACK payload size. For multicast SPS GC-PDSCH, the PUCCH resources for HARQ-ACK feedback can be:

- Opt1: a separate SPS-PUCCH-AN-List for multicast is configured common for all SPS GC-PDSCH configurations, different than that of SPS PDSCH configurations.

- Opt2: a common SPS-PUCCH-AN-List is configured for all SPS PDSCH and SPS GC-PDSCH configurations

- Opt2 can be used by default if Opt1 is not configured.

Proposal 8: For ACK/NACK-based feedback of RRC_CONNECTED UEs receiving SPS GC-PDSCH,

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS GC-PDSCH configurations.

- Otherwise, a common SPS-PUCCH-AN-List applies to all SPS PDSCH and SPS GC-PDSCH configurations.

Although more than one HARQ-ACK feedback mode is supported for multicast GC-PDSCH, UE-specific ACK/NACK feedback is needed for the activation/release for SPS GC-PDSCH. For SPS GC-PDSCH without PDCCH, it can be configured with no feedback, ACK/NACK-based or NACK-based feedback, FFS per UE or per SPS-Config.

Proposal 9:

- Support UE-specific ACK/NACK-based feedback for activation/release for SPS GC-PDSCH

- Support no feedback, ACK/NACK-based or NACK-based feedback for SPS GC-PDSCH without PDCCH

## SPS GC-PDSCH repetition

RAN1 has agreed to support semi-static or dynamic configuration of GC-PDSCH repetition. We would like to clarify that the scheme can be applied to dynamic GC-PDSCH as well as SPS GC-PDSCH. The repetition value configured for dynamic GC-PDSCH could be different than that of SPS GC-PDSCH. Similar as dynamic GC-PDSH, the semi-static repetition and dynamic repetition is not simultaneously applied to the same SPS GC-PDSCH.

Proposal 10: Support semi-static and dynamic slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

- Repetition configuration for SPS GC-PDSCH can be different than that of dynamic GC-PDSCH.

## CSI acquisition

RAN1 has agreed that the existing CSI measurement/reporting framework can be largely reused for multicast communication, but some modifications may be needed.

The multicast beam is targeting a group of UE, different than that of unicast beam. The CSI-RS for multicast beam measurement should be limited within the MBS CFR, so that common CSI-RS is shared among the UE group. The CSI-RS if out of the CFR cannot be group-common since it may be out of some UE’s dedicated BWP. From network point of view, it is too much overhead to send UE-specific CSI-RS for the multicast CSI measurement.

For legacy CSI-RS configuration, the CSI-RS power is defined relative the PDSCH DMRS configured per dedicated BWP for RRC_CONNECTED UEs. In case of CSI-RS for multicast, the CSI-RS power should be common for the group UEs. Therefore, the common CSI-RS power needs to be associated with GC-PDSCH DMRS, instead of unicast PDSCH DMRS.

Proposal 11: For RRC_CONNNECTED UES, configure the CSI-RS resource per MBS CFR.

- CSI-RS bandwidth is limited within the MBS CFR.

- CSI-RS power is associated with GC-PDSCH power.

- UL Grant to trigger A-CSI-RS for multicast transmission

- DL Grant to trigger A-CSI-RS for multicast transmission

Figure 2 UL Grant vs. DL grant to trigger A-CSI-RS for multicast transmission

The existing procedure for CSI measurement/reporting is designed mainly for single UE case. How to efficiently trigger CSI-RS transmission and CSI reporting for a group of multicast UEs need to be further studied.

In NR, UE-specific UL grant is used to trigger aperiodic CSI-RS transmission, where the A-CSI-RS will be transmitted after the PDCCH slot carrying UL grant with an RRC-configured time offset. Following the legacy A-CSI-RS procedure, the gNB has to send multiple UL grants for every UE in a multicast group to trigger multiple A-CSI-RS transmissions. As shown in Figure 2(a), two A-CSI-RSs are to be transmitted for UE1 and UE2 in the MC BWP. More users in the multicast group will result in significant DL CSI-RS overhead.

A more efficient way is to use group-common DL grant (GC-PDCCH to schedule GC-PDSCH) to trigger one A-CSI-RS transmission. As shown in Figure 2(b), the A-CSI-RS transmitted by gNB is used for a group of multicast UEs. Based on the common A-CSI-RS, each UE can report CSI measurement results on per-UE PUSCH scheduled by gNB separately.

Proposal 12: Support GC-PDCCH to trigger A-CSI-RS transmission in MBS CFR.

For multicast communication, the beam for multicast transmission may be different from that of unicast communication to a particular UE. As illustrated in Figure 3, the multicast beam may be transmitted from multiple cells in an SFN area. The configuration of the DL TRS/CSI-RS for TCI states for GC-PDCCH/PDSCH would be separate from unicast one.

Figure 3 Multicast beam vs. unicast beam

For multicast beam operation, the CSI report is needed, and the corresponding beam management is different than unicast one. The beam failure detection for multicast is associated with GC-PDCCH/GC-PDSCH in the MBS BWP. When a UE fails to receive the unicast PDCCH, the UE would trigger the beam failure and recovery over PRACH or PUCCH. For RRC_CONNECTED UEs, the unicast connection is the pre-condition to receive multicast control and data. When a UE detects multicast beam failure, the beam recovery for multicast transmission can be assisted by using unicast connection for Multicast RRC_CONNECTED UEs, i.e., the link radio recovery (LRR) and LRR response are via unicast link by serving cell.

Proposal 13: For RRC_CONNECTED UEs, support beam management for multicast assisted by unicast connection.

Besides DL CSI-RS-based scheme, CSI can be achieved by the measurement based on UL reference signal. When gNB with significant large antenna arrays in TDD band, the UL SRS-assisted CSI for multicast is also a candidate to simplify the UE complexity on the CSI measurement and reporting. The SRS is transmitted in the MBS BWP. The tradeoff between CSI feedback overhead and CSI accuracy for different feedback information should be taken into consideration in case of the multicast communication.

Proposal 14: Consider SRS configuration for CSI measurement of multicast transmission in MBS CFR.

# 3. Conclusion

In this contribution, we discussed various aspects related to UE feedback for NR multicast with the following proposals:

For ACK/NACK and NACK-only feedback,

Proposal 1: For RRC_CONNECTED UEs, support selection of UE-specific ACK/NACK and NACK-only for different UEs in the same group.

Proposal 2: For RRC_CONNECTED UEs, support separate PUCCH power control configuration for ACK/NACK-based and NACK-based multicast HARQ feedback.

- FFS power control parameters

- FFS signaling details

For HARQ-ACK codebook,

Proposal 3: For multiplexing of ACK/NACK feedback for unicast and multicast, determining the PUCCH resources for transmission is based on the PRI indicated in the last DCI across unicast and multicast.

Proposal 4: For HARQ-ACK codebook construction of ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, prefer Alt1 to determine PDSCH reception candidate occasions.

- Alt 1 assuming K1 set for unicast termed as set A and K1 set for multicast termed as set B

  - for slot timing values K1 in the intersection of A and B, based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

Proposal 5: For multiplexing of ACK/NACK feedback for unicast + multicast, or multicast + multicast

- For Type-1 codebook, the HARQ-ACK information bits are concatenated in increasing order of

  - PDSCH reception occasion index at first

  - and then serving cell index

  - and X-RNTI (C-RNTI before G-RNTIs, and G-RNTI1, G-RNTI2… for different multicast)

  - and CORESETPoolIndex (if configured for mTRP)

- For Type-2 codebook, the number of HARQ-ACK bit(s) and the bit order for multicast are according to the DAI of DL DCI per multicast G-RNTI.

Proposal 6: For RRC_CONNECTED UEs, also support Type 3 HARQ-ACK codebook for multicast ACK/NACK based HARQ-ACK feedback.

For enabling/disabling of HARQ feedback,

Proposal 7: For RRC_CONNECTED UEs, HARQ-ACK feedback can be enabled/disabled by RRC signaling.

- The configuration of HARQ-ACK feedback can be configured for a given G-RNTI (corresponding to a service) or for a UE receiving a service.

For SPS GC-PDSCH feedback,

Proposal 8: For ACK/NACK-based feedback of RRC_CONNECTED UEs receiving SPS GC-PDSCH,

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS GC-PDSCH configurations.

- Otherwise, a common SPS-PUCCH-AN-List applies to all SPS PDSCH and SPS GC-PDSCH configurations.

Proposal 9:

- Support UE-specific ACK/NACK-based feedback for activation/release for SPS GC-PDSCH

- Support no feedback, ACK/NACK-based or NACK-based feedback for SPS GC-PDSCH without PDCCH

For SPS GC-PDSCH repetition,

Proposal 10: Support semi-static and dynamic slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

- Repetition configuration for SPS GC-PDSCH can be different than that of dynamic GC-PDSCH.

For CSI acquisition,

Proposal 11: For RRC_CONNNECTED UES, configure the CSI-RS resource per MBS CFR.

- CSI-RS bandwidth is limited within the MBS CFR.

- CSI-RS power is associated with GC-PDSCH power.

Proposal 12: Support GC-PDCCH to trigger A-CSI-RS transmission in MBS CFR.

Proposal 13: For RRC_CONNECTED UEs, support beam management for multicast assisted by unicast connection.

Proposal 14: Consider SRS configuration for CSI measurement of multicast transmission in MBS CFR.

# References

[1] R1-2104695, “Views on group scheduling for Multicast RRC_CONNECTED UEs”, Qualcomm, RAN1#105-e, May 10th – 27th, 2021.

# Appendix

In RAN1#104bis-e meeting, we have the following agreements for UE feedback of Multicast RRC_CONNECTE UEs.

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;