3GPP TSG RAN WG1 #105-e                                                                                 	         R1-2104763

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Discussion on the remaining issues with shared spectrum operation determination

Agenda Item:	7.2.2

Document for:	Discussion and Decision

Introduction

In this contribution, we continue analysing the uncertainty on the determination of shared spectrum operation, which may cause different understanding between gNB and UE.

Discussion

In last meeting, we have raised an issue about how a UE determines the ‘operation in shared spectrum’. For some cases when DL and UL are not in the same band type, the issue becomes more severe, e.g. DL is in unlicensed band and the SUL is in licensed band. Or the DL is in licensed band and SUL in unlicensed band.  In these cases, the UE has to determine whether the operation is considered in shared spectrum or non-shared spectrum. This is because it directly relates with the DCI field interpretation in RACH procedure, e.g. RAR UL grant and PDCCH-RAR detection.

In RACH procedure, there are some UE behavior or signaling field interpretation dedicated to NRU, including:

- For PDCCH-RAR detection, the UE needs to check the SFN field

- For RAR UL grant scheduling: the UL grant field positions, e.g. PUSCH TDRA, MCS, TPC, CSI request, are changed according to whether or not operation with shared spectrum

What is unclear is that the term ‘operation’ may be interpreted as uplink operation or may also be interpreted as cell operation. One example is that for a PCell operating in unlicensed band, it may have a SUL link in licensed band, e.g. PCell downlink and normal uplink are in unlicensed band, SUL link is in licensed band. For this case, how to determine the term ‘operation’ when a UE selects SUL to transmit PRACH.

We can discuss the following two options:

Option 1: ‘operation’ is determined by whether the PDCCH scheduling RAR is in unlicensed band or licensed band.

Option 2: ‘operation’ is determined by whether the PRACH is transmitted in unlicensed band or licensed band.

Proposal: RAN1 shall clarify the understanding of ‘operation’ with/without shared spectrum channel access.