3GPP TSG RAN WG1 #105-e		R1-2104766

e-Meeting, May 10th -27th , 2021

Source:	OPPO

Title:	Discussion on enhancements for PUCCH format 0/1/4

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In RAN1#104bis-e meeting, extensive discussions were conducted on the aspect of PUCCH enhancements and the following agreements were made:

Agreement:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Decision: As per email decision posted on April 17th,

Agreement:

Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured

  - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

Agreement:

For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 only, as in Rel-15/16.

Agreement:

For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

Agreement:

For UCI of enhanced PF4, support pre-DFT blockwise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e).

From GTW session:

Agreement:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz.

R1-2104060	[DRAFT] LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band	Ericsson

Decision: As per email decision posted on April 20th, the draft LS is endorsed. Final version is approved in R1-2104061.

Agreement:

User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

In this contribution, we present some of the simulation results and provide some analysis on the potential enhancement and the benefits. The details of MIL calculation in this contribution can be found in the attached excel file in Appendix section.

- Discussion

  - The maximum values for the configured number of RBs

In this section, we provide an analysis on the maximum number of RBs that can be configured. It has been agreed to increase the number of RBs under PSD limitation in shared spectrum operation for the benefits for transmission power increase. It is a common understanding that the transmission power for PUCCH is directly linked to its coverage. Thus, any contribution results in transmission power increase will be beneficial for PUCCH coverage increase.

In the unlicensed spectrum, the PUCCH output power is limited by different factors. In addition to regulatory power limits, UE hardware limits on conducted power and EIRP have also been considered in the previous determination of the maximum value for the configured number of RBs. To summarize, they are

- Factor 1: Conducted power limit due to EIRP limit (Pmax_EIRP)

- Factor 2: Additional regional conducted power limit (Pmax_P), which is a region-specific factor.

- Factor 3: UE maximum conducted power (UE_P)

- Factor 4: UE conducted power limit due to EIRP limit (UE_EIRP)

Table 1 provides regulatory limits on EIRP and conducted power, i.e., factor 1 and 2, for various regulatory regions:

|  | Pmax_EIRP | Pmax_P |
| --- | --- | --- |
| EU | 40 dBm-TxBF | 23 dBm/MHz + max(0, 10*log10(BW)) - TxBF |
| US | 40 dBm-TxBF | 27 dBm – max(0, 10*log10(100 / BW)) |
| South Koera | 43 dBm – TxBF when an equipment is >=300m from an astronomical antenna 27 dBm – TxBF when an equipment is <300m from an astronomical antenna | 13 dBm/MHz + max(0, 10*log10(BW)) - TxBF |

Table 1: Regulatory Power Limits by Region [1]

In the table 2, RAN1 is currently discussing whether or not larger values for UE hardware limits on conducted power and EIRP, i.e. factor 3 and 4, can be assumed in the 52.6-71 GHz band:

| UE_P | [21] dBm - backoff |
| --- | --- |
| UE_EIRP | [25] dBm-TxBF or 40 dBm-TxBF |
| TxBF | [6] dBi |

Table 2: agreed UE hardware limits [1]

If the maximum PUCCH transmission power is decided by regulatory power limits and UE hardware limits simultaneously, Pmax = min(Pmax_EIRP, Pmax_P, UE_P, UE_EIRP). When we apply UE hardware limits in Table 2 and US regulatory power limit, the observation is as shown in Fig. 1. The decisive factor for the PUCCH transmission power limit in the narrow band region is Pmax_P up to 11 RB assuming 120 kHz SCS. Then, further increasing PUCCH bandwidth cannot benefit power increase as the decisive factor switches to UE_P, which is aligned with the agreement in the last meeting[2].

Fig. 1: Pmax calculation with regulatory power limits and agreed UE hardware limits

When we assume the values for UE hardware limits are larger than these in Table 2, e.g., UE_EIRP = 40 dBm, UE_P = 25dBm, TxBF = 0 dBi, the observation is different as shown in Fig. 2. In this case, the maximum value for the configured number of RBs is increased to 64 RBs.

Fig. 2: Pmax calculation with regulatory power limits and larger values for UE hardware limits

As shown form the above results, the maximum value for the configured number of RBs is highly related to the values for the UE hardware limits. If RAN4 specifies UE power class(es) with specific values for maximum UE conducted power and maximum UE EIRP, it is necessary to determine the maximum value for UE conducted power and EIRP before determining the maximum values for the configured number of RBs.

Observation 1: the maximum value for the configured number of RBs is highly related to the values for the UE hardware limits.

Proposal 1: determine the maximum value for the UE conducted power and EIRP before determining the maximum values for the configured number of RBs.

In addition, RAN4 only specify the minimum UE conducted power and EIRP currently to meet coverage requirement. If RAN4 does not specify maximum values for UE conducted power and EIRP, regulatory power limits should be considered when determining the maximum values for the configured number of RBs.

Proposal 2: If RAN4 does not specify maximum values for UE conducted power and EIRP, regulatory power limits should be considered when determining the maximum values for the configured number of RBs.

Moreover, when we apply EU regulatory power limit, the PUCCH transmission power can take full benefit from increasing the PUCCH bandwidth up to 32 RB, as shown in Fig. 3.

Fig. 3: Pmax calculation with EU regulatory power limits

  - PUCCH format 0 sequence

In this section, we investigate the performance comparison between different alternatives on the sequence generation for case N_RB>1. There are two alternatives discussed in the last meeting. One is to use a long sequence based on R15 ZC sequence generation. The other is the RB cycling used for R16 interlacing PUCCH format 0 sequence generation. In our analysis, we have simulated the power backoff (CM) of these two options.

The simulation results are given in table 3. The CM of long sequence is generally lower than that of RB cycling, implying that the long sequence can keep better ZC property, leading to a lower PAPR. In addition, The CM of long sequence is more stable than that of RB cycling, so the conducted power corresponding to the UE_P limit will be less influenced by the variation of the number of RBs, which is appreciate to support flexible configuration of various integer values in the range [1 .. max(NRB)] for each SCS.

| CM 95% | N_RB=1 | N_RB=2 | N_RB=4 | N_RB=8 | N_RB=12 | N_RB=32 |
| --- | --- | --- | --- | --- | --- | --- |
| Long seq. | 0.78 | 0.78 | 2.16 | 1.59 | 2.04 | 2.38 |
| RB cycling | 0.78 | 2.67 | 3.30 | 2.36 | 1.47 | 3.60 |

Table 3: CM comparison between long sequence and RB cycling

Observation 2: The CM of long sequence is lower and more stable than that of RB cycling, which is appreciate to support flexible configuration of various integer values in the range [1 .. max(NRB)] for each SCS.

Moreover, the required SNR for these two options is also reported in table 4 and the corresponding MIL comparison is given in Fig. 4. It can be clearly seen that in general the CM of long sequence is smaller than that of RB cycling. Moreover, the MIL with long sequence is also better than that of RB cycling in most cases. The detailed simulation parameters and more simulation results can be found in our companion contribution[3]. With the above analysis, we can draw the following observation and proposal.

| Target SNR for 120 kHz |  | DS=5 ns | DS=10 ns | DS = 20 ns |
| --- | --- | --- | --- | --- |
| N_RB=12 | Long seq. | -10.33 | -10.40 | -9.62 |
|  | RB cycling | -10.86 | -10.42 | -9.49 |
| N_RB=32 | Long seq. | -14.38 | -13.91 | -12.20 |
|  | RB cycling | -14.33 | -13.87 | -11.63 |
| Target SNR for 480 kHz |  |  |  |  |
| N_RB=3 | Long seq. | -5.59 | -4.65 | -3.53 |
|  | RB cycling | -5.46 | -4.65 | -3.81 |
| N_RB=8 | Long seq. | -8.10 | -6.84 | -4.26 |
|  | RB cycling | -8.03 | -6.91 | -4.40 |
| Target SNR for 960 kHz |  |  |  |  |
| N_RB=2 | Long seq. | -1.99 | -2.46 | -1.24 |
|  | RB cycling | -1.97 | -2.02 | -0.77 |
| N_RB=4 | Long seq. | -4.46 | -4.76 | -1.91 |
|  | RB cycling | -4.49 | -4.53 | -2.02 |

Table 4: required SNR comparison between long sequence and RB cycling

Fig. 4: MIL comparison between long sequence and RB cycling

Observation 3: using long sequence can lead to larger MIL than using RB cycling.

Proposal 3: Adopt long sequence for PUCCH format 0 and format 1 when N_RB>1.

  - Sub-PRB PUCCH resource allocation for 120 kHz

In the last meeting, for N_RB>1 PUCCH with 120 kHz SCS, the following alternatives were discussed.

- Alt-1: All REs within each RB are mapped

  - Note: PRB and sub-PRB interlaced mapping is not considered further

- Alt-2: Subset of REs within each RB are mapped (sub-PRB interlaced mapping)

Compared with full-PRB mapping, the sub-PRB mapping can have the following two advantages.

- Advantage 1: Sub-PRB mapping can boost the PSD on the active RE, leading to an increased SNR per RE.

- Advantage 2: by transmitting the same amount of HARQ-ACK information, the sub-PRB mapping uses less resources, leading to an increased spectral efficiency.

In our simulations, we assume that only 1 RE in a PRB is used for PUCCH mapping, i.e. for PUCCH format 0 with N_RB=12, only 12 RE are used for PUCCH allocation, which is based on a ZC sequence of length 12. The CM, required SNR and the MIL comparison between these two alternatives are depicted in table 5 and Fig. 5, respectively. It is to note that the reported SNR in table 5 with sub-PRB does not take into account the RE level power boosting, which is however factored in with our MIL simulation. For MIL simulation, we assume US regulatory power limit is applied as well as the UE specific power limit. As shown from our simulation results, the sub-PRB mapping leads to comparable MIL to full-PRB mapping, thanks to the RE power boosting.

| PF 0 | SCS 120 (kHz) | N_RB (RB) | mapping | sequence | CM | Required SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DS=5 ns | DS = 10 ns | DS = 20 ns |
|  |  | 12 | Full-PRB | 144-ZC | 2.048 | -10.33 | -10.40 | -9.62 |
|  |  | 12 | Sub-PRB | 12-ZC | 0.7756 | 0.44 | 0.36 | 1.21 |

Table 5: CM comparison between full-PRB and sub-PRB mapping

Fig. 5: MIL comparison between full-PRB and sub-PRB mapping

Observation 4: For 120 kHz SCS, compared with full-PRB mapping, sub-PRB mapping leads to better MIL and less resource consumption.

In addtion, user multiplexing has also been considered in this section. When CDM multiplexing is adopted between different users, the CM and required SNR for the UE with and without interference between these two alternatives are depicted in table 6. As shown in our simulation results, sub-PRB mapping provides comparable CDM multiplexing capacity with full-PRB mapping. Moreover, sub-PRB mapping allows for orthogonal FDM multiplexing of users additionally.

| PF 0 | SCS 120 (kHz) | Scenario | N_RB (RB) | mapping | sequence | CM | Required SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | DS=5 ns | DS = 10 ns | DS = 20 ns |
|  |  | Without interference | 12 | Full-PRB | 144-ZC | 2.048 | -10.33 | -10.40 | -9.62 |
|  |  |  |  | Sub-PRB | 12-ZC | 0.7756 | 0.44 | 0.36 | 1.21 |
|  |  | With interference |  | Full-PRB | 144-ZC | 2.048 | -10.15 | -10.16 | -9.49 |
|  |  |  |  | Sub-PRB | 12-ZC | 0.7756 | 0.49 | 0.29 | 0.91 |

Table 6: multiplexing capacity comparison between full-PRB and sub-PRB mapping

Observation 5: For 120 kHz SCS, sub-PRB mapping provides comparable CDM multiplexing capacity with full-PRB mapping, while allowing for orthogonal FDM multiplexing of users additionally.

Proposal 4: For 120 kHz SCS, adopt sub-PRB mapping instead of full-PRB mapping, where only 1 RE is mapped in a RB.

- Conclusion

This contribution discusses the potential enhancements to PDCCH monitoring. The following observations and proposals are made.

Observation 1: the maximum value for the configured number of RBs is highly related to the UE hardware limits.

Observation 2: The CM of long sequence is lower and more stable than that of RB cycling, which is appreciate to support flexible configuration of various integer values in the range [1 .. max(NRB)] for each SCS.

Observation 3: using long sequence can lead to larger MIL than using RB cycling.

Observation 4: For 120 kHz SCS, compared with full-PRB mapping, sub-PRB mapping leads to better MIL and less resource consumption.

Observation 5: For 120 kHz SCS, sub-PRB mapping provides comparable CDM multiplexing capacity with full-PRB mapping, while allowing for orthogonal FDM multiplexing of users additionally.

Proposal 1: determine the maximum value for UE conducted power and EIRP before determining the maximum values for the configured number of RBs.

Proposal 2: If RAN4 does not specify maximum values for UE conducted power and EIRP, regulatory power limits should be considered when determining the maximum values for the configured number of RBs.

Proposal 3: Adopt long sequence for PUCCH format 0 and format 1 when N_RB>1.

Proposal 4: For 120 kHz SCS, adopt sub-PRB mapping instead of full-PRB mapping, where only 1 RE is mapped in a RB.

- Reference

| [1] | R1-2102127, FL Summary 4 for Enhancements for PUCCH formats 0/1/4, Moderator (Ericsson), RAN1#104-e |
| --- | --- |
| [2] | RAN1#104b-e meeting chairman notes |
| [3] | R1-2104769, “Discussion on other aspects”, OPPO, RAN1#105-e meeting |

Appendix: Details of MIL