3GPP TSG RAN WG1 #105-e		                                    		  R1-2104871

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.5.1

Source:	InterDigital, Inc.

Title:	Discussion on accuracy improvements by mitigating timing delays

Document for:	Discussion and Decision

# Introduction

In RAN#104e and RAN1#104-bis-e the following agreements were made to study specification impact of reference devices [1,2]

Agreement: - Study specification impact for enabling a reference device with known location to support the following functionalities: - Measure DL PRS and report associated measurements (e.g., RSTD, Rx-Tx time difference, RSRP) to the LMF; - Transmit SRS and enable TRPs to measure and report measurements (e.g., RTOA, Rx-Tx time difference, AOA) associated with the reference device to the LMF; - FFS: The details of the signalling, the measurements, the parameters related to the Rx and Tx timing delays, AoD and AOA enhancements and measurement calibrations; - FFS: The report of device location coordinate information to the LMF if the LMF does not have the information - FFS: The device with the known location being a UE and/or a gNB - FFS: Precision to which location of reference device is known - Note: RAN1 assumes using these enhancements for the purpose of network synchronization is NOT within the scope of the WI Agreement: - Support the following for mitigating TRP Tx timing errors and/or UE Rx timing errors for DL TDOA - Support a UE to provide the association information of RSTD measurements with UE Rx TEG(s) to the LMF when the UE reports the RSTD measurements to the LMF if the UE has multiple TEGs - Support a TRP providing the association information of DL PRS resources with Tx TEGs to the LMF if the TRP has multiple TEGs - Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs - FFS: the details of the signalling, procedures, and UE capability - Send an LS to RAN4 to check if there is any issue to support the above enhancements Agreement: Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA - Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs - Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the Signaling, procedures, and UE capability Agreement: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS; - Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF according to the one of the 2 following options: - Option 1: the UE RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether UE provides the association information of DL PRS resources to UE Rx TEG to LMF for UE RxTx measurements specifically - Option 2: the UE RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS. - For both alterntives, the UE may provide the association information of SRS resources for positioning to UE Tx TEG to LMF - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the signalling, procedures, and UE capability Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signalling, procedures

In this contribution, we discuss possible specification impacts of the reference device and TEG.

# Specification impact of the reference device

A reference device may be needed if the network detects uncertainties in the measurements and perform calibration. In this section, possible specification impacts are discussed.

## Distribution of the location of the reference device

The location of the reference device is known by the LMF. In practice, the reference device may be a stationary device in the network or access point or gNB. The need for the reference device may depend on the positioning method implemented by the UE. For example, in the presence of unknown timing offsets or angle offsets, differential positioning methods may be needed.

Differential positioning methods can be enabled by the network or UE for UE-assisted or UE-based positioning method. For example, the presence and location of the reference devices can be broadcasted by the network and UE can obtain a list of the reference devices. A reference station ID, associated with the location of the reference station, is needed. If the UE identifies reference devices in the proximity, based on the positioning estimate obtained by a UE-based positioning method, the UE can initiate a differential positioning method. The list of reference devices used for positioning can be maintained by the LMF.

Observation 1: For UE-based positioning method, the UE needs to obtain a list of reference devices and their positions.

To enable a differential positioning method for UE-based positioning, measurements made by reference devices will be needed. The network may need to send measurements report to the UE such that the UE can perform a differential positioning method.

For UE-assisted positioning, the network can also collect the measurements reported by the UE and compare them against measurements reported by the reference device. Using collected measurements, the network can implement differential positioning to eliminate timing offsets that may exist in the measurements.

Observation 2: Broadcast of reference device(s) may be needed for the UEs performing UE-B positioning.

Observation 3: For UE-based differential positioning, the UE requires measurements made by reference device(s).

Proposal 1: Specification impact of reference devices includes at least assistance information which contains at least reference device ID, locations of reference devices.

Proposal 2: Study positioning procedures to support differential positioning techniques.

Regarding the FFS point, “FFS: The device with the known location being a UE and/or a gNB”, since a  reference device is expected to make measurements and report them to the LMF, the reference device will be considered as a UE. If the reference device is treated as gNB, contents of information exchanged between LMF and gNB and procedure may need modifications, causing large specification impact. Thus, we make the following proposal.

Proposal 3: A reference device is classified as a UE.

Regarding the FFS point, “FFS: The report of device location coordinate information to the LMF if the LMF does not have the information”, it may be straightforward to assume that reference device locations are known by the LMF. Enlistment of reference device(s) may require additional requirements such as integrity related to the reported position of the reference device or capability of the device to serve as a reference. Once the framework for the reference-based positioning becomes clear, mechanisms to enable enlistment of reference devices can be considered.

Proposal 4: Do not support features to allow enlistment of reference device(s) during the initial phase of reference-based positioning standardization study/work.

# Procedure related to TEG

TEG grouping:

As agreed in the last meetings, TEG is associated with a group of transmission and/or reception having timing errors within a certain margin. One outstanding issue is how to determine the margin of each TEG. In our view, the TEG margin (i.e., the maximum difference between any timing errors within a TEG) can be configured by the LMF. Specifically, LMF can configure the margin of the TEG based on the positioning method, the accuracy requirement, and the PRS configuration and measurement reporting.

Proposal 5: Support the LMF to configure a maximum difference between any two timing errors within a TEG.

Association of resource and TEG:

Regarding the association between transmission resource and TEG, one remaining FFS is whether to support the UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to LMF if the UE has multiple Tx TEGs. In our view, such information is necessary to precisely calculate the positioning of the UE. Specifically, if the UE uses two different UL SRS resources associated with two TEGs, the TEG associated with each UL SRS resource should be informed to the LMF.

Proposal 6: Support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs.

For OTDOA positioning method, if the UE or gNB uses the same TEG for Tx and Rx in the resources measured for RSTD, the timing error associated with each measurement resource in the RSTD measurement can be cancelled out. As a result, the positioning calculation can be improved. Since the network is responsible for the positioning accuracy of the UE, it should be able to indicate which TEG to use for each transmission/reception resource to support positioning measurement and reduce the effect of timing error. Therefore, for DL TDOA, the network should be able to indicate which Rx TEG to use for reception DL-PRS, and for UL TDOA the network should be able to indicate which Tx TEG to use for transmission of UL-PRS.

Proposal 7: For DL TDOA, support the LMF to indicate which Rx TEG(s) to use for the reception of one or more DL-PRS resources.

Proposal 8: For UL TDOA, support the LMF to indicate which Tx TEG(s) to use for transmission in one or more UL PRS resources.

TEG for DL+UL (RTT):

For DL+UL positioning method, it was agreed in the last meeting to downselect between Alt. 1 and Alt. 2 on how to provide the UE and gNB Rx-Tx timing difference measurement. In our view, Alt. 2, in which the UE or the network provides the RxTx TEG to LMF is more preferable. It is because RxTx TEG is the actual indication of the timing error of the Rx-Tx timing difference measurement. Providing separate Rx TEG and Tx TEG associated with DL-PRS reception and UL-PRS transmission cannot demonstrate the real timing error of the Tx-Rx timing difference measurement. Therefore, Alt. 1 may result in less accurate positioning performance. In Alt. 2, both Options can provide the accurate RxTx TEG; however, Option 2 can provide more information about Rx TEG and Tx TEG. Therefore, Option 2 is more preferable.

Proposal 9: For TEG for DL+UL positioning, support Option 2 of Alt. 2 for both UE and gNB cases.

TEG information reporting:

For UE-B positioning methods, the TEG information is necessary. Based on the positioning accuracy requirement, different TEG granularity may be required. In such a scenario, the UE should request TEG information to support itself in positioning measurement. Therefore, the system should support the UE to request the TEG information.

Proposal 10: For UE-B positioning methods, support the UE to request the information of gNB TEG.

TEG reporting:

When the UE has multiple TEGs, the UE does not need to provide TEG information for every measurement reporting. It is because if the UE uses the same TEG compared to the previous reporting, the UE may not need to report the TEG information. In this case, the LMF may assume that TEG is not changed and use TEG from the previous reporting. Therefore, the UE only needs to report TEG information if TEG information is changed compared to the previous reporting.

Proposal 11: Support a UE to indicate TEG in the measurement reporting when TEG information is changed compared to the previous reporting.

# Conclusion.

The following observations and proposals are made in this contribution.

Observation 1: For UE-based positioning method, the UE needs to obtain a list of reference devices and their positions.

Observation 2: Broadcast of reference device(s) may be needed for the UEs performing UE-B positioning.

Observation 3: For UE-based differential positioning, the UE requires measurements made by reference device(s).

Proposal 1: Specification impact of reference devices includes at least assistance information which contains at least reference device ID, locations of reference devices.

Proposal 2: Study positioning procedures to support differential positioning techniques.

Proposal 3: A reference device is classified as a UE.

Proposal 4: Do not support features to allow enlistment of reference device(s) during the initial phase of reference-based positioning standardization study/work.

Proposal 5: Support the LMF to configure a maximum difference between any two timing errors within a TEG.

Proposal 6: Support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs.

Proposal 7: For DL TDOA, support the LMF to indicate which Rx TEG(s) to use for the reception of one or more DL-PRS resources.

Proposal 8: For UL TDOA, support the LMF to indicate which Tx TEG(s) to use for transmission in one or more UL PRS resources.

Proposal 9: For TEG for DL+UL positioning, support Option 2 of Alt. 2 for both UE and gNB cases.

Proposal 10: For UE-B positioning methods, support the UE to request the information of gNB TEG.

Proposal 11: Support a UE to indicate TEG in the measurement reporting when TEG information is changed compared to the previous reporting.

Reference

[1] RAN1 Chairman’s notes, RAN1#104e, Feb. 2021.

[2] RAN1 Chairman’s notes, RAN1#104-bis-e, Apr. 2021.