3GPP TSG RAN WG1 #105-e		                                    		   R1-2104872

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.5.2

Source:	InterDigital, Inc.

Title:	Discussion on enhancements for UL-AoA positioning solutions

Document for:	Discussion and Decision

# Introduction

In RAN1 #104bis-e, the agreements related to UL-AoD enhancements were made [1]. In this contribution, enhancements related to UL-AoA are described and proposals are made.

# Improving accuracies of UL AoA

Agreement: - Uncertainty range for expected UL AoA/ZoA is defined as follows - Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) - φAOA - expected azimuth angle of arrival, ΔφAOA – uncertainty range for expected azimuth angle of arrival - Expected zenith angle of arrival as (θAOA - ΔθAOA/2, θAOA + ΔθAOA/2) - θAOA - expected zenith angle of arrival, ΔθAOA – uncertainty range for expected zenith angle of arrival - Select one of the following coordinate system alternatives for signaling UL AoA/ZoA assistance information - Alt.1: Only GCS is supported for AoA/ZoA assistance information indication - Alt.2: Both GCS and LCS are supported for AoA/ZoA assistance information indication - FFS: Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range)

In RAN1#104bis-e, it was agreed to signal expected UL AoA/ZoA and uncertain range from the LMF to gNB/TRP. Such assistance information can aid gNB/TRP to align its reception beam to receive SRS or SRS for positioning transmitted from the UE.

In TS 38.455, UL-AoA are expressed in GCS and LCS-to-GCS translation is provided optionally which is provided by the gNB to the LMF [2].  Thus, expected and uncertainty related to UL AoA/ZoA can also be transferred from the LMF to the gNB in terms of GCS as gNB/TRP is capable of converting GCS to LCS. In addition, by transferring information only in GCS, additional overhead can be reduced in signalling. Thus the following proposal is made.

Proposal 1: With regards to coordinate system for uncertainty range for expected UL AoA/ZoA , support Alt. 1 “Only GCS is supported for AoA/ZoA assistance information indication”

In addition, the following proposals are made in [3] as the result of discussion.

- Signaling of AoA/ZoA assistance information (expected value and uncertainty range) is also supported for UL-TDOA and Multi-RTT positioning methods - Whether RAN1 needs to provide some guidance to other WGs in terms of functionality for AoA/ZoA assistance information or all other signalling/procedure details related to AoA/ZoA assistance information can be discussed directly in RAN2/RAN3?

It was discussed whether the uncertainty range for expected UL AoA/ZoA is supported for both UL-TDOA and multi-RTT. For UL-TDOA, it can be beneficial for UL timing based approach to use expected and uncertain range in AoA/ZoA estimation at the gNB/TRP to improve RSRP at the receiver. Similarly, for multi-RTT, expected and uncertainty of AoA/ZoA at the gNB/TRP will be helpful to obtain more accurate AoA/ZoA at the gNB/TRP.

Proposal 2: Support AoA/ZoA assistance information (expected value and uncertainty range) transferred from the LMF to gNB/TRP for UL-TDOA and multi-RTT

Further signalling details related to UL AoA/ZoA can be discussed in RAN2 and RAN3. RAN1 may assist the discussion if a request for input is received from other working groups.

# Conclusion

The following proposals are made in this contribution:

Proposal 1: With regards to coordinate system for uncertainty range for expected UL AoA/ZoA , support Alt. 1 “Only GCS is supported for AoA/ZoA assistance information indication”

Proposal 2: Support AoA/ZoA assistance information (expected value and uncertainty range) transferred from the LMF to gNB/TRP for UL-TDOA and multi-RTT

Reference

[1] 3GPP RAN1 Chairman’s notes, RAN1#104-bis-e, Apr ,2021.

[2] 3GPP, “NR Positioning Protocol A (NRPPa*”, TS 38.455, ver. 16.2.0, Jan. 2021.

[3] R1-2103863, “Feature Lead Summary #2 for Enhancements of UL-AOA Positioning,” Intel, RAN1#104-bis-e, Apr. 2021.