3GPP TSG RAN WG1 Meeting #105-e	R1-2104894

e-Meeting, May 19 – 27, 2021

Source:	Intel Corporation

Title:	Discussion on initial access aspects for extending NR up to 71 GHz

Agenda item:	8.2.1

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss aspects related to initial access for extending NR up to 71 GHz. The WI for extending NR up to 71 GHz was approved in [1].

The main issues discussed in the following section for initial access are numerology supported for synchronization signal block (SSB), SSB transmission patterns (within a slot and within SSB periodicity), PRACH related issues, and discovery reference signal (DRS) related operations.

# Discussion on Initial Access Aspects

## Numerology for SSB and Support for Single Numerology Operation

In RAN1 #104-bis-e, it was agreed to support 480kHz and 960kHz based SSB for non-initial access case without Type0-PDCCH configuration in the MIB. The following is the agreement from RAN1 #104-bis-e.Agreement: For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB - Note: Strive to minimize specification impact due to the new SCS for SSB

There were additional debates on whether to support CORESET#0/Type0-PDCCH configuration in MIB for 480kHz and 960kHz and whether or not support 480kHz and 960kHz for initial access [7].

There are several issues to consider in supporting of larger SCS for SSB.

- Enablement of single numerology operation (for standalone and/or non-standalone)

- Support of ANR and CGI reporting for neighbor cells

- Timing estimation for channels and signals with 960kHz SCS

- Specification standardization effort

- Added complexity during cell search (if multiple SCS is searched)

First aspect is single numerology operation. With the support of 120 kHz, 480 kHz, and 960 kHz subcarrier spacing (SCS) for data and control channels for NR operating in the 60 GHz band, it’s important to review how different SCS will be utilized from use case perspective. As this gives insights on how the rest of the signals should be used together. Although there could be several use cases that could be applicable for different SCS, one of the main, and from our perspective the most important, use case for the larger SCS, e.g. 960 kHz, deployments that are expected to require very high aggregated throughput but not necessarily require large coverage. Some examples could be industrial deployment scenarios such as data center inter-rack connectivity or enterprise indoor hot spot type of deployments.

Because achieving high data rates above 10Gbps is no simple feat, the ability to optimize transceiver processing is important. From this perspective, it is critical for NR operating in 60 GHz band to have the ability to work with a single numerology. Transceivers that only require to work with a single numerology for all signals and channels can be easily optimized and signal processing can be pipelined in an efficient manner. While support of mixed numerology in FR1 was fairly critical to address URLLC services, the slots durations for 480 kHz and 960 kHz are small enough that additional multiplexing of other SCS together doesn’t necessarily result in improvements or enable specific application services. Multiplexing of 480 or 960 kHz SCS with a different numerology only adds operational and implementation complexity.

Observation 1:

- Single numerology operation can enable efficient transceiver implementation and operation.

Support of single numerology operation for one of the wider bandwidths also mean that RAN1 should support initial access with one or more of 480kHz and 960kHz based SSB. One of the main concerns for expanding the support to initial access was UE complexity. However, we do not think there is any significant difference for the UEs implementing for non-initial access and initial access.

UEs supporting 480kHz and/or 960kHz based SSB for non-initial access cases, i.e. strictly for PScell or Scell, still requires the implement a cell searcher that is able to handle 480kHz and/or 960kHz based SSB (based on capability) for any frequency carrier (within the supported band(s)). Because SSB positions between Pcell and Scell is not limited by specification to be in the same half radio frame, let alone have the same periodicity, UE capable of supporting SSB for non-initial access need to support the capability to blindly search for SSB. The hardware requirements for supporting non-initial access is actually much greater than the initial access cases, since the SSB periodicity can be larger than 20msec. For very large SSB periodicities, UE may need to support even larger PSS/SSS correlator buffer memory compared to initial access case.

Figure 1. Illustration of SSB transmission for Pcell and Scell.

The SSB for Scell basically performs the same functionality as Pcell, and therefore from UE implementation perspective, we do not believe there is any difference from non-initial access and initial access. The potential difference for the UE would be the need to perform multiple SSB detection in various SSB raster frequency entries. However, as analyzed in [9], the total number of SSB raster entries that UE may need to potentially search for is expected to be even smaller than what UE needs to perform for FR2. Additionally, because there is no requirement on the latency for initial cell search process, UE can take additional time to search for different SSB raster entries. This should be able to be leveraged using completely identical hardware that supports non-initial access SSB.

Observation 2:

- UE implementation complexity for supporting non-initial access is greater than initial access case, and support of initial access case is a subset of non-initial access case from hardware implementation perspective.

- The only difference between non-initial access and initial access case is number of frequency entries that UE needs to perform search. However, there are method to mitigate this complexity and lower SSB raster entries.

Second aspect is about ANR and CGI reporting support. ANR and CGI reporting are important features for operators to deploy systems in unlicensed spectrum with minimal coordination effort among different operators and vendors. This feature relies on the fact that UE is able to read neighbor cell SIB1 which contains PLMN and other network information. We believe support of ANR and CGI reporting for 60 GHz unlicensed will be equality as important and therefore strongly believe for any SSB SCS, the ability for UE to obtain Type0-PDCCH configuration of neighbor cell without assistance information (other than carrier frequency of the SSB). If this feature are to be supported regardless of whether SSB may be used for initial access or non-intial access, RAN1 specification will need to support Type0-PDCCH configuration indication (and associated SCS of the CORESET#0/initial active DL BWP) for any SSB SCS to be supported.

Third aspect is about how accurate timing for 960kHz channel and signal could be done based on SSB with lower SCS, for example 120kHz. In our previous contribution [8], residual timing error from 120kHz SSB could easily exceed 30ns which is about half the CP length for 960kHz. Since the timing is derived from PSS and SSS, which only span 127 subcarriers, the timing resolution of such band limited signals would be at most 65ns ( = 1/(127 x 120 kHz)), which is the order of the CP length for 960 kHz. In case UE is trying to perform RRM measurements of 960kHz signals (such as CSI-RS) based off timing derived from 120kHz SSB, this would result in significant degradation in measurement accuracy. In case UE is trying to receive PDCCH/PDSCH with 960kHz based on timing derived from 120kHz SSB, it can lead to reception failure for PDCCH and PDSCH. Therefore, only supporting 120kHz SSB would not be feasible option to enable support of 960kHz for control and data transmission, let alone connectivity for the system.

Fourth aspect is about specification standardization effort. Currently, the numerology of SSB and Type0-PDCCH that has been agreed so far is 120 kHz SCS. The support for 240, 480, and/or 960 kHz SCS is left for further discussion. When 120 kHz SCS data and control channel is used, use of 120 kHz SCS for SSB and initial BWP allows single numerology operation. For 480 and 960 kHz SCS data and control channel, we propose to support 480 and 960 kHz SCS for SSB and initial BWP, to benefit from single numerology operation. This does require RAN1 to work and specify the SSB patterns for 480 and 960 kHz SCS. However, we believe same framework and design could be applicable simply by scaling the numerology between 480 and 960 kHz SCS. Therefore, amount of standardization effort would be minimal.

The fifth aspect is about additional cell search complexity. For NR operating in 60 GHz band, use of 480 kHz and 960 kHz SCS was made optional for UEs. Therefore, any potential added complexity will not be mandated for all UEs. In addition, UEs performing cell search for multiple SCS is not something new for Rel-17. Many bands that were defined in Rel-15 required UE to perform cell search for multiple SSB SCS. Therefore, we do not think cell search complexity is an issue.

Based on discussions above, we suggest the following:

Proposal 1:

- Support 480 kHz and 960 kHz SCS for SSB for initial access cases.

  - Note: support of 480kHz and/or 960kHz SCS for SSB is optional.

- Support CORESET#0/Type0-PDCCH configuration indication in MIB of SSB for all supported SSB SCS.

## SSB and CORESET#0 Resources

In Rel-16 (FR1), 15kHz and 30kHz SCS was supported for SS/PBCH block (SSB) in 5 GHz and 6 GHz bands and only multiplexing pattern 1 (TDM of Type0-PDCCH and SSB) was supported. In FR2, for 120 kHz, SSB pattern within a pair of slots is defined as SSBs located at symbols {4, 5, 6, 7}, {8, 9, 10, 11} in the first slot of the slot pair and at {2, 3, 4, 5}, {6, 7, 8, 9} in the second slot of the slot pair, where symbol indices are enumerated from 0 starting from beginning of a slot.

Figure 2: SSB Pattern when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

For the design of SSB SCS of 480 kHz and 960 kHz, we can leverage the design criterion that were considered for Rel-15 SSB resource pattern designs. The following are some considerations that we believe are useful and important for SSB resource and Type0-PDCCH resource placements.

- Support possibility of time contiguous transmission within a DRS transmission occasion and to maximize the transmission opportunities of a SSB transmission. It is beneficial to co-locate a RMSI-CORESET with an associated SSB, since it will allow transmission of SSBs in the second half of a slot in case of failed LBT attempts at the beginning of the slot. Even if LBT is not utilized by SSB and RMSI, co-locating the resources for SSB and LBT, placing the SSB and RMSI in the same slot allows same beam to be utilized for both signals, which enabled efficient transmission of broadcast channel that require to be transmitted in multiple beam directions.

- Support of sufficient gap for beam switching between consecutive SSBs. When subcarrier spacing of SSB was small, the CP at the beginning of SSB might have been sufficient to absorb any beam transitional effects between two different SSB and different beam. However, for larger SCS, such as 480 kHz and 960 kHz, the CP duration may be not sufficient margin to absorb beam switching gaps. Therefore, supporting at least 1 symbol gap between SSB could help with beam switching.

- Support of sufficient gap for Rx-Tx switching. In FR2, the Rx-Tx switching time is 7µs, while the power ON/OFF transient times are 5µs. For larger SCS, such as 960kHz, we would need 7 symbols to absorb 7µs Tx-Rx switching gap and the gap between SSB and potential uplink transmission in existing SSB patterns would not be sufficient. Absorbing Rx-Tx switching time needed between SSB and potential uplink transmission could be achieved by introducing non-SSB carrying slots in between SSB carrying slots or modifying the SSB pattern such that sufficient symbol gap exists by reducing the number of SSB transmitted. Introducing non-SSB carrying slots is preferred as this does not necessarily result in complex SSB position design within a slot.

While the support of CORESET#0/Type0-PDCCH configuration in MIB for 480kHz and 960kHz based SSB has not been agreed for Rel-17. Even if the CORESET#0/Type0-PDCCH configuration in MIB is not supported in Rel-17, we believe consideration of CORESET#0/Type0-PDCCH should be considered as part of SSB pattern design to ensure support of CORESET#0/Type0-PDCCH for future releases. The SSB design once implemented cannot be changed in future releases. Therefore, careful consideration of functionality for future release is extremely important.

Observation 3:

- SSB pattern should be designed to also take into account any aspects that may be considered for future releases, as SSB pattern once implemented in Rel-17 cannot be changed in later releases.

In case UE needs to perform measurement of neighbour cells at the same time as serving cell SSB reception, the UE may not be expected to receive PDCCH on SSB symbols to be measured, and on 1 data symbol before and after each consecutive SSB symbols to be measured. This scheduling restriction was imposed in RAN4 to alleviate complexity burden at the receiver and to allow UEs operating with a single antenna array panel (as simultaneous reception of neighbour cell SSB and serving cell signals could require multiple beams and panels). One aspect that was considered during Rel-15 SSB resource pattern design was support of short uplink transmission in the same slot as SSB. For SSB with 480 or 960 kHz, we believe this specific consideration is no longer needed. The slot duration for 480 and 960 kHz is 4 and 8 times magnitude shorter than 120 kHz. Therefore, instead of having few symbols not occupied by SSB in a slot, it is much more beneficial to have an UL slot every few slots that do not contain SSB. Non-SSB carrying slots can be then further utilized for any potential uplink transmission. Taking the above design factors into account, we propose the SSB slot pattern shown in Figure 2, where 3 consecutive (potentially) SSB containing slots are followed by 1 slot that SSB cannot be transmitted on for 480 kHz, and 6 consecutive (potentially) SSB containing slots are followed by 2 slot that SSB cannot be transmitted on for 960 kHz. We also propose the SSB pattern shown inFigure 3, where SSBs are located at symbols {2, 3, 4,5}, {9, 10, 11, 12} in both the slots. Type0-PDCCH CORESET can be placed {0,1} and {7,8}, which will allow co-located placement of RMSI and SSB for efficiency and provide sufficient gap between SSB for any potential beam switching delay. To avoid further complication, we propose to use the same SSB placement within a slot for all SSB containing slots. This can simplify specification and implementation efforts.

Figure 3: Proposed SSB slot pattern for SCS 120 kHz and higher

Figure 4: Proposed SSB Pattern for SCS 120kHz and higher

Proposal 2:

- Consider SSB pattern in a slot with 3 SSB containing slots, each slot with 2 SSB position, followed by 1 non-SSB carrying slot for 480 kHz and 6 SSB carrying slots followed by 2 non-SSB carrying slots for 960kHz, to accommodate Rx-Tx switching gap.

  - For 480kHz and 960kHz SCS based SSB, first symbols of the candidate SSB have indexes {2,9} + 14×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

  - For 480kHz, n = 0,1,2, 4,5,6, 8,9,10, 12,13,14, 16,17,18, 20,21,22, 24,25,26, 28,29,30, 32,33,34,  36,37,38, 40,41.

  - For 960kHz, n = 0,1,2,3,4,5, 8,9,10,11,12,13, 16,17,18,19,20,21, 24,25,26,27,28,29, 32,33,34,35,36,37, 40, 41.

As mentioned above, Type0-PDCCH search space and can be placed in symbols {0,1} and {7, 8} for each SSB. If used with type 1 CORESET#0 multiplexing pattern (i.e. TDM), all symbols occupied by SSB can be also utilized by RMSI for transmission, which allows efficient multiplexing of RMSI and SSB and save beam sweeping overhead. In case, we do not need to support different SCS between Type0-PDCCH and SSB for 480kHz and 960kHz, we can simply the Type0-PDCCH configuration design and minimize specification effort. The need to support mixed numerology for 480kHz and 960kHz SSB is not clear, therefore we suggest to only support same SCS for SSB and CORESET#0 that is configured by MIB.

Proposal 3:

- Consider only same SCS for SSB and CORESET#0 (configured by MIB) for 480 and 960 kHz SCS.

## Supported PRACH Formats and PRACH RO Configuration

In RAN1 #104-bis-e, the following agreement was made for PRACH.

Agreement: - PRACH configuration for 480/960 kHz SCS (if agreed) - The minimum PRACH configuration period is 10 ms (as in FR2) - For RO configuration for PRACH with 480/960kHz SCS, - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least: - location of 480/960 kHz PRACH slot per reference slot - location of duration containing 480/960khz PRACH slot pattern within 10ms - potential impact to RA-RNTI calculation

To benefit from single numerology operation in the UL, we propose to support SCS 480 kHz and 960 kHz for PRACH transmissions. In this case, there is no issues with PRACH preamble detection performance as reported in the most recent study on NR extension up to 71 GHz [2] where candidate SCSs 120 kHz, 480 kHz and 960 kHz have demonstrated almost no difference in terms of required SINR to achieve the target misdetection rate of 1%. Some debates could happen around insufficient coverage of PRACH preamble if SCS 480 kHz and 960 kHz would be specified for it. However, our link-budget analysis, conducted for the study on NR coverage enhancement [3], has shown that PRACH has never become a bottleneck in the UL (for example, Figure 3).

(a) Indoor Scenario	(b) Urban Outdoor Scenario

Figure 5: Example MIL results to demonstrate PRACH coverage comparing to the coverage of other channels

Someone may argue that SCS 480 kHz and 960 kHz cannot be applicable to PRACH preamble because of detection issues arising in channels with large signal propagation delays. To address that, we first refer to our analysis on the channel RMS delay spread observed in different scenarios [4] where it has been shown the CP of numerologies with SCS 480 kHz and 960 kHz is able to absorb significant portion of channel’s delay (Figure 4).

(a)	(b)

Figure 6: CDFs of channel RMS delay spread

Secondly, preamble detection in channels with large signal propagation delay can be handled by proper receiver implementation. One example is energy detection technique, as described in [5], which allows to detect PRACH preamble even when channel delay exceeds the length of PRACH OFDM symbol. The discussion above leads us to the following proposal.

Proposal 4:

- Support 480 kHz and 960 kHz SCS for PRACH in NR extension up to 71 GHz.

  - Note: no need to distinguish whether the PRACH is for initial access or non-initial access, as such distinction does not exist for RAN1 specification.

The discussion regarding PRACH preamble formats for SCS 480 kHz and 960 kHz, supported in NR extension up to 71 GHz in addition to the formats for LRA=139, should consider minimal channel bandwidth. The random access (RA) bandwidth and recommended minimal channel bandwidth are given in Table 2-1 for LRA∈{571, 1151} and ΔfRA∈ {480 kHz, 960 kHz}.

Table 2-1. RA bandwidth and recommended min channel bandwidth

| LRA | NRBRA | ΔfRA | RA bandwidth | Recommended min channel bandwidth |
| --- | --- | --- | --- | --- |
| 571 | 48 | 480 kHz | 276.48 MHz | 400 MHz |
| 1151 | 96 | 480 kHz | 552.96 MHz | 800 MHz |
| 571 | 48 | 960 kHz | 552.96 MHz | 800 MHz |
| 1151 | 96 | 960 kHz | 1105.92 MHz | 1600 MHz |

In RAN4 #98-bis-e meeting, the minimal channel bandwidth of 400 MHz was agreed for SCS 480 kHz and 960 kHz [10]. From Table 2-1 it can be seen that the recommended min channel bandwidth for SCS 960 kHz exceeds the agreed min channel bandwidth in case of LRA=571 and LRA =1151. For SCS 480 kHz, the recommended min channel bandwidth is the same as the agreed min channel bandwidth only in case of LRA =571. Based on this comparison, it’s proposed to support optionally the PRACH preamble formats A1~A3, B1~B4, C0, C2 for LRA=571 and LRA=1151 in case of PRACH SCS 480 kHz or 960 kHz.

Proposal 5:

- Optional support of PRACH formats A1~A3, B1~B4, C0, C2 for LRA= 571 and LRA= 1151 with SCS 480 kHz and 960 kHz, i.e., μ∈{5, 6}.

Regarding PRACH RO configurations, Table 6.3.3.2-4 from TS 38.211, which defines random access configurations for FR2 in TDD, is reused for SCS 120 kHz in NR extension from 52.6 GHz up to 71 GHz. The same table should be also used in case of PRACH SCS 480 kHz or 960 kHz to minimize the amount of specification work. This can be enabled by applying the following design principles:

- The numerology for reference slot counting within a system frame remains corresponding to SCS 60 kHz;

- The max number of starting positions for PRACH slots within a reference slot (which has SCS 60 kHz) is equal to 2;

- Fix the starting position(s) of PRACH slots within the reference slot by properly setting the values of parameter nslotRA (TS 38.211, Section 5.3.2).

  - The starting position(s) should be aligned with the SSB slot patterns in order to avoid systematic overlapping between SSBs and ROs.

These principles are illustrated in Figure 5 for nslotRA=4 and nslotRA=8 when there is a single PRACH slot with SCS 480 kHz and 960 kHz, respectively, within the reference slot, and for nslotRA∈{0, 4} and nslotRA∈{0, 8} when there are two PRACH slots with SCS 480 kHz and 960 kHz, respectively, within the reference slot. In the figure, starting position for PRACH slots for counting PRACH ROs are depicted with hatch.

(a) nslotRA=4 (480 kHz) and nslotRA=8 (960 kHz)	(b) nslotRA∈{0, 4} (480 kHz) and nslotRA∈{0, 8} (960 kHz)

Figure 7. PRACH slot locations within reference slot

Proposal 6: Regarding PRACH RO configurations for SCS 480 kHz and 960 kHz:

- The numerology for reference slot counting within a system frame remains corresponding to SCS 60 kHz;

- The max number of starting positions for PRACH slots within a reference slot (which has SCS 60 kHz) is equal to 2;

- Fix the starting position(s) of PRACH slots within the reference slot by properly setting the values of parameter nslotRA (TS 38.211, Section 5.3.2).

  - The starting position(s) should be aligned with the SSB slot patterns in order to avoid systematic overlapping between SSBs and ROs.

- Reuse PRACH RO configurations listed in Table 6.3.3.2-4 from TS 38.211.

Another discussion point, which arose first in RAN1#104-e, was regarding the support of non-consecutive PRACH ROs in the time domain. Currently, multiple ROs within a PRACH slot are defined in a consecutive manner. Configuration of non-consecutive PRACH ROs assumes a time gap between the ROs which lasts one or several OFDM symbols. In case of PRACH SCS 480 kHz or 960 kHz, these time gaps could be used, for example, for Rx beam switching. Typically, the CP duration is utilized to switch the beams as the time needed for that is estimated by RAN4 around 100 ns, which is lower than CP length of numerology with SCS 120 kHz. However, as the CP length further shrinks with the increase of SCS, it may not be sufficient to fully accommodate a beam switch transient period in case of SCS 480 kHz or 960 kHz. Therefore, it’s proposed to have an option to introduce a gap between consecutive PRACH ROs for SCS 480 kHz and 960 kHz. One straightforward approach is to slightly modify the equation for computation of the first OFDM symbol of PRACH RO. This equation is given in Section 5.3.2 from TS 38.211 and copied below:

l=l0+ntRANdurRA+14nslotRA.

In the equation above, ntRA is the index of RO within a PRACH slot. It ranges from 0 to NtRA,slot-1. NdurRA is PRACH RO duration given in a number of OFDM symbols. The values for both NtRA,slot and NdurRA are listed in Table 6.3.3.2-4 from TS 38.211. The modification is presented below:

l=l0+ntRANdurRA+NgapRA+14nslotRA,

where NgapRA is the number of OFDM symbols for the time gap. Typically, NgapRA=1 could be sufficient. It should be noted, that for some of PRACH RO configurations provided in Table 6.3.3.2-4 from TS 38.211, the following may be possible: l0+ntRANdurRA+NgapRA≥14. This means nothing but that non-consecutive RO with time gaps in between occupy two consecutive PRACH slots as illustrated in Figure 6. Therefore, the total number of PRACH slots occupied by PRACH ROs may be greater than 2 in case of non-consecutive ROs with time gaps in between. However, the RO density, which is assumed as the total number of ROs within a reference slot, for PRACH SCS 480 kHz or 960 kHz remains the same as for SCS 120 kHz. Also, the number of starting positions for PRACH slots, which is controlled by the number of values the parameter nslotRA could take, remains equal to 2. Based on that understanding, no changes to the entries of the table with all PRACH RO configurations (Table 6.3.3.2-4 from TS 38.211) is needed.

(a) nslotRA=4 (480 kHz) and nslotRA=8 (960 kHz)	(b) nslotRA∈{0, 4} (480 kHz) and nslotRA∈{0, 8} (960 kHz)

Figure 8. Non-consecutive PRACH ROs with gaps in between occupy two consecutive PRACH slots.

Proposal 7:

- For PRACH SCS 480 kHz and 960 kHz, introduce optional time gaps between consecutive ROs;

- Modify equation defining the first OFDM symbol of PRACH RO given Section 5.3.2 from TS 38.211 as follows:

  - l=l0+ntRANdurRA+NgapRA+14nslotRA,

  - where NgapRA is the gap duration (number of OFDM symbols) and NgapRA=0 for no gap.

## Discussion on RA-RNTI Calculation

The current procedure of RA-RNTI computation is described in Section 5.1.3 of TS 38.321 from which the corresponding equation is excerpted and given by:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id.

In the equation, the index of the first slot of the PRACH occasion in a system frame, denoted as t_id, corresponds to PRACH SCS and has a range: 0 ≤ t_id < 80. In RAN1#104-e, it was identified that the current RA-RNTI calculation results in an overflow of RA-RNTI value for larger PRACH SCS (i.e., 480 kHz and 960 kHz). In particular, for SCS 960 kHz: 0 ≤ t_id < 640, and the resulting RA-RNTI exceeds the maximum provided by the current 16-bit RNTI. Taking into account configuration of PRACH RO described in Section 2.3, the following simple modification of RA-RNTI computation equation could be proposed:

- for PRACH SCS 480 kHz: RA-RNTI = 1 + s_id + 14 × ët_id/4û + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id;

- for PRACH SCS 960 kHz: RA-RNTI = 1 + s_id + 14 × ët_id/8û + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id.

After summarizing for all possible PRACH SCS values (μ=0,1,2,3,5,6), the modified equation can be given by:

- RA-RNTI = 1 + s_id + 14 × ët_id / max1, 2μ-3û + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id.

Proposal 8:

- RA-RNTI computation equation should be adjusted to avoid overflow in case of PRACH SCS 480 kHz and 960 kHz;

- Support the following modified equation for RA-RNTI computation:

  - RA-RNTI = 1 + s_id + 14 × ët_id / max1, 2μ-3û + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id,

  - where t_id is based on the value of μ specified in clause 5.3.2 of TS 38.211.

## DRS and Short Signal Exemption

In RAN1#104-bis-e, the following agreement was made regarding discovery burst (DB):

Agreement: - For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0 - FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements - PBCH payload size is no greater than that for FR2 - Duration of DBTW is no greater than 5 ms - Number of PBCH DMRS sequences is the same as for FR2 - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs - FFS: how to support UEs performing initial access that do not have any prior information on DBTW. - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands - FFS: details of how to inform UEs of the configuration of DBTW

Regarding the short control signalling exception, the following was agreed in RAN1 #104-bis-e:

Agreement: - Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH. - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule - FFS: Whether this can be applied to all supported SCS or specific SCS. - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval) - FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc.

Short signal exemption (SSE) is a mechanism to send relatively short signalling transmissions (compared to regular data transmissions) without sensing the channel for presence of other signals. For frequency ranges from 52.6 GHz up to 71 GHz, SSE is described in the most recent draft of EN 302 567 v2.2.0 [6] with application to short management and control frames. The total duration of the short control signaling transmissions is required to be constrained to less than 10 msec within 100 msec observation period.

EN 302 567 v2.2.0 classifies “short control signal” as control and management frames, which are terminology used in IEEE 802.11 system. These control and management frames do not have specific definition in EN 302 567, and therefore some discussion is needed in RAN1 to determine which signals and channel could be qualified as control and management frames.

Some candidates that may be considered for short control signal are:

- SS/PBCH Block

- Type0-PDCCH (for RMSI)

- Type0A-PDCCH (for OSI)

- Type1-PDCCH (for RAR)

- Type2-PDCCH (for paging)

- Type3-PDCCH (for DCI formats for other purposes such as TPC, SFI, Pre-emption indication, or cancellation indication)

- PRACH

- PUCCH carrying HARQ-ACK

- PUCCH carrying SR

- Msg 3 (after successful PRACH)

For 960 kHz SSB and Type0-PDCCH, if we assume the worst case transmission overhead and 20 msec SSB periodicity, the total transmission duration would be less than 7.5 msec (32 slots required for 64 SSB, 64 slots required for 64 Type0-PDCCH and PDSCH carrying RMSI results in 96 slots, which is approximately 1.5 msec in duration every 20 msec). Therefore, there may not be much difficulty in utilizing short control signal exemption for SSB and Type0-PDCCH and corresponding PDSCH.

However, for 480 kHz or 120 kHz SSB and Type0-PDCCH, if we assume the worst-case transmission overhead of 96 slots every 20 msec SSB period, this will exceed the 10 msec limits for short control signal exemption. Therefore, specification would need to at least support transmission of SSB by gNB and Type0-PDCCH with LBT even with short control signal exemption.

Even if we only consider SSB as part of short control signal exemption. 64 SSB transmission with 120 kHz SCS requires 32 slots, which is approximately 4 msec every SSB period. Given that 20 msec is a default SSB period, even SSB alone would not satisfy the 10 msec limit within 100 msec observation period constraint for short control signal exemption.

Observation 4:

- For 120 kHz SCS SSB, transmission of 64 SSB with 20 msec SSB periodicity exceed 10 msec transmission duration within a 100 msec observation period required for short control signal exemption.

- For 480 kHz SCS SSB, transmission of 64 SSB and 64 Type0-PDCCH with associated PDSCH with 20 msec SSB periodicity exceed 10 msec transmission duration within a 100 msec observation period required for short control signal exemption.

- For 960 kHz SCS SSB, transmission of 64 SSB and 64 Type0-PDCCH with associated PDSCH with 20 msec SSB periodicity does not exceed 10 msec transmission duration within a 100 msec observation period required for short control signal exemption.

To enable SS burst as DRS at least for SCS 120 kHz, the number of SSB transmission opportunities should be increased to deal with possible LBT events. The following options could be proposed to address this:

- Option 1: Reuse the approach of Rel-16 NR-U;

- Option 2: Introduce a floating discovery burst transmission window (DBTW).

Option 1 implies that a larger number of candidate SSB indices should be provided. Additionally, some means are needed for QCL assumption indication for SSBs within an SS burst with the increased number of candidate SSB indices considering the max number of SSB beams is limited by 64 according the WID [1].

Assuming the duration of DBTW is upper bounded by 5 ms, the max number of candidate SSB indices for SCS 120 kHz is L max=80. Therefore, supporting Option 1 for 120kHz SSB would essentially add 8 more slots to be used as candidates positions for SSB. Figure 9 shows the candidate SSB slots for 120kHz that has been agreed. There are 8 additional slots that is not currently utilized. Since the candidates SSB positions are not always consecutive, if the currently unused SSB slots positions are used, this may result in re-enumeration of SSB indices (for candidate position indication) for 120kHz.

Figure 9. Illustration of candidate SSB slots for 120kHz (agreed so far)

The side effect of increasing the available SSB slots, i.e., Option 1, is that in case of DB and DBTW is not needed as regulatory requirements do not require it so, or used in licensed operation, the SSB candidate slots positions would be different, and potentially the SSB indexing of the enumerated SSB candidate position could be different. This may require some signaling or mechanism to indicate whether DB/DBTW is enabled or disabled during initial access.

Option 2 allows the DBTW to be shifted by an offset smaller than 5 msec. The more granular the supported shift values are, the greater the ability to deal with LBT. This does not require more SSB candidate positions to be defined for 120kHz SCS. However, it does require to relax the SSB transmission constraint that was placed in Rel-15, where all transmitted SSBs are always confined within the same half radio frame, and have SSB transmission spill over half radio frames and even possibly radio frames. Figure 10 shows an example of DBTW shifted by 2.5msec.

Figure 10. Illustration of shifted DBTW by 2.5msec

The side effect of Option 2 is that new measurement gap patterns and RRM measurement configuration may need to be supported as the transmission of the SSB is no longer confined a half radio frame, but can be shifted depending on LBT. This potentially means support of larger measurement gap than 6msec, and asking UEs to perform SSB searches for longer periods during RRM measurements.

Both Option 1 and 2 do have some RAN1 design impact. However, without consideration of either Option 1 or 2, in case gNB has 64 SSBs, the existing NR design does not leave additional SSB candidate positions to be cope with LBT failure. This also means, in case of LBT failure events, the first SSB indices are more likely to consistently fail for transmission compared to latter SSB indices. This can result in RRM measurement performance loss for the first SSB indices and potentially impact to cell coverage in specific beam directions. If neither of Option 1 nor 2 is attractive, then specification should consider some mechanisms to balance out SSB DTX among all SSB beams from LBT failure.

Looking at the potential options, MIB may be required to support additional information than what is already supported in NR. For example, for 120kHz SSB L max=64 is used currently in NR, which requires 6 bits to encode a candidate SSB index, where 3 MSB are transmitted within PBCH payload and 3 LSB are indicated by PBCH DM-RS sequence index NDM-RSPBCH. Therefore, 1 additional bit is required for encoding a candidate SSB index when L max=80.

In Rel-16 NR-U, the parameter NSSBQCL was introduced to indicate QCL relationship across SSBs so that the UE assumes SSBs are QCL-ed if a value of NDM-RSPBCH mod NSSBQCL is the same among SSBs. The parameter NSSBQCL∈1, 2, 4, 8 can be indicated as part of PBCH payload. The number of actually transmitted SSBs within a DBTW cannot exceed NSSBQCL. Similar approach could be applied to SS burst-based DRS with SCS 120 kHz for QCL indication: SSBs are QCL-ed if they have the same value of ι mod NSSBQCL where ι is the candidate SSB index (0≤ι<Lmax ) and maxNSSBQCL=64. In NR extension from 52.6 GHz up to 71 GHz, it is very unlikely to operate relying on a single Tx beam only. Because of that NSSBQCL=1 could be excluded. Therefore, a possible set of NSSBQCL values could be defined as NSSBQCL∈ 2, 4, 8,64 or NSSBQCL∈8,64 which would require 2 bits or 1 bit for indication, respectively. Based on the discussion above, up to 3 additional bits are needed to indicate a larger number of candidate SSB indices and NSSBQCL for enabling DRS based on SS burst with SCS 120 kHz.

In Rel-16 NR-U, to indicate NSSBQCL parameter value in PBCH payload, two MIB bits, subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset, are reinterpreted as defined in Table 4.1-1 from TS 38.213 copied below:

Table 2-2. Table 4.1-1 from TS 38.213

| subCarrierSpacingCommon | LSB of ssb-SubcarrierOffset | NSSBQCL |
| --- | --- | --- |
| scs15or60 | 0 | 1 |
| scs15or60 | 1 | 2 |
| scs30or120 | 0 | 4 |
| scs30or120 | 1 | 8 |

This is possible due to certain UE assumptions available in NR-U: the same SCS between SSB and RMSI and restricted sync raster positions relative to the channel raster. Similar assumptions could be taken for NR extension from 52.6 GHz up to 71 GHz to borrow MIB bits for indication of candidate SSB index and NQCLSSB. For example, the following options are possible assuming SCS is always the same between SSB and RMSI:

Option A.	subCarrierSpacingCommon bit is reused to indicate NSSBQCL∈8,64 value, and LSB of ssb-SubcarrierOffset is reused for indication of candidate SSB index (in addition to existing mechanism);

Option B.	subCarrierSpacingCommon bit and LSB of ssb-SubcarrierOffset are reused to indicate NSSBQCL∈2,4,8,64 value, and 1 bit is additionally borrowed from ssb-SubcarrierOffset for indication of candidate SSB index.

Both options A and B above require clarification from RAN4 regarding channel and sync raster relationship in NR extension from 52.6 GHz up to 71 GHz. For example, if RAN4 would confirm that sync raster and channel raster are PRB-aligned (kSSB=0) or sync raster could only be a half-PRB shifted to channel raster (kSSB=6) then up to 3 bits could be borrowed from ssb-SubcarrierOffset out of 4 bits.

Although whether we can support DRS operation in 60GHz band by barrowing bits from unused fields in MIB may depend on RAN4’s raster design, based on analysis so far, we believe the support of DRS operation without increasing MIB size is certainly feasible. Therefore, it’s suggested to agree to at least supporting DBTW for SSB with 120kHz SCS.

Proposal 9: At least for SSB SCS 120 kHz,

- Support DBTW

  - Support signaling of enable/disable of DB and DBTW

- Consider supporting Option 1 and/or 2 for DB and DBTW for 120kHz SSB:

  - Option 1:

    - Increase the number of candidate SSB indices up to 80, i.e., L max=80;

    - Support additional values of n, such as 4, 9, 14, 19, in the equation defining the first symbols of candidate SS/PBCH blocks

    - For QCL relationship indication across SSBs, reuse Rel-16 NR-U mechanism by introducing NQCLSSB parameter

      - FFS: NSSBQCL∈ 2, 4, 8,64 or NSSBQCL∈8,64;

    - No changes to MIB payload size. Further discuss and consider reinterpreting bits from some bit fields within MIB to extend candidate SSB index and NQCLSSB information.

  - Option 2:

    - Support floating DBTW, where the time (or slot) offset for DBTW can be smaller than 5msec.

      - FFS: smallest supported DBTW offset (i.e. granularity of the floating DBTW)

- If neither Option 1 nor 2 is supported, RAN1 to support mechanism to balance out SSB DTX (among all SSB beams) from LBT failure.

For PRACH transmission, given that PRACH is only transmitted during initial access and other rare occasions, it might be possible to always consider utilizing short control signal exemption. UE is expected to retransmit PRACH only after RAR reception failure, therefore even with repetitive PRACH transmissions, random access protocol creates a natural gap between two consecutive PRACH transmissions. From our analysis, even if we utilize 120 kHz SCS for PRACH, we do not believe the UE could ever exceed total transmission duration of 10 msec within 100 msec observation period.

Observation 5:

- For 120 kHz, 480kHz, and 960 kHz PRACH transmission, UE does not exceed total transmission duration of 10 msec for PRACH within a 100 msec observation period.

Proposal 10:

- Consider applying short control signal exemption to PRACH transmission by the UE.

# Conclusions

In this contribution, we discussed issues related to initial access for extending NR up to 71 GHz. The following is a summary of the observation and proposals:

Observation 1:

- Single numerology operation can enable efficient transceiver implementation and operation.

Observation 2:

- UE implementation complexity for supporting non-initial access is greater than initial access case, and support of initial access case is a subset of non-initial access case from hardware implementation perspective.

- The only difference between non-initial access and initial access case is number of frequency entries that UE needs to perform search. However, there are method to mitigate this complexity and lower SSB raster entries.

Observation 3:

- SSB pattern should be designed to also take into account any aspects that may be considered for future releases, as SSB pattern once implemented in Rel-17 cannot be changed in later releases.

Observation 4:

- For 120 kHz SCS SSB, transmission of 64 SSB with 20 msec SSB periodicity exceed 10 msec transmission duration within a 100 msec observation period required for short control signal exemption.

- For 480 kHz SCS SSB, transmission of 64 SSB and 64 Type0-PDCCH with associated PDSCH with 20 msec SSB periodicity exceed 10 msec transmission duration within a 100 msec observation period required for short control signal exemption.

- For 960 kHz SCS SSB, transmission of 64 SSB and 64 Type0-PDCCH with associated PDSCH with 20 msec SSB periodicity does not exceed 10 msec transmission duration within a 100 msec observation period required for short control signal exemption.

Observation 5:

- For 120 kHz, 480kHz, and 960 kHz PRACH transmission, UE does not exceed total transmission duration of 10 msec for PRACH within a 100 msec observation period.

Proposal 1:

- Support 480 kHz and 960 kHz SCS for SSB for initial access cases.

  - Note: support of 480kHz and/or 960kHz SCS for SSB is optional.

- Support CORESET#0/Type0-PDCCH configuration indication in MIB of SSB for all supported SSB SCS.

Proposal 2:

- Consider SSB pattern in a slot with 3 SSB containing slots, each slot with 2 SSB position, followed by 1 non-SSB carrying slot for 480 kHz and 6 SSB carrying slots followed by 2 non-SSB carrying slots for 960kHz, to accommodate Rx-Tx switching gap.

  - For 480kHz and 960kHz SCS based SSB, first symbols of the candidate SSB have indexes {2,9} + 14×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

  - For 480kHz, n = 0,1,2, 4,5,6, 8,9,10, 12,13,14, 16,17,18, 20,21,22, 24,25,26, 28,29,30, 32,33,34,  36,37,38, 40,41.

  - For 960kHz, n = 0,1,2,3,4,5, 8,9,10,11,12,13, 16,17,18,19,20,21, 24,25,26,27,28,29, 32,33,34,35,36,37, 40, 41.

Proposal 3:

- Consider only same SCS for SSB and CORESET#0 (configured by MIB) for 480 and 960 kHz SCS.

Proposal 4:

- Support 480 kHz and 960 kHz SCS for PRACH in NR extension up to 71 GHz.

  - Note: no need to distinguish whether the PRACH is for initial access or non-initial access, as such distinction does not exist for RAN1 specification.

Proposal 5:

- Optional support of PRACH formats A1~A3, B1~B4, C0, C2 for LRA= 571 and LRA= 1151 with SCS 480 kHz and 960 kHz, i.e., μ∈{5, 6}.

Proposal 6: Regarding PRACH RO configurations for SCS 480 kHz and 960 kHz:

- The numerology for reference slot counting within a system frame remains corresponding to SCS 60 kHz;

- The max number of starting positions for PRACH slots within a reference slot (which has SCS 60 kHz) is equal to 2;

- Fix the starting position(s) of PRACH slots within the reference slot by properly setting the values of parameter nslotRA (TS 38.211, Section 5.3.2).

  - The starting position(s) should be aligned with the SSB slot patterns in order to avoid systematic overlapping between SSBs and ROs.

- Reuse PRACH RO configurations listed in Table 6.3.3.2-4 from TS 38.211.

Proposal 7:

- For PRACH SCS 480 kHz and 960 kHz, introduce optional time gaps between consecutive ROs;

- Modify equation defining the first OFDM symbol of PRACH RO given Section 5.3.2 from TS 38.211 as follows:

  - l=l0+ntRANdurRA+NgapRA+14nslotRA,

  - where NgapRA is the gap duration (number of OFDM symbols) and NgapRA=0 for no gap.

Proposal 8:

- RA-RNTI computation equation should be adjusted to avoid overflow in case of PRACH SCS 480 kHz and 960 kHz;

- Support the following modified equation for RA-RNTI computation:

  - RA-RNTI = 1 + s_id + 14 × ët_id / max1, 2μ-3û + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id,

  - where t_id is based on the value of μ specified in clause 5.3.2 of TS 38.211.

Proposal 9: At least for SSB SCS 120 kHz:

- Support DBTW

  - Support signaling of enable/disable of DB and DBTW

- Consider supporting Option 1 and/or 2 for DB and DBTW for 120kHz SSB:

  - Option 1:

    - Increase the number of candidate SSB indices up to 80, i.e., L max=80;

    - Support additional values of n, such as 4, 9, 14, 19, in the equation defining the first symbols of candidate SS/PBCH blocks

    - For QCL relationship indication across SSBs, reuse Rel-16 NR-U mechanism by introducing NQCLSSB parameter

      - FFS: NSSBQCL∈ 2, 4, 8,64 or NSSBQCL∈8,64;

    - No changes to MIB payload size. Further discuss and consider reinterpreting bits from some bit fields within MIB to extend candidate SSB index and NQCLSSB information.

  - Option 2:

    - Support floating DBTW, where the time (or slot) offset for DBTW can be smaller than 5msec.

      - FFS: smallest supported DBTW offset (i.e. granularity of the floating DBTW)

- If neither Option 1 nor 2 is supported, RAN1 to support mechanism to balance out SSB DTX (among all SSB beams) from LBT failure.

Proposal 10:

- Consider applying short control signal exemption to PRACH transmission by the UE.

# References

- RP-RP-210862, “Revised WID: Extending current NR operation to 71GHz,” CMCC.

- 3GPP TR 38.808 V17.0.0. Study on supporting NR from 52.6 GHz to 71 GHz,” March. 2021.

- R1-2007953, “On baseline coverage performance for FR2,” Intel Corp. 3GPP TSG RAN WG1 #103-e. Oct. 26th – Nov. 13th, 2020.

- R1-2009379, “Discussion on Required Changes to NR in 52.6 – 71 GHz,” Intel Corp. 3GPP TSG RAN WG1 #103-e. Oct. 26th – Nov. 13th, 2020.

- R1-1702127, “NR PRACH design,” Ericsson. 3GPP TSG-RAN WG1 Meeting #88. Athens, Greece, 13th – 17th Feb. 2017.

- Draft ETSI EN 302 567 V2.2.0, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,” Dec. 2020.

- R1-2104124, “Summary #3 of email discussion on initial access aspects of NR extension up to 71 GHz,” Moderator (Intel Corporation).

- R1-2009379, “Discussion on Required Changes to NR in 52.6 – 71 GHz,” Intel Corporation

- R1-2103229, “Initial access aspects for NR from 52.6 GHz to 71 GHz,” Samsung

- R4-2105411, “Reply LS on the maximum/minimum channel bandwidths and channelization for NR operation in 52.6 to 71GHz”, RAN WG4. 3GPP TSG-RAN WG4 Meeting #98-bis-e. Electronic Meeting, 12th - 20th April, 2021.