3GPP TSG RAN WG1 #105e                                                             R1-2104905

e-Meeting, May 10th – 27th, 2021

Source: 	Intel Corporation

Title: 	Mitigation of UE/gNB TX/RX Timing Errors

Agenda item:	8.5.1

Document for: 	Discussion and Decision

# Introduction

The RAN WG1 agreed to study the specification impact for enabling a reference device to perform measurements and reporting of timing related parameters and support gNB and UE RX/TX timing errors mitigation. The concept of Timing Error Groups (TEGs) was introduced to facilitate mitigation of RX/TX timing errors for the timing-based positioning methods (DL-TDOA, UL-TDOA, and Multi-RTT).

It was agreed to support the UE RX TEG(s) association with the DL RSTD measurement and the TRP TX TEGs association with the DL PRS resources for the DL-TDOA positioning method, [1]. Similar agreement was done for the UL-TDOA positioning methods, where TRP RX TEG(s) is associated with the RTOA measurement and the UE TX TEGs are associated with the UL Positioning SRS resources, [1].

In case of the Multi-RTT positioning method two alternatives were considered to support TEG(s) association with the gNB Rx-Tx and UE Rx-Tx time difference measurements. The following agreement for the gNB Rx-Tx time difference measurement was captured in the chairman’s notes, [1]:

Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signaling, procedures

Similar agreement was captured for the UE Rx-Tx time difference measurement in the chairman’s notes, [1]:

Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signaling, procedures

The proposal for the reference device specification impact evaluation was discussed but was not agreed yet. It was captured in the FL’s summary, [2]:

Proposal: - RAN1 has evaluated the use of reference devices, which can either be UE or TRP, for positioning and observes improvements in using reference devices for enhancing the positioning performance. - Note 1: The position of the reference device is known - Note 2: If the device is a TRP, it needs at least to support some of the Rel-16 positioning functionalities, which will be defined by RAN2. For example, the device positioning functionalities may include, but not limited to, the following: - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) - Transmit the UL SRS signals for positioning - Note 3: If the device is a UE, it may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the device is known, the information may also be requested by the LMF - Note 4: The impact on the specification, the measurement reports, and the procedure for supporting a UE/TRP to be a reference device will be determined by RAN2/RAN3/SA2 - Note 5: Up to RAN2/RAN3 discussions what type(s) of UE/TRP can be reference devices and any capabilities if/as needed - Note 6: RAN1 has not identified specification enhancements needed in RAN1 specifications - Send an LS to RAN2/RAN3/SA2 once RAN1 reaches the agreement for the above proposal and kindly asks RAN2 and RAN3 to determine if and what specification enhancements are needed in the relevant WGs to enable reference devices for positioning.

In this contribution we provide our view on RX/TX TEG(s) association information reporting for the DL + UL (Multi-RTT) positioning method and provide further details on the reference device definition and reporting information.

Our view on other NR positioning enhancements including the multipath/NLOS mitigation techniques can be found in our companion contributions [3]-[7].

# Accuracy Improvements by Mitigating UE and gNB RX/TX Timing Delays

## RX/TX TEG(s) Association Information Reporting for DL+UL Positioning

The RX/TX TEG(s) association information signalling for the DL+UL positioning was discussed at the previous meeting to facilitate mitigating of UE/TRP RX/TX timing errors. The following agreement has been captured for the UE Rx-Tx time difference measurement association with the RX/TX TEGs in the chairman’s notes, [1]:

Agreement: For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF, where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS; - Alt.2: Support a UE to provide the association information of a UE Rx-Tx time difference measurement with a UE RxTx TEG to LMF according to the one of the 2 following options: - Option 1: the UE RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether UE provides the association information of DL PRS resources to UE Rx TEG to LMF for UE RxTx measurements specifically - Option 2: the UE RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the DL PRS and the Tx TEG is used to transmit the UL Positioning SRS. - For both alternatives, the UE may provide the association information of SRS resources for positioning to UE Tx TEG to LMF - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF - FFS: the details of the signaling, procedures, and UE capability

Similar agreement has been captured for the gNB Rx-Tx time difference measurement association with the RX/TX TEGs in the chairman’s notes, [1]:

Agreement: - For mitigating UE/TRP Tx/Rx timing errors for DL+UL positioning, support one of the following alternatives: - Alt.1: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a pair of {Rx TEG, Tx TEG} to LMF - Alt. 2: Support a gNB to provide the association information of a gNB Rx-Tx time difference measurement with a TRP RxTx TEG to LMF, if the TRP has multiple RxTx TEGs, according to the one of the 2 following options: - Option 1: the TRP RxTx TEG is associated with one or more {DL PRS resource, UL Positioning SRS resource} pairs - FFS: whether gNB provides the association information of UL Positioning SRS resources to TRP Rx TEG to LMF, if the TRP has multiple Rx TEGs, for gNB RxTx measurements specifically - Option 2: the TRP RxTx TEG is associated with one or more {Rx TEG, Tx TEG} pairs where the Rx TEG is used to receive the UL Positioning SRS and the Tx TEG is used to transmit the DL PRS. - For both alternatives, the gNB may provide the association information of DL PRS resources to TRP Tx TEG to LMF if the TRP has multiple Tx TEGs. - FFS: the details of the signaling, procedures

There are two alternatives to support TX and RX TEGs association with the UE Rx-Tx and gNB Rx-Tx time difference measurements.

The Alt. 1 is a straightforward solution introducing the TX and RX TEG IDs associated with the UE Rx-Tx and gNB Rx-Tx time difference measurements. This solution is consistent to ones, previously introduced for the DL RSTD and UL RTOA measurements reporting.

The Alt. 2 introduces a new type of the TRP and UE RxTx TEG ID associated with the gNB Rx-Tx and UE Rx-Tx time difference measurement, respectively.

In Option 1 of Alt. 2, the RxTx TEG ID is associated with one or more {DL PRS, UL Positioning SRS} pairs. This option is unclear to us, since we believe that the RX TEG ID should be associated with the measurement and not with the transmission resource.

Option 2 of Alt. 2 just introduces additional mapping between the RxTx TEG ID and the {TX TEG ID, RX TEG ID} pairs, which is redundant in our opinion.

Moreover, if TX TEG ID is associated with the DL PRS resources and/or with the UL Positioning SRS resources, then the reporting of the TX TEG ID is not required.

Based on the above considerations, we think that Alt. 1 is a preferred option with possible simplification of the RX TEG ID reporting only, if association of the TX TEG ID with the DL PRS and/or UL Positioning SRS resources is supported.

  - For mitigating UE/gNB RX/TX timing errors for the DL+UL positioning, support the following:

    - Support a UE to provide the association information of a pair of {TX TEG ID, RX TEG ID} with a UE Rx-Tx time difference measurement to LMF, where TX TEG ID is used to transmit the UL Positioning SRS and RX TEG ID is used to receive the DL PRS

    - The UE may provide the association information of the UE TX TEG ID with the UL Positioning SRS resources to LMF, if the UE has multiple TX TEGs

      - Note: if association information of the TX TEG ID with the UL Positioning SRS resources is provided, then a UE may report the RX TEG ID only associated with the UE Rx-Tx time difference measurement

  - For mitigating UE/gNB RX/TX timing errors for the DL+UL positioning, support the following:

    - Support a gNB to provide the association information of a pair of {TX TEG ID, RX TEG ID} with a gNB Rx-Tx time difference measurement to LMF, where TX TEG ID is used to transmit the DL PRS and RX TEG ID is used to receive the UL Positioning SRS

    - The gNB may provide the association information of the TRP TX TEG ID with the DL PRS resources to LMF, if the TRP has multiple TX TEGs

      - Note: if association information of the TX TEG ID with the DL PRS resources is provided, then a gNB may report the RX TEG ID only associated with the gNB Rx-Tx time difference measurement

## Reference Device Information Reporting

The concept of reference device and its definition was discussed at the previous meeting but was not agreed yet. The following proposal was captured in the FL’s summary, [2]:

Proposal: - RAN1 has evaluated the use of reference devices, which can either be UE or TRP, for positioning and observes improvements in using reference devices for enhancing the positioning performance. - Note 1: The position of the reference device is known - Note 2: If the device is a TRP, it needs at least to support some of the Rel-16 positioning functionalities, which will be defined by RAN2. For example, the device positioning functionalities may include, but not limited to, the following: - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) - Transmit the UL SRS signals for positioning - Note 3: If the device is a UE, it may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the device is known, the information may also be requested by the LMF - Note 4: The impact on the specification, the measurement reports, and the procedure for supporting a UE/TRP to be a reference device will be determined by RAN2/RAN3/SA2 - Note 5: Up to RAN2/RAN3 discussions what type(s) of UE/TRP can be reference devices and any capabilities if/as needed - Note 6: RAN1 has not identified specification enhancements needed in RAN1 specifications - Send an LS to RAN2/RAN3/SA2 once RAN1 reaches the agreement for the above proposal and kindly asks RAN2 and RAN3 to determine if and what specification enhancements are needed in the relevant WGs to enable reference devices for positioning.

There are two alternatives to define a reference device summarized in the proposal above.

The first alternative refers to the case when reference device is a UE which has known coordinate and possibly known antenna orientation in space. It can be used to calibrate the gNB/TRP TX and RX timing errors based on the UL RTOA, DL RSTD, gNB Rx-Tx, and UE Rx-Tx time measurements reported to LMF. The solution with a reference UE may benefit from reusing of the UL SRS and DL PRS resource structures already defined in the specification.

The second alternative refers to the case when reference device is a TRP with extended functionalities, possibly including, but not limited to reporting of the DL RSTD as well as the transmitting of the UL SRS for positioning.

We believe that the second alternative is more challenging and requires significant specification change, for example, to support UL SRS transmission and perform measurements using the UL SRS resources.

  - Support solution, where reference device is a UE, which may provide the following information based on the extended capabilities:

    - It may be requested by LMF to provide its own known location coordinate information to LMF

    - It may be requested by LMF to provide its antenna orientation information to LMF, if this information is available

If TRP is considered as a reference device, then the calibration procedure itself can be left up to implementation and only reporting of the timing measurements can be supported from gNB/TRP to LMF, if necessary.

To mitigate gNB RX/TX timing errors at LMF side, the inter-TRP time measurements can be supported

A reference UE is required to provide its highly accurate 3D coordinate to the LMF, for this purpose an existing IEs can be used. For example, following existing IEs [TS 37.355 LPP] can be reused without any modification with assumption that the provided data is highly confident:

Ellipsoid point with altitude and uncertainty ellipsoid IE

High accuracy ellipsoid point with altitude and uncertainty ellipsoid

Other existing IEs describing the UE location can be reused with modifications:

Ellipsoid point IE with altitude information and indication of highly accurate coordinate estimation

Ellipsoid point with uncertainty circle IE with zero level of uncertainty and altitude information

Ellipsoid point with uncertainty ellipse IE with zero level of uncertainty and altitude information

Ellipsoid arc IE with zero level of uncertainty and altitude information

High accuracy ellipsoid point with uncertainty ellipse with zero level of uncertainty and altitude information

Based on the above considerations, we have the following proposals:

  - Continue discussion on reporting format of the precisely known reference UE location coordinates from UE to LMF and whether additional indication/signaling is needed so that LMF can distinguish reference and basic UEs

    - FFS: the details of the signaling, procedures

  - Specify reporting format of the reference UE antenna orientation in space from UE to LMF

    - FFS: the details of the signaling, procedures

The multi-TEG gNB/TRP calibration procedure is performed in assumption that all links are the LOS links (i.e. LOS centric calibration solution). If one of the used TX/RX links in the calibration procedure is an NLOS link, then in addition to the timing errors (related to implementation), it might introduce an excess propagation delay caused by NLOS propagation phenomenon.

This issue is specifically important if reference device is defined as a reference UE. In that case, it may be located at the lower height (compared to the gNB/TRP) and therefore experience an NLOS propagation.

To avoid the NLOS excess propagation delay estimation during calibration procedure and introduction of additional bias into the timing error estimate, the reference UE and/or gNB/TRP needs to inform LMF, that the used link is essentially an NLOS link.

We propose that a reference UE and gNB/TRP can report to LMF the LOS/NLOS indicator associated with the UL RTOA, DL RSTD, gNB Rx-Tx, and UE Rx-Tx time difference measurements. For more details, please see our companion contribution on multipath/NLOS mitigation, [5].

Based on the above considerations, we have the following proposals:

  - Support introduction of the LOS/NLOS indicator associated with the DL RSTD and UE Rx-Tx time difference measurements

  - Support introduction of the LOS/NLOS indicator associated with the UL RTOA and gNB Rx-Tx time difference measurements

Conclusions

In this contribution we provided our view on RX/TX TEG(s) association information reporting for the DL + UL (Multi-RTT) positioning method and provided further details on the reference device definition and reporting information. In summary, we have the following list of proposals:

  - For mitigating UE/gNB RX/TX timing errors for the DL+UL positioning, support the following:

    - Support a UE to provide the association information of a pair of {TX TEG ID, RX TEG ID} with a UE Rx-Tx time difference measurement to LMF, where TX TEG ID is used to transmit the UL Positioning SRS and RX TEG ID is used to receive the DL PRS

    - The UE may provide the association information of the UE TX TEG ID with the UL Positioning SRS resources to LMF, if the UE has multiple TX TEGs

      - Note: if association information of the TX TEG ID with the UL Positioning SRS resources is provided, then a UE may report the RX TEG ID only associated with the UE Rx-Tx time difference measurement

  - For mitigating UE/gNB RX/TX timing errors for the DL+UL positioning, support the following:

    - Support a gNB to provide the association information of a pair of {TX TEG ID, RX TEG ID} with a gNB Rx-Tx time difference measurement to LMF, where TX TEG ID is used to transmit the DL PRS and RX TEG ID is used to receive the UL Positioning SRS

    - The gNB may provide the association information of the TRP TX TEG ID with the DL PRS resources to LMF, if the TRP has multiple TX TEGs

      - Note: if association information of the TX TEG ID with the DL PRS resources is provided, then a gNB may report the RX TEG ID only associated with the gNB Rx-Tx time difference measurement

  - Support solution, where reference device is a UE, which may provide the following information based on the extended capabilities:

    - It may be requested by LMF to provide its own known location coordinate information to LMF

    - It may be requested by LMF to provide its antenna orientation information to LMF, if this information is available

  - Continue discussion on reporting format of the precisely known reference UE location coordinates from UE to LMF and whether additional indication/signaling is needed so that LMF can distinguish reference and basic UEs

    - FFS: the details of the signaling, procedures

  - Specify reporting format of the reference UE antenna orientation in space from UE to LMF

    - FFS: the details of the signaling, procedures

  - Support introduction of the LOS/NLOS indicator associated with the UE DL RSTD and UE Rx-Tx time difference measurements

  - Support introduction of the LOS/NLOS indicator associated with the gNB UL RTOA and gNB Rx-Tx time difference measurements

References

- 3GPP RAN1 “Chairman's Notes RAN1#104b-e 8.5,” April 12 – 20, 2021

- R1- 2103992 “FL Summary #4 for accuracy improvements by mitigating UE RxTx and gNB RxTx timing delays_v042_Intel_FL,” April 12 – 20, 2021

- R1-2104906	NR Positioning UL-AoA Enhancements, Intel Corporation, May 2021

- R1-2104907	NR Positioning DL-AoD Enhancements, Intel Corporation, May 2021

- R1-2104908	NR Positioning Latency Reduction, Intel Corporation, May 2021

- R1-2104909	Mitigation of NLOS Problem for NR Positioning, Intel Corporation, May 2021

- R1-2104910	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state, Intel Corporation, May 2021