3GPP TSG- RAN WG1 Meeting #105-e                    R1-2105098

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.3.1.2

Source:	Apple Inc.

Title:	Views on eIIoT/URLLC CSI feedback enhancements

Document for:	Discussion/Decision

# Introduction

At RAN1 #104bis-e, the following were agreed:

Agreements:

For new reporting Case 2, focus study on reporting of delta-CQI/MCS (Case 2-3):

- Note: this delta-CQI/MCS is determined based on UE implementation (for example, using SINR, LLR, raw BER, flipped bits, LDPC iterations, BLEP, # fail parity checks, etc.)

  - Companies are encouraged to provide more details in their analysis

- FFS: Granularity of new report type (e.g. units of CQI or MCS, how many bits)

- FFS: Whether quantity reported is relative to the scheduled MCS

Agreement: Focus study on the following for new reporting Case 1:

- Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection.

  - Downselect by RAN1#105 to at most a single method from the following options:

    - Mean-CQI/SINR and stdev-CQI/SINR (FFS details)

    - CSI based on worst IMR occasion (FFS details)

    - Interference standard deviation (FFS details)

    - Worst-M CQI (FFS details)

  - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type

- Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

  - Applicable for same reporting quantity as R16 for CQI.

  - FFS: Whether network configured channel and interference measurement interval can also be applied

  - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated

  - FFS: how to report the updated CQI

In this contribution we provide our initial views on those enhancements.

# Discussion on Case 2 enhancements

On Case 2 enhancements, the resulted CSI payload depends on the PDSCH decoding outcome: if the PDSCH is decoded successfully with a high margin, then a simple ACK is enough, on the other hand, if the PDSCH decoding is not successful or succeeds with a low margin, beyond HARQ-ACK, additional information is required.

First, we observe Case 2 feedback can be treated as something between HARQ-ACK feedback and CSI feedback.

Figure 1 Soft NACK/ACK schemes as example for Case 2 enhancement

Assume there can be up to four states for PDSCH decoding outcome: high ACK, low ACK, low NACK, high NACK as shown in Figure 1. With high ACK, PDSCH decoding succeeds with a high margin, little or no additional feedback information is generated; with low ACK, PDSCH decoding succeeds with a low margin, additional feedback information is generated such as (delta CQI/MCS, equivalent CQI/MCS for the reception up to now, etc); with low NACK, PDSCH decoding fails with a low margin (PDSCH decoding almost succeeds), additional feedback information is generated such as (delta CQI/MCS, equivalent CQI/MCS for the reception up to now, etc); with high NACK, PDSCH decoding fails with a high margin (PDSCH decoding fails miserably), additional feedback information is generated such as (delta CQI/MCS, equivalent CQI/MCS for the reception up to now, etc).

In general, the Case 2 feedback payload can be small for some code states, and it can be large for other cases.

First we consider the case where the Case 2 feedback is carried over PUCCH, and we have a similar consideration for PUSCH. Due to the uncertainty in the feedback payload, in general, we want to divide the UCI payload into two parts, in the first part (UCI part I), a fixed payload is carried irrespective of PDSCH decoding outcome; in the second part (UCI part II), additional feedback information may be carried according to the payload content in the first part.

We can consider the following scheme:

a modified HARQ-ACK is carried in UCI part I, ”1” for high ACK, ”0” for the other cases (note in this case low ACK will be represented by ”0” in UCI part I). And delta MCS/CQI is present in in UCI part II, and further information about HARQ-ACK is included in UCI part II.

high ACK: UCI part I:  ”1”, UCI part II: empty

low ACK: UCI part I: ”0”, UCI part II, delta MCS/CQI value in range 1, or a code state for low ACK and delta MCS/CQI

low NACK: UCI part I: ”0”, UCI part II, delta MCS/CQI value in range

2, or a code state for low NACK and delta MCS/CQI

high NACK: UCI part I: ”0”, UCI part II, delta MCS/CQI value in range 3, or a code state for high NACK and delta MCS/CQI

the delta MCS/CQI ranges can be RRC configured, MAC CE indicated, or specified.  For example,

there are n1 levels for delta MCS/CQI  or MCS/CQI for low ACK

there are n2 levels for delta MCS/CQI  or MCS/CQI for low NACK

there are n3 levels for delta MCS/CQI  or MCS/CQI for high NACK

then log2(n1+n2+n3) bits can be used to indicate one code state out of n1+n2+n3 code states.

In this case, UCI part I provides a bitmap: for each PDSCH with a single transport block, “1” means the PDSCH is received correctly and no further information is provided; “0” means delta CQI/MCS is provided for the corresponding PDSCH, which may include further information.

We have

Proposal 2-1:  If Case 2 enhancement feedback is supported, the feedback is as part of enhanced HARQ codebook.

# Discussion on Case 1 enhancements

Interference can change at a faster pace than the desired signal. The variation of the desired signal strength can be due to UE mobility and/or updated beamforming due to updated CSI feedback. The variation in interference can be caused by scheduled transmissions in adjacent cells, e.g. with mini-slot transmissions at adjacent cells for different UEs, the interference can change quickly even within a single slot. To characteristics in the fluctuation in CSI, approaches were proposed in previous meetings. The following was agreed in RAN1 #104bis-e:

Agreement: Focus study on the following for new reporting Case 1:

- Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection.

  - Downselect by RAN1#105 to at most a single method from the following options:

    - Mean-CQI/SINR and stdev-CQI/SINR (FFS details)

    - CSI based on worst IMR occasion (FFS details)

    - Interference standard deviation (FFS details)

    - Worst-M CQI (FFS details)

  - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type

- Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

  - Applicable for same reporting quantity as R16 for CQI.

  - FFS: Whether network configured channel and interference measurement interval can also be applied

  - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated

  - FFS: how to report the updated CQI

In general, generating CQI/SINR statistics can be more involved than generating statistics for interference alone. Between CQI statistics and SINR statistics, the CQI statistics provides feedback can be directly used by gNB in for MCS selection if the CQI statistics is assumed for a distribution actually experienced by the UE, as CQI is tied to the hypothetical BLER with which the CQI is calculated. Mean and standard deviation can quantify the tail behavior of the Gaussian distribution well. It can be expected if feedback with mean and standard deviation for CQI/SINR is specified, some quantization schemes need to be specified.  Other distributions, such as Gamma distribution, truncated Gaussian distribution, or lognormal distribution, can have rather different tail behaviors, and the case is  still to be made in terms of the selection of distribution, it is not clear mean and standard deviation still are the right quantities for feedback.

Compared with CQI/SINR statistics, SINR statistics does not lead to a direct use at the gNB side, as mapping from SINR to CQI may be nontrivial, and testability issue for SINR statistics needs to be addressed.  If CQI statistics, SINR statistics or interference statistics is to be specified, then a reference CSI reporting needs to be reported by the UE:

- With CQI statistics, a reference CSI reporting with a report quantity with CQI included in its report and the associated SINR should be reported in the new report type:

  - CQI statistics for the assumed distribution

  - A reference CSI report with reportQuantity from {cri-RI-PMI-CQI, cri-RI-i1-CQI, cri-RI-CQI, cri-RI-LI-PMI-CQI}

  - The CQI associated with the reference CSI report

- With SINR statistics, a reference CSI reporting with a report quantity with CQI included in its report and the associated SINR should be reported in the new report type:

  - SINR statistics for the assumed distribution

  - A reference CSI report with reportQuantity from {cri-RI-PMI-CQI, cri-RI-i1-CQI, cri-RI-CQI, cri-RI-LI-PMI-CQI}

  - The SINR associated with the reference CSI report

- With interference statistics, a reference CSI reporting with a report quantity with CQI included in its report and the associated interference should be reported in the new report type:

  - interference statistics for the assumed distribution

  - A reference CSI report with reportQuantity from {cri-RI-PMI-CQI, cri-RI-i1-CQI, cri-RI-CQI, cri-RI-LI-PMI-CQI}

  - The measured/assumed interference associated with the reference CSI report

We have

Proposal 3-1: a reference CSI report is configured for reporting with Case 1 enhancements (if supported).

For URLLC traffic, wideband scheduling can be used to mitigate frequency selective fading and/or interference. It seems worst-M CQI, worst-Best CQI, etc can be beneficial if localized scheduling (e.g. several clusters of resources in the BWP), and interference profile over those clusters does not change from CSI feedback to PDSCH scheduling. It is not clear such stationarity for interference can be safely assumed. If some network coordination for cells can be used to make interference more predictable, e.g. with fractional frequency use following certain reuse patterns across cells, then CSI measurement with reportFreqConfiguration can be already used, assume there are 12 subbands are present in the active BWP, a bitmap with subbands12 such as [110011…1] can be used from Rel-15 to limit the subbands where the UE will report the CQI. Thus the benefits of those schemes are not clear.

As for finer differential CQI granularity, a fundamental fact is that while there are 5 bits for MCS levels and there are only 4 bits for CQI; some kind of OLLA adaption is inevitable. Also to justify the finer differential CQI granularity, the stationarity of interference/CQI is also implicitly assumed, the question raised for worst-M CQI can be asked here also. From these considerations, the benefits with such schemes are also not clear.

We have

Proposal 3-2: Clarify the assumption on stationarity of interference for worst-M CQI/finer granularity for sub-band CQI.

# 3. Conclusions

In this contribution we share our views on HARQ/CSI-RS enhancements. We have

Proposal 1:  If Case 2 enhancement  feedback is supported, the feedback is as part of enhanced HARQ codebook.

Proposal 3-1: a reference CSI report is configured for reporting with Case 1 enhancements (if supported).

Proposal 3-2: Clarify the assumption on stationarity of interference for worst-M CQI/finer granularity for sub-band CQI.

# References

- RP-193233, “New WID on enhanced Industrial Internet of Things (IoT) and URLLC support”, Nokia, Nokia Shanghai Bell, RAN #86, December 2019

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, RAN #88e, June 2020

- R1-2006515, “CSI feedback enhancements for URLLC”, Apple, RAN1 #102-e, August 2020.