3GPP TSG RAN WG1 #105-e	R1-2105142

e-Meeting, May 10th – 27th, 2021

Agenda item:		8.3.2

Source:		Nokia, Nokia Shanghai Bell

Title:	Further UL enhancements for IIoT/URLLC in unlicensed controlled environment

Document for:		Discussion and Decision

# 1	Introduction

The revised work item description for “Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” was agreed at RAN#88-e [1].

Specifically, the objectives regarding the support of unlicensed operation on FR1, considering controlled environments, were defined as follows:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, we present our views about the UL enhancements for URLLC in unlicensed controlled environments. We build on top of the agreements and discussions from meeting RAN1#104bis-e which are summarized in [2]. The following agreements were reached in the meeting:

Support for UE-initiated COT for FBE Agreements: - Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode. Agreements: - For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity. Agreement: - For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell: - Do not support any additional period value Agreement: - For semi-static channel access mode, the starting point of first UE FFP for a serving cell - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement). Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The DL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Conclusion: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI. UL CG harmonization Agreement: - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance Agreements - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details Agreements - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details Agreement: - Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

We focus specifically on the items defined for the agenda of this meeting, i.e., only to handle topics of CG harmonization and decision between Alt-a/Alt-b for semi-static channel access mode when a UE can operate as UE-initiated COT for both CG and DG (see sections 2.3/2.4/2.5 of [4]). In Section 2.1 we discuss CG harmonization, while in Section 2.2 we discuss the support for UE-initiated COT for FBE.

# 2	UL enhancements for URLLC in unlicensed band

## 2.1 NR/NR-U CG enhancements harmonization

The assumption for the WI is that the operation on unlicensed spectrum occurs in a controlled environment, i.e. where interference from other systems is not expected. In that sense, as observed in [3] the possibility of LBT failure would depend on the deployment. If in a controlled facility it can be ensured that LBT failures are negligible, then NR IIoT based CG (defined primarily for licensed spectrum use) can potentially be employed without NR-U specific enhancements. While if the effect of potential LBT failures are considered, due to intra-system interference or sporadic interference from other RAT, it is beneficial for unlicensed IIoT/URLLC to use the NR-U CG mechanisms, which were designed to overcome the impact of frequent LBT failures.

Time domain consecutive allocations and PUSCH repetitions

There are clear differences between the types of time domain resource allocations supported in NR-U CG and Rel-16 IoT:

- NR-U introduced support for an allocation of a burst of consecutive slots (cg-nrofSlots) and multiple CG PUSCH resources within a slot (cg-nrofPUSCH-InSlot) configured by RRC,

- Rel-16 for IIoT introduced a support for PUSCH repetition Type B for intra-slot repetition including actual repetitions across slot boundary.

It is worth noting that repetitions are already supported in Rel-16 NR-U for consecutive PUSCH allocations as defined as part of Type A repetition specification for CG in 38.214 clause 6.1.2.3.1. I.e., if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration.

Observation 1: Back-to-back PUSCH repetitions are already supported with NR-U as part of Type A repetitions.

Non-back-to-back Type A repetitions (slot-level repetition) will cause transmission gaps between two repetitions. Additional LBT operation may be required for the UE to resume transmission after a gap. Additional LBT overhead and uncertain LBT outcome make such operation (i.e., non-back-to-back Type A repetition) being not that attractive in unlicensed band.

Proposal 1: Non-back-to-back Type A repetitions are not supported in unlicensed band.

As agreed in RAN1#104-e, PUSCH repetitions Type B should be supported in unlicensed band when using NR IIoT Rel-16 based CG. In our view, that also means that Type B repetitions should not be combined with NR-U multi-PUSCH allocations. So, we don’t see any additional enhancement needed for multi-PUSCH allocations, which should be only used if NR-U based CG is used, i.e. including CG-UCI.

Proposal 2: The use of PUSCH repetition Type B together with NR-U based multi-slot allocations should not be considered.

Enhancements for PUSCH repetition type B

It was discussed in the previous meeting whether PUSCH segmentation should take into account the idle period of an FFP. In our understanding, that can be handled with minimal specification impact, by defining the symbols overlapping with the idle period of an FFP where the UE is not allowed to transmit as an invalid symbol. The PUSCH segmentation behavior around the invalid symbol should follow what was already defined in Rel-16. The resuming of the actual repetitions after the idle period may be subject to the outcome of LBT made during the idle period interval.

Proposal 3: For PUSCH repetition type B, symbols overlapping with an idle period of an FFP where the UE is not allowed to transmit should be specified as invalid symbol.

In previous meeting it was agreed to further study whether orphan symbol(s) are transmitted if they are between two PUSCH repetitions type B that are transmitted. As a short background about “orphan symbol”, it was specified for URLLC CG in Rel-16 that in case of an actual repetition segmentation results in a 1-symbol segment, that repetition transmission should be dropped (i.e. not transmitted). In unlicensed operation, that results in a gap which can cause a transmitting UE to miss the COT if another initiating device acquires the COT during the gap. Therefore, we think it would be beneficial to allow transmission during the orphan symbol in unlicensed band, for the sake of reliability of an ongoing PUSCH type B repetition.

A cyclic prefix extension can be used to fill the orphan symbol. The extended cyclic prefix should be of one full symbol duration, prior to the actual transmission located after the orphan symbol, i.e. the symbol following the orphan symbol is copied in the place of orphan symbol. The specification change can be based on the CP extension design defined for NR-U in Rel-16.

Proposal 4: A 1-symbol cyclic prefix extension (i.e. repetition of the following symbol) can be transmitted for filling the orphan symbol(s) gap that is between two actual repetitions that are transmitted, in order to avoid missing a COT during a type B repetition to other initiating device.

Operation with/without CG retransmission timer

RAN2 reached the following agreements related to CG retransmission timer in #112-e meeting:

2	cg-RetransmissionTimer can be configured optionally for shared spectrum

3	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

4	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

5	As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U.

6	HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured.

As agreed by RAN2, configuration of (cg-RetransmissionTimer) can be optional on shared (i.e. unlicensed) spectrum. In case cg-RetransmissionTimer is not configured, at least autonomous retransmission and explicit ACK by DFI should clearly not be used for NR-U CG.

In spite of the RAN2 agreement, it is not yet clear whether other NR-U CG functionalities can be used when cg-RetransmissionTimer is not configured. Such functionalities include CG-UCI (including COT sharing), NR-U CG specific time domain resource allocation, etc. In our view, in deployments where LBT failures are not considered so that cg-RetransmissionTimer is not configured, other NR-U enhancements to overcome LBT failures are also not needed, and NR IIoT based CG can be adopted. It is up to RAN2 to decide which parameter should be used for determining if NR-U CG or NR IIOT CG procedure is to be followed, while in our view a single parameter such as cg-RetransmissionTimer is enough.

Following the suggested table for discussion by the moderator in previous meeting, and the remaining alternatives agreed in RAN1#104-bis-e (i.e. only Option 1 and Option 2a to be considered),  we summarize how each option would work, as well as potential issues and specification impact:

(Note: 0 means the feature is disabled/not configured, 1 means enabled/configured)

|  | CG-UCI | CG-DFI | cg-RetransmissionTimer-r16 | Comments |
| --- | --- | --- | --- | --- |
| IIoT like mechanism: option 1 | 0 | 0 | 0 | NR-U CG specific features disabled by RRC configuration, i.e., Rel-16 URLLC CG operation mode is used in unlicensed spectrum including LBT procedure according regulation, though the effect of LBT failures is assumed negligible. Minor RAN1 specification impact, if any. |
| Opt-2a: | 0 | 1 | 1 | Based on RAN2 agreement, when cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. Since CG-UCI is disabled there is no way for gNB to know which HARQ ID and RV the UE has selected, at least not without major specification impact. |
| Opt-2a: | 1 | 0 | 0 | According RAN2 agreement, when cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection. Therefore, the HARQ related fields in CG-UCI have no use in this option. While the remaining benefit of UE COT information is questionable if LBT failures are assumed negligible, since it should be expected that gNB can easily gain access the channel itself in LBE. And in FBE, the gNB configures the UE FFP so it knows the COT size already. |
| NR-U like mechanism: option 1 | 1 | 1 | 1 | NR-U CG specific features enabled by RRC configuration, i.e., NR-U based CG should be followed including features to overcome sporadic LBT issues. Minor RAN1 specification impact, if any. |

It was mentioned by some companies in previous meeting, that CG-UCI is useful e.g. to reduce latency when the traffic is not aligned with the CG-PUSCH. However, the benefit and use case of that is not clear to us, since such issue could be avoided either by assigning multiple CG configurations to a LCH, or simply by adjusting/increasing the periodicity of the CG.

The remaining benefit of CG-UCI would be for UE COT sharing information, however that benefit is also questionable in the unlicensed controlled environment. If LBT failures are considered negligible, it should be expected that gNB can easily gain access to the channel itself in LBE. While in FBE, the gNB configures the UE FFPs, and therefore it should know the COT duration of a UE initiating the transmitting. If LBT failures are not negligible, enabling CG-UCI alone cannot be justified while disabling the other NR-U enhancements as CG-DFI procedures.

Some companies also mentioned that CG-DFI could be used for early termination of repetitions. However, in our view that the feature was not designed for early termination. The processing time for using it for that purpose is a bottleneck, i.e. it is very challenging to process the data and deliver a CG-DFI before most of the repetitions are done (assuming that URLLC mini-slot repetitions need to be fast, otherwise one would use dynamic grant). In addition, early termination was not defined for NR URLLC CG, so it is not clear why that should be optimized for NR-U in the scope of harmonization.

Based on the discussion above, we emphasize our support for Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

Proposal 5: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter (Option 1  from RAN1#103-e), i.e. when cg-RetransmissionTimer is not configured, NR-U specific CG features such as CG-UCI, COT sharing indication, UE-selected HARQ process ID and RV, autonomous retransmission, consecutive allocations for different TB and CG-DFI are not supported.

CG-UCI and PUCCH multiplexing and prioritization

Rel-16 NR-U CG features include CG-UCI multiplexing with PUCCH on the CG-PUSCH, when a PUCCH carrying HARQ-ACK overlaps with the CG-PUSCH within a PUCCH group, if the higher layer parameter cg-UCI-Multiplexing is configured. Otherwise, if that parameter is not configured and the overlap occurs, the CG-PUSCH is skipped, i.e. the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information on a PUCCH transmission or on another PUSCH transmission according TS 38.213.

Rel-16 IIoT CG features include UCI enhancements which allow two HARQ-ACK codebooks of different priorities to be constructed. Separate PUCCH configurations for each priority are provided to the UE, and the priority is indicated in the scheduling DCI or configured. For CG the priority is configured by the higher layer parameter phy-PriorityIndex. If two physical channels of different priority, e.g. PUCCH carrying HARQ-ACK and CG-PUSCH, overlap in time, the UE should transmit the one of higher priority. On the other hand, if the channels have the same priority, the UE should multiplex the information, e.g. HARQ-ACK is multiplexed on PUSCH.

For Rel-17 IIoT, the PHY layer multiplexing of channels of different priorities is being discussed under Agenda Item 8.3.3 . In RAN1#102-e meeting, different multiplexing scenarios were agreed including the multiplexing of HARQ-ACK and PUSCH with/without UCI. In addition, the following FFS item was also listed, which may relate to NR-U CG:

Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). - Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only) - Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. - Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. For the above multiplexing scenarios, - Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations. - FFS for other separate configurations. - FFS: value range of beta-offset (e.g. <1). - FFS the conditions, if needed, for multiplexing, e.g. - FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling)? - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding). - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). - How to handle multiplexing of UCI of different priorities and CG-UCI in a CG-PUSCH

We see that PHY multiplexing/prioritization based on priority index introduced in Rel-16 for licensed spectrum should also be considered in the case of unlicensed spectrum operation, including the case when CG-UCI is present and NR-U specific CG enhancements are used for deployments where sporadic LBT issues are non-negligible for IIOT. The exact UE behavior for UCI multiplexing/prioritization in the presence of CG-UCI is FFS.

Proposal 6: PHY multiplexing/prioritization introduced in Rel-16 is supported also with NR-U CG. Interaction of CG-UCI and HARQ-ACK codebooks of different priorities is FFS.

## 2.2 On the support for UE-initiated COT for FBE

### 2.2.1 COT-initiator determination

#### 2.2.1.1 Scheduled UL transmission

With respect to determination of the COT initiator for scheduled UL transmissions, the following alternatives were down-selected during RAN1meeting #104-e meeting:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

These alternatives were further discussed in RAN1 meeting #104bis-e with neither conclusion nor agreement.

In our opinion, standardized means for determination of the COT initiator for UL transmissions with semi-static channel access should be very similar to standardized means for determination of the LBT type for UL transmission with dynamic channel access. For scheduled UL transmissions, the gNB indicates the COT initiator in the scheduling DCI, i.e. Alt-a. Alt-a adds robustness to Alt-b with neither additional complexity for the UE nor increased signaling overhead. The ChannelAccess-CPext field has already been separately defined in Rel-16 for FBE for both fallback DCI and non-fallback DCI. Three values of the ChannelAccess-CPext field have been specified. One spare value can be used to signal “UE as COT initiator”. One codepoint is sufficient as there is neither need to signal multiple LBT types nor CP extensions.

Proposal 7: Alt-a is used for determination of the COT initiator for intra-FFP scheduled UL transmissions.

Regarding the first FFS in Alt-a, the following options were discussed in RAN1meeting #104bis-e meeting:

- Option 1: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

- Option 2: Study whether to support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

The need for channel access related fields in DCI 0_2 and 1_2 was discussed during Rel-16 NR-U design, but due to lack of time, was not introduced in the end. We see that the compact DCi format can also be useful on unlicensed spectrum and propose that both DCI 0_2 and 1_2 support channel access related fields, with both dynamic and semi-static channel access.

Proposal 8: The field(s) indicating the COT initiator in the scheduling DCI cannot be absent. Channel access related fields in Rel-16 DCI 0_1 and 1_1 are included in Rel-17 DCI 0_2 and 1_2, respectively and supported with both dynamic as well as semi-static channel access.

Regarding the second FFS, with the following agreement from RAN1 meeting #104bis-e, the gNB scheduling an UL transmission in the next gNB’s FFP is now allowed.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

However, how to handle cross-FFP scheduling is still FFS. For example, it is yet to be discussed whether the gNB may indicate either the UE or the gNB as COT indicator in the DCI. In case the gNB signals the UE as COT initiator, the UE may still assume the gNB as COT initiator if it detects that the gNB has initiated a new COT prior to the scheduled UL transmission. If the gNB signals the gNB as COT initiator, the UE may still need to check if the gNB has initiated a new COT before it can initiate transmission. In the latter case, if the UE does not detect a gNB COT, it should be discussed if the UE is still allowed to transmit assuming UE initiated COT (possibly using modified PUSCH starting position to align the UL transmission with start of the UE FFP) or the UL transmission should be dropped.  The impact of the UE processing delay in the detection of the gNB COT based on signals transmitted by the gNB at the start of the gNB FFP should also be considered.

Proposal 9: Use a combination of Alt-a and Alt-b for determination of the COT initiator for cross-FFP scheduled UL transmissions. The UE processing delay for gNB COT detection should also be taken into account. Details are FFS.

#### 2.2.1.2 Configured UL transmissions

With respect to determination of the COT initiator for configured UL transmissions, the following alternatives were down-selected in RAN1meeting #104-e meeting:

- Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB has initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

- Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

These alternatives were further discussed in RAN1 meeting #104bis-e with neither conclusion nor agreement.

To avoid that an NR-U system operating with semi-static channel occupancy may occupy an unlicensed channel for more than 90-95% of the time, thus potentially violating the ETSI regulatory requirements, or at least causing coexistence problems, we think that, when a serving gNB acquires a COT during a gNB FFP, all its connected UEs should operate as responding devices according to gNB-shared COT during the corresponding gNB FFP. Therefore, we prefer Alt-a above, i.e. if a configured UL transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that the gNB has initiated that gNB FFP, the UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

The main arguments for Alt-b camp are simpler UE implementation and no misalignment between UE and gNB in case of COT misdetection. On the other hand, according to Rel-15 NR and Rel-16 NR-U specifications, the gNB may always override/cancel UL CG resources using DCI 2_0. Moreover, as by #104bis-e meeting agreements, a Rel-17 UE may also need to detect DCI 2_0 to determine if symbols of the gNB idle period are configured as DL in SFI. So, even with Alt. b, an RRC connected UE shall always try to detect DCI 2_0. Therefore, the benefits of Alt-b as compared of Al-t a are not clear, and we propose to support Alt-a. As for COT determination in case of DG-PUSCH transmissions (see Section 2.2.1.1), the impact of the UE processing delay for detection of gNB COT based on signals transmitted by the gNB at the start of gNB FFP should also be considered. I.e., the UE may be allowed to transmit assuming UE-initiated COT if it does not have enough processing time to detect a gNB COT at the start of the gNB FFP.

Proposal 10: Alt-a is chosen for determination of the COT initiator for configured grant UL transmissions.

As Alt-a is currently formulated in the agreement from RAN1 meeting #104-e, it is unclear what “otherwise” means. Does “otherwise” mean that the UE has not determined that the gNB has initiated the gNB FFP, or could it also mean that the UE has determined that the gNB has initiated the gNB FFP, but the configured UL transmission is NOT confined within the gNB FFP before the idle period of the gNB FFP (i.e. the configured UL transmission “overlaps” with the idle period of the gNB FFP)? In the latter case, we do not think the UE should assume that the configured UL transmission is to be transmitted according to UE-initiated COT, as this may result in violations of the ETSI regulatory requirements, or at least may cause coexistence issues, as discussed above. In this case, we think RAN1 should down-select one of the following alternatives:

- Alt-a1: the UE does not transmit the corresponding configured UL transmission

- Alt-a2: the UE only transmits on a subset of the configured UL (time) resources to ensure no transmissions during the idle period of that gNB FFP

We prefer Alt-a2 as it provides better utilization of the available UL resources. Based on the discussion above, we propose to capture the following proposal in the RAN1 agreements:

Proposal: In semi-static channel access mode, when a UE can be COT initiator, and a configured UL transmission is aligned with a UE FFP boundary, and ends before the idle period of that UE FFP:

- If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that the gNB has initiated that gNB FFP, the UE assumes that the configured UL transmission corresponds to gNB-initiated COT

- if the UE determines that the gNB has NOT initiated that gNB FFP, the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- if the transmission is NOT confined within a gNB FFP before the idle period of that gNB FFP (i.e. the transmission overlaps at least partially with the idle period of that gNB FFP) and the UE has already determined that the gNB has initiated that gNB FFP, the UE only transmits on a subset of the configured UL (time) resources to ensure no transmissions during the idle period of that gNB FFP.

Note: this is a slightly clarified version of Alt-a from RAN1 #104-e.

### 2.2.2 gNB-to-UE and UE-to-gNB COT sharing

With semi-static channel access in RRC CONNECTED mode, gNB-to-UE COT sharing mechanism can be based on COT signaling by the gNB using GC-PDCCH (DCI 2_0). However, according to 37.213, Section 4.3 below, a UE may not even be required to read GC-PDCCH to determine if a gNB COT is present or not. Furthermore, to support UL transmissions with semi-static channel occupancy in IDLE/INACTIVE mode, gNB-to-UE COT sharing based on detection of a DL transmission burst(s) within the gNB channel occupancy time is assumed in Rel-16 NR-U. In this case, an idle/inactive UE determines the gNB acquired COT based on existing signals (such as SSB or RMSI PDCCH) as 37.213, Section 4.3 states:

A channel occupancy initiated by a gNB and shared with UE(s) satisfies the following: - The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period. - The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16us. - The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16us. - A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows: - If the gap between the UL and DL transmission bursts is at most 16us, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel. - If the gap between the UL and DL transmission bursts is more than 16us, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us within a 25us interval ending immediately before transmission.

The problem with UE-initiated COT with semi-static channel occupancy and UE-to-gNB COT, is that detection of a DL transmission burst(s) within the gNB channel occupancy time may not imply that the gNB has initiated a COT, as the gNB may be sharing the COT initiated by another UE. As UE-to-UE COT sharing through the gNB is not supported as by RAN1 #104-e agreements, this in practice requires specification of specific constraints on the timing and type of DL signals the UE should use for gNB COT detection, which on the other hand would impose restrictions on the timing and type of DL signals that a gNB can transmit when operating as responding device. While this could work for Rel-17 UEs, this may not work for Rel-16 UEs, unless the timing and type of DL signals the UE should use for gNB COT detection is clearly specified also for Rel-16 UEs. So, without further clarifications on Rel-16 UE assumptions for gNB COT detection and sharing, in practice it may not be possible for the gNB to operate as responding device with semi-static channel access, which is a major limitation to Rel-17 enhancements.

Observation 2: With semi-static channel occupancy, the mechanism used by a legacy (Rel-16) UE to detect and share a serving gNB COT needs further clarifications if UE-to-gNB COT sharing is supported in Rel-17.

# 3	Conclusions

In this contribution, we discussed uplink enhancements for IIOT/URLLC in unlicensed band including and CG harmonization and support for UE initiated COT. Based on the discussions we have the following proposals:

NR/NR-U CG enhancements harmonization:

Observation 1: Back-to-back PUSCH repetitions are already supported with NR-U as part of Type A repetitions.

Proposal 1: Non-back-to-back Type A repetitions are not supported in unlicensed band.

Proposal 2: The use of PUSCH repetition Type B together with NR-U based multi-slot allocations should not be considered.

Proposal 3: For PUSCH repetition type B, symbols overlapping with an idle period of an FFP where the UE is not allowed to transmit should be specified as invalid symbol.

Proposal 4: A 1-symbol cyclic prefix extension (i.e. repetition of the following symbol) can be transmitted for filling the orphan symbol(s) gap that is between two actual repetitions that are transmitted, in order to avoid missing a COT during a type B repetition to other initiating device.

Proposal 5: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter (Option 1  from RAN1#103-e), i.e. when cg-RetransmissionTimer is not configured, NR-U specific CG features such as CG-UCI, COT sharing indication, UE-selected HARQ process ID and RV, autonomous retransmission, consecutive allocations for different TB and CG-DFI are not supported.

Proposal 6: PHY multiplexing/prioritization introduced in Rel-16 is supported also with NR-U CG. Interaction of CG-UCI and HARQ-ACK codebooks of different priorities is FFS.

On the support for UE-initiated COT for FBE:

Proposal 7: Alt-a is used for determination of the COT initiator for intra-FFP scheduled UL transmissions.

Proposal 8: The field(s) indicating the COT initiator in the scheduling DCI cannot be absent. Channel access related fields in Rel-16 DCI 0_1 and 1_1 are included in Rel-17 DCI 0_2 and 1_2, respectively and supported with both dynamic as well as semi-static channel access.

Proposal 9: Use a combination of Alt-a and Alt-b for determination of the COT initiator for cross-FFP scheduled UL transmissions. The UE processing delay for gNB COT detection should also be taken into account. Details are FFS.

Proposal 10: Alt-a is chosen for determination of the COT initiator for configured grant UL transmissions.

Observation 2: With semi-static channel occupancy, the mechanism used by a legacy (Rel-16) UE to detect and share a serving gNB COT needs further clarifications if UE-to-gNB COT sharing is supported in Rel-17.

# References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, TSG RAN#88-e, July 2020.

- RAN1 Chairman Notes, 3GPP TSG RAN WG1 Meeting #104b-e, April 12th – 20th, 2021

- R1-2008843, On UL intra-UE prioritization and multiplexing enhancements, Nokia, Nokia Shanghai Bell, 3GPP TSG RAN WG1 Meeting #103-e, October 2020.

- R1-2103960, Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band, Moderator (Ericsson), 3GPP TSG-RAN WG1 Meeting #104-bis-e, April 2021

# Appendix A1 – Agreements from previous meetings

Agreements from RAN1#102e:

Agreements: - For semi-static channel access mode, - If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity. - For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported. Agreements: - For semi-static channel access mode, - When gNB operates as an initiating device - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT - When a UE operates as an initiating device - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB - When the gNB shares a COT initiated by a UE during an FFP associated with the UE - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE - FFS whether/how to support additional restrictions to the idle period Agreements: - For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode - FFS the case when the UE is IDLE/INACTIVE mode Agreements: - A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst. Update on 8/26 Agreements: - At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum. Conclusion: Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision. Agreements: - Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles. Agreements: - UE-to- gNB COT sharing in semi-static channel access mode is supported. - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP. - FFS details - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP. - FFS whether/how UE to gNB COT sharing when the gap is >16us Update from 8/28 GTW Agreements: For semi-static channel access mode, o Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT. o FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT. Agreements: - For semi-static channel access mode, - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling. - FFS on to be provided by SIB-1 - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreements from RAN1#103-e:

Agreements: - In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms Conclusion: - For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB. Agreements: - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported Conclusion: If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X. If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y. Note 1: One of the devices X and Y is a UE and the other is its serving gNB. Note 2: Whether or not there is additional restriction on idle period is still FFS. Agreements: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions. Update on 11/11 Agreements: - The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s). - Note: UE initiated FBE configuration is configured per serving cell Agreements: In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT. o Note: Any value for the period, shall be at least 1ms and at most 10ms. o Note: Aim for low complexity operation to handle gNB and UE COT interactions Agreements: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements from RAN1#104-e:

Agreement: - PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG - FFS whether/how to enhance Agreement: - In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}. - FFS on other values Agreement: - In semi-static channel access mode: - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary. - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT - FFS on X (e.g. X=0, or X= even index number) Agreement: In semi-static channel access mode when a UE can operate as initiating device, - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period - Alt-b: Determination based on the rules applied for a configured UL transmission Agreement: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. Agreement: - In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.