3GPP TSG-RAN WG1 e-Meeting #105	R1- 2105207

May 10th – May 27th, 2021

Title:	Discussion on Timing Relationship Enhancements for NTN

Source: 	Fraunhofer IIS, Fraunhofer HHI

Agenda Item:	8.4.1– Timing relationship enhancements

TDoc Type:	Discussion

Document for:	Decision

# Introduction

In RAN1#104b-e, several agreements were achieved regarding the agenda item (AI) 8.4.1, timing relationship enhancement. The summary of the achieved agreements and FL recommendations are listed below:

RAN1#104b-e:

Agreement:

For updating K_offset after initial access, at least one of the following options is supported:

- Option 1: RRC reconfiguration

- Option 2: MAC CE

FFS: Other options

Agreement:

- For determination of cell-specific K_offset in system information, down-select one option from below:

  - Option 1: Signal one offset value for K_offset

    - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point

  - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values

    - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Agreement:

Confirm the following working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

Agreement:

When UE is not provided with K_offset value other than the one signaled in system information, the K_offset value signaled in system information is used for all timing relationships that require K_offset enhancement.

Agreement:

UE can be provided by network with a K_mac value.

- When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

# Discussion

In this contribution, we focus on the aspects of the signaling of Koffset during the initial access. Two options suggested in the previous meeting. Option 1, in particular, assumes that one offset value is signaled, where the offset value expects to cover the RTT of the service link plus the RTT of the satellite to the reference point (RP) with respect to which UL and DL frames are aligned. On the other hand Option 2 considers the signaling of two offset values. One offset value is expected to capture the RTT of the satellite to the reference point, while the second offset value takes the RTT of the service link into account. In the following, we focus on the implicit signaling design of Option 1 and Option 2.

## Option-1 Implicit Signaling Design

Implicit signaling design for Option-1 is based on system information parameter(s) that captures the RTT of the service link plus the RTT of the satellite to the reference point. For the case where RP is chosen to be at the gNB, the offset value must cover the RTT of the service link and feeder link together. An example of such parameters includes RRC timers T300, T301, T319, and T310. The RRC timers are configured within system information parameter “ue-TimerAndConstants” and broadcast via SIB1. Any configured value of timers T300, T301, T319, and T310 for NTN must at least take into account the end-to-end RTT of UE to gNB. In particular, the RRC timers take the following range of values:

T300/T301/T319 = { 100, 200, 300, 400, 600, 1000, 1500, 2000 }  ms, T310 = { 0, 50, 100, 200, 500, 1000, 2000 } ms. We can observe that the RRC timers can provide values greater than maximum experienced RTT in GEO and LEO satellites, i.e., 541.46 and 25.77, respectively [1]-[2]. As a result of this, the RRC timers can also be exploited for derivation of the value of initial Koffset as follows

Koffset=α×T3xx,

where T3xx is the certain value of RRC timer T3XX broadcast in SIB1, XX = {00,01,19,10}, and α is an scaling factor that can be obtained based on scenario, e.g. LEO or GEO and satellite altitudes.

Observation 1: NTN UE acquires RRC timers T300, T301, T319, and T310 before performing random access procedure.

Proposal 1: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., RRC timers T300, T301, T319, and T310.

Observation 2: RRC timers T300/301/319 are most suited for GEO satellites, while RRC timer T310 is suited for LEO scenario.

## Option-2 Implicit Signaling Design

The rationale behind Option-2 is to split the RTT between UE and RP into two parts, as mentioned above. First part (or first offset) captures the RTT between the satellite and the RP, while the second part (or second offset) covers the RTT of the service link. Since the value of Koffset is common among all UEs before random access channel (RACH) procedure, the second offset must be chosen based on at least the maximum service link RTT values, in order to account for the UEs at the cell edge. Maximum values of the service link RTT are reported in TR 38.821 [2]. In particular, for GEO, LEO 600 km, and LEO 1200 km, the maximum service link RTTs are 270.73 ms, 12.89 ms, and 20.89 ms, respectively.

Moreover, In RAN1#104be, an agreement achieved regarding the timing advance (TA) in NTN. In particular, it was agreed that the Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+ NTA,common+NTA,offset×Tc

In the equation above, NTA,common is referred to as “common TA” an captures the part of the TA originates from the satellite to the RP. As can be seen, NTA,common can be employed for calculation of the first offset in Option-2 approach. This particular choice has the advantages of reducing signaling overhead and avoid duplicate signaling, as NTA,common is broadcast via SIB1.

Observation 3: Common TA captures the RTT of the satellite to the reference point.

Observation 4: Adoption of common TA reduces signaling overhead and avoids duplicate signaling for determination of Koffset.

Proposal 2: Common Timing Advance should be used for determination of the first offset value, capturing the RTT of the satellite to RP, in Option-2 to reduce signaling overhead and avoid duplicate signaling.

The second offset value should cover the RTT o the service link. This can be obtained e.g. from “ra-ContentionResolutionTimer”. In particular, “ra-ContentionResolutionTimer” is a parameter configured within RACH-ConfigCommon parameter structure which is broadcast via SIB1. There, “ra-ContentionResolutionTimer” takes values in the range of {8, 16, 24, 32, 40, 48, 56, 64} subframe [ms] which, in turn, captures the range of the values of service link RTTs in NTN, especially for LEO scenarios. Service link RTT of the GEO scenario can be easily obtained via scaling of the parameter “ra-ContentionResolutionTimer”. As a result, “ra-ContentionResolutionTimer” together with common RTT can be employed for deriving a value for the initial Koffset as follows:

Koffset=RTTcommon+RTTmax, ServiceLink,

where RTTcommon is derived from NTA,common and RTTmax, ServiceLink is obtained based on the maximum RTT of the service link. Given the discussion above, Koffset can be calculated as

Koffset=RTTcommon+(α×TConResTimer),

where TConResTimer is a specific value of “ra-ContentionResolutionTimer” and α is scaling factor that depends on the scenario. Here, for simplicity, we assumed that all parameters have the unit of millisecond.

Observation 5: NTN UE acquires “ra-ContentionResolutionTimer” and the RTT value of the satellite to the RP via common TA before performing random access procedure.

Proposal 3: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., “ra-ContentionResolutionTimer” and common TA for option-2.

## Down-selection

When comparing Option-1 and Option-2, the advantages of Option-2 are more than Option-1. For both options, and considering implicit signaling design, only the scaling factor should be sent via the gNB. However, generally, the value ranges of Option-2 is less than Option-1, as part of the RTT between the UE and RP is already signalled via common TA. Thus, signaling overhead is reduced.

Proposal 4: RAN1 to down-select Option-2 for determination of the value of initial Koffset.

# Conclusions

Observation 1: NTN UE acquires RRC timers T300, T301, T319, and T310 before performing random access procedure.

Observation 2: RRC timers T300/301/319 are most suited for GEO satellites, while RRC timer T310 is suited for LEO scenario.

Observation 3: Common TA captures the RTT of the satellite to the reference point.

Observation 4: Adoption of common TA reduces signaling overhead and avoids duplicate signaling for determination of Koffset.

Observation 5: NTN UE acquires “ra-ContentionResolutionTimer” and the RTT value of the satellite to the RP via common TA before performing random access procedure.

Proposal 1: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., RRC timers T300, T301, T319, and T310.

Proposal 2: Common Timing Advance should be used for determination of the first offset value, capturing the RTT of the satellite to RP, in Option-2 to reduce signaling overhead and avoid duplicate signaling.

Proposal 3: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., “ra-ContentionResolutionTimer” and common TA for option-2.

Proposal 4: RAN1 to down-select Option-2 for determination of the value of initial Koffset.

# References

[1]	3GPP TR 38.811, “Study on New Radio (NR) to support non terrestrial networks (Release 15),” 3rd Generation Partnership Project; Technical Specification Group Radio Access Network, Version 15.1.0, June 2019.

[2]	3GPP TR 38.821 v16.0.0 (2019-12): 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)

[3]	3GPP TSG RAN WG1 Meeting #104b-e, “RAN1 Chairman’s Notes 8.4 eom”, April 2021.