3GPP TSG RAN WG1 Meeting #105-e		             R1-2105334

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.11.1.1

Source: 	Samsung

Title: 	On resource allocation for power saving

Document for:	Discussion and Decision

# Introduction

UE-to-UE direct communication is one important scenario developed by 3GPP with a series of techniques including LTE D2D, LTE V2X and NR sidelink. The first standard of NR sidelink was completed in Rel-16 Work Item “5G V2X with NR sidelink”, in which the sidelink technique focusing on vehicle-to-everything (V2X) was specified. In Rel-17, a wider scope of sidelink applications is expected, and a new Rel-17 Work Item on “NR sidelink enhancement” was approved in RAN#86 [1] to support various use cases in both public safety and commercial scenarios. The WID was further updated in RAN#90-e [2] to include potential impact of sidelink DRX in power saving scope.

Rel-16 NR sidelink mainly focus on vehicle UEs which have sufficient battery supply. In Rel-17, power sensitive UEs such as pedestrian UE and UE in public safety and commercial use cases are involved in the typical use cases. Therefore, power saving for NR sidelink was introduced in Rel-17 WI to allow SL UE operates in a power efficient manner with the following objective:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution, possible solutions for NR sidelink power saving, including how to reuse LTE technique and necessary enhancements are provided.

# Resource allocation schemes

In Rel-16 NR sidelink mode 2 operation, long term sensing window was specified to detect resource reservations and avoid collision as much as possible. However, the sensing procedure can hardly be supported by power sensitive UEs with limited battery capacity. Random selection and partial sensing were agreed as NR SL power saving schemes to achieve a trade-off between power consumption and reliability.

## Random selection

The following agreements regarding random selection was made in RAN1#104-e meeting [3]:

Agreements: - Random resource selection is applicable to both periodic and aperiodic transmissions - FFS conditions for random resource selection

Random selection significantly reduces power consumption by skipping sensing procedure, but cannot avoid collision by selecting resource randomly, thus leading to a degradation in PRR performance of legacy sidelink UEs. In addition, the collision between Rel-17 UEs using random selection may become non-negligible as the density of UEs using random selection increases. Therefore, some evaluation on the performance of random selection is necessary to estimate the feasibility of random selection and suitable scenarios.

In LTE V2X, the principle of random selection is that a UE determines all resources within a resource selection window as a candidate resource set, and randomly select among resources within the candidate resource set unless the resource are unavailable e.g. restricted by half-duplex or simultaneous transmission. Enhancements on LTE random selection should also be considered in Rel-17 to improve the PRR performance of random selection and reduce its impact on sidelink UEs using full/partial sensing.

Resource partitioning can be introduced to improve reliability. For example, the partitioning can be based on UE ID, UE service class, priority of SL data, or some other characteristic of the UE or SL data. The partitioning of resources can be done in such a way so as to minimize the probability of collision and make the probability of collision related to the priority of SL data, higher priority SL data having lower probability of collision of and vice versa.

Figure 1 (a) shows the impact of increasing the number of user groups on PRR. As the number of groups that users are partitioned into increases, the randomness within each group is reduced leading to less collision and higher PRR. Another benefit of partitioning users into groups is to control the collision probability as a function of the traffic priority. The simulation results in Figure 1 (b), show the benefit of improving the PRR of high priority traffic, albeit at the expense of a lower PRR for low priority traffic, by allocating more resources to the high priority traffic. In this simulation it is assumed that half the users have low priority traffic and half the users have high priority traffic. Figure 1 (b) shows three different scenarios:

- The 3 bars on the left show the PRR when all users share the same resources. The bar on the left of the group is the PRR for low priority users. The two bars on the right of the group is the PRR for high priority users. As expected, the PRR is the same in all cases (within the tolerance of the simulation).

- The 3 bars in the middle show the PRR when the resources are partitioned into three groups. The low priority users (half of the total users) are allocated one group, while the high priority users (the other half of the total users) are allocated two groups. The bar on the left of the group is the PRR for low priority users. The bar in the middle of the group is the PRR for the high priority users when the high priority users share the resources of the two allocated groups. The bar on the right of the group is the PRR for high priority users, when the high priority users are further partitioned between the two groups allocated to high priority users. As expected, the PRR of the low priority users goes down, while the PRR of the high priority users improves. Furthermore, by partitioning the high priority users between the high priority groups, there is a further slight improvement in the PRR.

- The 3 bars on the right show the PRR when the resources are partitioned into four groups. The low priority users (half of the total users) are allocated one group, while the high priority users (the other half of the total users) are allocated three groups. The bar on the left of the group is the PRR for low priority users. The bar in the middle of the group is the PRR for the high priority users when the high priority users share the resources of the three allocated groups. The bar on the right of the group is the PRR for high priority users, when the high priority users are further partitioned between the three groups allocated to high priority users. As expected, the PRR of the low priority users goes down, while the PRR of the high priority users improves. Furthermore, by partitioning the high priority users between the high priority groups, there is a further slight improvement in the PRR.

Figure 1. Impact of user grouping on PRR for random resource selection.

- PRR as a function of the number of groups.

- PRR for priority-dependent user grouping.

Proposal 1: For NR SL random resource selection, consider partitioning of candidate SL resources to reduce collision probability and improve PRR of high priority traffic.

## Periodic-based partial sensing

The following agreements regarding periodic-based partial sensing was made in RAN1#104bis-e meeting [4]:

Agreements: - In periodic-based partial sensing, - For the set of Preserve values, down-select to one of the following in RAN1#105-e - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList - FFS if support multiple sets of Preserve values based on one or more metrics - FFS whether/how to restrict the set of values - For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible) - Alt 1: Option 1 as in RAN1#104-e - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1 - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc. - Note: companies are encouraged to provide more evaluations Agreement: - When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met: - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool - The resource pool is (pre-)configured to enable partial sensing - Partial sensing configured by higher layer in the UE

Due to the difference between NR and LTE traffic characteristics, how to determine the values of Preserve and k used to derive sensing occasions needs further discussion. Considering the allowed periods of inter-period resource reservation are configured with resource pool, it is straightforward to determine Preserve based on the configured set sl-ResourceReservePeriodList. It could be beneficial to support flexible determination of Preserve to further reduce power consumption of sensing at the expense of acceptable performance degradation. For example, if some typical services correspond to the most part of PSSCH transmissions in a resource pool, UE could only monitor the most part of PSSCH transmissions with a subset of configured periods associated with the typical services. Therefore, Alt 2 in the agreement above is preferred.

The determination of the subset could be based on at least (pre-)configuration or UE determination. In addition, traffic priority at TX UE side could also be considered as a valid metric to better protect reliability of high priority traffic. For example, gNB could configure two subsets from sl-ResourceReservePeriodList with different number of values, for PSSCH with priority lower than threshold (i.e. higher priority) a subset with more values of Preserve is used in order to monitor more potential collision and enhance reliability with higher power consumption, otherwise for PSSCH with priority higher than threshold the other subset with less values of Preserve is used to save power.

Figure 2 and table 1 shows the performance comparison of two alternatives. In the simulation, sl-ResourceReservePeriodList is set as {100, 200} where 2/3 traffic are generated with period of 100 and 1/3 traffic are generated with period of 200. In Alt 1, UE perform sensing with Preserve corresponds to full set i.e. {100, 200} and in Alt 2 UE perform sensing with Preserve = 100 only. K equals to only most recent occasion in the simulation.

As illustrated in Figure 2, in alt 2 most traffic in the resource pool are transmitted with period of 100ms and can be detected during sensing procedure, the additional collision caused by remaining traffic with period of 200ms impact the one-shot PRR performance, but the PRR degradation of Alt 2 compared with Alt 1 is acceptable. Since the sensing occasions monitored by UE in alt 2 is much lower than alt 1 for each transmission, even after introducing additional power consumption of HARQ-based retransmission, the overall power consumption of alt 2 is reduced by ~28%. Therefore, Alt 2 of configuring Preserve value as a subset of sl-ResourceReservePeriodList is preferred as an appropriate trade-off between one-shot PRR performance and power consumption.

Furthermore, the trade-off between one-shot PRR performance and power consumption can be utilized to satisfy the different requirements of NR traffic. For example, high priority traffic needs higher one-shot PRR performance to guarantee its reliability requirement and reduce latency, and low priority traffic is more latency-tolerant and may allow more HARQ retransmissions. Therefore, Alt 2 can be used for low priority traffic to reduce overall power consumption, and Alt 1 can be used for high priority traffic to improve one-shot PRR performance. UE can be configured with both sets and dynamically determine which set to use for a given transmission according to priority.

Figure 2. PRR performance and power consumption based on Preserve value set

Table 1. Average power consumption of full sensing, partial sensing and random selection

| Sensing scheme | Power consumption (%) |
| --- | --- |
| Alt 1: Periodic sensing with Preserve corresponds to all values | 100 |
| Alt 2: Periodic sensing with Preserve corresponds to a subset | 72 |

Proposal 2: Preserve corresponds to a subset of values derived from the configured set sl-ResourceReservePeriodList

- Support configuration of multiple sets, and UE can select one set based on priority of a given transmission.

The principle of deciding value of k in NR partial sensing is not similar as in LTE-V. For determination of candidate resources in LTE-V, both RSRP-based resource exclusion and RSSI-based resource quality ranking are used since LTE-V was developed mainly targeting for periodic traffic. Correspondingly, LTE-V configures the values of k with a bitmap, since monitoring on multiple values of k is used to collect sufficient S-RSSI measurement result for linear averaging.

However, the determination of candidate resources in NR SL only include RSRP-based resource exclusion in previous one period, and no resource exclusion based on inter-period resource reservation before more than one period was specified in Rel-16. Consequently, monitoring k>1 periods for a given periodicity seems less meaningful. Therefore, Option 1 in the agreement above is preferred.

Furthermore, although sensing occasion is determined corresponding to candidate resources, UE may not be able to monitor resource pool on a given sensing occasion. For example, UE may need to perform SL/UL transmission on a slot containing sensing occasion, thus the occasion cannot be sensed due to half duplex. If the most recent sensing occasion cannot be sensed by TX UE, one possible solution is to define the sensing occasion as invalid, then UE will skip the invalid sensing occasion and try to monitor the second recent sensing occasion if the second one is valid, and so on. With this solution UE could always sense at least one occasion for a given reservation periodicity to avoid potential collision with the given periodicity from other UE.

Proposal 3: k equals to only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction.

- FFS additional enhancements if the most recent sensing occasion cannot be monitored (e.g. due to half duplex).

Since periodic-based sensing corresponds to sensing window before slot n, on which UE is triggered to perform resource selection, it is sensing in advance of the trigger, thus the UE should be able to expect the timing point of being triggered to select PSSCH resource in advance of the start of periodic-partial sensing. Correspondingly, the condition for which periodic-based partial sensing is performed by UE should at least include that UE is able to expect a subsequent resource selection trigger in advance of the start of periodic-partial sensing.

Proposal 4: Periodic-based partial sensing can be performed by TX UE if UE expects resource selection will be triggered in slot n.

## Contiguous partial sensing

The following agreements regarding contiguous partial sensing was made in RAN1#104-e meeting [3]:

Agreements: - In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option: - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable). - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.) - FFS whether n can be replaced by e.g., index of some of Y candidate slots - FFS condition(s) in which contiguous partial sensing is performed by UE - FFS interaction with SL-DRX, if any - FFS interaction with periodic-based partial sensing, if any - Other options are not precluded - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

In NR sidelink system, potential future transmissions from other UE, such as retransmission of periodic traffic, can be reflected by detected resource reservation indicated in SCI. Single SCI format may reserve up to 2 additional sidelink resources within a 32ms time window. Correspondingly, for resource selection triggered in slot n, it was agreed to introduce contiguous sensing window [n+TA, n+TB] to avoid the potential collision with intra-period resource reservation by other UEs.

According to the value of TA, TB, the contiguous sensing can be before slot n with negative TA, TB, which allows UE to perform sensing before resource selection trigger, thus selects PSSCH transmission resource as quickly as possible to reduce latency; or after slot n with positive TA, TB, which allows UE to perform sensing-based resource selection even if UE collects no existing sensing result when being triggered; or straddling slot n with a negative TA and a positive TB. In addition, specific entry e.g. TA=TB =0 can also be considered to allow UE skipping sensing procedure e.g. using random selection instead. Each possibility should be supported in Rel-17 NR SL to support various characteristics of NR SL traffic.

Proposal 5: From system design perspective, if UE is triggered to perform resource selection in slot n, both contiguous partial sensing starting and/or ending before slot n and after slot n are supported.

Contiguous partial sensing before slot n is also kind of pre-sensing, thus UE should be able to expect the timing point of being triggered to select PSSCH resource in the future. A similar condition of periodic-based partial sensing should be also be introduced for contiguous partial sensing before slot n.

Proposal 6: Contiguous partial sensing before slot n can be performed by TX UE if UE expects resource selection will be triggered in slot n.

The resource selection window of periodic-based partial sensing was agreed in last meeting, but the resource selection window of contiguous partial sensing was not discussed. For contiguous partial sensing before slot n, in which UE is triggered to select transmission resource, the situation is similar to periodic-based partial sensing that UE can collect sufficient sensing result at slot n by sensing in advance, thus the rule of periodic-based partial sensing can be reused, i.e. reusing the mechanism in Rel-16 NR V2X according to step 1 in TS 38.214. Similarly, no further enhancement is needed on top of the legacy procedure.

Proposal 7: For contiguous partial sensing before slot n, the resource selection window [n+T1, n+T2] is determined by reusing legacy Rel-16 NR-V procedure without any further enhancement.

Compared with contiguous partial sensing before slot n, contiguous partial sensing after slot n has no restriction on whether UE should expect future timing of being triggered resource selection. However, contiguous partial sensing after slot n increases transmission delay thus its condition should be carefully considered. One possible metric is traffic priority, e.g. only high-priority and latency-tolerant traffic could use contiguous partial sensing after slot n to ensure reliability one-shot transmission, and reduce the possibility of HARQ-based retransmissions. Latency should also be considered as necessary condition, e.g. if remaining PDB corresponding to one transmission is larger than threshold, contiguous partial sensing after slot n can be enabled.

Proposal 8: Contiguous partial sensing after slot n can be performed by UE if UE is triggered to perform resource selection in slot n and all or a subset of the following conditions are met:

- Priority < threshold

- Remaining PDB > threshold

- FFS other conditions

Since the main purpose of contiguous sensing is to avoid intra-period reservation, it is expected the position of contiguous sensing window could be determined according to intra-period reservation span in SCI. One straightforward solution is using the resource selection trigger as reference timing point and defining a window with up to 31 slots, e.g. contiguous sensing window during [n-31, n-Tproc,0). However, considering the T1-slot gap between slot n and starting slot of selection window n+T1, the beginning T1 slots in contiguous sensing window [n-31, n-Tproc,0) actually cannot reserve PSSCH transmission later than slot n+T1. Therefore, sensing in slot [n-31, n-31+T1) seems inefficient.

Other reference timing point can be considered to further reduce power consumption in contiguous partial sensing. One possible solution is using the starting slot n+T1 of resource selection window instead of slot n. In addition, since per slot  within candidate resource set corresponds to a set of sensing slots in which resource reservation on slot  may be detected, the location of contiguous sensing window can also be determined by [[n'-31, n-Tproc,0)] where n' is the earliest slot containing candidate resource .

Proposal 9: For contiguous sensing window [n+TA, n+TB], the reference timing point n can be considered with the following options:

- n

- n+T1

- The beginning slot index of selected Y candidate slots.

## Re-evaluation & pre-emption

Pre-emption check and re-evaluation was introduced in Rel-16 NR SL to allow the UE to re-select SL resources if potential collision with higher priority is detected on selected/reserved resources. Considering it was agreed to support the combination of partial sensing, random selection and full sensing within same resource pool, SL collision may occur more frequently e.g. caused by UEs using random selection, it is also beneficial to support re-evaluation check and pre-emption check procedures in Rel-17 partial sensing.

In addition, introducing NR SL enhancements developed in Rel-16 to random selection such as re-evaluation and pre-emption check is also beneficial to improve the PRR performance of random resource selection. In our companion Tdoc [7], we present preliminary simulation results and power consumption analysis on the benefit of introducing re-evaluation and pre-emption check to both random resource selection and partial sensing.

On the contrary, power consumption of pre-emption check and re-evaluation check in SL power saving mode should be carefully evaluated. Similar to partial sensing, pre-emption check and re-evaluation check could improve the HARQ performance of a single-shot transmission and reduce number of potential retransmissions at the expense of increased power for resource monitoring. Therefore, it is preferred to enable or disable pre-emption check and re-evaluation check by higher layer configuration e.g. in resource pool configuration.

Proposal 10: Both re-evaluation and pre-emption with power saving mode(s) can be enabled/disabled by resource pool (pre-) configuration.

Since pre-emption check and re-evaluation check are relatively independent with resource allocation schemes, the legacy procedure can be reused for Rel-17 power saving mode as a baseline. For the purpose of reducing power consumption used by pre-emption check and re-evaluation check, the corresponding sensing window could be modified with a fixed size W, e.g. up to 31 slots, to efficiently detect potential collisions and keep consistency with the principle of contiguous partial sensing.

Proposal 11: The procedure of pre-emption check and re-evaluation check in Rel-16 NR V2X is reused for Rel-17 power saving mode with a fixed window size of W slots.

For a sidelink transmission in slot m, the UE can perform a re-evaluation check or a pre-emption check in slot m-T3. The end of the sensing window for a pre-emption check or a re-evaluation check in slot m-T3 is m-T3-Tproc,0. The start of the sensing window should be W slots prior to slot m, where W is the furthest slot that can be reserved by SCI, i.e. 31 slots in Rel-16. Therefore, the sensing window is: [m-W,m-T3-Tproc,0).

Proposal 12: For a UE configured with enabled pre-emption check or enabled re-evaluation check, if UE reserves a sidelink transmission in slot m, the UE monitors slots between [m-W, m-T3-Tproc,0), where

- m-T3 is the slot in which pre-emption check and re-evaluation check is performed

- Tproc,0 corresponds to the processing time of pre-emption check and re-evaluation check

## Determination of resource allocation scheme

It was agreed in RAN1#103-e meeting that a Mode 2 transmission resource pool can be configured to enable full sensing, partial sensing, random selection or any combination of them. In addition, multiple schemes related to partial sensing as well as support of pre-emption check and re-evaluation check are discussed and there is no common understanding on how to use them. The conditions/restrictions need to be carefully discussed to ensure UE could select the most appropriate scheme for a given transmission as full sensing, partial sensing or random selection.

Some straightforward criteria such as higher layer configuration, traffic priority, CBR and remaining PDB can be used to select or exclude a RA scheme. HARQ-ACK error rate, (e.g. the UE can calculate a sliding window or exponential average of the HARQ error rate) can be used to switch from one scheme with lower reliability (e.g. random selection) to another scheme with higher reliability (e.g. partial/full sensing). The availability of inter-UE coordination information is also a valid criteria, e.g. if inter-UE coordination is available and can be used for resource selection, UE could skip sensing procedure. Existing sensing result could also be used to determine resource allocation schemes, e.g. if there exists sufficient pre-sensing result at resource selection trigger time in slot n, UE could select partial sensing as resource allocation scheme, otherwise UE uses random selection or start contiguous sensing after slot n. If UE could acquire the information of battery level from application layer similarly as in NR Uu power saving mode, UE could switch its resource allocation scheme according to remaining power.

Proposal 13: In a resource pool configured with more than one resource allocation scheme, study how UE determines resource allocation for a given transmission, according to one or more of the following criteria:

- Higher layer configuration

- Priority of traffic

- CBR

- Remaining PDB

- HARQ-ACK error rate

- Status of inter-UE coordination information

- Existing sensing result

- Battery level

- DRX configuration

- UE capability

- UE implementation

Different partial sensing schemes including periodic-based partial sensing, contiguous partial sensing before resource selection trigger and contiguous partial sensing after resource selection trigger are discussed. The relationship between different partial sensing schemes e.g. how could UE determine partial sensing for a given transmission, whether UE can use more than one partial sensing scheme for a given transmission or not, whether more than one partial sensing scheme can coexist in the same resource pool, etc., should be discussed further to ensure that the UE uses the most appropriate partial sensing scheme(s).

Proposal 14: Study the condition of determining partial sensing scheme(s), according to one or more of the following criteria:

- (Pre-)configuration in a resource pool

- Higher layer decision e.g. by traffic type

- UE implementation

# Sidelink DRX

The following agreements regarding sidelink DRX (SL DRX) were made in RAN2#112-e meeting [5]:

Agreements: - If a UE is in SL active time, UE should monitor PSCCH. FFS on PSSCH. FFS for sensing impacts. - Working assumption: SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

Based on the above decision, in RAN#90-e meeting the Rel-17 eSL Work Item description was revised to include RAN1 work on resource allocation for power saving considering the impact of SL DRX, if any [2]. Additionally, the following agreement was made in RAN2#113-e meeting [6]:

Agreements: - For data reception, RAN2 defines the behaviour for monitoring the SCI reception (i.e., PSCCH and 2nd SCI on PSSCH) during the SL active time for SL DRX. For data reception, the UE may skip monitoring of PSCCH and 2nd SCI on PSSCH during inactive time for SL DRX. Sensing aspect is not considered in this agreement.

Accordingly, although the working assumption above can be interpreted as that a UE cannot perform sensing in inactive time of SL DRX since sensing is defined as L1 SL-RSRP measurement when the corresponding PSCCH is decoded, there is no clear agreement about the sensing behavior during SL DRX inactive time. Therefore, the UE behavior for sensing in inactive time of SL DRX should be clarified at first.

Proposal 15: Down select between two options below for UE behavior on sensing with SL DRX:

- Option 1: Sensing (PSCCH monitoring and L1 SL-RSRP measurement) is allowed in inactive time of SL DRX.

- Option 2: Sensing (PSCCH monitoring and L1 SL-RSRP measurement) is allowed only in active time of SL DRX.

Specifically, Option 1 means that regardless of whether the UE is in SL DRX mode or not, the UE may monitor PSCCH in sensing window when the UE performs sensing. With Option 1, RAN1 impact of SL DRX can be avoided at least for sensing operation. Unlike Opion 1, Option 2 means that similar to data reception, in sensing operation the UE may skip monitoring of PSCCH during inactive time of SL DRX. In aspect of power saving, Option 2 would be more desiable than Option 1. Therefore, our preference is Option 2.

If Option 2 is selected, RAN1 needs to discuss further about enhancement on sensing behavior for SL DRX. With Option 2, the sensing window can be outside or partially inside of SL DRX active time. The UE thus may not perform sensing operation at all or perform sensing only on sensing window within SL DRX active time. Therefore, the following can be considered for the possible sensing behavior with Option 2:

- Alt 1-1: Perform sensing by adjusting the sensing window to occur in the SL DRX active time in order to guarantee sensing duration.

- Alt 1-2: Perform sensing within configured sensing window only during the slot(s) within SL DRX active time (inactive time is not used for sensing)

- Alt 1-3: No sensing (random selection)

- Alt 1-4: The sensing window becomes active time for SL DRX

Alt 1-1 is to ensure the length of already configured sensing window by moving the sensing window into available SL DRX active time. If enough sensing duration is not ensured due to SL DRX, there may be performance degradation in Mode2 resource allocation. For Alt 1-2, sensing is performed if the slot is within configured sensing window and in active time of SL DRX. Therefore, if there is no overlapping between SL DRX active time and the configured sensing window as shown in Figure 3 (a) or (c), the UE may not perform sensing. In case of Alt 1-3, a UE performs random selection without sensing if the UE is in SL DRX mode. For Alt 1-4, if a UE performs sensing, the sensing window is defined as active time for SL DRX. Notice that Alt 1-4 is effectively the same as Option 1 in Proposal 6 above. In aspect of power saving, Alt 1-2 and Alt 1-3 would be more desiable solusions than others but sensing in Alt 1-2 may be infeasible if SL DRX active time is not overlapped with sensing window. Therefore, we propose the combinition of Alt 1-2 and Alt 1-3. As shown in Figure 3 (b) or (d), if there is available sensing slot(s) with SL DRX active time, Alt 1-2 can be used. If sensing slot(s) is not available due to SL DRX as shown in Figure 3 (a) or (c), Alt 1-3 can be used.

Proposal 16: For UE with enabled SL DRX, sensing window is adjusted into DRX active time with configured sensing window.

- Sensing is not performed (e.g. using random selection instead) if no sensing slot is within active time.

Besides, as depicted in Figure 3 (c) and (d), the impact of SL DRX should be considered not only for partial sensing but also for full sensing.

Proposal 17: Consider the impact of SL DRX for full sensing also.

- Case A: Partial sensing and sensing slot(s) is not available by SL DRX

- Case B: Partial sensing and there is available sensing slot(s) with SL DRX

- Case C: Full sensing and sensing slot(s) is not available by SL DRX

- Case D: Full sensing and there is available sensing slot(s) with SL DRX

Figure 3. The impact of SL DRX for partial (Periodic based partial sensing is assumed and k is the most recent sensing occasion for a given reservation periodicity) and full sensing.

Moreover, for the impact of SL DRX with resource allocation, we need to consider resource selection behavior in addition to sensing behavior. Specifically, for the case where TX UE performs resource selection for RX UE which has SL DRX operation enabled, resource selection may need to be carried out only in active time of RX UE. For instance, assuming that SL DRX wake-up timing is aligned between TX and RX UE(s), if the TX UE selects resource(s) but the resource(s) is transmitted in SL DRX inactive time of RX UE, the transmission cannot be delivered to the RX UE(s). Accordingly, the following can be considered as the possible behavior for resource selection with SL DRX:

- Alt 2-1: The resource selection window of TX UE is adjusted (within remaining PDB) to be contained in SL DRX active time of RX UE

- Alt 2-2: The resource selection window of TX UE is configured resource selection window within SL DRX active time (inactive time is not used for slot(s) for resource selection) of RX UE

- Alt 2-3: The resource selection window becomes the active time for SL DRX

Alt 2-1 is to ensure the resource selection window by adjusting it to occur during the SL DRX active time of RX UE. However, by SL DRX and remaining PDB constraints, it may not be possible to find available slot(s) for resource selection. In this case, we can consider the use of exceptional resource pool. For Alt 2-2, a UE performs resource selection within the overlapped part of configured resource selection window and active time for SL DRX of RX UE. Therefore, if enough active time doesn’t exist within configured resource selection window, the number of candidate resource(s) for selection would be decreased and there may be performance degradation in Mode2 resource allocation. For Alt 2-3, if a UE performs resource selection, the resource selection window is defined as active time for SL DRX. Thus, Alt 2-3 is not efficient in the power saving aspect, and it requires the RX UE to align its DRX active time to the resource selection window of the Tx UE. Considering pros and cons of above alternatives, we propose Alt 2-1 as:

Proposal 18: When TX UE performs resource selection for RX UE with SL DRX enabled, the resource selection window is adjusted (within remaining PDB) according to SL-DRX active time of RX UE.

- Exceptional resource pool is used if there is no available slot for resource selection

In addition, SCI can be considered as one of solutions to align SL DRX wake-up time among the UEs communicating with each other. Other mechanism without RAN1 impact can be considered to align SL DRX timing as well. However, SCI for alignment of SL DRX timing can be a promising solution since it can be applied commonly for all cast types of broadcast, groupcast, and unicast. Therefore, we propose to consider that SCI is used to indicate SL DRX activation and deactivation to align SL DRX timing.

Proposal 19: SCI is used to align SL DRX wake-up time between TX UE and RX UE(s).

Lastly, the impact of SL DRX should be considered with respect to PSFCH reception at the TX UE. On one hand, similar to data reception for SL DRX, the TX UE may skip PSFCH reception in inactive time of SL DRX. On the other hand, regardless of whether the TX UE is in SL DRX active time or not, the TX UE can receive PSFCH when expected. For the latter case, even though the PSFCH resource is located in the tail end of a slot, we can consider further allowing PSCCH and PSSCH reception in the same slot as PSFCH. The TX UE behavior for PSFCH reception during inactive time of SL DRX should be clarified as:

Proposal 20: Down select between two options below for TX UE behavior on PSFCH reception with SL DRX enabled:

- Option 1: TX UE may skip PSFCH reception in inactive time of SL DRX

- Option 2: TX UE can receive PSFCH when it is expected regardless of whether the UE is in SL DRX mode or not.

  - FFS whether the PSFCH receiving slot becomes part of the active time of SL DRX.

In the power saving aspect, Option 1 would be more desiable than Option 2. However, if Option 1 is selected, RAN1 needs to discuss further about the behavior PSFCH reception with SL DRX. With Option 1, depending on TX UE’s decision for transmission of PSSCH, PSFCH reception slot can be outside or inside SL DRX active time in the aspect of TX UE. Therefore, the followings can be considered for the possible UE behavior of PSFCH reception with Option 1 as:

- Alt 3-1: TX UE always disables SL HARQ feedback by 2nd SCI if SL DRX is enabled.

- Alt 3-2: If the expected PSFCH reception is within inactive time of SL DRX, TX UE disables SL HARQ feedback by 2nd SCI. Otherwise, PSFCH reception can be enabled by TX UE.

- Alt 3-3: TX UE reselects the PSSCH resource until the associated PSFCH resource is located within SL-DRX active time

- Alt 3-4: TX UE selects/adjusts the resource selection window to include candidate PSSCH resources where the associated PSFCH resource is located within SL-DRX active time

- Alt 3-5: PSFCH resource is the first slot in the active time of SL DRX containing PSFCH after a minimum PSSCH-PSFCH time gap

Alt 3-1 is to disable HARQ feedback if TX UE is in SL DRX mode. Unlike Alt 3-1, in case of Alt 3-2, TX UE can disable HARQ feedback only if PSFCH resource which is associated with PSSCH is within inactive time of SL DRX. Alt 3-3 and Alt 3-4 are to enable PSFCH reception by modifying Mode 2 resource selection procedure. Alt 3-5 is to enable PSFCH reception by modifying association between PSSCH and PSFCH considering SL DRX. We need to study further the TX UE behavior of PSFCH reception with Option 1.

Proposal 21: Study a TX UE behavior of PSFCH reception considering that PSFCH reception in inactive time of SL DRX is not allowed.

# Conclusions

In this contribution, the issues in random selection and partial sensing procedures are discussed with the following proposals:

Proposal 1: For NR SL random resource selection, consider partitioning of candidate SL resources to reduce collision probability and improve PRR of high priority traffic.

Proposal 2: Preserve corresponds to a subset of values derived from the configured set sl-ResourceReservePeriodList

- Support configuration of multiple sets, and UE can select one set based on priority of a given transmission.

Proposal 3: k equals to only the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction.

- FFS additional enhancements if the most recent sensing occasion cannot be monitored (e.g. due to half duplex).

Proposal 4: Periodic-based partial sensing can be performed by TX UE if UE expects resource selection will be triggered in slot n.

Proposal 5: From system design perspective, if UE is triggered to perform resource selection in slot n, both contiguous partial sensing starting and/or ending before slot n and after slot n are supported.

Proposal 6: Contiguous partial sensing before slot n can be performed by TX UE if UE expects resource selection will be triggered in slot n.

Proposal 7: For contiguous partial sensing before slot n, the resource selection window [n+T1, n+T2] is determined by reusing legacy Rel-16 NR-V procedure without any further enhancement.

Proposal 8: Contiguous partial sensing after slot n can be performed by UE if UE is triggered to perform resource selection in slot n and all or a subset of the following conditions are met:

- Priority < threshold

- Remaining PDB > threshold

- FFS other conditions

Proposal 9: For contiguous sensing window [n+TA, n+TB], the reference timing point n can be considered with the following options:

- n

- n+T1

- The beginning slot index of selected Y candidate slots.

Proposal 10: Both re-evaluation and pre-emption with power saving mode(s) can be enabled/disabled by resource pool (pre-) configuration.

Proposal 11: The procedure of pre-emption check and re-evaluation check in Rel-16 NR V2X is reused for Rel-17 power saving mode with a fixed window size of W slots.

Proposal 12: For a UE configured with enabled pre-emption check or enabled re-evaluation check, if UE reserves a sidelink transmission in slot m, the UE monitors slots between [m-W, m-T3-Tproc,0), where

- m-T3 is the slot in which pre-emption check and re-evaluation check is performed

- Tproc,0 corresponds to the processing time of pre-emption check and re-evaluation check

Proposal 13: In a resource pool configured with more than one resource allocation scheme, study how UE determines resource allocation for a given transmission, according to one or more of the following criteria:

- Higher layer configuration

- Priority of traffic

- CBR

- Remaining PDB

- HARQ-ACK error rate

- Status of inter-UE coordination information

- Existing sensing result

- Battery level

- DRX configuration

- UE capability

- UE implementation

Proposal 14: Study the condition of determining partial sensing scheme(s), according to one or more of the following criteria:

- (Pre-)configuration in a resource pool

- Higher layer decision e.g. by traffic type

- UE implementation

Proposal 15: Down select between two options below for UE behavior on sensing with SL DRX:

- Option 1: Sensing (PSCCH monitoring and L1 SL-RSRP measurement) is allowed in inactive time of SL DRX.

- Option 2: Sensing (PSCCH monitoring and L1 SL-RSRP measurement) is allowed only in active time of SL DRX.

Proposal 16: For UE with enabled SL DRX, sensing window is adjusted into DRX active time with configured sensing window.

- Sensing is not performed (e.g. using random selection instead) if no sensing slot is within active time.

Proposal 17: Consider the impact of SL DRX for full sensing also.

Proposal 18: When TX UE performs resource selection for RX UE with SL DRX enabled, the resource selection window is adjusted (within remaining PDB) according to SL-DRX active time of RX UE.

- Exceptional resource pool is used if there is no available slot for resource selection

Proposal 19: SCI is used to align SL DRX wake-up time between TX UE and RX UE(s).

Proposal 20: Down select between two options below for TX UE behavior on PSFCH reception with SL DRX enabled:

- Option 1: TX UE may skip PSFCH reception in inactive time of SL DRX

- Option 2: TX UE can receive PSFCH when it is expected regardless of whether the UE is in SL DRX mode or not.

  - FFS whether the PSFCH receiving slot becomes part of the active time of SL DRX.

Proposal 21: Study a TX UE behavior of PSFCH reception considering that PSFCH reception in inactive time of SL DRX is not allowed.

# References

[1] RP-193231, “NR Sidelink enhancement,” RAN#86, Stiges, Spain, December, 2019.

[2] RP-202846, “Revised WID on NR Sidelink enhancement,” RAN#90-e, e-meeting, Dec, 2020.

[3] 3GPP TSG RAN1#104-e, Final Report of 3GPP TSG RAN

[4] 3GPP TSG RAN1#104bis-e, Final Report of 3GPP TSG RAN

[5] 3GPP TSG RAN2#112-e, Final Report of 3GPP TSG RAN

[6] 3GPP TSG RAN2#113-e, Final Report of 3GPP TSG RAN

[7] R1-2103259, “On Sidelink Issues and RAN1 Impacts”, Samsung, RAN1#104bis-e, e-meeting, April, 2021.

[8] R1-2103257, “On resource allocation for power saving”, Samsung, RAN1#104bis-e, e-meeting, April, 2021.