3GPP TSG RAN WG1 #105e                                                R1-2105394

e-Meeting, May 19th – May 27th, 2021

Agenda Item:	8.8.1.3

Source:	MediaTek Inc.

Title:	Discussion on joint channel estimation for PUSCH

Document for:		Discussion

# Introduction

An LS [1] was sent to RAN4, triggering the discussion on phase continuity and power consistency for UL repetition. A reply LS [2] was sent back to RAN1 from RAN4

In this paper, we provide our views on some remaining open issues to enable the joint channel estimation.

# Discussion

Un-scheduled UL gap symbols

In Rel-15, if there are X un-scheduled UL OFDM symbols in-between the PUSCH or PUCCH repetitions, the typical UE behavior is to switch off the PA. In this case, phase continuity is not guaranteed.

To maintain the phase continuity, UE may have to keep the PA (as well as some Tx RF circuits) running during those unscheduled symbols. Additional power consumption at UE side is a problem. In addition to the wasted UE power, the existing OFF power requirements cannot be met. In this case, our suggestion is to de-prioritize this scenario and allow only back-2-back UL repetition.

Observation 1. In order to maintain phase continuity during those unscheduled symbols between UL repetition, UE power consumption would get higher and the OFF power requirement cannot be met.

Proposal 1: UE power consumption should be taken into account in addition to OFF power requirement.

Carrier aggregation

In our view, when UE is very far apart from the BS. The UL signal power should be concentrated in one single CC, rather than distributed into multiple CCs. Otherwise, the UL coverage of each CC is reduced, and eventually UE is probably only wasting Tx power. On the other hand, in order to keep the same power level on a target CC, there are also some limitation on other CCs, e.g., target CC is not impacted by other concurrent CC(s) that are configured for inter-band CA with dynamic power sharing and no change in any configured CCs that are part of configured intra-band UL CA. The scenario seems limited. Therefore, whether/how to support phase continuity and power consistency for UL repetition under CA scenario should be clarified with RAN4 feedback.

Observation 2: The use case to remain phase continuity and power consistency for UL repetition is not clear under CA scenario.

Proposal 2: Whether/how to support phase continuity and power consistency for UL repetition under CA scenario should be clarified with RAN4 feedback.

Dual connectivity

The problem in DC is very similar to CA. Note that the scheduling and power control in EUTRAN side are completely independent to those in NR side. Therefore, to maintain the power level on the target CC seems even more challenging than CA.

Proposal 3: Whether/how to support phase continuity and power consistency for UL repetition under DC scenario should be clarified with RAN4 feedback.

Gap symbols/slots with different power or PRB configurations

If power level and PRB configuration cannot be maintained, then the transmitter configuration will likely change and retuning may be required. There is no guarantee that the phase can be sufficiently maintained in such a scenario and identifying the specific configurations where it may be achievable would seem challenging.

Proposal 4: phase cannot be guaranteed to be maintained if other signals are present during the gap with different power settings or different PRB configuration.

Impact of different TPMI

Changing the TPMI precoder across repetitions may result in changing the Tx RF configuration and has the ability to impact phase continuity. It is also not clear to us whether from BS receiver side joint channel estimation could be performed if TPMI changes across repetitions.

Proposal 5: The same TPMI precoder is supported to guarantee phase continuity.

TA adjustment

This may lead to an additional change in the transmitter phase compensation required, which would impact phase continuity. Therefore, we would propose that “no TA adjustment” is a necessary condition.

Proposal 6: “no TA adjustment” is preferred to keep phase continuity.

# Summary

In this paper, we discussed the issue of phase continuity and power consistency for UL repetition. We have the following observations and proposals.

Observation 1. In order to maintain phase continuity during those unscheduled symbols between UL repetition, UE power consumption would get higher and the OFF power requirement cannot be met.

Proposal 1: UE power consumption should be taken into account

Observation 2: The use case to remain phase continuity and power consistency for UL repetition is not clear under CA scenario.

Proposal 2: Whether/how to support phase continuity and power consistency for UL repetition under CA scenario should be clarified with RAN4 feedback.

Proposal 3: Whether/how to support phase continuity and power consistency for UL repetition under DC scenario

Proposal 4: phase cannot be guaranteed to be maintained if other signals are present during the gap with different power settings or different PRB configuration.

Proposal 5: The same TPMI precoder is supported to guarantee phase continuity.

Proposal 6: “no TA adjustment” is preferred to keep phase continuity.

# Reference

[1] R4-2100014, “LS on PUCCH and PUSCH repetition”, RAN WG1

[2] R4-2103393, “Reply LS on PUCCH and PUSCH repetition”, RAN WG4