3GPP TSG RAN WG1 #105-e		                    R1-2105477

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.4.1

Source:	LG Electronics

Title:	Discussions on timing relationship enhancements in NTN

Document for:	   Discussion and Decision

- Introduction

Agreement:

For updating K_offset after initial access, at least one of the following options is supported:

- Option 1: RRC reconfiguration

- Option 2: MAC CE

FFS: Other options

Agreement:

- For determination of cell-specific K_offset in system information, down-select one option from below:

  - Option 1: Signal one offset value for K_offset

    - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point

  - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values

    - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Agreement:

Confirm the following working assumption:

Introduce K_offset to enhance the adjustment of uplink transmission timing upon the reception of a corresponding timing advance command.

Agreement:

When UE is not provided with K_offset value other than the one signaled in system information, the K_offset value signaled in system information is used for all timing relationships that require K_offset enhancement.

Agreement:

UE can be provided by network with a K_mac value.

When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

In RAN1#104b-e meeting, the following agreements are captured in the chairman’s note as:

In this contribution, we further discuss the remaining issues for timing relationship enhancements in NTN.

- Discussion

  - Signaling of K_offset in initial access

In the RAN1#103-e meeting, it was agreed K_offset configured in system information and used in initial access is supported by at least cell specific configuration. The first remaining issue is whether the signaling of K_offset is based on the implicit or explicit signaling. In case of implicit signaling based solution, two options are listed as follows. One option is K_offset equals to common TA or UE specific TA, and the other option is K_offset is derived from ra-ResponseWindow and an offset. Both options can provide the benefit in terms of signaling overhead reduction. However, coupling K_offset with other information such as TA or ra-ResponseWindow may restrict the network flexibility. Moreover, the main purpose of K_offset is to remove the ambiguity between gNB and UE caused by the long round-trip delay in NTN, so mixture of two different functions is not desirable. Lastly, explicit signaling is more cleaner way and more forward compatible compared to implicit signaling. Thus, explicit signaling of K_offset is preferred.

Proposal 1: Support explicit signaling of K_offset.

The second issue is determination of cell-specific K_offset in system information between two options where option 1 is to signal one offset value for K_offset, and option 2 is to signal a first offset value and a second offset value so that K_offset in option 2 is equal to the sum of the two offset values. The K_offset is introduced to cover RTT between feeder link and service link. In our view, the functionality for both options are the same. The one advantage of option 2 may be reducing some signaling overhead by replacing feeder link RTT to common TA. As mentioned above, implicit signaling for K_offset is not preferred. Besides, signaling via two different fields seems not to provide significant benefit in terms of flexibility. Thus, signaling one offset value for K_offset seems sufficient.

Proposal 2: For determination of cell-specific K_offset in system information, support signaling one offset value for K_offset (option 1).

The second issue is whether or not to additionally support beam-specific K_offset. The cell-specific K_offset signaling has a merit in the overhead perspective. However, in general, the cell size in NTN is much larger than that of TN, therefore differential timing offset would not be negligible between UE in cell center and cell boundary. For example, the maximum satellite beam size can be up to 1000km for LEO which results in up to 3.2 ms maximum differential delay within a satellite beam. Thus, beam-specific K_offset indication would be more suitable in NTN. If the signaling overhead in SIB is really a problem, we may consider beam-group specific K_offset. Compared to beam-specific K_offset, beam-group specific K_offset can effectively reduce the signaling overhead while it can provide finer K_offset granularity. In this beam group specific K_offset signaling, how to make beam-group can be further studied.

Proposal 3: Support beam (group)-specific K_offset signaling in addition to cell-specific K_offset in initial access.

  - Update of K_offset

In the last meeting, for updating K_offset after initial access, it is agreed at least one of the following options is supported:

•	Option 1: RRC reconfiguration

•	Option 2: MAC CE

In our understanding, both option 1 and 2 will work properly. Compared to option 2, option 1 has longer delay than option 2. In case of LEO-600, frequent update of K_offset is expected, and thus in order to achieve fast timing adjustment, option 2 is more preferable.

One related issue is when to apply the updated K_offset value. For solving this issue, two options can be considered. One is to apply new K_offset value X slot after the reception of update signaling. The other one is to apply new K_offset value X slot after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception. Among the options, our preference is latter one since gNB can remove some ambiguity whether the UE successfully decode the update signaling or not.

Another issue is how and when to trigger K_offset update. One possible way can include that update of K_offset is triggered when the estimated/calculated TA value is larger than the configured or most updated K_offset. In Rel-17 NTN, UE autonomous TA estimation/calculation is supported, so UE can initiate/trigger the update of K_offset.

Proposal 4: Support MAC-CE based K_offset update after initial access.

Proposal 5: Apply new K_offset value X slot after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception.

So far, it is agreed to introduce K_offset for the following timing cases as:

- The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH).

- The transmission timing of RAR grant/fallback scheduled PUSCH.

- The transmission timing of HARQ-ACK on PUCCH.

- The CSI reference resource timing.

- The transmission timing of aperiodic SRS.

- The transmission timing of HARQ-ACK on PUCCH to MsgB/Msg4

- The transmission timing of PUSCH in configured Type 2

- The transmission timing upon the reception of a corresponding TAC

It is still open whether K_offset values corresponding to above cases are the same or different. If the all of K_offset values are configured to be the same, the K_offset value should be the maximum value among the candidate K_offset values. If different K_offset values are allowed according to applied timing cases, system performance may be improved at the expense of increased signaling overhead and complexity. In this case, RAN1 also need to discuss whether the update of K_offset is applied all the timing cases or not.

  - Indication of PDSCH-to-HARQ_feedback timing

In RAN1#104 meeting, it was agreed to increase the range of K1 values from (0,…,15) to (0,…,31) for unpaired spectrum. Then, the FFS point is whether there is an impact on the size of K1 field in DL DCI. For non-fallback DCI, gNB preconfigure candidate K1 values via RRC, and then DL DCI is used to indicate actual K1 value. Thus, one simple solution without increasing the size of K1 field is simply increasing the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31) as below. Note that regarding the parameter, we think simple extension for rel-17 NTN UE (i.e., dl-DataToUL-ACK-r17) can be introduced.

dl-DataToUL-ACK-r17                         SEQUENCE (SIZE (1..8)) OF INTEGER (0..31)                             OPTIONAL, -- Need M

The other solutions can be increasing the size of K1 field in an explicit or implicit manner. For the implicit indication, we may consider slot index based as discussed for the HARQ process id identification, but it restricts scheduling flexibility. Therefore, among the listed potential solutions, we prefer to have a simple solution that only increases the range of dl-DataToUL-ACK.

In case of fall-back DCI (i.e., DCI format 1_0), the candidate K1 values are fixed to {1, 2, 3, 4, 5, 6, 7, 8} as in the TS38.213. Thus, increasing K1 range does not provide any benefit in this case. To resolve this issue, we can consider introducing fixed or configurable offset for non-fallback DCI case. For example, we can use minimum K1 value configured for the non-fallback DCI as an offset.

Proposal 6: Do not increase the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- For non-fallback DCI, increase the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31).

- For fallback DCI, consider introducing fixed or configurable offset.

  - RACH procedure triggered by PDCCH order

According to TS 38.213, the UE transmits a PRACH in the selected PRACH occasion for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to the minimum gap of NT,2+ ∆BWPSwitching+∆Delay+Tswitch msec. This minimum gap is depicted in the Figure 1. In NR, UE assumes initial TA = 0 for PRACH transmission.

Figure 1. An illustration of RACH procedure triggered by PDDCH order without timing offset

In Rel-17 NTN, the initial TA for PRACH transmission can be determined based on UE specific TA (estimated by UE) + common TA (if provided). Therefore, as discussed in RAN1#104e meeting, gNB may not know which PRACH occasion was selected by the UE according to the UE’s “next available PRACH occasion”. To alleviate this problem, there are two possible solutions. One is based on the gNB implementation which means gNB may perform additional blind detection for a PDCCH ordered PRACH. The other one is to introduce timing offset which may correspond to UE specific TA + common TA in addition to minimum gap. As shown in Figure 2, the additional timing offset can remove the ambiguity on PDCCH ordered PRACH transmission. Between two solutions, we think introducing additional offset is more cleaner way to resolve this issue.

Figure 2. An illustration of RACH procedure triggered by PDDCH order with timing offset

Proposal 7: For RACH procedure triggered by PDCCH order in Rel-17 NTN, define timing offset in addition to minimum gap, NT,2+ ∆BWPSwitching+∆Delay+Tswitch.

- Conclusion

In this contribution, we discussed on timing relationship enhancements for NTN. Based on the above discussion, we have following proposals:

Proposal 1: Support explicit signaling of K_offset.

Proposal 2: For determination of cell-specific K_offset in system information, support signaling one offset value for K_offset (option 1).

Proposal 3: Support beam (group)-specific K_offset signaling in addition to cell-specific K_offset in initial access.

Proposal 4: Support MAC-CE based K_offset update after initial access.

Proposal 5: Apply new K_offset value X slot after transmission of RRC reconfiguration complete message or acknowledgement for MAC-CE reception.

Proposal 6: Do not increase the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

- For non-fallback DCI, increase the range of dl-DataToUL-ACK in PUCCH-config IE from (0,…,15) to (0,…,31).

- For fallback DCI, consider introducing fixed or configurable offset.

Proposal 7: For RACH procedure triggered by PDCCH order in Rel-17 NTN, define timing offset in addition to minimum gap, NT,2+ ∆BWPSwitching+∆Delay+Tswitch.