3GPP TSG RAN WG1 #105-e    			       	 R1-2105484

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.5.3

Source: 	LG Electronics

Title: 	Discussion on accuracy improvement for DL-AoD positioning

Document for:   Discussion and decision

Introduction

This contribution discusses Rel-17 NR positioning enhancements specifically in the DL-AoD (Angle of Departure) based positioning solution.

Downlink positioning technique enhancement

Expected uncertainty window

Considering some advantages such as reducing Rx beam sweeping overhead or beamforming determination, the expected UL AoA/ZoA is agreed to enhance UL AoA measurements. In this respect, similar additional information such as the expected uncertainty window was agreed to be introduced in the previous meeting [1], the related agreement is as below:

Agreement: For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options: - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP] - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP] - Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced. FFS: details of signaling

In our understanding, in case of option 1, it considers proving the expected DL-AoD/ZoD in the view of TRP side to the UE. That is, it can be interpreted as providing UE with the beam width/adjacent beams. However, if the UE does not know its location, the provided information shall no longer be utilized. We think that using DL-AoA (option 2) is more straightforward way in theUE perspective, and it seems more reasonable than option 1 based on above discussion.

Observation 1:

- If single Expected DL-AoD/ZoD and uncertainty is supported for enhancement of DL-AoD, the provided information shall no longer be utilized when UE does not know its location.

Proposal 1:

- For indication of expected uncertainty window to enhance DL-AoD, the configuration of both expected DL-AoA/ZoA value and uncertainty (option 2) is supported.

Furthermore, even though UE receives above information related with an expected uncertainty window, UE-based positioning cannot make sure whether the information is valid or not at the current time. If the LMF provides the pre-calculated location of UE, it is helpful for UE to judge whether the information is useful or not since the UE can obtain rough information of current location by using RAT-independent techniques. So, pre-calculated location of UE needs to be additionally provided.

Observation 2:

- Even though UE receives information related with an expected uncertainty window, UE cannot sure whether the information is valid or not at the current time when UE-based positioning measurement is operated.

Proposal 2:

- For UE to judge whether configured expected uncertainty window with is useful or not, pre-calculated location of UE needs to be additionally provided for the UE-based positioning measurement.

Rx beam index information

In the DL-AoD positioning technique, LMF would estimate UE’s location by using information on PRS resource(s) and the corresponding RSRP measurement values for one or multiple TRP(s). Conceptually, the UE’s relative direction from a TRP could be found by one or two PRS resource index(es) which shows the maximum RSRP among a set of PRS resources. In case of the DL-AoD positioning, it would be reasonable to use a fixed reception beam across multiples PRS resources to estimate variation of RSRP measurements for the different PRS resources. Thus, for the effective location estimation, it was introduced that the UE can report its reception beam index for two or more DL PRS measurements if the PRS measurements were made with the same reception beam where the index of reception beam is up to 8. In the current phase, however, it is unclear how the LMF utilizes the reception beam index to find UE’s location and the location server cannot know the reception beam direction corresponding to the RX beam index, even if the UE reports the reception beam index. However, this feature needs to be further discussed since the reception beam highly affects RSRP measurement values.

Regarding the technical feature on when the reception beam index reporting can be used, the following cases can be considered. 1) The UE can use the same reception beam for the multiple PRS resources unless those PRS resources have a different source of QCL type-D RS. 2) The UE uses the same reception beam for the multiple PRS resources even if those PRS resources have a different source of QCL type-D RS regardless of the QCL type-D configuration. In our understanding, the second case is also feasible by implementation. However, even if the reception beam selection is up to the UE, further restriction might be required so that the UE uses a proper reception beam to ensure the positioning accuracy or to avoid worst case of reception beam selection. For example, the UE's fixed reception beam for the PRS measurements with regard to a specific TRP should be the best reception beam in terms of the RSRP.

In addition, there is another issue on how to interpret the same reception beam index reported in the different reporting time instances. Although the UE reports the same RX beam index at two different reporting time instances, the same reception beam index does not guarantee the same beam direction due to the variation of UE’s position and/or rotation. In Rel-17 NR positioning, it is necessary to discuss how to utilize the reception beam index to improve the location estimation, such as finding UE’s location when the UE is located between the transmission beams. Also, we need discussions on how to inform LMF of the reception beam information corresponding to the reported reception beam index, if needed.

Proposal 3:

- A further restriction would be required so that the UE uses a reception beam to avoid worst case of the reception beam selection, even if the UE can ignore QCL type-D configuration of the PRS resources to use a fixed reception beam for DL-AoD technique.

Proposal 4:

- Need discussions on how to utilize the reception beam index for the accuracy improvements of DL-AoD based positioning, such as finding UE’s location when the UE is located between the transmission beams.

Resolution problem

For the accuracy of the DL-AoD positioning technique, it would be necessary to discuss on how to estimate the UE’s location when the UE is located between two different transmission beams. As a simple example, a UE is located in a certain region between two transmission beams represented by PRS resource #1 and PRS resource #2, respectively, depicted in Figure 1. In this example, it can be envisioned that, by using the reception beam direction oriented to PRS resource#2, the RSRP measurement of the PRS resource #2 could be the maximum among overall PRS resources transmitted from the TRP. Based on the reported RSRP measurements, the LMF would estimate UE’s location on the direction of transmission beam of PRS resource #2.

Thus, Rel-17 NR positioning enhancements should discuss how to address the accuracy degradation according to the resolution problem. As a possible solution, we could consider the following two steps. Firstly, each TRP can transmit the PRS resources with the relatively wide beams as depicted on the left-hand of Figure 2 and the UE obtains PRS RSRP measurements with a fixed reception beam. Then, the LMF may roughly estimate that the UE is located in a certain area/direction. Secondly, after finding the rough location of the UE, the TRP can transmit the PRS resources with narrow beams where each beam covers relatively small area, and the UE needs to measure each PRS resource with multiple reception beams in order to find the best Rx/Tx beam pair. The best beam pair would be a combination of reception and transmission beam maximizing RSRP or maximizing the received signal power of the first arrival signal path.

In consideration of the current TS 37.355, at least, it is necessary to support that the UE needs to report the maximum RSRP for each PRS resource considering multiple reception beams.

Figure 1 An illustrative example of the UE’s location between two transmission beams.

Figure 2 An illustrative example of the DL-AoD positioning based on two-step method.

Proposal 5:

- To overcome beam resolution problem, 2-step beam adjustment procedure needs to be considered.

Conclusion

In this contribution, we have discussed potential enhancements for DL-AOD positioning in Rel-17, and our proposals are summarized below.

Expected uncertainty window

Observation 1:

- If single Expected DL-AoD/ZoD and uncertainty is supported for enhancement of DL-AoD, the provided information shall no longer be utilized when UE does not know its location.

Proposal 1:

- For indication of expected uncertainty window to enhance DL-AoD, the configuration of both expected DL-AoA/ZoA value and uncertainty (option 2) is supported.

Observation 2:

- Even though UE receives information related with an expected uncertainty window, UE cannot sure whether the information is valid or not at the current time when UE-based positioning measurement is operated.

Proposal 2:

- For UE to judge whether configured expected uncertainty window with is useful or not, pre-calculated location of UE needs to be additionally provided for the UE-based positioning measurement.

Rx beam index information

Proposal 3:

- A further restriction would be required so that the UE uses a reception beam to avoid worst case of the reception beam selection, even if the UE can ignore QCL type-D configuration of the PRS resources to use a fixed reception beam for DL-AoD technique.

Proposal 4:

- Need discussions on how to utilize the reception beam index for the accuracy improvements of DL-AoD based positioning, such as finding UE’s location when the UE is located between the transmission beams.

Resolution problem

Proposal 5:

- To overcome beam resolution problem, 2-step beam adjustment procedure needs to be considered.

Reference

- 3GPP RAN1, Chairman’s Notes RAN1#104bis-e meeting.