3GPP TSG RAN WG1 #105-e    			       	 R1-2105487

e-Meeting, May 10th – May 27th, 2021

Agenda Item:	8.5.6

Source: 	LG Electronics

Title: 	Discussion on other enhancements for positioning

Document for:   Discussion and decision

Introduction

This contribution discusses Rel-17 NR positioning enhancements specifically in positioning for RRC inactive state and other enhancements for latency reduction.

Positioning for UEs in RRC inactive state

By supporting positioning in RRC inactive state, the power consumption and latency would be reduced since UEs does not need to be in RRC connected mode for positioning measurement. However, there are potential areas that need to be enhanced because the LPP connection between UE and LMF is disconnected in RRC inactive state. In this document, the problems when UE tries to positioning measurement in RRC inactive state are discussed and suggest solutions to resolve them are also presented.

DL method in RRC inactive state

For Rel-16, there are two ways to give UEs an information related with DL positioning method, i.e. through LPP PDUs (i.e. NAS) or positioning system information. In case of assistance data, UEs in RRC inactive state can obtain the information via system information even though UEs cannot received the information through LPP PDUs. As a result, UE may estimate the position based on PRS measurement regardless of positioning methods (e.g. UE-based, UE-assist). However, in case of UE-assist method, since the UL resources that are for measurement report are not configured, we need to consider how to report DL-PRS measurement result to LMF. In addition, there might be some deployment scenarios where only minimum SIB is accessible where the minimum SIB does not contain posSIB. And then, if dedicated PRS configuration is possible, accurate location more estimation can be acquired.

Observation 1:

- To support DL positioning measurement in inactive states, followings are observed:

  - For both UE-assist and UE-based method,

    - Regardless of positioning methods (e.g. UE-based, UE-assist), UE may perform positioning measurement based on information that is transmitted through system information.

    - There might be some deployment scenarios where only minimum SIB is accessible where the minimum SIB does not contain posSIB.

    - If dedicated PRS configuration is possible, more accurate location estimation can be acquired.

  - For UE-assist method,

    - UEs cannot transmit measurement reports unless the resources for measurement report is configured.

To resolve this problem, dedicated signalling should be considered without having transition to RRC connected state. Since there are some procedures such as paging and RACH before transition to RRC connected state, paging and/or msg2 and/or msg4 and/or msgB can be used for it. Likewise, msg 1 and/or msg 3 and/or msg A can be used for UL data (e.g. measurement gap request, measurement report) transmission. In addition, if signalling is needed for simple purpose such as triggering of positioning measurement, WUS (Wake-up signal) also can be considered. Furthermore, since CG-based and RACH-based has currently been considered for SDT, we think they also can be used for positioning measurement.

Proposal 1:

- To support positioning measurement for UEs in RRC inactive state, at least followings should be considered:

  - gNB  UE: assistance data(PRS configuration), measurement report configuration, measurement gap configuration, LCS request)

    - Paging (DCI and/or PDSCH)

    - Msg 2 and/or msg4 (in four-step RACH), msgB (in two-step RACH)

    - WUS (if signalling is needed for simple purpose such as triggering of positioning measurement)

  - UE  gNB: measurement gap request, measurement report

    - Msg1 and/or msg3 (in four-step RACH), msgA (in two-step RACH)

UL method in RRC inactive state

In case of SRS, the SRS configuration is only provided by serving gNB in RRC connected mode and the configuration granularity of SRS for positioning is per BWP. From this perspective, if UL positioning measurement is supported for UE in RRC inactive state, we also need to consider how to provide UEs with SRS configuration in RRC inactive state. For this, some channels and signal as described above can be used to deliver the information such as SRS configuration, LCS request and etc.

Proposal 2:

- If UL positioning measurement is supported for UE in RRC inactive state, RAN1 needs to consider how to provide UEs with SRS configuration in RRC inactive state.

Other enhancements for latency reduction

AP/SP PRS

The support of AP/SP PRS would be beneficial for latency reduction, but it is questionable that the measurement accuracy/ quality of AP PRS could be guaranteed or not. In addition, for DL-TDOA and Multi-RTT techniques, PRSs transmitted from more than three TRPs should be triggered. To support AP PRS, several points need to be considered as follows.

    - PRS configuration structure: PRS is hierarchically configured with the frequency layer(s), TRP(s), PRS resource set(s), and PRS resource(s). If the gNB triggers a specific PRS resource, the frequency layer, TRP, and PRS resource set need to be indicated.

    - Triggering with reporting or PRS transmission: If the UE needs to report positioning measurements, AP PRS can be triggered with AP PRS reporting.

SFN PRS with Cyclic shift

In our view, it is essential to reduce the latency while ensuring accuracy performance above a certain level. The UE needs to obtain PRS measurement transmitted from many TRPs in one symbol. However, the UE’s DL PRS processing capability is limited, so it should also be considered.

SFN PRS with cyclic shift would be a good approach especially for the InF scenarios, since the propagation delay is about 1 us even in a huge industrial hall of length 30 m. A common PRS sequence applied with intentional time-domain delays can be used to detect TOA(s) from multiple gNBs/TPs, which is beneficial to the UE complexity since it only requires a single cross-correlation computation. Furthermore, by this approach, the orthogonal transmission of more than 12 TRPs is possible in a single symbol, so the UE can attain measurements for a lot of TRPs with low latency and low complexity.

More specifically, if multiple gNBs/TPs transmit a common PRS sequence with different delays and those delay information is known to the UE(s), the UE(s) can detect the ToA(s) for the PRS transmitted from each gNB/TP. It can be considered that the cyclic shifted version of a common sequence based on the current sequence and orthogonal sequence.

Proposal 3:

- NR should consider cyclic shift based SFN transmission of PRS.

  - Study on benefit of the simultaneous transmission of a common PRS sequence with different intentional cyclic time-domain delays.

In consideration of the physical layer latency, 1-symbol PRS resource needs to be introduced in Rel-17 NR positioning, and the coverage of 1-symbol PRS resource could be enough for the InF scenarios since the distance between a TRP and a UE is not far. For example, the size of small hall is 120m x 60m, and ISD is 20 m.

Proposal 4:

- Support 1-symbol PRS resource for Rel-17 NR positioning.

Conclusion

In this contribution, we have discussed positioning for UEs in RRC inactive state and other enhancements for latency reduction, and our proposals are summarized below.

Positioning for UEs in RRC-inactive state

Observation 1:

- To support DL positioning measurement in inactive states, followings are observed:

  - For both UE-assist and UE-based method,

    - Regardless of positioning methods (e.g. UE-based, UE-assist), UE may perform positioning measurement based on information that is transmitted through system information.

    - There might be some deployment scenarios where only minimum SIB is accessible where the minimum SIB does not contain posSIB.

    - If dedicated PRS configuration is possible, more accurate location estimation can be acquired.

  - For UE-assist method,

    - UEs cannot transmit measurement reports unless the resources for measurement report is configured.

Proposal 1:

- To support positioning measurement for UEs in RRC inactive state, at least followings should be considered:

  - gNB  UE: assistance data(PRS configuration), measurement report configuration, measurement gap configuration, LCS request)

    - Paging (DCI and/or PDSCH)

    - Msg 2 and/or msg4 (in four-step RACH), msgB (in two-step RACH)

    - WUS (if signalling is needed for simple purpose such as triggering of positioning measurement)

  - UE  gNB: measurement gap request, measurement report

    - Msg1 and/or msg3 (in four-step RACH), msgA (in two-step RACH)

Proposal 2:

- If UL positioning measurement is supported for UE in RRC inactive state, RAN1 needs to consider how to provide UEs with SRS configuration in RRC inactive state.

Other enhancements for latency reduction

Proposal 3:

- NR should consider cyclic shift based SFN transmission of PRS.

  - Study on benefit of the simultaneous transmission of a common PRS sequence with different intentional cyclic time-domain delays.

Proposal 4:

- Support 1-symbol PRS resource for Rel-17 NR positioning.