3GPP TSG RAN WG1 #105-e			R1-2105601

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.12.2 Mechanisms to improve reliability for RRC_CONNECTED UEs

Title:	Discussion on reliability enhancement for RRC_CONNECTED UEs

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Agreements:

For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast,

- Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback.

  - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled

    - FFS details on RRC signalling and DCI indicating.

  - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback.

    - FFS whether enabling or disabling the feedback is the default mode.

- Option 2: RRC indicates enabling/disabling.

- FFS: whether down-selection between option 3 and option 2 is needed or support the both options.

- FFS: enabling/disabling by MAC-CE.

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

In the past RAN1 meetings, the following agreements were reached on MBS reliability enhancement [1], [2]:

In this contribution, we provide our views on the potential reliability enhancements to support the NR multicast and broadcast.

# Discussion

In the past RAN1 meetings, the discussion on HARQ-ACK feedback enabling/disabling mechanisms for MBS continued and further down-selection may be performed. In our view, option 3, i.e., RRC configures the enabling/ disabling function and DCI indicates enabling /disabling, is more beneficial as it can provide more flexibility for the network to enable/disable the HARQ feedback.

Proposal 1: For MBS HARQ-ACK feedback enabling/disabling, option 3 (i.e., RRC configures the enabling/ disabling function and DCI indicates enabling /disabling) is supported.

For retransmission, using group-common PDCCH, i.e., PTM transmission scheme 1, has the advantage of low signaling overhead when the number of the UEs to be scheduled is large. However, as another option, using UE-specific PDCCH to schedule the PDSCH for MBS, e.g., using PTP transmission or PTM transmission scheme 2, may be beneficial in some cases, e.g., when the number of the UEs to be scheduled is small, for the purpose of improving the robustness of the DCI transmission, etc. Therefore, we propose PTM transmission scheme 2 should be supported for retransmission for MBS.

Proposal 2: PTM transmission scheme 2 should be supported for MBS retransmission.

Both ACK/NACK based HARQ feedback scheme and NACK only based HARQ feedback scheme are supported for NR MBS. Also, a UE may receive the MBS service and unicast service simultaneously. In some cases, a PUCCH resource allocated for an MBS transmission e.g., TB 1, using NACK only HARQ feedback may colloid with a PUCCH resource allocated for a transmission (either MBS or unicast), e.g., TB 2, using ACK/NACK HARQ feedback. Behaviour need to be defined for the UE to handle this case. As the PUCCH resource for NACK only HARQ is shared by many UEs, it is more reliable for the UE to multiplex the HARQ feedback for TB 1 on the PUCCH resource allocated for TB 2 as it is dedicated to the UE.

Proposal 3: When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUCCH resource allocated for a transmission (2nd transmission) using ACK/NACK HARQ feedback, the UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission.

Another question is when the UE performs the multiplexing, what type, e.g., NACK only or ACK/NACK based, HARQ feedback should be used for the TB 1. As the PUCCH resource is dedicated to the UE, we think, instead of still sending NACK only based feedback, it will be beneficial for the UE to send ACK/NACK based feedback for TB 1. By doing this, it can help gNB know the accurate HARQ feedback result for TB1 which can help the gNB to make better decision on the scheduling and rescheduling.

Proposal 4: When a UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUCCH.

Similar issue occurs for the case when a PUCCH resource allocated for an MBS transmission e.g., TB 1, using NACK only HARQ feedback may colloid with a PUSCH transmission. We think the similar behaviour can be used. So, we propose the following proposals.

Proposal 5: When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUSCH transmission, the UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource.

Proposal 6: When a UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUSCH.

In the RAN1 #104-e meeting, it has been agreed to support HARQ-ACK feedback for SPS group-common PDSCH for MBS and the details will be discussed in AI 8.12.2. Similar to dynamic scheduling, ACK/NACK based and NACK-only based HARQ-ACK feedback have their pros and cons. Both of them should be considered for MBS SPS PDSCH.

Proposal 7: Both ACK/NACK based HARQ-ACK feedback and NACK-only based HARQ-ACK feedback should be considered for MBS SPS PDSCH.

As the gNB needs to activate or deactivate a group of UEs in MBS SPS, it is also important to enhance the reliability of the activation DCI and deactivation DCI. Supporting HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI should be also considered.

Proposal 8: Consider to support HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI.

# Conclusion

In this contribution, we have the following proposals

Proposal 1: For MBS HARQ-ACK feedback enabling/disabling, option 3 (i.e., RRC configures the enabling/ disabling function and DCI indicates enabling /disabling) is supported.

Proposal 2: PTM transmission scheme 2 should be supported for MBS retransmission.

Proposal 3: When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUCCH resource allocated for a transmission (2nd transmission) using ACK/NACK HARQ feedback, the UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission.

Proposal 4: When a UE multiplexes the HARQ feedback for the 1st transmission on the PUCCH resource allocated for the 2nd transmission, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUCCH.

Proposal 5: When a PUCCH resource allocated for an MBS transmission (1st transmission) using NACK only HARQ feedback collides with a PUSCH transmission, the UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource.

Proposal 6: When a UE multiplexes the HARQ feedback for the 1st transmission on the PUSCH resource, instead of gernerating the NACK only feedback, the UE generates the ACK/NACK based HARQ feedback for the 1st transmission and multiplexes it on the PUSCH.

Proposal 7: Both ACK/NACK based HARQ-ACK feedback and NACK-only based HARQ-ACK feedback should be considered for MBS SPS PDSCH.

Proposal 8: Consider to support HARQ-ACK feedback for MBS SPS activation DCI and deactivation DCI.

# References

- Chairman Notes, RAN1 #104-e, January 25th – February 5th, 2021

- Chairman Notes, RAN1 #104b-e, April 12th – 20th, 2021