3GPP TSG RAN WG1 #105-e			R1-2105607

e-Meeting, May 10th – 27th, 2021

Title:	[Draft]Reply LS to SA4 on Status Update on XR Traffic

Response to:	R1-2104023(S4-210684)

Release:	Rel-17

Work Item:	FS_XRTraffic, FS_NR_XR_eval

Source:	ZTE, Sanechips[To be RAN1]

To:	SA4

Contact Person:

Name:	Yuzhou Hu

E-mail Address:	<Hu.Yuzhou@zte.com.cn>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

- Overall Description:

RAN1 would like to thank SA4 on the update of their progress in terms of VR1 traffic, Q trace based evaluation as well as considerations on RAN1 evaluation to push additional information including priority related packet importance into 5GS.

RAN1 has so far finalized the traffic model for DL single stream and is in the course of discussing the DL multiple streams. One of the candidate options for DL multiple stream, i.e. FoV + omnidirectional stream is related to related to the ongoing discussing of VR1 traffic characteristic. Thus further SA input on the QoS setting including PDB/PER requirements for FoV and non-FoV streaming is needed.

Given RAN1 agreement on traffic models, the following values required of SA test channels are derived from the baseline settings

DL

VR/AR

| Resource type | Delay-critical GBR |
| --- | --- |
| Packet Delay Budget | 10 ms |
| Maximum Data Burst Volume | 30Mbps@60fps: 93750 Bytes 45Mbps@60fps: 140625 Bytes |

CG

| Resource type | Delay-critical GBR |
| --- | --- |
| Packet Delay Budget | 15 ms |
| Maximum Data Burst Volume | 30Mbps@60fps: 93750 Bytes 8Mbps@60fps: 25000 Bytes |

UL

Pose/Control

| Resource type | Delay-critical GBR |
| --- | --- |
| Packet Delay Budget | 10 ms |
| Maximum Data Burst Volume | 300 Bytes |

AR Video

| Resource type | Delay-critical GBR |
| --- | --- |
| Packet Delay Budget | 60 ms |
| Maximum Data Burst Volume | 10Mbps@60fps: 125000 Bytes 20Mbps@60fps: 250000 Bytes |

Action 3)	To kindly support SA4 in the definition of appropriate and representative test channels and provide feedback and comments on the initial definition in clause 7.6 of the PD.

Response: 	RAN1 has agreed to perform evaluation based on frame level model and a unified mapping relationship between FER and PER is supposed to be agreed by RAN1 in the first place to guarantee the Q trace evaluation in SA4.

2. Actions:

To 3GPP TSG SA WG4: RAN1 kindly asks SA4 to take above information into account.

3. Dates of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #106-e	16 Aug – 27 Aug 2021	eMeeting

TSG RAN WG1 Meeting #106b-e	11 Oct – 19 Oct 2021	eMeeting