3GPP TSG RAN WG1 Meeting #105-e                                                                 	   R1-2105777

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.8.3

Source:	Lenovo, Motorola Mobility

Title:	Type A PUSCH repetitions for Msg3

Document for:	Discussion and decision

# Introduction

In RAN#90e, a WID on NR coverage enhancements  was approved. It was further updated in RAN#91e [1]. One objective of the WI is to specify mechanism(s) to support Type A PUSCH repetitions for Msg3.

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide analysis on the differentiation between UEs supporting/not-supporting Msg3 repetition, and indication of the number of repetitions for Msg3 transmission.

# Discussion

Differentiation between CE UEs and legacy UEs

For contention based random access, the gNB does not know UE capability before Msg3 scheduling. Therefore, mechanisms are needed to differentiate UEs supporting Msg3 repetition from UEs not supporting it. It was agreed in RAN1#104b-e that gNB could identify UEs supporting/requesting Msg3 repetitions from the received preamble in separate PRACH resources:

Agreement: For Msg3 PUSCH repetition,  support the following modified Option 2-1.

- Option 2-1: For UE requested Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can request Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.).

  - Whether a UE would request is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is requested by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual

- FFS details if any.

One point FFS is how the separate PRACH resources are configured. From our view, the gNB could flexibly determine if configuring separate ROs for Msg3 PUSCH repetition or configuring separate preambles and shared ROs for Msg3 PUSCH repetition. It is noted that latter scheme introduces preamble space fragmentation. However, by properly configuring the condition (e.g., a RSRP threshold) for Msg3 repetition, only limited number of UEs in the system would potentially request Msg3 repetition. Therefore, it is expected that the contribution of preamble fragmentation due to Msg3 repetition to not be significant. On the other hand, if the preamble fragmentation does affect performance impact e.g., for legacy UEs, the gNB could configure separated ROs for the UEs requesting Msg3 repetition.

Proposal 1: Based on gNB configuration, separate ROs or separate preambles with shared ROs could be configured for Msg3 PUSCH repetition.

Another FFS is how the condition for requesting Msg3 repetition is defined. One example in the agreements is that the condition is based on measured SS-RSRP threshold, from which gNB knows the UE channel conditions. Specifically, UE could compare the measured SS-RSRP with a threshold, and if it is lower than the threshold, the UE will choose a preamble from the dedicated preambles for Msg3 repetition. Otherwise, the UE may just use the legacy preambles.

For flexibility, the SS-RSRP threshold could be configured by gNB. A reasonable threshold could be configured depending on the PRACH capacity and the number of different types of UEs in the cell. For example, if there are comparatively large ratio of legacy UEs in the system, a higher threshold could be configured such that only limited number of preambles is dedicated for UEs requesting Msg3 repetitions. On the other hand, if e.g., majority of the UEs in the system support Rel.17 CE UEs, a smaller threshold could be configured such that the gNB could know the UEs capable of Msg3 repetition.

Proposal 2: The gNB could configure a RSRP threshold for the UEs capable of Msg3 PUSCH repetition for preamble selection in a dedicated PRACH resource.

Indication of the number of repetitions for Msg3 transmission

Regarding the indication of the number of repetitions for Msg3 initial transmission, it was agreed in RAN1#104b-e that,

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted.

- FFS additionally using MAC RAR for indication.

And it was required to maintain backward compatibility of legacy UE interpretation of RAR, as agreed in RAN1#104e,

Agreements:

- Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively

Actually, from high level, there are two mechanisms to meet the requirement of backward compatibility of not impacting legacy UE interpretation of RAR.

- Alt.1: the UL grant for scheduling Msg3 repetition is unchanged.

- Alt.2: the UL grant for scheduling Msg3 repetition could be changed, and the RAR for CE UEs is placed at the end of RAR MAC PDU and is interpreted as padding bits for legacy UEs.

Alt.2 enables more flexible Msg3 scheduling in time domain. For example, for the solution of introducing a new PUSCH TDRA table with repetitions for Msg3 repetition, if the size of TDRA table shall be aligned with legacy (i.e., Alt.1), the flexibility of time domain resource allocation will be impacted, especially considering a few candidates of repetitions might be introduced for Msg3.  Placing RAR for CE UEs in the legacy padding bits enables larger RAR for indicating Msg3 repetition and does not impact legacy UE behavior of interpreting RAR.

Proposal 3: To enable flexible scheduling of Msg3 with repetition and meet the requirement of backward compatibility of not impacting UE interpretation of RAR, the RAR for CE UEs is placed at the end of RAR MAC PDU and is interpreted as padding bits for legacy UEs.

# Conclusions

As a summary, we have the following proposals on Type A PUSCH repetitions for Msg3,

Proposal 1: Based on gNB configuration, separate ROs or separate preambles with shared ROs could be configured for Msg3 PUSCH repetition.

Proposal 2: The gNB could configure a RSRP threshold for the UEs capable of Msg3 PUSCH repetition for preamble selection in a dedicated PRACH resource.

Proposal 3: To enable flexible scheduling of Msg3 with repetition and meet the requirement of backward compatibility of not impacting UE interpretation of RAR, the RAR for CE UEs is placed at the end of RAR MAC PDU and is interpreted as padding bits for legacy UEs.

# References

RP-210855, New WID on NR coverage enhancements, RAN#91e, China Telecom