3GPP TSG RAN WG1 #105-e                                                   R1-2105801

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.6.1.3

Source:			ASUSTeK

Title:                 Discussion on aspects related to duplex operation

Document for: Discussion

- Introduction

In RP-210918, “Revised WID on support of reduced capability NR devices”, which has been approved in RAN#91-e, has the objective of specifying support for UE complexity reduction features [1]:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]:

  - Reduced maximum UE bandwidth:

    - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz.

    - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

  - Reduced minimum number of Rx branches:

    - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

    - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

    - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

  - Maximum number of DL MIMO layers:

    - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

    - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

  - Relaxed maximum modulation order:

    - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE.

    - No other relaxations of maximum modulation order are specified for a RedCap UE.

  - Duplex operation:

    - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

In RAN1#104-e and #104b-e [2][3], agreements and working assumptions on half duplex operation have been reached by RAN1:

RAN1 #104e

Agreements:

For HD-FDD, for cases (if any) where collision handling needs to be specified, then the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum are used as a starting point if deemed applicable.

Agreements:

- (Working assumption) For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

- Sending an LS to RAN4 to inform the above working assumption, and to ask for feedback if any

  - The LS will not include the two FFS bullets

Agreements:

- For HD-FDD operation for RedCap UEs, consider at least the following DL/UL collision cases collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

  - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

    - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH, or RO

  - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

    - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

  - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

  - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

    - e.g., PUSCH, PUCCH, PRACH, SRS

  - Case 6: Monitoring for UL cancellation indication (if supported) while transmitting in UL

  - Case 7: Collision due to BWP switching (if supported)

  - Case 8: Dynamic or semi-static DL vs. valid RO

  - Case 9: Collision due to direction switching

RAN1 #104b-e

Agreements:

For Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum.

- FFS whether the timeline is extended to include the RX/TX switching time for HD-FDD

For Case 4: dynamically scheduled DL reception vs. dynamic scheduled UL transmission, reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum

- That is, it is considered as an error case if a dynamically scheduled DL reception overlaps with a dynamically scheduled UL transmission

For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier/single cell in unpaired spectrum

- The semi-statically configured DL reception may include PDCCH (excluding ULCI), SPS PDSCH, CSI-RS or PRS.

  - FFS on PDCCH carrying ULCI, including whether or not it is supported by RedCap UEs (including potential difference between HD vs. FD RedCap UEs)

- The dynamically scheduled UL transmission may include PUSCH, PUCCH, SRS or PRACH triggered by PDCCH order

Agreements:

For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both cell specific higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- FFS on cell-specifically configured DL reception vs. cell-specifically configured UL transmission

- FFS: whether or not there are conditions that need to be considered

Working assumption: For HD-FDD, no additional UE behavior for switching position determination is specified as compared to the existing specification.

Conclusion: Enhancement for potential UL and DL collision handling due to TA misalignment is not considered for Type-A HD-FDD operation of RedCap UEs

Working Assumption: For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

- A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell

- A HD-FDD UE is not expected to receive in the downlink earlier than[NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell

- FFS NTX-RX and NRX-TX

- FFS: how it jointly works with the agreement for other collision cases

Working assumption:

- If a dynamically scheduled UL transmission overlaps with an SSB, down-select one of the following options:

  - Option 1: Follow the handling of case 2 that dynamic UL is prioritized over SSB

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamic UL

  - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- If a semi-static configured UL transmission overlaps with an SSB, down-select one of from the following options

  - Option 1: Up to gNB configuration to avoid such collision and if it happens it is an error case

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL

  - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- FFS: whether/how to account for Tx/Rx switching time before and after the set of SSB symbols

- FFS: whether or not the semi-static configured UL transmission includes a valid RO

In this contribution, we discuss the collision handling bewteen DL reception and UL transmission on HD-FDD operation for RedCap UE.

- Discussion

The agreement has been made in RAN1# 104-e on the collision handling rule for HD-FDD operation of RedCap UE that principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum will be used as a starting point. For collision handling rule on case of dynamic scheduling DL reception /UL transmission collide with semi-static configured UL/DL and dynamic scheduling DL collide with dynamic scheduling UL, principles in legacy TDD operation are reused as agreed in RAN1 #104b-e. For case on semi-static configured DL collide with semi-static configured UL, concern was raised on cell specific configured DL colliding with cell specific configured UL. If gNB avoid cell specific DL and UL colliding with each other as legacy R15/16 TDD operation, non-RedCap UE which is capable of full duplex operation may be impacted since gNB can perform reception and transmission simultaneously in paired spectrum. Cell specific configured DL comprise SSB, PDCCH in CSS, and corresponding PDSCH, and cell specific configured UL comprise RO. It may consider that leave to UE implementation when cell specific configured DL collide with specific configured UL. Since UE may not always receive SSB or acquire system information, UE may send PRACH if UE need to perform random access procedure.

Proposal 1: For Case 3 on cell specific configured DL colliding with cell specific configured UL, consider leave to UE implementation whether perform DL reception or UL transmission.

For case of dynamic scheduled UL collide with SSB, working assumption has been made for down selecting from 3 options. For option 1: dynamic scheduled UL is prioritized over SSB as same handling on dynamic scheduled UL colliding with semi static configured DL, this is reasonable to follow the dynamic UL grant as UE may need to perform transmission and does not need to receive SSB. For option 2: SSB is prioritized over dynamic scheduled UL, it may increase the scheduling latency and reduce resource efficiency since UE ignore the UL scheduling grant and gNB send a UL grant which cannot schedule UL. For option 3: leave to UE implementation, it may have flexibility for UE acquire SSB when UE need to perform synchronization and acquire MIB or follow the dynamic scheduling grant if UE does not need to perform DL reception.

Proposal 2: For dynamically scheduled UL colliding with SSB, consider selecting option 1, option 3 from working assumption.

For case of semi static configured UL transmission colliding with SSB, there are also 3 options in working assumptions. For option 1: gNB configuration to avoid collision, it may decrease the scheduling flexibility and may be complex for gNB always avoid semi static configured UL overlapping with SSB. For option 2: SSB is prioritized over semi static configured UL, since gNB in paired spectrum is capable of performing reception and transmission simultaneously, UE may transmit semi static configured UL if UE does not need to receive SSB as the behaviour in option 3.

Proposal 3: For semi static configured UL colliding with SSB, consider selecting option 3 from working assumption.

For valid RO defined in R15/16, all ROs are valid in paired spectrum, and a RO is valid if it is within UL symbols or it start Ngap symbols after last downlink symbol and SSB in unpaired spectrum. The Ngap symbols can avoid the interference of UL and DL in TDD system, which may not be necessary for UE in HD-FDD operation since UE is served in paired spectrum. If a RO collide with DL reception or DL /UL switching gap, UE is not able to send PRACH in the RO. For case of dynamic scheduled DL colliding with valid RO, dynamic scheduling DL may be prioritized or prioritization may be left to UE implementation, i.e., UE may perform DL reception if UE does not need to perform random access procedure or send PRACH if UE need to perform random access procedure.

Proposal 4: For dynamic scheduled DL colliding with valid RO, consider prioritizing dynamic scheduling DL over valid RO or leaving to UE implementation.

- Conclusion

In this contribution, we discuss the collision handling rule for RedCap UE operating in HD-FDD mode and have following proposal.

Proposal 1: For Case 3 on cell specific configured DL colliding with cell specific configured UL, consider leave to UE implementation whether perform DL reception or UL transmission.

Proposal 2: For dynamically scheduled UL colliding with SSB, consider selecting option 1, option 3 from working assumption.

Proposal 3: For semi static configured UL colliding with SSB, consider selecting option 3 from working assumption.

Proposal 4: For dynamic scheduled DL colliding with valid RO, consider prioritizing dynamic scheduling DL over valid RO or leaving to UE implementation.

- Reference

- RP-210918,” Revised WID on support of reduced capability NR devices”.

- Chairman’s Notes RAN1#104-e.

- Chairman’s Notes RAN1#104b-e.