3GPP TSG RAN WG1 #105-e			R1-2105873

e-Meeting, May 10th – 27th, 2021

Source:	WILUS Inc.

Title:	Discussion on enhancement for unlicensed URLLC/IIoT

Agenda item:	8.3.2

Document for:	Discussion

- Introduction

From RAN1#102-e to RAN1#104b-e meeting [1]-[4], the followings were agreed to support UE initiated COT for FBE operation for URLLC in unlicensed spectrum under the NR_IIOT_URLLC_enh work item.

Agreements at RAN1#102-e: - For semi-static channel access mode, - When gNB operates as an initiating device - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT - When a UE operates as an initiating device - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB - When the gNB shares a COT initiated by a UE during an FFP associated with the UE - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE - FFS whether/how to support additional restrictions to the idle period Agreements at RAN1#102-e: - For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode - FFS the case when the UE is IDLE/INACTIVE mode Agreements at RAN1#102-e: - UE-to-gNB COT sharing in semi-static channel access mode is supported. - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP. - FFS details - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP. - FFS whether/how UE to gNB COT sharing when the gap is >16us Agreements at RAN1#102-e: For semi-static channel access mode, o Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT. o FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT. Agreements at RAN1#102-e: - For semi-static channel access mode, - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling. - FFS on to be provided by SIB-1 - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters Conclusion at RAN1#103-e: If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X. If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y. Note 1: One of the devices X and Y is a UE and the other is its serving gNB. Note 2: Whether or not there is additional restriction on idle period is still FFS. Agreements at RAN1#103-e: - The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s). - Note: UE initiated FBE configuration is configured per serving cell Agreements at RAN1#103-e: In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT. - Note: Any value for the period, shall be at least 1ms and at most 10ms. - Note: Aim for low complexity operation to handle gNB and UE COT interactions Agreements at RAN1#103-e: In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT. - UE determines the initiator of a COT based on at least one of the following alternatives: - Alt 1: Introduce additional bit field in the scheduling DCI - Alt 2: Based on ChannelAccess-CPext field in DCI - Alt. 3: Based on a predetermined rule(s) - Alt. 4: Based on RRC signalling - Alt. 5: Based on MAC CE - FFS other alternatives - FFS on overriding possibility and/or the assumption. - Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements at RAN1#103-e: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreement at RAN1#104-e: In semi-static channel access mode when a UE can operate as initiating device, - Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period - Alt-b: Determination based on the rules applied for a configured UL transmission Agreement at RAN1#104-e: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. Agreement at RAN1#104-e: - PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG - FFS whether/how to enhance Agreement at RAN1#104b-e: - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance. Agreement at RAN1#104b-e: - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details. Agreement at RAN1#104b-e: - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details. Conclusion at RAN1#104b-e: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

In this contribution, we discuss uplink enhancement for unlicensed URLLC and its specification impact to support 1) determination of gNB and UE’s COT for configured UL transmission and for dynamically scheduled UL transmission, and 2) enhanced/harmonized configured-grant PUSCH for URLLC in the unlicensed band.

Discussion on UL enhancement for Unlicensed URLLC/IIoT

On determination of gNB’s and UE’s COT for CG and DG

Inter-relation between FFP configurations and UE-initiated/gNB-shared COT

Regarding COT-associated indication for scheduled UL transmission, from the perspectives of UE’s transmission, it can be allowed to transmit UL transmission at the time-point except for the idle period of any FFP associated with the UE in the case that the UE initiates a COT without any knowledge regarding whether the UL transmission belongs to the idle period of gNB’s FFP or not. However, in the other case that the UE is indicated by gNB that the scheduled UL transmission is within shared gNB COT and the time-point is located at the idle period of gNB’s FFP, the UE behavior for scheduled UL transmission at the idle period of gNB’s FFP can be different from the previous case. Therefore, to provide the gNB capability to control the scheduled UE transmission, it seems beneficial to have a signaling mechanism for indicating whether the scheduled UL transmission is belonging to UE-initiated COT or gNB-shared COT. We prefer to have an explicit or implicit signaling mechanism based on DCI of scheduled UL transmission to determine whether a scheduled UL transmission should be transmitted according to UE-initiated COT or gNB-shared COT.

- Proposal 1: We support an explicit or implicit signaling mechanism based on the content in the scheduling DCI to determine whether a scheduled UL transmission should be transmitted according to UE-initiated COT or gNB-shared COT.

Regarding COT-associated indication for a configured UL transmission, it was discussed from RAN1#103-e to RAN1#104b-e meeting, and it was agreed as the following, but UE’s assumption for COT association of the configured UL transmission should be decided.

Agreements at RAN1#103-e [2]: In semi-static channel access mode: - When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration. - When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE: - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT. - FFS on other conditions for determining the corresponding UE or gNB initiated COT - Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT. Agreements at RAN1#104-e [3]: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

When the configured UL transmission is confined within a gNB FFP before the idle period of that gNB FFP, it seems beneficial to follow the same behavior regardless of whether or not the configured UL transmission is starting from being aligned with a UE’s FFP boundary if a UE has already determined that the gNB has initiated FFP. Therefore, we propose to support Alt-a as the UE behavior for the configured UL transmission, when a UE can operate as UE initiated COT.

- Proposal 2: We support Alt-a with the same UE behavior on determination of gNB and UE’s COT for the configured UL transmission regardless of whether or not the configured UL transmission is starting from being aligned with a UE’s FFP boundary.

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Enhanced configured-grant PUSCH for URLLC in an unlicensed band.

An LBT is considered as a channel access procedure by default to perform UL/DL transmission in unlicensed band operation. Depending on the success and failure of LBT, a separate operation may be required for the reception by a UE and transmission from a gNB. In particular, the SS/PBCH block in the NR system for the initial access procedure must be transmitted successfully from the base station to a UE for time/frequency synchronization and cell ID detection by the UE.

However, in the unlicensed band, because the transmission of the SS/PBCH block may vary depending on the results of the LBT performed by the gNB, the gNB and the UE are operated by setting the position of the candidate SS/PBCH block reception with an assumption of multiple candidate positions of same SS/PBCH block index for the UE.

For configured grant PUSCH, in the case of the PUSCH repetition type A specified from Rel-15 and the PUSCH repetition type B specified in the Rel-16 URLLC work item, those operations were originally set for licensed operation. Therefore, when operating in the licensed band, the UE always assumes that the gNB transmits the SS/PBCH block at the positions configured to the UE, so the time and frequency resources in the positions for the SS/PBCH block reception are excluded at the resources to perform the UE’s PUSCH repetition type A/type B. However, for the transmission of the SS/PBCH block in the unlicensed spectrum, as described above, the locations of the candidate SS/PBCH blocks that the UE should assume for the reception of the SS/PBCH block are set to multiple positions for the same index of SS/PBCH block. In particular, when the UE detects a certain SS/PBCH block index since the UE assumes that the number of transmitted SS/PBCH blocks with the same SS/PBCH block index is not larger than one from the UE’s perspective in Rel-16, the corresponding resources at other candidate positions of SS/PBCH block with the same index can be used as a time/frequency resource for PUSCH repetition type A or type B. Therefore, it should be further discussed whether or not to allow transmission of configured grant PUSCH with repetition at candidate SS/PBCH block positions configured with the same SS/PBCH block index after the UE successfully detects the SS/PBCH block index.

- Proposal 3: It should be further discussed whether or not to possibly transmit configured-grant PUSCH with repetition at candidate SS/PBCH block positions for the same SS/PBCH block index after the detection of the SS/PBCH block index.

Additionally, at the RAN1#104-e meeting, it was agreed that PUSCH repetition Type-B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG and it is FFS whether/how to enhance this. Regarding enhancement of PUSCH repetition Type-B for URLLC/IIoT in the unlicensed band, it should be further discussed how to handle on LBT gap/switching gap between segmented transmissions of nominal repetition by slot boundary or between non-contiguous PUSCH type-B repetitions by DL reception (e.g., candidate SS/PBCH blocks, or others). In Rel-16 URLLC WI, it was specified to have an RRC parameter for DL-UL switching, i.e., numberOfInvalidSymbolsForDL-UL-Switching-r16 as RRC signaling. However, this parameter does not apply to segmented transmission due to SS/PBCH blocks or CORESET#0 as well as non-contiguous UL transmission for PUCCH type B repetition. It can be simply extended for URLLC/IIoT operation in the unlicensed band by reusing numberOfInvalidSymbolsForDL-UL-Switching-r16 for both DL-UL switching and LBT gap for both segmented UL repetitions, which occur after SS/PBCH block or CORESET#0 and non-contiguous UL transmission for PUSCH repetition type B repetition.

- Proposal 4: To enhance PUSCH repetition Type-B for URLLC/IIoT in the unlicensed band, it should be further discussed how to handle on LBT gap/switching gap between segmented transmissions of nominal repetition by slot boundary or between non-contiguous PUSCH Type-B repetitions by DL reception (e.g., candidate SS/PBCH blocks, or others).

Conclusion

In this contribution, we have discussed issues to support determination of gNB and UE’s COT for configured UL transmission and for dynamically scheduled UL transmission and enhanced configured-grant PUSCH in NR-U and URLLC introduced in Rel-16 in unlicensed band. As a conclusion, we summarize our view as follows:

- Proposal 1: We support an explicit or implicit signaling mechanism based on the content in the scheduling DCI to determine whether a scheduled UL transmission should be transmitted according to UE-initiated COT or gNB-shared COT.

- Proposal 2: We support Alt-a with the same UE behavior on determination of gNB and UE’s COT for the configured UL transmission regardless of whether or not the configured UL transmission is starting from being aligned with a UE’s FFP boundary.

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

- Proposal 3: It should be further discussed whether or not to possibly transmit configured-grant PUSCH with repetition at candidate SS/PBCH block positions for the same SS/PBCH block index after the detection of the SS/PBCH block index.

- Proposal 4: To enhance PUSCH repetition Type-B for URLLC/IIoT in the unlicensed band, it should be further discussed how to handle on LBT gap/switching gap between segmented transmissions of nominal repetition by slot boundary or between non-contiguous PUSCH Type-B repetitions by DL reception (e.g., candidate SS/PBCH blocks, or others).

References

[1] Final Report of 3GPP TSG RAN WG1 #102-e meeting v1.0.0

[2] Final Report of 3GPP TSG RAN WG1 #103-e meeting v1.0.0

[3] Final Report of 3GPP TSG RAN WG1 #104-e meeting v1.0.0

[4] Draft Report of 3GPP TSG RAN WG1 #104b-e meeting v0.2.0

[5] 3GPP TS 37.213 v16.4.0