3GPP TSG-RAN WG1 #105-e	R1-2105892

e-Meeting, 10th May – 27th May, 2021

Agenda item:		8.5.6

Source:	MediaTek Inc.

Title:	Potential physical layer impact to the RAN2-led topics

Document for:		Discussion

# Introduction

In this contribution, we provide the view for the following discussion point,

- On-demand transmission of DL-PRS

# On-demand transmission of DL-PRS

DL-PRS pattern is designed to be full staggering in order to support long propagation uncertainty. When UE requests the on-demand transmission, the main intention could be for the purpose of accuracy enhancement. As such, we don't see the need to re-use all the full staggering DL-PRS patterns.

The partial staggering pattern with or without repetition, or the non-staggering pattern with repetition could be considered, since the propagation uncertainty has been resolved. The repetition could improve the received SINR for further improving the measurement accuracy. Furthermore, the transmission of partial staggering patterns may also reduce the RS overhead and shorten the processing time.

Proposal 2-1: Support dynamic symbol number configuration for each comb type of Rel-16 DL-PRS for on-demand transmission, and UE may request the symbol number

# Conclusion

Proposal 2-1: Support dynamic symbol number configuration for each comb type of Rel-16 DL-PRS for on-demand transmission, and UE may request the symbol number

# Reference