3GPP TSG RAN WG1 #105-e	R1-2105895

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.4

Source:	Ericsson

Title:	Corrections to SL procedures

Document for:	Discussion, Decision

# 1	Discussion

In this contribution we discuss on issue related to the priorities associated with SL HARQ-ACK reporting in uplink.

# 2	Correction for SL procedures

During last RAN1#104-b-e there was some discussion about some remaining prioritization issues and the following was agreed:

Agreement: - After prioritization with other PUCCH transmissions, when PUCCH carrying SL HARQ-ACK information is prioritized and overlaps with a PUSCH of priority index 1, - PUCCH carrying SL HARQ-ACK information is dropped if it overlaps with a PUSCH of priority index 1. - After prioritization with other PUCCH transmissions, when PUCCH carrying SL HARQ-ACK information is prioritized and overlaps with a PUSCH of priority index 0, - the SL HARQ-ACK information is multiplexed on the overlapping PUSCH of priority index 0 following Clause 9.2.5 and 9.2.6 if the PUSCH does not contain Uu UCI. - The PUSCH with SL HARQ-ACK is subject to the prioritization with any other UL transmissions as per the existing specifications. - The corresponding CR is to be prepared in RAN1#105-e, - Including checking whether or not any other essential corrections is necessary based on the above agreement

One issue to was not agreed and left to be discussed during this meeting is the wording of the CR corresponding to this agreement and whether any further enhancement or additional rule in addition to the previous agreement is needed.

In order the capture the above agreement, we propose to have the following TP with additions marked in red:

## ============================= Start of text proposal for 38.213 ========================== # 9 UE procedure for reporting control information <Unchanged parts omitted> When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in Clause 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH with SL HARQ-ACK report would overlap in time with a transmission of a PUSCH with priority index 1, the UE cancels the transmission of the first PUCCH before the first symbol that would overlap with the PUSCH with priority index 1. - if a transmission of a first PUCCH with SL HARQ-ACK report would overlap in time with a transmission of a PUSCH with priority index 0, the SL HARQ-ACK information is multiplexed on the overlapping PUSCH of priority index 0 following Clause 9.2.5 and 9.2.6 if the PUSCH does not contain Uu UCI. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission <Unchanged parts omitted> ============================= End of text proposal ==========================

Include the above TP to capture the agreement regarding the issue of SL-HARQ ACK report in PUCCH from RAN1#104-b-e

Additionally, in our view, there is no need to have any extra rule for prioritization during this maintenance phase since only essential correction should be added and the procedure is in general working properly.

No extra enhancements or corrections are needed for the prioritization procedure, i.e., extra rules, during this Rel-16 maintenance phase.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include the above TP to capture the agreement regarding the issue of SL-HARQ ACK report in PUCCH from RAN1#104-b-e

Proposal 2	No extra enhancements or corrections are needed for the prioritization procedure, i.e., extra rules, during this Rel-16 maintenance phase.