3GPP TSG RAN WG1 #105-e	R1-2105897

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.4

Source:	Ericsson

Title:	Condition to stop the infinite loop for Mode 2 RA

Document for:	Discussion, Decision

# 1	Discussion

In RAN1 WG, the following was agreed in RAN1#104-bis-e during e-mail discussion:

Agreements: - Update the specification of identification of candidate resources for Mode-2 resource allocation in section 8.1.4 of TS 38.214 to handle the case when X·M_total number of identified resources could not be reached after any number of loop iterations - If the number of the excluded resources in step 5 is larger than (1-X)·M_total, a UE skips step 5

It was intended during RAN1#104-bis-e to provide a CR text regarding the previous agreement but based on some companies comments and views, there was not a common understanding about the precise text to reflect the agreement in a proper way.

We propose the following TP to introduce a stop condition within the Step 5 in [TS 38.214, 8.1.4] in order to avoid an infinite loop in the case where the number of potential/identified resources cannot meet the loop exit conditions.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> 4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 5-1) If the number of candidate single-slot resources Rx,y excluded from the set SA in step 5 is greater than (1-X)⋅Mtotal, the set SA is initialized to the set of all the candidate single-slot resources as indicated in Step 4.

Include the proposed TP to define a condition to avoid an infinite loop in the procedure in [TS 38.214, 8.1.4] when the number of potential/identified resources cannot meet the loop exit conditions according to the agreement in RAN1#104-bis-e.

# 2	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include the proposed TP to define a condition to avoid an infinite loop in the procedure in [TS 38.214, 8.1.4] when the number of potential/identified resources cannot meet the loop exit conditions according to the agreement in RAN1#104-bis-e.