3GPP TSG RAN WG1 Meeting #105-e                                                                     R1-2106103

e-Meeting, May 10th – 27th, 2021

Source:              ZTE

Title:                   Draft Text Proposals for 38.214 on TEI-17 CW mapping

Agenda Item:     8.16

Document for:   Discussion and Decision

Introduction

In Rel-17 TEI agenda, we propose to enhance the current NR codeword mapping for PUSCH transmission. The proposal is shown as follows.

Proposal: Support 2 codewords with 2 MCS for rank 2-4 uplink transmission (i.e. LTE-like CW mapping)

The detailed test case, simulation results and analysis can be found in our companion contribution [1]. Based on the above TEI proposal, we provide the corresponding text proposals for 38.214 [2] in section 2 of this contribution.

Proposed TPs

- 6.1.4.2	Transport block size determination

If the higher layer parameter EnableTwoCodewords-R17 is configured, up to two codewords are enabled, then one of the two transport blocks is disabled by DCI format 0_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. UE does not expect to be enabled with two transport blocks when UCI is carried on PUSCH.

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by fallbackRAR UL grant or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or

for a PUSCH transmission with configured grant, or

for a MsgA PUSCH transmission,

<unchanged is omitted>

Conclusion

In Rel-17 TEI agenda, we propose to support 2 codewords with 2 MCS for rank 2-4 uplink transmission (i.e. LTE-like CW mapping) for NR.

In this contribution, we provide the corresponding text proposals for 38.214 in section 2.

References

- R1-2104596  TEI-17 proposal on NR codeword mapping  ZTE, Sanechips, CMCC, China Telecom, China

Unicom, SoftBank, vivo, NTT DOCOMO

- TS 38.214 g50, Physical layer procedures for data (Release 16), v16.5.0.