3GPP TSG-RAN WG1 #105-e 																	R1-2106155

e-Meeting, 10th – 27th May, 2021

Agenda item:			8.8.1.1

Source:	Moderator (Sharp)

Title:	FL Summary on Enhancements on PUSCH repetition type A

Document for:	Discussion and Decision

Introduction

For PUSCH enahancements the following objectives are described in the Coverage Enhancement WID.

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

This document is intended to facilitate view exchange and discussions on the enhancements on PUSCH repetition type A, for the following assigned email discussion.

[105-e-NR-R17-CovEnh-01] Email discussion regarding enhancements for PUSCH repetition type A – Toshi (Sharp)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

Open Issues summary

Increasing the maximum number of repetitions

In Rel-15/16, RRC parameter pusch-AggregationFactor configures the number of repetitions for PUSCH, where the candidate value set of pusch-AggregationFactor = {2, 4, 8}. TDRA based dynamic repetition number indication introduced in Rel-16 is applied when configured, where the candidate value set of numberOfRepetitions-r16 = {1, 2, 3, 4, 7, 8, 12, 16}. For CG-PUSCH, RRC parameter repK configures the number of repetitions, where the candidate value set of repK = {2, 4, 8}. For Type 2 configured PUSCH repetition, TDRA based dynamic repetition number indication with numberOfRepetitions-r16 using activation DCI is also applicable.

In RAN1#104-e, we discussed several aspects including the maximum number itself, other candidate values, repetitions for configure grant, RRC parameters to be extended, and TDRA list.

Issue#1-1: Value of the maximum number of repetitions

In RAN1#104-e, although the majority supported the maximum number of 32, some companies wanted to first see companies’ views on assumptions for designing of the maximum value, e.g. whether the number of repetitions is counted based on contiguous slots or available slots, whether to consider both FDD and TDD or either of them and whether to consider both VoIP and eMBB or either of them.

When discussing how much the maximum repetition factor should be increased, the following three cases were raised by companies.

- Case 1: FDD or SUL

- Case 2: TDD with contiguous-slot-based counting

- Case 3: TDD with available-slot-based counting

Although most of the companies believed that, once the increased maximum repetition factor is decided, it should be applicable to all the three cases, there were different views on which cases should assumed when evaluating if proposed values achieve sufficient PUSCH coverage. Some company said the value should be decided based on Case 1 while other companies argued it should be Case 2 or Case 3. This divergence came from different views on the “bundle” of two enhancements, (a) increasing the maximum number of repetitions and (b) the number of repetitions counted on the basis of available slots. The majority thought that the two enhancements are not bundled (i.e. can be configured separately/independently) while a few companies said that the two enhancements are always bundled. The most of the majority companies were also thinking that the maximum value should be extended to 32 by the enhancement (a) so that sufficient coverage can be achieved without the enhancement (b). Furthermore, the some of the companies who preferred “always-bundle” were also saying that the maximum value should be extended to 32 even with the enhancement (a).

According to the contributions for RAN1#105-e, companies’ preferences on the maximum repetition number are:

- 32

  - Supported by: ZTE [3], vivo [4], CATT [5], Qualcomm [7], OPPO [8], China Telecom [9], Intel [11], Apple [12], Panasonic [13], Samsung (if a need is identified) [15], Xiaomi [18], Sharp [19], NTT DOCOMO [21], Lenovo/Motorola Mobility [22]

- 16 (i.e. the same as in Rel-16)

  - Supported by: Huawei/HiSilicon (based on available slots) [1], CMCC (based on available slots) [6], Samsung [15], Ericsson (based on available slots) [20]

- 20

  - Supported by: Ericsson (for FDD with 15kHz SCS) [20]

- 24

  - Supported by: Samsung (if a need is identified) [15]

- 40

  - Supported by: Huawei/HiSilicon (based on contiguous slots, and for 30kHz SCS) [1]

Some of the observations from contributions are also listed below.

- Reasons to propose 32 for the maximum repetition number

  - Coverage enhancements specified in this WI should be also applicable for NTN scenarios. Since FDD is assumed for core specification work for NTN scenarios, there are enough consecutive UL slots to transmit the maximum 32 repetitions transmission to obtain the performance gain.

  - This WI is not aiming at LPWA scenario, in which the minimum number of the maximum repetition number among the typical LPWA systems is 32.

  - Excessive repetition number will reduce the performance such as UL UPT.

  - HARQ retransmission mechanism can cooperate with repetition transmission. There is no need to pursue hard one-shot BLER (iBLER) in all scenarios.

  - Considering VoIP as a motivating example, a voice packets gets generated once every 20ms, with voice packet aggregation, an aggregated packet may get generated once every 40ms. With 15 kHz subcarrier spacing, 40 uplink slots are available to transmit an aggregated packet in a FDD system.

  - 2-3dB performance gain can be observed compared with repetition factor of 16.

- Reasons to propose 16 for the maximum repetition number

  - Counting on the basis of available slots for repetition should be as mandate feature of CE UE capability. Based on the available slot counting method, repetition factor of 16 can compensate the coverage gaps. (CMCC)

- Reasons to propose 20 for the maximum repetition number

  - For FDD with 15kHz SCS, the number of actual repetitions of 20 result in 16 kbps (the lowest VoIP data rate).

- Reasons to propose 40 for the maximum repetition number

  - Considering the 20ms data arrival period and typical TDD configuration with 30KHz subcarrier spacing (i.e. has 40 slots), a maximum repetition number of being 40 can be supported for full occupation of all 40 slots within 20ms.

Based on the above, the large majority is still thinking that 32 is a reasonable value for the maximum number of repetitions. Moreover, even if 32 is adopted, the network may still have a choice to configure a smaller value, such as 20 or 24, depending on the outcomes from Issue#1-3. Considering these observations, companies are asked again if it is acceptable to take 32 as the maximum number of repetitions.

Initial FL proposal #1-1

- The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32.

Issue#1-2: RRC parameters to be extended for supporting the increased maximum number

In Rel-16, there are three RRC parameters which are used to configure repetition factors, pusch-AggregationFactor, numberOfRepetitions, and repK. In RAN1#104-e, we discussed which parameter(s) should be extended to support the increased maximum repetition factor. Although the large majority supported extension of all the three parameters, several companies expressed that extension of numberOfRepetitions (i.e. the one associated with TDRA list) is sufficient. In RAN1#104-e, it was agreed that Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI. There was still the sub-bullet saying that “FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig”. In other words, the repetition factors semi-statically configured without using the TDRA list can be set to 2, 4 or 8 in Rel-15/16, but it should be decided whether those semi-static repetition factors also support the increase of maximum number of repetitions or not.

According to the contributions for RAN1#105-e, companies’ preferences on extensions on the repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

- Repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports increase of the maximum number of repetitions.

  - Supported by: Intel [10], Samsung [15], LG [16], Lenovo/Motorola Mobility [22], Nokia/Nokia Shanghai Bell [24]

  - Not supported by: ZTE [3], vivo [4], CATT [5], CMCC (studied further) [6], NTT DOCOMO [21]

  - Up to RAN2: Xiaomi [18]

Based on the above analysis, companies have different views on this issue. Therefore, it is suggested having more discussions on whether to support this function.

Initial FL proposal #1-2

- Discuss if repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions.

Issue#1-3: Other candidate value set for configuration of the number of repetitions

In Rel-16, 8 candidates for repetition factors are supported. The exact value set is {1, 2, 3, 4, 7, 8, 12, 16}.

In RAN1#104-e, several companies proposed adding {20, 24, 28, 32}, as finer granularity among the value set improves resource efficiency. At the same time, it was commonly understood that the exact values should be discussed after concluding the discussion on Issue#1-1.

According to the contributions for RAN1#105-e, companies’ preferences on other candidate values are summarized as the following.

- The number of candidate repetition factors to be increased from 8 to 16.

  - Supported by ZTE [3]

- {20, 24, 28} are also supported.

  - Supported by: vivo [4], CATT [5], Lenovo/Motorola Mobility [22]

In addition, in RAN1#104-e there was a discussion about the number of rows of TDRA list. Although several companies expressed that it should remain unchanged from Rel-16, the large majority wanted to postpone this discussion as this issue should be affected by the number of candidate repetition factors. According to the contributions for RAN1#105-e, Samsung [15], LG [16], Xiaomi [18] are suggesting that the number of rows of the TDRA table should remain unchanged from Rel-16, though Xiaomi [18] is proposing having multiple TDRA tables which correspond to different CE targets.

As discussed in the previous meeting, it is suggested discussing Issue#1-3 after concluding Issue#1-1 discussion.

Initial FL proposal #1-3

- Discuss Issue#1-3 after concluding Issue#1-1 discussion.

Issue#1-4: Other issues

According to contributions for RAN1#105-e, no other issue related to the increased maximum number of repetitions is provided.

The number of repetitions counted on the basis of available UL slots

In Rel-16, transmission occasions for a PUSCH with repetition type A are derived based on K consecutive slots, and then transmissions at some occasions may be omitted according to TDD configuration, dynamic SFI, PUSCH priority, and Cancelation Indication. Rel-15/16 also support PUCCH with N-time repetition in which only slots having sufficient UL/flexible symbols for the allocated PUCCH resource are counted as part of N slots, where UL/flexible symbols are determined by only semi-static configurations (i.e. TDD configuration and SSB configuration).

In RAN1#104-e, there were two different directions proposed for the determination of ”available slots for PUSCH repetition”, one was to follow Rel-16 PUSCH omission rule, and the other was to follow Rel-15/16 PUCCH repetition rule. Both of the rules refer to TDD configuration and SSB configuration. Therefore, it is straightforward that TDD configuration and SSB configuration are also used for the dermination of available slots in Rel-17. On the other hand, there are two aspects which identify the difference between those rules.

The first aspect is whether or not dynamic signal (dynamic SFI, PUSCH priority, and cancelation indication) is used for the determination of available slots. This aspect was described the following agreement in RAN1#104-e. For Alt 1, we discussed which semi-static configurations should be considerd for the available slot determination. Many companies preferred to reuse Rel-15/16 PUCCH repetition rules, i.e. using TDD configuration and SSB configuration, while a few companies wanted to use more configuration, e.g. invalid UL symbol configuration or Type0-CSS / CORESET#0 configuration. For Alt 2, we discussed which dynamic signaling should be considerd for the available slot determination. Although not many companies provided views on it, all the companies proving their views preferred to reuse Rel-16 PUSCH omission rule, i.e. to use all of SFI, PUSCH priority and Cancelation Indication.

Agreements: Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded) - Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC). - Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Relating to this aspect, in RAN1#104bis-e it was taken as a working assumption that the number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3. It is obvious that neither dynamic SFI, PUSCH priority nor cancelation indication is applicable for the determination of available slots for Msg3. In this sense, it can be said that Alt1 needs to be supported al least for Msg3.

The second aspect is whether or not the determination of available slots is done o prior to the first transmission of the repetition. This aspect was mentioned the following conclusion made in RAN1#104-e.

Conclusion: Discuss further to select one of the following alternatives: - Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions. - Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

In Rel-16 PUSCH repetition Type A, the indicated TDRA is applied to K consecutive slots, and then the UE determines to omit the PUSCH transmission in each slot depending on whether the TDRA causes any collision/overlapping or not. RAN1#104bis-e, we discussed whether or not the same principle applies, and it was agreed that, for defining available slots, a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. On the other hand, whether this is applied to special slots or not is still for further study. In RAN1#104-e, 5 companies expressed their views that PUSCH symbol allocation in special slots can be different from UL slots so that UL portion of the special slots can be fully utilized by Rel-17 PUSCH repetition Type A transmissions. At the same time, there were also some companies which see no need of special handling of special slots.

Issue#2-1: Use of dynamic signaling for the determination of available slots

As in the agreement from RAN1#104-e, it should be discussed whether or not the determination of all the available slots depends on dynamic signaling.

- Alt 1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations and does not depend on dynamic signaling.

- Alt 2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations and also depends on dynamic signaling.

During the email discussions in RAN1#104-e, 19 companies preferred Alt 1, i.e. dynamic signaling is not used for the determination of availble slots, while 5 companies preferred Alt 2, i.e. dynamic signaling is not used for the determination of availble slots, to Alt 1. The proponents of Alt 1 expressed several reasons to support it, which includs (1) Alt 1 simplifies UE implementation in terms of processing timeline, and (2) Alt 1 resolves different understanding of available slots between gNB and UE due to detection failure of the dynamic signaling at the UE side. Meanwhile, the proponents of Alt 1 argued that (a) the available slots imply mean the slots with actual transmissions in order to ensure the sufficient number of repetitions, and (b) there is no increase of difficulty over Rel-16 PUSCH repetition omission.

According to the contributions for RAN1#105-e, one more alternative solution is raised, which takes into account issue#2-3’s aspect as well. More specifically, Alt 3 determines available slots by referring to dynamic signaling in the DCI which schedules the concerned PUSCH, so that the determination can be done prior to the 1st actual transmission of the PUSCH repetitions. Companies’ preferences can be summarized as the following. The concern on Alt 2 raised by the Alt 1 proponents is that detection failures of dynamic signaling lead to different understanding of available slots between UE and gNB.

For PUSCH repetition Type A counted on the basis of available slots,

- Alt 1: The determination of available slots does not depend on any dynamic signaling.

  - Note: Further omission of PUSCH repetition in the available slot is subject to dynamic signaling.

  - Supported by : Huawei/HiSilicon (if the determination is done prior to the 1st transmission.) [1], vivo [4], CATT [5], CMCC [6], Qualcomm [7], OPPO [8], China Telecom [9], InterDigital [10], Intel [11], NEC [14], LG [16], Sierra Wireless [17], Xiaomi [18], Sharp [19], Ericsson [20], NTT DOCOMO [21], WILUS [23]

- Alt 2: The determination of available slots depends on dynamic signaling including e.g., dynamic SFI.

  - Supported by : Huawei/HiSilicon (if the determination is done per slot.) [1], ZTE [3], Samsung [15], Lenovo/Motorola Mobility [22], Nokia/Nokia Shanghai Bell [24]

- Alt 3: The determination of available slots depends on dynamic signaling in the scheduling DCI only.

  - OPPO [8], Panasonic [13]

Some of the observations from contributions are also listed below.

- Reasons to propose Alt 1

  - With Alt 2, reception failures of dynamic signaling at the UE side lead to different understanding of available slots between the UE and gNB.

  - Alt 1 makes UE implementation easier, as it does not require re-counting when receiving dynamic signaling.

- Reasons to propose Alt 2

  - Alt 2 can ensure enough number of actual repetitions and meet the requirement of PUSCH coverage performance, while Alt 1 may cause insufficient coverage performance due to PUSCH dropping in a determined available slot.

  - It is not apparent why for coverage enhancement a UE cannot determine whether or not to transmit in a slot based on DCI indication when that is already supported or why a network should be practically prohibited from benefiting from coverage enhancements when it dynamically adapts the UL-DL TDD configuration.

- Reasons to propose Alt 3

  - Well-balanced design in terms of scheduling flexibility and UE complexity.

In addition, more detailed determination rules are also proposed, which are listed below.

- The slot where a PUSCH repetition is dropped due to overlapping with PUCCH should be determined as unavailable.

  - ZTE [3]

- The flexible symbols configured with SSB based measurement by SMTC should be determined as unavailable.

  - Vivo [4]

- The slot with PUSCH dropping due to power reduction should be counted as an available slot.

  - OPPO [8]

- Support opportunistic UL transmission on “non-available” UL slots dynamically indicated by SFI

  - InterDigital [10]

- Overlapping with SR with larger priority should not have any impact on the determination of available slots.

  - Sharp [19]

- Flexible symbol should be determined as available for UL transmissions

  - Nokia/Nokia Shanghai Bell [24]

Alt 1 now has more supports than in the previous meeting. Also, many companies pointed that Alt 2 has the problem that UE and gNB may have different understanding on available slots, which leads the issues described in Issue#2-2 and Issue#2-3.

Initial FL proposal #2-1

- The determination of available slots does not depend on any dynamic signaling. (i.e. Taking Alt 1 of the previous agreement)

Issue#2-2: RV Cycle

In Rel-16 PUSCH repetition Type A, RV to be applied on a given slot is derived from when this slot comes from the start of the K consecutive slots as in the following TS38.214 descriptions.

For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2. Table 6.1.2.1-2: Redundancy version for PUSCH transmission rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B). n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

For PUSCH repetition based on the K available slots, it should be discussed how RV in each slot of the K available slots is determined.

According to the contributions for RAN1#105-e, only two companies are providing their views, which are listed below.

- RV in a slot is derived by the index of the slot, where the indexing is based on available slots according to semi-static configurations, and the index=0 at the slot Ks derived from the slot offset K2.

  - Sharp [19]

- RV cycling should be based on available slots which are determined by RRC configurations only.

  - Ericsson [20]

Both of them are suggesting that RV cycling should be affected only by RRC configurations. Otherwise, reception failures of dynamic signaling lead to different understanding of RV. Therefore, it is suggested discussing the following proposal as a starting point.

Updated Initial FL proposal #2-2

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

  - Note: this has no spec impact in terms of the redundancy version derivation.

For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2. Table 6.1.2.1-2: Redundancy version for PUSCH transmission rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B). n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.

Issue#2-3: Inter-Slot Frequency Hopping Cycle

According to contributions for RAN1#105-e, Qualcomm [7] raises the issue related to inter-slot frequency hopping. Similar to RV cycling, in Rel-15/16, inter-slot frequency hopping cycle is determined on the basis of consecutive physical slots. More specifically, hop index in a slot is determined based on whether the slot index is odd or even. However, it causes an uneven distribution of hops in TDD system. InterDigital [10] also mentions the same issue.

Figure: Uneven distribution of hops in TDD [7]

A possible solution proposed in [7] is that, for inter-slot frequency hopping, hop index is determined based on indexing within the determined available slots.

- For inter-slot frequency hopping, hop index is derived based on indexing within the determined K transmission occasions.

The proposal seems a good starting point.

Initial FL proposal #2-3

- For inter-slot frequency hopping, hop index is derived based on the indexing of n within K transmission occasions (may be in K non-contiguous slots) which are determined by only RRC configurations, where K is the indicated/configured repetition factor.

Issue#2-4: Timeline aspect for the determination of available slots

As in the conslusion from RAN1#104-e, it should be discussed whether or not the determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

As this aspect was raised in the email discusion in RAN1#104-e, companies did not have enough time to deeply look into this aspect during RAN1#104-e. FL asked companies to provide their analyses in RAN1#105 on what is a possible bar/bottleneck in terms of timeline requirements in the available-slot-based PUSCH repetition, if any, compared to the omission rules of Rel-16 PUSCH repetitin Type A.

According to the contributions for RAN1#105-e, companies’ views on this aspect are summarized as the following.

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

  - Supported by: vivo [4], Qualcomm [7], OPPO [8], China Telecom [9], InterDigital [10], Intel [11], LG [16], Sierra Wireless [17], Xiaomi [18], Ericsson [20], Lenovo/Motorola Mobility (if not adopting the limitation of overall duration (i.e. Issue#2-6)) [22]

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

  - Supported by: ZTE [3], Lenovo/Motorola Mobility (if adopting the limitation of overall duration (i.e. Issue#2-6)) [22], Nokia/Nokia Shanghai Bell [24]

Some of the observations from contributions are also listed below.

- Reasons to propose Alt-a

  - It enables cross-slot channel estimation/DMRS bundling.

  - It simplifies UE implementation as it does not require recounting.

  - Alt-a also simplifies hopping determination.

- Reasons to propose Alt-b

  - Alt-b reuses Rel-15/16 PUSCH omission mechanism (i.e. per-slot based timeline requirements).

Initial FL proposal #2-4

- The determination of all the available slots has to be done prior to the first actual transmission of the repetitions. (i.e. Take Alt-a of the previous conclusion)

Issue#2-5: Semi-static configurations to be used for the determination of available slots

Regarding semi-static configurations to be used for the determination of available slots, there was some email discussions in RAN1#104-e. Many companies preferred to use TDD configuration and SSB configuration as in Rel 15/16 PUCCH repetition, while a few companies wanted to use more configuration, e.g. invalid UL symbol configuration or Type0-CSS / CORESET#0 configuration as in Rel 16 PUSCH repetition Type B. The agreement in RAN1#104-e states that at lease TDD configuration is used and FFS for other semi-stataic configurations, and there was no company which disagree with using of SSB configuration.

Since the previous meeting, no one has objected to refer to ssb-PositionsInBurst (i.e. SSB configuration) for determination of available slots. Therefore, the following should be agreeable.

- ssb-PositionsInBurst (i.e. SSB configuration) is used for determination of available slots.

  - Flexible symbol(s) for the reception of SSB is determined as unavailable.

For other semi-static configurations, according to the contributions for RAN1#105-e, several companies are proposing using the following RRC configurations for the determination of available slots.

- SSB based measurement by SMTC

  - Vivo [4]

- CORESET0 with Type0-PDCCH CSS set

  - Intel [11], Samsung [15], WILUS [23]

- Invalid UL symbols for DL-to-UL switching purpose

  - Intel [11], Samsung [15], WILUS [23]

- Other CG-PUSCH with larger priority index

  - Sharp (for CG-PUSCH) [19]

- PUCCH with larger priority index carrying HARQ-ACK for SPS

  - Sharp (for CG-PUSCH) [19]

- Semi-static PUCCH with repetition

  - WILUS [23]

Base on the above, the following proposals were made.

Initial FL proposal #2-5

- ssb-PositionsInBurst (i.e. SSB configuration) is used for determination of available slots. Flexible symbol(s) for the reception of SSB is determined as unavailable.

- Discuss further use of the following RRC configurations for determination of available slots:

  - SSB based measurement by SMTC

  - CORESET0 with Type0-PDCCH CSS set

  - Invalid UL symbols for DL-to-UL switching purpose

  - Other CG-PUSCH with larger priority index

  - PUCCH with larger priority index carrying HARQ-ACK for SPS

  - Semi-static PUCCH with repetition

Issue#2-6: Special slot handling

In RAN1#104-e, 5 companies expressed their views that PUSCH symbol allocation in special slots can be different from UL slots so that UL portion of the special slots can be fully utilized by Rel-17 PUSCH repetition Type A transmissions. At the same time, there were also some companies which see no need of special handling of special slots. RAN1#104bis-e, it was agreed that, for defining available slots, a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. On the other hand, there was a sub-bullet saying ”FFS:details”. With this sub-bullet, the current situation is that whether this agreement is applied to special slots or not is still for further study.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

According to the contributions for RAN1#105-e, only one company is discussing the special slot handling.

- For the number of repetitions for PUSCH repetition type A counted on the basis of available UL slots, the special slot is determined as an available UL slot. For the special slots, the available UL symbols can be used for PUSCH transmission.

  - China Telecom [9]

Updated Initial FL proposal #2-6

- The agreement in RAN1#104-e is applied to all slots including special slots.

Issue#2-7: Limitation of overall duration of PUSCH repetitions

In RAN1#104-e, several companies proposed having a time window/limitation of overall time duration for a signle set of PUSCH repetitions so that an excessive delay can be avoided. Meanwhile, more companies thought that the network can control the overall time duration by setting an appropreate repetition factor.

- Alt 1: Count of available slots continues until reaching the indicated/configured repetition factor.

- Alt 2: Count of available slots continues until reaching the indicated/configured repetition factor or reaching the limitation of overall duration for a set of PUSCH repetitions, whichever comes first.

At the same time, it was widely understood that whether this issue needs to be discussed depends on the outcome of Issue#2-1, because the overall duration is certainly deterministic and controlable if the available slots are determined by semi-static configurations only while the postponement of PUSCH repetition due to dynamic scheduling may lead to difficulty to predict when the repetitions finish.

According to the contributions for RAN1#105-e, the following companies are proposing introducing the limitation of the overall duration.

- Alt 2: Count of available slots continues until reaching the indicated/configured repetition factor or reaching the limitation of overall duration for a set of PUSCH repetitions, whichever comes first.

  - Supported by: OPPO [8], Intel [11], Samsung [15], Lenovo/Motrola Mobility [22]

Initial FL proposal #2-7

- Discuss Issue#2-7 after concluding the discussion on Issue#2-1.

Issue#2-8: Enhancements on PUSCH dropping

Similar to special slot handling, in the previous meeting it was also proposed introducing the mechanizm to fully utilize UL slots. More specifically, it was proposed that, even if some of the UL symbols allocated for a PUSCH are determined as unavailable for the PUSCH transmission due to e.g., overlapping with higher priority channels or cancellation indication, the PUSCH transmission using only remaining UL symbols should be possible.

According to contributions for RAN1#105-e, three companies are providing their views on this issue, as shown below.

- Huawei/HiSilicon is proposing that, if available UL slot is determined prior to 1st actual PUSCH repetition while other burst signals overlap with PUSCH repetition on determined available UL slot, then non-overlapped symbols of PUSCH repetition within this overlapped available UL slot can be used for PUSCH repetition to make a full use of UL resource.

- CMCC mentions that confliction between PUSCH repetitions and other uplink transmissions (e.g. SRS) should be further studied.

- Ericsson is also discussing the issue of overlapping of PUSCH and other UL channels (e.g. SPS HARQ-ACK discussed in Rel-17 IIoT/URLLC WI) and signal (A-SRS discussed in Rel-17 FeMIMO WI) within a slot.

Initial FL proposal #2-8

- Discuss further:

  - Necessity of collision handling between PUSCH repetition Type A and the other UL transmissions, e.g. SPS HARQ-ACK, SRS

  - How to handle the collision, if any, e.g.

    - Dropping whole part of either PUSCH repetition or the colliding UL transmission

    - Partially dropping either PUSCH repetition or the colliding UL transmission

Issue#2-9: Enhancement on UCI multiplexing on PUSCH repetitions

This issue was raised in TEI-17 agenda item (TEI proposal #6) in RAN1#104-e meeting, and the conclusion was to continue discussion in RAN1#105-e.

In Rel-15/16, UE does not expect to detect a DCI, indicating a PUCCH resource for HARQ-ACK in a later slot, if UE detects a DCI scheduling a PUSCH transmission in a previous slot and the HARQ-ACK information be multiplexed on the PUSCH transmission. For Rel-17 Coverage Enhancement, two enhancements are considered, both enhancements result in increase of overall time duration for a single set of PUSCH repetitions. Therefore, the restriction to PDSCH scheduling may have more impact to the gNB scheduler.

Figure: Rel-15/16 PDSCH scheduling restriction when PUSCH repetitions is scheduled [4]

According to contributions for RAN1#105-e, vivo [4] is proposing enhancement on UCI multiplexing on PUSCH repetitions such that HARQ-ACK multiplexing on PUSCH repetitions can be allowed even if HARQ-ACK for the scheduling DCI comes after the UL grant of the PUSCH repetition transmission. Panasonic [13] also mentions the same issue and suggests discussing it.

- HARQ-ACK multiplexing on PUSCH repetitions can be allowed even if HARQ-ACK for the scheduling DCI comes after the UL grant of the PUSCH repetition transmission

It should be noted that R1-2105536 sourced by Huawei/HiSilicon and China Unicom under AI 8.16 (TEI-17) is also addressing exactly the same issue.

A fundamental problem is that total UCI bit size to be reported is not known when the gNB schedules the PUSCH. This uncertainty leads to two sub-issues to be resolved. One is how to perform rate-matching of UCI around the PUSCH. The other is which DAI to be used to determine the UCI bit size. The solutions proposed in [4], [13] and R1-2105536 are listed below:

- Option 1: HARQ-ACK bits for later DL assignments puncture the PUSCH repetition.

- Option 2: When HARQ-ACK bits for the DL assignments later than UL grant is received, PUCCH with HARQ-ACK is transmitted and the PUSCH repetition is dropped or postponed.

- Option 3: The time restriction on scheduling HARQ after UL grant is only applied to initial PUSCH repetition, and HARQ information bits corresponding to the PDSCH(s) scheduled after UL grant which triggers the PUSCH transmission are allowed to be multiplexed on the non-initial repetitions, where DAI in the last DCI applies.

Updated Initial FL proposal #2-9

- Discuss Issue#2-9 in the TEI-17 proposal#6 discussion under AI 8.16.

Issue#2-10: Configuration/indication of CovEnh functions

In RAN1#104-e, we had discussions on configurability of CovEnh functions and relationship of two enhancements, i.e., increase of the maximum  number of repetitions and the repetitions counted on the basis of available slots. Although this issue is higher related to the UE features that should be discussed under a dedicated agenda item later, it is good to exchange companies’ views on it in order to have better understanding among companies on their proposed designs.

According to contributions for RAN1#105-e, the following views have been provided.

- Counting on the basis of available slots for repetition should be as mandate feature of CE UE capability.

  - CMCC

- Flexible configurability of two enhancemets is suitable. Two enhancements can be configured independently (either one or both of them can be configured).

  - CATT, OPPO, China Telecom, Apple

- One between three repetition options, i.e. legacy repetition, increased max repetition factor and repetitions based on available slots, is configured.

  - Ericsson

- One of the two counting methods (i.e., contiguous slot basis / available slot basis) is RRC configured or dynamically indicated by gNB

  - OPPO (implicitly indicated), Panasonic (dynamically indicated via TDRA table), Lenovo/Motorola Mobility (implicitly indicated)

- Separate capabilities for the two enhancements

  - Apple

- Single feature which support the two enhancements

  - Nokia/Nokia Shanghai Bell

- The (Increase of max repetition factor feature or the TBoMS feature) and the “Counting based on available UL slots” feature should be allowed to be enabled at the same time.

  - Sierra Wireless

Initial FL proposal #2-10

- Discuss further the following options, in terms of configurations/indications of two enhancements, i.e., (a) increase of the maximum number of repetitions and (b) the repetitions counted on the basis of available slots:

  - Option 1: The enhancement (a) is RRC-configurable. The enhancement (b) is a basic feature of CovEnh capability.

  - Option 2: A set of the enhancements (a) and (b) is RRC-configurable (i.e., configured together).

  - Option 3: Either enhancements (a) or (b) is RRC-configurable (i.e., not configured together).

  - Option 4: Either enhancements (a) or (b) or both is RRC-configurable.

  - Option 5: Either enhancements (a) or (b) is dynamically-indicated.

Issue#2-11: Modification on βoffsetPUSCH

This issue raised by Samsung [15] is not strongly related to the CovEnh WI scope but is a kind of a correction proposal on the existing UCI on PUSCH repetition procedures.

In RAN1#91 and RAN1#92bis the following agreements were made.

Agreement: (RAN1#91) - For UCI on PUSCH with UL-SCH, the amount of resources used for HARQ-ACK is calculated based on the following equation. Q'=O⋅MscPUSCH⋅NsymbPUSCH⋅βoffsetHARQ-ACKr=0C-1Kr where O is the number of ACK/NACK bits, MscPUSCH is the scheduled bandwidth for PUSCH transmission in the current PUSCH transmission period for the transport block, expressed as a number of subcarriers. C, and Kr are obtained from the PDCCH scheduling the PUSCH transmission. NsymbPUSCH is the number of OFDM symbols in the PUSCH transmission duration excluding DMRS. REs occupied by PTRS are also excluded. - FFS: if an upper bound on the number of symbols for HARQ-ACK resource is needed Agreement: (RAN1#92bis) For HARQ-ACK, CSI part 1, and CSI part 2 (if exists) transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1, and CSI part 2 (exists), are determined as follows: QACK'=minOACK+LACK⋅βoffsetPUSCHSE,α⋅l=l0Nsymb,allPUSCH-1MscUCIl QCSI-1'=minOCSI-1+LCSI-1⋅βoffsetPUSCHSE,l=0Nsymb,allPUSCH-1MscUCIl-QACK' QCSI-2'=l=0Nsymb,allPUSCH-1MscUCIl-QACK'-QCSI-1' SE is the spectrum efficiency which is code rate * modulation order

On the other hand, TS 38.212 v16.5.0 computes the number of coded modulation symbols as follows, where the value range of βoffsetPUSCH is from 1 to 126.

- 6.3.2.4.1.1 HARQ-ACK For HARQ-ACK transmission on PUSCH not using repetition type B with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows: … For HARQ-ACK transmission on an actual repetition of a PUSCH with repetition Type B with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as QACK', is determined as follows: QACK'=minOACK+LACK∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr, α∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl, l=0Nsymb,actualPUSCH-1Msc,actualUCIl … - 6.3.2.4.1.2 CSI part 1 For CSI part 1 transmission on PUSCH not using repetition type B with UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined as follows: QCSI-1'=minOCSI-1+LCSI-1∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=0Nsymb,allPUSCH-1MscUCIl-QACK/CG-UCI' … For CSI part 1 transmission on an actual repetition of a PUSCH with repetition Type B with UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as QCSI-part1', is determined as follows: QCSI-1'=minOCSI-1+LCSI-1∙βoffsetPUSCH∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIlr=0CUL-SCH-1Kr, α∙l=0Nsymb,nominalPUSCH-1Msc,nominalUCIl-QACK/CG-UCI' , l=0Nsymb,actualPUSCH-1Msc,actualUCIl-QACK/CG-UCI'

The current TS38.213 is not according to RAN1 agreements as UCI is multiplexed in only one repetition while r=0CUL-SCH-1Kr is over N repetitions. Scaling  βoffsetPUSCH by N may make it more aligned to the intended behaviour. Samsung [15] is suggesting the correction as part of the Rel-17 coverage enhancements.

One discussion point would be whether or not the maximum βoffsetPUSCH value of 126 in current specification provide sufficient reliability of UCI on a PUSCH repetition.

Updated Initial FL proposal #2-11

- No need to discuss modification of  βoffsetPUSCH in this WI.

Issue#2-12: Other issues

According to contributions for RAN1#105-e, NICT/TOYOTA are proposing that additional methods may be necessary, because applications require low latency. However, any specific methods have not been provided. Therefore, the proponents are asked to provide more details on what methods they have in mind.

First round discussion

Issues for the 1st round discussion

Issue#1-1: The maximum number of repeitions

According to the contributions for RAN1#105-e, there is almost nothing newly added to the discussions, compared to what we had in RAN1#104-e. The large majority is still thinking that 32 is a reasonable value for the maximum number of repetitions. Moreover, even if 32 is adopted, the network may still have a choice to configure a smaller value, such as 20 or 24, depending on the outcomes from Issue#1-3. Considering these observations, it is suggested taking 32 as the maximum number of repetitions for Rel-17 PUSCH repetition Type A.

Initial FL proposal #1-1

- The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32.

Companies are asked if the above proposal #1-1 is acceptable.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. |
| Intel | We are fine with the proposal. |
| China Telecom | We support this proposal. |
| ZTE | Support |
| vivo | Support |
| Samsung | OK with proposal |
| Apple | Support |
| Ericsson | Although 20 is enough in our view, we’re fine to support up to 32 repetitions counted based on physical slots which should also be reflected in the proposal. Given above, we propose following updates to the FL proposal to reflect the fact that we’re talking about Type A PUSCH repetition enhancement with increasing the number of repetitions counted based on physical slots: Initial FL proposal #1-1 - The maximum number of repetitions counted based on physical slots and supported by Rel-17 PUSCH repetition Type A is 32. |
| NTT DOCOMO | We support the FL proposal. |
| Panasonic | We support the proposal. |
| Sharp | Support the FL proposal #1-1. |
| CATT | We are OK with the proposal. |
| LG | We are fine with the proposal. |
| OPPO | Support |
| Xiaomi | Support |
| NEC | Support |
| Lenovo, Motorola Mobility | We agree with Ericsson’s proposed update to the FL proposal. Basically, we don’t think that maximum number of repetitions should be 32 when counting is done based on available slots. For that case, 16 is sufficient |
| Nokia/NSB | We are fine with the FL’s proposal. |
| Sierra Wireless | We agree with Ericsson’s proposed update to the FL proposal Understanding what type of slots and how the counting occurs is important to make clear. |
| FL | The updated Proposal#1-1 is trying to capture the alternatives which were raised in the GTW session. Updated FL proposal #1-1 - Select one of the following Alt 1 and Alt 2. The maximum number of repetitions supported by REl-17 PUSCH repetition Type A is: - Alt 1: 32 irrespective of counting method, - Alt 2: 32 for the counting based on physical slots, and 16 (i.e. no change from Rel-16) for the counting based on available slots. - No other option is considered. |
| Huawei, HiSilicon | We share the similar view with Ericsson & Lenovo that 32 repetitions is supported only on when the maximum number of repetition is counted based on contiguous slots. If repetition is counted based on available slot, then current 16 is sufficient without further increasing. |

Issue#1-2: RRC parameters to be extended for supporting the increased maximam number

As described in section 2.1.2, companies have different views on whether or not repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions. Therefore, it is suggested having more discussions on whether to support this function.

Initial FL proposal #1-2

- Discuss if repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions.

Companies are invited to provide their views/justifications on whether or not repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. We are okay to not update these parameters. Updating the TDRA tables seems sufficient. |
| Intel | We think repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig should support the increased maximum number of repetitions. When UE is not configured with number of repetitions in the TDRA table, UE needs to follow the pusch-AggregationFactor configured in PUSCH-Config. In this case, sufficient number of repetitions for PUSCH can still be possible, which can help meet the coverage enhancement target. This also applies for repK in ConfiguredGrantConfig |
| China Telecom | Agree with Intel. Repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig should support the increased maximum number of repetitions. |
| ZTE | Similar as Rel-16 URLLC enhancement, increasing the repetition factor in TDRA table is sufficient. |
| vivo | Extension on numberOfRepetitions is sufficient, which can be applied to both dynamic grant and configured grant. |
| Samsung | Coverage issues can exist also with configured transmissions. It is fine to increase the number of repetitions for the above parameters. |
| Apple | Repetition indication in the TDRAT table provide full flexibility and is enough. |
| Ericsson | Supporting the further increased number of repetitions in TDRA list is enough in Rel-17, following the same logic as we enhanced type A PUSCH repetition in Rel-16. Dedicated TDRA list is already flexible enough, there’s no need to increase the repetitions indicated in semi-static signalling. |
| NTT DOCOMO | We are fine with the FL proposal. We are open for the discussion, and currently, the motivation/necessity of increasing the maximum number of repetition in PUSCH-Config and/or ConfiguredGrantConfig is not clear for us. |
| Sharp | The repetition factor provided by TDRA list, which was agreed in RAN1#104-e, is applicable to both Type 1 and Type 2 configured grant PUSCH transmissions. We are wondering if repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig needs to support the increased maximum number of repetitions. |
| CATT | As expressed in our contribution, since repetition indication in the TDRA table has higher priority on repetition number determination than repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig, increasing the maximum number in PUSCH-Config and/or ConfiguredGrantConfig does not provide further flexibility or coverage enhancement. Only increasing the number of repetitions in TDRA list is enough. |
| LG | Increasing the maximum repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig seems beneficial to enhance UL coverage. |
| OPPO | The proposal only introduces a new parameter in TDRA to indicate the number of repetitions. This would limit gNB using only that one parameter for R17 Coverage Enhancement. Not good for the number of repetitions for R17 can only be dynamically indicated。 |
| Xiaomi | Support to increase RRC parameters about the maximum repetition factor, but whether in PUSCH-config and/or in ConfiguredGrantConfig depends on RAN2. |
| CMCC | Extension on numberOfRepetitions is sufficient for both dynamic grant and configured grant transmissions. The motivation of enhance the repetition factor in PUSCH-Config and/or ConfiguredGrantConfig is not clear. |
| NEC | Increase the repetition factor in TDRA table should be baseline. Be open to increase repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig either. |
| Lenovo, Motorola Mobility | Just increasing the maximum number of repetitions in the TDRA should be enough. |
| Nokia/NSB | Support the FL’s proposal. We support increasing the maximum repetition factor configured in PUSCH-Config and ConfiguredGrantConfig. It is our understanding that the Rel-16 dynamic indication of number of repetitions configured in numberofRepetitions is an optional feature, therefore increasing the maximum repetition factor configured in PUSCH-Config can be supported to make use of the Rel-17 feature on those UEs that cannot support dynamic indication of repetition factor in Rel-16. |
| Sierra Wireless | OK to discuss but updating the TDRA tables should be sufficient. |

Issue#1-3: Other candidate value set for configuration of the number of repetitions

Issue#1-3 will be discussed after concluding the discussion on Issue#1-1.

Issue#1-4: Other issues

Companies are invited to provide other issues to be discussed in this meeting, if any, for the increase of the maximum number of repetitions.

| Company | Comments |
| --- | --- |
| XXX |  |

Issue#2-1: Use of dynamic signaling for the detemination of available slots

For the determination of available slots, the following three alternatives have been proposed. Alt 1 and Alt 2 was captured in the agreement in RAN1#104, and Alt 3 is a sub-option of Alt-2 considering Issue#2-4.

- Alt 1: The determination of available slots does not depend on any dynamic signaling.

- Alt 2: The determination of available slots depends on dynamic signaling including e.g., dynamic SFI.

- Alt 3: The determination of available slots depends on dynamic signaling in the scheduling DCI only.

According to the contributions for RAN1#105-e, Alt 1 now has more supports than in the previous meeting. Also, many companies pointed that Alt 2 has the problem that UE and gNB may have different understanding on available slots.

Initial FL proposal #2-1

- The determination of available slots does not depend on any dynamic signaling. (i.e. Taking Alt 1 of the previous agreement)

Companies are invited to provide their views on the above proposal #2-1. If still prefer Alt 2, also provide the views on the mis-alignment issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. Relying on dynamic signaling is likely to make the system less robust. |
| Intel | We are fine with the proposal. The major concern for Alt. 2/3 is the potential misalignment between gNB and UE on the PUSCH repetition, which may create some interference in the network. |
| China Telecom | We support this proposal. |
| ZTE | Do not support. We think at least dynamic SFI should be considered for determining available slots. In Rel-15/16, when dynamic SFI is configured, there is no ambiguity on the number of repetitions between gNB and UE, regardless of the SFI is detected or not. Because, for DG PUSCH, it can be transmitted on UL symbols or semi-static flexible symbols, and no conflict between dynamic grant and SFI is expected on the flexible symbols. For CG PUSCH, the UE will not transmit on flexible symbols, and therefore there is no conflicts with SFI. So, could any proponent of this proposal could clarify in which case there is ambiguity on the number of repetitions between gNB and UE if dynamic SFI is considered? We have no strong view on other dynamic signaling. |
| vivo | Support. The available slots should be determined based on semi-static configurations only, dynamic signaling should not be considered, otherwise, NW and UE would have ambiguity on the resources used for PUSCH repetitions. |
| Samsung | Do not support. If dynamic signaling does not work, Rel-15 and Rel-16 do not work. The issues from a potential miss of dynamic signaling are identical. No support of dynamic signaling means that Rel-17 coverage enhancements cannot coexist with deployment of basic NR features. Legacy behavior is also modified which makes no sense for a network. Moreover, the reasons for proposing that restriction are unclear. |
| Apple | We support this proposal. |
| Ericsson | Fine with the proposal to only consider semi-static TDD configuration for available slot determination, and whether a Type A PUSCH can be actually transmitted on the available slot can be based on rules related to SFI/CI/priority handling etc. same as legacy. Regarding the misalignment between gNB and UE on the SFI signaling, in our understanding, - For CG PUSCH, when dynamic SFI is receive, it can be only transmitted on uplink symbols - meaning that SFI needs to change flexible symbols to be uplink, if this SFI is mis-detected, then CG PUSCH may be not transmitted/counted, but gNB may assume it will be transmitted/counted, then the issue happens. - For DG PUSCH, when dynamic SFI is received, it can be transmitted on both flexible and uplink symbols - There should be no mis-alignment issue since gNB will never transmit an SFI to change a set of flexible symbols to be downlink for DG PUSCH, and even if an SFI to change flexible symbols to be uplink symbols for DG PUSCH transmission is mis-detected, DG PUSCH can still be transmitted on the set of flexible symbols |
| NTT DOCOMO | We support the FL proposal. |
| Panasonic | Before concluding whether dynamic signaling is used or not, we think whether semi-static flexible symbol is used or not should be discussed. If semi-static flexible symbol is not used for PUSCH repetition. we support the proposal. If semi-static flexible symbol can be used for PUSCH repetition, the design not depending on any dynamic signaling prevent to use flexible symbol as DL by SFI. We think to limit SFI usage of DL may not be acceptable way of the operation if the number of repetitions or repetition window is large. However, if the postponement depends on the dynamic SFI indication, more complexity on the determination on when to transmit PUSCH is necessary. Therefore, from flexible gNB control and UE complexity point of view, we support Alt.3. In Alt.3, whether semi-static flexible symbol(s) is intended for UL transmission or not is determined by flag in scheduling DCI. |
| Sharp | Support the FL proposal #2-1. |
| CATT | Support. As stated by FL, in Alt1, available slot only determines the ‘possible transmission slot candidate’, but some of the slots may still be dropped due to dynamic signaling, which does not conflict with current mechanism. |
| LG | We support the proposal. |
| OPPO | We are OK with the proposal, although we prefer Alt3. In our view, dynamic SFI is not used to determine whether or not a slot is an available slot, but it can influence the actual repetition in available slot, i.e. if dynamic SFI make some symbols overlapped with TDRA for PUSCH repetition change to DL, the repetition in this available slot is dropped. Alt 3 can ensure UE and gNB have the same understanding on available slots, it also a well-balanced design in terms of scheduling flexibility and UE complexity. The miss-detection problem is solvable. |
| Xiaomi | Support the proposal. |
| CMCC | Support. Considering the dynamic indications would make the situation more complex. More discussions and specification effort would be spent to solve the ambiguities between gNB and UE. |
| NEC | OK with the proposal. |
| WILUS | We support the FL’s proposal. |
| Lenovo, Motorola Mobility | Do not support. To determine the actual number of available slots, it is necessary to take into account any dynamic signaling that might impact the availability of UL slots. |
| InterDigital | We support the FL’s proposal. CI and URLLC deprioritization do not change slot “availability” and whether a slot is considered available should therefore solely depend on RRC slot type configuration. |
| Nokia/NSB | Can we clarify that this is for counting the number of repetition and it does not limit the scheduling flexibility for gNB, and in case there is overlapping due to dynamic signaling the legacy Rel-15/16 behavior of collision handling should be reused? If these issues can be clarified, then we are Ok with the proposal for the sake of progress. |
| Sierra Wireless | Support. Relying on dynamic signaling will be much more complex, make the system less robust, and may create issue with HD-FDD Redcap UEs. |
| Huawei, HiSilicon | Suggest to clarify the proposal is for counting the number of available slot. In our understanding, they are two UE actions of the determination of available slots, i.e. the determination on which available slots are counted and the determination on which counted slots can be actually transmitted. These two UE actions can have different timeline for determination, and depend on different signaling. For example, in the current Rel-16 PUSCH repetition type A the first action is to determine an overall time window for the whole repetition based on RRC signaling and the scheduling DCI before the transmission of the first repetition, and the second action for actual transmission may be interrupted later by dynamic URLLC signaling to transmit URLLC PUSCH of higher priority. With the clarification for counting only, the proposal is more compatible to legacy collision handling, so some companies’ concerns may be resolved. Proposal: For Rel-17 PUSCH repetition Type A, the determination counting of available slots does not depend on any dynamic signalling except for the DCI scheduling the PUSCH repetition. (i.e. Taking Alt 1 of the previous agreement) |

Issue#2-2: RV Cycle

Based on the proposals in contributions for this meeting, it is suggested discussing the following proposal as a starting point.

Initial FL proposal #2-2

- RV cycling is based on rvid indicated by the DCI scheduling the PUSCH and the indexing of n within K transmission occasions (may be in K non-contiguous slots) which are determined by only RRC configurations, where K is the indicated/configured repetition factor.

Companies are invited to provide their views on the above proposal#2-2.

| Company | Comments |
| --- | --- |
| Qualcomm | Not too sure if any changes are necessary. The spec is already clear and says that the RV index is updated for each transmission occasion. Once available slots are determined, the transmission occasions are determined as well. |
| Intel | Our view is that we can simply follow the existing spec for RV cycling. The only difference may be determination of K. However, if we agree to use RRC configuration to determine available UL slots for repetition type A, it seems we do not need to change the spec. |
| ZTE | Similar understanding as Qualcomm. |
| vivo | Agree with Qualcomm. |
| Samsung | The RV is intended for an actual transmission. It should be same as legacy. |
| Apple | Agree with Qualcomm. |
| Ericsson | Considering available slot is introduced in Rel-17, it’s enough to have a proposal: - RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17 Note that in legacy, the RV is cycled on slots determined without any misunderstanding between UE and gNB so that the RV pattern is aligned between gNB and UE side. |
| Panasonic | We share the similar view as Qualcomm. |
| Sharp | Support the FL proposal #2-2, as it is aligned with what we have in Rel-15/16. We are fine with Ericsson’s proposal, too. RV cycling should be done within the K available slots so as to avoid to lose systematic bits of the TB due to non-uniform RV distribution. The thing is that, given that RV cycling is done based on the available slots, the same understanding of the available slots between UE and gNB has to be ensured, because wrong RV causes significant performance degradation. |
| CATT | We agree that RV cycling can be based on available slots naturally. |
| LG | Agree with Qualcomm. |
| OPPO | There is no description in the spec about whether unavailable slots can be counted as transmission occasions. In our view, R15/R16 indicate the number of repetitions K on the base of contiguous slots, if a slot is not for PUSCH repetition in the K contiguous slots, or the actual PUSCH repetition is dropped in a slot, the indexing of rvid is still counted. Similarly with Ericsson, we suggest to make some changes for proposal: RV cycling is based on K available slots at least for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17, where K available slots are determined by only RRC configurations (/RRC configuration and scheduling DCI), where K is the indicated/configured repetition factor. |
| Xiaomi | Agree with Qualcomm and intel |
| CMCC | Share similar views as Qualcomm |
| NEC | Agree with Qualcomm |
| WILUS | Agree with Qualcomm. |
| Lenovo, Motorola Mobility | Similar views as Qualcomm |
| InterDigital | One issue to consider is the following. Let us assume from Issue #2-1 that determination of available slots does not depend on any dynamic signalling, i.e., available slots are determined by RRC. CI or high prioirty URLLC transmission may cancel some of the available slots (after the UE determines available slots with the RRC config). Since the objective is to keep incrementing the RV despite of cancellation even, the UE should keep incrementing the repetition counter. CI and URLLC deprioritization should therefore not change the definition of slot “availability”, and whether a slot is considered available should therefore solely depend on RRC slot type configuration. |
| Nokia/NSB | It is our understanding from the proposal that this is just to reuse the Rel-15/16 behavior. Should be make it simple that the Rel-15/16 behavior is reused for RV cycling? The only difference here seems to be that transmission occasions can be on non-contiguous slots. |
| Sierra Wireless | We share the similar view as Qualcomm. |
| FL | The updated Proposal#2-2 is trying to capture the majority’s views on Issue#2-2, i.e. no need to change Rel-15/16 RV cycling, but just consider the available slots instead of the physical slots. Here I borrow Ericsson’s proposal. FL proposal #2-2 for the 2nd round discussion - RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17 For your reference, the related paragraph from TS38.214 is copied below. The red parts may need to be updated anyway such that the number of repetitions K is counted on the basis of available slots. On the other hand, the RV derivation is described in the third sentence and the Table 6.1.2.1-2. The above proposal #2-2 intends not to change the third sentence and the Table 6.1.2.1-2. Hope this clarifies the intention. For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2. Table 6.1.2.1-2: Redundancy version for PUSCH transmission rvid indicated by the DCI scheduling the PUSCH; rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B). n mod 4 = 0; n mod 4 = 1; n mod 4 = 2; n mod 4 = 3. 0; 0; 2; 3; 1. 2; 2; 3; 1; 0. 3; 3; 1; 0; 2. 1; 1; 0; 2; 3.. |
| Huawei, HiSilicon | Some clarifications are needed. - In the current spec, the RV is derived based on K consecutive slots as the spec excerpt shown by FL. Some spec changes are needed to replace them with “available slots”, then how can the note saying no spec impact for it at all? - The available slots may be time-varying, e.g. it can be pre-empted by URLLC PUSCH. Does the “available slots” in the proposal mean the available slots during counting, or the available slots for actual transmission? |

Issue#2-3: Inter-Slot Frequency Hopping Cycle

Similar to Issue#2-2, it is suggested discussing the following proposal as a starting point.

Initial FL proposal #2-3

- For inter-slot frequency hopping, hop index is derived based on the indexing of n within K transmission occasions (may be in K non-contiguous slots) which are determined by only RRC configurations, where K is the indicated/configured repetition factor.

Companies are invited to provide their views on the above proposal#2-3.

| Company | Comments |
| --- | --- |
| Intel | Our view is that we can simply follow the existing spec for inter-slot frequency hopping. The only difference may be determination of K. However, if we agree to use RRC configuration to determine available UL slots for repetition type A, it seems we do not need to change the spec. |
| ZTE | It’s not clear why any change is needed. The issue (potential uneven distribution of hops in TDD system) exists in Rel-15/16, the proposed enhancement may not be able to provide clear performance gain. In addition, inter-slot FH with inter-slot bundling is discussing in AI 8.8.1.3. It’s better to wait for the design there. |
| Apple | We don’t see the necessity to change the spec. |
| Ericsson | For Type A PUSCH repetition, start PRB of a hop is determined based on slot index, which can be reused by enhanced Type A PUSCH repetition in Rel-17 in our view, no specification change is needed. For joint channel estimation, we may need to determine different FH patterns, e.g. change startPRB every N slots. However, these should be discussed in 8.8.1.3 as is also pointed out by ZTE. |
| Panasonic | We share the same view as ZTE that it is better to wait for the design on inter-slot hopping in AI 8.8.1.3. |
| Sharp | OK to discuss it. |
| LG | Specification change is not necessary. Frequency hop index for PUSCH repetition Type A is determined based on the slot number within a radio frame. |
| OPPO | It would be good to have hopping index based on the real repetition as in the previous questions. |
| Xiaomi | Specification change is not necessary. |
| NEC | Agree with Intel. |
| Lenovo, Motorola Mobility | Don’t see the need for this enhancement |
| Nokia/NSB | We are open for further discussion on this aspect. However, this should be considered as advanced design aspect (further optimization) hence can be considered after the basic framework of this feature has been finalized. |
| Sierra Wireless | Specification change is not necessary. |
| Huawei, HiSilicon | Specification change is not necessary. |

Issue#2-4: Timeline aspect for the detemination of available slots

The following two alternative were listed in the conclusion from RAN1#104-e.

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

If Alt 1 in Issue#2-1 is agreed as suggested in FL proposal #2-1, there is no need to discuss this issue, since Alt 1 automatically leads to Alt-a.

Initial FL proposal #2-4

- The determination of all the available slots has to be done prior to the first actual transmission of the repetitions. (i.e. Take Alt-a of the previous conclusion)

At the same time, exchanging views on the following points may help the progress of Issue#2-1 discussion. Therefore, companies are invited to provide their views on the following points:

- Reasons to propose Alt-a

  - It enables cross-slot channel estimation/DMRS bundling.

  - It simplifies UE implementation as it does not require recounting.

  - Alt-a also simplifies hopping determination.

- Reasons to propose Alt-b

  - Alt-b reuses Rel-15/16 PUSCH omission mechanism (i.e. per-slot based timeline requirements).

| Company | Comments |
| --- | --- |
| Qualcomm | Support. Adding to the two reasons the FL has already captured, here is one more reason: asking the UE to perform a check for every slot makes the system error prone --- one instance of misunderstanding between UE and gNB derails entire sequence of transmissions since RV indexing then goes out of sync. Its good to keep in mind that we are designing for a cell-edge UE, so the issue of UE missing DCIs could occur more frequently. |
| Intel | We are fine with the proposal. |
| China Telecom | We support this proposal. |
| ZTE | It depends on the outcome of issue#2-1. If the available slot is only determined by RRC configuration and SFI, we are fine with the proposal. Note that, as we commented above, dynamic SFI would not cause any ambiguity, and the collision between SFI and PUSCH transmission is not expected to happen based on current specification. If the available slot can also be affected by other dynamic signalling, e.g., UL CI or high priority PUCCH, Alt-b is preferred as it can relieve gNB scheduling restriction. We want to highlight that this also depends on the outcome of Issue#2-9. |
| vivo | Support the FL proposal |
| Samsung | This proposal is not needed and is not agreeable. Rel-15 and Rel-16 already implement Alt-b (e.g. for PUCCH repetitions or for cancellations of repetitions). There is obviously no issue with UE complexity. There is also no relevance to cross-slot channel estimation (it is either possible if a repetition is not dropped, or not possible otherwise) and hopping determination can be based on the number of physical slots for either Alt-a or Alt-b (no difference). |
| Apple | Support this proposal. The Alt-a could avoid the misalignment on repetitions between gNB and UE. |
| Ericsson | Fine with the proposal. As we comment on issue 2-3, for hopping determination, we can reuse what we have in Rel-16 for startPRB determination for Type A PUSCH repetition, no specification change is needed. |
| NTT DOCOMO | We support the FL proposal, so that simple and steady approach to determine the available slots is reasonable. |
| Panasonic | We support the proposal. If Alt.1 or Alt.3 is taken, Alt.a is supported automatically. |
| Sharp | Rel-15/16 PUSCH omission mechanism cannot apply to the Rel-17 available slot determination as it is. Rel-15/16 PUSCH omission is affected by overlapping with SR or CG-PUSCH (for CG PUSCH with repetition case), but the gNB is not aware of the presence of those UL transmissions. Therefore, such simple reuse of Rel-15/16 PUSCH omission mechanism leads to different understanding of available slots between UE and gNB. |
| CATT | Support. This is corresponding to the proposed Issue#2-1. |
| LG | We support the proposal. |
| OPPO | We support this proposal. Even a slot is determined as available, it does not mean it is an actual repetition slot, i.e. the PUSCH repetition may be dropped in available slot. |
| Xiaomi | We are fine with the proposal. |
| CMCC | Support. It is important to keep the gNB and UE have a same understanding of how many repetitions or slots could be used. There is no strong need to further discuss the relation with other features, e.g. channel estimation or hopping, though it could benefits those operations. |
| NEC | Support the proposal. |
| WILUS | We support the FL’s proposal. |
| Lenovo, Motorola Mobility | We support the proposal |
| InterDigital | We support the proposal. CI and URLLC deprioritization do not change slot “availability” and whether a slot is considered available should therefore solely depend on RRC slot type configuration |
| Nokia/NSB | We can discuss this proposal after having progress on Issue#2-1. It’s our understanding that Alt-b fully aligned with the intention of “counting number of repetitions on the basis of available UL slots”. |
| Sierra Wireless | We support this proposal. |
| Huawei, HiSilicon | Support FL’s proposal in principle. But clarification is needed as commented to issue#2-1. |

Issue#2-5: Semi-static configurations to be used for the detemination of available slots

Base on the analysis in section 2.2.5, the following proposals were made.

Initial FL proposal #2-5

- ssb-PositionsInBurst (i.e. SSB configuration) is used for determination of available slots. Flexible symbol(s) for the reception of SSB is determined as unavailable.

Companies are asked if the above proposal #2-5 is agreeable, and also encouraged to provide their views on use of the following RRC configurations for determination of available slots:

  - SSB based measurement by SMTC

  - CORESET0 with Type0-PDCCH CSS set

  - Invalid UL symbols for DL-to-UL switching purpose

  - Other CG-PUSCH with larger priority index

  - PUCCH with larger priority index carrying HARQ-ACK for SPS

  - Semi-static PUCCH with repetition

| Company | Comments |
| --- | --- |
| Qualcomm | Support. We can also add CORESET0 to the list. Others don’t seem critical. |
| Intel | We think on top of SSB transmission, we also need to consider CORESET0 with Type0-PDCCH CSS set and invalid UL symbols for DL-to-UL switching purpose to ensure that UE has sufficient number of symbols for each repetition. |
| ZTE | Support the proposal. We think the last two bullets in the list can be considered also. |
| vivo | For RRM measurement based on SSB, SMTC can be configured for UE to determine the measurement resource. The SMTC configuration is provided per frequency layer, which may cover SSB occasions from multiple serving cells in the frequency layer. Hence, the SSB occasions configured by SMTC configuration is not necessary the same as the SSBs indicated by ssb-PositionsInBurst for the serving cell. As shown in the following figure, the serving cell only transmit SSB#0, and symbols for SSB#1 is not configured with SSB for serving cell. While SSB#1 is transmitted on the neighbouring cell in the same frequency layer, UE can not transmit PUSCH on these occasions. As specified in 38.133, on the SSB symbols configured by SMTC in TDD bands, UE is not expected to transmit PUSCH. ##### 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.. Both NW and UE are aware of the SMTC configurations, there is no ambiguity if these symbols are counted as not available. Hence, the flexible symbols configured with the SSB measurement by SMTC is not considered as available for type-A PUSCH repetition. |
| Samsung | Support. Also OK with CORESET#0. |
| Ericsson | Not necessary. Available slot determination based on the semi-static TDD-UL-DL configuration is simply enough in our view. We do not have to associate the availability of a slot with all legacy omission rules used to determine whether a PUSCH should be transmitted since the omission rules can be further applied on the determined available slots anyway. For example, following rules with respect to the collision between SSB and other uplink channels in current spec. is enough and there’s no need to use these rules to determine available slots: For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. If a UE - is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells.. |
| NTT DOCOMO | We support the FL proposal. |
| Panasonic | We support the proposal. |
| Sharp | Support the FL proposal #2-5. We think at “least CORESET0 with Type0-PDCCH CSS set” configuration and “Semi-static PUCCH with repetition” configuration should be used for the determination of available slots so that unnecessary omissions can be avoided. |
| CATT | Understand the motivation. However, we think whether a slot is available or not is determined just the same with legacy rules. As point out by Ericsson, the specification already shows the case (mentioned by FL) when a slot is unavailable for PUSCH transmission in TDD, by taking semi-static parameters into consideration. For another example, current TS 38.213 already precludes the PUSCH transmission overlapped with CORESET#0 for Type0-PDCCH CSS in TDD case. And vivo’s suggestion is also another example (already captured in TS 38.133). If any RRC parameter should be taken into consideration additionally, we are open to discuss. But currently we do not feel there is any difference. |
| LG | Support the proposal. We’d like to apply the same principle with the available slots determination rule for Rel-16 PUCCH repetitions. |
| OPPO | We support this proposal. The bullet can be: For determination of available slots, flexible symbol(s) for the reception of SSB indicated by ssb-PositionsInBurst is determined as not for PUSCH transmission. |
| Xiaomi | Support the proposal and add CORESET0 with Type0-PDCCH CSS set into consideration As for invalid UL symbols for DL-to-UL switching, it should be considered for some half duplex FDD redcap UE use cases, we are open to discuss it. |
| CMCC | Support the proposal. We share a similar view that current specification is enough to determine the available slot. |
| NEC | Support the proposal. |
| WILUS | Support. Also, “CORESET0 with Type0-PDCCH CSS set” and “Semi-static PUCCH with repetition” can be take into accounted for the determination of available slots. |
| Lenovo, Motorola Mobility | We support the proposal For further consideration, CORESET#0 and PUCCH with larger priority index carrying HARQ-ACK for SPS can be added as well |
| Nokia/NSB | We are fine with the proposal. Any channel configured by RRC (i.e., no discrepancy due to dynamic signaling) that may overlap with PUSCH repetitions should be considered for the determination of available slots. |
| Sierra Wireless | As mentioned by others, there are other factors to consider. One way to fix this is to add “at least” in front of the proposal: - At least, ssb-PositionsInBurst (i.e. SSB configuration) is used for determination of available slots. Flexible symbol(s) for the reception of SSB is determined as unavailable. |

Issue#2-6: Special slot handling

According to the contributions for RAN1#105-e, only one company prefer having an exception that use of special slots does not require all the symbols indicated by TDRA to be valid for UL transmissions, although in RAN1#104-e more companies were in favor of it. Considering the situation, it is suggested checking if the following proposal is acceptable.

Initial FL proposal #2-6

- For defining available slots: a special slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the special slot overlaps with the symbol not intended for UL transmissions.

Companies are asked if the above proposal #2-6 is acceptable.

| Company | Comments |
| --- | --- |
| Qualcomm | This discussion seems unnecessary. We already have an agreement that says in each available slot, all symbols needed |
| Intel | It is not clear to why we need to discuss this proposal. In the RAN1#104-e meeting, it was agreed - For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions This agreement also covered all the slots, including special slots. We do not think we need to discuss this proposal. |
| China Telecom | As pointed out by FL in section 2.2.6, the use of special slots should be clarified based on the FFS in the following agreements. Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details In our understanding, FL proposal #2-6 cannot make full use of special slots. TDRA should be separately indicated for special slots. |
| ZTE | Agree with Qualcomm and Intel. |
| vivo | Agree with Qualcomm and Intel. |
| Samsung | We assume that the existing agreement applies to any slot. |
| Apple | Agree with Qualcomm and Intel. |
| Ericsson | No need to discuss this, the agreement is clear as pointed out by Intel. |
| Panasonic | We share the same view as Qualcomm, Intel, ZTE, vivo, and Apple. This issue was concluded in RAN1#104e. |
| Sharp | This issue can be de-prioritized. The previous agreement should be considered to also apply to special slots unless the necessity of the exception is justified. |
| CATT | While the main bullet of the aforementioned agreement seems clear, we are not sure the what ‘FFS details’ is about. Is it left to further discussion on the ‘TDRA for a PUSCH’ or ‘not intended for UL’ or anything else? |
| LG | Agree with Qualcomm and Intel. |
| OPPO | We want to clarify whether flexible symbols can be as the symbol intended for UL transmissions. |
| Xiaomi | No need to discuss it, existed agreement is clear. |
| CATT | While the main bullet of the aforementioned agreement seems clear, we are not sure the what ‘FFS details’ is about. Is it left to further discussion on the ‘TDRA for a PUSCH’ or ‘not intended for UL’ or anything else? |
| NEC | Agree with Qualcomm. |
| WILUS | Agree with Qualcomm and Intel. |
| Lenovo, Motorola Mobility | Agree with Qualcomm and Intel |
| Nokia/NSB | We share the same view with the majority that the agreement made in RAN1#104e clarified this issue. |
| Sierra Wireless | Agree with Qualcomm and Intel and this goes against a previous agreement. |
| Huawei, HiSilicon | OK with FL’s proposal |

Issue#2-7: Limitation of overall duration of PUSCH repetitions

Issue#2-7 will be discussed after concluding the discussion on Issue#2-1.

Issue#2-8: Enhancements on PUSCH dropping

Since not many views have been provided in terms of enhancements on PUSCH dropping, it is suggested collecting companies’ views on the necessity of any new collision handling with other UL transmissions.

Initial FL proposal #2-8

- Discuss further:

  - Necessity of collision handling between PUSCH repetition Type A and the other UL transmissions, e.g. SPS HARQ-ACK, SRS

  - How to handle the collision, if any, e.g.

    - Dropping whole part of either PUSCH repetition or the colliding UL transmission

    - Partially dropping either PUSCH repetition or the colliding UL transmission

Companies are invited to provide their views on the above proposal #2-9.

| Company | Comments |
| --- | --- |
| Qualcomm | Don’t see any need to revise the existing rules. |
| Intel | Our view is that for repetition type A, we can simply follow existing collision handling rule after determination of available UL slots. We do not need additional rules to handle this. |
| ZTE | Share with Qualcomm and Intel, the motivation to revise the existing rules is unclear to us. |
| vivo | Agree with Qualcomm and intel. |
| Samsung | Changes of Rel-15/16 collision rules with other UL transmissions may depend on other proposals. We can revisit this proposal later. Overall we prefer to maintain the same Rel-15/16 overlapping rules. |
| Apple | Agree with Qualcomm and intel. |
| Ericsson | Collision handling may be needed between the enhanced Type A PUSCH repetitions and other UL channels introduced in Rel-17, e.g. enhancements of SPS HARQ-ACK, A-SRS are discussed in NR IIoT and URLLC WI and FeMIMO WI respectively. Priorities should be defined among these channels though we agree that the existing priority handling mechanisms in Rel-16 should be used for the collision handling in Rel-17. |
| Panasonic | We share the same view as Qualcomm and Intel. |
| Sharp | The existing omission rule should be reused unless any collision handling mechanism is investigated in other WIs. |
| CATT | Currently we do not see issues with the existing rules. If there is any, we can discuss later. |
| LG | In general, we are ok to keep the existing rules. However, as SRS with symbol length 8 and 12 is introduced in Rel-17 FeMIMO, it seems difficult and inefficient to avoid the collision of PUSCH and SRS. Hence, we can discuss prioritizing SRS transmission in case of SRS and PUSCH overlapping. |
| OPPO | We can postpone the discussion. |
| Xiaomi | Share the same view with intel and Qualcomm. | Share the same view with intel and Qualcomm. |
| CMCC | Since the maximum repetition number have been increased, more collisions could happens compared with Rel-15/16. We are open to discuss if the current rule needs to be updated. |
| NEC | Agree with Qualcomm. |
| Lenovo, Motorola Mobility | Agree with Qualcomm and Intel that existing rules are enough and no need to be enhanced |
| InterDigital | We can discuss this issue later. |
| Nokia/NSB | We share the same view with the majority that Rel-15/16 rules of collision handling should be reused. |
| Sierra Wireless | Although we feel that legacy dropping rules can be applied, we can also postpone the discussion as collision rules are typically discussed later in the work item. |
| Huawei, HiSilicon | For uplink coverage enhancement, a full utilization of precious UL resource is typically important as an enhancement solution. Thus, for collision with other UL transmissions, we prefer - Partially dropping either PUSCH repetition or the colliding UL transmission where remaining non-overlapping resource can be reused for PUSCH repetition. |

Issue#2-9: Enhancement on UCI multiplexing on PUSCH repetition

Based on the analysis described in section 2.2.9, the following proposal is made.

Initial FL proposal #2-9

- Discuss further whether/how HARQ-ACK multiplexing on PUSCH repetitions can be allowed if HARQ-ACK for the scheduling DCI comes after the UL grant of the PUSCH repetition transmission, taking the following options into account.

  - Option 1: HARQ-ACK bits for later DL assignments puncture the PUSCH repetition.

  - Option 2: When HARQ-ACK bits for the DL assignments later than UL grant is received, PUCCH with HARQ-ACK is transmitted and the PUSCH repetition is dropped or postponed.

  - Option 3: The time restriction on scheduling HARQ after UL grant is only applied to initial PUSCH repetition, and HARQ information bits corresponding to the PDSCH(s) scheduled after UL grant which triggers the PUSCH transmission are allowed to be multiplexed on the non-initial repetitions, where DAI in the last DCI applies.

Companies are invited to provide their views on whether/how HARQ-ACK multiplexing on PUSCH repetitions can be allowed if HARQ-ACK for the scheduling DCI comes after the UL grant of the PUSCH repetition transmission, taking the following options into account.

| Company | Comments |
| --- | --- |
| Qualcomm | Prefer to let this be discussed under R17-TEI. This seems out of scope of this sub-agenda. |
| Intel | We do not think we need to discuss this issue here. It is out of scope for enhancement on repetition type A. |
| ZTE | Propose to postpone the discussion considering this is discussing in Rel-17 TEI. |
| vivo | Generally, support this proposal. Suggest to discuss under this Coverage Enh AI, and this issue becomes more important because the DL scheduling would be even restrictive if the number of repetitions is extended or the repetition is counted based on available slots, which is also enhanced under this AI. For the UCI multiplexing on initial PUSCH repetition, the legacy timeline restriction can be reused. While for the later repetitions, the UCI can be multiplexed to the PUSCH by puncturing some PUSCH symbols, thus UE does not need to re-generate the PUSCH due to the UCI multiplexing. |
| Samsung | Option 3 for PUCCH without repetitions. Option 2 for PUCCH with repetitions (that is Rel-15 behavior – no agreement needed). |
| Apple | Agree with others, this enhancement is out of scope of WID. |
| Ericsson | This seems not related to the Type A PUSCH repetition. |
| NTT DOCOMO | We think the proposal is raised because of the enhancement of PSUCH Type A repetitions, so it’s better to discuss this topic in CovEnh WI. And for the technical pints, we understand the motivation. On the other hands, Options 1-3 cause miss understanding between gNB and UE, if UE can’t decode DCI for PDSCH scheduling, so that we need to carefully decide whether it’s necessary or not. |
| Panasonic | We share the same view as ZTE. |
| Sharp | OK to discuss it unless other companies disagree. We understand the motivation of the proposal on time restriction relaxation, and agree with enhancing the UCI multiplexing on PUSCH repetition. As for the options, Option 3 seems the simplest. |
| CATT | Seems not essential to the Type A PUSCH repetition enhancement. We can comeback after finishing the essential part if necessary. |
| LG | Discussion on this issue seems not necessary. |
| OPPO | Propose to discuss it latter |
| Xiaomi | No need to discuss it now or discuss it later. |
| CMCC | We share a similar view that in the CE WI, at least, we should have some discussion for this issue and do not leave this issue completely to the other agenda. Since the maximum repetition number could be very high, the collision would be more severe than that in Rel-15/16. |
| Lenovo, Motorola Mobility | We also think that this enhancement is not within the scope of discussion here |
| Nokia/NSB | We share the same view with the majority that this topic is out-of-scope of this sub-AI and should be discussed in TEI. |
| Huawei, HiSilicon | Prefer to discuss it in TEI-17 |

Issue#2-10: Configuration/indication of CovEnh functions

Although this issue is higher related to the UE features that should be discussed under a dedicated agenda item later, it is good to exchange companies’ views on it in order to have better understanding among companies on their proposed designs.

Initial FL proposal #2-10

- Discuss further the following options, in terms of configurations/indications of two enhancements, i.e., (a) increase of the maximum number of repetitions and (b) the repetitions counted on the basis of available slots:

  - Option 1: The enhancement (a) is RRC-configurable. The enhancement (b) is a basic feature of CovEnh capability.

  - Option 2: A set of the enhancements (a) and (b) is RRC-configurable (i.e., configured together).

  - Option 3: Either enhancements (a) or (b) is RRC-configurable (i.e., not configured together).

  - Option 4: Either enhancements (a) or (b) or both is RRC-configurable.

  - Option 5: Either enhancements (a) or (b) is dynamically-indicated.

Companies are invited to provide their views on the above option2.

| Company | Comments |
| --- | --- |
| Qualcomm | Option (1) seems closest to our preference. Although its not clear what is meant by RRC configuration for enhancement (a). Just to elaborate a bit, we think R17 should go with a single counting method. Hence (b) should be a basic feature of CovEnh capability and not need any further configuration. Regarding (a) UE can indicate capability to support increase repetitions. Besides this nothing more seems necessary; DG/CG configs can take care of the rest. |
| Intel | We slightly prefer Option 3. We think these two features need to be configurable, but they can be independent feature. For instance, if we increase the maximum number of repetition level, we do not see the need to transmit PUSCH repetition type A based on available UL slots. |
| China Telecom | Two enhancements in Rel-17 are independent to each other. UE can be configured both or one of them subject to UE capability. |
| ZTE | Option 4 is preferred, and we are also fine with Option 1. |
| vivo | Prefer option 3. Enabling both features simultaneously seems not necessary. |
| Samsung | Option 1 or Option 3. |
| Apple | Option 3 is preferred. If the maximum repetition is the same for (a) and (b), configuring both doesn’t make sense, configuring (b) is enough. |
| Ericsson | Option 3. No need to have the increased number of repetitions counted based on available slots. |
| NTT DOCOMO | We prefer Option 3. We think both features are independent, so that they can be configured separately. When applying (a), RRC signaling with Rel-18-IE is necessary to indicate number of repetition. And also it’s better to have RRC signaling for (b) to avoid the miss alignment between UE and gNB. We also think that applying both features simultaneously is not necessary. |
| Panasonic | We prefer Option 1 or Option 4. |
| CATT | From view of coverage improvement, (a) is more suitable for FDD bands and (b) is more suitable for TDD bands. If can be determined separate for FDD and TDD, we would suggest (a) a basic feature for FDD and (b) a basic feature for TDD. The other one is subject to UE capability and RRC-configurable. If not, we prefer Option 1 or 2 to avoid missing (b) in TDD case. |
| LG | We prefer Option 1. |
| OPPO | Option 1 and Option 3 are preferred. |
| Xiaomi | We prefer Option 4 and Option1. |
| CMCC | Option 1 is preferred. As discussed in the paper and previous meetings, the available slot is an efficient counting method, which could increase the actual repetition numbers even without the increase of the maximum repetition number. Then, it is more fair to define the enhancement (a) as RRC configured, in which the repetition factors could depends on gNB’s configuration. We have the sympathy that configure both (a) and (b) would increase the complexity of the UE and induce a much larger delay. But in the option 3, the UEs still have to realize both enhancements but only not use them at the same time. If a single configuration could realize a same effect, we do not see the necessary to have the overlapped functions but depending on gNB’s preference. |
| NEC | Prefer option 3. |
| Lenovo, Motorola Mobility | In our view, an RRC parameter can be used to signal that Rel-17 PUSCH repetition type A enhancement is configured. Then dynamically, TDRA table can imply whether (a) or (b) is applied. If the number of repetitions indicated by TDRA table is > 16, then (a) is applied, otherwise for 16 or a smaller number of repetitions, (b) is applied. In terms of above options, a combination of option 2 and option 5 should be agreed. |
| Nokia/NSB | We strongly prefer Option 1 with the understanding that Rel-17 only uses one unique counting method based on the available slots for UL transmissions (i.e., enhancement (b)). Then whether the maximum number of repetitions is further increased or not (i.e., enhancement (a)) is up to UE capability and, in the end, the practical number of repetitions is configured by the gNB. |
| Sierra Wireless | Slight preference to option 4 (as the functions are decoupled) but can also support option 1. |

Issue#2-11: Modification on βoffsetPUSCH

Since this issues was newly raised for this meeting, it is suggested discussing first if this issue is to be discussed in this AI.

Initial FL proposal #2-11

- Discuss first if Issue#2-11 is discussed in this AI.

Companies are invited to provide their views on whether to discuss modification of  βoffsetPUSCH. If yes, also provide views on the following proposal in [15]:

- Scaling  βoffsetPUSCH by N so as to make it more aligned to the originally intended behaviour, where N is the number of repetitions.

| Company | Comments |
| --- | --- |
| Qualcomm | Seems out of scope. |
| Intel | We do not think this is needed. |
| ZTE | This is out of scope, and it could be discussed as Rel-17 TEI if needed. |
| Samsung | The issue directly relates to aspects of PUSCH repetitions (UCI multiplexing). Open to suggestions for a more appropriate AI than this one. |
| Apple | This is out of scope of WID. |
| Ericsson | No need to discuss this in this aganda. |
| Panasonic | We don’t think this enhancement is needed. |
| Sharp | If necessary, TEI is a more suitable place to bring this proposal. |
| CATT | Not essential and no need to discuss now. |
| LG | Discussion on this issue is not necessary. UCI multiplexing is specified only for PUCCH without repetition, and the UCI is multiplexed within one PUSCH repetition. If more resource for UCI transmission is required, PUCCH repetition can be applied. |
| Xiaomi | No need to discuss. |
| CMCC | It seems out of the scope of this WI. |
| Lenovo, Motorola Mobility | This is out of scope |
| Nokia/NSB | We also think that this is out-of-scope of this sub-AI. |
| Huawei, HiSilicon | No need to discuss. |

Issue#2-12: Other issues

Companies are invited to provide other issues to be discussed in this meeting, if any, for the repetitions counted on the basis of available slots for UL transmissions.

| Company | Comments |
| --- | --- |
| XXX |  |

Summary of the 1st round discussion

Issue#1-1: The maximum number of repeitions

The following Initial FL proposal#1-1 was discussed in the Thursday GTW session. Although most of the companies were OKed with the proposal during the 1st round discussion. In the GTW session, 5 companies preferred agreeing the proposal without any condition, while 5 companies preferred adding the condition saying “ for a case of counting based on available slots”. The difference of the views come from different preferences on how the two enhancements are configured, which we have been discussing under Issue#2-10.

Initial FL proposal #1-1

- The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32.

After the Thursday GTW session, FL updated the proposal as the following, in order to capture two options raised in the GTW session.

Updated FL proposal #1-1

•         Select one of the following Alt 1 and Alt 2. The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is:

- Alt 1: 32 irrespective of counting method,

- Alt 2: 32 for the counting based on physical slots, and 16 (i.e. no change from Rel-16) for the counting based on available slots.

•         No other option is considered.

However, this proposal was still a bit too contentious to converge.

Issue#1-2: RRC parameters to be extended for supporting the increased maximam number

Initial FL proposal #1-2

- Discuss if repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions.

The 1st round discussion on the above proposal #1-2 is summarized as follows:

Whether repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions or not:

- Support (9 companies): Intel, China Telecom, vivo, Samsung, LG, OPPO, Xiaomi (IEs are up to RAN2), Nokia/NSB

- Not support (11 companies): Qualcomm, ZTE, Apple, Ericsson, NTT DOCOMO, Sharp, CATT, CMCC, Lenovo/Motorola Mobility, Sierra Wireless

Issue#1-3: Other candidate value set for configuration of the number of repetitions

Void

Issue#1-4: Other issues

No other issue identified.

Issue#2-1: Use of dynamic signaling for the detemination of available slots

According to the 1st round discussion,

- Alt 1: The determination of available slots does not depend on any dynamic signaling. Further omission of PUSCH repetition in the available slot is subject to dynamic signaling.

  - Support (19 comnapies): Qualcomm, Intel, China Telecom, vivo, Apple, Ericsson, NTT DOCOMO, Sharp, CATT, LG, OPPO, Xiaomi, CMCC, NEC, WILUS, InterDigital, Nokia/NSB, Sierra Wireless

- Alt 2 (4 companies): The determination of available slots depends on dynamic signaling including e.g., dynamic SFI.

  - ZTE (at least dynamic SFI), Samsung, Lenovo/Motorola Mobility

- Alt 3 (2 companies): The determination of available slots depends on dynamic signaling in the scheduling DCI only.

  - Panasonic, OPPO (1st preference)

The questions/concerns to Alt 1 were:

- For DG PUSCH, no conflict between dynamic grant and dynamic SFI is expected. Therefore, no ambiguity caused by dynamic SFI.

- For CG PUSCH, the UE will not transmit on flexible symbols. Therefore, no ambiguity caused by dynamic SFI.

- The issues from a potential miss of dynamic signaling exist in Rel-15/16.

- No support of dynamic signaling means that Rel-17 coverage enhancements cannot coexist with deployment of basic NR features.

- If semi-static flexible symbol can be used for PUSCH repetition, the design not depending on any dynamic signaling prevent to use flexible symbol as DL by SFI.

Issue#2-2: RV Cycle

According to the 1st round discussion, almost all the companies shared the views that the existing RV cycling mechanism should be reused as it with replacing “physical slot” to “available slot”. Based on it, the following updated proposal #2-2 was provided.

Updated FL proposal #2-2

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

  - Note: this has no spec impact in terms of the redundancy version derivation

The above proposal was discussed in the email discussion after the Thursday GTW session, and 13 companies supported the proposal while no company objected to the main bullet. For the sub-bullet, there were several comments saying that it was a bit too early to conclude with no spec impact at all considering the transmission occasions should corresponds to available slots, but not to physical slots as in Rel-15/16, though the mechanism of RV cycling on the transmission occasions should be reused. Therefore, the following updated FL proposal #2-2 was agreed. In the next round discussion, the meaning of available slots and the relation between transmission occasions and the available slots would be discussed.

Updated FL proposal #2-2

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

Issue#2-3: Inter-Slot Frequency Hopping Cycle

The companies’ inputs during the 1st round discussion are summarized as follows.

- For inter-slot frequency hopping, whether hop index is based on available slots or not.

  - Support (1 company): OPPO

  - No need (9 companies): Intel, Apple, Ericsson, LG, Xiaomi, NEC, Lenovo/Motorola Mobility, Sierra Wireless

  - Wait for joint channel estimation discussion (2 companies): ZTE, Panasonic

  - Open to discuss (3 companies): Sharp, Nokia/NSB

Issue#2-4: Timeline aspect for the detemination of available slots

The companies’ preferences according to the 1st round discussion are summarized as follows.

- The determination of all the available slots has to be done prior to the first actual transmission of the repetitions. (i.e. Take Alt-a of the previous conclusion)

  - Support (18 companies): Qualcomm, Intel, China Telecom, vivo, Apple, Ericsson, NTT DOCOMO, Panasonic, CATT, LG, OPPO, Xiaomi, CMCC, NEC, Lenovo/Motorola Mobility, InterDigital, Sierra Wireless

  - Not support (1 company): Samsung

  - Depends on Issue#2-1 (3 companies): ZTE, Nokia/NSB

Issue#2-5: Semi-static configurations to be used for the detemination of available slots

Base on the inputs, companies’ preferences are summarized below.

For the determination of available slots, use of:

- ssb-PositionsInBurst (i.e. SSB configuration)

  - Support (17 companies): Qualcomm, Intel, ZTE, Samsung, NTT DOCOMO, Panasonic, Sharp, LG, OPPO, Xiaomi, CMCC, NEC, WILUS, Lenovo/Motorola Mobility, Nokia/NSB

  - Not support: Ericsson, CATT

- SSB based measurement by SMTC

  - Support (1 company): vivo

- CORESET0 with Type0-PDCCH CSS set

  - Support (6 companies): Qualcomm, Intel, Samsung, Sharp, Xiaomi, WILUS

- Invalid UL symbols for DL-to-UL switching purpose

  - Support (1 company): Intel

  - Open to discuss (1 company): Xiaomi (for half duplex FDD RedCap)

- Other CG-PUSCH with larger priority index

- PUCCH with larger priority index carrying HARQ-ACK for SPS

  - Support (1 company): ZTE

- Semi-static PUCCH with repetition

  - Support (3 companies): ZTE, Sharp, WILUS

- Any channel configured by RRC (i.e., no discrepancy due to dynamic signaling) that may overlap with PUSCH repetitions

  - Support (1 company): Nokia/NSB

Issue#2-6: Special slot handling

Initial FL proposal #2-6

- For defining available slots: a special slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the special slot overlaps with the symbol not intended for UL transmissions.

The companies’ input to the above proposal during the 1st round discussion is summarized as follows.

- This discussion is not necessary / the agreement from RAN1#104-e covers special slots as well.

  - Qualcomm, Intel, ZTE, vivo, Samsung, Apple, Ericsson, Panasonic, Sharp, LG, OPPO, Xiami, NEC, WILUS, Lenovo/Motorola Mobility, Nokia/NSB, Sierra Wireless, Huawei/HiSilicon (21 companies)

- Should discuss

  - China Telecom (1 company)

- The sub-bullet in the previous agreement is not clear

  - CATT (1 company)

Issue#2-7: Limitation of overall duration of PUSCH repetitions

Void.

Issue#2-8: Enhancements on PUSCH dropping

The outcomes from the 1st round discussion are as follows.

- Necessity of modifications to the existing collision handling rules

  - Consider (4 companies): Ericsson (the existing rules + priorities on UL transmissions) , LG (SRS with symbol length 8 and 12), Huawei/HiSilicon (support partial dropping of PUSCH repetition)

  - Keep the existing rules as is  (16 companies): Qualcomm, Intel, ZTE, vivo, Samsung, Apple, Panasonic, Sharp, CATT, LG, Xiaomi, NEC, Lenovo/Motorola Mobility, Nokia/NSB

  - Open to discuss / Depends on other proposal / Defer the discussion (6 companies): Samsung, Sharp, OPPO, CMCC, InterDigital, Sierra Wireless

Issue#2-9: Enhancement on UCI multiplexing on PUSCH repetition

During the 1st round discussion, 14 companies expressed their views that this issue is out of CovEnh WI scope and/or this issue does not need to be discussed in this agenda. Meanwhile, 3 companies suggested discussing this issue under this agenda item.

Issue#2-10: Configuration/indication of CovEnh functions

Accortding to the inputs on the configuration/indication aspect during the 1st round discussion, companies’ preferences diverge a lot.

  - Option 1: The enhancement (a) is RRC-configurable. The enhancement (b) is a basic feature of CovEnh capability.

    - Support (9 companies): Samsung, Panasonic, CATT (if can be separate for FDD and TDD), LG, OPPO, Xiaomi, CMCC, Nokia/NSB, Sierra Wireless (2nd choice)

  - Option 2: A set of the enhancements (a) and (b) is RRC-configurable (i.e., configured together).

    - Support (1 company): CATT (if cannot be separate for FDD and TDD)

  - Option 3: Either enhancements (a) or (b) is RRC-configurable (i.e., not configured together).

    - Support (8 companies): Intel, vivo, Samsung, Apple, Ericsson, NTT DOCOMO, OPPO, NEC

  - Option 4: Either enhancements (a) or (b) or both is RRC-configurable.

    - Support (4 companies): China Telecom, Panasonic, Xiaomi, Sierra Wireless (1st choice)

  - Option 5: Either enhancements (a) or (b) is dynamically-indicated.

    - Support (2 companies): Lenovo/Motorola Mobility

  - Option 6: The enhancement (a) is tied to the UE capabitily for (a). The enhancement (b) is a basic feature of CovEnh capability.

    - Support (1 company): Qualcomm

  - Option 7: For FDD, the enhancement (a) is a basic feature of CovEnh capability, and the enhancement (b) is RRC-configurable. For TDD, the enhancement (a) is RRC-configurable, and the enhancement (b) is a basic feature of CovEnh capability.

    - Support (1 company): CATT (if can be separate for FDD and TDD)

Issue#2-11: Modification on βoffsetPUSCH

During the 1st round discussion, 15 companies expressed their views that this issue is out of CovEnh WI scope and/or this issue does not need to be discussed in this agenda.

Issue#2-12: Other issues

No other issue identified.

Second round discussion

Issues for the 2nd round discussion

[Open] Issue#1-1: The maximum number of repeitions

It is suggested collecting companies’ views on the maximum number of repetitions for each counting method (i.e. the counting based on physical slots and the counting based on available slots).

- The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is:

  - Alt 1: 32 irrespective of counting method

    - Qualcomm, CATT, vivo, Xiaomi, ZTE, Sharp, OPPO, China Telecom (1st choice)

  - Alt 2: 32 for the counting based on physical slots, and 16 (i.e. no change from Rel-16) for the counting based on available slots

    - Intel, Panasonic, China Telecom (2nd choice)

  - Alt 3: 20 for the counting based on physical slots, and 16 (i.e. no change from Rel-16) for the counting based on available slots

    - Ericsson

Companies are encouraged to provide their preferences if not provide yet.

| Company | Comments |
| --- | --- |
| Samsung | Alt 1 |
| Lenovo, Motorola Mobility | Alt 2, and Alt 3 as second preference. With Alt 1, it is overoptimization by combining the two solutions i.e. increasing the number of repetitions and then further counting on basis of available slots. This is could also result in quite a long duration over which 32 repetitions can be done on available UL slots. In order to deal with this, further discussion/agreement on restricting the overall duration is additionally required. Furthermore, with Alt 2, even the signaling aspect would be simpler. For indicated repetitions > 16, no explicit signaling will be needed to indicate whether counting is done based on physical slots or available slots. |
| Ericsson-2rd | As we’ve discussed a lot in the first round, we share the majority view that the 2 Type A PUSCH enhancement features in Rel-17 should be treated independently, and actually the 2 features were meant to be down-selected in WI phase in the end of the study item phase (see below agreement) though finally both features were agreed to be supported. Agreements: Capture the following observation into the TR. - Enhancements on PUSCH repetition type A is beneficial for PUSCH coverage enhancements for TDD. It is recommended to support enhancements on PUSCH repetition type A in Rel-17, including the following two options (potential down-selection during the WI phase): - Option 1: Increasing the maximum number of repetitions, e.g., up to 32. - Option 2: The number of repetitions counted on the basis of available UL slots. Alt1 indicates that the 2 features will be combined which should be at least discussed separately and this depends on how we solve Issue#2-10 summarized in this document. So, we do not think we need this alternative at this stage and propose to discuss issue #2-10 first. For the maximum number of increased repetitions counted based on physical slots (option 1 in above agreement), 20 is enough as we proposed and clarified in our contributions. As a compromise, our second preference is to support up to 32 repetitions counted based on physical slots. For the maximum number of repetitions counted based on available slots (option 2 in above agreement), there’s need to discuss the maximum number of repetitions which should be the same as the maximum number of repetitions in NR Rel-16 Type A PUSCH repetition, what we need to do is just 1 bit to decide whether to determine the repetitions in Rel-16 based on available slots. Given above, to move forward, we propose: Initial FL proposal #1-1 - The maximum number of Type A PUSCH repetitions counted based on physical slots in Rel-17 PUSCH is 32 or 20 to be down-selected in RAN1 #105-e meeting. - FFS the maximum number of Type A PUSCH repetitions counted based on available slots in NR Rel-17, depending on whether the 2 Type A PUSCH repetition enhancement options can be combined or not. |
| Apple | Alt 1 is preferred. Based on the evolution results from SI, the coverage gap of PUSCH is larger than 6dB. If repetition number is doubled, it could provide 08.dB SINR gain. If the same maximum repetition number for both counting methods is adopted, it provides additional scheduling flexibility to compensate the coverage gap via counting on available slot. |
| CMCC | Alt 2 and alt 3 is slightly preferred as 16 repetitions based on available slot counting is good enough for the coverage enhancement. |
| Intel | We prefer Alt. 2 and share similar view as CMCC. |
| NTT DOCOMO | We prefer Alt.2, since 16 repetitions is enough for PUSCH repetition based on available slot. |
| vivo | Alt.1 is preferred. |
| CATT | We prefer Alt1. Though it may be a little over-optimized in some cases with all possible configurations, it is still under gNB’s control anyway. With max=32, A gNB can still configure/schedule actual repetition number <32 by implementation. In some scenarios, a gNB may like to combine both increased repetition number and count on available slot for extreme coverage compensation. |
| Qualcomm | We prefer Alt 1. Alt 2 appears to unnecessarily couple the enhancements. We don’t see any technical benefits of Alt 2 as well. |
| OPPO | Support Alt 1. - Regarding on the counting based on available slots, there is also the case that actual PUSCH repetition is dropped on the available slots. That is, the number of actual PUSCH repetition is less than the configured number. Maybe 16 actual PUSCH repetitions can guarantee the coverage performance, it is also beneficial to support a larger number of repetitions than 16 because the number of actual repetitions may be less than the configured number. - The maximum number 32 counting based on available slots is also beneficial for PUSCH repetition in NTN. |
| LG | Prefer Alt 1. |
| Nokia/NSB | We prefer Alt. 1. |
| Xiaomi | Alt.1 |
| Huawei, HiSilicon | Alt.2 or Alt.3 |

[Open] Issue#1-2: RRC parameters to be extended for supporting the increased maximam number

According to the 1st round discussion, 11 companies expressed that the motivation to support repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports unclear.

Companies (especially those who prefer to support it) are encouraged to provide justifications to support repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

| Company | Comments |
| --- | --- |
| Samsung | Coverage enhancement is also beneficial for configured transmissions. A UE should be able to benefit from a larger repetition factor also when the repetition factor in the TDRA table is not configured, by using pusch-AggregationFactor or repK. If an increase of the repetition factor in the TDRA table is agreed, the repetition factor configured in PUSCH-Config and ConfiguredGrantConfig is also increased. |
| Ericsson-2rd | Same comments as we provided in 1st round. Increasing number of repetitions in TDRA table based on NR Rel-16 is enough. |
| Apple | As we already agreed to indicate the increased the repetition number via TDRA table. The necessity of introducing RRC configured repetition number is not clear. |
| CMCC | The extension of numberOfRepetitions could support both configured grant and dynamic grant transmissions. When numberOfRepetitions is configured, the repetitions will follow the numberOfRepetitions no matter which value is configured for pusch-AggregationFactor, or repK. |
| Intel | As commented in the first round of discussion, we think repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig should support the increased maximum number of repetitions. When UE is not configured with number of repetitions in the TDRA table, UE needs to follow the pusch-AggregationFactor configured in PUSCH-Config. In this case, sufficient number of repetitions for PUSCH can still be possible, which can help meet the coverage enhancement target. This also applies for repK in ConfiguredGrantConfig |
| vivo | If the repetition factor is configured for TDRA, the TDRA table with repetition factor also applies to configured grant. For a cell edge UE, we don’t think it is typical case that the repetition factor is not configured in TDRA for dynamic grant, while repetitions for configured grant is required. Hence, we think repetition number in TDRA is sufficient. |
| CATT | The maximum repetition number is already achieved by parameters in TDRA table. No need to bother other parameters. The handling is similar to the case in Rel-16 of increasing repetition number to 16 for URLLC, but no change for pusch-AggregationFactor or repK. |
| OPPO | There is no good for the number of repetitions for R17 can only be indicated by a parameter. |
| Nokia/NSB | As mentioned in the 1st round, it is our understanding that the Rel-16 dynamic indication of number of repetitions configured in numberofRepetitions is an optional feature, therefore increasing the maximum repetition factor configured in PUSCH-Config can be supported to make use of the Rel-17 feature on those UEs that cannot support dynamic indication of repetition factor in Rel-16. |
| Xiaomi | The maximum repetition number is supported for config grant and dynamic grant transmission. Thus, other RRC parameters are not necessary. |

[Pending] Issue#1-3: Other candidate value set for configuration of the number of repetitions

Issue#1-3 will be discussed after concluding the discussion on Issue#1-1.

[Closed] Issue#1-4: Other issues

[Open] Issue#2-1: Use of dynamic signaling for the detemination of available slots

As the large majority supported Alt 1, the 2nd round discussion focuses on clarifications on Alt 1.

- Alt 1: The determination of available slots does not depend on any dynamic signaling. Further omission of PUSCH repetition in the available slot is subject to dynamic signaling.

The questions/concerns to Alt 1 were:

- For DG PUSCH, no conflict between dynamic grant and dynamic SFI is expected. Therefore, no ambiguity caused by dynamic SFI.

- For CG PUSCH, the UE will not transmit on flexible symbols. Therefore, no ambiguity caused by dynamic SFI.

- The issues from a potential miss of dynamic signaling exist in Rel-15/16.

- No support of dynamic signaling means that Rel-17 coverage enhancements cannot coexist with deployment of basic NR features.

- If semi-static flexible symbol can be used for PUSCH repetition, the design not depending on any dynamic signaling prevent to use flexible symbol as DL by SFI.

Companies (especially Alt 1 supporting companies) are invited to provide their views on above questions/concerns.

| Company | Comments |
| --- | --- |
| Samsung | We agree with all concerns on Alt.1. It is better to set aside this proposal and consider whether or not it applies on a case-by-case basis as related to CovEnh features. For example, for determining hopping intervals, it is fine to not consider dynamic signaling; however, for determining whether or not a repetition can be transmitted in a slot, it does not make sense (and reverts R15/R16) to not consider whether or not that repetition has been cancelled - that may even be due to a DCI triggering a PUCCH that overlaps with a PUSCH repetition (that is also dynamic signalling). It would be unprecedented to make any agreement that is incompatible with existing specifications and NR operation and is functionally flawed. |
| Ericsson-2rd | Same comments as we’ve provided in first round. Semi-static RRC signaling is enough to determine the availability of a slot. Omission rules due to dynamic signaling to determine the actual transmissions of PUSCH will still be reused, meaning that whether flexible symbols can be used or not for DG/CG PUSCH are still the same as legacy. Available slot determination is just a step before actual transmission determination to filter out some of the downlink slots indicated by semi-static TDD UL/DL configurations. |
| ZTE | Agree with above concerns on Alt.1. As commented in the first round, at least dynamic SFI should be considered as no ambiguity could be caused by dynamic SFI no matter it is received or not by the UE. For the updated Alt1, could you clarify the intention of treating the ‘counting’ and ‘omission’ mechanism differently. Is it correct understanding that, the omission mechanism will follow legacy rules (as copied below), and we will only focus on the counting mechanism here? ‘For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].’ |
| Apple | Seems there is an editorial error in the proposal - Alt 1: The determination of available slots does not depend on any dynamic signaling. Further omission of PUSCH repetition in the available slot is NOT subject to dynamic signaling. We don’t any issues for first three question. For the fourth question, don’t why can’t co-exist with deployment of basic NR features? For the fifth question, don’t see the issue here, the network makes sure there is no conflict between PUSCH repetition with DL reception on flexible symo |
| CMCC | For the concerns of Alt 1, The 1st and 3rd bullets are related with the mis-alignment between UE and gNB. Especially for the 1st bullet, not all the flexible symbols could be used for the transmission, some of which should cover the round trip time. We cannot accept the idea that the flexible symbols could always be available for the dynamic indicated transmission and repetitions. From my understanding, the 4th bullet is also focused on the SFI indication, it is confusing why there is a co-existence issue for not taking the SFI into account. For the other dynamic signaling, it could be solved by the post-operations, such as do not counting it as a repetition actually happened. |
| Intel | We do not think not supporting dynamic signalling means that coverage enhancement feature cannot exist with deployment of basic NR feature. In our view, there are two steps for actual transmission of repetition type A - Step 1: determination of available slot for N repetitions based on RRC configuration - Step 2: if any additional collision due to dynamic signaling, e.g., dynamic SFI, UL CI, priority indication, etc., UE drops the PUSCH repetition, but does not further postpone the repetition. Note that same design principle was already supported and specified for PUCCH repetition in TDD in Rel-15. We do not see any issue to follow similar mechanism for PUSCH repetition type A. |
| Sharp | Regarding the procedure of the counting and the omission, I fully agree with Intel. Indeed, it is also aligned with Rel-15/16 PUSCH procedure described in 38.214. TS38.214 For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2. <Omitted> For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].. In Rel-15/16, the number of repetitions, K, is counted based on consecutive slots. K slots corresponds to K transmission occasions. A PUSCH in a transmission occasion can be omitted according to the PUSCH omission rules (defined in Clauses 9, 11.1 and 11.2A.) In Rel-17, K is counted based on available slots, and as discussed in Issue#2-2, K transmission occasions correspond to the K available slots. There is no need to change to the omission procedure in which a PUSCH in a transmission occasion can be omitted according to the PUSCH omission rules. To clarify this, we propose modifying the Alt 1 as follows: - Alt 1: The determination of available slots during repetition count does not depend on any dynamic signaling. Further omission of PUSCH repetition in the available slot is subject to dynamic signaling Rel-16 PUSCH omission rules. To Samsung and ZTE, the following comments are to address the above listed concerns: - For DG-PUSCH with dynamic SFI, Rel-15/16 do not allow any conflict. In other words, dynamic SFI does not affect the determination of whether the DG PUSCH is actually transmitted or not. In this sense, as in Rel-15/16, the determination of available slots for Rel-17 DG PUSCH with repetition should NOT depend on dynamic SFI. - For DG/CG-PUSCH with UL-CI or with overlapping with high priority channels, different understanding of available slots between UE and gNB happens if the UE fails to detect the UL-CI or the DCI scheduling the high priority channels. Such mis-alignment leads to RV mismatch between UE and gNB. In Rel-16, detection failure didn’t cause the RV mismatch. - For CG PUSCH with dynamic SFI, when the UE fails to detect the dynamic SFI which cancels the CG-PUSCH, the same problem as DG/CG-PUSCH with UL-CI or with overlapping with high priority channels happens. - As mentioned above, Rel-15/16 also had a potential miss of dynamic signaling. However, such miss causes the RV mismatch in Rel-17 PUSCH with the counting based on available slots, while Rel-15/16 did not have the RV mismatch problem. In the operation with Alt 1, the network can use dynamic signaling as in Rel-15/16. Even with Alt 1, PUSCHs on the available slots should be able to be omitted according to dynamic signaling later on. As this omission procedure is exactly the same as in Rel-16, the counting method does not cause any difference in terms of flexibility of use of the dynamic signaling. |
| WILUS | We share the similar view with Intel and Sharp. Available slot means the slot that PUSCH transmission is dropped and deferred to the next available slot, while unavailable slot means the slot that PUSCH transmission is dropped and not deferred. |
| vivo | Agree with intel’s understanding. |
| CATT | Several preliminary views from us: 1. No ambiguity on SFI, but seems also no need for SFI. Understand that DG-PUSCH can be transmitted in flexible symbols (determined by semi-static parameters), and transmissions on flexible symbols does not expected to conflict with SFI indication. Doesn’t it show that whether a slot is available does not need to consider SFI? 2. Similarly, CG-PUSCH cannot be transmitted in flexible symbols (determined by semi-static parameters) regardless of SFI indication. If so, seems CG-PUSCH does not need to determine available slots based on SFI either. 3. Not sure why this is an argument... we do not have to additionally introduce something risky in Release-17. 4. Hard to understand. Anyway, with Alt.1 even a slot is determined as available (by semi-static parameters), it is not the ‘actual transmitted slot’ and may still ‘be omitted’ due to dynamic signaling (e.g. PUCCH with repetition). This does not conflict with current specification. 5. If a gNB would like to indicate the flexible symbols as DL by SFI, it should avoid dynamically scheduling PUSCH transmission to overlap with the flexible symbols. |
| Qualcomm | Echo Intel’s comments. This is a two step procedure. Step 1 identified available slots and must be as robust as possible. Step 2 determines if an actual transmission is possible; here dynamic signaling is taken into account. |
| OPPO | Similar comments as we’ve provided in first round. The determination of available slots depends on “RRC configuration” or “RRC configuration+ scheduling DCI” prior to the first actual transmission of the repetitions. Dynamic signal can influence the actual repetition in available slot, i.e. if any additional collision due to dynamic signaling, the transmission in this available slot is dropped, but still counted in the total number of repetitions for PUSCH repetition. |
| Nokia/NSB | We support the modified version from Sharp, which further clarifies the “available slots” and “omission rules”. This seems to also reflect the understanding on two-step procedure mentioned by other companies. |
| Xiaomi | Agree with intel |

[Open] Issue#2-2: RV Cycle

According to the email discussion after Thursday GTW session, the following proposal seems pretty much close to the consensus.

FL proposal #2-2 for the 2nd round discussion

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

Note: this has no spec impact in terms of the redundancy version derivation

Companies are invited to provide their views on if removing the sub-bullet is acceptable.

| Company | Comments |
| --- | --- |
| Samsung | No strong opinion on the note. The clarification should be only about the slots, not on any procedure. |
| FL | This discussion is closed. |

In the email discussions before Quiet period, the main bullet of the above proposal was agreed.

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

To further clarification of the above agreement, it is suggested to discuss the following aspect.

FL proposal #2-2 for the 2nd round discussion

- Each available slot identified by the UE is treated as a transmission occasion for PUSCH repetition.

Companies are invited to provide their views.

| Company | Comments |
| --- | --- |
| Ericsson-2rd | Seems this discussion is not necessary in our view. The available slot determination is handled in issue 2-1, and a PUSCH may also be not able to be transmitted on an available slot due to dynamic signaling. |
| ZTE | Given it is currently proposing to treat the ‘counting’ and ‘omission’ mechanism differently, we would like to clarify the intention of this proposal. Does it mean, as long as one slot is determined as available for counting, it will be regarded as one transmission occasion when determining the RV cycling, no matter the transmission occasion would be omitted or not? |
| Apple | Agree with FL’s Proposal. |
| CMCC | General fine with the current version. |
| Panasonic | We support the FL proposal. |
| Intel | We are fine with the proposal. |
| NTT DOCOMO | We support the FL proposal. |
| Sharp | Agree. In the current spec, the RV cycling on the transmission occasions is defined, and the transmission occasions are corresponding to “consecutive slots”. For PUSCH repetitions counted based on available slots, the transmission occasions should be tied to the available slots in order to achieve the agreed behavior by the minimal spec changes |
| WILUS | We support the FL’s proposal. |
| InterDigital | We are ok with the FL’s proposal as long as RV is cycled for transmission occasion that is omitted. Jut to be clear that the behavior is aligned with Rel. 15/16, would it be possible to modify the proposal as below? Each available slot identified by the UE is treated as a transmission occasion for PUSCH repetition, i.e., RV is cycled for an omitted transmission occasion |
| vivo | Agree. It seems nature, considering we already have the agreement that the RV cycling is based on available slots… |
| CATT | Agree with the proposal. |
| Qualcomm | Support. @ZTE, our understanding is similar to yours. This reduces any scope of confusion with RV usage. |
| OPPO | In our view, as long as one slot is determined as available prior to the first actual transmission of the repetitions, it will be regarded as one transmission occasion when determining the RV cycling, no matter the transmission occasion would be omitted or not. We would like to clarify the meaning of “Each available slot identified by the UE”. Does it mean all slots determined as available prior to the first actual transmission of the repetitions, or just the slots UE transmit PUSCH actually? |
| LG | We don’t think this proposal is necessary but ok to make agreement for clarification. |
| Nokia/NSB | It’s still unclear that the “available slots” identified by the UE is on which step, i.e., for counting or after applying omission rules? |
| Xiaomi | Support the proposal. |

[Open] Issue#2-3: Inter-Slot Frequency Hopping Cycle

FL proposal #2-3 for the 2nd round discussion

- For PUSCH repetition Type A without joint channel estimation, inter-slot frequency hopping is based on physical slot index as in Rel-15/16.

Companies are invited to provide their views on the above proposal#2-3.

| Company | Comments |
| --- | --- |
| Samsung | No need for this agreement. |
| Lenovo, Motorola Mobility | Fine with proposal, but not really needed |
| Ericsson-2rd | Looks fine. Even with joint channel estimation, could you clarify why FH cannot be based on physical slots? In our understanding it’s just every N physical slots instead of every 1 physical slot that the start PRB is changed, where in the N is the number of physical slots that phase coherency is kept. It seems physical slots is more reasonable if the phase coherency is supposed to be kept in time no matter whether DL/UL/Flexible slots are included in the time window. Anyway, we also agree that this might be good to be discussed together with JCE agenda to have some unified solution for FH pattern determination. |
| ZTE | Support. We prefer to have an explicit agreement to conclude. |
| Apple | As this proposal has no specification impacts. Maybe it can be treated as a conclusion. |
| CMCC | No need for this discussion. |
| Panasonic | Although the operation itself is reasonable, we don’t think the agreement is not needed. |
| Intel | We are fine with the proposal, but may not needed. |
| Sharp | Fine with the proposal. |
| CATT | OK with such conclusion, though it seems just the same with current specification. |
| Qualcomm | We don’t see the connection to 8.8.1.3 that other companies seem to be seeing. Hopping based on transmission occasions/available slots has its merits compared to physical slots. The issue here is uneven distribution of hops. Consider the following TDD pattern: DDDU. If PUSCH with 2 repetitions is scheduled along with inter slot frequency hopping, then in this case, no hopping actually occurs if we simply follow the current spec! This is not ideal and a solution to fix this is worth considering. For now, prefer to present both alternatives, and we can downscope in the next meeting. Might give companies more time to assess pros and cons. |
| OPPO | We see this topic should be decided in thread 3. |
| LG | Fine to make it as conclusion. |
| Nokia/NSB | We are fine with the proposal. |
| Xiaomi | Fine with the proposal |
| Huawei, HiSilicon | OK. |

[Closed] Issue#2-4: Timeline aspect for the detemination of available slots

Looking at the inputs for the 1st round discussion, the companies’ opinions on this issue are largely correlated with those of Issue#2-1. Therefore, it is suggested discussing this aspect together with Issue#2-1.

[Open] Issue#2-5: Semi-static configurations to be used for the detemination of available slots

There seems to be two options to deal with ssb-PositionsInBurst (i.e. SSB configuration).

- Alt 1: If PUSCH symbol is a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

- Alt 2: If PUSCH symbol is a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as available during the counting of repetitions. As PUSCH overlaps with SSB, the PUSCH in the slot is omitted.

Companies are invited to provide their views on the above alternatives.

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We support Alt 1for counting based on available slots. |
| Ericsson-2rd | Alt 2 is preferred, since the omission rules for determining the transmissions of uplink channels colliding with SSB symbols are already specified in current spec. which should be reused and is enough in our view. For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. If a UE - is configured with multiple serving cells and is provided half-duplex-behavior = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in any of multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon, when provided to the UE, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in any of multiple serving cells.. |
| ZTE | Support Alt 1. Alt 2 will cause the number of actual transmitted repetitions smaller than Alt1. |
| Apple | Try to understand the proposal. First, it seems there is editorial error. “If PUSCH symbol is in a slot overlaps with flexible symbol(s) with SSB transmission…..” Second, does the flexible symbol here mean the guard symbol for DL to UL switching? “If PUSCH symbol is in a slot overlaps with flexible symbol(s) with for SSB transmission reception…..” Third, even above updates are the right understanding, the UL symbols could not be used for transmission if it is occupied by switching. So the proposal could be further updated as. “If PUSCH symbol is in a slot overlaps with flexible symbol(s) and/or UL symbol(s) with for SSB transmission reception…..” If our understanding on this proposal is correct, Alt 1 is preferred. BTW, we would like to align the understanding on previous meeting’s agreements. “symbol not intended for UL transmissions” means only DL symbol or both DL symbol and flexible symbol? We think it only refers to DL symbols. Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions |
| CMCC | Support Alt 1. |
| Panasonic | We support Alt.1. |
| Intel | We support Alt. 1 Flexible symbols with SSB transmission can be considered as part of RRC configuration. We should use this for the determination of available slots. |
| NTT DOCOMO | We support Alt.1. |
| Sharp | We prefer Alt 1, as even Rel-15 PUCCH repetition supports it to avoid the unnecessary dropping. Alt 2 reduces the actual repetitions when they overlap with SSB. |
| WILUS | Alt 1. Symbols configured for SSB transmission can be regarded as seme-static DL symbols. Therefore, if a symbol of PUSCH transmission overlaps with a symbol for SSB in a slot, the slot is not counted as available slot for UL transmission, and defer the PUSCH repetition to the next available slot. |
| vivo | Support Alt.1. |
| CATT | We think the ‘flexible symbols’ here does not (only) mean the flexible symbols for DL-UL switching, but a general scenario for SSB transmission in the symbols that marked as flexible by TDD configuration. Currently for repetition type A counted on ‘physical’ slots, Alt2 is more aligned with the legacy manner. But now we recognize that it may be beneficial to adopt Alt1 for larger number of actual transmission. |
| Qualcomm | Support Alt 1. |
| OPPO | We are discussing the determination of available slots whether or not depend on the flexible symbol(s) with SSB transmission in this section, and the counting is on the basic of available slots. There is no necessary to add “during the counting of repetitions”. We propose modifying as follows: - Alt 1: If PUSCH symbol is in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot. - Alt 2: If PUSCH symbol is in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as available during the counting of repetitions. As PUSCH overlaps with SSB, the PUSCH in the slot is omitted, and is still counted in the total number of repetitions for PUSCH repetition. |
| LG | Support Alt 1, which is aligned with the available slots determination mechanism for Rel-16 PUCCH repetitions. |
| Nokia/NSB | We support Alt. 1, which is more aligned with the goal of this feature, i.e., “increasing the number of repetitions”. |
| Xiaomi | Support alt1. |
| Huawei, HiSilicon | Alt.1 |

[Open] Issue#2-6: Special slot handling

FL proposal #2-6 for the 2nd round discussion

- The agreement in RAN1#104-e is applied to all slots including special slots.

Provide your concern, only if the above proposal is not acceptable.

| Company | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | We support the FL proposal |
| Ericsson-2rd | If in legacy special slot is treated for Type A PUSCH repetition, it will be treated for enhanced Type A PUSCH repetition in Rel-17 in the same ways as a normal slot. No further agreement is needed specifically for special slot. |
| ZTE | Support |
| Apple | Agree with this clarification. |
| CMCC | Support the FL proposal. Though we understand the intention after checking the section 3.2, when going into the agreements, the proposal should be more specific for checking thereafter. |
| Panasonic | We support the FL proposal. |
| Intel | We are fine, but our view is that we do not need this proposal as this is already agreed. |
| NTT DOCOMO | We support the FL proposal. |
| Sharp | Support the proposal. |
| WILUS | We support the FL’s proposal. |
| CATT | OK |
| OPPO | Support. |
| LG | Support the proposal for clarification. |
| Nokia/NSB | We support the FL proposal. Maybe the exact agreement in RAN1#104-e can be quoted in the final proposal to avoid confusion on which agreement in RAN1#104-e is applied. |
| Xiaomi | Support |

[Pending] Issue#2-7: Limitation of overall duration of PUSCH repetitions

Issue#2-7 will be discussed after concluding the discussion on Issue#2-1.

[Open] Issue#2-8: Enhancements on PUSCH dropping

Although 16 companies expressed that the existing collision handling rules is sufficient, 8 companies were open to discuss once some issue is identified due to the other WI’s works. So far, SRS with symbol length 8 and 12 which is discussed in feMIMO WI is raised as a possible UL transmission which potentially requires some handling of collisions with PUSCH repetitions. How it affects PUSCH repetition is still not so clear. For SPS HARQ-ACK discussed in IIoT/URLLC WI, they may be likely to define dropping rules by themselves if necessary.

FL proposal #2-8 for the 2nd round discussion

- Whether/how the UL transmissions investigated in the other WIs affect PUSCH repetition Type A should be identified first. Once identified, revisit Issue#2-8.

| Company | Comments |
| --- | --- |
| Samsung | OK |
| Ericsson-2rd | It would be good if we can have an proposal to summarize that - Reuse existing collision handling rules for collision between enhanced Type A PUSCH repetitions and other UL channels, FFS the collision with UL channels/signals introduced in other topics in Rel-17 if identified. |
| ZTE | Fine with the intention of this proposal, while we feel no need to have an agreement/conclusion for it now. |
| Apple | OK |
| CMCC | Fine with the proposal. |
| Panasonic | We are fine with the FL suggestion. |
| Intel | We are fine to discuss it further, but we think it is important to reuse the existing collision handling mechanism. |
| Sharp | Fine with the proposal. Also OK with Ericsson’s suggestion. |
| InterDigital | Support the FL’s proposal. |
| CATT | We are fine to have a conclusion. |
| Qualcomm | Prefer to go with Ericsson’s version. |
| OPPO | We are OK with the proposal. |
| LG | OK |
| Nokia/NSB | We are fine with the suggestion from the FL. |
| Xiaomi | OK |
| Huawei, HiSilicon | A clarification is needed. In case of SRS transmission overlapping with PUSCH repetition Type A, in Rel-15/16 spec, it requires gNB scheduling to avoid such overlap, e.g. a P-SRS is configured at the last two symbols of a slot, then an 8-repetition can only be scheduled with the first 12 symbols of each slot, which makes the 2*7=14 symbols wasted. As the number of repetition is increased, such waste becomes larger. Therefore, we suggest to discuss the case before decide to reuse existing collision handling rules. |

[Closed] Issue#2-9: Enhancement on UCI multiplexing on PUSCH repetition

Since most of the companies expressed that this issue is out of CovEnh WI scope and/or this issue does not need to be discussed in this agenda, Issue#2-9 discussion is closed.

[Open] Issue#2-10: Configuration/indication of CovEnh functions

It seems companies’ views on configuration/indication of CovEnh functions diverge. In the 2nd round discussion, it is suggested focusing on some particuler aspects.

- Question #1: Should dynamic switching between two counting methods, i.e. the counting based on physical slots and the counting based on available slots, be supported?

- Question #2: When gNB and UE are both capable of the counting based on available slots, is the counting based on available slots always used? (i.e. The counting based on available slots cannot be deactivated in this case.)

- Question #3: When gNB and UE are both capable of the increased maximum repetition number, is it still allowed to configure Rel-16 TDRA-list-based PUSCH repetition Type A?

Companies are invited to provide their answers to the above two questions.

| Company | Comments |
| --- | --- |
|  | Answer to Question #1: Yes/No Answer to Question #2: Yes/No Answer to Question #3: Yes/No Other comments: |
| Samsung | Question #1: No. There is absolutely no technical reason to support and such support would complicate the specifications and introduce problems/error cases. Question #2: No. There is no reason to force the network choice when it has no impact on the UE procedures – that is not something that RAN1 should be doing or is allowed to do. Also, there can be benefit to have the counting based on physical slots as the network can easily predict when the repetitions end (even if only RRC based configurations are considered). Question #3: Yes. Again, there is no reason to force the network behavior or any role for RAN1 to be making such decisions. Overall, no agreement on any of the above questions is needed. |
| Lenovo, Motorola Mobility | Question#1: Yes, we think that implicit method should be applied to determine whether counting is done based on available slots or physical slots. This can be done based on number of repetitions indicated by the signaled index of TDRA. For example, if the number of repetitions is greater than 16, then counting is done based on physical slots, otherwise, counting is done based on available slots. (and RRC configuration can be used to enable/disable Rel-17 PUSCH repetition type A) Question#2: No, only if Rel-17 PUSCH repetition type A is indicated (enabled), for example by RRC indication Question#3: Yes, it should be up to network to configure |
| Ericsson-2rd | The 2 enhancement features (option 1: increasing legacy maximum number of repetitions, option2: counting legacy repetition factors based on available slot) should be independent from each other. It should be up to network configuration to determine whether increasing number of repetitions is supported or counting based on available slot is supported, or whether legacy repetition is supposed. For option 1: a flag to indicate whether a legacy repetition factor is based on available slot or still based on physical slot is needed. For option 2: a separate Rel-17 TDRA table (for backward compatible) is needed to include new set of repetition factors with maximum number increased One of the options supported at the same is enough. To question 1: this is needed since for available slot counting feature, it’s just a matter of whether legacy repetition factors should be treated as counted based on available slot or physical slot, which has nothing to do with the new TDRA table to be introduced for the other feature “increasing number of repetitions based on physical slots”. To question 2: for enabling/disabling counting legacy repetitions based on available slot, it’s up to gNB to configure whether available slot counting is configured/reconfigured or not, isn’t this similar to Rel-16 when we introduced Rel-16 repetition? To question 3: for increasing legacy maximum number of repetitions, when Rel-16 TDRA table is also provided, Rel-17 can overwrite Rel-16 TDRA table. E.g. as is shown in below table, in Rel-16 the repetition factor is determined as following on top of the aggregation factors defined in Rel 15 For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1.. |
| ZTE | Question #1: No. Regarding Lenovo’s comments, we don’t think we should limit the counting to be based on available slots only, when the number of repetition is small, and limit the counting to be based on physical slots only, when the number of repetition is large. Question #2: No. It’s up to gNB configuration. Question #3: Yes. It’s up to gNB configuration. |
| Apple | Question #1: No This is related to UE capability discussion. If two counting methods are separate UE features, then dynamic switching is not applied to all UEs. Even for UE with both capabilities, we don’t see the strong motivation to supporting dynamic switching, dynamic switching indication via DCI? Question #2: No It’s up to network scheduling/configuration which counting is applied. Question #3: No Rel.17 solution is enhancement of Rel.16, if UE supports the Rel.17 feature already, which network use Rel.16 configuration. |
| CMCC | Question #1: No. The dynamic switching would complicate the specification. And we see no motivation why UEs supporting the repetitions based on available slot would fall back to the counting based on physical slots. Question #2: Yes. When the UE reports that Rel-17 CE are supported, based on the RRC configuration, UE should count the repetitions based on the available slots. Question #2: Yes. It should based on gNB configurations. Please FL further clarify the motivation of the Question #3 ? For the question #2 and #3, from my understanding, there is no need to add “gNB” after the when. |
| Panasonic | Question #1: Yes/No. Both RRC based configuration only or dynamic switching indicating repetition type in the TDRA table can be further considered. Question #2: No. It is up to network configuration. Question #3: Yes. It is up to network configuration. |
| Intel | Answer to Question #1: No. RRC configuration should be sufficient. No need for dynamic switching. Answer to Question #2: No. It depends on the configuration. Answer to Question #3: Yes. It depends on the configuration. |
| NTT DOCOMO | Question #1: No. We don’t see any motivation to support the dynamic indication. Question #2: No. NW cam configure. Question #3: Yes. NW can configure. |
| Sharp | Question #1: No Question #2: No. Agree with Samsung. Irrespective of whether “the counting based on available slots” is a basic CovEnh feature or not, some kind of the configuration to activate/deactivate the function is necessary. Otherwise, the network capable of “the counting based on available slots” is forced to use that function. Question #3: Yes. Agree with Samsung. With this reason, some kind of the configuration to activate/deactivate the function, i.e. the increased maximum repetition number, is necessary. |
| vivo | Answer to Question #1: No Both of these options are designed to achieve the same goal, it is not necessary to configure these two features simultaneously, and even dynamic switching between these two options. Answer to Question #2: No It is up to NW configuration, NW can configure type-A repetition based on consecutive slots only. Answer to Question #3: Not sure In our understanding, numberOfRepetitions-r17 can be provided for enhanced type-A repetition, in addition to numberOfRepetitions-r16. NW can provide the same max number in numberOfRepetitions-r17 as that for numberOfRepetitions-r16. While from signalling design perspective, it does not necessary mean exactly the same RRC parameter. |
| CATT | Answer to Question #1: No No benefit is foreseen for dynamic switching between different counting rules. Answer to Question #2: No From UE point of view, a feature should be up to gNB configuration, after the UE reports the capability. If configured, then follow new counting method based on available slots. Answer to Question #3: Yes Similar to Question #2, a feature should be up to gNB configuration, after the UE reports the capability. If configured. If the gNB configures the increased maximum number, then use Release-17 IE with increased maximum number. If not, follow legacy parameter. |
| Qualcomm | Q1: No. Q2: Yes. UE unlikely to maintain two implementations. |
| OPPO | Question #1: No. Question #2: No. Spec. can not restrict the NW configuration. Question #3: Yes. But this will depend on UE capability report of Rel-16 features. |
| LG | Question #1: No In case of FDD, dynamic/semi-static switching between counting methods seems not necessary. For TDD case, we don’t see any motivation for dynamic switching. Before the discussion, the necessity of semi-static switching should be discussed first. For Question #2 and #3, we’d like to handle these issues in UE capability discussion. |
| Nokia/NSB | Answer to Question #1: No We don’t see any use case for dynamic indication. Why do we need a dynamic switching while both features solve the same issue? Answer to Question #2: No Answer to Question #3: Yes For both Questions #2 and #3, the NW should have full flexibility to configure. |
| Xiaomi | Q1:No Q2:Yes Q3: Yes It depends on the configuration. |

[Closed] Issue#2-11: Modification on βoffsetPUSCH

Since most of the companies expressed that this issue is out of CovEnh WI scope and/or this issue does not need to be discussed in this agenda, Issue#2-11 discussion is closed.

[Closed] Issue#2-12: Other issues

Void

Summary of the 2nd round discussion

Issue#1-1: The maximum number of repeitions

Companies’ views on the maximum number of repetitions for each counting method (i.e. the counting based on physical slots and the counting based on available slots) seems diverging. It is suggested listing alternatives for further down-selection in the future round of discussions.

FL proposal #1-1 after the 2nd round discussion

- Select one of the following Alt 1 and Alt 2 depending on whether two enhancements (i.e. the increased maximum repetition number and the repetitions counted based on available slots) can be used simultaneously or not (under the discussion in Issue#2-10),

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

    - Qualcomm, CATT, vivo, Xiaomi, ZTE, Sharp, OPPO, China Telecom (1st choice), Samsung, Apple, LG, Nokia/NSB, Xiaomi

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: X for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

  - FFS: X is either 32 or 20, to be down-selected in RAN1#105-e

    - X=32: Intel, Panasonic, China Telecom (2nd choice), Lenovo/Motorola Mobility, CMCC, NTT DOCOMO, Huawei, HiSilicon

    - X=20: Ericsson, Lenovo/Motorola Mobility, CMCC, Huawei, HiSilicon

Issue#1-2: RRC parameters to be extended for supporting the increased maximam number

Whether repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig supports the increased maximum number of repetitions or not was discussed in the 1st and 2nd round discussions. Companies’ views provided so far are summarised as follows.

- Support (8 companies): Intel, China Telecom, Samsung, LG, OPPO, Xiaomi (IEs are up to RAN2), Nokia/NSB

- Not support (13 companies): Qualcomm, ZTE, Apple, Ericsson, NTT DOCOMO, Sharp, CATT, CMCC, Lenovo/Motorola Mobility, Sierra Wireless, vivo, XIaomi

Based on the current situation, at this moment there is no consensus to support the increased maximum number of repetitions by the repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig. Proponents can try to convince the other party in the future discussions.

Issue#1-3: Other candidate value set for configuration of the number of repetitions

Void

Issue#1-4: Other issues

Void.

Issue#2-1: Use of dynamic signaling for the detemination of available slots

In the 1st round discussion, the following alternatives were dicussed.

- Alt 1: The determination of available slots does not depend on any dynamic signaling. Further omission of PUSCH repetition in the available slot is subject to dynamic signaling.

  - Support (19 comnapies): Qualcomm, Intel, China Telecom, vivo, Apple, Ericsson, NTT DOCOMO, Sharp, CATT, LG, OPPO, Xiaomi, CMCC, NEC, WILUS, InterDigital, Nokia/NSB, Sierra Wireless

- Alt 2 (4 companies): The determination of available slots depends on dynamic signaling including e.g., dynamic SFI.

  - ZTE (at least dynamic SFI), Samsung, Lenovo/Motorola Mobility

- Alt 3 (2 companies): The determination of available slots depends on dynamic signaling in the scheduling DCI only.

  - Panasonic, OPPO (1st preference)

The 2nd round discussion focused on several concerns to Alt 1 which were raised during the 1st round discussion. The biggest concern was that mechanism without considering dynamic signaling (e.g. dynamic SFI) would lead to the restriction on the use of the dynamic signaling. However, the proponents of Alt 1 argued that no additional restriction is necessary, since, even with Alt 1, dynamic signaling is referred to for PUSCH dropping in exactly the same way as in Rel-15/16. It is suggested making further clarifications on Alt 1 of the previous agreement, and agreeing Alt 1 with this clarification.

FL proposal #2-1 after the 2nd round discussion

Take Alt 1 of the agreement in RAN1#104-e as an agreement with the following clarifications

- Alt 1 of the agreement in RAN1#104-e includes the following two steps

  - Step 1: Determination of available slot for counting K repetitions, where the determination is based on RRC configurations.

  - Step 2: If any additional collision happens due to e.g., dynamic SFI, UL CI, priority indication, or some RRC configurations (as discussed under Issue#2-5), etc., the PUSCH repetition is omitted, but is not further postponed.

Issue#2-2: RV Cycle

In the 2nd round discussion, 19 companies provided their views on the main bullet of the following proposal, and only 1 company didn’t think this discussion was necessary. 4 companies preferred further clarifications on whether “available slots” is for counting or after applying omission rules. Therefore, it is suggested adding the note saying “RV is cycled for a transmission occasion, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.”

FL proposal #2-2 after the 2nd round discussion

- Each available slot identified by the UE is treated as a transmission occasion for PUSCH repetition.

  - Note: RV is cycled for a transmission occasion, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not.

Issue#2-3: Inter-Slot Frequency Hopping Cycle

- For PUSCH repetition Type A without joint channel estimation, inter-slot frequency hopping is based on physical slot index as in Rel-15/16.

There were 17 inputs to the above proposal during the 2nd round discussion

  - Support (12 companies): ZTE, Apple, Lenovo/Motorola Mobility, Sharp, CATT, LG, Nokia/NSB, Xiaomi, Huawei, HiSilicon

  - No need for this agreement (4 companies): Samsung, CMCC, Panasonic, Intel

  - Good to discuss with AI8.8.1.3 (2 companies): Ericsson, OPPO

  - Need to consider (1 company): Qualcomm (Revisit in the next meeting)

Although the majority is thinking Rel-16 inter-slot frequency hopping works with the counting based on available slots, several companies are still thinking some more discussions may be necessary. Since this is the first meeting to discuss this issue, we can revisit this issue in the next meeting if any new analysis is brought up.

FL proposal #2-3 after the 2nd round discussion

- For enhancement on inter-slot frequency hopping cycle, revisit in the next meeting if the further analysis is provided, e.g., assessment of pros/cons, any relation with the discussions in AI 8.8.1.3.

Issue#2-4: Timeline aspect for the detemination of available slots

Void.

Issue#2-5: Semi-static configurations to be used for the detemination of available slots

In terms of use of with ssb-PositionsInBurst for Rel-17 PUSCH repetitions counted based on the available slots, the companies’ inputs are summarized as follows.

- Alt 1: If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

  - Support (19 companies): Lenovo/Motorola Mobility, ZTE, Apple (both flexible and UL symbols for SSB receptions), CMCC, Panasonic, Intel, NTT DOCOMO, Sharp. WILUS, vivo, CATT, Qualcomm, LG, Nokia/NSB, Xiaomi, Huawei, HiSilicon

- Alt 2: If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as available during the counting of repetitions. As PUSCH overlaps with SSB, the PUSCH in the slot is omitted.

  - Support (1 company): Ericsson

Since the clear majority supports Alt 1, it is suggested going with Alt 1. And discuss the other RRC configurations in the next round discussion.

FL proposal #2-5 after the 2nd round discussion

- If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot.

Issue#2-6: Special slot handling

In the 2nd round discussion, 17 companies provided their view on the following proposal, and there was no objection though 2 companies thought no additional agreement was necessary. Based on the situation, it is suggested making it as a conclusion.

FL proposal #2-6 after the 2nd round discussion

Conclusion:

- The following agreement in RAN1#104-e is applied to all slots including special slots.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

Issue#2-7: Limitation of overall duration of PUSCH repetitions

Void.

Issue#2-8: Enhancements on PUSCH dropping

In the 2nd round discussion, 16 companies provided their view on the following proposal, and there was no objection. 2 companies preferred stating “Reuse existing collision handling rules” that had been proposed by Ericsson. It is suggested going with Ericsson’s version.

FL proposal #2-8 after the 2nd round discussion

- Whether/how the UL transmissions investigated in the other WIs affect PUSCH repetition Type A should be identified first. Once identified, revisit Issue#2-8.

- Reuse existing collision handling rules for collision between enhanced Type A PUSCH repetitions and other UL channels, FFS the collision with UL channels/signals introduced in other topics in Rel-17 if identified.

On the other hand, 1 company proposed discussing collision handling between SRS and PUSCH repetition Type A (e.g. support of partial dropping of PUSCH when overlapping with SRS). It is suggested collecting companies’ views on this particular case in the next round of discussion.

Issue#2-9: Enhancement on UCI multiplexing on PUSCH repetition

Void.

Issue#2-10: Configuration/indication of CovEnh functions

During the 2nd round discussion, several aspects related to configuration/indication of CovEnh functions were discussed.

The first aspect was whether to support dynamic switching between two countring methids. 4 compnaies were in favor of it while 15 companies did not want to support it. Seems futher discussions is necessary.

- Dynamic switching between two counting methods, i.e., the counting based on physical slots and the counting based on available slots.

  - Support (4 companies): Lenovo/Motorola Mobility, Ericsson, Panasonic (can be further considered)

  - Not support (15 companies): Samsung, ZTE, Apple, CMCC, Intel, NTT DOCOMO, Sharp, vivo, CATT, Qualcomm, OPPO, LG, Nokia/NSB, Xiaomi

The second and third aspects were configurability of two CovEnh enhancements.

The second aspect is whether to support configurability of “the counting based on available slots” function. The large majority supports the configurability.

- Rel-17 supports the configurability of “the counting based on available slots” function. A UE uses “the counting based on available slots” function if Rel-17 IE activates the function, otherwise the UE uses the legacy counting.

  - Support (15 companies): Samsung, Lenovo/Motorola Mobility, Ericsson, ZTE, Apple, Panasonic, Intel, NTT DOCOMO, Sharp, vivo, CATT, OPPO, Nokia/NSB

  - Not support (The UE uses “the counting based on available slots” function if the corresponding UE capability is reported.) (3 companies): CMCC, Qualcomm, Xiaomi

  - Discuss in UE feature discussions (1 company): LG

The second aspect is whether to support configurability of “the increased maximum number of repetitions” function. The large majority prefers having the configurability.

- Rel-17 supports the configurability of “the increased maximum number of repetitions” function. A UE uses “the increased maximum number of repetitions” function if Rel-17 IE activates the function, otherwise the UE uses the legacy maximum repetition factors.

  - Support (16 companies): Samsung, Lenovo/Motorola Mobility, Ericsson, ZTE, CMCC, Panasonic, Intel, NTT DOCOMO, Sharp, vivo, CATT, OPPO, Nokia/NSB, Xiaomi

  - Not support (1 company): Apple

  - Discuss in UE feature discussions (1 company): LG

In the next round discussion, it is suggested discussing whether to support simultaneous use of two enhancements by a single UE capable of both enhancements.

FL proposal #2-10 after the 2nd round discussion

- Rel-17 supports the configurability of “the counting based on available slots” function.

  - Note: whether “the counting based on available slots” is a basic function for CovEnv or not will be discussed later in UE feature dicsussions.

- Rel-17 supports the configurability of “the increased maximum number of repetitions” function.

- FFS: whether to support simultaneous use of the above two functions by a single UE capable of both functions.

Issue#2-11: Modification on βoffsetPUSCH

Void.

Issue#2-12: Other issues

Void.

Issues for the 3rd round discussion

To be updated

Summary of the 3rd round discussion

To be updated

Issues for the 4th round discussion

To be updated

Summary of the 4th round discussion

To be updated

References

- R1-2104240	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

- R1-2104291	Discussion on enhancements on PUSCH repetition Type A for low latency requirement	NICT, TOYOTA MOTOR CORPORATION

- R1-2104330	Discussion on enhanced PUSCH repetition type A	ZTE

- R1-2104376	Discussion on enhancement for PUSCH repetition type A	vivo

- R1-2104537	Discussion on enhancements on PUSCH repetition type A	CATT

- R1-2104625	Discussion on enhancements on PUSCH repetition type A	CMCC

- R1-2104685	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

- R1-2104792	Enhancements on PUSCH repetition type A	OPPO

- R1-2104846	Enhancements on PUSCH repetition type A	China Telecom

- R1-2104859	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

- R1-2104919	Enhancements on PUSCH repetition type A	Intel Corporation

- R1-2105119	Discussion on PUSCH repetition type A enhancement	Apple

- R1-2105146	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

- R1-2105255	Discussion on PUSCH repetition type A	NEC

- R1-2105325	Enhancements on PUSCH repetition type A	Samsung

- R1-2105488	Discussions on PUSCH repetition type A enhancements	LG Electronics

- R1-2105511	Design considerations for PUSCH repetition Type A Enhancements	Sierra Wireless, S.A.

- R1-2105575	Enhancements on PUSCH repetition type A	Xiaomi

- R1-2105640	Enhancements on PUSCH repetition type A	Sharp

- R1-2105652	PUSCH Repetition Type A Enhancement	Ericsson

- R1-2105711	Enhancement on PUSCH repetition type A	NTT DOCOMO, INC.

- R1-2105773	Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

- R1-2105877	Discussion on enhancements on PUSCH repetition type A	WILUS Inc.

- R1-2105901	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

List of agreements

Agreements in RAN1#104-e

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

-        Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

-        Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

Agreements in RAN1#105-e