3GPP TSG-RAN WG1 Meeting #105-e	R1- 2106340

e-Meeting, May 10th – 27th, 2021

Title:	DRAFT Reply LS on TA pre-compensation

Reply to:	LS on TA pre-compensation (R2-2104376)

Release:	Release 17

Work Item:	NR_NTN_solutions-Core

Source:	Moderator (OPPO), [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Hao Lin

#### E-mail Address:	lin.hao@oppo.com

Attachments:	None

1. 	Overall Description:

RAN1 would like to thank RAN2 for sending their LS on TA pre-compensation. RAN1 has discussed the questions asked by RAN2 and RAN1 answers are provided below.

RAN2 Q2: RAN2 respectfully requests RAN1 to provide input on: (i) how UE determines UE-gNB RTT, and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

RAN1 answer: In RAN1#105-e meeting, RAN1 reached the following agreement on UE-gNB RTT determination, which is used to delay the starts of ra-ResponseWindow and msgB-ResponseWindow. The UE-gNB RTT is estimated according to the UE's TA and K_mac. RAN2 may take this information into account for future discussion.

Agreement:

The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT.

- The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac.

Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by  NTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×TcTTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc. The estimate of gNB-satellite RTT is equal to the sum of NTA,common×Tc and K_mac.  How to treat NTA and NTA,offset can be further discussed.

Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0.

Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed.

Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

2. 	Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account for future work.

3. 	Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #106-e 		16 – 27 August 2021		Electronic Meeting

TSG-RAN WG1 Meeting #106-bis-e 	11 – 19 October 2021		Electronic Meeting