3GPP TSG-RAN WG1 Meeting #105-e	R1-21xxxxx

e-Meeting, 19th – 27th May 2021

Agenda Item:	6.2.1

Source:	Moderator (Ericsson)

Title:	FL summary on clarification of UE procedure for UL multi-TB scheduling in TDD for Rel-16 LTE-MTC

Document for:	Discussion, Decision

# 1	Introduction

This document provides a summary of the following RAN1 email discussion.

[105-e-LTE-eMTC5-02] Clarification of UE procedure for UL multi-TB scheduling in TDD for LTE-MTC – Johan (Ericsson) - Discuss and decide on the potential clarification in 36.213 discussed in these contributions: - R1-2105267, “Clarification on UE procedure for uplink MTB scheduling in TDD”, ZTE - R1-2105268, “Discussion on UE procedure for uplink MTB scheduling in TDD”, ZTE - Discussion and decision by May 24, TPs by May 27

# 2	Discussion

Contribution [2] discusses a potential need to clarify the UE procedure for UL multi-TB scheduling in CE mode A in TDD UL/DL configuration 0. In this TDD UL/DL configuration, the ‘UL index’ field in the DCI achieves dual-TB scheduling already since Rel-13. Section 2.1 in [2] makes the following observations and proposal:

Observation 1: For TDD configuration 0, when repetition number N = 1 and UL index = ‘11’, the corresponding HARQ process number for the two scheduled TBs are consecutive.

Observation 2: When 2 TBs are scheduled, the HARQ process number determined by ‘UL index’ and ‘Scheduling TBs for Unicast’ field are conflicted.

Observation 3: When more than 2 TBs are scheduled, the 2 HARQ process number determined by UL index is conflicted with the HARQ process number indicated by ‘Scheduling TBs for Unicast’ field if multi-TB scheduling feature is configured.

Proposal 1: When ce-PUSCH-MultiTB-Config is configured and ‘UL index’ in DCI format 6-0A is set as ‘11’, if multiple TBs are scheduled, further clarification is needed on how to determine the HARQ process number for each TB.

Question 1: Companies are invited to comment on the observations and proposals listed above.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | We agree the observation 1-3 For the proposal part: When ce-PUSCH-MultiTB-Config is configured, no matter how many TBs are scheduled, even only one TB is scheduled, the UL index in DCI format 6-0A should not set as “11”. If one TB is scheduled by ‘Scheduling TBs for Unicast’ field, and we use “UL index = ‘11’” to schedule 2 TB, this is another confliction. |
| Qualcomm | We agree with the analysis. We think the simplest way would be to disallow ‘11’ when multiple TBs are scheduled. |

Section 2.2 in [2] lists the following potential solutions and proposes to adopt Option 1. A draft 36.213 CR corresponding to Option 1 has been provided in [1].

- Option 1: UE is not expected to receive DCI format 6-0A with both the MSB and LSB of ‘UL index’ set to 1 if multiple TBs are scheduled when ce-PUSCH-MultiTB-Config is configured.

- Option 2: When ce-PUSCH-MultiTB-Config is configured, the ‘UL index’ field is ignored.

- Option 3: The multi-TB scheduling feature is not supported for TDD configuration 0.

- Option 4: When UL index = 11 and ce-PUSCH-MultiTB-Config is configured, only single TB scheduling is supported.

Question 2: Please comment on the options listed above and express your preference, if any.

| Company | Comments |
| --- | --- |
| Lenovo, MotoM | Based on the observation, if we want to down select from the 4 options above, we prefer option 1 in general. |
| Qualcomm | We think Option 1 is preferred (it is not clear to us what is the difference with Option 4, though). We think the CR can be simplified as follows: with both the MSB and LSB of the UL index set to 1 when N>1 or NTB>1. |

# References

- R1-2105267, “Clarification on UE procedure for uplink MTB scheduling in TDD”, ZTE

- R1-2105268, “Discussion on UE procedure for uplink MTB scheduling in TDD”, ZTE