3GPP TSG RAN WG1 Meeting #105-e                                          	                    R1-210xxxx

e-Meeting, May 19th – 27th, 2021

Source:	Moderator (CATT)

Title:	              Feature lead summary #1 on AI 7.2.4 Sidelink synchronization mechanism

Agenda Item:	7.2.4

Document for:	Discussion and Decision

- Introduction

This feature lead summary document captures the remaining issues of sidelink synchronization mechanism aspects for Rel-16 NR V2X based on the submitted contributions [1]-[2]. The issue list can be found as following subsection.

- Issue list

| Issue# | Descriptions | Type | Tdocs |
| --- | --- | --- | --- |
| SY-1 | NR SL-TDD-Config in the coverage of eNB | Discussion with TP | [vivo] |
| SY-2 | Indication of the non-TDD case in sl-TDD-Config | Discussion with TP | [Sharp] |
| SY-3 | Clarification of the notation of “uslotsSL” | Discussion with TP | [Sharp] |

Issues/corrections proposed in contributions

NR SL-TDD-Config in the coverage of eNB

In inter-RAT deployment, eNB provides NR SL configurations. NR UE in coverage of eNB should use the LTE TDD configuration for PSBCH determination. It is not clear in current spec that how to determine the SL-TDD-Config in PSBCH in this case. The rules should be defined.

Contribution Proposals:

[vivo] Discussion with TP

Observation 1. The uplink resources of LTE TDD configurations are not always placed at the end of a period, therefore directly reusing the formulae agreed for NR TDD configuration conversion to determine SL-TDD-Config in the inter-RAT case is impossible.

Proposal 1. The codepoints 9~15 of a1, a2, a3,a4 when a0=0 can be used for LTE TDD configuration indication in PSBCH as follows, a5, a6, a7,a8, a9, a10, a11 are set to ‘1’.

Indication of LTE TDD Configuration (X=0)

| a1,a2, a3, a4 | LTE TDD configuration |
| --- | --- |
| 1, 0, 0, 1 | 0 |
| 1, 0, 1, 0 | 1 |
| 1, 0, 1, 1 | 2 |
| 1, 1, 0, 0 | 3 |
| 1, 1, 0, 1 | 4 |
| 1, 1, 1, 0 | 5 |
| 1, 1, 1, 1 | 6 |

TP#1 for 38.213: PSBCH content

------------------------------------------------------ Start of Draft TP of 213--------------------------------------------- 16.1 Synchronization procedures For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots. For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], - a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1'; else - when UE determines sl-TDD-Config based on tdd-UL-DL-ConfigurationCommon or sl-TDD-Configuration-r16 according to clause 5.8.9.4.3 in [TS 38.331] - a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 - a1, a2, a3,a4 are determined based on - P in pattern1 as described in Table 16.1-1 for a0=0 - P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1 where P and P2 are as described in Clause 11.1 - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix = "ECP"; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are the parameters of TDD-UL-ConfigurationCommon as described in Clause 11.1, or the parameters of sl-TDD-Configuration-r16 as defined in [9.3, TS 38.331] - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] - If tdd-Config as described in [12, TS 36.331] is provided - a0=0 - a1, a2, a3,a4 are determined based on Table 16.1-3 - a5, a6, a7,a8, a9, a10, a11 are set to '1' Table 16.1-1: Slot configuration period when one pattern is indicated a1, a2, a3,a4; Slot configuration period of pattern1 P (msec). 0, 0, 0, 0; 0.5. 0, 0, 0, 1; 0.625. 0, 0, 1, 0; 1. 0, 0, 1, 1; 1.25. 0, 1, 0, 0; 2. 0, 1, 0, 1; 2.5. 0, 1, 1, 0; 4. 0, 1, 1, 1; 5. 1, 0, 0, 0; 10. Reserved; Reserved. <Unchanged parts are omitted> Table 16.1-3: Indication of LTE TDD Configuration a1,a2, a3, a4; LTE TDD configuration. 1, 0, 0, 1; 0. 1, 0, 1, 0; 1. 1, 0, 1, 1; 2. 1, 1, 0, 0; 3. 1, 1, 0, 1; 4. 1, 1, 1, 0; 5. 1, 1, 1, 1; 6. --------------------------------------------------------- End of Draft TP ----------------------------------------------------

Indication of the non-TDD case in sl-TDD-Config

In latest version of TS 38.213, the description does not consider supplementary uplink band. It is proposed to add the case of shared SL carrier and SUL carrier as another “non-TDD” case in deriving sl-TDD-Config.

Contribution Proposals:

[Sharp] Discussion with TP#1

-------------------------------------------- Start of TP of 38.213 ------------------------------------------- < Unchanged parts are omitted > For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots. For paired spectrum or supplementary uplink band, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], - a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1'; else - a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 < Unchanged parts are omitted > -------------------------------------------- End of TP --------------------------------------------

Clarification of the notation of “uslotsSL”

In latest version of TS 38.213 for parameter sl-TDD-Config, “uslotsSL” is not clearly explained in any spec text. It is proposed to add clarification for “uslotsSL” in section 16.1 of TS 38.213.

Contribution Proposals:

[Sharp] Discussion with TP#2

-------------------------------------------- Start of TP of 38.213 ------------------------------------------- < Unchanged parts are omitted > For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], - a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to '1'; else - a0=0 if pattern1 is provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by sl-TDD-Configuration-r16 or tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 - a1, a2, a3,a4 are determined based on - P in pattern1 as described in Table 16.1-1 for a0=0 - P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1 where P and P2 are as described in Clause 11.1 - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of a number of UL slots uslotsSL, respectively < Unchanged parts are omitted > -------------------------------------------- End of TP --------------------------------------------

References

  - R1-2105464, “Maintenance on NR sidelink synchronization and procedure”, vivo, e-Meeting, 3GPP RAN1#105-e, May 19th – 27th, 2021.

  - R1-2105628, “Remaining issues on synchronization mechanism and QoS management for NR sidelink”, Sharp, e-Meeting, 3GPP RAN1#105-e, May 19th – 27th, 2021.